3GPP TSG RAN WG2 Meeting #131			R2-2505653

Bengaluru, India, August 25- 29, 2025

Agenda Item: 	8.2.4

Source:		Sony

Title:			Energy level indication

Document for:	Discussion & Decision

# Introduction

At RAN#106 meeting, a new WID for Ambient IoT in NR was approved including the following RAN2 objectives [1].

- RAN2 scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes. - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129). - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R. - Only MAC layer is included

At RAN2#130 meeting [2], the following agreements were made:

Agreements

- R2D message scheduling non-first segment (re)transmission does not include upper layer command.

- For the first segment and unsegmented packet (re)transmission, the “offset” indicator in R2D is not present.

- This implies that the R2D message will either have command or offset (but not both).  FFS whether we define two message types or one message type with optional fields.

Agreements

- The device is expected to send a MAC response to the reader in the D2R occasion.   The MAC response contains the NAS message if available at the D2R occasion.   If there is no NAS message available to transmit at the D2R occasion then the response contains MAC with 0 SDU and padding as needed.

- Send LS to CT1 to inform the agreement 1 to CT1 and explain that we have an issue with delayed NAS write success response.   RAN2 would prefer that this is handled by CT1 (and give the example of sending NAS response upon successful reception of write command).    Ask if this can be handled by CT1

Agreement on MAC PDU format

- A mandatory length field directly indicates the length of D2R data MAC SDU to support varying lengths of D2R data.    The size of length field is 7-bit in bytes.

- The offset indication for transmission/retransmission of the segments after the first segment of a D2R message is 7-bit length in bytes.  Segmented SDUs are also byte aligned.

- FFS D2R message type.  Current running CR will capture no message type,  but we can revisit this next meeting and also consider if any other bits are needed for the MAC header

- The length field inside MAC for SDU is not needed for R2D messages, assuming R2D MAC padding is not needed.  FFS can come back if padding is needed depending on granularity of TBS  (only if needed)

Agreements

-	For CBRA, to avoid AS ID being occupied for unnecessary time and to keep alignment between reader and device on AS ID release, device can release AS ID upon receiving paging message with different transaction ID, no matter the paging message is for it or not.   FFS for CFRA

-	FFS for need for release message

In this contribution, we discuss our view on energy level indication.

# Discussions on energy level indication

At the latest RAN2#130 meeting it was agreed to follow up on D2R message type, but also if any other bits may be need.

FFS D2R message type.  Current running CR will capture no message type,  but we can revisit this next meeting and also consider if any other bits are needed for the MAC header

It has been discussed and we have proposed earlier to have some sort of energy level indication, and we think that some bits e.g., in the MAC header could solve this  open question when there is not energy enough to transmit the whole data packet received from upper layers and which layer is aware of the amount of energy.

In the following we discuss potential alternatives to address this

Option 1: Upper layer knows energy level.

If the upper layer knows the amount of available energy, then it can decide to only respond to the reader if there is enough energy. This may be good for AS layer, since the AS layer does not have to be involved in energy status aspects.

The drawback is that upper layers may not know if packet for different reasons needs to be segmented or re-transmitted, which may lead to an energy shortage.

Option 2: MAC layer has the information.

Then MAC layer can do segmentation based on how large segment can be transmitted, before harvesting is needed.

This is good since MAC can know max Transport Block Sizes etc.

The drawback is that MAC may not have instantaneous information on the remaining level of stored energy during an ongoing transmission.

Option 3: PHY has the information.

For this case MAC can create e.g. max TRB of 1000 bits and push it down to PHY.

Then PHY has to do segmentation based on either energy level, or based on number of re-transmissions etc., if not handled in the MAC layer.

We think that MAC layer may be the best option, since MAC likely should have the best knowledge about amount of data to transmit as well as PHY configuration and energy storage status.

Proposal 1: RAN2 to agree to add bits in the MAC header for energy level indication.

# Summary

In this contribution, we have discussed our views on aspects related to segmentation and remaining bits, offset and energy level.

Proposal 1: RAN2 to agree to add bits in the MAC header for energy level indication.

# References

- RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR

- Minutes RAN2#130 meeting