3GPP TSG RAN WG2 Meeting #131			R2-2505658

Bengaluru, India, August 25- 29, 2025

Agenda Item: 	8.7.3

Source: 		Sony

Title: 			UE Assistance Information (UAI) for recommended gap cancellation ratio.

Document for: 	Discussion & Decision

# Introduction

eXtended Reality (XR) for NR phase 3 work item (WI) was revised in RANP#105 [1]. The WI aims for enhancements addressing system capacity, efficient and effective mechanisms to meet QoS requirements and lower device power consumption, in the context of the demanding scenarios and traffic characteristics requirements of XR, including also multi-modal applications. The WI also aims to realize system capacity gains by enabling transmission/reception in gaps/restrictions that are caused by RRM measurements, while keeping impact to mobility performance limited.

The Rel-19 XR ph3 objective that being addressed in this contribution is:

- Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4]

- Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4]

At RAN2#127bis [2], the following agreements were made related to RRM measurement gap skipping.

Agreements for RRM measurement gap skipping - RAN2 assumes that at least some impact on DSR from MG skipping can be avoided by NW implementation. FFS whether there is an impact which would require some specification changes/enhancements. - No need to have delay-aware LCP enhancements specific for MG skipping, i.e. MG skipping and delay-aware LCP are designed as independent features - RAN2 can further evaluate whether there is any impact on DRX from MG skipping. For the moment, the issue is unclear. - RAN2 will focus its work on supporting the solution chosen by RAN1/RAN4 - RAN2 can discuss whether there is a need to additionally have other solution (e.g. RRC-based) which can be decided after RAN1/RAN4 evaluation and knowing more details of DCI-based solution

At RAN2#128 [3], the following agreements were made:

Agreements on RRM measurement gaps impacts - No MG-specific enhancements is needed on DSR operation. - RAN2 assumes that UE follows DRX pattern as currently, even when MG is indicated as skipped - No MG-specific enhancements is needed for DRX operation.

At RAN2#129 meeting [4], the following agreements were made:

Agreements on MG skipping - From MAC perspective, the UE behaves as if there is no activated measurement gap during a skipped/cancelled measurement gap occasion. - “Cancelled” or “skipped” terminology will be aligned with RAN1 specifications when implementing changes in MAC. - RAN2 will not work on semi-static MG skipping solutions unless requested by RAN4/RAN1

At RAN2#130 [5]. the following agreements were made:

Agreements on measurement gap skipping - (RRC-05) UE reports the assistance information of recommended gap cancellation ratio when the configuration is received (and the UE has preference) or when the assistance information changes since the last report. - (RRC-05) It is left to UE implementation to decide whether the recommended gap cancellation ratio changes. - (RRC-06) A prohibit timer is used to limit frequent transmission of the UAI with recommended gap cancellation ratio. FFS the granularity of prohibit timer - Consider RAN4 progress on the granularity for the recommended gap cancellation ratio when drafting the RRC CR - (UE capability-03) RAN2 defines a per UE capability for reporting the ratio of gap occasions. And its prerequisite condition is that it supports enabling TX/RX during measurement gap scheduling restrictions by DCI.

This contribution provides our views on introducing support for UE Assistance Information for measurement gap skipping

# Discussion on UE assistance Information (UAI)

To support XR application, the UE may require continuing the transmission and skip the RRM measurement. And it has been agreed to support UE Assistance Information for the UE to report a recommended gap cancellation ratio.

RAN4 has been addressed to find a suitable ratio and RAN2 has received recommendation from RAN4

RAN2 received the following LS from RAN4, [6]

R2-2503328	Reply LS on UE assistance information (R4-2504972; contact: Nokia)	RAN4	LS in	Rel-19	NR_XR_Ph3-Core	To:RAN1, RAN2

Noted

RAN2 assumes granularity of indication should be decided by RAN4. If no input from RAN4 is received, then we will decide this in RAN2 next meeting.

Observation 1: It was noted that if no further information is received from RAN4, then RAN2 will have to decide.

At RAN4 meeting the following was agreed according to the LS.

Regarding UE assistance information related to measurement occasions, the following was agreed on RAN4#114 (R4-2502700):

## Issue 5-1: General on UAI <Agreement>: - Agree to define UAI based on the following assumptions, and further discuss the content of UAI - RAN4 will not specify any explicit or implicit requirement or expectation on gNB behaviour in response to any XR UAI (from RAN4 perspective) - Not define UE behaviour and the corresponding requirements based on UAI - Supporting of UAI is optional for UE with optional UE capability.

And the following was agreed in RAN4 #114bis (R4-2504911):

- The information in UAI: a ratio (R) of gap occasions that is recommended for cancellation during a time period of 1 s. - R ∈ {0, 20, 40, 60}%

ACTION:  RAN4 respectfully asks RAN2 to implement the signaling of the above mentioned UAI reporting for XR.

Based on this, we don’t see any further information will be provided to RAN2. Hence RAN2 should commence with the requested action. This means introducing support in the UE Assistance Information by including a information about the ratio for which the UE can skip measurements occasions during a time period.

The ratio to be selected by the UE may depend on different conditions, like traffic situation or radio conditions or mobility status.

The UE needs to be configured to report UAI related to skipping of measurements gaps and the different ratios that can be reported as well as trigger conditions.

Proposal 1: Introduce support for UE Assistance Information (UAI) indicating the gNB the MG(s) to be skipped by the UE with the value range of [0%, 20%, 40%, 60%].

At this meeting, RAN2 should decide on values for a prohibit timer to limit and settle how frequent the UE can report UAI for measurement gap skipping. Since the traffic is relate to XR, the reception and transmission interval can be down to milliseconds, on the other hand, measurements are seldom needed that frequent. E.g. connected mode periodic reporting the interval is in the range from 120 ms upto 30 min.

For this case e.g. value ranges could be in the range from 100 ms up to 10 sec. [ms100, ms200, ms 500, ms1000, ms2000, ms5000, ms10000].

Proposal 2. Agree on values for Prohibit timer e.g. [ms100, ms200, ms500, ms1000, ms2000, ms5000, ms10000].

Proposal 3. Send replay LS to RAN4 and inform about RAN2 progress.

# Conclusion

In this contribution, we provide our views on enabling transmission/reception during measurement gap. The observations and proposals are listed below.

Observation 1: It was noted that if no further information is received from RAN4, then RAN2 will have to decide.

Proposal 1: Support UE Assistance Information (UAI) indicating the gNB the MG(s) to be skipped by the UE.

Proposal 2. Agree on values for Prohibit timer e.g. [ms100, ms200, ms500, ms1000, ms2000, ms5000, ms10000].

Proposal 3. Send replay LS to RAN4 and inform about RAN2 progress.

# References

- RP-241771, “Revised WID: XR (eXtended Reality) for NR Phase 3”, RANP #105, September 2024

- RAN2#127bis, Hefei

- RAN2#128, Orlando

- RAN2#129

- RAN2#130

- RAN4#114 (R4-2502700