3GPP TSG-RAN WG2 Meeting #131	R2-2505716

Bengaluru, India, 25 – 29 August 2025

Agenda item:	8.8.4

Source:	Nokia

Title:	Remaining Issues for MBS in NTN

WID/SID:	NR_NTN_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

This TDoc handles open issues regarding the support of MBS for NR NTN such as:

- Intended service area inclusion in Service announcement for MCCH reading based energy savings, also based on SA4 reply LS

- MII transmission when the UE enters or leaves the intended MBS service area

- Broadcast area versus intended broadcast area

# 2	Discussion

## 2.1	ISA in Service Announcement

Intended Service Area in Service Announcement has been discussed in RAN2#129 based on [15]. The main motivation used by the authors of [15] was to allow the UE to skip MCCH reading if the UE determined (based on Service Announcement) that it is outside of the ISA. An LS to SA4 and SA2 has been sent [14] asking about the feasibility of such scheme in Release 19 timeframe. SA4 has provided the following reply to the LS [20]:

SA4 thanks RAN2 for bringing up the topic of “MBS via NTN”. SA4 is still studying the realization for including an Intended Service Area into Service announcement, leveraging the RAN2 Work Item code NR_NTN_Ph3-Core. SA4 has agreed to extend the Service Announcement and add a targeted service area in form of a list of polygon or circle shapes. However, SA4 is still discussing the technical realization and possibly a generalization, so that the target service area can be extended to represents the intended service area in case of NR NTN. SA4 confirms, that inclusion of the geographical information, i.e. in form of list of polygon or circle shapes in the Service Announcement can be supported within the Rel-19 timeframe.

Our understanding of the LS reply is that SA4 has agreed to extend the Service Announcements by adding the targeted service area. Although, this agreement is indeed stating that it may be possible to include the area, the work has not progressed in SA4 and a request for SA4 work at this point in the WI will result in a workload across working groups which does not match the gain.

One disadvantage of the originally proposed approach is that it not only requires extra information in the Service Announcement but also introduces new requirements to the application function (AF) and also to Multicast/Broadcast Service Function (MBSF). As such, RAN should not rely on such solutions and instead focus on RAN-centric ones.

Observation 1: Including ISA in Service Announcement may provide the UE with limited power saving options, which are not relevant to completing the NTN WI scope.

Observation 2: SA4 response states that additional work indeed is needed to complete the ISA in Service announcement for NR NTN.

Additionally to the AF and MBSF impact, the proposed solution might require a coordination between the operator and the application function if the application shall know about ISA. Eventually, AF does not need to know that certain service is transmitted via NTN RAN and not the “ordinary” TN.

Observation 3: ISA in Service Announcement will result in new requirements to the application function (AF) and also to Multicast/Broadcast Service Function (MBSF).

Observation 4: ISA in Service Announcement might require a coordination between the operator and the application function if the application shall know about ISA.

Thus, in our view the gains of ISA in Service Announcement are too small compared to the pain of introducing this.

Proposal 1: RAN2 does not consider including ISA in Service Announcement, irrespective of the SA4 conclusions on targeted service area in Service Announcement.

## 2.2	Broadcast service area

At RAN2#127bis the following has been discussed and not resolved:

Proposal 4: For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature may initiate the MBS Interest Indication procedure when UE is entering or leaving the intended area. - QC thinks this is up to UE and there is nothing to reflect in the spec. Xiaomi agrees - HW thinks that when entering the area this makes sense, but wonder about the case when the UE leaves the area. - Apple thinks the UE could initiate the MII procedure in any case and then it’s up to NW implementation how to behave. LG agrees with Apple. Oppo also agrees. CMCC thinks in this case the NW has no idea of the UE location in this case. Come back in the next meeting

RAN2#130 opened a discussion based on R2-2503560 on whether the term broadcast area referred to the intended service area and not a hard line of the where MBS session content is available, this understanding should be captured in the spec i.e. stage 2 [21]:

R2-2503560 Discussions on supporting broadcast service Fujitsu discussion Rel-19 NR_NTN_Ph3-Core Observation 1: Behaviours with respect to the initiation of MII procedure upon entering or leaving the area of broadcast service has already been defined. Proposal 1: RAN2 to confirm that, with respect to the MBS service, the term ‘broadcast service area’ refers to the ‘intended service area’. It could be captured in Stage 2 specification. Come back in the next meeting

This understanding is even more prominent if the broadcast area is a polygon rather than a circle, which is not feasible to limit actual signal transmission to.

We don’t see a specific reason that the UE would be confused on the definition of area since legacy areas are already not defined by where physical signals are bounded. The discussion also mentions that there are no need to define additional MBS Interest Indication (MII) events, which we see no advantages to have introduced.

Proposal 2: There is no need to explicitly define that the broadcast service area from a physics perspective may be different than the intended service area.

## 2.3	On MBS Interest Indication

Another open point is the MBS Interest indication. In section 5.9.4.2 of [7] the following text can be found:

An MBS capable UE in RRC_CONNECTED may initiate the procedure in several cases including upon successful connection establishment/resume, upon entering or leaving the broadcast service area, upon MBS broadcast session start or stop, upon change of interest, upon change of priority between MBS broadcast reception and unicast/multicast reception, upon change to a PCell providing SIB21 (i.e. where the SIB1 scheduling information contains SIB21), upon receiving SIB20 of an SCell via dedicated signalling, upon handover, and upon RRC connection re-establishment, upon change to a PCell providing nonServingCellMII in SIB1, upon starting or stopping reception of MBS broadcast service on a non-serving cell, upon change of CFR information or subcarrier spacing for MBS broadcast reception on a non-serving cell.

As can be seen, the MII might be sent when UE enters or leaves the broadcast service area and this is already supported by NR RRC [7].

Observation 5: As per legacy NR RRC the MBS capable UE may send the MBS Interest Indication upon entering or leaving the broadcast service area.

Thus, agreeing to MII being sent when the UE enters or leaves the broadcast service area requires no changes to MII design in NTN. Furthermore, the behaviour described in [7] is not binding (the term used in the procedure is “UE may initiate”), so in our view this issue is not worth spending additional time. Note that the MII feature can be used by the UE in case UE cannot find the MBS broadcast service that it is expecting in a cell. A gNB can decide whether to handover the UE to a possible neighbor gNB/cell providing MBS broadcast service UE has interest in. Thus, as has been indicated at RAN2#127bis, it is a UE implementation issue, and a UE can decide when to send the MII (which could be somehow related to the intended service area).

Proposal 3: RAN2 does not specify any new behaviour with respect to MBS Interest Indication when Rel-19 NTN capable UE enters or leaves the MBS broadcast service area.

# 3	Conclusion

This document has made the following observations:

Observation 1: Including ISA in Service Announcement may provide the UE with limited power saving options, which are not relevant to completing the NTN WI scope.

Observation 2: SA4 response states that additional work indeed is needed to complete the ISA in Service announcement for NR NTN.

Observation 3: ISA in Service Announcement will result in new requirements to the application function (AF) and also to Multicast/Broadcast Service Function (MBSF).

Observation 4: ISA in Service Announcement might require a coordination between the operator and the application function if the application shall know about ISA.

Observation 5: As per legacy NR RRC the MBS capable UE may send the MBS Interest Indication upon entering or leaving the broadcast service area.

And proposed the following:

Proposal 1: RAN2 does not consider including ISA in Service Announcement, irrespective of the SA4 conclusions on targeted service area in Service Announcement.

Proposal 2: There is no need to explicitly define that the broadcast service area from a physics perspective may be different than the intended service area.

Proposal 3: RAN2 does not specify any new behaviour with respect to MBS Interest Indication when Rel-19 NTN capable UE enters or leaves the MBS broadcast service area.

# References

- RP-240775 Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3 3GPP TSG RAN Meeting #103 Maastricht, The Netherlands, March 18-22, 2024

- R2-2409213 Report from Break-out session on NR-NTN and IoT-NTN 3GPP TSG-RAN WG2 Meeting #127bis Changsha, China, Apr 15th – 19th, 2024

- R2-2410913	Report from Break-out session on NR-NTN and IoT-NTN 3GPP TSG-RAN WG2 Meeting #128 Orlando, USA, Nov 18th – 22nd, 2024

- R2-2501333	Report from Break-out session on NR-NTN and IoT-NTN 3GPP TSG-RAN WG2 Meeting #129 Athens, Greece, Feb 17th -21st, 2025

- S1-244857 Reply LS on requirements for ETWS primary notification 3GPP TSG SA WG WG1 Meeting #108 Orlando, FL, USA, 18th – 22nd November 2024

- S1-244753 Warning notification with geofencing for ETWS over satellite access (CR to TS 22.268) 3GPP TSG-SA1 Meeting #108	 Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

- 3GPP TS 38.331 NR RRC, v18.4.0, December 2024

- R2-2407823 LS on requirements for ETWS primary notification 3GPP TSG-RAN WG2 Meeting #127 Maastricht, Netherlands, August 19th – 23rd, 2024

- R2-2405703 Report from Break-out session on NR-NTN and IoT-NTN 3GPP TSG-RAN WG2 Meeting #126 Fukuoka, Japan, May 20th – 24th, 2024

- R2-2407573 Report from Break-out session on NR-NTN and IoT-NTN 3GPP TSG-RAN WG2 Meeting #127 Maastricht, The Netherlands, Aug 19th – 23rd, 2024

- R2-2407619 [Offline 307] [R19 NR NTN] service area information (Apple) 3GPP TSG-RAN WG2 Meeting #127 Maastricht, The Netherlands, Aug 19th – 23rd, 2024

- R2-2402355, Discussion on support of broadcast service via NR NTN, CATT, China Broadnet, 3GPP TSG RAN2#125bis meeting, Rel-19, April 2024.

- R3-245844 LS on Supporting MBS broadcast service for NR NTN 3GPP TSG RAN WG3 125-bis Hefei, China, 14 - 18 October, 2024

- R2-2501583 LS on Inclusion of NTN intended service area in the Service Announcement 3GPP TSG-RAN WG2 Meeting #129 Athens, Greece, Feb 17th -21st, 2025

- R2-2501306  Support for broadcast services in NR NTN 3GPP TSG-RAN WG2 Meeting #129 Athens, Greece, Feb 17th -21st, 2025

- R2-2502983 Report from Break-out session on NR-NTN and IoT-NTN 3GPP TSG-RAN WG2 Meeting #129bis	Wuhan, China, Apr 7th – 11th, 2025

- R2-2503057 Report of [AT129bis][304][R19 NR NTN] Service continuity 3GPP TSG-RAN WG2 Meeting #129bis	Wuhan, China, Apr 7th – 11th, 2025

- R2-250xxxx NR RRC Running CR for Rel-19 NTN

- R2-250xxxx Remaining RRC open issues for NR NTN Rel-19

- S4-251099 Reply on Inclusion of NTN intended service area in the Service Announcement

- R2-2504673 Report from Break-out session on NR-NTN and IoT-NTN 3GPP TSG-RAN WG2 Meeting #130	Malta, May 19th – 23rd, 2025