3GPP TSG-RAN WG2 Meeting #131	R2-2505739

Bengaluru, India, Aug 25th – 29th, 2025

Agenda Item:	8.18.2

Source:	Huawei, HiSilicon

Title:	Discussion on time/frequency interleavers for MBMS

Document for: Discussion and Decision

1	Introduction

During the RAN2#130 meeting, the following agreements were made about the time and frequency interleaving for LTE- based 5G Terrestrial Broadcast:

Baseline signalling design - RAN2 assumes the following needs to be specified in RRC, but the final parameters/values/structure/terminology needs to be confirmed based on L1 parameters list from RAN1: - Follow RAN1 agreement (i.e. per PMCH configuration) on the configuration for time interleaving transmission. - Wait for RAN1 for the candidate values of the new MCH scheduling period(s). - Introduce a field to indicate TBS scaling factor (i.e. N) with candidate values {2,4,8,16,ffs}. - Introduce a field to indicate the separation between two transmissions of the same TB (i.e. M) with candidate values {4,8,16,32,ffs}. - Presence of the parent IE containing fields for N and M indicates time interleaving is enabled. - FFS (pending RAN1 decision): Introduce a flag to indicate whether frequency interleaving for MTCH is enabled. Co-existence with legacy MBMS and MAC impacts - We aim to support co-existence of legacy (pre-Rel19) and R19 transmission and UEs by defining R19 PMCHs to cater to Time Interleaving based configurations, scheduling and transmissions in addition to legacy PMCHs. FFS exact signalling - RAN2 to adopt and specify minimal multiplexing enhancements for MCH reception that are applied only for scheduling of R19 PMCHs based MTCHs and may include, e.g.: a) Not allowing multiplexing of two MTCHs in same subframe b) Not applying Time interleaving to subframe carrying MSI/eMSI/MCCH c) Not allowing multiplexing of MTCH with MSI/eMSI/MCCH in a sub-frame due to Time interleaving difference d) FFS (pending RAN1 discussion): Inserting and/ interpreting padding to account for remaining portion of the subframe in above scenarios Impact on cell reselection - FFS cell reselection impact (e.g. need to understand whether there is a practical scenario where some changes would be needed)

Besides, RAN1 sends an LS to RAN2 providing some RAN1 agreements which may have RAN2 impact [2].

In this contribution, we will further analyse the RAN2 impacts to support time and frequency interleaver for MBMS reception.

2	Discussion

## 2.1 Impact to MAC

In the LS from RAN1, there are two issues may have impact on MAC specification:

In relation to the expected number of subframes used for time-interleaving derived from “stop MTCH (x+1)”. Agreement The number of subframes used for time-interleaving derived from ‘stop MTCH (x+1)’ is expected to be an integer multiple of MxN. Spec impact is up to RAN2. In relation to HARQ handling in the MAC specification Agreement RAN1 asks RAN2 whether/how to address the HARQ handling in the MAC specification for the Rel-19 time-interleaved PMCH transmission with M TBs and N RVs.

For the first agreement, it is about how to set the “stop MTCH” field in the MSI MAC CE. For legacy, UE assumes that “the other scheduled MTCH(s) start immediately after the previous MTCH, at the earliest in the subframe where the previous MTCH stops.” When time and frequency interleaving is introduced, there may be different understanding among different eNBs on how to set the “stop MTCH”, e.g., whether it should be the subframe where MTCH transmission actually stops or the subframe in MxN as agreed by RAN1. This is different from the legacy behaviour since the actual transmission may already stop before the (MxN)th subframe. Different NW settings may result in errors in UE side for MBSFN handling.

Let’s assume M=3, N=2, then the actual stop subframe should be Subframe 5, but according to RAN1 agreement it should be set as Subframe M * N=6 that the next MTCH starts. Without capturing anything, different NWs may set different values. So we need to capture this in MAC.

Proposal 1: Following RAN1 agreement, RAN2 should capture in MAC that the number of subframes used for time-interleaving derived from ‘stop MTCH (x+1)’ is expected to be an integer multiple of MxN.

For the HARQ handling, we think it is not specific to interleaved MBMS transmission. In other words, interleaved MBMS should be the same as legacy MBMS regarding HARQ handling and interleaving mainly affects the PHY which is close-looped within PHY layer. In legacy, HARQ handling for MBMS is not specified in the spec. If we introduce this for interleaving, it may cause confusion that HARQ is only applied to interleaved MBMS.

Proposal 2: No need to introduce HARQ handling specific to interleaving MBMS.

## 2.2 Impact to the cell prioritization

In the RAN2#130 meeting, RAN2 discussed the cell prioritization, and the following FFS was left:

- FFS cell reselection impact (e.g. need to understand whether there is a practical scenario where some changes would be needed)

Considering the resource efficiency, one cell may choose to only provide MBMS with interleaving or without interleaving. It is not practical to require all cells to simultaneously provide MBMS with both mechanisms. In this case, since legacy UEs cannot receive MBMS with interleaving from a cell, the corresponding frequency shouldn’t be prioritized, which can be up to NW configuration. In the meantime, for UEs supporting interleaving, the same frequency should be prioritized so that UE can be served in a better way. See the following figure where UE1 support interleaving while UE2 doesn’t:

To solve this issue, a straightforward way is to introduce a separate frequency list in the system information for the UEs supporting interleaving, following similar principle as legacy. The spec impact is marginal.

Proposal 3: A separate frequency list is introduced in SIB15 to indicate provision of MBMS with interleaving, following similar principle as legacy.

# 3	Conclusion

Based on the above discussion we have the following observation and proposals:

Impact to MAC

Proposal 1: Following RAN1 agreement, RAN2 should capture in MAC that the number of subframes used for time-interleaving derived from ‘stop MTCH (x+1)’ is expected to be an integer multiple of MxN.

Proposal 2: No need to introduce HARQ handling specific to interleaving MBMS.

Impact to the cell prioritization

Proposal 3: A separate frequency list is introduced in SIB15 to indicate provision of MBMS with interleaving, following similar principle as legacy.

4	Reference

[1] R1-2504922, LS on RAN2 aspects for LTE-based 5G Broadcast Phase 2, RAN1(EBU)