3GPP TSG-RAN WG2 #131	R2-2505808

Bengaluru, India, 25th – 29th August, 2025

Agenda Item:	8.12.3

Source:	Ericsson

Title:	Remaining aspects from other NR MIMO Ph.5 objectives

Document for:	Discussion

# 1	Introduction

This contribution discusses remaining open issues for NR MIMO phase 5 objectives that have impact to RAN2 specifications (except Asymmetric DL sTRP/UL mTRP, which is discussed separately).

# 2	Discussion

## 2.1 User Plane Impacts

In [Post130][219][MIMO_Ph5] Running CR for 38.321 it was discussed whether a UE should initiate Random Access (RA) when UEI report is triggered and to be transmitted but TAT is expired. The discussion was a continuation of what RAN2 considered in RAN2# 130 but did not conclude (R2-2504626, R2-2504369).

In summary, the UE may transmit a Mode-A UEI report on PUSCH, but upon TAT expiry as PUCCH/PUSCH are released, there are no valid UL resources for this transmission.

There are a few remaining options to consider (c.f.  [Post130][219][MIMO_Ph5]):

- Legacy behaviour following the handling of CSI report is that the UEI report is not transmitted - no specification impact.

  - UEI BR reporting can resume upon UL resynchronization and RRC reconfiguration. BFD/BFR may be triggered for failure cases.

- UE initiates RA when UEI report is triggered while no valid PUCCH/PUSCH resources are present (TAT expired) and includes or indicates a RA cause. It remains to be concluded then if:

  - Whether this is beneficial for timely reconfiguration of new resources for UEI report early (e.g., in Msg4). This assumes that:

    - A UEI report can be retained over time, meaning a delayed UEI report is still valid and useful once new resources are available

    - Whether BFR-RA for an actual failure case is insufficient

    - Other possible implications yet to be determined.

From the discussion, it seems the main motivation for this optimization is that it supports timely resource reconfiguration – if the UE has lost its uplink (UL) resources due to TAT expiry, triggering RA immediately including a UEI indication/cause can help quickly re-establish connectivity and allocate new PUCCH/PUSCH resources. This may allow the UEI report to be transmitted without waiting for a full periodic reconfiguration or allow the initiation of new UEI reporting trigger.

However, the functionality as defined in RAN1 is based on measurement entailing a number of beam events within a measurement period (T_window), for which a trigger and report indicate timely the triggering condition for beam management purposes. The report triggered in mode A is thus very similar to a normal aperiodic beam report including the configuration of the triggering offsets. For situations where a UE is out of sync, the actions and beam management purpose is as a result not useful as UEI are intended to enable fast, event-driven adjustments (e.g., beam switching) using real-time or near-real-time triggered measurements and reporting). If the UE is out of sync, the system delay to regain uplink sync (via RA, for example) negates the 'fast' nature of the report

Additionally, beam state changes rapidly in FR2/sTRP, especially in mobility or blockage scenarios. Any measurements collected before losing sync could be obsolete by the time UL resources are restored, making the UEI payload irrelevant or even misleading.

To send UEI after losing UL sync, the UE needs to re-establish a connection via RA and scheduling. This adds signaling overhead and may duplicate existing failure recovery processes (BFR/BFD), without delivering meaningful beam-management benefit because the triggering context has likely changed.

In no-sync situations, network reconfiguration after RA would typically also trigger new CSI/beam management procedures automatically, rendering at least the previously triggered UEI redundant.

If many UEs in poor UL conditions try to regain sync for reporting, collisions and delays on RACH will further deteriorate responsiveness—which is counter to the low-latency beam management goal.

In essence, UEI’s intended advantage—low-overhead, fast, and opportunistic reporting—largely disappears once the UE loses synchronization, and the re-establishment path already has existing signaling to handle link and beam reconfigurations.

Following the legacy option above would thus render no specification impact and still fulfil the intended UEI functionality in Rel-19

UEI’s intended advantage—low-overhead, fast, and opportunistic reporting—largely disappears once the UE loses synchronization, and the re-establishment path already has existing signaling to handle link and beam reconfigurations.

UE does not initiate RA due to TAT expiry when UEI report is triggered and no transmission resources are present.

## 2.2	Control Plane Impacts

The following issues were left open in [Post130][218][MIMO_Ph5]:

| UEI BMR | FFS whether UEI BM parameters should be moved to CSI-MeasConfig. | Since now most of the parameters are stable this could be discussed based on company contributions. |
| --- | --- | --- |
| CSI-CRI | Editor’s note: FFS on how to define additionalOneSlotOffsetDoppler as a list. | This could be handled during a round of updates for 38.331 based on companies input. |
| CSI-CJTC | Parameter referenceAntennaPort is indicated in RAN1 list as follows: up to RAN2 regarding {1,..,x}, where x is x in ‘xTyR’ when SRS usage set to antenna switching. | This could be discussed based on company contributions. |

For UEI Report, currently the related parameters are defined within CSI-ReportConfig. This is also in line with the L1 parameter list, which included the UE IBR parameters on CSI-ReportConfig. Therefore, RAN2 should only deviate from RAN1 guidance if there is an outstanding issue.

Some parameters in CSI-ReportConfig are mandatory but not applicable to UEI report case as the fields within reportConfigType. To address this, we think it is sufficient to capture restrictions in field description on which fields the UE should ignore when configured for UEI report, as already done for eventTypeUE-IBR:

eventTypeUE-IBR

Indicates the event type for UE initiated beam reporting and associated fields as specified in clause 5.2.1.5.4 of TS 38.214 [19]. When this field is configured, the UE ignores reportConfigType. When this field is set to event1, eventThreshold can only be configured with values from 14 to 113.

Therefore, it seems doable to define UEI report parameters within CSI-ReportConfig and thus no specification change is needed.

Keep UEI report parameters within CSI-ReportConfig. No specification impact.

For CSI-CJTC, it is left up to RAN2 how to define the range of referenceAntennaPort. In principle, this field can have values of 1, 2, or 4 depending on ‘xTyR’ used for SRS antenna switching. Alternatively, it could be defined as 1, 2 or 3 where each number points to a certain ‘xTyR’ used for SRS antenna switching. But since both options would anyway require 2 bits, defining value 1, 2 and 4 seems clearer.

referenceAntennaPort is defined with values 1, 2 and 4.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	UEI’s intended advantage—low-overhead, fast, and opportunistic reporting—largely disappears once the UE loses synchronization, and the re-establishment path already has existing signaling to handle link and beam reconfigurations.

Based on the discussion in the previous sections we propose the following:

Proposal 1	UE does not initiate RA due to TAT expiry when UEI report is triggered and no transmission resources are present.

Proposal 2	Keep UEI report parameters within CSI-ReportConfig. No specification impact.

Proposal 3	referenceAntennaPort is defined with values 1, 2 and 4.

# 4	References

- RP-242394 NR MIMO Phase 5, Samsung (Moderator), RAN meeting #102 Scotland, December 11-15, 2023