3GPP TSG-RAN WG2 #131	R2-2505835

Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	8.10.3

Source:	Ericsson

Title:	Discussion on SON-MDT enhancements for Slicing and NTN

Document for:	Discussion/Decision

# Introduction

RAN#103 decided to support further data collection for Self-organizing Networks (SON). One of the topics that was endorsed was the SON and MDT enhancements for mobility in non-terrestrial networks and slicing.

# Discussion

## SON-MDT for Slicing

In the meeting RAN2#130 RAN2 discussed logging the NSAG information when the UE fails to find a suitable cell upon cell selection/reselection based on slice priorities. In the end RAN2 agreed to log the highest ranked NSAG information potentially in a report.

- We aim to specify that: If the UE supports slice-based cell reselection does not find any suitable cell in the frequencies corresponding to the highest ranked NSAG, the UE logs the highest ranked NSAG. FFS the need of cell or frequency info.

In this section we discuss considerations regarding the need for logging such information and possible SON/MDT report where it should be included.

The main problem with this scenario is that the UE could not find a suitable cell in the frequency at which it searches for suitable cells based on the provided NSAG priorities. The question of associating NSAG and possible other information in a SON/MDT report is open in RAN2.

## Inclusion of additional information along with NSAG

It was agreed that inclusion of NSAG information allows network to identify the issue of UE not re-selecting to a cell according to NSAG priorities and thus introduces delay in UE accessing to intended services. Logged MDT was selected as a promising candidate for the report. However, from network perspective, in logged MDT, only NSAG information is not sufficient to perform optimization and reconfiguration.

Consider the following scenario-

Cell A

Cell B

Cell C

Cell D

tl1

tl2

tl3

tl4

t1

t2

t3

The UE is configured with a logging periodicity of tlog. It performs periodic logging at instances tl1, tl2, tl3, tl4. However, the UE performs cell re-selection at t1, t2, t3 to Cell B, C, D, respectively. However, if the UE fails to select a cell according to NSAG priority and performs a sub-optimal cell reselection to cell B at t1, it will log such information at tl2 while being served by cell C. Hence, if the UE logs only NSAG information, network would not be able find problem that the UE performed sub-optimal cell reselection to cell B. However, UE may go to any cell selection state and may not have the cell information available in some scenarios. Network would also be unable to determine the location of the problem along with time information. Hence, inclusion of NSAG information in insufficient.

- Inclusion of NSAG without additional information such as cell  identity and UE location at the time of failure is not beneficial to the network.

Hence, RAN2 needs to agree to add cell, location time information along with NSAG information in the logged MDT report.

[Open issue RRC-3] RAN2 logs the cell reselection failure information in the next/first MDT sample logged after the detected cell reselection failure.

[Open issue RRC-3] RAN2 include the following information along with NSAG information in the logged MDT report
a) Cell information (if available, it may not be available if the UE goes to any cell selection state)
b) location where UE fails to access a cell according to NSAG priority

A TP outlining the proposal 1 in provided in the Annex

## SON-MDT for NTN

### Maximum number of geographical areas

We noticed that the maximum number of geographical areas that the UE can be configured has not been discussed. One of the reasons for introducing a geographical area scope is to address the large coverage of an NTN cell and to allow finer granularity in MDT area scope configuration. It is reasonable to define several geographical areas within a single NTN cell, without dividing it into an excessive number of geographical areas.

Therefore, the current maximum number of geographical areas 8 which is implemented in the current RRC running CR is a reasonable number.

Set 8 as the Maximum number of geographical areas that the UE can be configured as implemented in the RRC running CR.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Inclusion of NSAG without additional information such as cell and frequency information, UE location and time information is not beneficial to the network.

Based on the discussion in the previous sections we propose the following:

Proposal 1	[Open issue RRC-3] RAN2 logs the cell reselection failure information in the next/first MDT sample logged after the detected cell reselection failure.

Proposal 2	[Open issue RRC-3] RAN2 include the following information along with NSAG information in the logged MDT report a) Cell information (if available, it may not be available if the UE goes to any cell selection state) b) location where UE fails to access a cell according to NSAG priority

Proposal 3	Set 8 as the Maximum number of geographical areas that the UE can be configured as implemented in the RRC running CR.

# References

RP-234038, “New WID: Data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity) Phase 4”, RAN#102, Edinburgh, Scotland, December 2023.

# Annex

TP for Proposal 1 based on current running CR:

#### 5.5a.3.1	General

This procedure specifies the logging of available measurements by a UE in RRC_IDLE and RRC_INACTIVE that has a logged measurement configuration. The actual process of logging within the UE, takes place in RRC_IDLE state could continue in RRC_INACTIVE state or vice versa.

#### 5.5a.3.2	Initiation

While T330 is running and SDT procedure is not ongoing, the UE shall:

1>	if measurement logging is suspended:

2>	if during the last logging interval the IDC problems detected by the UE is resolved, resume measurement logging;

1>	if intendedAreaScopeList is included in VarLogMeasConfig:

2> if location information, if available, is outside of all areas indicated by intendedAreaScopeList:

3> skip the execution of the remainder of clause 5.5a.3.2 for the current logging interval (i.e. do not perform measurement logging for this interval);

1>	if not suspended, perform the logging in accordance with the following:

2>	if the reportType is set to periodical in the VarLogMeasConfig:

3>	if the UE is in any cell selection state (as specified in TS 38.304 [20]):

4>	perform the logging at regular time intervals, as defined by the loggingInterval in the VarLogMeasConfig;

3>	if the UE is in camped normally state on an NR cell and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport; or

3>	if the UE is in camped normally state on an NR cell and if the registered SNPN identity is included in snpn-ConfigID-List stored in VarLogMeasReport:

4>	if areaConfiguration is not included in VarLogMeasConfig; or

4>	if the serving cell is part of the area indicated by areaConfig in areaConfiguration in VarLogMeasConfig; or

4>	if the serving cell is part of the area indicated by cag-ConfigList in areaConfiguration in VarLogMeasConfig; or

4>	if the serving cell is part of the area indicated by snpn-ConfigList in areaConfiguration in VarLogMeasConfig:

5>	perform the logging at regular time intervals, as defined by the loggingInterval in the VarLogMeasConfig;

2>	else if the reportType is set to eventTriggered, and eventType is set to outOfCoverage:

3>	perform the logging at regular time intervals as defined by the loggingInterval in VarLogMeasConfig only when the UE is in any cell selection state;

3>	upon transition from any cell selection state to camped normally state in NR:

4>	if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport, or if the registered SNPN identity is included in snpn-ConfigID-List stored in VarLogMeasReport; and

4>	if areaConfiguration is not included in VarLogMeasConfig or if the current camping cell is part of the area indicated by areaConfig of areaConfiguration in VarLogMeasConfig, or if the current camping cell is part of the area indicated by cag-ConfigList of areaConfiguration in VarLogMeasConfig, or if the current camping cell is part of the area indicated by snpn-ConfigList of areaConfiguration in VarLogMeasConfig:

5>	perform the logging;

2>	else if the reportType is set to eventTriggered and eventType is set to eventL1:

3>	if the UE is in camped normally state on an NR cell and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport; or

3>	if the UE is in camped normally state on an NR cell and if the registered SNPN identity is included in snpn-ConfigID-List stored in VarLogMeasReport:

4>	if areaConfiguration is not included in VarLogMeasConfig; or

4>	if the serving cell is part of the area indicated by areaConfig in areaConfiguration in VarLogMeasConfig; or

4>	if the current serving cell is part of the area indicated by cag-ConfigList of areaConfiguration in VarLogMeasConfig, or if the current camping cell is part of the area indicated by snpn-ConfigList of areaConfiguration in VarLogMeasConfig;

5>	perform the logging at regular time intervals as defined by the loggingInterval in VarLogMeasConfig only when the conditions indicated by the eventL1 are met;

2>	when performing the logging:

3> if InterFreqTargetInfo is configured and if the UE detected IDC problems on at least one of the frequencies included in InterFreqTargetInfo or any inter-RAT frequency during the last logging interval, or

3>	if InterFreqTargetInfo is not configured and if the UE detected IDC problems during the last logging interval:

4>	if measResultServingCell in the VarLogMeasReport is not empty:

5>	include inDeviceCoexDetected;

5>	suspend measurement logging from the next logging interval;

4>	else:

5>	suspend measurement logging;

3>	set the relativeTimeStamp to indicate the elapsed time since the moment at which the logged measurement configuration was received;

3>	if location information became available during the last logging interval, set the content of the locationInfo as in 5.3.3.7:

3>	if the UE is in any cell selection state (as specified in TS 38.304 [20]):

4>	set anyCellSelectionDetected to indicate the detection of no suitable or no acceptable cell found;

4>	if the UE is configured with network slice(s) by NAS for cell reselection, and failed to perform a cell reselection to a cell asscoiated with the highest ranked NSAG (included in sliceInfoList or sliceInfoListDedicated):

5>	set nsagID to the highest ranked NSAG-ID associated with the network slice(s) provided by NAS, included in sliceInfoList, or sliceInfoListDedicated;

5> set the content of the locationInfo-NSAG to the location wherein the UE failed to perform cell reselection, if available;

4>	if the reportType is set to eventTriggered in the VarLogMeasConfig; and

4>	if the RPLMN at the time of entering the any cell selection state is included in plmn-IdentityList stored in VarLogMeasReport or if the registered SNPN identity at the time of entering the any cell selection state is included in snpn-ConfigID-List stored in VarLogMeasReport; and

4>	if areaConfiguration is not included in VarLogMeasConfig or if the last suitable cell that the UE was camping on is part of the area indicated by areaConfig of areaConfiguration in VarLogMeasConfig, or if last suitable cell that the UE was camping on is part of the area indicated by cag-ConfigList of areaConfiguration in VarLogMeasConfig, or if last suitable cell that the UE was camping on is part of the area indicated by snpn-ConfigList of areaConfiguration in VarLogMeasConfig:

5>	set the servCellIdentity to indicate global cell identity of the last suitable cell that the UE was camping on;

5>	set the measResultServingCell to include the quantities of the last suitable cell the UE was camping on;

4>	else if the reportType is set to periodical in the VarLogMeasConfig:

5>	set the servCellIdentity to indicate global cell identity of the last logged cell that the UE was camping on;

5>	set the measResultServingCell to include the quantities of the last logged cell the UE was camping on;

3>	else:

4>	set the servCellIdentity to indicate global cell identity of the cell the UE is camping on;

4>	set the measResultServingCell to include the quantities of the cell the UE is camping on;

3>	if available, set the measResultNeighCells, in order of decreasing ranking-criterion as used for cell re-selection, to include measurements of neighbouring cell that became available during the last logging interval and according to the following:

4>	include measurement results for at most 6 neighbouring cells on the NR serving frequency and for at most 3 cells per NR neighbouring frequency and for the NR neighbouring frequencies in accordance with the following:

5>	if interFreqTargetInfo is included in VarLogMeasConfig:

6>	if earlyMeasIndication is included in VarLogMeasConfig;

7>	include measurement results for NR neighbouring frequencies that are included in both interFreqTargetInfo and either in measIdleCarrierListNR (within the VarMeasIdleConfig) or SIB4;

6>	else:

7>	include measurement results for NR neighbouring frequencies that are included in both interFreqTargetInfo and SIB4;

5>	else:

6>	if earlyMeasIndication is included in VarLogMeasConfig;

7>	include measurement results for NR neighbouring frequencies that are included in either measIdleCarrierListNR (within the VarMeasIdleConfig) or SIB4;

6>	else:

7>	include measurement results for NR neighbouring frequencies that are included in SIB4;

4>	include measurement results for at most 3 neighbours per inter-RAT frequency in accordance with the following:

5>	if earlyMeasIndication is included in VarLogMeasConfig:

6>	include measurement results for inter-RAT neighbouring frequencies that are included in either measIdleCarrierListEUTRA (within the VarMeasIdleConfig) or SIB5;

5>	else:

6>	include measurement results for inter-RAT frequencies that are included in SIB5;

4>	for each neighbour cell included, include the optional fields that are available;

NOTE 1:	The UE includes the latest results of the available measurements as used for cell reselection evaluation in RRC_IDLE or RRC_INACTIVE, which are performed in accordance with the performance requirements as specified in TS 38.133 [14].

NOTE 2:	For logging the measurements on frequencies (indicated in measIdleCarrierListNR/ measIdleCarrierListEUTRA) in the logged measurement, the qualityThreshold in measIdleConfig should not be applied, and how the UE logs the measurements on the frequencies is left to the UE implementation.

3>	if UE failed to perform cell reselection t a cell according to NSAG priorities by NAS, according to sliceInfoList and/or sliceInfoListDedicated:

4>	set nsagID to the highest ranked NSAG-ID associated with the network slice(s) provided by NAS, or provided in system, or provided in dedicated signalling for cell reselection;

4> set the reselectedCellId to the cell UE reselected to if it is different from servCellIdentity;

4> set the content of the locationInfo-NSAG to the location wherein the UE failed to perform cell reselection, if available;

2>	when the memory reserved for the logged measurement information becomes full, stop timer T330 and perform the same actions as performed upon expiry of T330, as specified in 5.5a.1.4.

Editor’s Note: FFS if the UE logs the MDT data or not when it cannot obtain its location when geographical area scope is configured.

ASN.1 example:

LogMeasReport-r16 ::=                SEQUENCE {

absoluteTimeStamp-r16                AbsoluteTimeInfo-r16,

traceReference-r16                   TraceReference-r16,

traceRecordingSessionRef-r16         OCTET STRING (SIZE (2)),

tce-Id-r16                           OCTET STRING (SIZE (1)),

logMeasInfoList-r16                  LogMeasInfoList-r16,

logMeasAvailable-r16                 ENUMERATED {true}                   OPTIONAL,

logMeasAvailableBT-r16               ENUMERATED {true}                   OPTIONAL,

logMeasAvailableWLAN-r16             ENUMERATED {true}                   OPTIONAL,

...

}

LogMeasInfoList-r16 ::=              SEQUENCE (SIZE (1..maxLogMeasReport-r16)) OF LogMeasInfo-r16

LogMeasInfo-r16 ::=                  SEQUENCE {

locationInfo-r16                     LocationInfo-r16                    OPTIONAL,

relativeTimeStamp-r16                INTEGER (0..7200),

servCellIdentity-r16                 CGI-Info-Logging-r16                OPTIONAL,

measResultServingCell-r16            MeasResultServingCell-r16           OPTIONAL,

measResultNeighCells-r16             SEQUENCE {

measResultNeighCellListNR            MeasResultListLogging2NR-r16    OPTIONAL,

measResultNeighCellListEUTRA         MeasResultList2EUTRA-r16        OPTIONAL

},

anyCellSelectionDetected-r16         ENUMERATED {true}                   OPTIONAL,

...,

[[

inDeviceCoexDetected-r17             ENUMERATED {true}                   OPTIONAL

]]

[[

nsagSelectionInfo-rxx            SEQUENCE {

nsagID                            NSAG-ID-r17,

locationInfo-NSAG-rxx                  LocationInfo-r16                  OPTIONAL,

reselectedCellId-rxx              CGI-Info-Logging-r16              OPTIONAL

}

]]

}