3GPP TSG RAN WG2 Meeting #131	R2-2505879

Bangalore, India, Aug. 25th - 29th. 2025

Agenda Item:	8.8.5

Source:	ETRI, Korea University

Title:		Remaining issues on support of regenerative payload

Document for:	Discussion and Decision

1	Introduction

Even though the impact of regenerative payload bas been investigated in RAN2, it can be said that no significant impact on the existing features are identified during the investigation. However, there seems to be a couple of descriptions in the current spec. that need to be clarified regarding regenerative payload. In this paper, we would like to discuss the required clarifications.

2	Discussion

Clarification of time-based measurement initiation for a feeder link switch

Time-based measurement initiation was adopted in Rel-17 for quasi-Earth-fixed cells and extended for a feeder link switch in Earth-moving cells by reusing t-Service in Rel-18.

With transparent payloads, the feeder link switch always results in a change of the serving cell. However, with regenerative payloads, the serving cell is not changed after the feeder link switch either for quasi-Earth-fixed cells or Earth-moving cells. Consequently, time-based measurement initiation for feeder link switch is not necessary for regenerative payloads. However, the current spec. does not explicitly specify that time-based measurement initiation for feeder link switch applies only to transparent payloads, which needs clarification.

To clarify that time-based measurement initiation for feeder link switch applies only to transparent payloads.

Clarification of t-Service

In the current spec., t-Service is used for two features: one is time-base measurement initiation and the other is satellite switch with resynchronization.

Regarding time-base measurement initiation, t-Service is used in TS 38.304 Sec. 5.2.4.2 as follows:

If the t-Service of the serving cell is present in SIB19, and if UE supports time-based measurement initiation, the UE shall perform intra-frequency, inter-frequency or inter-RAT measurements before the t-Service, regardless of the distance between UE and the serving cell reference location or whether the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, or Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, The exact time to start measurement before t-Service is up to UE implementation. UE shall perform measurements of higher priority NR inter-frequency or inter-RAT frequencies according to TS 38.133 [8] regardless of the remaining service time of the serving cell (i.e. time remaining until t-Service).

Regarding satellite switch with resynchronization, t-Service is used in TS 38.300 Sec. 5.7.19 as follows:

A UE capable of hard satellite switch with resynchronization in RRC_CONNECTED initiates the procedure when SatSwitchWithReSync and t-Service are included in SIB19. Upon initiating the procedure, the UE shall: 1> if t-ServiceStart is included in SIB19 and the UE supports soft satellite switch with resynchronization: 2> start acquiring DL synchronization with the SpCell served by the satellite indicated by ntn-Config in SatSwitchWithReSync between the time indicated by t-ServiceStart and the time indicated by t-Service for the serving cell; 1> upon the time indicated by t-Service: 2> stop timer T430 if running; 2> inform lower layers that UL synchronisation is lost due to satellite switch with resynchronization; ...

Referring to the text above, we can observe that t-Service is used in the spec. without any mention of whether the payload is transparent or regenerative.

In TS 38.331 Sec. 6.3.1, t-Service is described as follows:

t-Service Indicates the time information on when a cell provided via NTN is going to stop serving the area it is currently covering. This field applies for both service link switches in NTN quasi-Earth fixed cell and feeder link switches for both NTN quasi-Earth fixed and Earth-moving cell.

We can observe that the description of t-Service explicitly states that t-Service applies for feeder link switches but it also does not mention whether the payload is transparent or regenerative.

From the observations made above, even though time-based measurement initiation for feeder link switch applies only for transparent payloads, the current spec. can mislead readers to understand that t-Service can be used feeder link switch regardless of the payload type. Therefore, the description needs clarification.

To clarify that t-Service applies for both service link switch regardless of the payload type, and feeder link switch for transparent payload.

Support of the satellite switch with re-synchronization for regenerative payloads was discussed in RAN2 but reached no conclusion, and finally it was decided not to support it in a RAN plenary meeting last year. However, the description of satellite switch with re-synchronization in TS 38.300 does not clearly state that satellite switch with re-synchronization is supported only by transparent payloads.

The description of t-Service in TS 38.331, which is shown above, does not mention satellite switch with re-synchronization even though t-Service can be used for it.

To clarify that satellite switch with re-synchronization is supported only by transparent payloads, clarifications in both TS 38.300 and TS 38.331 seems to be unnecessary. One of two clarifications is enough to resolve the ambiguity in payload type supporting satellite switch with re-synchronization, where the two clarifications are 1) “satellite switch with re-synchronization is supported by transparent payloads” for the description of satellite switch with re-synchronization in TS 38.300 and 2) “t-Service is used for satellite switch with re-synchronization for transparent payloads” for the description of t-Service in TS 38.331.

To adopt one of two clarifications: 1) “satellite switch with re-synchronization is supported by transparent payloads” for the description of satellite switch with re-synchronization in TS 38.300 and 2) “t-Service is used for satellite switch with re-synchronization for transparent payloads” for the description of t-Service in TS 38.331.

3	Conclusion

In this paper, we propose the followings:

Proposal 1	To clarify that time-based measurement initiation for feeder link switch applies only to transparent payloads.

Proposal 2	To clarify that t-Service applies for both service link switch regardless of the payload type, and feeder link switch for transparent payload.

Proposal 3	To adopt one of two clarifications: 1) “satellite switch with re-synchronization is supported by transparent payloads” for the description of satellite switch with re-synchronization in TS 38.300 and 2) “t-Service is used for satellite switch with re-synchronization for transparent payloads” for the description of t-Service in TS 38.331.