3GPP TSG-RAN WG2 Meeting #131	R2-2505918

Bengaluru, India, 25th – 29th August, 2025

Agenda item:	8.9.4

Source:	Samsung

Title:	Open issues on PWS for NB-IoT NTN

WID/SID:	IoT_NTN_Ph3-Core

Document for:	Discussion and Decision

# Introduction

In RAN#107 the latest WID for IoT NTN Phase 3 was agreed [1], where the PWS component is the following:

- Support broadcast of PWS messages for NB-IoT re-using the LTE mechanisms [RAN2] Note: The solutions will be developed for NTN but can be applicable to TN (if possible). Specific NB-IoT TN optimizations will not be considered as part of this WI. Note: Support for indication of intended service area for ETWS will be considered based on NR NTN discussions/outcome. Note: PWS notifications in paging, direct indication, and system information in RRC connected mode is not supported.

In this contribution, we provide some discussions related to PWS broadcasting for NB-IoT.

# Discussion

Public Warning System broadcasting is a feature in 4G LTE and 5G NR that allows a network to broadcast public warning notifications. PWS supports Earthquake and Tsunami Warning System (ETWS) and Commercial Mobile Alert System (CMAS) messages which are sent over a set of System Information Blocks. ETWS uses the SIB10 and SIB11 in LTE and SIB6 and SIB7 in NR while CMAS uses SIB12 in LTE and SIB8 in NR. The network notifies UEs using Paging messages, which configures the UEs to acquire the related system information. The eNB or gNBs are configured via S1 or NG-AP to start broadcasting the PWS broadcasting.

## Skipping acquiring SIB1-NB

In the RRC open issue list, there is the following proposal:

Proposal 1: (RRC-2) RAN2 to discuss whether to allow the UE to skip reading SIB1-NB to shorten the latency of PWS acquisition.

When a UE receives an indication that PWS is broadcasted, then the UE will first acquire SIB1 in order to acquire how the PWS SIB(s) are scheduled. It would indeed be beneficial to skip acquiring SIB1 as this will take time. However, the options for solving this problem will also introduce problems and/or complexity.

One option would be that the UE acquires the PWS SIB scheduling in advance, and when the UE receives a Paging message indicating PWS being broadcasted, then the UE uses the stored PWS SIB scheduling. This has drawbacks in that the PWS scheduling needs to be pre-configured, which may be difficult if the eNB has not received the PWS message yet.

Another option is that the PWS SIB scheduling is sent with the paging message. This has less drawbacks, but makes the Paging more complex. This means that the highly efficient Direct Indication can no longer be used. This takes away from the simplicity of the current scheme, which also leads to risk for a relatively robust procedure.

With these potential options, we think it is safer accept some delay and rely on an already robust procedure.

Proposal 1: Do not introduce enhancements to skip acquiring SIB1-NB to shorten the latency of PWS acquisition.

## PWS segmentation

### Assembling PWS segments from different cells

In the RRC open issue list there is the following proposal:

Proposal 2: (RRC-3) RAN2 to discuss whether to allow UE to receive and assemble PWS segments from different cells during mobility based on proponent’s TP.

Compared to other SIBs, PWS SIBs are unique because the content of a SIB can be segmented. This can be useful as it can increase the coverage by dividing up a large message to multiple parts. When a satellite and consequently the NTN cell moves, there may be the case where a UE has managed to acquire some segments from one cell when the UE performs mobility to a second cell. For this purpose, it could be beneficial to allow for a UE to continue to acquire PWS SIB segments in the next cell.

However, in order to allow for PWS segments to continue to be acquired in a next cell there are some conditions that need to be fulfilled. First of all, both cells need to be segmenting its PWS SIBs in the same manner and the UE needs to be aware whether the cells are segmenting the PWS SIBs in the same manner. The complexity in this, is that the UE needs to be aware which other cells that are segmenting the PWS SIBs in the same manner, or the UE would have to assume that all cells are segmenting the PWS SIB in the same manner. All cells segmenting the PWS in the same manner in a satellite network is however quite unlikely, as the PWS message would likely be different in different geographical area. A simple 1-bit indication would not be enough. One solution would be to use some type of ID which indicates whether the cell is applying the same type segmentation and same PWS content would need to be used. It should for instance be made clear that a non-terrestrial cell and a terrestrial cell cannot be relied upon to use the same type of segmentation.

Observation 1: It is unlikely that all cells will segment the PWS SIB in the same manner, as the PWS messages would likely be different in different areas in a satellite network.

Proposal 2: To support assembling PWS segments from different cells, PWS segmentation and PWS message would need to be assured to be the same across different cells through an ID.

Proposal 3: To support assembling PWS segments from different cells, it should be made clear that segmentation between a terrestrial and non-terrestrial network can be assumed to not be the same.

### SI accumulation of PWS segments across SI windows

In RAN2#130, the following issue was discussed:

R2-2504335	Clarification to NTN SI message accumulation over SI-windows	Nordic Semiconductor ASA	discussion	Rel-17

Observation 3: The SI accumulation feature should not be used when acquiring SIB31(-NB) and SIB33(-NB).

Proposal 2: Clarify in the specifications that the SI accumulation shall not be used when acquiring SIB31(-NB) and SIB33(-NB).

- Vivo agrees with the intention but thinks the current spec already allows this UE behaviour.

- Google thinks this also depends on the SI window length

- Samsung thinks that at least in GEO case this would be useful

- Nokia thinks this issue is not new

- Ericsson wonders if the same problems exists for other SIB

RAN2 understands that the UE is not required to perform SI accumulation across SI windows when acquiring SIB31(-NB) and SIB33(-NB) (no need for spec change)

In both eMTC and NB-IoT, it has been specified that a UE that fails to acquire an SI Message containing SIBs, may continue to accumulate SI decodings across SI windows. This was introduced for eMTC and NB-IoT in order to increase the coverage, mainly because system information would be unlikely to change quickly in an IoT network and since any change of the system information should result in system information update procedures. However, for PWS the segmentation of the PWS message means that the network may change the SIB information for the different segments after an SI window. This means that a UE that fails to decode a PWS SIB segment 1 in an SI window may then continue to accumulate PWS SIB segment 2 in the next SI window, believing the SI Message to be the same. This will lead failed decoding or at least severely reduced coverage. Potentially, accumulating PWS segments across SI windows may need to be turned off for PWS SIBs.

Observation 2: A UE failing to decode a PWS segment in one SI message may continue accidentally decoding the PWS segment in the next SI window, leading to failed decoding.

Proposal 4: RAN2 to discuss how to deal with accumulating SI message decoding across SI windows when PWS is segmented.

## Text proposal for releasing a UE to acquire PWS SIB

In RAN2#130 the following was agreed:

Agreements:

1.	In case of PWS notification the network may release a RRC_CONNECTED UE to idle, in case the UE reports the capability to receive PWS in idle (no other spec impact other than the introduction of a UE capability for supporting PWS reception in idle)

Although the agreement states that there is no additional specification impact, we still believe that the agreement should be captured as part of specifications. This is to make it clear for an implementer that the only manner in which an NB-IoT UE in RRC_CONNECTED can be made to acquire the PWS SIB is by releasing the UE to RRC_IDLE.

Proposal 5: Capture descriptive text on releasing NB-IoT to RRC_IDLE to acquire the PWS SIB in 36.331.

Proposal 6: Agree to Text proposal in Annex A.

## Acceptable cell for NB-IoT

- In the open issue list on 36.304, the following has been captured:

Acceptable Cell Support for NB-IoT

Proposal 1:  Introduce acceptable cell support for NB-IoT in Rel-19. Further changes to restrict the support for PWS capable UE can be concluded based on contributions and online discussion.

Proposal  1A: No changes needed in existing text on 5.2.7 and 5.2.8 related to acceptable cell camping. Following changes will be considered as baseline for running CR update (can be updated based on contribution or online discussion)

Figure in 5.2.2-2 to be updated.

Changes to 5.2.8a to clarify the PWS support of NB-IoT for acceptable cell

- Most importantly it should first be confirmed whether acceptable cell is supported for NB-IoT:

Proposal 7: Acceptable cell is supported for NB-IoT NTN.

Second question concerns what type of changes are necessary to introduce the acceptable cell for NB-IoT.

Section 5.2.7, which is the section for non-NB-IoT UEs has two parts. One is when the UE is released to RRC_IDLE in a normal case, and the second part is when the UE is being released to RRC_IDLE during an emergency call when UE has been camping on an acceptable cell. For NB-IoT in Section 5.2.7a, only the first part has been captured, because the NB-IoT UE does not support emergency call, which will continue to be the case. However, one missing part of 5.2.7 that is not in 5.2.7a is the part on performing Stored Information Cell Selection in case no suitable cell found.

Proposal 8: Do not capture text describing camping on acceptable cell after returning to RRC_IDLE after having camped on any cell state in 5.2.7a.

Proposal 9: Capture the NB-IoT performing Store Information Cell Selection in case no suitable cell found in 5.2.7a.

Section 5.2.8 is the section on Any cell selection state for non-NB-IoT UEs. In this is it described when to camp on an acceptable cell. This needs to be added to the NB-IoT section in 5.2.8a.

Proposal 10: Capture the NB-IoT performing Store Information Cell Selection in case no suitable cell found in 5.2.7a.

Proposal 11: Agree to text proposal in Annex B.

Lastly, the Figure in 5.2.2-2 misses the acceptable cell camping procedures. Therefore the acceptable cell procedures as in 5.2.2-1 shall be added. However there are aspects (leaving idle mode to enter connected mode for Emergency calls only) related to emergency calls that shall not be added.

Proposal 12: Update Figure 5.2.2-2 to include acceptable cell camping as in Figure 5.2.2-1 excluding emergency call procedures.

- One FFS is related to whether there is a need for the network to indicate supporting PWS, which is used for a UE to decide whether to camp on an NB-IoT cell as an acceptable cell. This is argued to be useful, as else there may be no need for a UE to camp on an NB-IoT cell as an acceptable cell. For instance, PWS is unlikely to be implemented in a terrestrial network as the current use cases does not call for it, so there would be not a lot of reason to camp on a terrestrial network as an acceptable cell. We believe that this can be discussed.

Proposal 13: Discuss whether a network indicates support for PWS, which is used to decide whether a UE camps on the cell as an acceptable cell.

# Conclusion

In this contribution, we discussed issues related to PWS signaling for NB-IoT.

Observation 1: It is unlikely that all cells will segment the PWS SIB in the same manner, as the PWS messages would likely be different in different areas in a satellite network.

Observation 2: A UE failing to decode a PWS segment in one SI message may continue accidentally decoding the PWS segment in the next SI window, leading to failed decoding.

Proposal 1: Do not introduce enhancements to skip acquiring SIB1-NB to shorten the latency of PWS acquisition.

Proposal 2: To support assembling PWS segments from different NTN cells, PWS segmentation and PWS message would need to be assured to be the same through an ID.

Proposal 3: To support assembling PWS segments from different cells, it should be made clear that segmentation between a terrestrial and non-terrestrial network can be assumed to not be the same.

Proposal 4: RAN2 to discuss how to deal with accumulating SI message decoding across SI windows when PWS is segmented.

Proposal 5: Capture descriptive text on releasing NB-IoT to RRC_IDLE to acquire the PWS SIB in 36.331.

Proposal 6: Agree to Text proposal in Annex A.

Proposal 7: Acceptable cell is supported for NB-IoT NTN.

Proposal 8: Do not capture text describing camping on acceptable cell after returning to RRC_IDLE after having camped on any cell state in 5.2.7a.

Proposal 9: Capture the NB-IoT performing Store Information Cell Selection in case no suitable cell found in 5.2.7a.

Proposal 10: Capture the NB-IoT performing Store Information Cell Selection in case no suitable cell found in 5.2.7a.

Proposal 11: Agree to text proposal in Annex B.

Proposal 12: Update Figure 5.2.2-2 to include acceptable cell camping as in Figure 5.2.2-1 excluding emergency call procedures.

Proposal 13: Discuss whether a network indicates support for PWS, which is used to decide whether a UE camps on the cell as an acceptable cell.

# Reference

RP-250472, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, Mediatek, RAN#107, Incheon, ROK, March 2025.

# Annex A – 36.331 TP on releasing NB-IoT UE

---------------------------- 36.331 ----------------------------

5.2.1.3	System information validity and notification of changes

Change of system information (other than for ETWS, CMAS, EAB, UAC, and satellite assistance information parameters except for discontinuous coverage scenarios and for NB-IoT, other than for AB parameters and satellite assistance information parameters except for discontinuous coverage scenarios) only occurs at specific radio frames, i.e. the concept of a modification period is used. System information may be transmitted a number of times with the same content within a modification period, as defined by its scheduling. The modification period boundaries are defined by SFN values for which SFN mod m= 0, where m is the number of radio frames comprising the modification period. The modification period is configured by system information. If H-SFN is provided in SystemInformationBlockType1-BR, modification period boundaries for BL UEs and UEs in CE are defined by SFN values for which (H-SFN * 1024 + SFN) mod m=0. For NB-IoT, H-SFN is always provided and the modification period boundaries are defined by SFN values for which (H-SFN * 1024 + SFN) mod m=0.

To enable system information update notification for RRC_IDLE UEs configured to use a DRX cycle longer than the modification period, an eDRX acquisition period is defined. The boundaries of the eDRX acquisition period are determined by H-SFN values for which H-SFN mod 256 =0. For NB-IoT, the boundaries of the eDRX acquisition period are determined by H-SFN values for which H-SFN mod 1024 =0.

NOTE 1:	If the UE in RRC_IDLE is configured to use extended DRX cycle, e.g., in the order of several minutes or longer, in case the eNB is reset the UE SFN may not be synchronized to the new eNB SFN. The UE is expected to recover, e.g., acquire MIB within a reasonable time, to avoid repeated paging failures.

NOTE 1a:	For the UE in RRC_INACTIVE, the idle mode extended DRX cycle, if configured, is used to compare with the modification period.

When the network changes (some of the) system information, it first notifies the UEs about this change, i.e. this may be done throughout a modification period. In the next modification period, the network transmits the updated system information. During a modification period where ETWS or CMAS transmission is started or stopped, the SI messages carrying the SIBs scheduled in schedulingInfoListExt and/or SI messages carrying the posSIBs scheduled in posSchedulingInfoList may change, so the UE might not be able to successfully receive those SIBs and/or posSIBs in the remainder of the current modification period and next modification period according to the scheduling information received prior to the change. These general principles are illustrated in figure 5.2.1.3-1, in which different colours indicate different system information. Upon receiving a change notification, the UE not configured to use a DRX cycle that is longer than the modification period acquires the new system information immediately from the start of the next modification period. Upon receiving a change notification applicable to eDRX, a UE in RRC_IDLE configured to use a DRX cycle that is longer than the modification period acquires the updated system information immediately from the start of the next eDRX acquisition period. The UE applies the previously acquired system information until the UE acquires the new system information. The possible boundaries of modification for SystemInformationBlockType1-BR are defined by SFN values for which SFN mod 512 = 0 except for notification of ETWS/CMAS for which the eNB may change SystemInformationBlockType1-BR content at any time. For NB-IoT, the possible boundaries of modification for SystemInformationBlockType1-NB are defined by SFN values for which (H-SFN * 1024 + SFN) mod 4096 = 0.

Figure 5.2.1.3-1: Change of system Information

The Paging message is used to inform UEs in RRC_IDLE and UEs in RRC_CONNECTED about a system information change. If the UE is in RRC_CONNECTED or is not configured to use a DRX cycle longer than the modification period in RRC_IDLE, and receives a Paging message including the systemInfoModification, it knows that the system information will change at the next modification period boundary. A UE in RRC_IDLE that is configured to use a DRX cycle longer than the modification period, and receives in an eDRX acquisition period at least one Paging message including the systemInfoModification-eDRX, shall acquire the updated system information at the next eDRX acquisition period boundary. Although the UE may be informed about changes in system information, no further details are provided e.g. regarding which system information will change, except if systemInfoValueTagSI is received by BL UEs or UEs in CE.

In RRC_CONNECTED, BL UEs or UEs in CE or NB-IoT UEs are not required to acquire system information except when T311 is running, or upon handover where the UE is only required to acquire the MasterInformationBlock in the target PCell, or for UEs in CE to receive ETWS/CMAS information, or upon expiry of T317 where the UE is required to acquire the SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT) and may acquire the SystemInformationBlockType33 (SystemInformationBlockType33-NB in NB-IoT). For an NB-IoT UE capable of receiving ETWS/CMAS, the E-UTRAN may initiate connection release for UE to acquire ETWS/CMAS. In RRC_IDLE, E-UTRAN may notify BL UEs or UEs in CE or NB-IoT UEs about SI update, and except for NB-IoT, ETWS and CMAS notification, EAB modification and UAC modification, using Direct Indication information, as specified in 6.6 (or 6.7.5 in NB-IoT) and TS 36.212 [22].

NOTE 2:	Upon system information change essential for BL UEs, UEs in CE, or NB-IoT UEs in RRC_CONNECTED, E-UTRAN may initiate connection release.

NOTE 3:	When acquiring SIB31(-NB) in RRC_CONNECTED, UE may assume that the scheduling is unchanged.

---------------------------- 36.331 ----------------------------

# Annex B – TP

---------------------------- 36.304 ----------------------------

5.2.7	Cell Selection at transition to RRC_IDLE or RRC_INACTIVE state

For NB-IoT cell selection at transition to RRC_IDLE state is defined in clause 5.2.7a.

At reception of RRCConnectionRelease message or RRCEarlyDataComplete message to move the UE into RRC_IDLE or RRC_INACTIVE, UE shall attempt to camp on a suitable cell according to redirectedCarrierInfo, if included in the RRCConnectionRelease message or RRCEarlyDataComplete message. If the UE cannot find a suitable cell, the UE is allowed to camp on any suitable cell of the indicated RAT. If the RRCConnectionRelease message or RRCEarlyDataComplete message does not contain the redirectedCarrierInfo UE shall attempt to select a suitable cell on an EUTRA carrier. If no suitable cell is found according to the above, the UE shall perform a cell selection starting with Stored Information Cell Selection procedure in order to find a suitable cell to camp on.

When returning to RRC_IDLE or RRC_INACTIVE state after UE moved to RRC_CONNECTED state from camped on any cell state, UE shall attempt to camp on an acceptable cell according to redirectedCarrierInfo, if included in the RRCConnectionRelease message. If the UE cannot find an acceptable cell, the UE is allowed to camp on any acceptable cell of the indicated RAT. If the RRCConnectionRelease message does not contain redirectedCarrierInfo UE shall attempt to select an acceptable cell on an EUTRA carrier. If no acceptable cell is found according to the above, the UE shall continue to search for an acceptable cell of any PLMN in state any cell selection.

5.2.7a	Cell Selection at transition to RRC_IDLE state for NB-IoT

At reception of RRCConnectionRelease-NB message or RRCEarlyDataComplete-NB message to move the UE into RRC_IDLE, UE shall attempt to camp on a suitable cell according to redirectedCarrierInfo, if included in the RRCConnectionRelease-NB message or RRCEarlyDataComplete-NB message. If the UE cannot find a suitable cell, the UE is allowed to camp on a suitable cell of any NB-IoT carrier. If the RRCConnectionRelease-NB message or RRCEarlyDataComplete-NB message does not contain the redirectedCarrierInfo UE shall attempt to select a suitable cell on a NB-IoT carrier. If no suitable cell is found according to the above, the UE shall perform a cell selection starting with Stored Information Cell Selection procedure in order to find a suitable cell to camp on.

5.2.8	Any Cell Selection state

For NB-IoT Any Cell Selection state is defined in clause 5.2.8a.

This state is applicable for RRC_IDLE and RRC_INACTIVE state. In this state, the UE shall perform cell selection process to find a suitable cell. If the cell selection process fails to find a suitable cell after a complete scan of all RATs and all frequency bands supported by the UE, the UE shall attempt to find an acceptable cell of any PLMN to camp on, trying all RATs that are supported by the UE and searching first for a high quality cell, as defined in clause 5.1.2.2.

The UE, which is not camped on any cell, shall stay in this state.

5.2.8a	Any Cell Selection state for NB-IoT

In this state, the UE shall attempt to find a suitable cell of any PLMN to camp on and searching first for a high quality cell, as defined in clause 5.1.2.2. For an NB-IoT UE supporting acceptable cell, if the cell selection process fails to find a suitable cell after a complete scan of all all frequency bands supported by the UE, the UE shall attempt to find an acceptable cell of any PLMN to camp on, searching first for a high quality cell, as defined in clause 5.1.2.2.

The UE, which is not camped on any cell, shall stay in this state until a suitable cell is found.

5.2.9	Camped on Any Cell state

In this state, the UE shall perform the following tasks:

-	monitor the paging channel of the cell as specified in clause 7 according to information sent in system information;

-	monitor relevant System Information as specified in TS 36.331 [3];

-	perform necessary measurements for the cell reselection evaluation procedure;

-	execute the cell reselection evaluation process on the following occasions/triggers:

1)	UE internal triggers, so as to meet performance as specified in TS 36.133 [10];

2)	When information on the BCCH or BR-BCCH used for the cell reselection evaluation procedure has been modified;

-	regularly attempt to find a suitable cell trying all frequencies of all RATs that are supported by the UE. If a suitable cell is found, UE shall move to camped normally state;

-	if the UE supports voice services and the current cell does not support emergency call as indicated in System information specified in TS 36.331 [3], the UE should perform cell selection/ reselection to an acceptable cell of any supported RAT regardless of priorities provided in system information from current cell, if no suitable cell is found.

NOTE:	The UE is allowed to not perform reselection to an inter-frequency E-UTRAN cell in order to prevent camping on a cell on which it cannot initiate an IMS emergency call.

---------------------------- 36.304 ----------------------------