3GPP TSG-RAN WG2 Meeting #131	R2-2505924

# Bengaluru, India, Aug 25th – 29th, 2025

Agenda item:		8.2.4 (Ambient_IoT_solutions)

Source:	LG Electronics Inc.

Title: 	Discussion on A-IoT data transmission

Document for:	Discussion and Decision

1.	Introduction

This document discusses ambient IoT (A-IoT) data transmission. In this contribution, we discuss mainly MAC PDU general format, TBS size and byte alignment, AS ID release, and data not available case. The related agreements in R2-130 meetings are as follows.

Agreements on segmentation 1. R2D message scheduling non-first segment (re)transmission does not include upper layer command. 2. For the first segment and unsegmented packet (re)transmission, the “offset” indicator in R2D is not present. 3. This implies that the R2D message will either have command or offset (but not both). FFS whether we define two message types or one message type with optional fields. Agreements on unsuccessful command 1. The device is expected to send a MAC response to the reader in the D2R occasion. The MAC response contains the NAS message if available at the D2R occasion. If there is no NAS message available to transmit at the D2R occasion then the response contains MAC with 0 SDU and padding as needed. 2. Send LS to CT1 to inform the agreement 1 to CT1 and explain that we have an issue with delayed NAS write success response. RAN2 would prefer that this is handled by CT1 (and give the example of sending NAS response upon successful reception of write command). Ask if this can be handled by CT1 Agreements on MAC PDU format 1. A mandatory length field directly indicates the length of D2R data MAC SDU to support varying lengths of D2R data. The size of length field is 7-bit in bytes. 2. The offset indication for transmission/retransmission of the segments after the first segment of a D2R message is 7-bit length in bytes. Segmented SDUs are also byte aligned. 3. FFS D2R message type. Current running CR will capture no message type, but we can revisit this next meeting and also consider if any other bits are needed for the MAC header 4. The length field inside MAC for SDU is not needed for R2D messages, assuming R2D MAC padding is not needed. FFS can come back if padding is needed depending on granularity of TBS (only if needed) Agreements on AS ID - For CBRA, to avoid AS ID being occupied for unnecessary time and to keep alignment between reader and device on AS ID release, device can release AS ID upon receiving paging message with different transaction ID, no matter the paging message is for it or not. FFS for CFRA - FFS for need for release message

In addition, regarding data not available case, the issue about how to set the value of More Data Indication (MDI) field has been raised and discussed during the post email discussion ([POST130][027][AIoT] MAC Running CR), however there has been no clear consensus.

2.	Discussion

In R2-130 meeting, it was decided that R2D message containing D2R Scheduling Info for the first segment or unsegmented packet (re)transmission does not include the offset indicator and R2D message scheduling non-first segment (re)transmission does not include upper layer command. However, for the two cases, whether to define two message types or one message type with optional fields has not been concluded in the meeting. The current A-IoT MAC running CR specifies 5 R2D message types and it defines the 3 bits long R2D Message Type field. There are 3 reserved values for future extension. Then, it seems worth using one message type with optional fields to save a value of message types for future. During the post email discussion ([POST130][027][AIoT] MAC Running CR), the 1-bit indication, which is called as Choice Indication (CI), is introduced to differentiate the two cases: R2D message containing upper layer command (i.e., Data SDU) and R2D message containing the offset indicator (i.e., Received Data Size) in the R2D Upper Layer Data Transfer message. This seems to be acceptable.

Proposal 1. One R2D message type with an optional field is used to differentiate the two cases: R2D message containing upper layer command and R2D message containing the offset indicator.

In R2-129bis meeting, it is agreed that the MAC PDU should be byte-aligned, assuming the allocated TBS value is in the unit of byte. Depending on granularity of TBS, R2D MAC padding may be needed. Based on the following RAN1 agreement, there is no restriction on the number of chips for R2D transmission. In other words, the number of chips is not restricted to multiples of M, where M is the number of chips within in an OFDM symbol. Therefore, we understand that any size of transport block (TB) can be assigned to R2D transmission within the maximum TB size (TBS), and R2D MAC padding is not needed as assumed in R2-130 meeting.

Agreement in R1-121 meeting Agreement When the generated number of chips for the R2D transmission does not fully occupy the last OFDM symbol, padding is used as follows: - Alt 1a: The content of padding is up to reader implementation and transparent to device. - The timeline determination of any timing relationship refers to the end of padding. - Note: it implies the device should be aware of the duration of padding or the last OFDM symbol boundary by implementation. - Note: the padding time could be used for extra time needed for calculating FEC/CRC for D2R (if applied) - The end chip(s) of the padding content shall follow the CP handling solution determined in RAN1, and may be affected by other agreements. - The chip(s) of the padding content shall avoid to result in SIP pattern.

For R2D MAC PDU it is byte-aligned, and the allocated TBS value is in the unit of byte. In R1-121 meeting, RAN1 agreed that TBS information for R2D is supported via higher layer R2D control signalling. It can be considered that the TBS information is added in R2D MAC PDU format mandatorily. During the post email discussion, it has been discussed whether Access Trigger message needs to include TBS information or not because the Access Trigger message consists of 3 bits long R2D message type only. There seems to be a consensus that TBS information needs not to be included in Access Trigger message. Then, the TBS information is mandatorily included in the R2D messages except Access Trigger message.

Proposal 2. The TBS information is mandatorily included in R2D messages except Access Trigger message.

An A-IoT device releases AS ID upon receiving a paging message with different transaction ID in case of CBRA. Considering that a device may have no transaction ID, it has been added in the MAC running CR for further clarification that, if a device has no transaction ID, an A-IoT device releases AS ID upon receiving a paging message. It is an acceptable device’s behavior. If a device is just powered-on, it has no transaction ID. In addition, there seems to be a consensus that transaction ID is not included in paging message for CFA. If it is agreed, a device has no transaction ID upon reception of a paging message for CFA.

Proposal 3. If a device has no transaction ID, an A-IoT device releases AS ID upon receiving a paging message.

For CFA, based on the assumption that a paging message for CFA has no transaction ID, whether to release AS ID upon receiving a paging for CFA or not can be discussed. During the post email discussion, it is captured in the MAC running CR that AS ID is released upon receiving a paging for CFA. This is acceptable considering that parallel services are not supported in Rel-19 A-IoT and a subsequent paging is not applied for CFA. The reader will initiate a new service after completing the previous service. The reader may use paging for CFA to initiate the new service. If a device receives a paging message for CFA, it needs to release AS ID because the old service is already completed, and a new service is initiated. It is noted again that the device has no transaction ID. If the device is involved in the new service, it will be assigned with a new AS ID. Even if the old service was triggered by paging for CFA, the device needs to release AS ID if it has an assigned AS ID. Since subsequent paging is not used for CFA, the following paging for CFA can be considered as a new paging for a new service.

Proposal 4. An A-IoT device releases AS ID upon receiving a paging message for CFA.

It has been discussed whether an AS ID release message is needed or not. Since AS IDs are newly assigned when a new service is initiated by a paging with a different transaction ID, an AS ID release message does not seem necessary. If the release message is introduced, the device can be released from AS ID maintenance upon receiving the release message. It may be beneficial for device power saving. However, in Rel-19, sleep or suspend mode is not defined. The device needs to be in active state for monitoring at least paging if it has enough energy. The power saving gain does not seem considerable.

Proposal 5. AS ID release message is not needed.

Data not available case has been discussed, and it is agreed that if there is no NAS message available to transmit at the D2R occasion then the response contains MAC PDU with 0 SDU and padding as needed. An LS is sent to CT1 to inform the agreement and to ask if data not available case can be handled by CT1.

For data not available case, an issue of how to set the value of More Data Indication (MDI) field is raised during the post email discussion. There seems to be a consensus that the reader can recognize data not available case based on reception of MAC PDU with 0 SDU. Without MDI field, the reader can be aware of data not available case. Initially, the MDI field is introduced to indicate whether the data SDU in the MAC PDU is the last segment or not. It is not for upper layer data not available case. It is preferred that MDI field is set to 0 if upper layer data is not available for transmission.

Proposal 6. More Data Indication (MDI) field is set to 0 if upper layer data is not available for transmission.

We think that the reader’s behavior may depend on CT1 response. If data not available case is handled by CT1, the reader just receives the padding only D2R MAC PDU without additional actions. The device sends padding only MAC PDU, and the reader receives the MAC PDU. If padding only D2R MAC PDU is successfully received, the reader MAC considers it as successful D2R transmission. It is noted that the reader MAC has no data to be transmitted to the upper layers, and it does not need to perform additional action. On the other hand, if data not available case cannot be fully handled by CT1, reader’s behavior for how to handle the case may be discussed further in RAN2. For example, the reader may schedule another D2R transmission, or it may indicate the situation to the upper layer on the network side. We may discuss reader’s behaviour, if needed, after receiving CT1 response. Now, we understand that RAN2 prefers that this is handled by CT1.

Proposal 7. Discuss reader’s behaviour for how to handle data not available case, if needed, after receiving the reply LS from CT1.

3.	Conclusion

This document discusses MAC PDU general format, TBS size and byte alignment, AS ID release, and data not available case.

Proposal 1. One R2D message type with an optional field is used to differentiate the two cases: R2D message containing upper layer command and R2D message containing the offset indicator.

Proposal 2. The TBS information is mandatorily included in R2D messages except Access Trigger message.

Proposal 3. If a device has no transaction ID, an A-IoT device releases AS ID upon receiving a paging message.

Proposal 4. An A-IoT device releases AS ID upon receiving a paging message for CFA.

Proposal 5. AS ID release message is not needed.

Proposal 6. More Data Indication (MDI) field is set to 0 if upper layer data is not available for transmission.

Proposal 7. Discuss reader’s behaviour for how to handle data not available case, if needed, after receiving the reply LS from CT1.