3GPP TSG-RAN WG2 #131	R2-2505935

Bengaluru, India, 25th – 29th August 2025

Source:	NTT DOCOMO, INC.

Title:	Discussion on remaining issues on AIoT paging

Document for:	Discussion

Agenda Item:	8.2.2

- Introduction

In the post e-mail discussion after RAN2#130 [1], following issues related to Paging remained unsolved in the end.

| Group 1: Paging |  |  |
| --- | --- | --- |
| Subgroup: Multi-reader scenario |  |  |
| Subgroup: Transaction ID |  |  |
| Issue 1-2: transaction ID | Whether/how to specify how the reader generate Transaction ID, and the size | Companies are invited to input views for Q#2 |
| Subgroup: Others |  |  |
| Issue 1-6: Paging ID visibility | Whether Paging ID is invisible or visible to MAC. | Not critical |

This contribution explains our views on Issue 1-2 since we don’t have strong view on Issue 1-6.

- Discussion on Issue 1-2

To summarize the e-mail discussion [1], companies have the common view that how the reader generates Transaction IDs can be left to reader implementation, and nothing needs to be specified. On the other hand, companies have diverging views on the length of Transaction ID from 2 bits to 6 bits, especially there are two big camps on 4 bits and 6 bits.

Basically, setting to a longer Transaction ID is preferable in terms of ease of operation. Note that one important background is the fact that we already agreed that we do not introduce reader ID in the Paging message and we rely on Transaction ID to let devices know whether every Paging message comes from a different neighboring reader from the previously received Paging to decide whether to ignore the new Paging message.

In our view, there are two reasons why the longer Transaction ID is better. Now we will explain the details.

In an environment like Release 19 where intermediate UE readers do not exist, the readers that may deliver paging to a device are, for example, the reader closest to the device and its neighboring readers. A simple example is shown in Figure 1.

Figure 1. Paging messages which come to one device in Rel-19 scenario.

Therefore, from an operational perspective, it is sufficient if the Transaction ID differs between each neighboring reader. However, as mentioned above, since the specification does not define how the reader generates the Transaction ID, and assuming Release 19 condition that coordination between readers is not possible, the Transaction ID is considered to be generated randomly. Therefore, it is better to have longer Transaction IDs to reduce the possibility of collisions of Transaction IDs randomly generated in neighboring readers.

Observation 1.	Longer Transaction ID can reduce the possibility of collision of Transaction IDs between neighboring readers.

The other reason is that if the same paging message format will used in Release 20, i.e., the same length of Transaction ID will be used also in Release 20, the number of readers that a single device needs to consider will increase significantly. A simple example is shown in Figure 2.

Figure 2. Paging messages which come to one device in Rel-20 scenario.

In a scenario where intermediate UEs exist, the readers that may deliver a paging message to a single device can be considered as follows. Here, we assume the case where the device is located at the coverage edge between neighboring cells, which results in the largest number of readers that need to be considered.

- All of the intermediate UE readers connected to the gNB A.

- All of the intermediate UE readers connected to the gNB B.

- If the gNB A is also working as a reader, gNB A reader.

- If the gNB B is also working as a reader, gNB B reader.

Therefore, it is necessary to introduce a Transaction ID that is long enough to allow different IDs to be assigned across all of these readers. Please note that since the Transaction ID assigned to an intermediate UE can be controlled by the gNB to which the UE is connected, collisions between Transaction IDs among UE readers connected to the same gNB are not expected. On the other hand, however, it is important to emphasize that collisions may occur between UE readers connected to different gNBs if the inter-gNB coordination is still not considered in Release 20.

Observation 2.	If the same length of Transaction ID is reused in Release 20, longer Transaction ID is much more desirable considering allocating different Transaction IDs among UE readers in the same gNB and neighboring gNBs.

For the two reasons mentioned above, we believe that the longer the Transaction ID, the better. Ideally, a length such as 8 bits would be desirable, but based on the consensus observed in the post-email discussion, we consider 6 bits to be a reasonable choice.

Proposal 1.	(Issue 1-2) The length of Transaction ID in the Paging message is 6 bits.

- Summary and proposal

Observation 1.	Longer Transaction ID can reduce the possibility of collision of Transaction IDs between neighboring readers.

Observation 2.	If the same length of Transaction ID is reused in Release 20, longer Transaction ID is much more desirable considering allocating different Transaction IDs among UE readers in the same gNB and neighboring gNBs.

Proposal 1.	(Issue 1-2) The length of Transaction ID in the Paging message is 6 bits.

- References

[1]	[POST130][027][AIoT] MAC Running CR (Huawei).