3GPP TSG-RAN WG2 #131	R2-2505981

Bengaluru, India, 25th – 29th August 2025

Source:	NTT DOCOMO, INC.

Title:	Discussion on remaining issues on AIoT random access

Document for:	Discussion

Agenda Item:	8.2.3

- Introduction

In the post e-mail discussion after RAN2#130 [1], following issues related to random access remained unsolved in the end.

| Group 2: Random access |  |  |
| --- | --- | --- |
| Subgroup: R2D trigger message and Msg1 related |  |  |
| Issue 2-3: R2D trigger message byte alignment | The R2D trigger message should be byte aligned or not. | Companies are invited to input views for Q#11 |
| Subgroup: CBRA procedure related |  |  |
| Issue 2-4: CBRA failure detection | Further down selection between option B and C for msg2 monitor window in CBRA. | To be discussed by company contributions |
| Subgroup: Msg2 content |  |  |
| Issue 2-5: random ID differentiation in Msg2 | Whether to include frequency index along with RN16 in MSG2 to reduce collisions of MSG1 between different devices. | To be discussed by company contributions |

This contribution explains our views on Issue 2-3, Issue 2-4 and Issue 2-5.

- Discussion

  - Issue 2-3: Whether the Access Trigger message should be bit-aligned

As a result of the post-email discussion [1], it still remains under discussion whether the Access Trigger message should be bit-aligned or byte-aligned.

In our view, the Access Trigger message can be bit-aligned, and from the perspective of reducing signalling overhead, it should be bit-aligned.

First, the advantage of a byte-aligned message is that it allows the message size information to be conveyed using fewer bits. For example, the SDU field of the D2R Upper Layer Data Transfer message is byte-aligned so that the SDU length can be indicated in byte units.

On the other hand, the Access Trigger message has a fixed size, so there is no need to indicate its length within the MAC layer. Even at the physical layer, where the type of R2D message is invisible, there is still no need to indicate the message length. This is because RAN1 agreed at the May meeting that the end of the R2D message would be indicated by the R2D postamble.

RAN1#121 Agreement R2D postamble is specified with 4 ON chips corresponding to M value of the PRDCH - R2D postamble is added immediately after the PRDCH - R2D postamble has always 4 ON chips - Note: For M=24, 2 ON chips at the end of OFDM symbol for CP handling are in addition to R2D postamble, and are not part of the R2D postamble - R2D padding duration is determined after R2D postamble insertion TBS information for R2D is supported via higher layer R2D control signalling. - Send LS to RAN2 asking to include R2D TBS information (excluding CRC length) in higher layer signaling, at least for messages with variable size. Note: Exact method for determining the end of PRDCH at the device is not specified.

From the reason above, Access Trigger message does not need to be byte-aligned. Considering that the necessary components in the Access Trigger message is only R2D Message Type with 3 bits, if Access Trigger message is forced to be byte-aligned there will be 5 bits of unnecessary overhead. Therefore, we think Access Trigger message should be bit-aligned.

Observation 1.	(Issue 2-3) The device can recognize the end point of R2D Access Trigger message by finding the postamble which RAN1 agreed to introduce.

Proposal 1.	(Issue 2-3) Access Trigger message is bit-aligned.

  - Issue 2-4: When the device detects CBRA failure

At the end of the previous meeting, the time boundary for a device to detect Msg2 and determine a CBRA failure remained under discussion between the following two options.

- Option B – the boundary is the reception of either the next R2D trigger message or the subsequent paging message

- Option C – the boundary is the reception of either the kth R2D trigger message or the subsequent paging message (K is FFS)

One potential concern we have with Option B is that it results in sequential handling of Msg1 reception and Msg2 transmission by the reader. In other words, once the reader sends an Access Trigger message, it would not be able to send the next Access Trigger message until it has completed sending Msg2 responses corresponding to all Msg1 messages received during that Access Occasion. However, we believe that it is more efficient for the reader to process received Msg1s and transmit Msg2 in parallel with sending the next Access Trigger message because this parallelization shortens the total time required to complete the random access procedure. Therefore, we prefer Option C. Considering the expected benefits, parallelizing the processing of up to two Access Occasions should be sufficient, so we suggest setting K=2.

Observation 2.	(Issue 2-4) Allowing the reader to handle generation of Msg2 and transmission of the next Access Trigger message could reduce the the total time required to complete the random access procedure.

Proposal 2.	(Issue 2-4) Down-select Option C (with K=2) for the time boundary for the device to detects CBRA failure.

  - Issue 2-5: Whether to include frequency index in Msg2

At the end of the previous meeting, it remained under discussion whether to introduce frequency index along with RN16 (and assigned AS ID) in which corresponding Msg1 was received in order to correctly respond to devices even when RN16s in one Access Occasion collided to each other.

As we pointed out during the previous meeting, the probability of the error case where RN16 collisions occur within a single Access Occasion, which is the reason for introducing frequency index in Msg2, is low.

Since RAN1 agreed that the maximum multiplexing number for FDM is 8, any collision would occur among these 8 frequencies. Even if the maximum of 8 devices were to transmit RN16 using different frequencies within a single Access Occasion (which we note is itself an unlikely scenario), the probability of RN16 collision among them is only 0.0427%.

| Number of devices which transmitted Msg1 in one Access Occasion | Probability of RN16 collision in Msg2 |
| --- | --- |
| 1 | 0.0000000% |
| 2 | 0.0015259% |
| 3 | 0.0045776% |
| 4 | 0.0091550% |
| 5 | 0.0152580% |
| 6 | 0.0228862% |
| 7 | 0.0320394% |
| 8 | 0.0427171% |

Therefore, the benefits of introducing a frequency index in Msg2 are not significant.

Observation 3.	(Issue 2-5) The probability of RN16 collision in one Msg2 is up to 0.0427%.

On the other hand, there are two potential risks raised when notifying the frequency index in Msg2.

First, after sending Msg1, the device must remember the frequency index of the Msg1 it transmitted. It is necessary to assess whether the impact on device memory implementation caused by this requirement is acceptable, given the relatively minor benefits mentioned above.

Additionally, although minor, there is a concern about the increased size of Msg2. Currently, Msg2 includes either an RN16 (2 bytes) or both RN16 and AS ID (4 bytes) per device. If the frequency index is also notified, an additional 1 byte will be required. This makes an increase of Transport Block size which could lead to situations where Msg2 cannot be received depending on the device’s battery level. (Note that we consider this risk relatively small, and it may be possible to treat it as a trade-off against the small benefits mentioned above.)

Onservation 4.	(Issue 2-5) Indicating frequency index in Msg2 can cause a bit of memory complexity in device and increases on Msg2 size.

On the other hand, even if the frequency index is not included in Msg2, there is still a low probability that RN16 collisions may occur within a single Access Occasion. We believe that in such cases, the reader should handle the exception by not replying with Msg2 to any of the devices involved in the collision. From the device’s perspective, this would result in Msg2 not being received after sending Msg1, leading to a detection of CBRA failure. Note that this is a situation that can also occur when the Msg1 transmission resource collides with that of another device. Therefore, similar recovery mechanisms, e.g., reinitiating paging, can be applied in this case as well.

Proposal 3.	(Issue 2-5) Frequency index in Msg2 is not introduced.

Proposal 4.	(Issue 2-5) The reader does not reply Msg2 to any of the devices involved in RN16 collision within an Access Occasion.

- Summary and proposal

Observation 1.	(Issue 2-3) The device can recognize the end point of R2D Access Trigger message by finding the postamble which RAN1 agreed to introduce.

Proposal 1.	(Issue 2-3) Access Trigger message is bit-aligned.

Observation 2.	(Issue 2-4) Allowing the reader to handle generation of Msg2 and transmission of the next Access Trigger message could reduce the the total time required to complete the random access procedure.

Proposal 2.	(Issue 2-4) Down-select Option C (with K=2) for the time boundary for the device to detects CBRA failure.

Observation 3.	(Issue 2-5) The probability of RN16 collision in one Msg2 is up to 0.0427%.

Onservation 4.	(Issue 2-5) Indicating frequency index in Msg2 can cause a bit of memory complexity in device and increases on Msg2 size.

Proposal 3.	(Issue 2-5) Frequency index in Msg2 is not introduced.

Proposal 4.	(Issue 2-5) The reader does not reply Msg2 to any of the devices involved in RN16 collision within an Access Occasion.

- References

[1]	[POST130][027][AIoT] MAC Running CR (Huawei).