3GPP TSG-RAN WG2 Meeting #131		       R2-2506030

Bengaluru, India: 25th Aug – 29th Aug 2025

Agenda Item:	8.3.2.1

Source:	ASUSTeK

Title:	Discussion on functionality management for RRM measurement prediction

Document for:	Discussion and Decision

- Introduction

In the previous RAN2 #130 meeting, the following agreements for performance monitoring of UE side models has been agreed. In this document, we would like to focus on the UE monitoring of UE side models.

Agreements 1 For UE-sided model, NW-sided monitoring, the performance monitor procedure contains - NW sends monitoring configuration and inference configuration to UE. - UE reports measurements and inference output based on the corresponding configurations to NW. - NW performs monitoring and makes decisions, which can be fully NW-implemented. 2 For UE-sided model, UE-sided monitoring, the performance monitor procedure as baselined contains - NW sends monitoring configuration to UE - UE measures monitoring data and generates monitoring results by comparing inference output and monitoring data. - (NW-decision): UE reports the monitoring result to NW, and NW makes the management decision. - Can further consider UE sided monitoring and (UE-decision) for some use cases: UE makes the management decision based on network configuration and sends the decision to NW. FFS which use cases. - For RRM measurement prediction - RSRP differences can be used as the performance metric for monitoring.

- Discussion

The previous meeting RAN2 #130 agreed for UE-sided model, UE-sided monitoring, the monitoring procedure includes NW sending a configuration and UE reporting. In addition, several companies raised proposals to consider the trigger of the report in the last RAN2 meeting. In [2], it proposed that network may configure the reporting type (periodic or event based reporting) of the performance metric for performance monitoring. In [3], it proposed RAN2 to study the options of periodic reporting and reporting triggered by NW. In [4], it proposed that trigger conditions for performance monitoring and monitoring result reporting are needed. In [5], it proposed that UE reports performance monitoring outcome in periodic or event triggered basis, according to the performance monitoring report configuration configured by the network. In summary, the performance monitoring reporting could be categorized into periodic reporting and event based reporting, based on how the report is triggered. However, it is unclear whether the UE performs periodic reporting and/or event based reporting.

In our understanding, periodic reporting of monitoring results can be the baseline. It is useful since the NW can obtain the monitoring results at least every configured time period. In addition, the configuration is quite simple. On the other hand, it may be redundant for the UE to periodically report when the performance is perfect. Therefore, event based reporting of monitoring results and/or UE decisions could also be useful in terms of signalling overhead reduction. Therefore, both periodic reporting and event based reporting should be supported.

Proposal 1: Periodic and event based reporting are both supported for reporting monitoring results.

However, the current measurement events based on cell measurements may not be suitable for performance monitoring. For example, the NW may be interested in performance for all radio conditions. For example, it may be too late to report monitoring results when the UE is in bad radio condition. Therefore, we think it may be useful to design new events, for example, based on the performance monitoring results.

Proposal 2: RAN2 to discuss about new events for monitoring, reporting, and/or UE decisions.

- Conclusion

Proposal 1: Periodic and event based reporting are both supported for reporting monitoring results.

Proposal 2: RAN2 to discuss about new events for monitoring, reporting, and/or UE decisions.

References

[1] Chair minutes for RAN2 #130

[2] R2-2503465, Discussion on AIML mobility performance monitoring, NEC.

[3] R2-2503594, Discussion on performance monitoring, CATT, Turkcell.

[4] R2-2503965, Discussion on performance monitoring for AI mobility, Spreadtrum, UNISOC.

[5] R2-2504297, Discussion on performance monitoring, Ericsson.

Annex