# 3GPP TSG-WG2 Meeting #131                                        			            R2-2506038

Bangalore, India Aug 25th – 29th, 2025

Source:                     Qualcomm Incorporated

Title:                         IDLE/Inactive LP-WUS disabling and enabling

Agenda Item:	8.4.2

Document for:	Discussion and Decision

# Introduction & Background

The following objective is approved for IDLE and Inactive mode LP-WUS, this contribution discusses the related issues.

- For IDLE/INACTIVE modes

  - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4)

# Discussion

During RAN2 #130 post-meeting email discussion, several solutions were discussed for LP-WUS disabling and enabling as follows.

Option 1: NAS based solution

In NAS based solution, UE indicates to AMF during registration procedure UE supports LP-WUS (i.e. UE-based subgroup), CN indicates explicitly to the UE to disable or enable LP-WUS in the registration procedure. Then a new parameter “Requested LP-WUS” in Registration Request is needed. When paging is triggered, AMF notifies gNB whether LP-WUS is enabled or disabled.

This option can achieve disabling and enabling per UE.

This solution has impact on SA2, CT1 and RAN3.

Option 2: RRC based solution

In RRC based solution, the source gNB can disable or enable LP-WUS using dedicated RRC message, e.g. RRC release message, and then notifies the AMF. When AMF pages the UE via a new gNB, the AMF needs to indicate the new gNB LP-WUS is enabled or disabled. One option to send the indication is the source gNB includes LP-WUS enabling or disabling in NGAP paging container, then AMF should be transparent to the indication.

This option can achieve disabling and enabling per UE.

This solution has RAN3 impact, may also has SA2 impact. Another drawback for this RRC based solution is the LP-WUS strategy is impacted in different gNB. The old gNB LP-WUS usage strategy is also used in the new gNB, which the new gNB may not expect. RAN2 has to consider the validity of the dedicated LP-WUS disabling and enabling.

Option 2a: RRC based solution without other WG impact

This is to address the drawback of RRC based solution in option 2. The gNB can disable or enable LP-WUS using dedicated RRC message, e.g. RRC release message, and the enabling and disabling indication is only valid in current cell. If the UE reselects to another cell, then the indication is not valid any more.

This option can achieve disabling and enabling per UE.

This solution only impacts RAN2 specification and has no impact on other WGs.

Option 3: Existing NAS based solution

In this solution, UE indicate support of CN-assigned subgrouping as today, AMF can disable or enable LP-WUS based on this UE NAS capability. From AMF perspective, AMF does not need to know whether UE supports UE-based subgrouping. AMF can blindly disable LP-WUS to some UEs.

This option can achieve disabling and enabling per UE.

This solution has no impact on specification.

Option 4: Comparing UE i-DRX with the offset between an LO and a reference PO/PF

This solution If UE i-DRX is no less than the value of the offset, the UE/network enables LP-WUS monitoring. Otherwise, the UE and network disables LP-WUS monitoring.

This solution will introduce new UE behaviour to compare the DRX and wake up offset to determine whether to use LP-WUS.

This option can achieve disabling and enabling per a group of UE.

Option 4a: Comparing UE supported wake up delay with the offset between an LO and a reference PO/PF

In current running TS 38.304 CR, it is already specified that UE should compare supported wake up delay and configured offset to determine whether to use LP-WUS or legacy paging as follows.

If single value is configured for lpwus-LoFrameOffsetList for a PO, and if the gap between the LO and the corresponding PO is no less than the wake-up delay that a UE supports, the UE monitors the LO associated with the offset, otherwise the UE follows the paging monitoring procedure as described in clause 7.1 or 7.2.

If more than one values are configured for lpwus-LoFrameOffsetList for a PO , and if the gap between the LO associated with at least one offset and the corresponding PO is no less than the wake-up delay that a UE supports, the UE monitors the LO associated with the smallest offset value with which the gap between the LO and the corresponding PO is no less than the wake-up delay, otherwise the UE follows the paging monitoring procedure as described in clause 7.1 or 7.2.

Then if gNB wants to disable or enable LP-WUS for some UEs, gNB can set proper offset, which could be smaller or larger than supported wake up delay supported by some UEs, and then these UE will be disabled or enabled for LP-WUS.

This solution has no impact on specification.

This option can achieve disabling and enabling per a group of UE.

Summary

The following table summarizes impact and granularity of the options

| Options | Impact | Effect |
| --- | --- | --- |
| Option 1: NAS based solution | RAN3, SA2, CT1 | Per UE |
| Option 2: RRC based solution | RAN2, RAN3, SA2(possible) | Per UE |
| Option 2a: RRC based solution without other WG impact | RAN2 | Per UE |
| Option 3: Existing NAS based solution | None | Per UE group |
| Option 4: Comparing UE i-DRX with lo-Offset | RAN2 | Per UE group |
| Option 4a: Comparing UE supported wake up delay with lo-Offset | None | Per UE group |

Considering this is the last RAN2 meeting to close the WID, a solution should be selected without other working group impact. Option 2a or option 4a only has RAN2 impact or no impact and can be considered.

- RAN2 selects one of the following solutions to disable or enable LP-WUS.

Option 4a: gNB sets proper offset between an LO and a reference PO/PF, UE determines whether to disable LP-WUS monitoring via comparing the offset and supported wake up delay.

Option 2a: The gNB can disable or enable LP-WUS using dedicated RRC message, e.g. RRC release message, and the enabling and disabling indication is only valid in current cell.

# Conclusion

LP-WUS disabling and enabling

Observation 1: UE will compare supported wake-up delay and gNB configured wake-up offset to determine whether to enable LP-WUS.

- RAN2 selects one of the following solutions to disable or enable LP-WUS.

Option 4a: gNB sets proper offset between an LO and a reference PO/PF, UE determines whether to disable LP-WUS monitoring via comparing the offset and supported wake up delay.

Option 2a: The gNB can disable or enable LP-WUS using dedicated RRC message, e.g. RRC release message, and the enabling and disabling indication is only valid in current cell.

# Reference

[1]    RP-234056: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)