3GPP RAN WG2 Meeting #131 	R2-2506062

Bangalore, India, August 25th – 29th, 2025

Agenda Item:	8.2.3

Source:	HONOR

Title: 	Discussion on A-IoT random access

Document for:	Discussion and decision

1	Introduction

During the after-meeting email discussion for the MAC open issues, two issues are identified as to be discussed by company contributions:

| Subgroup: CBRA procedure related |  |  |
| --- | --- | --- |
| Issue 2-4: CBRA failure detection | Further down selection between option B and C for msg2 monitor window in CBRA. - 1 Exclude the option of MSG2 transmission and any retransmission of MSG2 happens within a predefined time window (based on timer) - 2 A device expecting MSG2 assumes CBRA failure if its MSG2 is not received before a boundary, where the boundary can be further downselected between option B and C below. A device receiving MSG2 within this boundary transmits MSG3. The device does not process MSG2 (re)transmission received after the boundary. - Option B – the boundary is the reception of either the next R2D trigger message or the subsequent paging message - Option C – the boundary is the reception of either the kth R2D trigger message or the subsequent paging message (K is FFS) - Option A (the boundary being the subsequent paging only) is excluded. - For option C, further discuss in terms of complexity at the device vs reader flexibility. - 3 Including frequency index along with RN16 in MSG2 to reduce collisions of MSG1 between different devices is feasible. FFS Discuss further whether to include it. - Status in running CR: the common part of option B and C is captured, while the difference part is in [] and highligted. | To be discussed by company contributions |
| Subgroup: Msg2 content |  |  |
| Issue 2-5: random ID differentiation in Msg2 | Whether to include frequency index along with RN16 in MSG2 to reduce collisions of MSG1 between different devices. - In previous meetings, RAN2 discussed whether Msg2 need to include more information on top of the random ID to avoid random ID collision, but there was no consensus. - RAN2#130 agreement: Including frequency index along with RN16 in MSG2 to reduce collisions of MSG1 between different devices is feasible. FFS Discuss further whether to include it. - Status in running CR: not captured yet. | To be discussed by company contributions |

In this contribution, we analyze the remaining issues of the A-IoT random access and provide proposals related to A-IoT random access procedure.

- Discussion

Ambient IoT, which is introduced as a new 3GPP IoT technology, aims to rely on ultra-low complexity devices with ultra-low power consumption for the very-low end IoT applications and address some representative use cases.

2.1 Issue 2-4: CBRA failure detection

In Release 19 A-IoT WI [1], the contention-based random access is limited to the 3-step only procedure. The monitoring window for the failure detection is one left issue in the CBRA procedure.

During last meeting, companies brought up different definitions of the boundary of the Msg2 reception as following:

- Option B – the boundary is the reception of either the next R2D trigger message or the subsequent paging message.

- Option C – the boundary is the reception of either the kth R2D trigger message or the subsequent paging message (K is FFS).

Option C is introduced to give more flexibility to the reader by extend the boundary of Msg2 reception to the Kth access trigger message or the subsequent paging message. The device would only deem the procedure as failure when there is no echoed random number (RN) in all the Msg2(s) before the extended boundary. The maximum number of echoed RN is 8 since frequency resource indication to schedule the Msg3 could only indicate 8 values of small frequency shift factor at most. Thus, even the reader could send the Msg2 for all the Msg1(s) received within boundary divided by the kth access triggered message, multiple Msg2(s) may be sent.

Observation 1: Multiple Msg2(s) may still be needed to echo all the Msg1(s) received within boundary divided by the kth triggered message since the Msg2 could only carry 8 RN at most.

Thus, the device needs to decode every Msg2 and compares every RN in the Msg2(s) to check whether it is echoed and transmit the Msg3 in the scheduled resource to complete the access procedure. This would result in more power consumption for the device. Even more, the device needs to perform the counting of the access trigger message. Extended boundary would also bring more RN collision.

Observation 2: From the perspective of the device, it would monitor the Msg2 for a longer period and decoded more Msg2 within the extended boundary.

Although the reader could not schedule the Msg3 across different access trigger message, the reader still has the flexibility to delay the next trigger message if it doesn’t want to echo some Msg1(s) immediately. The delay is acceptable since there is no latency requirement for A-IoT services and there is no parallel service.

Proposal 1: (Issue 2-4) RAN2 to confirm the Option B which reduces the device power consumption and complexity.

2.2 Issue 2-5: Random ID differentiation in Msg2

Since there may be some cases that the same RN is transmitted in the same slot or different slot. For the same RN is transmitted in the same slot case, if only the RN is echoed in the Msg2, the two devices would both transmit the Msg3 with the same frequency resource based on explicit indication in the PRDCH for Msg2 transmission.

Observation 3: Devices would transmit the Msg3 with the same frequency resource if the same RN is transmitted within the boundary.

Based on the current mechanism, the device would compare each RN in Msg2 with its RN transmitted in Msg1 to check if it needs to transmit the Msg3 with its device ID. This is workable with limited efficiency. Thus, we prefer to have a mechanism to provide information about whether the RN is echoed to the device based on the information related to the Msg1 transmission resource. Based on the information related to the Msg1 transmission resource, the device could jump to the entry for its resource and could identify whether it is echoed even when its RN is collision with another device access in the same resource set.

Proposal 2: (Issue 2-5) Information mapping to the resources following the access trigger message could be used to indicate if RN is echoed the device using the resource to transmit its Msg1.

Observation 4: Based on the information mapping to the resources, the number of the echoed devices in the Msg2 could be derived.

2.3 Other open issues

Extended use cases of the NACK feedback

Msg3 carries the device ID in a higher layer container to ack the inventory request. That is, from the perspective of devices, the format of Msg3 message is similar to that of the message containing D2R data since the D2R data and device ID are both included in the higher layer container instead of MAC PDU. Furthermore, there may be some circumstances that the reader would like to terminate the data transmission of one device to enable remaining devices in the following slot to complete their access/data transmission. The NACK to trigger re-access could be used to terminate the on-going procedure after D2R data transmission.

Proposal 3: NACK could also be applied to D2R data (e.g., NACK for command response).

3	Conclusions

In this contribution, we discussed the issues related to the random access procedure and failure handling aspects on Ambient IoT. Based on the  discussion, the following observation and proposals are concluded:

Observation 1: Multiple Msg2(s) may still be needed to echo all the Msg1(s) received within boundary divided by the kth triggered message since the Msg2 could only carry 8 RN at most.

Observation 2: From the perspective of the device, it would monitor the Msg2 for a longer period and decoded more Msg2 within the extended boundary.

Proposal 1: (Issue 2-4) RAN2 to confirm the Option B which reduces the device power consumption and complexity.

Observation 3: Devices would transmit the Msg3 with the same frequency resource if the same RN is transmitted within the boundary.

Proposal 2: (Issue 2-5) Information mapping to the resources following the access trigger message could be used to indicate if RN is echoed the device using the resource to transmit its Msg1.

Observation 4: Based on the information mapping to the resources, the number of the echoed devices in the Msg2 could be derived.

Proposal 3: NACK could also be applied to D2R data (e.g., NACK for command response).

4	References

- RP-250796 New Work Item: Solutions for Ambient IoT (Internet of Things) in NR