3GPP TSG-RAN WG2 Meeting #131	R2-2506152

Bengaluru, India, 25 – 29 August 2025                                       Revision of R2-2504617

Agenda item:	8.9.2

Source:	Sateliot, Thales, Novamint, CATT, Samsung, Ericsson, Nordic

Title:	On Satellite ID aspects for S&F Satellite operation

WID/SID:	IoT_NTN_Ph3 - Release 19

Document for:	Discussion and Decision

# 1	Introduction

As part of the 5GSAT_Ph3-ARC WID, SA2 agreed a core solution to support Store and Forward (S&F) operation as specified mainly in [1] – the main highlights are outlined below with emphasis on the expected UE behaviour when receiving a S&F Wait Timer and/or a S&F Monitoring List:

- The Store and Forward Satellite operation applies in E-UTRAN with satellite access and is suitable for delay-tolerant communication services (e.g. CIoT/MTC, SMS, etc.).

- When an on-board MME supports S&F Satellite operation and is operating in S&F Mode, the on-board eNB broadcasts an indication of operating in the S&F Mode, which the UE uses to determine the current operation mode of the satellite. If a satellite is operating in the S&F Mode and if a UE is enabled for S&F Satellite operation, then the UE indicates the S&F capability during Attach and Tracking Area Update.

- The MME may optionally provide to the UE a S&F Wait Timer, a S&F Monitoring List or both when accepting or rejecting a NAS procedure.

- When the S&F Wait Timer expires, the UE may perform a NAS procedure, which can be a subsequent NAS procedure or a reattempt of a NAS procedure previously rejected with a S&F reject cause

- How a UE behaves when receiving the S&F Monitoring List is up to UE implementation. When a UE receives a S&F Monitoring List and the UE accesses a satellite that supports Store and Forward Satellite operation that is not on the S&F Monitoring List there is increased probability that it will not be able to complete the NAS procedure.

- The S&F Wait Timer or S&F Monitoring List doesn't affect the UE when accessing an eNB that does not broadcast an indication of operating in the S&F Mode.

A more detailed discussion on the intended use of the “S&F Monitoring List” and “S&F Wait Timer” in a deployment based on the split-MME architecture is provided in R2-2504617 [4]. The main objective of the discussion is to stress that the use of these two features is not motivated by UE power saving but mainly intended to prevent UE from accessing satellites with no UE context or where the UE context has not been yet updated/synchronized.

In a different front, RAN WID on IoT_NTN_Ph3 [3] indicated the following in relation to supporting S&F operations although any impact to ME is generally within the scope of this WID:

- Strive to minimise UE impact

In a recent RAN #130, agreement was reached on the following points:

Agreements:

1.	When the S&F operation indication is present, the agreed time information in SIB31 (i.e., t-ModeSwitching) indicates when the normal mode will start; otherwise, the agreed time information in SIB31 (i.e., t-ModeSwitching) indicates when the S&F mode will start.

2.	In a S&F network deployment which also exhibits discontinuous coverage, existing mechanisms to handle discontinuous coverage can be leveraged (e.g. satellite assistance information, UE not needing to perform idle mode tasks when the UE determines that is out of coverage, etc.). There is no need to modify existing discontinuous coverage features due to the addition of S&F Satellite operation. FFS if we clarify in discontinuous coverage procedure in idle mode that the UE also takes into account the information about NAS configured S&F monitoring list.

3.	Clarify in Stage 2 from Rel-18 (so in general, not only for S&F) that the values used in “satelliteId-r17” are expected to correspond to the values used in “satelliteId-r18” in other SIBs (e.g. the value used to identify the serving satellite in SIB33, SIB32 and SIB31 should be the same). Come back in the next meeting with an actual Rel-18 CR for this. FFS if in Rel-19 we need to say anything about the correspondence between the “satelliteId” IE and “S&F Monitoring List”.

Working Assumption:

1.	In the neighbour cell list we introduce an indication whether the cell operates in the S&F mode or not (FFS if we also include the transition time). This WA can only be confirmed if we converge on the corresponding UE behaviour.

Some remaining issues      as identified in RAN2 #130 especially in relation to the S&F mode of operation are:

- Cell (re)selection enhancements to consider “S&F Monitoring List” and “S&F Wait Timer”.

- Relaxation of idle mode tasks in a S&F network deployment exhibiting discontinuous coverage.

- WA on introducing the S&F mode indication in the neighbour cell list

- Aligning the use of satellite IDs.

- Scope and uniqueness of satelliteId.

This paper covers issues iv) and v).

Issues i), ii) and iii) are covered in R2-2506151.

# 2	Discussion

## 2.1      Aligning the use of Satellite IDs

The following discussion was held in RAN2#130:

Complementing above information, SatelliteIds used in the different SIBs (i.e., SatelliteId-r17 and SatelliteId-r18) are encoded as a binary coded integer value from 0 to 255 (i.e., INTEGER (0..255)). This is also the case for the satellite ID defined by CT1 in [7] and used in the S&F monitoring list. Therefore, all satellite IDs defined so far and used in AS and NAS are of the same type.

In this respect, a UE that has been provided with a “S&F Monitoring List”, which includes a list of satelliteIDs, is expected to determine the satelliteID value of NTN cells operating in S&F from the satelliteID information broadcast in the SIBs.

Therefore, it is important to properly establish the correspondence between the satelliteIDs values broadcast in SIBs and the values used by the NAS layer in the MME for S&F Satellite operation.

Observation 1     : AS-based and NAS-based satellite IDs are of the same type.

Observation 2     : A UE that has been provided with a “S&F Monitoring List”, which includes a list of satelliteIDs, is expected to determine the satelliteID value of NTN cells operating in S&F from the satelliteID information broadcast in the SIBs.

Proposal 1     : RAN2 is respectively requested to add a stage-2 description in Rel-18 specs to clarify that the satellite identifier value used in different SIB messages to identify the satellite assistance information of one certain satellite should be the same.

Proposal 2     : RAN2 is respectfully requested to add a stage-2 description in Rel-19 specs to state that the satellite identifier values used at the AS-level (e.g., satellite identifiers included in SIB messages) and the NAS-level (e.g. satellite identifiers included in the S&F Monitoring list) are sent consistently, i.e. with the same value, when they refer to the same satellite (related TP provided in the Annex)

## 2.2      Scope and uniqueness of Satellite ID

The following discussion was held in RAN2#128:

As indicated above, it was decided as of RAN2#128 that the respective discussion on how to maintain unique satellite IDs would continue once aspects as discussed in section 2.5 got resolved. Given that maintaining uniqueness of satellite IDs is equally important without having to use much of scarce SIB space, a combination of satelliteID+PLMN code may be considered to maintain global unique identifiers for satellites.

Proposal 3: RAN2 is respectfully requested to continue discussion on the scope and uniqueness of satellite IDs, considering options such as:

- A satellite identifier value should uniquely identify a satellite to the UE within a PLMN

- Quasi(?)-Global uniqueness based on an identifier such as PLMN

- Cell-specific uniqueness

- Up to network/UE implementation (e.g. the satellite Id does not have to be unique for the whole life of the network, but maybe only for a sufficiently long period of time ensuring that an assistance information got from SIB by the UE has expired and so, not be used)

Proposal 4: RAN2 is respectfully requested to capture outcome of P3 in Stage 2.

Proposal 5: If uniqueness is established to be quasi(?)-global and the value range of existing defined satellite IDs (satelliteId-r17/r18) is not enough, then introduce new satelliteId-r19.

Proposal 6: RAN2 discuss informing other WGs for alignment.

# 3	Conclusion

This paper has analysed the remaining open issues as identified as of RAN2 #130 in the context of S&F operation and makes the following observations and proposals:

Alignment of Satellite IDs:

Observation 1     : AS-based and NAS-based satellite IDs are of the same type.

Observation 2     : A UE that has been provided with a “S&F Monitoring List”, which includes a list of satelliteIDs, is expected to determine the satelliteID value of NTN cells operating in S&F from the satelliteID information broadcast in the SIBs.

Proposal 1     : RAN2 is respectively requested to add a stage-2 description in Rel-18 specs to clarify that the satellite identifier value used in different SIB messages to identify the satellite assistance information of one certain satellite should be the same.

Proposal 2     : RAN2 is respectfully requested to add a stage-2 description in Rel-19 specs to state that the satellite identifier values used at the AS-level (e.g., satellite identifiers included in SIB messages) and the NAS-level (e.g. satellite identifiers included in the S&F Monitoring list) are sent consistently, i.e. with the same value, when they refer to the same satellite (related TP provided in the Annex)

Scope and uniqueness of Satellite IDs:

Proposal 3: RAN2 is respectfully requested to continue discussion on the scope and uniqueness of satellite IDs, considering options such as:

- A satellite identifier value should uniquely identify a satellite to the UE within a PLMN

- Quasi(?)-Global uniqueness based on an identifier such as PLMN

- Cell-specific uniqueness

- Up to network/UE implementation (e.g. the satellite Id does not have to be unique for the whole life of the network, but maybe only for a sufficiently long period of time ensuring that an assistance information got from SIB by the UE has expired and so, not be used)

Proposal 4: RAN2 is respectfully requested to capture outcome of P3 in Stage 2.

Proposal 5: If uniqueness is established to be quasi(?)-global and the value range of existing defined satellite IDs (satelliteId-r17/r18) is not enough, then introduce new satelliteId-r19.

Proposal 6: RAN2 discuss informing other WGs for alignment.

# Reference

- TS 23.401, General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access, V19.2.0 (2025-03).

- S2-2502450, Reply LS on Satellite IDs for store-and-forward operation, Greece, Feb. 2025.

- RP-242397, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, MediaTek, Sep. 2025.

- R2-2504617, Discussion on “S&F Monitoring List” and “S&F Wait Timer” and potential RAN2 impacts, Sateliot, Malta, May. 2025.

- TS 36.304, Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode, V18.4.0, (2025-06)

- TS 23.122, Non-Access-Stratum (NAS) functions related to Mobile Station (MS) in idle mode, V19.3.0 (2025-06).

- TS 24.301, Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3, V19.2.0 (2025-03).

- TS 36.331, Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification, V18.5.0 (2025-04)

# Annex: TP on TS 36.300

## 23.21	Support for BL UEs, UEs in enhanced coverage and NB-IoT UEs over Non-Terrestrial Networks

### 23.21.1  General

Support for BL UEs, UEs in enhanced coverage and NB-IoT UEs over Non-Terrestrial Networks (see clause 4.12) is only applicable to E-UTRA connected to EPC. UEs not supporting NTN are barred from accessing an NTN cell.

In NTN, only BL UEs, UEs in enhanced coverage and NB-IoT UEs with GNSS capability are supported in this release of the specification.

To accommodate long propagation delays in NTN, increased timer values and window sizes, or delayed starting times are supported for the physical layer and for higher layers.

UL segmented transmission is supported for UL transmission with repetitions. The UE shall apply UE pre-compensation per segment of UL transmission of PUSCH/PUCCH/PRACH for BL UEs or UEs in enhanced coverage and NPUSCH/NPRACH for NB-IoT UEs from one segment to the next segment.

In this release of the specification, NTN is only applicable to FDD system.

In NTN, the distance refers to Euclidean distance.

The satellite identifier value used in different SIB messages to identify the satellite assistance information of one certain satellite should be the same.

The satellite identifier values used at the AS-level (e.g., SIB31, SIB32, SIB33) and the NAS-level (e.g. satellite identifiers included in the S&F Monitoring list) are sent consistently, i.e. with the same value, when they refer to the same satellite.