3GPP TSG-RAN WG2 Meeting #131	R2-2506166 is a revision R2-2506131

Bengaluru, India, 25 – 29 August 2025

Agenda item:	8.11.3

Source:	Nokia

Title:	Other Aspects of SBFD

WID/SID:	NR_duplex_evo-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In RAN#102 meeting, a Work Item (WI) on Evolution of NR duplex operation: Subband full duplex (SBFD) was approved. The work item description has been updated in RAN#104 meeting and the objectives are mentioned in [RP-241614].

In this contribution we discuss RAN2-aspects relevant to SBFD operation which are not covered in the Random Access agenda item. In particular, we cover L3 CSI-RS measurements, semi-persistent L1 CLI measurement resources, emergency call handling, as well as also issues brought by rapporteur in the email discussion [Post130][217][SBFD] List of open issues of RRC impact (Huawei)

# 2		Mobility aspects in SBFD

Mobility measurements, e.g. RSRP and RSSQ, can be based on SSB or CSI-RS signals. CSI-RS are impacted by SBFD operation as noted in the following agreements made by RAN1:

RAN1#117: Working Assumption For frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs, support one contiguous CSI-RS resource allocation with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL usable PRBs. - No impact on CSI-RS sequence generation - CSI-RS sequence mapping is applied to CSI-RS resources within DL usable PRBs only (effectively, this is same as the case when the CSI-RS sequence mapped to the RBs outside the DL usable PRBs are punctured) - RAN1 to further study the impact on CSI processing timeline in SBFD symbols to process the CSI-RS across the two DL subbands Agreement For a contiguous CSI-RS resource which overlaps with SBFD subband boundaries, only CSI-RS frequency resources within DL usable PRBs are valid for SBFD-aware. - No impact on CSI-RS sequence generation - CSI-RS sequence mapping is applied to CSI-RS resources within DL usable PRBs only (effectively, this is same as the case when the CSI-RS sequence mapped to the RBs outside the DL usable PRBs are punctured)

One open aspect related to CSI-RS based L3 mobility is about the UE assumptions for the inter-cell CSI-RS measurements on the time/frequency slots configuration of neighbouring cells. We distinguish 2 options:

- Option 1: Explicit signalling of the time and frequency location of the SBFD symbols/subbands of neighbouring cells. This would resolve any ambiguity at the UE side about whether all the configured CSI-RS resources are available, or whether some of the CSI-RS resources are not available due to overlapping with an UL subband.

- Option 2: No additional signalling is required. The UE configured for inter-cell CSI-RS is not informed about the time and frequency configuration of the neighbour cells.

Our preferred solution would be Option 1 since it removes any ambiguity at the UE. However, given the limited TUs left, we can compromise and therefore Option 2 is supported. In such case, the UE assumes that the configured inter-cell CSI-RS resources are available for measurements and therefore no specification impact is expected.

Proposal 1: Do not introduce enhancements for L3 mobility CSI-RS measurements in SBFD. No explicit signalling of the time and frequency configuration of the SBFD symbols/subbands of neighbour cells.

# 2 	SBFD with NR-DC

RAN1 replied in [R1-2504857] about the feasibility of simultaneous configuration of SBFD and NR-DC. According to RAN1, SBFD and NR-DC can be configured simultaneously only for inter-band NR-DC as long as a UE is capable of simultaneous transmission and reception indicated by UE capability simultaneousRxTxInterBandCA, where SBFD operation is applicable on only one NR TDD carrier. During the post RAN2#130 offline discussions, companies provided their views on the support of SBFD and NR-DC. The conclusion from the discussion is that, from a RAN2 point of view, the simultaneous configuration of SBFD and NR-DC can be supported, given the conditions expressed by RAN1. However, some companies expressed concerns on impact to other WGs such as RAN4. Thus, we prefer to follow the rapporteur and ask RAN4 whether they see any issue with the support of these 2 features together.

Proposal 2: RAN2 understands the simultaneous configuration of SBFD and NR-DC can be supported from RAN2 perspective. The SBFD operation is only applicable on only one NR TDD carrier and subject to UE capability. Send reply LS to RAN4 (CC RAN1) with this understanding.

# 3	Semi-persistent L1 CLI measurements

Regarding SP CLI measurements, RAN2 agreed the following working assumption in #129bis:

Working assumption: The configured SP CLI measurement resource sets are initially deactivated upon (re-) configuration by upper layers and after reconfiguration with sync.

We would like to confirm the working assumptions. This way of configuring the semi-persistent resources is common practice in MAC specifications, and it has been used for other measurement resources such as CSI-RS/CSI-IM and ZP-CSI-RS resource sets.

Proposal 3: The configured SP CLI measurement resource sets are initially deactivated upon (re-) configuration by upper layers and after reconfiguration with sync.

# 4	Emergency call handling

Emergency calls are critical, thus it is imperative to minimize the risk of interruptions or temporarily too poor quality in the voice transmission. In cells utilizing SBFD this can be an issue if not taken into account when allocating resources for emergency calls or mission critical services with a low data rate. This is particularly important for non SBFD aware UEs. Assuming that the cell operates with a mix of DL-only, UL-only and SBFD slots, the DL resource allocation for a UE making an emergency call should take place in DL-only slots, and the UL resource allocation should take place in UL-only slots for legacy UEs but likely also for SBFD aware UEs as the NW may not be aware of the UEs' SBFD capability at the call setup procedure.

Proposal 4: The NW should ensure that when establishing voice calls and especially emergency calls (at least for non SBFD aware UEs), resources are not allocated to SBFD symbols (i.e. resources should be DL-only or UL-only symbols).

# 5	Conclusion

Proposal 1: Do not introduce enhancements for L3 mobility CSI-RS measurements in SBFD. No explicit signalling of the time and frequency configuration of the SBFD symbols/subbands of neighbour cells.

Proposal 2: RAN2 understands the simultaneous configuration of SBFD and NR-DC can be supported from RAN2 perspective. The SBFD operation is only applicable on only one NR TDD carrier and subject to UE capability. Send reply LS to RAN4 (CC RAN1) with this understanding.

Proposal 3: The configured SP CLI measurement resource sets are initially deactivated upon (re-) configuration by upper layers and after reconfiguration with sync.

Proposal 4: The NW should ensure that when establishing voice calls and especially emergency calls (at least for non SBFD aware UEs), resources are not allocated to SBFD symbols (i.e. resources should be DL-only or UL-only symbols).