3GPP TSG-RAN WG2 meeting #131bis	R2-2506702

Agenda Item:	2.2

Source: 	ETSI MCC

Title:		Report of 3GPP TSG RAN WG2 meeting #131, Bengaluru, India

Report of 3GPP TSG RAN WG2 meeting #131

Bengaluru, India

25 - 29 August, 2025

Document for: Approval

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

# Contents

Organisation of the meeting	7

Statistics/Executive Summary	7

General	8

1	Opening of the meeting	8

1.1	Call for IPR	8

1.2	Network usage conditions	8

1.3	Other	8

2	General	9

2.1	Approval of the agenda	9

2.2	Approval of the report of the previous meeting	9

2.3	Reporting from other meetings	9

2.4	Instructions	9

2.5	Others	10

3	Incoming liaisons	11

4	EUTRA Rel-17 and earlier	12

4.1	EUTRA corrections Rel-17 and earlier	12

4.3	Positioning corrections Rel-16 and earlier	13

5	NR Rel-15 and Rel-16	13

5.1	Common	13

5.1.1	Stage 2 and Organisational	14

5.1.2	User Plane corrections	14

5.1.2.1	MAC	14

5.1.2.2	RLC PDCP SDAP BAP	15

5.1.3	Control Plane corrections	15

5.1.3.1	NR RRC	15

5.1.3.2	UE capabilities	18

5.1.3.3	Other	19

5.3	NR Positioning Support	19

6	NR Rel-17	22

6.1	Common	22

6.1.1	Stage 2 and Organisational	23

6.1.2	User Plane corrections	23

6.1.3	Control Plane corrections	25

6.1.3.1	NR RRC	25

6.1.3.2	UE capabilities	27

6.1.3.3	Other	28

6.3	NR positioning enhancements	29

7	Rel-18	29

7.0	Common	29

7.0.1	UE Capabilities	29

7.0.2	Rel-18 corrections	29

7.0.2.1	RACH-less HO	29

7.0.2.2	NR network-controlled repeaters	29

7.0.2.3	NR support for UAV	29

7.0.2.4 	Mobile Terminated Small Data Transmission	29

7.0.2.5	IDC enhancements for NR and MR-DC	29

7.0.2.6	Mobile IAB (Integrated Access and Backhaul) for NR	29

7.0.2.7	Timing Resiliency and URLLC Enh	29

7.0.2.8	Enhanced support of reduced capability NR devices	30

7.0.2.9	Further NR coverage enhancements	30

7.0.2.10	Network energy savings for NR	30

7.0.2.11	Further enhancement of data collection for SON MDT in NR and EN-DC	30

7.0.2.12	Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR	32

7.0.2.13	NR MIMO evolution	33

7.0.2.14	Enhancements of NR Multicast and Broadcast Services	34

7.0.2.15	Enhancement on NR QoE management and optimizations for diverse services	34

7.0.2.16	XR Enhancements for NR	34

7.0.2.17	NR NTN enhancements	34

7.0.2.18	IoT NTN enhancements	35

7.0.2.19	Enhanced NR Sidelink Relay	36

7.0.2.20	NR Sidelink evolution	38

7.0.2.21	Expanded and improved NR positioning	38

7.0.2.22	Further NR mobility enhancements	42

7.0.2.23	TEI18	44

7.0.2.24	Others	44

8	Rel-19	45

8.0	General	45

8.0.0	In-principle agreed CRs	45

8.0.1	Other	49

8.1	AI/ML for NR air interface	52

8.1.1	Organizational	52

8.1.2	Functionality based LCM	59

8.1.2.1	LCM for NW-sided model for Beam Management use case	59

8.1.2.2	LCM for UE-sided model for Beam Management use case	59

8.1.2.3	LCM for Positioning use case	69

8.1.3	NW side data collection	73

8.1.4	UE side data collection	81

8.1.5	Model transfer/delivery	83

8.2	Ambient IoT	83

8.2.1	Organizational	83

8.2.2	A-IoT Paging	88

8.2.3	A-IoT Random Access	90

8.2.4	A-IoT Data Transmission and Other general aspects	92

8.3	AI/ML for Mobility	96

8.3.1	Organizational	96

8.3.2	UE sided model	97

8.3.2.1	Functionality management for RRM measurement prediction	97

8.3.2.2	Functionality management for RRM measurement event prediction	101

8.3.2.3	Data collection	103

8.3.3	Network sided model	107

8.4	Low-power wake-up signal and receiver for NR (LP-WUS/WUR)	110

8.4.1	Organizational	111

8.4.2	Procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE	117

8.4.3	RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	122

8.4.4	Procedures for LP-WUS in RRC_CONNECTED	125

8.5	Network Energy Saving Enh.	126

8.5.1	Organizational	127

8.5.2	On-demand SSB SCell operation	130

8.5.3	On-demand SIB1	133

8.5.4	Adaptation of common signal/channel transmissions	137

8.6	Mobility Enhancement Ph4	139

8.6.1	Organizational	139

8.6.2	Inter-CU LTM	145

8.6.3	L1 event triggered measurement reporting	147

8.6.4	Conditional intra-CU LTM	149

8.7	XR Enhancements Ph3	152

8.7.1	Organizational	152

8.7.2	Multi-modality support	156

8.7.3	RRM measurement gaps/restrictions related enhancements	156

8.7.4	Scheduling enhancements	156

8.7.5	RLC enhancements	159

8.7.6	XR rate control	164

8.8	NTN for NR Ph3	169

8.8.1	Organizational	169

8.8.2	Downlink coverage enhancements	174

8.8.3	Uplink Capacity/Throughput Enhancement	181

8.8.4	Support of Broadcast service	181

8.8.5	Support of regenerative payload	183

8.8.6	LTE to NR NTN mobility	184

8.9	IoT NTN Ph3	184

8.9.1	Organizational	184

8.9.2	Support of Store & Forward	189

8.9.3	Uplink Capacity Enhancement	193

8.9.4	Support of PWS	203

8.10	SON/MDT Ph4	204

8.10.1	Organizational	204

8.10.2	MRO enhancements for Rel-18 mobility features	207

8.10.3	Other	211

8.11	Evolution of NR duplex operation: Sub-band full duplex (SBFD)	213

8.11.1	Organizational	213

8.11.2	Random access in SBFD	216

8.11.3	Other aspects	220

8.12	NR MIMO Phase 5	221

8.12.1	Organizational	221

8.12.2	Asymmetric DL sTRP/UL mTRP	224

8.12.3	Others	225

8.13	NR sidelink multi-hop relay	228

8.13.1	Organizational	228

8.13.2	Relay discovery and (re)selection	234

8.13.3	Control Plane Procedures and SRAP impact	239

8.13.4	Service continuity	244

8.14	Additional topological enhancements	247

8.15	NavIC L1 SPS A-GNSS support	247

8.16	BDS B2b in A-GNSS	247

8.17	IoT-NTN TDD mode	248

8.18	LTE-based 5G Broadcast	254

8.18.1	Organizational	254

8.18.2	Other	255

8.19	TEI19	257

8.19.1	RAN2-led	258

8.19.2	Other WG-led	268

8.20	NR Others	272

8.20.1	RAN4	272

8.20.2	Other WGs	277

9	Breakout session reports	279

9.1	Session on V2X/SL, R19 NES and MOB	279

9.2	Session on Rel-18 MIMO, Rel-19 MIMO, LPWUS, SBFD, NR Others	279

9.3	Session on NR NTN and IoT NTN	280

9.4	Session on positioning and sidelink relay	280

9.5	Session on R19 XR and LTE-based 5G Broadcast	280

9.6	Session on maintenance and SON/MDT	280

Closing of the meeting	280

Annex A:	List of participants	280

Annex B:	List of Tdocs	280

Annex C:	Incoming liaison statements	280

Annex D:	Outgoing liaison statements	285

Annex E:	List of agreed CRs	286

Annex F:	List of email discussions during the meeting	296

Annex G:	Post-meeting email discussions after RAN2#131	297

Long email discussions, for R2-131bis, Sept 26th 10:00 UTC (unless otherwise stated)	308

Annex H:	History	317

# Organisation of the meeting

Meeting:	3GPP TSG RAN2#131

Meeting location:	Bengaluru, India

Duration:	25 - 29.08.2025

Host:	TSDSI

TSG RAN WG2 Chair	Diana Pani (InterDigital) (diana.pani@interdigital.com)

TSG RAN WG2 Vice chair:	Kyeongin Jeong (Samsung) (kyeongin.j@samsung.com)

TSG RAN WG2 Vice chair:	Erlin Zeng (CATT) (erlin.zeng@catt.cn)

TSG RAN WG2 MCC Support:	Juha Korhonen (ETSI MCC) (juha.korhonen@etsi.org)

Email reflector:	3GPP_TSG_RAN_WG2@LIST.ETSI.ORG

Technical documents:	ftp://ftp.3gpp.org/tsg_ran/WG2_RL2/TSGR2_130/Docs

Next meetings:	TSG RAN2#131bis	13 - 17.10.2025, Praque, Czech Republic

TSG RAN2#132	17 - 21.11.2025, Dallas, USA

# Statistics/Executive Summary

TSG RAN2#131bis was a normal face-to-face meeting, with a possibility for two-way remote access.

The topics discussed were:

-	NR Rel-15, Rel-16 and Rel-17 User Plane corrections, Rel-18 UE Capabilities, Network energy savings for NR, Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR, Enhancement on NR QoE management and optimizations for diverse services, XR Enhancements for NR, TEI18, Rel-18 Other, Rel-19 General, AI/ML for NR air interface, Ambient IoT, AI/ML for Mobility, TEI19 - Diana Pani (Chair)

-	NR Sidelink evolution, Further NR mobility enhancements, Network Energy Saving Enh, Mobility Enhancement Ph4 - Kyeongin Jeong (VC)

-	NR MIMO evolution, Low-power wake-up signal and receiver for NR (LP-WUS/WUR), Evolution of NR duplex operation: Sub-band full duplex (SBFD), NR MIMO Phase 5, NR Others - Erlin Zeng (VC)

-	EUTRA corrections Rel-17 and earlier, Rel-17 NR stage 2, RRC and UE capabilities, IoT NTN enhancements, NR NTN enhancements, NTN for NR Ph3, IoT NTN Ph3, IoT-NTN TDD mode, TEI19 - Sergio Parolari

-	NR positioning support, Expanded and improved NR positioning, Enhanced NR Sidelink Relay, NR sidelink multi-hop relay, NavIC L1 SPS A-GNSS support, BDS B2b in A-GNSS, SRS frequency hopping, Periodic assistance data for integrity service alert, On-demand posSIB - Nathan Tenny

-	XR Enhancements for NR, XR Enhancements Ph3, LTE-based 5G Broadcast - Dawid Koziol

-	EUTRA corrections Rel-17 and earlier, NR Rel-15, Rel-16 and Rel-17 (except user plane), Further enhancement of data collection for SON MDT in NR and EN-DC, SON/MDT Ph4 - Mattias Bergström

The statistics from this meeting are:

-	371 f2f participants

-	1617 Tdoc numbers allocated with 1585 available contributions. (See the attached tdoc list)

-	68 incoming liaison statements, out of which 66 were noted. The remaining non-treated liaison will be treated in RAN2#131bis meeting.

-	23 outgoing liaison statements

-	68 at-meeting email discussions

-	152 email approvals/discussions scheduled after the RAN2#131 meeting (95 short and 57 long), see Annex G for details.

Number of CRs submitted: 553. Out of these, 217 were agreed. See Annex E for details.

# General

This meeting was an ordinary meeting and had full decision power, i.e. full decision power to make agreements and approvals according to RAN WG2 terms of reference, without any need to ratify decisions at a later RAN2 or other meeting.

# 1	Opening of the meeting

## 1.1	Call for IPR

The attention of the delegates of this Working Group is drawn to the fact that 3GPP Individual Members have the obligation under the IPR Policies of their respective Organizational Partners to inform their respective Organizational Partners of Essential IPRs they become aware of. The delegates were asked to take note that they were hereby invited: - to investigate whether their organization or any other organization owns IPRs which were, or were likely to become Essential in respect of the work of 3GPP. - to notify their respective Organizational Partners of all potential IPRs, e.g., for ETSI, by means of the IPR Statement and the Licensing declaration forms (https://www.etsi.org/images/files/IPR/etsi-ipr-form.doc)

NOTE:	IPRs may be declared to the Director-General or Chairman of the SDO, but not to the RAN WG2 Chairman.

## 1.2	Network usage conditions

1/ 	To avoid email system overload, please don’t attach files and documents to emails e.g. for offline email discussions, but instead use files placed on the meeting server instead. Inbox/Drafts folder is used for meeting offline discussions.

## 1.3	Other

In accordance with the Working Procedures it is reaffirmed that: (i) compliance with all applicable antitrust and competition laws is required; (ii) timely submissions of work items in advance of TSG or WG meetings are important to allow for full and fair consideration of such matters; and (iii) the chair will conduct the meeting with strict impartiality and in the interests of 3GPP

Note on (i): In case of question please contact your legal counsel.

Note on (ii): WIDs don’t need to be submitted to the RAN2 meeting and will typically not be discussed here either.

Consensus principles reminder The attention of the delegates to the meeting is drawn to the fact that 3GPP endeavours to reach consensus on all decisions and therefore depends on a cooperative spirit of the Individual Members. In particular, Individual Members are encouraged to seek a consensus-based solution and only to sustain objections as a very last resort, and where absolutely necessary and well justified. The leadership will conduct the present meeting in a manner whereby informal methods of reaching consensus are encouraged, whilst ensuring that well justified concerns are taken into account

RAN endorsed working principle for 6G (RP-250766) 3GPP to create lean and streamlined standards for 6G, e.g., by dimensioning an appropriate set of functionalities, minimizing the adoption of multiple options for the same functionality, avoiding excessive configurations, etc. Any exception to the above shall be well justified.

# 2	General

## 2.1	Approval of the agenda

R2-2505001	Agenda for RAN2#131	Chairman	agenda

Approved

## 2.2	Approval of the report of the previous meeting

R2-2505002	RAN2#130 Meeting Report	MCC	report

Approved

## 2.3	Reporting from other meetings

## 2.4	Instructions

CRs

- Use latest CR template version 12.3 for all CRs submitted to RAN2 meeting

Rel-18 and earlier maintenance CRs

- Only essential/critical corrections are expected

- Editorial and clarification corrections should be sent to be reviewed and approved by spec rapporteurs prior to submission.

- Editorials corrections should be collected and submitted by spec rapporteurs.

- NOTE: the tdoc limit applies to all CRs (i.e. WI spec rapporteurs are NO longer expected to submit individual contributions).  They can submit a company CR where they also include miscellaneous corrections that have been sent to them.

Rel-18 UE capabilities

-	EUTRA UE capabilities corrections are covered by separate CRs

-	RAN1/RAN4 NR UE capabilities (new) and corrections are covered in Rel-18 common MegaCRs (38306 and 38331) covering all rel-18 WIs (end outcome).

-	UE capabilities in LPP 37355 and SLPP 38355 are covered in the main CRs for the Positioning WI.

Tdoc limitations

Tdoc limitations doesn’t apply to Rapporteur Input, i.e.

-	Assigned summary rapporteur input of the summary.

-	Email / offline discussions outcomes by discussion rapporteur,

-	Limit of 1 WI/SI  rapporteurs input for WI planning.  The work plan is not expected to be updated/submitted every meeting, unless needed.   It can include progress of other WG groups in the same Tdoc (i.e. separate Tdocs on other WG agreements are not required).

-	TS rapporteur input for TS maintenance.

-	Contact Company of a LSin that triggers RAN2 action may submit one tdoc to facilitate the LS reply. This only applies to one of the contact companies in case there are several (default the first).

Tdoc limitations doesn’t apply to Input created at the meeting, revisions, assigned documents etc.

Tdoc limitations doesn’t apply to shadow / mirror CRs (Cat A), or In-Principle Agreed CRs.

Tdoc limitations applies to all other submitted tdocs (e.g. discussion tdoc and CR tdoc are counted as two).

Postponed CRs still count towards tdoc limit unless 3 or more companies are co-sourcing it.

For each R19 feature, 1 additional tdoc on top of the limit is allowed for a primary co-sourcing company for co-sourced contribution with 4 or more companies.

Open issues

- CR Rapporteurs (as indicated in email discussion scope) are expected to provide open issue list

- Please refer to RAN2 chair guidance document in [POST129bis][001][Organizational] Open issue list.

- CR rapporteurs are expected to ask for inputs, provide proposals on how to resolve the issues or provide limited options to resolve the issue for further discussion online.

- For each issue (before the email discussion deadline), rapporteurs are requested to explicitly indicate whether further contribution input on the open issue is needed.   Input should be requested only for difficult to resolve issues and/or new open issues for which there wasn’t sufficient discussion time to resolve it.

- Companies should follow rapporteurs guidance (i.e. only address open issues for which the rapporteur indicates further input is needed).

- Companies should clearly indicate the open issue number they are addressing in their section and proposal, e.g. Proposal x: (RRC-1) Agree to bla bla

- Companies can discuss UE capabilities in their topic-specific Tdocs

Rel-19 CRs

- CR already agreed in principle but not yet officially agreed must be submitted to RAN2#131 for formal approval under in-principle agreed CRs AIs

- All Rel-19 WI CRs for approval to RAN#109 should be submitted as real CRs to this meeting (i.e. no draftCRs).  All WI CR rapporteurs should ensure that the CRs resulting from post email discussions are submitted as real CRs from beginning of the meeting.

- All Rel-19 CRs should be based on the latest the June version of the specs

- All CRs should follow the CR and formatting rules.

Rel-19 UE capabilities

-	EUTRA UE capabilities are covered by separate CRs

-	All NR UE capabilities will be included common Mega CRs (38306 and 38331) covering all Rel-19 WIs (end outcome).

During the work on NR UE caps:

-	In a Common Rel-19 Agenda Item (AI): RAN1 and RAN4 feature corrections are handled jointly under a common AI, with some explicit exceptions. UE capabilities will be included in UE cap MegaCR directly from UE capability rapporteur

-	In WI-specific Rel-19 Agenda Items: RAN2 specific UE capabilities are handled per WI and endorsed as individual CRs.  Final endorsed CRs will be merged into mega CR post meeting.

Tdoc request/submission for RAN2#131 deadlines:

- Tdoc Submission deadline: August 15th, 1000 UTC

## 2.5	Others

RAN2 election

Chair, 1st Vice Chair, and 2nd Vice Chair to be elected, see 3GPP web page, where information is/will be posted.

RAN2#131 Chair Election

RAN2: Diana PANI - InterDigital Communications (ATIS)

RAN2#131 Vice Chair Election Position 1

RAN2 Vice Chair: Kyeongin JEONG - Samsung Electronics Co., Ltd (TTA)

RAN2#131 Vice Chair Election Position 2

RAN2 Vice Chair:  Erlin ZENG - CATT (CCSA)

R2-2505003	RAN2 Handbook	MCC	discussion

Noted

R2-2506505	RAN2 6G SI Agenda	Chairlady	discussion

-	Tmobile would like to have more proposals per tdoc if co-sourced.

-	Ericsson, LG, Huawei and Nokia thinks that we can have co-sourced but it would count in the quota.

-	Huawei thinks that quota of 5 is a little small so we should at least have 6 so it matches with 6G.

-	Huawei thinks that the NOTE on AI/ML and sensing seems restrictive.

-	LG thinks that we should have 3 lines per proposals and avoid small font size.

-	Qualcomm thinks that since the general AI is very big, it would be good to slightly increase the quota.

Remove co-sourced contribution additional Tdoc limits.  Companies can have co-sourced contributions but will count towards total quote

Increase limit to 6

Proposals should have reasonable readable font and length.  Observations and proposals should fit in one page conclusion

Noted

Review of RAN3 Endorsed stage-2 CRs:

[Post131][999][ Review of RAN3 endorsed CRs (MCC)

Scope: Review and agree RAN3 endorsed stage-2 CRs

Intended outcome: set of agreed CRs

Deadline: two sets of CRs expected from RAN2.

The deadline for the first set:05/09

The deadline for the second set: 10/09

=> Agreed in the 1st set:

- R2-2506506 (38.300); a.k.a. R3-255765

- R2-2506507 (38.300); a.k.a. R3-255766

- R2-2506508 (36.300); a.k.a. R3-255767

- R2-2506509 (36.300); a.k.a. R3-255768

- R2-2506510 (38.300); a.k.a. R3-255818

- R2-2506511 (38.300); a.k.a. R3-255911

- R2-2506512 (38.305); a.k.a. R3-255914

=> Agreed in the 2nd set:

- R2-2506593 (38.300); a.k.a. R3-255981

- R2-2506618 (37.340) (rev of R2-2506594); a.k.a. R3-255992

- R2-2506595 (37.340); a.k.a. R3-255993

- R2-2506596 (38.300); a.k.a. R3-255994

- R2-2506597 (38.300); a.k.a. R3-256013

- R2-2506598 (37.340); a.k.a. R3-256017

- R2-2506599 (37.340); a.k.a. R3-256028

- R2-2506600 (36.300); a.k.a. R3-256030

- R2-2506601 (38.305); a.k.a. R3-256044

- R2-2506602 (38.300); a.k.a. R3-256050

# 3	Incoming liaisons

Note: LSs are moved to the respective agenda items if any.

R2-2505007	LS on draft-ietf-6man-deprecate-router-alert, “Deprecation of the IPv6 Router Alert Option for New Protocols” (contact: Huawei)	IETF IPv6 Maintenance	LS in	To:RAN2, SA2, CT1, CT3 CT4

Noted

# 4	EUTRA Rel-17 and earlier

Only essential corrections. No documents should be submitted to 4. Please submit to 4.x

## 4.1	EUTRA corrections Rel-17 and earlier

(NB_IOTenh4_LTE_eMTC6-Core; leading WG: RAN1; REL-17; WID: RP-211340)

(UPIP_EN-DC_UE; leading WG: RAN3; REL-17; WID: RP-213669)

(LTE TEI17)

Essential corrections to LTE Rel-17 topics not covered by other agenda items.

(NB_IOTenh3-Core; leading WG: RAN1; REL-16; started: Jun 18; Completed: June 20; WID: RP-200293); REL-15 and Earlier NB-IoT WIs are in scope but not listed explicitly (long list).

(LTE_eMTC5-Core; LTE_eMTC5-Core; leading WG: RAN1; REL-16; started: Jun 18; Completed:  June 20; WID: RP-192875;), REL-15 and Earlier eMTC WIs are in scope but not listed explicitly (long list).

(LTE_feMob-Core; leading WG: RAN2; REL-16; started: Jun 18; Completed: June 20; WID: RP-190921);

(LTE_terr_bcast-Core, LTE_DL_MIMO_EE-Core, LTE_high_speed_enh2-Core; LTE TEI16 Non-positioning);

(LTE_NBIOT_eMTC_NTN; leading WG: RAN1; REL-17; WID: RP-211601)

REL-16 and Earlier EUTRA WIs are in scope but not listed explicitly (long list), Except Positioning WI, which is addressed by AIs below.

NOTE that LTE corrections related to NR WIs or Joint NR LTE WIs should be submitted to NR AIs below.

NOTE that LTE corrections which are the same as an NR correction should be submitted to the respective NR AI (so the NR CR and LTE CR can be treated together).

This Agenda Item is treated in the Maintenance Breakout session (Corrections for LTE_NBIOT_eMTC_NTN might be treated in the NTN breakout session)

Mobility

R2-2505202	Correction on LTE RLF report logging	CATT	CR	Rel-16	36.331	16.20.0	5134	-	F	LTE_feMob-Core

R2-2505203	Correction on LTE RLF report logging	CATT	CR	Rel-17	36.331	17.13.0	5135	-	A	LTE_feMob-Core

R2-2505204	Correction on LTE RLF report logging	CATT	CR	Rel-18	36.331	18.6.0	5136	-	A	LTE_feMob-Core

-	Xiaomi agrees that we have a similar sentence in NR, but why not use exactly the same? CATT has no strong view but clarifies that CATT used the LTE style for this sentence.

-	Ericsson wonders if this is a correction of an enhancement? Samsung think this is an enhancement. Samsung suggests Cat B. CATT thinks its not a Cat B. Huawei think it’s a Cat F since the underlying feature was there before. Ericsson is OK with Cat F.

Tick the RAN box, with this they are agreed unseen in R2-2506351, R2-2506352 and R2-2506353

MBMS

R2-2505700	Correction on MBMS Interest Indication	Samsung, Nokia, Ericsson, Qualcomm Incorporated, ZTE	CR	Rel-18	36.331	18.6.0	5125	1	F	LTE_terr_bcast-Core, TEI18	R2-2504325

-	Some of the added text is underlined.

Remove underlining from the text, with this the CR is agreed unseen in R2-2506354.

R2-2505881	Clarification on UTC time offset in IoT NTN	ZTE Corporation, Sanechips	CR	Rel-17	36.331	17.13.0	5145	-	F	LTE_NBIOT_eMTC_NTN-Core

- Xiaomi thinks this CR is similar to a previous one from Nordic and we could have the same approach

- MTK support the principle but legacy UEs can cannot support the change and then the NW ha sto cope with that. Could accept a R19 correction

- Nokia thinks the issue is valid but not a strong view on how to address this

R2-2505886	Clarification on UTC time offset in IoT NTN	ZTE Corporation, Sanechips	CR	Rel-18	36.331	18.6.0	5146	-	A	LTE_NBIOT_eMTC_NTN-Core

Revised in R2-2506282

R2-2506282	Clarification on UTC time offset in IoT NTN	ZTE Corporation, Sanechips	CR	Rel-18	36.331	18.6.0	5146	1	F	LTE_NBIOT_eMTC_NTN-Core

- Ericsson thinks we could have a Rel-17 CR

- MTK thinks that there are already UEs in the NW and then prefer to have a Rel18 CR with a note on the early implementation

Revised in R2-2506283 to add a TEI18 in the coversheet

R2-2506283	Clarification on UTC time offset in IoT NTN	ZTE Corporation, Sanechips	CR	Rel-18	36.331	18.6.0	5146	2	F	LTE_NBIOT_eMTC_NTN-Core, TEI18

Agreed (unseen), but then coversheet revised by MCC: duplicated meeting number

R2-2506630	Clarification on UTC time offset in IoT NTN	ZTE Corporation, Sanechips	CR	Rel-18	36.331	18.6.0	5146	3	F	LTE_NBIOT_eMTC_NTN-Core, TEI18

=> Agreed

Latency Reduction

R2-2506025	Correction on UL spatial multiplexing	ASUSTeK	CR	Rel-17	36.321	17.7.0	1595	-	F	TEI17, LTE_LATRED_L2-Core

R2-2506026	Correction on UL spatial multiplexing	ASUSTeK	CR	Rel-18	36.321	18.4.0	1596	-	A	TEI17, LTE_LATRED_L2-Core

-	Lenovo asks why change from Rel-17, the feature was added in Rel-14? AsusTek thinks is too late to change that far back, AsusTek can consider Rel-18.

-	Lenovo thinks its correct but not essential and can consider Rel-18.

Rel-18 CR is agreed unseen in R2-2506374, but cat F.

## 4.3	Positioning corrections Rel-16 and earlier

(LTE_NavIC-Core, LTE TEI16 Positioning), REL-15 and Earlier WIs related to positioning are in scope but not listed explicitly (long list).

Tdoc Limitation: 1 tdoc

# 5	NR Rel-15 and Rel-16

Essential corrections only.

Tdoc Limitation: 4 Tdocs in total for agenda item 5 (incl. its sub agenda items) and agenda item 6 (incl. its sub agenda items)

In case a correction need to be reflected in both NR TS and LTE TS, the corrections should be submitted under one single AI (so the NR and LTE correction can be treated together), the sub-Ais below this

## 5.1	Common

Includes the following WIs and input that doesn’t fit elsewhere.

(NR_newRAT-Core; leading WG: RAN1; REL-15; started: Mar. 17; closed: Jun. 19: WID: RP-191971)

(NR_IAB-Core; leading WG: RAN2; REL-16; started: Dec 18; target Aug 20; WID: RP-200840)

(NR_unlic-Core; leading WG: RAN1; REL-16; started: Dec 18; Closed June 20; WID: RP-192926).

(NR_IIOT-Core; leading WG: RAN2; REL-16; started: Mar 19; Completed: Jun 20; WID: RP-200797)

(NR_UE_pow_sav-Core; leading WG: RAN1; REL-16; started: Mar 19; Completed Jun 20; WID: RP-200494).

(NR_2step_RACH-Core; leading WG: RAN1; REL-16; started: Dec 18; Completed: June 20; WID: RP-200085).

(SRVCC_NR_to_UMTS-Core; leading WG: RAN2; REL-16; started: Dec 18; Completed; Mar 20; WID: RP-190713)

(RACS-RAN-Core, leading WG: RAN2; REL-16; started: Mar 19; completed: Jun 20; WID: RP-191088)

(NG_RAN_PRN-Core; leading WG: RAN3; REL-16; started: Mar 19; completed: June 20; WID: RP-200122)

(NR_eMIMO-Core, leading WG: RAN1; REL-16; started: Jun 18; target; Aug 20; WID: RP-200474)

(NR_CLI_RIM; leading WG: RAN1; REL-16; started: Dec 18; Completed: Jun 20; WID: RP-191997)

(NR_L1enh_URLLC-Core, leading WG: RAN1; REL-16; Completed: June 20; WID: RP-191584)

(LTE_NR_DC_CA_enh-Core; leading WG: RAN2; REL-16; started: Jun 18; Target Aug 20; WI RP-200791)

(NR_Mob_enh-Core; leading WG: RAN2; REL-16; started: Jun 18; Completed June 20; WID: RP-192277).

(NR_SON_MDT-Core; leading WG: RAN3; REL-16; started: Jun 19; Completed June 20; WID: RP-191776)

(5G_V2X_NRSL-Core; leading WG: RAN1; REL-16; started: Mar 19; completed; Aug 20; WID: RP-200129)

(NR_HST, NR_RRM_enh-Core, NR_RF_FR1, NR_RF_FR2_req_enh, NR_n66_BW, LTE_NR_B41_Bn41_PC29dBm-Core, NR_CSIRS_L3meas,)

(NR TEI16)

LTE mob enh corrections that are common with NR mobility enhancements should be submitted to this AI.

### 5.1.1	Stage 2 and Organisational

Incoming LSs, etc. You should discuss your stage 2 CRs with the specification rapporteurs before submission. Includes impact to 38.300, 36.300, 37.340

R2-2505049	LS on mandatory gaps capability (R4-2508379; contact: Ericsson)	RAN4	LS in	Rel-16	NR_RRM_enh-Core	To:RAN2

Noted

### 5.1.2	User Plane corrections

User Plane corrections will be handled in the User Plane break out session

#### 5.1.2.1	MAC

R2-2506159	Correction to F field in MAC subheader for DL-SCH and UL-SCH	NTT DOCOMO INC., Huawei, HiSilicon, MediaTek Inc., Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, Samsung, ZTE	CR	Rel-15	38.321	15.13.0	2116	-	F	NR_newRAT-Core

-	Xiaomi doesn’t see the issue and this CR was discussed in 2019.

-	Qualcomm thinks in theory it is NBC but it doesn’t impact the UEs that have implemented this feature.   ZTE and Mediatek agrees and the product is implemented according to this CR.  Ericsson thinks that this is NBC for the network but supports it.

-	Vivo thinks that we can keep the spec as it was.

-	Docomo has tested this and there are no UEs detected that have implemented something else.

-	Apple wants to confirm that this impacts only tx side.  Confirmed.

Add Non backward compatible description in cover page

[AT131][008][UP] F field in MAC (NTT docomo)

Intended outcome: update cover page with NBC and agree by email

Deadline:  Thursday by email

R2-2506435	Correction to F field in MAC subheader for DL-SCH and UL-SCH	NTT DOCOMO INC., Huawei, HiSilicon, MediaTek Inc., Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, Samsung, ZTE	CR	Rel-15	38.321	15.13.0	2116	1	F	NR_newRAT-Core

The CR is agreed

R2-2506161	Correction to F field in MAC subheader for DL-SCH and UL-SCH	NTT DOCOMO INC., Huawei, HiSilicon, MediaTek Inc., Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, Samsung, ZTE	CR	Rel-16	38.321	16.20.0	2117	-	A	NR_newRAT-Core

=> Revised in R2-2506436

R2-2506436	Correction to F field in MAC subheader for DL-SCH and UL-SCH	NTT DOCOMO INC., Huawei, HiSilicon, MediaTek Inc., Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, Samsung, ZTE	CR	Rel-16	38.321	16.20.0	2117	1	A	NR_newRAT-Core

=> Revised in R2-2506476

R2-2506476	Correction to F field in MAC subheader for DL-SCH and UL-SCH	NTT DOCOMO INC., Huawei, HiSilicon, MediaTek Inc., Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, Samsung, ZTE	CR	Rel-16	38.321	16.20.0	2117	2	A	NR_newRAT-Core

The CR is agreed

R2-2506162	Correction to F field in MAC subheader for DL-SCH and UL-SCH	NTT DOCOMO INC., Huawei, HiSilicon, MediaTek Inc., Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, Samsung, ZTE	CR	Rel-17	38.321	17.13.0	2118	-	A	NR_newRAT-Core

=> Revised in R2-2506437

R2-2506437	Correction to F field in MAC subheader for DL-SCH and UL-SCH	NTT DOCOMO INC., Huawei, HiSilicon, MediaTek Inc., Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, Samsung, ZTE	CR	Rel-17	38.321	17.13.0	2118	1	A	NR_newRAT-Core

The CR is agreed

R2-2506163	Correction to F field in MAC subheader for DL-SCH and UL-SCH	NTT DOCOMO INC., Huawei, HiSilicon, MediaTek Inc., Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, Samsung, ZTE	CR	Rel-18	38.321	18.6.0	2119	-	A	NR_newRAT-Core

=> Revised in R2-2506438

R2-2506438	Correction to F field in MAC subheader for DL-SCH and UL-SCH	NTT DOCOMO INC., Huawei, HiSilicon, MediaTek Inc., Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, Samsung, ZTE	CR	Rel-18	38.321	18.6.0	2119	1	A	NR_newRAT-Core

The CR is agreed

#### 5.1.2.2	RLC PDCP SDAP BAP

### 5.1.3	Control Plane corrections

#### 5.1.3.1	NR RRC

Corrections to 38331, and related change to other TS if applicable, e.g. 36331, Stage-2 etc.

SON/MDT

R2-2505288	Discussion on tracking area code information in SON reports	ZTE Corporation,  Sanechips	discussion	Rel-16	NR_SON_MDT-Core, NR_ENDC_SON_MDT_enh-Core, NR_ENDC_SON_MDT_enh2-Core

Proposal 1: Correct since Rel-16, that when cellId in RA report is set to CGI, the TAC is logged only when available and agrees on the TP included in Annex for P1.

Proposal 2a: Correct since Rel-17, that if the UE logs sourcePCellID in SHR, it shall be set to the global cell identity and tracking area code of the source PCell. i.e., remove the ‘if available’ from the procedure text when setting sourcePCellID in SHR.

Proposal 2b: Correct since Rel-18, that if the UE logs pCellId in SRP, it shall be set to the global cell identity and tracking area code of the PCell, i.e., remove the ‘if available’ from the procedure text when setting pCellId in SPR.

Proposal 3: Correct since Rel-18, that when sourcePSCellId and targetPSCellId in SPR report are set to CGI, the TAC is logged only when available.

Proposal 4: If P1-P3 are agreed, RAN2 discusses and agrees on the TPs provided in Annex.

-	Huawei agrees with the intention of P1, but think current spec is already clear. Nokia think the proposal does not impact the network, only the UE. Nokia thinks that the spec is perhaps a bit ambiguous but not wrong. Nokia is neutral since it doesn’t impact the network. Samsung think the intention is correct but the current spec is also fine as-is. Samsung has not seen any issues. ZTE think the spec is unclear and there might be a risk for misunderstanding, but no errors hve been spotted in the field. Xiaomi does not think current spec is not wrong. Qualcomm think the risk if misunderstanding is not high, and if we keep on doing this exercise (i.e. polishing the specs) we will keep on forever, Qualcomm therefore would not like to do this change.

Noted

R2-2506074	Correction on previousPCellId in RLF report	Huawei, HiSilicon, CATT, CMCC	CR	Rel-16	38.331	16.20.0	5451	-	F	NR_SON_MDT-Core

R2-2506075	Correction on previousPCellId in RLF report	Huawei, HiSilicon, CATT, CMCC	CR	Rel-17	38.331	17.13.0	5452	-	A	NR_SON_MDT-Core

R2-2506076	Correction on previousPCellId in RLF report	Huawei, HiSilicon, CATT, CMCC	CR	Rel-18	38.331	18.6.0	5453	-	A	NR_SON_MDT-Core

-	Samsung think that for the first change we should not refer to LTE messages in the NR spec but instead talk about mobilityFromNR. Ericsson agrees with Samsung. Huawei is OK to improve the wording in this way. ZTE points out that the CR is all in portrait layout and this should be fixed.

[AT131][602][Maint] Correction on previousPCellId in RLF report (Huawei)

Scope:

    - Discuss offline (face-to-face preferred, if suitable)

    - Produce agreeable CRs

Intended outcome:

    - Agreed CR(s) in R2-2506355, R2-2506356, R2-2506357

Deadline:

    - Wednesday 17:00. The intention is to agree over email.

R2-2506355	Correction on previousPCellId in RLF report	Huawei, HiSilicon, CATT, CMCC	CR	Rel-16	38.331	16.20.0	5451	1	F	NR_SON_MDT-Core

R2-2506356	Correction on previousPCellId in RLF report	Huawei, HiSilicon, CATT, CMCC	CR	Rel-17	38.331	17.13.0	5452	1	A	NR_SON_MDT-Core

R2-2506357	Correction on previousPCellId in RLF report	Huawei, HiSilicon, CATT, CMCC	CR	Rel-18	38.331	18.6.0	5453	1	A	NR_SON_MDT-Core

All 3 are agreed

RRC procedure delay

R2-2505734	Clarification on RRC procedure delay for BWP switching	Samsung	CR	Rel-15	38.331	15.29.0	5432	-	F	NR_newRAT-Core

R2-2505735	Clarification on RRC procedure delay for BWP switching	Samsung	CR	Rel-16	38.331	16.20.0	5433	-	A	NR_newRAT-Core

R2-2505743	Clarification on RRC procedure delay for BWP switching	Samsung	CR	Rel-17	38.331	17.13.0	5434	-	A	NR_newRAT-Core

R2-2505745	Clarification on RRC procedure delay for BWP switching	Samsung	CR	Rel-18	38.331	18.6.0	5436	-	A	NR_newRAT-Core

-	Nokia are unsure if this is necessary, but its important to understand how companies really interpret this sentence. CATT doesn’t think this is necessary. Docomo think the CR is correct and think that we should not talk about these processing delays in RAN2 specs. Qualcomm sees misalignment between RAN2 and RAN4 specs, but the CR is not correct and instead we can just remove this sentence completely. Ericsson think the RAN4 requirement is clear from their specs and either can do as Samsung but also removing the sentence completely is OK. Xiaomi are OK to remove the sentence. Nokia are OK to remove. Huawei also want to remove.

-	Nokia prefers to change from R18. Samsung wants to remove all specs (from Rel-15) since otherwise it seems that there is different behaviour in different releases. Xiaomi prefers to do this only from Rel-18.

Update the CR to remove the sentence. We do this clarification from Rel-18.

[AT131][603][Maint] Clarification on RRC procedure delay for BWP switching (Samsung)

Scope:

    - Produce agreeable CR

Intended outcome:

    - Agreed CR(s) in R2-2506358

Deadline:

    - Wednesday 17:00. The intention is to agree over email.

R2-2506358	Clarification on RRC procedure delay for BWP switching	Samsung	CR	Rel-18	38.331	18.6.0	5436	1	F	NR_newRAT-Core

Agreed, but then coversheet revised by MCC: TEI18 needs to be added as a WI code.

R2-2506636	Clarification on RRC procedure delay for BWP switching	Samsung	CR	Rel-18	38.331	18.6.0	5436	2	F	NR_newRAT-Core, TEI18

=> Agreed

MIMO

R2-2506120	Clarification of releasing elements from Rel-16 PUCCH Spatial Relation Info List	ZTE Corporation	CR	Rel-16	38.331	16.20.0	5459	-	F	NR_eMIMO-Core

R2-2506121	Clarification of releasing elements from Rel-16 PUCCH Spatial Relation Info List	ZTE Corporation	CR	Rel-17	38.331	17.13.0	5460	-	A	NR_eMIMO-Core

R2-2506122	Clarification of releasing elements from Rel-16 PUCCH Spatial Relation Info List	ZTE Corporation	CR	Rel-18	38.331	18.6.0	5461	-	A	NR_eMIMO-Core

R2-2506124	Summary Report of [Post130][602][Maint] Spatial Relation Info list extension (ZTE)_Summary	ZTE Corporation	discussion	Rel-16	NR_eMIMO-Core

-	The email discussion rapporteur (ZTE) requested to discuss this offline during the week.

[AT131][601][Maint] Spatial Relation Info list extension (ZTE)

Scope:

    - Discuss offline (face-to-face preferred, if suitable)

    - Produce agreeable CRs if needed

Intended outcome:

    - Agreed CR(s)

Deadline:

    - Wednesday 17:00. The intention is to agree over email if possible, otherwise we come back in the Thursday comeback session.

R2-2506453	Summary Report of [AT131][601][Maint] Spatial Relation Info list extension (ZTE)

Proposal 1 (easy agreement): RAN2 clarifies that “If the network includes spatialRelationInfoToAddModListExt in a RRCReconfiguration message, it shall include the same number of entries, and listed in the same order, as in the concatenation of included spatialRelationInfoToAddModList and included spatialRelationInfoToAddModListSizeExt in the same message”. No spec change is needed.

Proposal 2(easy agreement): In the case where there are 8 entries have already been created by the patialRelationInfoToAddModList (i.e.Rel-15 ToAddModlist), it is allowed for NW to use the spatialRelationInfoToAddModList alone for adding more elements if the available Id of the added entry is no more than 8, or together with spatialRelationInfoToAddModListExt if the available Id of the added entry is greater than 8.  No spec change is needed.

Proposal 3(Easy Agreement): RAN2 confirms that an entry created using spatialRelationInfoToAddModList can be modified using spatialRelationInfoToAddModListSizeExt, and vise versa. No spec change is needed.

Proposal 5(Easy Agreement): RAN2 clarifies that the PUCCH-SpatialRelationInfoId-r16 is considered as the same as pucch-SpatialRelationInfoId or pucch-SpatialRelationInfoId-v1610 when UE receiving the spatialRelationInfoToReleaseListExt-v1610. No spec change is needed.

Agreements - RAN2 confirms that if the network includes spatialRelationInfoToAddModListExt in a RRCReconfiguration message, it shall include the same number of entries, and listed in the same order, as in the concatenation of included spatialRelationInfoToAddModList and included spatialRelationInfoToAddModListSizeExt in the same message. No spec change is needed. - RAN2 confirms that in case where 8 entries have already been added by the spatialRelationInfoToAddModList (i.e. the Rel-15 ToAddModlist), it is allowed for NW to use the spatialRelationInfoToAddModList alone for adding more elements if the Id of the added entry is not greater than 8, or together with spatialRelationInfoToAddModListExt if the Id of the added entry is greater than 8. No spec change is needed. - RAN2 confirms that an entry created using spatialRelationInfoToAddModList can be modified using spatialRelationInfoToAddModListSizeExt, and vice versa. No spec change is needed. - RAN2 confirms that the PUCCH-SpatialRelationInfoId-r16 is considered the same as pucch-SpatialRelationInfoId and pucch-SpatialRelationInfoId-v1610 when the UE receives the spatialRelationInfoToReleaseListExt-v1610. No spec change is needed.

ZTE Corporation	discussion	Rel-16	NR_eMIMO-Core

#### 5.1.3.2	UE capabilities

UE cap corrections 38306, 38331

SCellWithoutSSB

R2-2505465	Correction on intraF-NeighMeasForSCellWithoutSSB capability	ZTE Corporation, Sanechips, Nokia, Huawei, HiSilicon	CR	Rel-15	38.306	15.28.0	1327	-	F	NR_newRAT-Core

R2-2505466	Correction on intraF-NeighMeasForSCellWithoutSSB capability	ZTE Corporation, Sanechips, Nokia, Huawei, HiSilicon	CR	Rel-16	38.306	16.21.0	1328	-	A	NR_newRAT-Core

R2-2505467	Correction on intraF-NeighMeasForSCellWithoutSSB capability	ZTE Corporation, Sanechips, Nokia, Huawei, HiSilicon	CR	Rel-17	38.306	17.13.0	1329	-	A	NR_newRAT-Core

-	ZTE clarifies that a separate capability might be needed for FR2/FR1. Lenovo think that since this is a per UE capa it should say “No” rather than “N/A”. ZTE did the same for SCell without SSB-capa. Nokia thinks it should be “No”.

-	ZTE thinks that Rel-18 can be discussed later if needed.

-	After further offline, ZTE indicates that it might be needed to create a Rel-18 CR together and hence want to discuss this further offline during this meeting.

[AT131][604][Maint] Correction on intraF-NeighMeasForSCellWithoutSSB (ZTE)

Scope:

    - Discuss offline (face-to-face preferred, if suitable)

    - Produce agreeable CRs

Intended outcome:

    - Agreed CR(s) in R2-2506359, R2-2506360, R2-2506361, R2-2506362

Deadline:

    - Wednesday 17:00. The intention is to agree over email.

R2-2506359	Correction on intraF-NeighMeasForSCellWithoutSSB capability	ZTE Corporation, Sanechips , Nokia, Huawei , HiSilicon	CR	Rel-15	38.306	15.28.0	1327	1	F	NR_newRAT-Core

R2-2506360	Correction on intraF-NeighMeasForSCellWithoutSSB capability	ZTE Corporation, Sanechips , Nokia, Huawei , HiSilicon	CR	Rel-16	38.306	16.21.0	1328	1	A	NR_newRAT-Core

R2-2506361	Correction on intraF-NeighMeasForSCellWithoutSSB capability	ZTE Corporation, Sanechips , Nokia, Huawei , HiSilicon	CR	Rel-17	38.306	17.13.0	1329	1	A	NR_newRAT-Core

R2-2506362	Correction on intraF-NeighMeasForSCellWithoutSSB capability	ZTE Corporation, Sanechips , Nokia, Huawei , HiSilicon	CR	Rel-18	38.306	18.6.0	1345	-	A	NR_newRAT-Core, Netw_Energy_NR-Core

=> 	All 4 CRs are agreed, but then CR#1345 is coversheet revised by MCC in R2-2506649: Cat A CR has to use the same WI as the corresponding cat F CR.

R2-2506649	Correction on intraF-NeighMeasForSCellWithoutSSB capability	ZTE Corporation, Sanechips , Nokia, Huawei , HiSilicon	CR	Rel-18	38.306	18.6.0	1345	1	A	NR_newRAT-Core

All 4 CRs are agreed

Mandatory gaps

R2-2506138	Discussion on R16 mandatory gap capability	ZTE Corporation, Sanechips	discussion	Rel-16	NR_RRM_enh-Core

R2-2505112	Discussion on R4 LS on Mandatory Gap Capability	OPPO	discussion	Rel-16	NR_RRM_enh-Core

-	Qualcomm thinks that unless there is a legacy UE issue, we think we shouldn’t update the specs as RAN4 requested. Ericsson understands but think that RAN4 already discussed this so it makes sense that RAN2 follows the RAN4 outcome. Xiaomi agrees with ZTE and Qualcomm. Samsung think there could be a UE that has issues. ZTE clarifies that there have been problematic UEs in the field.

-	Nokia think we can capture something in the minutes. Ericsson think we need to clarify something at least. Huawei are OK to capture something in the minutes. Ericsson think we already have minutes from RAN4 and think that we should do a spec change now. Xiaomi think we can capture in the minutes that the UE shall set R15 and R16 capas consistently. Ericsson think we cannot capture mandatory behaviours in minutes. Xiaomi think we might need to change more places if we do this change. Nokia agrees with Xiaomi but think we should just have a suggestion in the minutes. Samsung are OK with capturing in the minutes.

RAN2 understands that the UE should set the supportedGapPattern and supportedGapPattern-NRonly-r16 consistently in the sense that the UE does not indicate support for a gap pattern in the Rel-15 field but indicates non-support in the Rel-16 field.

R2-2505809	Correction to supportedGapPattern-NRonly-r16	Ericsson	CR	Rel-16	38.306	16.21.0	1335	-	F	NR_RRM_enh-Core

R2-2505810	Correction to supportedGapPattern-NRonly-r16	Ericsson	CR	Rel-17	38.306	17.13.0	1336	-	A	NR_RRM_enh-Core

R2-2505811	Correction to supportedGapPattern-NRonly-r16	Ericsson	CR	Rel-18	38.306	18.6.0	1337	-	A	NR_RRM_enh-Core

R2-2506145	Correction to supported gap patterns	Huawei, HiSilicon	CR	Rel-16	38.306	16.21.0	1342	-	F	NR_RRM_enh-Core

R2-2506146	Correction to supported gap patterns	Huawei, HiSilicon	CR	Rel-17	38.306	17.13.0	1343	-	A	NR_RRM_enh-Core

R2-2506147	Correction to supported gap patterns	Huawei, HiSilicon	CR	Rel-18	38.306	18.6.0	1344	-	A	NR_RRM_enh-Core

The 6 above are not pursued

#### 5.1.3.3	Other

This agenda item addresses the idle and inactive behaviour specified in 38.304 or 36.304, LTE-specific changes for the applicable WIs, Other parts not covered elsewhere.

## 5.3	NR Positioning Support

(NR_newRAT-Core; leading WG: RAN1; REL-15; started: Mar. 17; closed: Jun. 19: WID: RP-191971)

(NR_pos-Core; leading WG: RAN1; REL-16; started: Mar 19; target; Jun 20; WID: RP-200218).

(NR TEI16 Positioning)

Stage 2 corrections shall be discussed with the specification rapporteur (Sven Fischer sfischer@qti.qualcomm.com) before submission. Stage 2 CRs not discussed with the specification rapporteur will not be treated.

R2-2505324	Correction to delivery of posSIB segments by dedicated signalling in RRC_CONNECTED	Huawei, HiSilicon, Ericsson, Qualcomm	CR	Rel-16	38.331	16.20.0	5407	-	F	NR_pos-Core

Revised in R2-2506301

Discussion:

Lenovo think this behaviour would be mandatory for the UE.  Huawei confirm that it is intended as a correction.

Nokia think the problem description is not crystal-clear, and they are not sure it is critical; it may be just a matter of efficiency.  They also have some concern that the changed text is in sections that are general to SIBs and posSIBs and include the broadcast aspects, and they wonder if it would be an alternative to have multiple SI messages in the dedicated message.  Huawei note that there is no ASN.1 change, and they think it is a genuine correction because there are cases where the posSIB needs to be delivered quickly.

vivo ask if there really is a restriction in the spec today.  Chair understands that the SI message has the restriction and the OCTET STRING contains the message format.

Ericsson think the CR just clarifies what the network would be allowed to do and distinguishes dedicated from broadcast.

Nokia are bothered by the change of the mapping so that it would be different for broadcast and unicast.  They also are concerned by the change to the section about SIBs and posSIBs.  Ericsson think the latter change is not essential to have.  Qualcomm disagree with Ericsson in this respect and think we need to fix the requirement for only one segment of a posSIB in the SI message, but they agree that it would be nicer if we had a clear separation between the broadcast and unicast cases.

Nokia would prefer not to change anything and think the efficiency issue raised in the coversheet is not critical.

Huawei understand that section 5.2.1 is not specific to broadcast but covers any form of the SI message.

Nokia can accept the change in principle but need to take a further look at the wording.

ZTE think we could address vivo’s question in the offline as well, and they think the change could also be applied to non-positioning SIBs.  Chair thinks we should not change the other SIBs in this session; CATT agree and think the use case for non-positioning SIBs would need to be discussed.

Huawei understand the difference here is that the posSIBs are generated by the LMF, not the gNB, so for non-positioning SIBs the gNB can do something in implementation.

[AT131][401][POS] CRs on posSIB segments in dedicated signalling (Huawei)

Scope: Check and update the CRs in R2-2505324 / R2-2505325 / R2-2505640, including confirming if there is a restriction in the spec today.  Intention is to allow multiple segments of a posSIB when sent by dedicated signalling, without changing the broadcast behaviour or the non-positioning SIBs.

Intended outcome: Agreeable CRs in R2-2506301 / R2-2506302 / R2-2506303 if needed, and report in R2-2506317

Deadline: Wednesday 2025-08-27 1900 IST

R2-2505325	Correction to delivery of posSIB segments by dedicated signalling in RRC_CONNECTED	Huawei, HiSilicon, Ericsson, Qualcomm	CR	Rel-17	38.331	17.13.0	5408	-	A	NR_pos-Core

Revised in R2-2506302

R2-2505326	Correction on delivery of posSIB segments by dedicated siganling in RRC_CONNECTED	Huawei, HiSilicon, Ericsson, Qualcomm	CR	Rel-18	38.331	18.6.0	5409	-	A	NR_pos-Core

Revised in R2-2505640

R2-2505640	Correction to delivery of posSIB segments by dedicated signalling in RRC_CONNECTED	Huawei, HiSilicon, Ericsson, Qualcomm	CR	Rel-18	38.331	18.6.0	5409	1	A	NR_pos-Core	R2-2505326

Revised in R2-2506303

R2-2506317	Summary for [AT131][401][POS] CRs on PosSIB segments in dedicated signalling (Huawei)	Huawei	discussion	Rel-16	NR_pos-Core

Noted

Proposal1: Provide following overall clarification in the section of 5.2.1 to highlight the difference in handling of posSIB segments between broadcast and dedicated delivery of posSIB. Remove the previous change in 5.2.1.

-	For a UE in RRC_CONNECTED, the network may provide multiple segments of a posSIB of the same posSIB type through dedicated signalling using the RRCReconfiguration message.

Proposal2: Adopt the following change for the field description for dedicatedPosSysInfoDelivery

dedicatedPosSysInfoDelivery

This field is used to transfer SIBPos to the UE in RRC_CONNECTED. This field may contain multiple posSIB segments of the same posSIB type. When the number of segments of posSIB of the same posSIB type exceeds the maximum number of posSIBs that this field could carry (i.e., 32), the posSIB segments of the same posSIB type could be delivered in multiple RRCReconfiguration messages.

R2-2506301	Correction to delivery of posSIB segments by dedicated signalling in RRC_CONNECTED	Huawei, HiSilicon, Ericsson, Qualcomm	CR	Rel-16	38.331	16.20.0	5407	1	F	NR_pos-Core

Not pursued

Discussion:

ZTE think this is gNB implementation behaviour and the spec already allows it.  They would be OK to capture guidance in the minutes.

Huawei think this is a question of how you interpret the signalling and what is in the container, but it is not completely clear from the spec.  Chair thinks it is unambiguous that the container does not contain a SystemInformation message.  CATT agree with the chair.

CATT think the first change in the CR is not essential because it is related to dedicated signalling rather than the SI message as such.  Huawei clarify that the intention is to isolate the change to posSIBs, and they think that the relevant section is about system information, not just the SI message.

Nokia think it might be OK to capture guidance in the minutes, but if something is captured in the spec it should be clear and thorough.

Huawei think implementation teams will read the spec, and it may not be clear there.  ZTE think it is business as usual that we capture implementation guidance in the notes when something is not broken.

ZTE understand that the previous discussion resulted in a lot of companies feeling the change was an optimization.  Huawei clarify that the earlier version of the CR had normative impact and this version is for clarification.

Nokia think we could add a NOTE.

Ericsson think it is clear that the ASN.1 already allows this behaviour, but it was not clear that the UE would be able to handle it.

Huawei and Ericsson would be OK with a NOTE.  ZTE think we do not need anything and it should be clear that anything not forbidden for the gNB is allowed.

Nokia think it is not uncommon to have network implementation guidance in the spec.

CATT think the bar for Rel-16 should be higher than this.

Huawei note that the RRC is more than just ASN.1, and how the network behaves may not be clear today, so they see the change as critical.  Ericsson agree with Huawei and note that there was discussion of whether a UE capability would be needed.

ZTE think we should not open the door for clarifications like this going back to Rel-16.

Agreement:

RAN2 confirm that for a UE in RRC_CONNECTED, the network may provide multiple segments of a posSIB of the same posSIB type through dedicated signalling using the RRCReconfiguration message.

R2-2506302	Correction to delivery of posSIB segments by dedicated signalling in RRC_CONNECTED	Huawei, HiSilicon, Ericsson, Qualcomm	CR	Rel-17	38.331	17.13.0	5408	1	A	NR_pos-Core

Not pursued

R2-2506303	Correction to delivery of posSIB segments by dedicated signalling in RRC_CONNECTED	Huawei, HiSilicon, Ericsson, Qualcomm	CR	Rel-18	38.331	18.6.0	5409	2	A	NR_pos-Core	R2-2505640

Revised in R2-2506458

[Post131][418][POS] Rel-18 CR on multiple posSIB segments in dedicated signalling (Huawei)

Scope: Update the CR in R2-2503603 to a category F CR reflecting the guidance in the chair notes from RAN2#131.

Intended outcome: Agreed CR in R2-2506458

Deadline: Short (for RP)

=> Agreed in R2-2506579 (38.331)

R2-2506579	Correction to delivery of posSIB segments by dedicated signalling in RRC_CONNECTED	Huawei, HiSilicon, Ericsson, Qualcomm	CR	Rel-18	38.331	18.6.0	5409	4	A	NR_pos-Core, TEI18

=> Agreed

# 6	NR Rel-17

Essential corrections only.  Editorial/clarifications should be sent to be reviewed and approved by spec rapporteurs prior to submission.  Editorials should only be submitted by spec rapporteurs.

Tdoc Limitation: 4 Tdocs in total for agenda item 5 (incl. its sub agenda items) and agenda item 6 (incl. its sub agenda items)

## 6.1	Common

(NR_MG_enh-Core; leading WG: RAN4; REL-17; WID: RP-211591)

(NR_UDC_enh-Core; leading WG: RAN2; REL-17; WID: RP-211203)

(NG_RAN_PRN_enh-Core; leading WG: RAN3; REL-17; WID: RP-202363)

(NR_IAB_enh-Core; leading WG: RAN2; REL-17; WID: RP-211548)

(NR_UE_pow_sav_enh-Core; leading WG: RAN2; REL-17; WID: RP-212630)

(LTE_NR_DC_enh2-Core; leading WG: RAN2; REL-17; WID: RP-201040)

(LTE_NR_MUSIM-Core; leading WG: RAN2; REL-17; WID: RP-212610)

(NR_Slice-Core; leading WG: RAN2; REL-17; WID: RP-212534)

(NR_QoE-Core; leading WG: RAN3; REL-17; WID: RP-211406)

(NR_ext_to_71GHz-Core; leading WG: RAN1; REL-17; WID: RP-212637)

(NR_cov_enh-Core; leading WG: RAN1; REL-17; WID: RP-211566): non-RACH-indication parts

(NR_redcap-Core; leading WG: RAN1; REL-17; WID: RP-211574)

(NR_feMIMO-Core; leading WG: RAN1; REL-17; WID: RP-212535)

(NR_SmallData_INACTIVE-Core, leading WG: RAN2; REL-17; WID: RP-212594)

(NR_IIOT_URLLC_enh-Core; leading WG: RAN2; REL-17; WID: RP-210854)

(NR_MBS-Core; leading WG: RAN2; REL-17; WID: RP-201038)

(NR_ENDC_SON_MDT_enh-Core; leading WG: RAN3; REL-17; WID: RP-201281)

(NR_NTN_solutions-Core; leading WG: RAN2; REL-17; WID: RP-211557)

(NR_SL_enh-Core; leading WG: RAN1; REL-17; WID: RP-202846)

(NR_SL_Relay-Core; leading WG: RAN2; REL-17; WID: RP-212601)

PRACH partitioning items

(NR TEI17)

Includes Rel-17 Work Items without specific R2 Agenda Item, e.g. RAN1 and RAN4 led items, SA2 and CT1 led items (was previously “Rel-17 Other”)

Includes aspects that does not fit under the more specific AIs, e.g. multi-WI aspects.

Corrections for NR_NTN_solutions-Core might be treated in the NTN breakout session.

### 6.1.1	Stage 2 and Organisational

Incoming LSs, etc. You should discuss your stage 2 CRs with the specification rapporteurs before submission. Includes impact to 38.300, 37.340, (36.300 if applicable)

R2-2505037	Reply LS on stage 1 requirements to support PWS over satellite NG-RAN in Rel-17 (R3-253867; contact: Ericsson)	RAN3	LS in	Rel-17	NR_NTN_solutions-Core, LTE_NBIOT_eMTC_NTN-Core	To:SA1, CT1, RAN2	Cc:SA2, CT4, RAN, SA

Noted

R2-2505053	Reply LS from RAN on removal of support of PWS over satellite NG-RAN in Rel-17 and 18 (RP-251859; contact: Aalyria)	RAN	LS in	Rel-17	NR_NTN_solutions-Core, LTE_NBIOT_eMTC_NTN-Core	To:CT1, SA1, CT, SA, RAN3, RAN2	Cc:SA2, CT4

Noted

R2-2505826	Support for PWS in NTN	Ericsson	discussion	Rel-17	NR_NTN_solutions-Core

Proposal 1	RAN2 should wait for CT1 decision before taking any further action.

RAN2 waits for CT1 decision before taking any further action on PWS support over NR NTN in Rel-17 an Rel-18

Paging capabilities

R2-2505062	Reply LS on Handling of UE Radio Capability for Paging (S2-2506082; contact: Ericsson)	SA2	LS in	Rel-17	NR_newRAT-Core, 5GS_Ph1, TEI17	To:RAN3	Cc:RAN2

Noted

Emergency

R2-2505060	Reply LS on emergency call back and paging (S2-2505938; contact: Qualcomm)	SA2	LS in	Rel-17	NR_newRAT-Core, NR_redcap-Core	To:RAN3	Cc:RAN2, CT1, RAN

-	Qualcomm suggest noting the LS without action

Noted

R2-2505853	Clarification on eDRX and emergency PDU session	Ericsson	CR	Rel-18	38.300	18.6.0	1017	-	F	NR_redcap-Core	Withdrawn

R2-2505854	Clarification on eDRX and emergency PDU session	Ericsson	CR	Rel-18	38.300	18.6.0	1018	-	A	NR_redcap-Core

R2-2505865	Clarification on eDRX and emergency PDU session	Ericsson	CR	Rel-17	38.300	17.13.0	1019	-	F	NR_redcap-Core

-	ZTE think there are similar CRs in RAN3 and they discuss there, there is an offline there today.

-	ZTE understands the motivation but think the CR is not needed, since current spec only (need to) say that the UE shall not be given eDRX when sent to INACTIVE, it doesn’t say the NW cannot keep the UE in CONNECTED. LG agrees with ZTE. Docomo supports the intention and are OK to clarify this in the spec, but capturing this in the IMS emergency call section would be a more suitable place. CATT and Nokia think current spec is fine. Qualcomm think we should leave the spec as is.

Not pursued (RAN3 has made changes in this area which renders no further RAN2 discussion on this topic needed).

### 6.1.2	User Plane corrections

User Plane Related aspects will be handled in the User Plane break out session. (exception: TEI new proposals if any).

R2-2505626	Correction on DCP considering MUSIM gaps	Huawei, HiSilicon	CR	Rel-17	38.321	17.13.0	2108	-	F	LTE_NR_MUSIM-Core, NR_UE_pow_sav-Core

-	Vivo, Apple, Qualcomm, supports but need to update stage 2 specs.

-	Apple points that we discussed something similar in LP-WUS.

The CR is agreed

R2-2505627	Correction on DCP considering MUSIM gaps	Huawei, HiSilicon	CR	Rel-18	38.321	18.6.0	2109	-	A	LTE_NR_MUSIM-Core, NR_UE_pow_sav-Core

The CR is agreed

[AT131][014][R17 UP] DCP stage 2 CR (Huawei)

Intended outcome: Review stage 2 CR

Deadline:  Thursday

R2-2506209	Correction on DCP considering MUSIM gaps	Huawei, HiSilicon, Nokia (Rapporteur), vivo	CR	Rel-17	38.300	17.13.0	1025	-	F	LTE_NR_MUSIM-Core, NR_UE_pow_sav-Core

Update checkbox

The CR is agreed in R2-2506479 with change above

R2-2506479	Correction on DCP considering MUSIM gaps	Huawei, HiSilicon, Nokia (Rapporteur), vivo	CR	Rel-17	38.300	17.13.0	1025	1	F	LTE_NR_MUSIM-Core, NR_UE_pow_sav-Core

=> Agreed

R2-2506210	Correction on DCP considering MUSIM gaps	Huawei, HiSilicon, Nokia (Rapporteur), vivo	CR	Rel-18	38.300	18.6.0	1026	-	A	LTE_NR_MUSIM-Core, NR_UE_pow_sav-Core

Update checkbox

The CR is agreed in R2-2506480 with change above

R2-2506480	Correction on DCP considering MUSIM gaps	Huawei, HiSilicon, Nokia (Rapporteur), vivo	CR	Rel-18	38.300	18.6.0	1026	1	A	LTE_NR_MUSIM-Core, NR_UE_pow_sav-Core

=> Agreed

R2-2506058	Correction on DRX Command handling	Samsung	CR	Rel-17	38.321	17.13.0	2113	-	F	NR_MBS-Core

-	Nokia doesn’t think this is needed.  Vivo, Qualcomm and Lenovo doesn’t see what is changed, the behaviour is the same.   Qualcomm agrees with intent but doesn’t see the need for change

-	Xiaomi is supportive

Cover page will be updated to describe the issue

if a DRX Command MAC CE indicated by PDCCH addressed to C-RNTI or CS-RNTI for unicast transmission

take latest CR template

[AT131][040][R17 UP] DRX command handling (Samsung)

Intended outcome: Agree to CR by email R2-2506478

Deadline:  Friday

R2-2506478	Correction on DRX Command handling	Samsung	CR	Rel-17	38.321	17.13.0	2113	1	F	NR_MBS-Core	R2-2506058

The CR is agreed

R2-2506071	Correction on DRX Command handling	Samsung	CR	Rel-18	38.321	18.6.0	2114	-	A	NR_MBS-Core

=> Revised in R2-2506481

R2-2506481	Correction on DRX Command handling	Samsung	CR	Rel-18	38.321	18.6.0	2114	1	A	NR_MBS-Core	R2-2506071

The CR is agreed

### 6.1.3	Control Plane corrections

#### 6.1.3.1	NR RRC

Corrections to 38331, and related change to other TS if applicable, except UE caps.

R2-2505841	Discussion on eventD1/D2 and condEventD1/D2/T1	Samsung, ASUSTek, CATT, Ericsson, Nokia, Huawei, Apple, Xiaomi, ZTE Corporation, vivo	discussion	Rel-17	NR_NTN_solutions, NR_NTN_enh-Core

Proposal 1: RAN2 clarifies for measId configured with only eventD1/D2 for neighbor cells, UE does not need to perform RRM measurement or report result for neighbor cells in the report of the measId.

RAN2 clarifies for measId configured with only eventD1/D2 for neighbor cells, UE does not need to perform RRM measurement or report result for neighbor cells in the report of the measId.

Proposal 2: For CHO, RAN2 clarifies UE does not need to perform RRM measurements for a measId configured with condEventD1/D2/T1 when evaluating condEventD1/D2/T1. This does not preclude the UE from detecting based on the associated measObject the applicable cell which has a physical cell identity matching the value in reconfigurationWithSync when evaluating condEventD1/D2/T1 for CHO with only condEventD1/D2/T1.

For CHO, RAN2 clarifies UE does not need to perform RRM measurements for a measId configured with condEventD1/D2/T1 when evaluating condEventD1/D2/T1. This does not preclude the UE from detecting based on the associated measObject the applicable cell which has a physical cell identity matching the value in reconfigurationWithSync when evaluating condEventD1/D2/T1 for CHO with only condEventD1/D2/T1.

Proposal 3: FFS the specification impact of Proposal 1 and 2 (e.g., no change, a NOTE for clarification, or change on procedural text).

R2-2505842	Corrections on eventD1 and condEventD1/T1	Samsung	CR	Rel-17	38.331	17.13.0	5365	1	F	NR_NTN_solutions	R2-2504204

- ZTE thinks the actual change should be about the reporting part, not the measurement

- Ericsson prefers to have a change to the procedural text rather than a Note. Nokia agrees, at least for the first note

- Huawei thinks the spec is already sufficiently clear

We work on a change to the procedural text

Revised in R2-2506272

R2-2506272	Corrections on eventD1 and condEventD1/T1	Samsung	CR	Rel-17	38.331	17.13.0	5365	2	F	NR_NTN_solutions	R2-2504204

Revised in R2-2506286

R2-2506286	Corrections on eventD1 and condEventD1/T1	Samsung	CR	Rel-17	38.331	17.13.0	5365	3	F	NR_NTN_solutions	R2-2504204

Change condEventD2 into condEventT1

Revised in R2-2506289

R2-2506289	Corrections on eventD1 and condEventD1/T1	Samsung, Huawei, HiSilicon, Ericsson, Nokia, Xiaomi	CR	Rel-17	38.331	17.13.0	5365	4	F	NR_NTN_solutions	R2-2504204

Agreed, but then coversheet revised by MCC: duplicated meeting number

R2-2506631	Corrections on eventD1 and condEventD1/T1	Samsung, Huawei, HiSilicon, Ericsson, Nokia, Xiaomi	CR	Rel-17	38.331	17.13.0	5365	5	F	NR_NTN_solutions

=> Agreed

R2-2505843	Corrections on eventD1/D2 and condEventD1/D2/T1	Samsung	CR	Rel-18	38.331	18.6.0	5366	1	F	NR_NTN_solutions, NR_NTN_enh-Core	R2-2504205

- Xiaomi thinks the change about CHO should refer to the evaluation part

Revised in R2-2506273

R2-2506273	Corrections on eventD1/D2 and condEventD1/D2/T1	Samsung	CR	Rel-18	38.331	18.6.0	5366	2	F	NR_NTN_solutions, NR_NTN_enh-Core	R2-2504205

Revised in R2-2506287

R2-2506287	Corrections on eventD1/D2 and condEventD1/D2/T1	Samsung	CR	Rel-18	38.331	18.6.0	5366	3	F	NR_NTN_solutions, NR_NTN_enh-Core	R2-2504205

Revised in R2-2506294

R2-2506294	Corrections on eventD1/D2 and condEventD1/D2/T1	Samsung	CR	Rel-18	38.331	18.6.0	5366	4	F	NR_NTN_solutions, NR_NTN_enh-Core	R2-2504205

Agreed, but then coversheet revised by MCC: duplicated meeting number

R2-2506632	Corrections on eventD1/D2 and condEventD1/D2/T1	Samsung	CR	Rel-18	38.331	18.6.0	5366	5	F	NR_NTN_solutions, NR_NTN_enh-Core

=> Agreed

R2-2506155	Correction of PDD reporting related descriptions	Ericsson	discussion	Rel-17	NR_NTN_solutions-Core

Proposal 1	Confirm that the descriptions for PDD reporting in current specifications incorrectly refer to cells.

- QC thinks this is not critical and if we clarify this here we would also need to make some changes to the UE capability

- Nokia thinks that logically Ericsson is right but this is not crititcal and changes to the specs are not needed. Huawei agrees

Proposal 2	Agree the CRs for TS 38.331 and TS 38.300 provided in the annex of this contribution.

MIMO

R2-2505784	Correction on configured grant power control in unified TCI framework	Ofinno	CR	Rel-17	38.331	17.13.0	5439	-	F	NR_FeMIMO-Core

R2-2505785	Correction on configured grant power control in unified TCI framework	Ofinno	CR	Rel-18	38.331	18.6.0	5440	-	A	NR_FeMIMO-Core

-	MediaTek think the intention is OK but the way the CR is not implemented well, we should use wording “UE ignores” as this is more commonly used. Nokia agrees with MediaTek.

Change to “If unifiedTCI-StateType is configured, the UE does not use this field”. Agreed unseen in R2-2506363 and R2-2506364

SON/MDT

R2-2505830	Correction on SCGFailureInformation	Ericsson	CR	Rel-17	38.331	17.13.0	5444	-	F	NR_ENDC_SON_MDT_enh-Core

R2-2505831	Correction on SCGFailureInformation	Ericsson	CR	Rel-18	38.331	18.6.0	5445	-	A	NR_ENDC_SON_MDT_enh-Core

-	Xiaomi wants more time since the delegate handling this is in another session. Wants to push this offline.

[AT131][605][Maint] Correction on SCGFailureInformation (Ericsson)

Scope:

    - Discuss offline (face-to-face preferred, if suitable)

    - Produce agreeable CRs

Intended outcome:

    - Agreed CR(s) in R2-2506365, R2-2506366

Deadline:

    - Wednesday 17:00. The intention is to agree over email.

-	Ericsson reports after the offline that the clause referred in the CR (R2-2505830) was originally added to capture the scenario of SCG failure shortly after a successful execution of the reconfiguration with synch for SCG. Therefore, this change should not be pursued

R2-2505830, R2-2506365, R2-2506366 are not pursued

#### 6.1.3.2	UE capabilities

UE cap corrections 38306, 38331.

Measurement gap enhancements

R2-2505602	Clarifications on the applicability of independent gap UE capability	Qualcomm Incorporated	CR	Rel-17	38.306	17.13.0	1331	-	F	NR_MG_enh-Core

Revised in R2-2506369

R2-2505603	Clarifications on the applicability of independent gap UE capability	Qualcomm Incorporated	CR	Rel-18	38.306	18.6.0	1332	-	A	NR_MG_enh-Core

Revised in R2-2506499

-	ZTE supports this CR, but why not from Rel-15? Qualcomm think we could do from Rel-15. ZTE wants this from Rel-15 since the capa is a Rel-15 capa.

-	After further offline: Qualcomm reports that it is not possible to simply port back the change to Rel-15 and therefore suggests to have a post meeting email discussion to more carefully investigate and agree how to port the change to Rel-15.

[Post131][604][Maint] Clarifications on the applicability of independent gap UE capability (Qualcomm)

Scope:

    - Produce agreeable CRs

Intended outcome:

    - Agreed CR(s)

Deadline:

    - Short

=> Agreed in:

- R2506367: 38.306 (Rel-15)

- R2506368: 38.306 (Rel-16)

- R2506369: 38.306 (Rel-17)

- R2506499: 38.306 (Rel-18)

R2-2506367	Clarifications on the applicability of independent gap UE capability	Qualcomm Incorporated	CR	Rel-15	38.306	15.28.0	1346	-	F	NR_MG_enh-Core

R2-2506368	Clarifications on the applicability of independent gap UE capability	Qualcomm Incorporated	CR	Rel-15	38.306	16.21.0	1347	-	A	NR_MG_enh-Core

R2-2506369	Clarifications on the applicability of independent gap UE capability	Qualcomm Incorporated	CR	Rel-17	38.306	17.13.0	1331	1	A	NR_MG_enh-Core

R2-2506499	Clarifications on the applicability of independent gap UE capability	Qualcomm Incorporated	CR	Rel-18	38.306	18.6.0	1332	1	A	NR_MG_enh-Core

=> All 4 CRs are agreed, but then coversheet revised by MCC: Wrong Rel-17 WI code for an earlier release CR

R2-2506643	Clarifications on the applicability of independent gap UE capability	Qualcomm Incorporated	CR	Rel-15	38.306	15.28.0	1346	1	F	TEI15

R2-2506644	Clarifications on the applicability of independent gap UE capability	Qualcomm Incorporated	CR	Rel-15	38.306	16.21.0	1347	1	A	TEI15

R2-2506645	Clarifications on the applicability of independent gap UE capability	Qualcomm Incorporated	CR	Rel-17	38.306	17.13.0	1331	2	A	TEI15

R2-2506646	Clarifications on the applicability of independent gap UE capability	Qualcomm Incorporated	CR	Rel-18	38.306	18.6.0	1332	2	A	TEI15

=> All 4 CRs are agreed

R2-2505897	Correction on inter-RAT FR2 measurement capabilities	Nokia	CR	Rel-17	36.331	17.13.0	5147	-	F	NR_MG_enh-Core

R2-2505898	Correction on inter-RAT FR2 measurement capabilities	Nokia	CR	Rel-18	36.331	18.6.0	5148	-	A	NR_MG_enh-Core

R2-2505899	Correction on inter-RAT FR2 measurement capabilities	Nokia	CR	Rel-17	36.306	17.9.0	1921	-	F	NR_MG_enh-Core

R2-2505900	Correction on inter-RAT FR2 measurement capabilities	Nokia	CR	Rel-18	36.306	18.5.0	1922	-	A	NR_MG_enh-Core

-	Qualcomm supports these CRs. ZTE agrees with the intention but think LTE stand-alone is also impacted. Xiaomi agrees.

-	Huawei wants to check these CRs a bit more offline.

[AT131][606][Maint] Correction on inter-RAT FR2 measurement capabilities (Nokia)

Scope:

    - Discuss offline (face-to-face preferred, if suitable)

    - Produce agreeable CRs

Intended outcome:

    - Agreed CR(s) in R2-2506370, R2-2506371, R2-2506372, R2-2506373

Deadline:

    - Wednesday 17:00. The intention is to agree over email.

R2-2506370	Correction on inter-RAT FR2 measurement capabilities	Nokia	CR	Rel-17	36.331	17.13.0	5147	1	F	NR_MG_enh-Core

R2-2506371	Correction on inter-RAT FR2 measurement capabilities	Nokia	CR	Rel-18	36.331	18.6.0	5148	1	A	NR_MG_enh-Core

R2-2506372	Correction on inter-RAT FR2 measurement capabilities	Nokia	CR	Rel-17	36.306	17.9.0	1921	1	F	NR_MG_enh-Core

R2-2506373	Correction on inter-RAT FR2 measurement capabilities	Nokia	CR	Rel-18	36.306	18.5.0	1922	1	A	NR_MG_enh-Core

All 4 are agreed (but then coversheet revised by MCC in R2-2506620 – R2-2506623: wrong tdoc and meeting numbers)

R2-2506620	Correction on inter-RAT FR2 measurement capabilities	Nokia, Qualcomm Incorporated	CR	Rel-17	36.331	17.13.0	5147	2	F	NR_MG_enh-Core

R2-2506621	Correction on inter-RAT FR2 measurement capabilities	Nokia, Qualcomm Incorporated	CR	Rel-18	36.331	18.6.0	5148	2	A	NR_MG_enh-Core

R2-2506622	Correction on inter-RAT FR2 measurement capabilities	Nokia, Qualcomm Incorporated	CR	Rel-17	36.306	17.9.0	1921	2	F	NR_MG_enh-Core

R2-2506623	Correction on inter-RAT FR2 measurement capabilities	Nokia, Qualcomm Incorporated	CR	Rel-18	36.306	18.5.0	1922	2	A	NR_MG_enh-Core

=> All 4 are agreed

#### 6.1.3.3	Other

Including idle and inactive behaviour specified in 38.304 or 36.304.

## 6.3	NR positioning enhancements

(NR_pos_enh-Core; leading WG: RAN1; REL-17; WID: RP-210903)

# 7	Rel-18

## 7.0	Common

Rel-18 WIs not covered under an explicit AI in 7.x.  Multi-WI Rel-18 items, e.g. cross-WI-issues not handled under another WI. UE capabilities.

### 7.0.1	UE Capabilities

Multi-WI handling of Rel-18 feature lists and UE capability Mega CRs.

R2-2505308	Corrections on Rel-18 UE capability descriptions, including [HARQ-ACK MUX on PUSCH]	Xiaomi	CR	Rel-18	38.306	18.6.0	1322	-	F	NR_MIMO_evo_DL_UL, NR_Mob_enh2, Netw_Energy_NR, NR_XR_enh, NR_NTN_enh, NR_SL_enh2, TEI18

[AT131][022][R18 UE caps] CR  (Xiaomi)

Intended outcome: review and agree by email

Deadline:  Thursday

R2-2506469	Corrections on Rel-18 UE capability descriptions, including [HARQ-ACK MUX on PUSCH]	Xiaomi	CR	Rel-18	38.306	18.6.0	1322	1	F	NR_MIMO_evo_DL_UL, NR_Mob_enh2, Netw_Energy_NR, NR_XR_enh, NR_NTN_enh, NR_SL_enh2, TEI18

The CR is agreed

### 7.0.2	Rel-18 corrections

Essential corrections only. For smaller corrections please contact CR editor / Rapporteur directly.  Coordinate with rapporteurs and chair if input above limit is required

Tdoc limitation: 5

#### 7.0.2.1	RACH-less HO

Corrections to generalized RACH-less HO procedure, including NTN, mIAB, and overlapping sections of the LTM cell switch procedure

#### 7.0.2.2	NR network-controlled repeaters

(NR_NetConRepeater; leading WG: RAN1; REL-18; WID: RP-230175)

#### 7.0.2.3	NR support for UAV

(NR_UAV-Core; leading WG: RAN2; REL-18; WID: RP-230782 and LTE WID: RP-230783 )

#### 7.0.2.4 	Mobile Terminated Small Data Transmission

(NR_MT_SDT-Core; leading WG: RAN2; REL-18; WID: RP-222993)

#### 7.0.2.5	IDC enhancements for NR and MR-DC

(NR_IDC_enh-Core; leading WG: RAN2; REL-18; WID: RP-221281)

#### 7.0.2.6	Mobile IAB (Integrated Access and Backhaul) for NR

( NR_mobile_IAB -Core; leading WG: RAN3; REL-18; WID: RP-232669)

#### 7.0.2.7	Timing Resiliency and URLLC Enh

(NR_TRS_URLLC; leading WG: RAN3; REL-18; WID: RP-230754)

#### 7.0.2.8	Enhanced support of reduced capability NR devices

(NR_redcap_enh-Core; leading WG: RAN1; REL-18; WID: RP-232671)

#### 7.0.2.9	Further NR coverage enhancements

(NR_cov_enh2-Core; leading WG: RAN1; REL-18; WID: RP-221858)

#### 7.0.2.10	Network energy savings for NR

(Netw_Energy_NR-Core; leading WG: RAN1; REL-18; WID: RP-223540)

R2-2505468	Presence condition of ssbFrequency in servingCellMO for SSB less Scell	ZTE Corporation, Sanechips, Huawei , HiSilicon, Nokia	discussion	Rel-18	Netw_Energy_NR-Core

Remove the Conditional Presence “SSBorAssociatedSSB2” of ssbFrequency in servingCellMO for SSB-less SCell and change back to “SSBorAssociatedSSB”.

Noted

[AT131][015][R18 NES] CRs (ZTE)

Intended outcome: Agree to CRs by email

Deadline:  Thursday

R2-2506442	Correction on Presence condition of ssbFrequency in servingCellMO for SSB-less SCell	ZTE Corporation, Sanechips, Huawei, HiSilicon, Nokia	CR	Rel-15	38.331	15.29.0	5470	-	F	NR_newRAT-Core

The CR is agreed

R2-2506443	Correction on Presence condition of ssbFrequency in servingCellMO for SSB-less SCell	ZTE Corporation, Sanechips, Huawei, HiSilicon, Nokia	CR	Rel-16	38.331	16.20.0	5471	-	A	NR_newRAT-Core

The CR is agreed

R2-2506444	Correction on Presence condition of ssbFrequency in servingCellMO for SSB-less SCell	ZTE Corporation, Sanechips, Huawei, HiSilicon, Nokia	CR	Rel-17	38.331	17.13.0	5472	-	A	NR_newRAT-Core

The CR is agreed

R2-2506445	Correction on Presence condition of ssbFrequency in servingCellMO for SSB-less SCell	ZTE Corporation, Sanechips, Huawei, HiSilicon, Nokia	CR	Rel-18	38.331	18.6.0	5473	-	A	NR_newRAT-Core

The CR is agreed

R2-2505812	Correction to scellWithoutSSB	Ericsson	CR	Rel-18	38.306	18.6.0	1338	-	F	Netw_Energy_NR-Core

The CR is agreed

#### 7.0.2.11	Further enhancement of data collection for SON MDT in NR and EN-DC

(NR_ENDC_SON_MDT_enh2-Core; leading WG: RAN3; REL-18; WID: RP-221825)

noSuitableCell

R2-2506055	Correction on reporting noSuitableCellFound	Samsung	CR	Rel-18	38.331	18.6.0	5448	-	F	NR_ENDC_SON_MDT_enh2-Core

-	CATT think there is no need to change this. ZTE also thinks that noSuitableCellFound is more about expiry at reestablishment and not fast recovery. Ericsson thinks this is not a correction but more a change of functionality by extending the functionality to a new scenario.

Not pursued

successPSCell-Config

R2-2506077	Correction on procedual text for successPSCell-Config	Huawei, HiSilicon, CATT, CMCC, Samsung, Ericsson	CR	Rel-18	38.331	18.6.0	5454	-	F	NR_ENDC_SON_MDT_enh2-Core

Agreed

RACH partition

R2-2506106	Discussion on parameters for RACH partition in RA-report	Sharp	discussion

	Option 1: the startPreambleForThisPartition and numberOfPreamblesPerSSB-ForThisPartition in RA report is set to that of the first used set of RA resource.

	Option 2: the startPreambleForThisPartition and numberOfPreamblesPerSSB-ForThisPartition in RA report is set to that of the last used set of RA resource.

	Option 3: define a new list of startPreambleForThisPartition and numberOfPreamblesPerSSB-ForThisPartition in RA report to include that of the used multiple sets of RA resource.

Proposal: RAN2 discusses how to clarify the startPreambleForThisPartition and numberOfPreamblesPerSSB-ForThisPartition setting in RA report in case of an RA procedure with msg1 repetition number ramping.

R2-2506107	Correction to startPreambleForThisPartition and numberOfPreamblesPerSSB-ForThisPartition in RA-report	Sharp	CR	Rel-18	38.331	18.6.0	5456	-	F	NR_ENDC_SON_MDT_enh2-Core

-	Ericsson think the UE always logs the latest used resources, so they agree, but the spec is already clear on this. Sharp does not agree that its clear. ZTE supports the CR. LG thinks its beneficial to clarify this in the spec but want to understand that indeed it is the case that two UEs in different situations (different RSRP) might end up with the same RA report since they both might end up using the same RA resources in the end of the procedure. Qualcomm is not sure a clarification is needed.

Agreed

R2-2506183	Correction on the dlRSRPAboveThreshold in RA-report	Xiaomi	CR	Rel-18	38.331	18.6.0	5466	-	F	NR_ENDC_SON_MDT_enh2-Core

-	Ericsson want to discuss the third change more offline. Nokia thinks we don’t need to add suffixes unless needed.

[AT131][608][SONMDT] Correction on the dlRSRPAboveThreshold in RA-report (Xiaomi)

Scope:

    - Discuss offline (face-to-face preferred, if suitable)

    - Produce agreeable CRs

Intended outcome:

    - Agreed CR(s) in R2-2506375

Deadline:

    - Wednesday 17:00. The intention is to agree over email.

R2-2506375	Correction on the dlRSRPAboveThreshold in RA-report	Xiaomi	CR	Rel-18	38.331	18.6.0	5466	1	F	NR_ENDC_SON_MDT_enh2-Core

Agreed

SHR

R2-2506108	Correction for setting choCandidate in SHR	Sharp	CR	Rel-18	38.331	18.6.0	5457	-	F	NR_ENDC_SON_MDT_enh2-Core

-	Samsung thinks the UE cannot know if masterCellGroup is included or not since the UE may not do early decoding of the CHO. Ericsson think we can simplify by saying that the cell is a CHO candidate. CATT think we shouldn’t consider SON/MDT enhancements for NES/NTN CHOs.

Not pursued

SPR

R2-2506182	Correction on SPR determination procedure	Xiaomi	CR	Rel-18	38.331	18.6.0	5465	-	F	NR_ENDC_SON_MDT_enh2-Core

-	Samsung thinks the CR is wrong.

Not pursued

Withdrawn/old revisions

R2-2505309	Correction on SPR determination procedure	Xiaomi	CR	Rel-18	38.331	18.6.0	5404	-	F	NR_ENDC_SON_MDT_enh2-Core	Withdrawn

R2-2505310	Correction on the dlRSRPAboveThreshold in RA-report	Xiaomi	CR	Rel-18	38.331	18.6.0	5405	-	F	NR_ENDC_SON_MDT_enh2-Core	Withdrawn

#### 7.0.2.12	Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR

(NR_DualTxRx_MUSIM-Core; leading WG: RAN2; REL-18; WID: RP-233071)

R2-2505052	LS to RAN2 on MUSIM gap and Measurement gap (R4-2508447; contact: R4-2508447)	RAN4	LS in	Rel-18	NR_DualTxRx_MUSIM-Core	To:RAN2

Noted

R2-2505996	Clarification on the MUSIM Gap	ZTE Corporation	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

Proposal 2: RAN2 to confirm that when network does not configure the priority even the UE has indicated the preferred MUSIM priority, UE behaviour is not specified.

-	Mediatek thinks this is not needed as RAN4 indicated that there are no requirements and that is different from UE behaviour not specified.

Proposal 3: RAN2 to confirm whether the NOTE below has included the case in the Proposal 2.

NOTE: If network does not configure the relative priorities among MUSIM gaps as indicated by the UE, UE behaviour is not specified.

-	Mediatek thinks these are different cases.  Ericsson think that it includes the cases where the network doesn’t provide the priorities.   Vivo thinks that further clarification are needed.

Proposal 3a: If not included, the NOTE should be further clarified, e.g.

NOTE: If network does not configure the relative priorities among MUSIM gaps (including the case that the network does not configure the priority for the MUSIM Gaps) as indicated by the UE, UE

behaviour is not specified.

No further clarification

Agreements

- No further RAN2 specification impact is required for resolving MUSIM gap and measurement gap collisions, based on the reply LS [1].

R2-2505997	Clarification to musim-AffectedBandsList	ZTE Corporation	CR	Rel-18	38.331	18.6.0	5447	-	F	NR_DualTxRx_MUSIM-Core

-	Vivo doesn’t think we can agree to the CR this meeting as it is up to UE implementation.   ZTE thinks we need clear specification.

-	Huawei thinks that we can agree on the understanding on the chair minutes and not agree to the CR.

[CB Thursday]

[AT131][016][MUSIM] Offline (ZTE)

Intended outcome: Discuss both CBs for MUSIM

Deadline:  Thursday

R2-2506446	Clarification to musim-AffectedBandsList	ZTE Corporation	CR	Rel-18	38.331	18.6.0	5447	1	F	NR_DualTxRx_MUSIM-Core

The CR is agreed

R2-2506447	Report of the  [AT131][016][MUSIM] Offline (ZTE)	ZTE	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

Noted

#### 7.0.2.13	NR MIMO evolution

(NR_MIMO_evo_DL_UL-Core; leading WG: RAN1; REL-18; WID: RP-233028)

To be treated in breakout session (Erlin)

(NR_MIMO_evo_DL_UL-Core; leading WG: RAN1; REL-18; WID: RP-233028)

R2-2505013	LS on maximum transmission power for STxMP (R1-2504839; contact: Huawei)	RAN1	LS in	Rel-18	NR_MIMO_evo_DL_UL-Core	To:RAN4	Cc:RAN2

Noted

R2-2505019	Reply LS on differentiation of sDCI based mTRP and sTRP (R1-2504885; contact: CATT)	RAN1	LS in	Rel-18	NR_MIMO_evo_DL_UL-Core	To:RAN2

Discusison

-	OPPO wonders whether R1 will discuss alternatives further. CATT think R1 will not discuss and we in R2 can decide.

Noted

R2-2505462	Correction on simultaneousU-TCI-UpdateListx for Unified TCI State Update	CATT, Samsung	CR	Rel-18	38.331	18.6.0	5418	-	F	NR_MIMO_evo_DL_UL-Core

The CR is not pursued

Discussion

-	Ericsson think the first sentence in the field description is sufficient, so no need to the CR.

-	HW think the added texts seem to be generally correct to any MAC CE, so not sure why we need to specifically highlight here.

RAN2 understand NW implementation can ensure the valid configuration in the serving cell(s) of the simultaneousU-TCI-UpdateListx for which the unified TCI state update MAC CE(s) can be applied.

R2-2506160	Correction to reportQuantity	Ericsson	CR	Rel-18	38.331	18.6.0	5462	-	F	NR_MIMO_evo_DL_UL-Core

The CR is agreed.

Discussion

-	OPPO think this is correct. Nokia agree.

-	Nokia think this is editorial and can go to the CR from the RRC spec Rapp.

#### 7.0.2.14	Enhancements of NR Multicast and Broadcast Services

(NR_MBS_enh-Core; leading WG: RAN2; REL-18; WID: RP-231829)

#### 7.0.2.15	Enhancement on NR QoE management and optimizations for diverse services

(NR_QoE_enh-Core; leading WG: RAN3; REL-18; WID: RP-223488)

R2-2505066	LS to RAN3 on MBS Communication Service Type (S4-251096; contact: Nokia)	SA4	LS in	Rel-18	NR_QoE_enh-Core	To:RAN3	Cc:RAN2, SA5

Noted

#### 7.0.2.16	XR Enhancements for NR

(NR_XR_enh-Core; leading WG: RAN2; REL-18; WID: RP-230786)

R2-2505327	Correction to the description of PDU set QoS parameter	Huawei, HiSilicon, Nokia (Rapporteur)	CR	Rel-18	38.300	18.6.0	1010	-	F	NR_XR_enh-Core

Add clarification that PSDB and PSER should be provided together

[AT131][017][R18 XR] PDU set QoS parameter CR (Huawei)

Intended outcome: update and agree to CR by email

Deadline:  Thursday

R2-2506459	Correction to the description of PDU set QoS parameter	Huawei, HiSilicon, Nokia (Rapporteur)	CR	Rel-18	38.300	18.6.0	1010	1	F	NR_XR_enh-Core

The CR is agreed

R2-2505408	Correction on DSR triggering	vivo	CR	Rel-18	38.321	18.6.0	2099	-	F	NR_XR_enh-Core

Postponed

- Xiaomi does not see strong need to change. It is an optimization. LGE agrees.

- Nokia thinks this makes sense.

- LGE thinks this would require to also change DSR cancellation condition. We can leave as is.

- Samsung agrees with LGE and Xiaomi. Network knows about the segment so will schedule the transmission.

- Ericsson does not think we need a change.

- Sharp agrees that an issue can happen.

- After offline discussion vivo clarifies that companies have different understanding on the current specification and would like to check with product teams.

#### 7.0.2.17	NR NTN enhancements

(NR_NTN_enh-Core; leading WG: RAN1; REL-18; WID: RP-232669)

R2-2505534	Clarification on reference point for hard satellite switch with resynchronization	Qualcomm Inc., Huawei, HiSilicon, Xiaomi	CR	Rel-18	38.331	18.6.0	5372	1	F	NR_NTN_enh-Core	R2-2504339

- QC would like to have a clarification for the description of the ssb-TimeOffset, for which the behaviour is not clarified anywhere

- ZTE thinks we can clarify that “…if this field is provided the cell is operating in soft satellite switch…”

- Ericsson thinks the previous agreement to capture the intention in the meeting notes is sufficient. No need for a spec change. Nokia agrees

- Vivo can accept to have a clarification

- CMCC agrees a clarification is useful, possibly also according the ZTE proposal

- Samsung is fine with the CR (but need to check whether the CR is using the latest version)

We only introduce a clarification for the ssb-TimeOffset along the lines of the proposal from ZTE

Revised in R2-2506274

R2-2506274	Clarification on reference point for hard satellite switch with resynchronization	Qualcomm Inc., Huawei, HiSilicon, Xiaomi	CR	Rel-18	38.331	18.6.0	5372	2	F	NR_NTN_enh-Core

Agreed

R2-2505577	Correction on the reference point of t-ServiceStart	MediaTek	CR	Rel-18	38.331	18.6.0	5423	-	F	NR_NTN_enh-Core

- HW thinks the current text is correct

- Vivo thinks that from the UE point of view there is no issue with the current description

- Ericsson also thinks there is no issue with the current description/modelling. CATT agrees. QC and CMCC also agree.

Not pursued

R2-2505715	Final Thoughts on IDC issue between GNSS and NTN UL	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_NTN_enh-Core	R2-2504133

Proposal 1: Do not consider an extension of autonomousDenialSlots which would increase the number of slots the UE can autonomously deny and bring additional complexity to UL scheduling.

Proposal 2: Do not consider extending the length of the activeDuration in IDC-TDM solution.

Noted

R2-2505420	Discussion on eventD1/D2 and condEventD1/D2/T1	Samsung	discussion	Rel-18	NR_NTN_solutions, NR_NTN_enh-Core

Withdrawn

#### 7.0.2.18	IoT NTN enhancements

(IoT_NTN_enh-Core; leading WG: RAN2; REL-18; WID: RP-223519)

R2-2505914	Various corrections on connected mode RRM for IoT NTN	Samsung	CR	Rel-18	36.331	18.6.0	5121	1	F	IoT_NTN_enh-Core	R2-2504095

1) Clarify that SIB33(-NB) scheduling can be assumed to be unchanged.

-	Xiaomi doesn’t agree on this

-	Nokia also thinks this is not needed

-	ZTE has sympathy for the first change (but we could say that the UE may skip SIB1). HW and CATT agree

-	vivo could agree

First change is agreed

2) Clarify that for NTN, E-UTRAN configures listed cells (cellsToAddModList) and that the UE is only required to measure on the listed cells.

-	Xiaomi doesn’t agree on this either

-	ZTE/HW disagree

- 	vivo/CATT think this is not needed

Second change is not pursued

3) Clarify that Layer 3 filtering is not applicable to event D1/D2/T1.

-	ZTE/HW disagree

-	CATT thinks this is not needed

Second change is not pursued

Revised in R2-2506275

R2-2506275	Corrections on IoT NTN SIB33	Samsung	CR	Rel-18	36.331	18.6.0	5121	2	F	IoT_NTN_enh-Core	R2-2504095

Agreed

R2-2506172	Clarification of satellite identifiers	THALES, Samsung, Sateliot, Nordic Semiconductor ASA, Novamint, CATT, Ericsson	CR	Rel-18	36.300	18.5.0	1430	-	F	IoT_NTN_enh-Core

- Ericsson thinks a Stage3 CR is better and sufficient. Toyota agrees

- Thales thinks that also a Stage 2 CR is beneficial.

- ZTE thinks a Stage2 CR is enough.

- ZTE thinks the cover page should be fixed. Changes on changes also need to be removed

Change is agreed. Actual wording can be further discussed. Coversheet also needs to be revised

Revised in R2-2506276

R2-2506276	Clarification of satellite identifiers	THALES, Samsung, Sateliot, Nordic Semiconductor ASA, Novamint, CATT, Ericsson	CR	Rel-18	36.300	18.5.0	1430	1	F	IoT_NTN_enh-Core

- ZTE is fine with this

Change the second sentence to “How the satellite identifiers are allocated to the NTN payloads is up to network implementation.”

Revised in R2-2506295

R2-2506295	Clarification of satellite identifiers	THALES, Samsung, Sateliot, Nordic Semiconductor ASA, Novamint, CATT, Ericsson	CR	Rel-18	36.300	18.5.0	1430	2	F	IoT_NTN_enh-Core

Agreed

R2-2506173	Clarification of satellite identifiers	THALES, Samsung, Sateliot, Nordic Semiconductor ASA, Ericsson	CR	Rel-18	36.331	18.6.0	5152	-	F	IoT_NTN_enh-Core

- ZTE wonders about the need and the wording of the CR (what does “correspond to” means?). MTK agrees

We agree to have a Stage3 change. Actual wording can be further discussed.

Revised in R2-2506277

R2-2506277	Clarification of satellite identifiers	THALES, Samsung, Sateliot, Nordic Semiconductor ASA, Ericsson	CR	Rel-18	36.331	18.6.0	5152	-	F	IoT_NTN_enh-Core

Agreed

#### 7.0.2.19	Enhanced NR Sidelink Relay

(NR_SL_relay_enh-Core; leading WG: RAN2; REL-18; WID: RP-223501)

R2-2505356	Correction on field description of sl-CapabilityInformationSidelink for U2U Relay	OPPO	CR	Rel-18	38.331	18.6.0	5413	-	F	NR_SL_relay_enh-Core

Discussion:

Huawei note that the Source to TSG is missing on the coversheet.

Nokia note that the ME should be ticked on the coversheet.

Source to TSG to be added in coversheet

ME to be ticked on coversheet

Agreed with these changes as R2-2506309

R2-2506309	Correction on field description of sl-CapabilityInformationSidelink for U2U Relay	OPPO	CR	Rel-18	38.331	18.6.0	5413	1	F	NR_SL_relay_enh-Core

=> Agreed, but then coversheet revised by MCC: duplicated meeting number

R2-2506634	Correction on field description of sl-CapabilityInformationSidelink for U2U Relay	OPPO	CR	Rel-18	38.331	18.6.0	5413	2	F	NR_SL_relay_enh-Core

=> Agreed

R2-2505543	Correction to SI reception by remote UE for multi path	LG Electronics Inc.	CR	Rel-18	38.331	18.6.0	5422	-	F	NR_SL_relay_enh-Core

Revised in R2-2506310

Discussion:

Ericsson assume we are not changing any of the behaviours for multi-path.  Xiaomi agree with Ericsson.  Apple also agree and think the new sentence is not needed.

[AT131][404][Relay] SI reception for multi-path relay (LG)

Scope: Discuss the CR in R2-2505543 and determine if a change is needed.

Intended outcome: Agreeable CR if applicable in R2-2506310

Deadline: Wednesday 2025-08-27 1900 IST

R2-2506310	Correction to SI reception by remote UE for multi path	LG Electronics Inc.	discussion	Rel-18	NR_SL_relay_enh-Core

Postponed

Discussion:

LG clarify that a CR is available but needs a tdoc number.

Apple think the intention is to exclude the MP case, and they are not sure the changed phraseology makes a big difference.  LG clarify that the difference is the part about the common search space.

Ericsson are not sure that the additional change is needed since we already have the Uu procedures.

R2-2505760	Corrections on N3C multi-path	ZTE Corporation, Sanechips	discussion	Rel-18	NR_SL_relay_enh-Core

Noted

Discussion:

Xiaomi think on P1/P2, it is UE implementation and does not need to be specified.

Nokia think we previously discussed P7 and agreed option 1 (split is a prerequisite).  Samsung have a different understanding and think we can have them independently; on P1/P2, they agree with Xiaomi.

OPPO agree that P1/P2 are not necessary; for P4/P5/P6 on the indication of the network capability, they think it is an optimisation.

Huawei think P3 to release the configuration during re-establishment is similar to other UAI configurations, so they have some sympathy, but they think the proposed change is NBC and nothing is broken without it.

Apple think P6 is not needed because N3C is handled by the UE implementation, but they think P4 is valid and some clarification of early detection would be useful.  Xiaomi wonder if there can really be different understandings of P4.  Huawei agree with Xiaomi and recall that we intended to leave the concept to UE implementation in the details.

Stage 2 CRs postponed from RAN2#130

R2-2505183	Correction to PDCP duplication description for L2 MP using SL relay or N3C indirect path	Huawei, HiSilicon, Nokia (Rapporteur), CMCC	CR	Rel-18	38.300	18.6.0	0989	1	F	NR_SL_relay_enh-Core	R2-2504002

Revised in R2-2506311

Discussion:

LG think PDCP duplication for the multi-path split bearer is already described.  Huawei agree that there is such a description, but in the PDCP duplication clause there is nothing on the multi-path scenario.  Qualcomm think we could reference the existing clause instead of having more material here.

OPPO agree with LG and think it is already covered, and the stage 3 is also clear.

Xiaomi agree that it is already described and the CR is not necessary.

Ericsson do not have a strong concern but think the current description is in terms of DC rather than multi-path, so they see the CR as valid.  Nokia agree with Ericsson and think the existing description is not clear in its applicability to multi-path.

Samsung support the CR.

Huawei indicate that a reader will know from the multi-path clause that PDCP duplication is supported, and the PDCP duplication clause does not by itself make this clear, but they could agree with Qualcomm’s suggestion to cross-reference.  Ericsson think we should not just cross-reference, and we should exclude changing the stage 3.

[AT131][405][Relay] PDCP duplication for multi-path (Huawei)

Scope: Check the CR in R2-2505183.

Intended outcome: Agreeable CR in R2-2506311

Deadline: Wednesday 2025-08-27 1900 IST

R2-2506311	Correction to PDCP duplication description for L2 MP using SL relay or N3C indirect path	Huawei, HiSilicon, Nokia (Rapporteur), CMCC, China Telecom	CR	Rel-18	38.300	18.6.0	0989	2	F	NR_SL_relay_enh-Core	R2-2505183

Agreed

[Chair’s note: This document was made available before the email discussion deadline and checked online for agreeability.  Companies were notified on the email thread, and a final check was made in the CB session after the email discussion deadline.]

R2-2505885	U2U Relays, Peer Remote UE Control Plane Procedures	Ericsson	CR	Rel-18	38.300	18.6.0	1020	-	F	NR_SL_relay_enh-Core

Postponed

Discussion:

Xiaomi think these steps are not that specific and either UE can be in either role.

LG think bidirectionality is already included.

Qualcomm have a similar view to LG.

#### 7.0.2.20	NR Sidelink evolution

(NR_SL_enh2-Core; leading WG: RAN1; REL-18; WID: RP-230077)

#### 7.0.2.21	Expanded and improved NR positioning

(NR_pos_enh2-Core; leading WG: RAN1; REL-18; WID: RP-232670)

LS on startSFN and related contributions

R2-2505014	Reply LS on startSFN for positioning SRS frequency hopping (R1-2504854; contact: ZTE)	RAN1	LS in	Rel-18	NR_pos_enh2-Core	To:RAN2

Noted

Discussion:

Lenovo have a related CR (R2-2505599), but it is aligned with the ZTE CR.

R2-2506181	Corrections on the startSFN of the UTW	ZTE Corporation, Ericsson, vivo, Qualcomm, Samsung, Nokia	CR	Rel-18	38.331	18.6.0	5464	-	F	NR_pos_enh2-Core

Revised in R2-2506313

Discussion:

Lenovo think on the description of the UTW, it should refer to the starting slot rather than the current slot, and the description terminology should be consistent across different field descriptions.

CATT wonder if the formula is necessary (in RRC or anywhere else in RAN2 specs) once startSFN is removed.

Qualcomm note that we agreed to capture the formula, and they are not aware of it being captured in a RAN1 spec.

Ericsson think Lenovo’s concern can be checked, but they think the wording with the current slot is correct.

Huawei understand that the first slot is slot 0 plus the offset, and it repeats periodically; they see that the current description is just an easier way to capture this.

vivo support the current CR and think the UE calculates the correct frame based on the current formula.

Lenovo think there are other detailed issues that should be checked offline.

[AT131][402][POS] CR on startSFN (ZTE)

Scope: Check the wording and details of the CR in R2-2506181 (can take into account also the related wording in R2-2505599).

Intended outcome: Agreed CR (without CB if possible) capturing the formula in accordance with previous agreements, in R2-2506313

Deadline: Wednesday 2025-08-27 1900 IST

R2-2506313	Corrections on the startSFN of the UTW	ZTE Corporation, Ericsson, vivo, Qualcomm, Samsung, Nokia, Lenovo	CR	Rel-18	38.331	18.6.0	5464	1	F	NR_pos_enh2-Core

Agreed

SP SRS frequency hopping

R2-2505323	Discussion on SP positioning SRS frequency hopping	Huawei, HiSilicon, Ericsson	discussion	Rel-18	NR_pos_enh2-Core

Noted

R2-2505266	Correction to SP positioning SRS transmission with frequency hopping	Ofinno	CR	Rel-18	38.321	18.6.0	2097	-	F	NR_pos_enh2-Core

Not pursued

Discussion (joint):

Ofinno clarify that they do not see a need to add a restriction on additional SP-SRS in the same BWP.

vivo support Huawei’s option and think there is no need to configure two SP-SRS in the same BWP.

Samsung note that there is no RAN1 agreement to have the restriction and think we should not decide this in RAN2 on our own; they prefer option 1 to introduce a new MAC CE, but they acknowledge that it would be a bigger spec impact.

ZTE agree that the restriction is not needed, and they note that a normal UE can be configured for legacy SRS and FH SP-SRS, so they think the Ofinno CR looks better; they do not think adding a new MAC CE would be desirable.

Ericsson think we should not bring in new MAC CEs that would require a capability.  They note that RAN1 left it to RAN2 to capture the restriction, if any, and they support the Huawei CR.

Qualcomm wonder if the non-RedCap FH differences mean that this situation does not apply to non-RedCap UEs, and for RedCap UEs, multiple SP-SRS configurations may not be possible.  So they do not see the need to avoid the restriction.

Lenovo think we should not open all the options that RAN1 discussed, and we should pick an option and inform RAN1.

Nokia think it is challenging to discuss in RAN2 a RAN1 feature where RAN1 could not decide, but they do not think RAN2 should introduce the restriction.

Xiaomi do not see a motivation to configure both SRS for the same BWP; they prefer Huawei’s solution but think the RRC impact is not needed.

Huawei understand that RAN1 delegated the choice to us and any option should be feasible from RAN1 perspective, so the question is how to support the feature with minimum spec impact.

Ofinno wonder why Huawei see their solution as having less impact; in both cases, the backward compatibility is similar, and option 2 does not touch the RRC.

Qualcomm think there are no backward compatibility issues because the dual-SRS feature does not exist; NRPPa cannot support it.

ZTE think from the gNB configuration perspective, the gNB can configure SRS via RRC and activate via the same MAC CE, but the FH can only be configured on a separate BWP, and the MAC CE can only activate one kind of SRS.

Ofinno have the same understanding as ZTE and think RAN1 did some work to handle the collision between FH SP-SRS and other SRS, so they think we would be surprising RAN1 if we took this decision.

vivo agree with Qualcomm’s interpretation and think if we limit to the bandwidth the UE can support, the network can only configure one SRS with FH to the UE and NRPPa cannot support the gNB configuring it with normal SRS.

Ericsson think it is a corner case for them to be configured together and RAN1 are not expecting that this would be done.

Qualcomm think the compatibility of the two SRS is a RAN2 issue, and they are not convinced there is a use case since the SRS are configured per positioning session and the request will never have two SRS configurations.

Lenovo think when we reply to RAN1 we should include RAN3.

Nokia understand that RAN1 have not actually sent us an LS and we are trying to adapt to their agreements without official guidance.  Since the feature comes from RAN1 they would like to have an authoritative instruction from RAN1.  Huawei think it is clear from the RAN1 summary that they mean to leave it to RAN2; vivo agree with Huawei and think it is totally in RAN2 scope.

Ofinno think companies agree that there is an issue, so we need to do something; they agree an LS would have been better, but we cannot leave the feature broken.

Nokia agree that RAN1 discussions left it to RAN2, but we have different understandings in the room about RAN1 status and whether two SRS configurations can coexist in this way.

vivo think we could use Qualcomm’s comment as a starting point and check whether the configuration exists.

Ofinno think option 2 does not break anything, even if the case that it leaves open is not used, while option 3 breaks something whose validity is contested.  Qualcomm agree with this point but consider the spec impact to be less in option 3, and they would prefer to go with the simplest way.

Ericsson think even if the SRS can coexist, it is clear that the acceptable solution in the room is to have the restriction.

Qualcomm think this is clearly not a RAN1 issue; it is affecting the LMF.

Ofinno think we do not have a strong technical reason to go with option 3, but they can accept the majority view.

[AT131][403][POS] SP-SRSp with frequency hopping (Huawei)

Scope: Draft agreeable CRs based on the TPs in R2-2505323.

Intended outcome: Agreeable CRs in R2-2506304 (MAC) / R2-2506305 (RRC).

Deadline: Wednesday 2025-08-27 1900 IST

R2-2506304	Correction on SP Positioning SRS frequency hopping	Huawei, HiSilicon, Ericsson, Ofinno	CR	Rel-18	38.321	18.6.0	2120	-	F	NR_pos_enh2-Core

Agreed

R2-2506305	Correction on SP Positioning SRS frequency hopping	Huawei, HiSilicon, Ericsson, Ofinno	CR	Rel-18	38.331	18.6.0	5467	-	F	NR_pos_enh2-Core

Agreed, but then coversheet revised by MCC: space in the middle of the tdoc number (causes problems in portal scripts)

R2-2506633	Correction on SP Positioning SRS frequency hopping	Huawei, HiSilicon, Ericsson, Ofinno	CR	Rel-18	38.331	18.6.0	5467	1	F	NR_pos_enh2-Core

=> Agreed

Other stage 3 CRs

R2-2505155	Correction on NR Sidelink measurements based on SL-PRS	vivo, Ericsson	CR	Rel-18	38.331	18.6.0	5398	-	F	NR_pos_enh2-Core

Agreed

R2-2505599	Miscellaneous corrections on NR positioning enhancements	Lenovo	CR	Rel-18	38.331	18.6.0	5425	-	F	NR_pos_enh2-Core

Changes 3, 4, and 5 on the coversheet are backed out

Agreed with these changes are R2-2506306

R2-2505600	Miscellaneous corrections on Aggregated SP Positioning SRS Activation/Deactivation MAC CE	Lenovo	CR	Rel-18	38.321	18.6.0	2107	-	F	NR_pos_enh2-Core

Agreed

Discussion:

Huawei wonder if we could merge these changes into another CR.  Lenovo think this is fine if we have another MAC CR.  Huawei acknowledge that these changes are separate from other things we have discussed in the MAC.

R2-2505896	Correction for timing Reporting Granularity Factor	Ericsson	CR	Rel-18	37.355	18.5.0	0560	-	F	NR_pos_enh2-Core

NOTE to be deleted

Interoperability analysis to be corrected on the coversheet

Agreed with these changes as R2-2506307

Discussion:

Xiaomi agree with the intention but think it could be handled with a NOTE.

ZTE also agree with the intention but think there is an internal contradiction between the NOTE and the field description text, and we should only keep one.  Samsung have the same view and think the NOTE could be removed; they also note that the interoperability analysis is for the wrong CR.

Qualcomm think the existing spec is clear, but if some clarification is needed they would prefer to remove the NOTE, which is not idiomatic for LPP language.  They think “configure” is the wrong verb.

Xiaomi can accept deleting the NOTE.

R2-2506027	Correction regarding SL-PRS Resource Request	ASUSTeK	CR	Rel-18	38.321	18.6.0	2112	-	F	NR_pos_enh2-Core

Resource request cancellation to appear in both the Uu and PC5 reset lists

Agreed with this change as R2-2506308

Discussion:

vivo agree with the second change but think the first change is not needed; they believe the spec is correct and the agreement may have a typo.

Samsung understand that the SL-PRS MAC CE is similar to the SL BSR, and they think both MAC resets should trigger the behaviour.  vivo do not understand why it would be in the PC5 list; ASUSTeK understand this is because the SL-PRS is per destination, and when the MAC resets for a destination the corresponding SL-PRS is no longer needed.

Huawei are fine with the second change, and on the first, if the connection is released they see no need to keep the SL-PRS, so they are fine with ASUSTeK’s proposal; for the Uu MAC reset, they think you should cancel everything, so they think it belongs in both places.

vivo are OK with Samsung’s proposal.

ZTE think for the second change, the original wording is not wrong, just unnecessary.  They note that the MAC wording follows the RRC signalling structure.

Stage 2 CRs

R2-2505124	Corrections on LPHAP, carrier phase, bandwidth aggregation and frequency hopping for positioning	CATT,   Ericsson, Nokia, ZTE Corporation	CR	Rel-18	38.305	18.6.0	0187	2	F	NR_pos_enh2-Core	R2-2504883

Not pursued (related to email discussion [Post131][410])

Discussion:

Lenovo are fine with adding the stage 2 description, but they think the proposed description is too much copy/paste from the WI summary and we could be more stylistically consistent in the stage 2, with flows; this is why they proposed removals of the RRC_INACTIVE / RRC_IDLE descriptions.

Huawei have no objection to the CR but think normally RAN1 would do this.  Lenovo note that RAN3 routinely populate the RAN3 part of the stage 2.

Ericsson think a spec reader will not normally read the WI description, and it is good to have some high-level description here.

Nokia think we have not historically had stage 2 input from RAN1 on positioning, and it makes sense to reverse-engineer from stage 3.

Qualcomm have some sympathy for Lenovo’s comment, and they consider that CPP is not a completely clearly defined term and is used unclearly in some places in the CR; they think the content of this CR is already covered in other sections and no information is missing.

CATT indicate that the intention is to clarify the principles and references for the positioning subfeatures, and most of the contents are derived from RAN1 specs.  They would be open to discuss the content further but think we need a new clause.

Lenovo do not see much value if the text does not give some insight into how the procedures work.

Qualcomm think we should target any parts that are not covered in existing specifications, but they have not identified anything critical that is missing; they also think we could try to include updates in the existing sections and avoid having too much growth in the spec.  Ericsson note that we do have dedicated sections for other positioning subfeatures, e.g., GNSS.  Qualcomm think this is right, but we do not, for instance, have a section on RTK as distinct from GNSS.

CATT understand that new subfeatures should in general be described briefly in 38.300.

Ericsson point out that the new subfeatures are used for multiple methods, which makes it not so clear where to capture them.

Qualcomm think we have this information captured in the per-method “information transferred” tables.

Nokia agree that the existing sections capture carrier phase measurements, for example, but for bandwidth aggregation and frequency hopping they think there is not already coverage.  They think we should not automatically capture the full WID detail in stage 2.

[Post131][410][POS] Stage 2 descriptions for Rel-18 positioning (CATT)

Scope: Determine how to structure descriptions of the Rel-18 positioning subfeatures in 38.305 and work towards an agreeable CR, starting from the proposals in R2-2505124 / R2-2505849.

Intended outcome: Agreeable CR if possible, and report to next meeting

Deadline: Long

R2-2505848	Removal of description for positioning in RRC_INACTIVE state	Lenovo	CR	Rel-17	38.305	17.8.0	0193	-	F	NR_pos_enh-Core

Not pursued

R2-2505849	Removal of descriptions for positioning in RRC_INACTIVE and RRC_IDLE state	Lenovo	CR	Rel-18	38.305	18.6.0	0194	-	F	NR_pos_enh2-Core

Not pursued (related to email discussion [Post131][410])

Withdrawn/Not available

R2-2505589	Corrections on the startSFN of the UTW	ZTE Corporation, Ericsson, vivo, Qualcomm, Samsung, Nokia	CR	Rel-18	38.331	18.6.0	5424	-	F	NR_pos_enh2-Core	Withdrawn

Withdrawn

#### 7.0.2.22	Further NR mobility enhancements

(NR_Mob_enh2-Core; leading WG: RAN2; REL-18; WID:RP-233970)

R2-2505718	Corrections to Rel-18 LTM (LTM Recovery to the source cell)	Nokia	discussion	Rel-18	NR_Mob_enh2-Core	R2-2504136

[Lenovo]: Why we didn’t have the problem in the earlier release, e.g. HO case? [Nokia]: For LTM, we have a candidate configuration and it can be different than actual serving cell configuration that has been used, which is different than L3 based HO. [Qualcomm]: Understand it is a corner case. [Lenovo]: Assume it can be handled by network. [Samsung]: Assume network needs to reconfigure the associated candidate configuration also when it reconfigures the serving cell configuration. Assume it is corner case. [Nokia]: Do not think it is a corner case. [Huawei]: Understand if network wants to have recovery function, network should be responsible for configurations in order this not to happen. [ZTE]: Do not agree with Samsung’s comment. network can configure the candidate configuration and the serving configuration differently, but network can handle each case by implementation. If network configures two resource differently, network should be able to handle two resources together.

Not pursued.

R2-2505814	Correction on when the UE applies the SR configuration	Ericsson	CR	Rel-18	38.331	18.6.0	5442	-	F	NR_Mob_enh2-Core

[Samsung]: Understand it is not a correction. It proposes a new mechanism. If SR resource is directly used during LTM cell switch procedure, we need a mapping between SSB and SR resource. [Lenovo]: If the intention is to use SR after LTM cell switch completion, it’s ok. However, if the intention is to use SR during LTM cell switch, it’s not acceptable as a correction. [MediaTek]: Understand, based on the sentence “Before RACH-less LTM procedure completion, the UE shall not trigger random access procedure if it does not have a valid PUCCH resource for triggered SRs.” in 38.300, SR resource can be available during LTM cell switch for RACH-less LTM case. [Session chair]: Let’s have offline to check which understanding is correct. [Ericsson]: Will take the offline until either Wednesday session or CB session.

Comeback:

[Ericsson]: Discussed with other companies that had concerns and all companies are ok with the proposal now. We do not need to agree the CR, but it’s good to capture it in the minutes. [ZTE]: Wonder why it cannot be applied to C-LTM case? [Ericsson]: C-LTM is different than LTM since the target candidate cannot receive the selected beam information from the source. [Samsung]: Do not see any additional spec impact for LTM since it is already clarified in 38.300. [Apple]: Wonder if it can be also applicable to RACH-less HO? [Nokia]: Want to have more time to consider. [Session chair]: Any companies want to have more time to check it until next meeting? No company.

SR may be available during RACH-less LTM cell switch procedure if TA is valid.

R2-2506012	Correction on Power Ramping for early TA	Samsung	CR	Rel-18	38.321	18.6.0	2111	-	F	NR_Mob_enh2-Core

Agreed.

R2-2506073	Correction for RRC segment handling	Samsung	CR	Rel-18	38.331	18.6.0	5450	-	F	NR_Mob_enh2-Core

[Huawei]: Wonder why we don’t have such a sentence in HO case? [Samsung]: For LTM, MAC is used to cell switch command. For HO, RRC Reconfiguration will be sent after all segmented RRC messages sent. [Ericsson]: Understand, for LTM, the UE clears everything except the specified ones. Assume it is ok without this correction. [MediaTek]: Although the intention is correct, understand network can avoid this situation. [ZTE]: Understand the difference compared to L3 based HO. Support the proposal. Also note it is already captured in other places in the spec. [Apple]: Agree with MediaTek. It can be avoided by network implementation. Note it was discussed for L3 based HO in the past and that’s why it is not specified for the cases where network can handle. [Ericsson]: Agree with Apple. [ZTE]: For LTM, not sure what network option is good to avoid this issue. If cannot avoid, what’s the consequence?

Noted.

R2-2506119	Clarification on PDCCH order triggered early RACH	ZTE Corporation	CR	Rel-18	38.321	18.6.0	2115	-	F	NR_Mob_enh2-Core

[Samsung]: Understand the concerned scenario is not valid since PDCCH order triggered early RACH is a kind of one-shot procedure. [ZTE]: Consider it is valid that the NW can send multiple PDCCH orders with the different PCI, but if we follow the current spec, RACH for the new candidate will not be triggered. [Samsung]: Actually, note was added for the same serving cell although it is not included in the note clearly. Otherwise we need other corrections for other cases (not only for LTM). [ZTE]: Want to have some time to talk more with companies.

Revised in R2-2506421.

R2-2506421	Clarification on PDCCH order triggered early RACH	ZTE Corporation	CR	Rel-18	38.321	18.6.0	2115	1	F	NR_Mob_enh2-Core

[Huawei]: In cover page, Radio Access Network should be clicked.

Radio Access Network should be clicked in affects.

CR in R2-2506232 is agreed with the change above.

R2-2505813	Correction on capabilities for LTM	Ericsson	CR	Rel-18	38.306	18.6.0	1339	-	F	NR_Mob_enh2-Core	Withdrawn

#### 7.0.2.23	TEI18

R2-2505783	Correction on uplink power control for Type-1 CG-PUSCH [PL RS Type 1 CG]	Ofinno	CR	Rel-18	38.331	18.6.0	5438	-	F	TEI18

-	CATT agrees with the first change.   For the second one the changes are not reflected in RAN1 specs.

Add inter-operability for change 1 and change 2 font (if agreable)

Only 1st change is pursued

R2-2506420	Correction on uplink power control for Type-1 CG-PUSCH [PL RS Type 1 CG]	Ofinno	CR	Rel-18	38.331	18.6.0	5438	1	F	TEI18

The CR is agreed, but then coversheet revised by MCC: Meeting header has redundant letters "rk"

R2-2506637	Correction on uplink power control for Type-1 CG-PUSCH [PL RS Type 1 CG]	Ofinno	CR	Rel-18	38.331	18.6.0	5438	2	F	TEI18

=> Agreed

#### 7.0.2.24	Others

Including NR Others, Multi-WI Rel-18 items, e.g. cross-WI-issues not handled under another WI

R2-2505269	Correction on the configuration of the joint TCI for multi-cell scheduling	Ofinno	CR	Rel-18	38.331	18.6.0	5401	-	F	NR_MC_enh-Core

Remove second paragraph on summary for change

-	Docomo and Huawei agree with the change.   As we are aligning with RAN1 we don’t need to be so severe.

Inter-operability needs to be updated

[AT131][018][R18 Others] MC CR (Offino)

Intended outcome: update cover page and agree to CR by email

Deadline:  Thursday

R2-2506419	Correction on the configuration of the joint TCI for multi-cell scheduling	Ofinno	CR	Rel-18	38.331	18.6.0	5401	1	F	NR_MC_enh-Core

The CR is agreed

R2-2505535	UE capability for support of event A4 based CHO for ATG	Qualcomm Inc., Samsung	CR	Rel-18	38.306	18.6.0	1330	-	F	NR_ATG-Core

-	Huawei and Xiaomi agrees with CR but it should be FR1 only like all ATG features.

-	ZTE thinks that this NBC

-	Nokia asks if you support CHO and ATG then you support event A4. Qualcomm explains that A4 was only introduced for NTN and it doesn’t meant it’s supported by ATG.

Update FR1 only

[CB Thursday – whether it is NBC and other concerns]

R2-2506422	UE capability for support of event A4 based CHO for ATG	Qualcomm Inc., Samsung	CR	Rel-18	38.306	18.6.0	1330	1	F	NR_ATG-Core

[AT131][041][R18] Event A4-based CHO (Qualcomm)

Intended outcome: agree to CRs after capturing NBC statement

Deadline:  Thursday

R2-2506483	UE capability for support of event A4 based CHO for ATG	Qualcomm Inc., Samsung	CR	Rel-18	38.306	18.6.0	1330	2	F	NR_ATG-Core	R2-2506422

The CR is agreed

R2-2506423	UE capability for support of event A4 based CHO for ATG	Qualcomm Inc., Samsung	CR	Rel-18	38.331	18.6.0	5468	-	F	NR_ATG-Core

=> Revised in R2-2506484

R2-2506484	UE capability for support of event A4 based CHO for ATG	Qualcomm Inc., Samsung	CR	Rel-18	38.331	18.6.0	5468	1	F	NR_ATG-Core	R2-2506423

The CR is agreed

R2-2505668	Miscellaneous non-controversial corrections Set XXVI	Ericsson	CR	Rel-18	38.331	18.6.0	5427	-	F	NR_newRAT-Core, TEI18

Withdrawn

R2-2505855	Clarification for DCP	Ericsson	CR	Rel-18	38.300	18.6.0	1016	-	F	NR_UE_pow_sav-Core

-	Huawei thinks that the current sentence is fine.  Vivo agrees and we have other places where we have used similar wording.

The CR is not pursued

R2-2506053	Correction to multi-Rx preference reporting [MultiRx]	Huawei, HiSilicon, Apple	CR	Rel-18	38.331	18.6.0	5449	-	F	TEI18, NR_FR2_multiRX_DL-Core

Remove TEI code  [MultiRx]

R2-2506411	Correction to multi-Rx preference reporting	Huawei, HiSilicon, Apple	CR	Rel-18	38.331	18.6.0	5449	1	F	NR_FR2_multiRX_DL-Core

The CR is agreed

# 8	Rel-19

## 8.0	General

### 8.0.0	In-principle agreed CRs

This AI is reserved for Rel-19 in-principle agreed CRs that need to be formally agreed

R2-2505270	SSB position restrictions for less-than-5MHz SCells	Qualcomm Incorporated	CR	Rel-19	38.331	18.6.0	5249	3	B	NR_FR1_lessthan_5MHz_BW_Ph2-Core	R2-2501389

The CR is agreed

To be treated in positioning offline

R2-2505317	Introduction of control parameters for on-demand posSIB request [OdPosSIB-Req]	Huawei, HiSilicon, Ericsson, Samsung	CR	Rel-19	38.331	18.6.0	5406	-	B	TEI19

Delivery interval to be removed and delivery amount to be replaced by “start” and “stop”

Revised in R2-2506322 (post-meeting email)

Discussion:

Huawei clarify that the control parameters have been added since last meeting.

Lenovo understood we agreed not to have the delivery interval.  Ericsson understand there was an expectation that the gNB would use the same periodicity as for broadcast, but they now think this will not work and the interval is needed.

Qualcomm think this repeats the discussion we already had; they do not see how the UE can select the periodicity, since it comes from the LMF.

Ericsson think we could agree that the gNB provides the data with the same periodicity as it receives from the LMF, and if we capture in the spec that the gNB will provide the posSIB when the content is updated, it would be an OK alternative.

Samsung note that we have the constraint that the interval requested by the UE should be larger than the legacy posSIB interval.  Qualcomm think this would mean the UE does not get every update if it requests a larger periodicity.

Ericsson are not sure what the deliveryAmount means without the deliveryInterval.

Huawei think Ericsson’s suggestion may make sense but is outside the scope of this CR; they do not want to exclude any solutions, and they are not sure all implementations will work with Ericsson’s suggestion; they think the gNB might want to deliver with a different periodicity.

Qualcomm think we do not necessarily need to capture the gNB implementation; they think the point is to avoid repeated requests for periodic assistance data.

Huawei agree that the default value would be the periodicity in broadcast, but if the UE requests a different periodicity the gNB should be able to meet the request.

Qualcomm think if we backed out the delivery interval, only the first and last instances of the assistance data would be useful.  Ericsson think it would be OK to have only start and stop.

Qualcomm indicate that the AIP version had an enumeration of values; they think the interpretation of the integer value for the amount is strange.

R2-2505318	Introduction of control parameters for on-demand posSIB request [OdPosSIB-Req]	Huawei, HiSilicon, Ericsson, Samsung	CR	Rel-19	38.306	18.6.0	1323	-	B	TEI19

R2-2505319	Introduction of control parameters for on-demand posSIB request [OdPosSIB-Req]	Huawei, HiSilicon, Ericsson, Samsung	CR	Rel-19	38.300	18.6.0	1009	-	B	TEI19

R2-2505320	Introduction of control parameters for on-demand posSIB request [OdPosSIB-Req]	Huawei, HiSilicon, Ericsson, Samsung	CR	Rel-19	38.305	18.6.0	0191	-	B	TEI19

[Post131][409][POS] Control parameters for on-demand posSIB request (Huawei)

Scope: Revise the CR in R2-2505317 in accordance with the decision of RAN2#131 to reduce the reporting to “start” and “stop”, and confirm the accompanying CRs in R2-2505318 / R2-2505319 / R2-2505320

Intended outcome: Agreed CRs in R2-2506322 (38.331) and original tdoc numbers (other specs)

Deadline: Short (for RP)

=> Agreed in:

- R2-2506654 (38.305)

- R2-2506655 (38.331)

- R2-2506656 (38.306)

- R2-2506657 (38.300)

R2-2506654	Introduction of control parameters for on-demand posSIB request [OdPosSIB_Req]	Huawei, HiSilicon, Ericsson, Samsung	CR	Rel-19	38.305	18.6.0	0191	2	B	TEI19

=> Agreed

R2-2506655	Introduction of control parameters for on-demand posSIB request [OdPosSIB_Req]	Huawei, HiSilicon, Ericsson, Samsung	CR	Rel-19	38.331	18.6.0	5406	3	B	TEI19

=> Agreed

R2-2506656	Introduction of control parameters for on-demand posSIB request [OdPosSIB_Req]	Huawei, HiSilicon, Ericsson, Samsung	CR	Rel-19	38.306	18.6.0	1323	3	B	TEI19

=> Agreed

R2-2506657	Introduction of control parameters for on-demand posSIB request [OdPosSIB_Req]	Huawei, HiSilicon, Ericsson, Samsung	CR	Rel-19	38.300	18.6.0	1009	3	B	TEI19

=> Agreed

R2-2506322	Introduction of control parameters for on-demand posSIB request [OdPosSIB-Req]	Huawei, HiSilicon, Ericsson, Samsung	CR	Rel-19	38.331	18.6.0	5406	1	B	TEI19

=> Revised in R2-2506576

R2-2506576	Introduction of control parameters for on-demand posSIB request [OdPosSIB-Req]	Huawei, HiSilicon, Ericsson, Samsung	CR	Rel-19	38.331	18.6.0	5406	2	B	TEI19

=> Agreed, but then coversheet revised by MCC in R2-2506655: Hyphen in TEI identifier

R2-2506526	Introduction of control parameters for on-demand posSIB request [OdPosSIB-Req]	Huawei, HiSilicon, Ericsson, Samsung	CR	Rel-19	38.306	18.6.0	1323	1	B	TEI19

=> Revised in R2-2506577

R2-2506577	Introduction of control parameters for on-demand posSIB request [OdPosSIB-Req]	Huawei, HiSilicon, Ericsson, Samsung	CR	Rel-19	38.306	18.6.0	1323	2	B	TEI19

=> Agreed, but then coversheet revised by MCC in R2-2506656: Hyphen in TEI identifier

R2-2506527	Introduction of control parameters for on-demand posSIB request [OdPosSIB-Req]	Huawei, HiSilicon, Ericsson, Samsung	CR	Rel-19	38.300	18.6.0	1009	1	B	TEI19

=> Revised in R2-2506578

R2-2506578	Introduction of control parameters for on-demand posSIB request [OdPosSIB-Req]	Huawei, HiSilicon, Ericsson, Samsung	CR	Rel-19	38.300	18.6.0	1009	2	B	TEI19

=> Agreed, but then coversheet revised by MCC in R2-2506657: Hyphen in TEI identifier

R2-2506528	Introduction of control parameters for on-demand posSIB request [OdPosSIB-Req]	Huawei, HiSilicon, Ericsson, Samsung	CR	Rel-19	38.305	18.6.0	0191	1	B	TEI19

=> Agreed, but then coversheet revised by MCC in R2-2506654: Hyphen in TEI identifier

R2-2505514	Corrections to TS 38.300 on multi-path relay enhancement [N3C_M_Relay]	CMCC,ZTE, MediaTek, vivo, Huawei, CATT, Nokia, Nokia Shanghai Bell, Xiaomi, Spreadtrum, UNISOC	CR	Rel-19	38.300	18.6.0	0991	1	B	TEI19	R2-2504382

Update title to introduction of … and remove spec number from title.

=> Revised in R2-2506408

R2-2506408	Introduction of Multi-path Relay Enhancement [N3C_M_Relay]	CMCC, ZTE, MediaTek, vivo, Huawei, CATT, Meta,Nokia, Nokia Shanghai Bell, Xiaomi, Spreadtrum, UNISOC	CR	Rel-19	38.300	18.6.0	0991	2	B	TEI19

The CR is agreed

R2-2505515	Corrections to TS 38.331 on multi-path relay enhancement [N3C_M_Relay]	CMCC, ZTE, MediaTek, vivo, Huawei, CATT, Meta, Nokia, Nokia Shanghai Bell, Xiaomi, Spreadtrum, UNISOC	CR	Rel-19	38.331	18.6.0	5373	1	B	TEI19	R2-2504383

Update as per offline comments

=> Revised in R2-2506409

R2-2506409	Introduction of Multi-path Relay Enhancement [N3C_M_Relay]	CMCC, ZTE, MediaTek, vivo, Huawei, CATT, Meta, Nokia, Nokia Shanghai Bell, Xiaomi, Spreadtrum, UNISOC	CR	Rel-19	38.331	18.6.0	5373	2	B	TEI19

The CR is agreed

R2-2505516	Corrections to TS 38.306 on multi-path relay enhancement [N3C_M_Relay]	CMCC, ZTE, MediaTek, vivo, Huawei, CATT, Nokia, Nokia Shanghai Bell, Xiaomi, Spreadtrum, UNISOC	CR	Rel-19	38.306	18.6.0	1296	1	B	TEI19	R2-2504384

=> Revised in R2-2506200

R2-2506200	Corrections to TS 38.306 on multi-path relay enhancement [N3C_M_Relay]	CMCC, ZTE, MediaTek, vivo, Huawei, CATT, Nokia, Nokia Shanghai Bell, Xiaomi, Spreadtrum, UNISOC	CR	Rel-19	38.306	18.6.0	1296	2	B	TEI19

R2-2506410	Introduction of Multi-path Relay Enhancement [N3C_M_Relay]	CMCC, ZTE, MediaTek, vivo, Huawei, CATT, Meta, Nokia, Nokia Shanghai Bell, Xiaomi, Spreadtrum, UNISOC	CR	Rel-19	38.306	18.6.0	1296	3	B	TEI19

The CR is agreed

R2-2506084	Introduction of ANR reporting of HSDN cells [ANR_HSDN]	Huawei, HiSilicon, CMCC, China Unicom, China Telecom, CATT, NTT DoCoMo, Samsung	CR	Rel-19	38.331	18.6.0	5318	4	B	TEI19	R2-2504894

=> Revised in R2-2506207

R2-2506207	Introduction of ANR reporting of HSDN cells [ANR_HSDN]	Huawei, HiSilicon, CMCC, China Unicom, China Telecom, CATT, NTT DoCoMo, Samsung	CR	Rel-19	38.331	18.6.0	5318	5	B	TEI19

The CR is agreed (but then coversheet revised by MCC: wrong tdoc number)

R2-2506624	Introduction of ANR reporting of HSDN cells [ANR_HSDN]	Huawei, HiSilicon, CMCC, China Unicom, China Telecom, CATT, NTT DoCoMo, Samsung	CR	Rel-19	38.331	18.6.0	5318	7	B	TEI19

=> Agreed

R2-2506085	Introduction of ANR reporting of HSDN cells [ANR_HSDN]	Huawei, HiSilicon, CMCC, China Unicom, China Telecom, CATT, NTT DoCoMo, Samsung	CR	Rel-19	38.306	18.6.0	1264	2	B	TEI19	R2-2504231

=> Revised in R2-2506197

R2-2506197	Introduction of ANR reporting of HSDN cells [ANR_HSDN]	Huawei, HiSilicon, CMCC, China Unicom, China Telecom, CATT, NTT DoCoMo, Samsung	CR	Rel-19	38.306	18.6.0	1264	3	B	TEI19	R2-2506085

The CR is agreed

R2-2506086	Introduction of ANR reporting of HSDN cells [ANR_HSDN]	Huawei, HiSilicon, CMCC, China Unicom, China Telecom, CATT, NTT DoCoMo, Samsung	CR	Rel-19	36.331	18.6.0	5110	3	B	TEI19	R2-2504232

The CR is agreed

R2-2506087	Introduction of ANR reporting of HSDN cells [ANR_HSDN]	Huawei, HiSilicon, CMCC, China Unicom, China Telecom, CATT, NTT DoCoMo, Samsung	CR	Rel-19	36.306	18.5.0	1911	2	B	TEI19	R2-2504233

The CR is agreed

R2-2506403	Restriction on RAT utilization	Apple, Vodafone, InterDigital, Ericsson	CR	Rel-19	36.300	18.5.0	1432	-	B	ECRATU

The CR is agreed

R2-2506404	Restriction on RAT utilization	Apple, Vodafone, InterDigital, Ericsson	CR	Rel-19	38.300	18.6.0	1024	-	B	ECRATU

The CR is agreed

R2-2506405	Restriction on RAT utilization	Apple, Vodafone, InterDigital, Ericsson, Nokia, OPPO, Samsung	CR	Rel-19	36.304	18.4.0	0884	-	B	ECRATU

=> Revised in R2-2506424

R2-2506424	Restriction on RAT utilization	Apple, Vodafone, InterDigital, Ericsson, Nokia, OPPO, Samsung	CR	Rel-19	36.304	18.4.0	0884	1	B	ECRATU

The CR is agreed

R2-2506406	Restriction on RAT utilization	Apple, Vodafone, InterDigital, Ericsson, Nokia, OPPO, Samsung	CR	Rel-19	38.304	18.4.0	0446	-	B	ECRATU

=> Revised in R2-2506425

R2-2506425	Restriction on RAT utilization	Apple, Vodafone, InterDigital, Ericsson, Nokia, OPPO, Samsung	CR	Rel-19	38.304	18.4.0	0446	1	B	ECRATU

The CR is agreed

### 8.0.1	Other

This AI is reserved for Rel-19 LSs from other WGs.  No contributions are expected on these LSs for this meeting

Reserved for UE capability rapporteur input and Rel-19 ASN.1 review

R2-2505008	LS on Rel-19 higher layers parameters list Post RAN1#120bis (R1-2503154; contact: Ericsson)	RAN1	LS in	Rel-19	NR_MIMO_Ph5, NR_duplex_evo, Netw_Energy_NR_enh, NR_LPWUS, NR_XR_Ph3, NR_Mob_Ph4, NR_MC_enh2, TEI19	To:RAN2, RAN3	Cc:RAN4

-	Ericsson highlights that we can still update the structure or descriptions

Noted

R2-2505009	LS on Rel-19 higher layers parameters list Post RAN1#121 (R1-2503242; contact: Ericsson)	RAN1	LS in	Rel-19	NR_AIML_air, NR_MIMO_Ph5, NR_duplex_evo, Netw_Energy_NR_enh, NR_LPWUS, NR_Mob_Ph4, NR_XR_Ph3, NR_NTN_Ph3, IoT_NTN_Ph3, IoT_NTN_TDD, NR_MC_enh2, NR_LBCA_Sw, LTE_terr_bcast_Ph2, TEI19	To:RAN2, RAN3	Cc:RAN4

Noted

R2-2505010	LS on updated Rel-19 RAN1 UE features lists for NR after RAN1#121 (R1-2504675; contact: NTT DOCOMO, AT&T)	RAN1	LS in	Rel-19	NR_AIML_air, NR_MIMO_Ph5, NR_duplex_evo, Netw_Energy_NR_enh, NR_LPWUS, NR_Mob_Ph4, NR_NTN_Ph3, NR_MC_enh2, TEI19	To:RAN2	Cc:RAN4

Noted

R2-2505011	LS on updated Rel-19 RAN1 UE features lists for LTE after RAN1#121 (R1-2504678; contact: NTT DOCOMO, AT&T)	RAN1	LS in	Rel-19	IoT_NTN_Ph3, IoT_NTN_TDD, LTE_terr_bcast_Ph2	To:RAN2	Cc:RAN4

Noted

R2-2505044	LS on Rel-19 RAN4 UE feature list for NR (version 1) (R4-2508078; contact: CMCC)	RAN4	LS in	Rel-19	NR_ENDC_RF_Ph4, NonCol_intraB_ENDC_NR_CA_Ph2, NR_ATG_enh, NR_RRM_Ph5, Netw_Energy_NR_enh, NR_LPWUS, NR_Mob_Ph4, NR_XR_Ph3, NR_FR1_lessthan_5MHz_BW_Ph2, NR_LBCA_Sw, NR_FR1_7MHz_BW	To:RAN2	Cc:RAN1

Noted

R2-2505306	Introduction of UE capability for Rel-19 R1 R4 feature lists, including [TN32HARQ], [Pos_SRSHop], [SRTrig_SSSGSwitch], [SRSCS_ULTxSwitch], [SimCSI_count]	Xiaomi	CR	Rel-19	38.306	18.6.0	1321	-	B	NR_MIMO_Ph5, Netw_Energy_NR_enh, NR_ENDC_RF_Ph4, NR_ATG_enh, TEI19

Endorsed as baseline

The CR will be further updated after RAN1/4 feature list LSs are received

R2-2505307	Introduction of UE capability for Rel-19 R1 R4 feature lists, including [TN32HARQ], [Pos_SRSHop], [SRTrig_SSSGSwitch], [SRSCS_ULTxSwitch], [SimCSI_count]	Xiaomi	CR	Rel-19	38.331	18.6.0	5403	-	B	NR_MIMO_Ph5, Netw_Energy_NR_enh, NR_ENDC_RF_Ph4, NR_ATG_enh, TEI19

Endorsed as baseline

The CR will be further updated after RAN1/4 feature list LSs are received

[POST131][005][UE caps] UE capability CRs (Xiaomi)

Intended outcome:

CR capturing RAN1/4 feature list – Thursday, Sept. 4

CR merging all RAN2 endorsed CRs as well - Tuesday, Sept. 9

Deadline:  Thursday

=> Agreed in:

- R2-2506610 (38.306)

- R2-2506611 (38.331)

R2-2506610	Introduction of Rel-19 UE capability, including [TN32HARQ], [Pos_SRSHop], [SRTrig_SSSGSwitch], [SRSCS_ULTxSwitch], [SimCSI_count], [SimCSI_countNES], [TxSwitch_R19]	Xiaomi	CR	Rel-19	38.306	18.6.0	1321	1	B	NR_AIML_air, NR_Mob_Ph4, NR_MIMO_Ph5, NR_duplex_evo, Netw_Energy_NR_enh, NR_LPWUS, NR_XR_Ph3, NR_NTN_Ph3, NR_ENDC_RF_Ph4, NR_ATG_enh, NR_RRM_Ph5, NonCol_intraB_ENDC_NR_CA_Ph2, NR_SL_relay_multihop, NR_ENDC_SON_MDT_Ph4, TEI19

=> Agreed but then coversheet revised by MCC (Error in WI name: "NR-NR_MIMO_Ph5)

R2-2506627	Introduction of Rel-19 UE capability, including [TN32HARQ], [Pos_SRSHop], [SRTrig_SSSGSwitch], [SRSCS_ULTxSwitch], [SimCSI_count], [SimCSI_countNES], [TxSwitch_R19]	Xiaomi	CR	Rel-19	38.306	18.6.0	1321	2	B	NR_AIML_air, NR_Mob_Ph4, NR_MIMO_Ph5, NR_duplex_evo, Netw_Energy_NR_enh, NR_LPWUS, NR_XR_Ph3, NR_NTN_Ph3, NR_ENDC_RF_Ph4, NR_ATG_enh, NR_RRM_Ph5, NonCol_intraB_ENDC_NR_CA_Ph2, NR_SL_relay_multihop, NR_ENDC_SON_MDT_Ph4, TEI19

=> Agreed

R2-2506611	Introduction of Rel-19 UE capability, including [TN32HARQ], [Pos_SRSHop], [SRTrig_SSSGSwitch], [SRSCS_ULTxSwitch], [SimCSI_count], [SimCSI_countNES], [TxSwitch_R19]	Xiaomi	CR	Rel-19	38.331	18.6.0	5403	1	B	NR_AIML_air, NR_Mob_Ph4, NR_MIMO_Ph5, NR_duplex_evo, Netw_Energy_NR_enh, NR_LPWUS, NR_XR_Ph3, NR_NTN_Ph3, NR_ENDC_RF_Ph4, NR_ATG_enh, NR_RRM_Ph5, NonCol_intraB_ENDC_NR_CA_Ph2, NR_SL_relay_multihop, NR_ENDC_SON_MDT_Ph4, TEI19

=> Agreed but then coversheet revised by MCC Error in WI name: "NR-NR_MIMO_Ph5

R2-2506628	Introduction of Rel-19 UE capability, including [TN32HARQ], [Pos_SRSHop], [SRTrig_SSSGSwitch], [SRSCS_ULTxSwitch], [SimCSI_count], [SimCSI_countNES], [TxSwitch_R19]	Xiaomi	CR	Rel-19	38.331	18.6.0	5403	2	B	NR_AIML_air, NR_Mob_Ph4, NR_MIMO_Ph5, NR_duplex_evo, Netw_Energy_NR_enh, NR_LPWUS, NR_XR_Ph3, NR_NTN_Ph3, NR_ENDC_RF_Ph4, NR_ATG_enh, NR_RRM_Ph5, NonCol_intraB_ENDC_NR_CA_Ph2, NR_SL_relay_multihop, NR_ENDC_SON_MDT_Ph4, TEI19

=> Agreed

R2-2505525	Per band and per BC capability	Samsung, Xiaomi	discussion	Rel-19

Proposal 1: The lower capability derived based on both per band and per BC capability is applied.

Proposal 2: If per band capability is absent for a band, the feature is not supported for the band even if per BC capability is indicated for a BC associating the band.

Proposal 3: per BC capability can be indicated when at least one band in a BC indicates corresponding per band capability.

Proposal 4: If per BC capability is absent for a BC, per band capability is applied for the BC.

Proposal 5: if proposal 1-4 is agreed, RAN2 agree to add high level description for per band and per BC capabilities in TS38.306.

Proposal 6: if proposal 1-4 is agreed, RAN2 agree to send an LS to RAN1 to check if the assumptions are correct for per band and per BC capabilities that they introduced.

Proposal 7: RAN2 agree to ask RAN1 to confirm if it is correct to assume that pre-requisite for per band and per BC capabilities is defined for per band and per BC separately.

-	Vivo thinks per band and BC are quite independent.

-	Nokia and Qualcomm think that we need to do this on a case by case basis.  There are too many capabilities.   We can send an LS to RAN1 after looking at them one by one.

-	Xiaomi thinks that if we can align with the examples we can have an understand offline line and then communicate with RAN1.

-	ZTE agrees with the intention of the paper but we can narrow the scope.

-	T mobile thinks we should make sure RAN4 is involved.

-	Ericsson think that the only capabilities we should look at are the ones that have same signaling for R16.

-	Apple thinks that a general principles on what RAN2 thinks would help narrow down RAN1/4 discussions.

Send LS to RAN1

[POST131][006][UE caps] Per band/BC  (Samsung)

Intended outcome: agree to LS

Deadline:  short

=> Approved in R2-2506565 (LS out)

R2-2506473	Draft LS on per band and per BC capability	Samsung, Xiaomi	LS out	Rel-19	NR_MIMO_Ph5, NR_AIML_air	To:RAN1	Cc:RAN4

R2-2506565	LS on per band and per BC capability	RAN2	LS out	Rel-19	NR_MIMO_Ph5, NR_AIML_air	To:RAN1	Cc:RAN4

=> Approved

R2-2505664	Report of [POST130][002][ASN.1] cross WI interactions	Ericsson	discussion	Rel-19	TEI19

continue to collect a list of cross-WI issues (and issues with respect to existing features) and try to propose solutions in an AT email discussion.

In POST email discussion, companies to discuss the list of cross-WI issues and specification impacts, conclude on specification impacts at RAN2#131bis.

[AT131][007][ASN.1] Cross WI interactions (Ericsson)

Intended outcome: continue to collect a list of cross-WI issues (and issues with respect to existing features)

Deadline:  Friday

R2-2506434	Report of [AT131][007][ASN.1] Cross WI interactions (Ericsson)	Ericsson (Rapporteur)	discussion

R2-2505665	ASN.1 review plan	Ericsson	discussion	Rel-19	TEI19

Noted and maybe updated after any offline comments

R2-2505781	LPP ASN.1 Review Plan	Qualcomm Incorporated (Rapporteur)	discussion

-	Lenovo thinks that we can use the new way as the macro causing issues.

ASN.1 plan is endorsed

R2-2505457	Rel-19 LTE ASN.1 review plan	Samsung	discussion	Rel-19

For the LTE ASN.1 review, do not split the review file into smaller files i.e. one LTE ASN.1 review file is used.

For the LTE ASN.1 review, same template used for NR ASN.1 review is used i.e. reduce to WI and RIL number and include a pointer to a comment in a table.

ASN.1 principle/guideline and ASN.1 review plan is common for LTE and NR ASN.1 review.

LS from SA4

R2-2505068	LS on the RAN simulation assumptions for ULBC (S4-251584; contact: Qualcomm)	SA4	LS in	Rel-20	FS_ULBC	To:RAN1, RAN2, RAN4, SA2, CT1	Cc:SA1

- -	Qualcomm thinks that we can try to respond with a simple answer on header sizes.

-	Vivo thinks we should wait as there will be discussions.  Samsung thinks that the header sizes will not change so we can at least respond on that.

-	ZTE thinks that in theory we can provide an answer like, if we go with CP vs. UP what would be the header size without deciding.

-	Vivo, Lenovo, Huawei and Xiaomi think that there are a lot of aspects to consider.  Lenovo doesn’t think it is urgent to provide a precise answer.

Noted

We will begin this work in RAN2#131bis and will attempt to answer to SA4 after some initial RAN2 work.

R2-2505077	Discussion on SA4 LS regarding RAN Simulation Assumptions for ULBC	vivo	discussion	Rel-20	FS_ULBC

R2-2505428	Reply LS on the RAN simulation assumptions for ULBC	Qualcomm Technologies Ireland	LS out	Rel-20	FS_ULBC	SA4	SA2, CT1, RAN1

R2-2505601	Response to SA4 LS on the RAN simulation assumptions for ULBC	ZTE Corporation, Sanechips	discussion	Rel-20	FS_ULBC

R2-2506125	Draft Reply LS on the RAN Simulation Assumptions for ULBC	vivo	LS out	Rel-20	FS_ULBC	SA4, SA2	RAN1, CT1

## 8.1	AI/ML for NR air interface

(NR_AIML_air-Core; leading WG: RAN1; REL-19; WID: RP-250792 and SID: RP-243245)

Time budget: 2.5 TU

Tdoc Limitation: 3 tdocs

### 8.1.1	Organizational

LS, Rapporteur input, including workplan.

Including outcome of [POST130][022][AI PHY] 38.300 Running CR (Vivo), [POST130][023][AI PHY] 38.305 Running CR (CATT), 	[POST130][025][AI PHY] 37.355 Running CR (Qualcomm), [POST130][026][AI PHY] 38.331 Running CR (Ericsson), and [POST130][038][AI PHY] UE capabilities (Xiaomi)

- Agreements

The AI/ML for PHY WI is considered complete from RAN2 point of view

LS

RAN2 in “To”

R2-2505029	LS on Rel-19 AI/ML positioning higher layer parameters list Post RAN1#121 (R1-2505073; contact: Ericsson)	RAN1	LS in	Rel-19	NR_AIML_air	To:RAN2, RAN3	Cc:RAN4

Noted

R2-2505036	Reply LS on OAM-centric solution for NW-side data collection (S5-252842; contact: Huawei)	SA5	LS in	Rel-19	AIML_MGT_Ph2	To:RAN2	Cc:RAN3

Noted

R2-2505042	Reply LS on signalling feasibility of dataset and parameter sharing (R3-253961; contact: Xiaomi)	RAN3	LS in	Rel-19	NR_AIML_air-Core	To:RAN2, RAN1	Cc:SA2, SA5, SA3

Noted

R2-2505043	Reply LS on OAM-centric solution for NW-side data collection (R3-253962; contact: ZTE)	RAN3	LS in	Rel-19	NR_AIML_air-Core	To:RAN2	Cc:SA5

Noted

RAN4 LS

R2-2505045	LS on AI/ML functionality activation (R4-2508085; contact: CMCC)	RAN4	LS in	Rel-19	NR_AIML_air-Core	To:RAN2	Cc:RAN1

Interpretation 1: the applicable functionalities are already activated (i.e., the applicable functionalities activation is completed) before reporting applicable functionalities via RRCReconfigurationComplete in step 4.

Interpretation 2: UE starts to activate the applicable functionalities (i.e., the applicable functionalities activation is not completed) upon reporting applicable functionalities via RRCReconfigurationComplete in step 4.

Noted

Interpretation 1 vs. 2

R2-2505510	Discussion and draft reply LS on functionality activation	CMCC,OPPO	discussion	Rel-19	NR_AIML_air-Core

Proposal 1: RAN2 confirm that Interpretation 1 is common understanding for AI/ML functionality activation for option A, and update the wording as follows:.

	Interpretation 1: the applicable functionalities are already considered activated (i.e., the applicable functionalities activation is completed) before upon reporting applicable functionalities via RRCReconfigurationComplete in step 4

Noted

R2-2505192	Discussion on LS on functionality activation and open issues on BM	vivo	discussion	NR_AIML_air-Core

Proposal 1. Reply to RAN4 that interpretation 2 is aligned with RAN2 agreements. That is, UE starts to activate the applicable functionalities (i.e., the applicable functionalities activation is not completed) upon reporting applicable functionalities via RRCReconfigurationComplete.

Noted

Discussion

-	ZTE doesn’t think that RRC reconfiguration should be coupled with model loading and handling, so RAN4 has the wrong

-	Xiaomi thinks that key point from RAN2 point of view is UE actions and when the UE is ready to apply inference.   When we respond applicable it means the UE is ready, when it is not ready it will not respond applicable.  How long the UE it takes to load the model it is up to UE implementation.  The time needed should be define in RAN4.  LG shares the same view as Xiaomi.

-	Qualcomm thinks interpretation 1 should be the right one.   For semi persistent reporting RAN1 has already addressed them, there is a time for activation.

-	MEdiatek thinks that time to load the model is implementation and it is in addition to other configuration loading.  We need to understand when it should and how long it takes and whether it is in RRC processing or activation time.

-	Nokia agrees we should decouple applicability reporting and inference and shouldn’t couple it with RRC processing delay.

-	Ericsson agrees with interpretation 1 and we should ensure there is no ambiguity in the network whether the UE is transmitting something or not.

-	Oppo thinks that we only care about RRC processing delay and more than that it is RAN1 scope.

-	Apple prefers interpretation 1.   RAN4 cares about CSI reporting requirement and they mix the concepts.   We should decouple the RRC processing and CSI reporting.

-	Clarify the agreement and tell RAN4 that RRC processing delay should be impacted by model loading.

-	Samsung agrees with interpretation 1.  If the UE is not ready by RRC processing delay it just tells the network the model is not applicable, but up to RAN4 how they define processing delays to enable the models.

-	Huawei thinks we should go with interpretation 2.

-	Nokia indicates that for CSI reporting if the UE reports non-applicable then the network removes the configuration.  Apple understands that the UE can report non-applicable without the release indication then the network shouldn’t release.   The time of loading can be very large so it is not possible to delay the message.

R2-2506456	[Draft] Reply LS on AI/ML functionality activation	Apple	LS out	Rel-19	NR_AIML_Air-Core	To:RAN4	Cc:RAN1

RRC Open Issues: Issues related to RAN4 [Offline after treating LS]

R2-2506455	Summary report of [AT131][003][AI PHY] Functionality activation (Apple)	Apple	discussion	Rel-19	NR_AIML_Air-Core

Noted

Need discussion

After offline discussion, a small group of companies further discuss the issue when UE submits inference configuration of periodic CSI to lower layer. The following two options are identified:

•	Option 1: Upon reception of RRC Reconfiguration message, UE immediately submits inference configuration of periodic CSI to lower layer.

•	Option 2: Upon reception of RRC Reconfiguration message, UE holds on submitting inference configuration of periodic CSI to lower layer upon reception of RRC Reconfiguration message until applicable.

-	Samsung and Xiaomi, CMCC, Vivo,  think that we should follow option 2 as RAN1 relies on getting the applicable configuration.   Nokia and Ericsson thinks that we should follow legacy mechanism.  Qualcomm thinks we should ask RAN1 if they see any issue.   Apple thinks it is up to UE implementations.   ZTE also thinks that we should follow legacy.

-	Oppo thinks that both options work but if we go with option 1 we will introduce RAN1 impacts and they have close their work items.   Lenovo thinks we should go with legacy and RAN1 can easily fix it.

-	LG and Huawei thinks we should go with option 2.   Huawei thinks that it is clear from LS that RRC should handle the configuration to the UE.

-	Samsung asks what is the problem if we go with option 2 as the ambiguity period also exists in legacy.   If we go with option 1 the lower layer will send garbage CSI report.

Companies don’t have consensus on option 1 vs option 2. Some companies think that RAN1 need to fix their issue (e.g. UCI multiplexing during ambiguity time) while some companies don’t think any issue. Some company want to ask what is RAN1’s understanding on how UE handles CSI-ReportConfig when CSI-ReportConfig for inference configuration is not applicable. But some companies think current RAN2 spec is not broken.

Given the situation, Rapporteur suggest to only not to include RAN1 question in the LS to RAN4.

Proposal 6: As no consensus on specific question to RAN1, Reply LS to RAN4 (RAN4) only includes response to RAN4 question (i.e. no specific action to RAN1).

Agreements

- On the time duration for an AI functionality to become available for inference, RAN2 conclude that it is up to UE implementation from RAN2 point of view and no further RAN2 work.

- RRCReconfigurationComplete containing applicability reports has a processing latency requirement of 16 ms with respect to the reception of RRCReconfiguration, from RAN2 point of view.

- RRC processing delay shouldn’t be impacted by the model loading delay

- If the UE is ready for inference by end of RRC processing delay, it reports model applicable.  If not, it reports model inapplicable and doesn’t set the release flag.   The network is not expected to release inference configuration (this will not be added to stage 3 specifcation).

- Once the model is applicable, UE reports applicability to network via UAI (applicable to all CSI reporting).

- Respond to RAN4

- On the time duration for an AI functionality to become available for inference, RAN2 conclude that it is up to UE implementation from RAN2 point of view and no further RAN2 work.

- RRCReconfigurationComplete containing applicability reports has a processing latency requirement of 16 ms with respect to the reception of RRCReconfiguration, from RAN2 point of view.

- LS to RAN1 - RAN2 has identified a problem.  From RAN2 point of view this can be solved by option 2, but needs to check with RAN1.   RAN2 also discussed option 1 and couldn’t conclude as it is outside scope of RAN2.  Would like too ask RAN1 which one is best.

- Current RAN2 specifications will not be updated to cover this problem for now

- In RAN4 LS, RAN2 will not mention interpretation but just provide agreement 1 - 6

[POST131][003][AI PHY] Functionality activation (Apple)

Intended outcome: LS to RAN4  and LS to RAN1

Deadline:  Thursday

=> Approved in:

R2-2506519: LS to RAN1

R2-2506520: LS to RAN4

Not treated

RRC-15: The time duration for an AI functionality to become available for inference after UE reports applicability

R2-2505470	Further Discussion on the Remaining RRC Issues on LCM	MediaTek Inc.	discussion

Proposal 6 (RRC-17): It is clarified that loading/preparation time for the model corresponding to the applicable configuration is not considered as a component in the processing latency between the reception of RRCReconfiguration and the reporting of RRCReconfigurationComplete.

R2-2505502	Remaining issues on LCM procedure of UE-sided model for AI/ML based beam management	Apple	discussion	Rel-19	NR_AIML_air-Core

Proposal 3 (Open issue RRC-15): On the time duration for an AI functionality to become available for inference, RAN2 conclude that it is up to UE implementation and no need of UE reporting. Whether to specify its requirement is left to RAN4.

RRC-17: Processing timing requirement of applicability/inapplicability report via RRCReconfigurationComplete

R2-2505778	RRC open issues for AIML for NR air interface	Ericsson	discussion (Moved from 8.1.1)

to determine the applicability.

Proposal 7: (RRC-17) RRCReconfigurationComplete containing applicability reports has a processing latency requirement of 16 ms with respect to the reception of RRCReconfiguration. FFS whether RAN4 input is needed. FFS whether this solves open issue RRC-15.

R2-2505301	Discussion on life cycle management open issues	Xiaomi	discussion	Rel-19	NR_AIML_air-Core

Proposal 3: (RRC-17) Introduce multiple RRC processing delay requirements for applicability reporting based on UE capability. Values of RRC processing delay requirement is up to RAN4.

RRC-45: How/where to capture activation of periodic inference CSI-ReportConfig in specifications

R2-2505502	Remaining issues on LCM procedure of UE-sided model for AI/ML based beam management	Apple	discussion	Rel-19	NR_AIML_air-Core

Proposal 7 (Open issue RRC-45): As no concept of “activation/deactivation of periodic CSI reporting” in RAN1, adopt the way of “submit/do not submit the configuration to lower layers” in RRC running CR.

R2-2505345	Remaining issues in LCM for BM and CSI prediction	Samsung	discussion	Rel-19	NR_AIML_air-Core

Proposal 4: (RRC-45) Handling of periodic CSI-ReportConfig is described in Section 5.3.5.3 when setting the content of RRCReconfigurationComplete.

RAN2 in “CC” (to be noted)

R2-2505034	Reply LS on per-UE UE performance metrics (R3-253816; contact: Nokia)	RAN3	LS in	Rel-18	NR_AIML_NGRAN-Core	To:SA5	Cc:RAN2

Noted

R2-2505057	Reply LS on AI/ML UE sided data collection (S2-2505713; contact: vivo, Samsung)	SA2	LS in	Rel-20	NR_AIML_air, FS_AIML_CN_Ph2	To:RAN	Cc:SA, SA3, SA5, RAN1, RAN2

Noted

Running CRs

37.320

R2-2506078	Introduction of AI for Air interface feature in TS 37.320	Huawei, HiSilicon, Ericsson, Nokia	CR	Rel-19	37.320	18.4.0	0143	-	B	NR_AIML_air-Core

Endorsed and will be used as baseline for further discussion

[POST131][023][AI PHY] 37.320 (Huawei)

Intended outcome: Agree to final CR

Deadline:  short

=> Agreed in R2-2506586 (37.320)

R2-2506586	Introduction of AI for Air interface feature in TS 37.320	Huawei, HiSilicon, Ericsson, Nokia	CR	Rel-19	37.320	18.4.0	0143	3	B	NR_AIML_air-Core

=> Agreed

37.355

R2-2505703	Summary of [POST130][025][AI PHY] 37.355 CR (Qualcomm)	Qualcomm Incorporated (Rapporteur)	discussion	NR_AIML_air-Core

Noted

R2-2505704	Introduction of AI/ML Positioning Accuracy Enhancements	Qualcomm Incorporated (Rapporteur)	CR	Rel-19	37.355	18.5.0	0559	-	B	NR_AIML_air-Core

Endorsed and will be used as baseline for further discussion

[POST131][024][AI PHY] 37.355 (Qualcomm)

Intended outcome: Agree to final CR and then trigger LS to RAN1 on questions related to LPP21

Deadline:  short for CR and 2 weeks after for LS

=> Agreed in R2-2506584 (37.355)

=> Approved in R2-2506658 (LSout)

R2-2506584	Introduction of AI/ML Positioning Accuracy Enhancements	Qualcomm Incorporated (Rapporteur)	CR	Rel-19	37.355	18.5.0	0559	2	B	NR_AIML_air-Core

=> Agreed

R2-2506658	LS on the implicit indication of TRP location coordinates via "Associated ID"	RAN2	LS out	Rel-29	NR_AIML_air-Core	To:RAN1

=> Approved

38.300

R2-2505191	Introduction of AI for Air interface feature in 38300	vivo(Rapporteur)	CR	Rel-19	38.300	18.6.0	1006	-	B	NR_AIML_air-Core

- -	Qualcomm thinks that we shouldn’t use the word model in the stage 2 and we should align with RAN1 specification.   ZTE doesn’t think this is an issue.  Ericsson also agrees with comment from Qualcomm.

Endorsed and will be used as baseline for further discussion

[POST131][025][AI PHY] 38.300 (Vivo)

Intended outcome: Agree to final CR

Deadline:  short

=> Agreed in R2-2506498

R2-2506498	Introduction of AI for Air interface feature in TS38.300	vivo	CR	Rel-19	38.300	18.6.0	1006	1	B	NR_AIML_air-Core

=> Agreed

38.305

R2-2505212	38.305 running CR for AIML Positioning	CATT	CR	Rel-19	38.305	18.6.0	0190	-	B	NR_AIML_air-Core

Endorsed and will be used as baseline for further discussion

[POST131][026][AI PHY] 38.305 (CATT)

Intended outcome: Agree to final CR

Deadline:  short

=> Agreed in R2-2506617

R2-2506617	Introduction of AIML Positioning	CATT	CR	Rel-19	38.305	18.6.0	0190	2	B	NR_AIML_air-Core

=> Agreed

38.321

R2-2505501	Running MAC CR for AI/ML for Air Interface	Apple (Rapporteur)	CR	Rel-19	38.321	18.6.0	2104	-	B	NR_AIML_air-Core

Endorsed and will be used as baseline for further discussion

[POST131][028][AI PHY] 38.321 (Apple)

Intended outcome: Agree to final CR

Deadline:  short

=> Agreed in R2-2506562

R2-2506448	Report of [AT131][028][AI PHY] UE capabilities (Xiaomi)	Xiaomi	discussion	Rel-19	NR_AIML_air-Core

R2-2506562	Introduction of AI/ML for Air Interface to TS 38.321	Apple (Rapporteur)	CR	Rel-19	38.321	18.6.0	2104	2	B	NR_AIML_air-Core

=> Agreed

38.331

R2-2505777	Introduction of AIML for NR air interface	Ericsson	CR	Rel-19	38.331	18.6.0	5437	-	B	NR_AIML_air-Core

=> Revised in R2-2506401

R2-2506401	Introduction of AIML for NR air interface	Ericsson	CR	Rel-19	38.331	18.6.0	5437	1	B	NR_AIML_air-Core

Endorsed and will be used as baseline for further discussion

[POST131][027][AI PHY] 38.331 (Ericsson)

Intended outcome: Agree to final CR

Deadline:  short

=> Agreed in R2-2506530

R2-2506530	Introduction of AIML for NR air interface	Ericsson	CR	Rel-19	38.331	18.6.0	5437	2	B	NR_AIML_air-Core

=> Agreed, but then coversheet revised by MCC: duplicated meeting number

R2-2506638	Introduction of AIML for NR air interface	Ericsson	CR	Rel-19	38.331	18.6.0	5437	3	B	NR_AIML_air-Core

=> Agreed

UE capabilities

R2-2505298	Report of [POST130][038][AI PHY] UE capabilities (Xiaomi)	Xiaomi	discussion	Rel-19	NR_AIML_air-Core

Proposal 2 FFS on whether UE can support other memory sizes and indicate to network via optional capability signaling.

-	Qualcomm thinks that if the UE supports more it is up to the UE and doesn’t need to report anything.  Nokia agrees.   Huawei thinks that if there is no indication then the threshold will be set to the minimum value.

UAI

-	ZTE thinks UAI should be mandatory for both option B.   Apple thinks that both for option A and B this should be mandatory.

Agreements - If UE supports NW side data collection, it is mandatory for UE to support the minimum AS layer memory size of 64kB for UE supporting AI/ML based beam management, which is shared across all use cases with NW-sided model. The assumption is that this will be shared across all use cases (i.e. Rel-20 as well) - FFS on whether UE can support other memory sizes and indicate to network via optional capability signaling. - Include RAN2 feature ‘UE can provide update of applicability reporting via UAI’ as part of RAN1 FGs (e.g., 58-0-1 and/or FG 58-1-2/3/4/5, the details of those feature group depend on RAN1 progress) once implemented. - Introduce two conditional mandatory capabilities (with signaling) for AI/ML based BM Option A and Option B, if UE supports FG58-0-1 and/or FG58-1-2/3/4/5 (the details of those feature group depend on RAN1 progress). - Include RAN2 feature ‘providing UE preferred configuration for UE-side data collection’ as part of RAN1 FG58-1-7/FG58-3-4 (once implemented). - UAI is mandatory for both Option A and B - Introduce an optional per UE capability ‘loggedDataCollection-r19’ to indicate supporting logged measurements of data collection for NW-side model, which includes the following components: - the minimum 64kB AS layer memory size - periodic logging - Provide full buffer indication, low power indication - Event-based logging is an optional per UE capability separate from ‘loggedDataCollection-r19’. UE supporting this feature shall also indicate the support of ‘loggedDataCollection-r19’. If UE supports event-based logging it shall support data threshold-based data availability indication. - RAN2 will not introduce separate CSI resource capability for logged NW-side data collection. Legacy capability will be used for logged NW-side data collection. Check with RAN1 on whether this assumption is ok. - Data threshold-based data availability indication is an optional per UE capability with signaling. A UE supporting this feature shall also indicate support of the basic logged NW-side data collection.

[POST131][043][AI PHY] UE capabilities (Xiaomi)

Intended outcome: align on remaining open issues and agreable proposals

Deadline:  Thursday

=> Endorsed in:

R2-2506524 (38.306)

- R2-2506525 (38.331)

R2-2506524	Introduction of AI air UE capability	Xiaomi, OPPO	draftCR	Rel-19	38.306	18.6.0	B	NR_AIML_air-Core

=> Endorsed

R2-2506525	Introduction of AI air UE capability	Xiaomi, OPPO	draftCR	Rel-19	38.331	18.6.0	B	NR_AIML_air-Core

=> Endorsed

R2-2506448	Report of [AT131][028][AI PHY] UE capabilities (Xiaomi)	discussion	Rel-19	NR_AIML_air-Core

To be discussed

Proposal 3: (7 proponents, 4 opponents) Data threshold-based data availability indication is an optional per UE capability with signaling. A UE supporting this feature shall also indicate support of the basic logged NW-side data collection.

Proposal 5: (7 proponents, 4 opponents) UE can support other memory sizes and indicate this to network via an optional capability signaling.

Noted

R2-2505299	Introduction of AI air UE capability	Xiaomi, Oppo	draftCR	Rel-19	38.306	18.6.0	B	NR_AIML_air-Core

Endorsed and will be used as baseline for further discussion

R2-2505300	Introduction of AI air UE capability	Oppo, Xiaomi	draftCR	Rel-19	38.331	18.6.0	B	NR_AIML_air-Core

Endorsed and will be used as baseline for further discussion

### 8.1.2	Functionality based LCM

Contributions should focus on general understanding of LCM procedure (except for data collection transfer (i.e. study item part) and model transfer/delivery), what is required to enable the UE to perform different steps of the LCM procedure, what is the granularity of functionality, dependencies with RAN1 and what is needed from RAN1 to progress in RAN2

Contributions should be submitted in 8.1.2.x and aspects related to data collections should be submitted in data collection section

Two-sided model discussions are out of scope of this AI

#### 8.1.2.1	LCM for NW-sided model for Beam Management use case

LCM related to NW-sided model for beam management use case.

No contributions expected for this meeting.

#### 8.1.2.2	LCM for UE-sided model for Beam Management use case

Including functionality identification, additional conditions and further reporting of applicable functionalities, and any necessary signaling/protocol aspects including, optiona A and Option B (except RRC parameter details in OtherCongif). Contributions should to take into consideration the reply LS from RAN1 (R1-2410898) on BM applicable functionality reporting and other RAN1 agreements.

Contributions can discuss aspects of LCM that are significantly different from BM for CSI prediction use case.

Including outcome of [POST130][037][AI PHY] UE candidate data collection (Xiaomi_Ericsson)

Email discussion: UE candidate data collection [Tuesday]

Reporting aspects

R2-2505296	Report of [POST130][037][AI PHY] UE candidate data collection (XiaomiEricsson)	Xiaomi, Ericsson	discussion	Rel-19	NR_AIML_air-Core

Noted

Data collection start/stop indication

R2-2505296	Report of [POST130][037][AI PHY] UE candidate data collection (XiaomiEricsson)	Xiaomi, Ericsson	discussion	Rel-19	NR_AIML_air-Core

Proposal 3: Besides ‘start/stop indication’, DataCollectionPreference indicated by UE via UAI also includes a list of ID(s) representing its preferred configuration(s).

R2-2505912	On Unifying and Simplifying ASN.1 and Procedures for AI/ML	Nokia, Qualcomm, Ericsson, T-Mobile USA Inc.	discussion	Rel-19	NR_AIML_air-Core

Proposal 11: The UE will report its preference for a candidate for UE-side data collection by sending a UEAssistanceInformation message. FFS how to signal, e.g., 1) UE indicates list of preferred configuration(s), wherein the presence indicates “start”, and the absence indicates “stop” 2) UE indicates a start list and a stop list, each containing a list of zero or more data collection candidate configurations or 3) UE indicates a list of data collection candidate configurations, each with a corresponding “start” or “stop” indication.

Noted

R2-2505345	Remaining issues in LCM for BM and CSI prediction	Samsung	discussion	Rel-19	NR_AIML_air-Core

Proposal 1: (RRC-03) Introduce configuration ID lists UE prefers to stop for UE-side data collection in UAI message. UE can include both 1) configuration ID lists it prefers to start and 2) configuration ID lists it prefers to stop together in a common UAI message.

Noted

R2-2505838	LCM for UE-side models for beam management	Ericsson	discussion

Proposal 1: (RRC-3) The UE does not request to start data collection for candidate configurations for which a corresponding UE data collection configuration is already configured via CSI-ReportConfig.

Noted

R2-2505199	Open Issues on LCM for UE-sided Models	Qualcomm Incorporated 	discussion	Rel-19

Proposal 1: To reduce signaling overhead, the UE should indicate preferred data collection configurations without a START / STOP indication.

Discussions

-	Qualcomm thinks that the UE should be able to indicate which configuration it wants to starts.

-	Huawei and Nokia thinks that if the UE says start then it should measure.   When the UE stops measuring it should indicate stop.   Huawei indicates that the UE may measure multiple configuration and it should be able to indicate which ones it wants to stop.

-	Apple thinks that the indication it applies to all configurations as if it wants to stop it is related to power issues or other issues.

Configuration aspects

R2-2505296	Report of [POST130][037][AI PHY] UE candidate data collection (XiaomiEricsson)	Xiaomi, Ericsson	discussion	Rel-19	NR_AIML_air-Core

Proposal 2: Adopt below text in the field description of dataCollectionCandidateConfig:

The UE is not expected to perform measurements solely based on the configurations provided by this IE.

Proposal 5: Solution 1 is adopted for providing candidate UE data collection configurations.

Solution 1: OtherConfig contains a list of candidate configurations as a list of a new IE, where each candidate configuration contains at least an identifier of the candidate configuration, CSI-ResourceConfigId for Set A, CSI-ResourceConfigId for Set B, and related associated IDs, as agreed in RAN2#130. Each candidate configuration is associated with a cell ID.

Proposal 5-1: Include TP in R2-2505297 in the RRC running CR of AI PHY.

Proposal 5-2: Send RAN1 LS about RAN2 agreement on candidate UE-side data collection configurations.

-	Nokia thinks we need to include the CSI report config IE.  The overhead is not that high as we aren’t providing many configuration.

-	ZTE is concerned with CSI-ResourceConfigId as this may have RAN4 specification impact.   Since the UE doesn’t need to measure there is not RAN4 impact.

-	Ericsson agrees with Nokia and in the offline companies agreed that we could use it for CSI prediction.   We should adopt the same approach for both.  Vivo thinks that if we do it in CSI report config we have to involve RAN1.   Qualcomm also agrees with Ericsson.

-	Oppo thinks that there only a subset of parameters needed but in CSI report config there are a lot and that causes overhead.

-	Apple thinks that it is more than just overhead, there are many mandatory IEs not related to inference.  And we need to identify spec work for UEs to ignore IEs.    RAN1 has discussed this issues and it is not related to RAN4.

Noted

R2-2505297	Text proposal of UE candidate data collection configuration	Xiaomi, Ericsson	discussion	Rel-19	NR_AIML_air-Core

Noted

Agreements on UE candidate data collection 1 Multiple preferred configurations within the list of candidate configurations provided by NW can be indicated by the UE via UAI. 2 No prohibit timer is needed for UE indicating its preferred data collection configuration. 3 On stop/start indication - The UE can send start indication (without a preferred list) to indicate preference to start data collection - The UE can send preferred list implying that it would like to start data collection on those configuration - The UE can send stop indication for all or a given actual CSI report config ID. - Rapporteur will determine best way of signaling. This doesn’t preclude merging 1 and 2. - Adopt below text in the field description of dataCollectionCandidateConfig: The UE is not expected to perform measurements solely based on the configurations provided by this IE. - Adopt the following solution: OtherConfig contains a list of candidate configurations as a list of a new IE, where each candidate configuration contains at least an identifier of the candidate configuration, CSI-ResourceConfigId for Set A, CSI-ResourceConfigId for Set B, and related associated IDs, as agreed in RAN2#130. Each candidate configuration is associated with a cell ID. We will also include individual IEs for CSI prediction case. - Ask RAN1 what IEs are needed for CSI prediction and inform them of our agreements on BM and confirm if anything else is needed.

[AT131][029][AI PHY] LS to RAN1 (Xiaomi)

Intended outcome:   to be approved by email

Deadline:  Thursday

R2-2506470	LS on candidate data collection	RAN2	LS out	Rel-19	NR_AIML_air-Core	To:RAN1	Cc:RAN4

The LS is approved

RRC Open Issues: Issues with Rapporteur proposal not addressed by contributions [Tuesday]

R2-2505778	RRC open issues for AIML for NR air interface	Ericsson	discussion (Moved from 8.1.1)

Proposal 1: (RRC-9)

Update the definition of ‘applicable AI/ML functionality’ in RRC clause 3.1 as: “Applicable AIconfiguration: Configuration according to which an AI/ML functionality is determined to be applicable by the UE, as defined in TS 38.300 [2].”

RRC Open Issues: Almost agreeable [Offline]

[AT131][002][AI PHY] LCM BM OI (Ericsson)

Intended outcome: agreeable proposals on some open issues

Deadline:  Wednesday

R2-2506414	Summary of offline discussion [AT131][002][AI PHY] LCM BM OI (Ericsson)	Ericsson	discussion	Rel-19	NR_AIML_air-Core

Proposal 1	(RRC-13) (all companies agree) The procedures for UE data collection for the CSI prediction use case are the same as for the beam management use case.

Proposal 2	(RRC-13) (1 company disagrees) The candidate UE data collection configuration for CSI prediction is a list of CSI-ReportConfig in otherConfig. To be checked with RAN1.

Proposal 3	(RRC-13) (1 company disagrees) The applicability reporting procedures for CSI prediction are the same as for beam management. RAN2 confirms that option B is not supported for CSI prediction.

-	Huawei would like to ask RAN1 about option B.  Companies think that if they want it they can provide parameters to RAN2.

Proposal 4	(RRC-14) (3 companies disagree) RAN2 understand that when network releases inference configurations of poor-performed applicable functionalities, network may also provide either non-AI/ML configuration in CSI-ReportConfig or may provide full inference configuration of other applicable functionalities, if previously not configured to UE. There is no spec impact and feedback from the NW to the UE to adjust the applicability determination procedure is not supported in Rel-19.

Noted

RRC-13: CSI prediction LCM framework

R2-2505778	RRC open issues for AIML for NR air interface	Ericsson	discussion (Moved from 8.1.1)

Proposal 2: (RRC-13) The procedures for UE data collection for the CSI prediction use case are the same as for the beam management use case (start/stop request from UE, candidate configuration provision from NW, request for preferred candidate configurations from UE, etc.).

Proposal 3: (RRC-13) The candidate UE data collection configuration for CSI prediction has the same content as for BM, except that associated IDs and differentiation between Set A/B are not needed.

Proposal 4: (RRC-13) The applicability reporting procedures for CSI prediction are the same as for beam management. RAN2 to confirm that option B is not supported for CSI prediction.

R2-2505076	Delta Part for CSI Prediction	OPPO	discussion	Rel-19	NR_AIML_air-Core

Proposal 1: (RRC-13) For applicability check, both Option A and Option B are supported for CSI prediction if this comes for free compared to BM use cases.

RRC-14: Repeated reports of applicability for configurations which consistently perform poorly

R2-2505778	RRC open issues for AIML for NR air interface	Ericsson	discussion (Moved from 8.1.1)

Proposal 5: (RRC-14) Feedback from the NW to the UE to adjust the applicability determination procedure is not supported in Rel-19.

R2-2505199	Open Issues on LCM for UE-sided Models	Qualcomm Incorporated 	discussion	Rel-19

Proposal 7: The network can indicate whether the deactivation signal is due to poor inference performance, which will be useful to the UE side for making retraining / fine-tuning decisions.

R2-2505301	Discussion on life cycle management open issues	Xiaomi	discussion	Rel-19	NR_AIML_air-Core

Proposal 1: (RRC-14) When network releases inference configurations of poor-performed applicable functionalities, network should also provide either non-AI/ML configuration in CSI-ReportConfig or provides full inference configuration of other applicable functionalities, if previously not configured to UE

Agreements

- The procedures for UE data collection request for the CSI prediction use case are the same as for the beam management use case.

- The applicability reporting procedures for CSI prediction are the same as for beam management. RAN2 confirms that option B is not supported for CSI prediction, given no parameters were provided by RAN1.

- RAN2 understand that when network releases inference configurations of poor-performed applicable functionalities, network may also provide either non-AI/ML configuration in CSI-ReportConfig or may provide full inference configuration of other applicable functionalities, if previously not configured to UE. There is no spec impact and feedback from the NW to the UE to adjust the applicability determination procedure is not supported in Rel-19.

- No enhancements are pursued for reporting applicability in RRCReestablishmentComplete.

- The UE can report applicability via RRCResumeComplete for SCG inference configurations received in RRCResume, without specification impact beyond already agreed applicability reporting procedure.

- Applicability reporting is added in RRCResumeComplete for inference configurations that exist at the UE based on legacy procedures (restored or received in RRCResume).

- RAN2 assumes for no NR DC enhancements are considered and will not ask RAN3 work in Rel-19.

RRC-42a: Whether applicability reporting comes for free in RRCReestablishmentComplete

R2-2505778	RRC open issues for AIML for NR air interface	Ericsson	discussion (Moved from 8.1.1)

Proposal 10: (RRC-42) No enhancements are pursued for reporting applicability in RRCReestablishmentComplete.

R2-2505103	Discussion signaling aspects of LCM for UE-sided model for BM	Transsion Holdings	discussion

Proposal 1   Base on the legacy RRC connection re-establishment procedure, when the UE decides to perform RRC connection re-establishment, the UE can report applicable functionalities only for inference configurations through RRCReestablishmentComplete.

Noted

RRC-42b: Whether applicability reporting comes for free in RRCResumeComplete

R2-2505778	RRC open issues for AIML for NR air interface	Ericsson	discussion (Moved from 8.1.1)

Proposal 11: (RRC-42) The UE can report applicability via RRCResumeComplete for SCG inference configurations received in RRCResume, without specification impact beyond already agreed applicability reporting procedure.

Proposal 12: (RRC-42) Applicability reporting is added in RRCResumeComplete for inference configurations that exist at the UE based on legacy procedures (restored or received in RRCResume).

Noted

Question on whether we support DC

-	Lenovo explains that in RAN3 doesn’t have time.   Nokia thinks that there are some RAN3 areas that are essential.

R2-2506117	Continuous Discussion On LCM for UE-sided model	ZTE Corporation	discussion	Rel-19	NR_AIML_air-Core

Proposal 4: (RRC-42) If inference configuration with periodic CSI reporting is configured in RRCResume, the UE autonomously activate the applicable inference configuration upon reporting applicability status via RRCResumeComplete.

Proposal 5: (RRC-42) RAN2 to discuss and decide one of following options regarding activation of the restored inference configuration with periodic CSI reporting:

Option 1: To follow the activation status of the inference configuration as the UE is transited to RRC_INACTIVE. UE activates autonomously the inference configuration with periodic CSI reporting if it is activated before the UE is transited to RRC_INACTIVE.

Option 2: To activate the inference configuration according to the applicability status upon RRC resumption. The UE autonomously activates the applicable inference configuration with periodic CSI reporting upon reporting applicability status via RRCResumeComplete.

Noted

[After RRC-46 is resolved]

Not treated

RRC-41: Value range for associated ID

R2-2505778	RRC open issues for AIML for NR air interface	Ericsson	discussion (Moved from 8.1.1)

Proposal 9: (RRC-41) RAN2 to discuss the length of the associated ID (e.g. 7, 8, 16, 24 bits, etc.), after resolving (RRC-46).

R2-2505757	LCM for UE-sided model for Beam Management and CSI prediction use cases	Huawei, HiSilicon	discussion	Rel-19	NR_AIML_air-Core

Proposal 12: (RRC-41) As a starting point, the length of associated ID is 8 bits.

R2-2505994	Discussion on LCM for UE-side model for BM	NTT DOCOMO, INC.	discussion	Rel-19

Proposal 3: Considering multi-vendor operation and support both single-cell specific associated ID and multi-cell specific associated ID, RAN2 to support much enough value range for associated ID (e.g., 24 bit or more) to enable rough allocation to each vendor within a PLMN.

RRC Open Issues: Issues requiring discussion [for online discussion]

RRC-16: UE behaviour when the associated ID is not provided by the network

R2-2505778	RRC open issues for AIML for NR air interface	Ericsson	discussion (Moved from 8.1.1)

Proposal 6: (RRC-16) RAN2 to discuss the following options:

(a) If the network does not provide the associated ID, the UE reports the applicability (applicable/inapplicable) based on UE-side additional conditions (internally known by UE), model availability in device and the other provided information in the configuration. FFS if the UE reports associated IDs, if available.

(b) If the network does not provide the associated ID, it is up to UE implementation how to determine the applicability.

Noted

R2-2505074	Leftover Issue Discussion on LCM for UE-sided model for BM use case	OPPO	discussion	Rel-19	NR_AIML_air-Core

Proposal 3: (RRC-16) If associated ID is not provided by the network for applicability check, UE can optionally provide associated ID(s) to NW per full inference configuration for Option A or per inference related parameters for Option B.

Noted

R2-2505502	Remaining issues on LCM procedure of UE-sided model for AI/ML based beam management	Apple	discussion	Rel-19	NR_AIML_air-Core

Proposal 4 (Open issue RRC-16): If the network does not provide the associated ID, it is up to UE implementation how to determine the applicability.

Proposal 5 (Open issue RRC-16): RAN2 don’t pursue UE reporting associated ID, which is an optimization for an unreasonable NW configuration (i.e. NW provides associated ID in training configuration but not in inference configuration).

Noted

R2-2505192	Discussion on LS on functionality activation and open issues on BM	vivo	discussion	NR_AIML_air-Core

Proposal 6: (RRC-16) If the other conditions (e.g., UE-side additional conditions, model availability in device and the other provided information in the configuration) are not fulfilled, the UE feedbacks inapplicable regardless of whether associated ID is provided by the network or not.

Proposal 7: (RRC-16) If the other conditions are fulfilled and the associated ID is not provided by the network, the UE feedbacks applicable if there is at least one AI model of the functionality which is applicable to all the associated IDs.

Proposal 8: (RRC-16) If the other conditions are fulfilled, the associated ID is not provided by the network and there is no AI model of the functionality which is applicable to all the associated IDs, RAN2 to discuss the following 3 alternatives:

-	Alt1: The UE always feedbacks non-applicable.

-	Alt2: The UE feedbacks non-applicable (in order not to activate the functionality by the UE) with indicating the applicable associated ID(s). Then the network may reconfigure the inference configuration with one of the applicable associated ID(s).

-	Alt3: The UE feedbacks applicable and also activates the functionality. In this case, the network performs the management based on performance monitoring.

Noted

RRC-46: Whether the associated ID is cell specific or multi-cell specific

R2-2505524	Support of associated ID	Samsung, Apple, Xiaomi, Qualcomm, Google, Interdigital, NTT DOCOMO, Nokia, LG Electronics, vivo	discussion	Rel-19	NR_AIML_air-Core

Proposal 1: Both single cell and multi-cell associated ID can be supported based on NW implementation (i.e., the network may allocate an Associated ID to a single cell and/or to multiple cells).

Proposal 2: Associated IDs shall be unique within a PLMN in that they can only be associated with one same/similar beam deployment.

Noted

R2-2505838	LCM for UE-side models for beam management	Ericsson	discussion

Proposal 7: (RRC-46) RAN2 to confirm that the associated ID is consistent within a cell, i.e. the same values can be reused in other cells, where they can correspond to different beam configurations.

Noted

R2-2505192	Discussion on LS on functionality activation and open issues on BM	vivo	discussion	NR_AIML_air-Core

Proposal 12: (RRC-46) It should be RAN1, not RAN2 to determine whether the associated ID is cell specific or multi-cell specific. RAN2 can assume multi-cell specific associated ID so far to define the value range before RAN1 does not reach the conclusion.

Noted

[AT131][030][AI PHY] Multicell v.s Single cell associate ID (Samsung)

Intended outcome: Agreable proposal for issue 46 and 41

Deadline:  Thursday

R2-2506439 	Summary report of [AT131][030][AI PHY] Multicell v.s Single cell associate ID	Samsung	discussion

Noted

Support both cell specific ID and multiple-cell specific ID but detailed signaling is FFS

-	ZTE thinks that RAN1 should discuss.  Samsung indicates that RAN1 feature lead confirms that this is a RAN2 issue.

-	Ericsson has some concerns on the complexity of coordination and the problem wouldn’t even be solved.   Even if you keep associated ID but due to different propagation the model would still need to be trained.    Samsung thinks that one suggestion was an area rather than full PLMN.

-	Vivo explains that from UE perspective everytime we change a cell we need to reload the model.   And to alleviate the concern from network vendor we can talk about area.   CMCC thinks one compromise is to be tracking area as it is difficult to have same ID throughout all PLMN.

-	Interdigital thinks that if it cell specific we are significantly increasing load and if we make it multicell then you will have to dimension once and it shouldn’t change often.  If the network doesn’t want to do multicell it can chose to do single cell.   Apple has same view and from UE vendor there are some very strong concerns.

-	Ericsson thinks that we can have per PLMN and per gNB.

-	Docomo supports multicell.

Agreements on associated ID

- Both single cell and multi-cell associated ID can be supported based on NW implementation (i.e., the network may allocate an Associated ID to a single cell and/or to multiple cells).

- Associated IDs shall be unique within a PLMN in that they can only be associated with one same/similar beam deployment.   FFS is we should have signalling indicating multi-cell.

- We will not define areas.  The Associated ID is 24 bits.

4	If the network does not provide the associated ID, it is up to UE implementation how to determine the applicability.

RRC-40: Configuration for Option B for applicability reporting

R2-2505778	RRC open issues for AIML for NR air interface	Ericsson	discussion (Moved from 8.1.1)

Proposal 8: (RRC-40) RAN2 confirms that UE receives RRCReconfiguration message including one set or multiple sets of inference related parameters via OtherConfig for option B.

Noted

R2-2506011	Discussion on Applicable Functionality Reporting Option B for BM	OPPO, Lenovo, ZTE Corporation, Apple, Huawei, HiSilicon, vivo, CMCC, Samsung, LG Electronics, MediaTek Inc., InterDigital	discussion	Rel-19	NR_AIML_air-Core

Proposal 2: For Option B for BM Case 1/2, one set or multiple sets of inference related parameters can be configured in OtherConfig, where each set in OtherConfig contains the following parameters according to RAN1#121 agreement:

	associatedIDforSetA-r19, resourcesForSetA-r19, resourcesForChannelMeasurement, associatedIDforSetB-r19, reportQuantity-r19, reportConfigType, nrofreportedpredictedrs-r19, TimeGap-r19, nroftimeinstance-r19, applicabilityConfigId-r19.

Proposal 3: For Option B for BM Case 1/2, inference related parameter set is configured per serving cell.

Proposal 4: Take the draft TP in this contribution into account for stage 3 spec design.

Noted

R2-2505912	On Unifying and Simplifying ASN.1 and Procedures for AI/ML	Nokia, Qualcomm, Ericsson, T-Mobile USA Inc.	discussion	Rel-19	NR_AIML_air-Core

Proposa1: Use CSI-ReportConfig as the IE to configure the UE with sets of inference-related parameters, each CSI-ReportConfig representing one set of inference-related parameters.

Proposal 3: The CSI-ReportConfig configured in inferenceRelatedSetConfig-r19 is not to be passed to lower layers for configuration. That is, there shall be no RAN1 impact due to the reuse of CSI-ReportConfig.

Proposal 4: Define a new IE called InferenceRelatedSet-r19, which is configured with an identifier, inferenceRelatedSetId-r19 for use as a reference in the applicability report and a configuration containing a set of inference-related parameters, inferenceRelatedSetConfig-r19, which will minimally include CSI-ReportConfig as one of the choices.

Proposal 5: The field description for inferenceRelatedSetConfig-r19 will be used to specify which fields are relevant for the applicability determination using sets of inference-related parameters based on the type of configuration.

Proposal 6: Add inferenceRelatedSetId as a choice in applicabilityReportConfigId-r19 under ApplicabilityReportConfigIdList-r19 to support the reporting of the applicability of sets of inference-related parameters.

Proposal 7: Move applicabilityCellId-r19 to the ApplicabilityReportConfigIdList-r19.

Proposal 8: RAN2 to discuss adopting the ASN.1 definition of Figure 2.2-1 to simplify the reporting of applicability, reporting the applicability of configurations pertaining to any cell in the same flat list.

Proposal 9: RAN2 to discuss adopting a generic procedure to support the reporting of the applicability of sets of inference-related parameters as exemplified in the provided procedural text prototype.

Noted

Agreements Other Config for option B - RAN2 confirms that UE receives RRCReconfiguration message including one set or multiple sets of inference related parameters via OtherConfig for option B. - For Option B for BM Case 1/2, one set or multiple sets of inference related parameters can be configured in OtherConfig, where each set in OtherConfig contains the following parameters according to RAN1#121 agreement: associatedIDforSetA-r19, resourcesForSetA-r19, resourcesForChannelMeasurement, associatedIDforSetB-r19, reportQuantity-r19, reportConfigType, nrofreportedpredictedrs-r19, TimeGap-r19, nroftimeinstance-r19, applicabilityConfigId-r19. - For Option B for BM Case 1/2, inference related parameter set is configured per serving cell.

RRC-44: Whether RRCReconfigurationComplete contains the (in)applicability of all inference configurations

R2-2505686	Left issues for AI based BM	Lenovo	discussion	Rel-19

Proposal 3: The RRCReconfigurationComplete message only contains applicability information of the inference configuration or inference related parameter sets configured (i.e., add or modify) in the respective received RRCReconfiguration.

Noted

R2-2505880	Remaining Open Issues: LCM for UE-Sided AI/ML Beam Management	SHARP Corporation	discussion

Proposal 3 (RRC 44): RRCReconfigurationComplete shall include applicability/inapplicability status for:

a)	All inference configurations included in the immediately preceding RRCReconfiguration message, and

b)	Any previously configured inference configurations for which applicability/inapplicability has already been reported and whose applicability status has changed since the last report.

Proposal 4 (RRC 44): All further changes in applicability status shall be reported by the UE using UAI messages.

Noted

Agreements on applicability/inapplicability

1	RRCReconfigurationComplete shall include applicability/inapplicability status for:

a)	All inference configurations included in the immediately preceding RRCReconfiguration message, and

b)	Any previously configured inference configurations for which applicability/inapplicability has already been reported and whose applicability status has changed since the last report.   [CB on how the UE handles previously configured periodic CSI config that becomes applicable]

2	Do not introduce a link, explicit or implicit, between a full inference configuration and a set of inference-related parameters. If the applicability of a full inference configuration changes and there is a corresponding set of inference-related parameters whose applicability changes at the same time, the UE shall report the applicability of both

RRC-47: FFS how to implement RAN1 parameters in CSI-ReportConfig

R2-2505762	Remaining open issues: LCM for UE-sided model for BM use case	InterDigital	discussion	Rel-19	NR_AIML_air-Core

Proposal 3: [RRC-47] Group RAN1 parameters with a CHOICE structure (i.e., Option 2)

Noted

R2-2505911	LCM for UE-side Beam Management	Nokia	discussion	Rel-19	NR_AIML_air-Core

Proposal 4: (RRC-47) RAN2 to support ungrouping RAN1 parameters and benefit from using extension markers and adding new fields, new sets of parameters for inference, monitoring and data collection.

Noted

R2-2505345	Remaining issues in LCM for BM and CSI prediction	Samsung	discussion	Rel-19	NR_AIML_air-Core

Proposal 6: (RRC-47) a separate structure should be defined for BM and CSI prediction.

Proposal 7: (RRC-47) csi-InferencePrediction-r19 is defined out of configurationForChannelPrediction-r19.

Noted

Continue this on CR review phase

RRC-48: Applicability reporting for parameter sets after the corresponding full inference configuration is provided

R2-2505911	LCM for UE-side Beam Management	Nokia	discussion	Rel-19	NR_AIML_air-Core

Proposal 5: (RRC-48) Do not introduce a link, explicit or implicit, between a full inference configuration and a set of inference-related parameters. If the applicability of a full inference configuration changes and there is a corresponding set of inference-related parameters whose applicability changes at the same time, the UE shall report the applicability of both.

Noted

R2-2505762	Remaining open issues: LCM for UE-sided model for BM use case	InterDigital	discussion	Rel-19	NR_AIML_air-Core

Proposal 4: [RRC-48] In a subsequent applicability report, UE only includes one configuration ID (e.g., the full inference configuration ID) if both a full inference configuration ID and corresponding parameter set configuration ID are available.

Noted

R2-2505301	Discussion on life cycle management open issues	Xiaomi	discussion	Rel-19	NR_AIML_air-Core

Proposal 9: (RRC-48) It is up to UE implementation which ID (either ID of Option A/B or both IDs of Option A/B) is reported when both Option A and Option B are configured.

Noted

RRC Open Issues: Issues to be addressed directly in RRC CR [if time allows]

RRC-11: How to configure RS configuration for UE sided data collection within CSI-ReportConfig

R2-2505074	Leftover Issue Discussion on LCM for UE-sided model for BM use case	OPPO	discussion	Rel-19	NR_AIML_air-Core

Proposal 1: (RRC-11) Data collection resource configuration is configured in CSI-MeasConfig, which is in parallel with csi-ReportConfigToAddModList.

R2-2506096	LCM for UE-side models for beam management	TCL	discussion

Proposal 1: We suggest that RAN2 consider the RAN1-agreed none-BM-r19 in the CSI-ReportConfig to explicitly prevent an actual CSI report.

RRC-12: Monitoring for AI based beam management

R2-2506059	Discussion on open issues of LCM for UE-side models (BM use case)	HONOR	discussion	Rel-19	NR_AIML_air-Core

Proposal 9: (RRC-12) When UE indicates a periodic CSI-ReportConfig is not applicable, the gNB is expected to release the configuration and the linked monitoring report configuration.

Proposal 10: (RRC-12) UE continues to perform the monitoring and reporting until the configuration is released.

Proposal 11: (RRC-12) When gNB instructs UE to release the CSI-ReportConfig for inference configuration and linked monitoring configuration, only the CSI-ReportConfigId of inference report configuration is included in the csi-ReportConfigToReleaseList.

R2-2506096	LCM for UE-side models for beam management	TCL	discussion

Proposal 2: For performance monitoring, event-triggered reporting shall be supported

RRC Open Issues: Issues listed by Rapporteur as “Already resolved” [If time allows]

RRC-1: Cause of inapplicability

R2-2505239	Remaining issues on LCM for UE-sided model for BM	LG Electronics	discussion	Rel-19	NR_AIML_air-Core

Proposal 1. [RRC1] UE includes a release flag only when requesting configuration release without specifying an explicit cause, e.g., due to model unavailability

R2-2505757	LCM for UE-sided model for Beam Management and CSI prediction use cases	Huawei, HiSilicon	discussion	Rel-19	NR_AIML_air-Core

Proposal 1: (RRC-1) The release indication is only in initial inapplicability report via RRCReconfigurationComplete message (i.e. add a condition that the inapplicabilityCause-r19 is only for RRCReconfigurationComplete message).

Proposal 2: (RRC-1) Specify the usage of the “release configuration” flag clearly in the field description (i.e. removing the “e.g.” in the running CR).

R2-2505880	Remaining Open Issues: LCM for UE-Sided AI/ML Beam Management	SHARP Corporation	discussion

Proposal 5 (RRC 1 & 44): The UE shall report an explicit cause value for inapplicability whenever applicability/inapplicability status is reported, both in the initial RRCReconfigurationComplete message and in subsequent UAI messages.

Proposal 6 (RRC 1 & 44): The network may request the cause for inapplicability from the UE when needed (for e.g., in RRCReconfiguration message).

RRC-4: Activation of a periodic CSI report configuration upon change from inapplicable to applicable

R2-2505470	Further Discussion on the Remaining RRC Issues on LCM	MediaTek Inc.	discussion

Proposal 1: (RRC-4) The UE is not required to perform applicability determine procedure for the functionalities/configurations that have already been indicated as inapplicable to the network.

R2-2505712	Discussion on LCM for UE-sided model for Beam Management	Spreadtrum, UNISOC	discussion	Rel-19

Proposal 2: For RRC4, if periodic CSI report configuration is not released and changes from inapplicable to applicable, it can be activated autonomously upon reporting its applicability via UAI.

RRC-8: Coexistence between option A and option B

R2-2505712	Discussion on LCM for UE-sided model for Beam Management	Spreadtrum, UNISOC	discussion	Rel-19

Proposal 3: For RRC8, for clarification, option A and option B cannot be configured simultaneously for the same AI functionality.

Not Treated

R2-2505213	Discussion on LCM for UE-sided model for BM use case	CATT	discussion	Rel-19	NR_AIML_air-Core

R2-2505582	Remaining open issues on LCM for UE-sided model for Beam Management	NEC Corporation	discussion	Rel-19

R2-2506102	Discussion on LCM for UE-Side Model for Beam Management Use Case	Futurewei Technologies	discussion	Rel-19

#### 8.1.2.3	LCM for Positioning use case

Contributions should focus on LCM for UE-sided model, but can discuss NW-sided model. Aspects related to data collection should be covered in 8.1.3

Rapporteur’s input (to be noted)

R2-2505702	LPP open issues for feature "AI/ML for NR air interface"	Qualcomm Incorporated (Rapporteur)	discussion	NR_AIML_air-Core (Moved from 8.1.1)

Noted

LPP Open Issues

LPP-6a: Number of PRUs in NR-PRU-DL-Info

R2-2505194	Discussion on LPP-6a for AI based positioning Case1	vivo	discussion	NR_AIML_air-Core

Proposal 1. (LPP-6a) UE is allowed to request the exact number of PRUs that it expects the PRU information.

Proposal 2. (LPP-6a) RAN2 to adopt that the NW can provide a list of PRUs’ information in one Provide Assistance Data message, considering signalling overhead to repeatedly sending LPP message.

Noted

R2-2505763	Remaining open issues: LCM for Positioning use case	InterDigital	discussion	Rel-19	NR_AIML_air-Core

Proposal 1: [LPP-6a] Do not introduce a request for additional PRUs (e.g., a number of PRUs) in the Request Assistance Data message.

Noted

Discussion

-	Nokia and Samsung is good with proposal 2.  Nokia doesn’t a see a reason to request additional PRUs

-	Ericsson doesn’t think any enhancements are needed.   SA2 is working on a lot of enhancements in this area so there is no need for further enhancements.

-	ZTE and LG thinks that this is essential for training and agree with both 1 and 2.

-	Xiaomi thinks that there was no RAN1 evaluation whether these additional PRUs would be helpful and there is no way for the UE to evaluate which PRU it is unless we provide the ID and that information shouldn’t be exposed.

-	Samsung and ZTE thinks that we just need to know the location and not exactly with PRU.

-	Qualcomm agrees with Interdigital and the PRU supports a lot of measurements but not all of those are useful for the UE.

-	Vivo explains that the proposals are also for performance monitoring.  Qualcomm thinks that model training is up to UE anyways.

-	Interdigital thinks that we can still do training with a bit more signaling overhead but it is not the most important issue.  Ericsson agrees we can save some signaling and not worth doing it.   Apple agrees with Ericsson.

LPP-10a: Applicability of "batch reporting" for AI/ML positioning.

R2-2505765	Remaining LPP open issues for "AI/ML Positioning Case 1"	Qualcomm Incorporated	discussion	NR_AIML_air-Core

Proposal 2 (LPP-10a): "Batch reporting", i.e., reporting of up to 32 location results in a single report as supported for the current NR positioning methods, is also applicable to "NR AI/ML Positioning Case 1".

Noted

R2-2505593	Discussion on LCM for positioning use case	ZTE Corporation	discussion	Rel-19	NR_AIML_air-Core

Proposal 3: Do not support batch reporting in AI/ML positioning method.

Noted

LPP-11: Details of IE NR-AI-ML-PositioningRequestLocationInformation

R2-2506056	Discussion on remaining open issues for AIML positioning	LG Electronics Inc.	discussion	Rel-19	37.355

Proposal 4: Keep NR-DL-AIML-RequestLocationInformation, excluding UE-assisted measurement parameters, and retain only UE-based and common parameters (e.g., nr-AssistanceAvailability).

Proposal 5: For AI/ML positioning Case 1, the LocationInformationType field in CommonIEsRequestLocationInformation shall be set to locationEstimateRequired. Other values, including locationMeasurementsRequired, locationEstimatePreferred, locationMeasurementsPreferred, and locationEstimateAndMeasurementsRequired, are not applicable and shall not be used.

Noted

LPP-14: Target device error causes

R2-2505593	Discussion on LCM for positioning use case	ZTE Corporation	discussion	Rel-19	NR_AIML_air-Core

Proposal 5: Introduce the following additional target device error cause in AI/ML positioning, the error cause are indicated with respect to each LCM procedure (training/inference/monitoring):

-	Error cause during training:

UE does not get enough training data to train a AI/ML model;

UE cannot download or acquire or train a AI/ML model;

-	Error cause during inference:

UE cannot generate an estimate location using a AI/ML model;

-	Error cause during performance monitoring:

UE cannot generate a performance monitoring metric.

Noted

R2-2506080	Discussion on LCM for Positioning use case of UE-sided model	Huawei, HiSilicon	discussion	Rel-19	NR_AIML_air-Core

Proposal 2: (LPP-14) It is proposed RAN2 to introduce “low power state” as new target device error.

Noted

Discussion

-	Xiaomi doesn’t think we need anymore error causes, we agreed last meeting.

-	Ericsson thinks that we don’t really address how it downloaded etc, so this is an overkill.

LPP-18: Consistency between training and inference [Unofficial offline – lead by Vivo]

R2-2505193	Discussion on open issue LPP-18 consistency between training and inference	vivo, OPPO, CATT,  ZTE Corporation, Sanechips, Fraunhofer IIS, Fraunhofer HHI	discussion	NR_AIML_air-Core

Proposal 1. RAN2 to adopt a list of global cell information (i.e., NCGIs, or PCIs with ARFCN) and TRP ID, as the request associated information to ensure consistency between training and inference.

Proposal 2. RAN2 to discuss how to introduce the specific group of TRPs associated with the requested assistance data:

-	to directly acquire assistance data of specific TRPs, i.e., right under NR-DL-AIML-RequestAssistanceData;

-	to ask specific TRPs for PRS transmission with on-demand PRS configuration, i.e., within NR-On-Demand-DL-PRS-Request.

Noted

R2-2505355	Remaining issues on LCM for POS use case	Samsung	discussion	Rel-19	NR_AIML_air-Core

Proposal. 11: To ensure consistency between training and inference, UE can request the assistance data (e.g., DL-PRS configuration) for a specific group of TRPs. I.e., UE can report a list of TRP information (i.e., combination of dl-PRS-ID, NCGI, PCI, ARFCN) to identify the group of TRPs for which the assistance data is requested.

Proposal. 12: To ensure consistency between training and inference, UE can report the associated ID value used during training phase for each TRP.

Noted

Discussion after offline

-	Vivo explains that we can support P1 but in the offline there was no big desire to have associated ID.

-	Samsung thinks we can leave it open.

LPP-19: Applicability of BM related agreements

R2-2505214	Issues to address for AIML Positioning stage-2	CATT	discussion	Rel-19	NR_AIML_air-Core

(LPP-19)Proposal 1: the following agreements made for BM applicability reporting is also applicable to positioning case 1:

-	UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device.

-	Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed.

-	Qualcomm thinks the second bullet doesn’t map to positioning as it is not clear what is initial report and what is subsequent report.   Apple agrees with Qualcomm

Noted

R2-2506056	Discussion on remaining open issues for AIML positioning	LG Electronics Inc.	discussion	Rel-19	37.355

Proposal 8. No LPP changes are needed for applicability reporting in AI/ML positioning Case 1, as the existing capability exchange and unsolicited reporting mechanisms are sufficient.

Proposal 9. Add an explicit field, e.g., nr-dl-aiml-positioning-supported-r19, to NR-DL-AIML-ProvideCapabilities to indicate the applicability of AI/ML positioning Case 1.

Noted

R2-2505866	Applicability reporting for positioning use case	Nokia	discussion	Rel-19	NR_AIML_air-Core

Proposal 1: Add an explicit bit in NR-DL-AIML-ProvideCapabilities-r19 to indicate dynamic change in UE DL AIML positioning applicability from supported to “currently not-applicable”.

Proposal 2: The NR-DL-AIML-ProvideAssistanceData IE signals a list of candidate DL PRS configurations which can be used by the target device to determine applicability of AI/ML positioning for the provided set of DL-PRS configurations.

-	Ericsson agrees.  Qualcomm thinks this is not an open issue this is a new feature discussion.

Proposal 3: The UE may optionally include in NR-DL-AIML-ProvideCapabilities IE the DL-PRS configurations from the set of candidate DL-PRS configurations assistance provided by the location server for which the DL AIML positioning is currently applicable in the UE.

Noted

LPP-20: LPP Impacts related to Case 3a/3b

R2-2505302	Discussion on AI positioning open issues	Xiaomi	discussion	Rel-19	NR_AIML_air-Core

Proposal 2: (LPP-20) Case 3a and Case 3b can be supported without new impact to LPP.

-	Ericsson thinks that there may be some impacts like case 3b, the location and time stamp and RAN3 has ensured that the time stamp is also provided.   Currently in RAN2 the time stamp is optional.

Noted

LPP-21: "Associated ID" for TRP Location Coordinates (IE TRP-ImplicitLocationInfo-r19) [Unofficial offline – lead by Qualcomm]

R2-2505765	Remaining LPP open issues for "AI/ML Positioning Case 1"	Qualcomm Incorporated	discussion	NR_AIML_air-Core

Proposal 4a (LPP-21): The new IE for the implicit TRP location information (NR-TRP-LocationInfo-Implicit-r19) has the same structure as the existing IE NR-TRP-LocationInfo-r16, but with the location coordinate information being replaced by an "Associated ID" at each level (TRP, ARPs level).

Proposal 4b (LPP-21): In all NR AI/ML assistance data IEs where Cell IDs (NCGIs, PCIs) can optionally be included for a TRP, the Cell IDs (NCGIs, PCIs) are always present in the IEs if the TRP is associated to a cell.

Proposal 4c (LPP-21): If Proposals 4a/4b are agreeable, the "Associated ID" can be defined with 8-bits.

Proposal 4d (LPP-21): Whether the IEs NR-TRP-LocationInfo-r16 and NR-TRP-LocationInfo-Implicit-r19 can be both provided together or not is left to implementation/deployment and does not need to be specified.

Proposal 4e (LPP-21): If a UE supports explicit TRP location info (NR-TRP-LocationInfo-r16) a UE must not mandatorily support also implicit location info (NR-TRP-LocationInfo-Implicit-r19) (and vice versa).

Noted

R2-2505908	RAN1 Agreements impacting RAN2 and Addressing Open Issues 	Ericsson	discussion	Rel-19	NR_AIML_air-Core

Proposal 5: (LPP-21) Associate ID for PRS only TP is not considered in Rel-20. The scope of Associate ID (to what granular level the associate ID needs to be updated) is left to NW implementation. Associate ID value range is 0 to 255. Explicit and Implicit TRP coordinates are mutually inclusive. In terms of UE capability, the UE supporting case1 AI/ML must support implicit location info.

Noted

Agreements on positioning

- Do not introduce a request for additional PRUs (e.g., a number of PRUs) in the Request Assistance Data message

- "Batch reporting", i.e., reporting of up to 32 location results in a single report as supported for the current NR positioning methods, is also applicable to "NR AI/ML Positioning Case 1".

- Keep NR-DL-AIML-RequestLocationInformation, excluding UE-assisted measurement parameters, and retain only UE-based and common parameters (e.g., nr-AssistanceAvailability).

- For AI/ML positioning Case 1, the LocationInformationType field in CommonIEsRequestLocationInformation shall be set to locationEstimateRequired. Other values, including locationMeasurementsRequired, locationEstimatePreferred, locationMeasurementsPreferred, and locationEstimateAndMeasurementsRequired, are not applicable and shall not be used.   No specification impact.

- We do not introduce new error cause for the target device error causes.

- Case 3a and Case 3b can be supported without new impact to LPP

- Introduce list of global cell information (i.e., NCGIs, or PCIs with ARFCN) and TRP ID, as the request associated information to ensure consistency between training and inference.

- The UE asks specific TRPs for PRS transmission with on-demand PRS configuration, i.e., within NR-On-Demand-DL-PRS-Request

- Similar to BM, UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device.  If nw side additional conditions are not provided then we follow BM conclusion.   No stage 3 impacts.

- Similar to AI PHY, when applicability changes the UE should report this to the LMF and only what changed.   For now capture this at least in stage 2.  Check offline if and how this would be implemented in stage 3.

- Wait for RAN1 for LPP-21.  Take what RAN1 gives us and we implemented.   Can compile an LS for next meeting if we have questions.

### 8.1.3	NW side data collection

Contributions should focus on the remaining aspects related to mechanisms and principles identified for data collection for network side model training.  Including outcome of [POST130][031][AI PHY] NW side data collection (Ericsson and ZTE) and [POST130][034][AI PHY] LS to RAN3 (Nokia).

Email discussion: Data collection configuration (open issue RRC-24) [for online discussion Tuesday]

R2-2505778	RRC open issues for AIML for NR air interface	Ericsson	discussion (Moved from 8.1.1)

Proposal 1: (RRC-9)

Proposal 1	(RRC-49) For L1-related content for NW-side data collection, RAN2 to agree on one of the two options: (a) it is sufficient to collect the L1-RSRP and/or beam ID as agreed by RAN2, or (b) RAN2 should ask RAN1 whether other data needs to be collected.

Proposal 2	(RRC-43) RAN2 to discuss the value range for the buffer threshold based on the outcome of the UE capability discussion. Possible values to consider are, e.g. 1 KB, 2 KB, 4 KB, 8 KB, 16 KB, 32 KB, 45 KB, 48 KB, 52 KB, 58 KB, 60 KB, 62KB, 64 KB, 128 KB, 256 KB, etc.

16KB, 32KB and 48KB.  FFS during CR phase in any higher value is needed depending on UE capability discussion.

R2-2505859	Report of email discussion [POST130][031][AI PHY] NW side data collection	Ericsson, ZTE Corporation	discussion

Noted

Can we revert the agreement on event triggered logging

-	Apple and Vivo doesn’t think we should revert as we have spent a lot of time on making decisions.  Huawei thinks that we shouldn’t continue this discussion as network vendors don’t think this is useful.  Ericsson and Nokia agrees and since the beginning didn’t see a need to have this as we can achieve it by implementation.

-	ZTE explains that there was a reason when we agreed to it and now we don’t those benefits.

-	Tmobile and AT&T also doesn’t see the need for it either.

Proposal 2	RAN2 to decide on whether to capture the event evaluation for the event-triggered logging in the existing A1/A2 events (in sub-clauses 5.5.4.2 and 5.5.4.3) or in new events.

Proposal 7	RAN2 to discuss and decide on alternative to introduce logging configuration for network data collection:

- logging configuration is introduced as a new list of configurations under CSI-MeasConfig, based on TP1 in R2-2505860, (Acceptable 8/20, Not acceptable 3/20)

- logging configuration is introduced in a new L3 measurement framework, at the same level as MeasConfig and CellGroupConfig, based on TP2 in R2-2505861, (Acceptable 15/20, Not acceptable 3/20)

Agreements on NW side data collection

1	RAN2 confirms that the network data logging is captured in a new clause (e.g. 5.5x) in the RRC specification.

2	A hysteresis should be configured and used (alongside threshold and timeToTrigger) for event-triggered logging for NW-side data collection.

3	The resource configuration does not have separate resources for Set A and Set B.

4	RAN2 to send an LS to RAN1 to inform about the RAN2 agreements on solution for network data logging, including L1 related content for NW-side data collection.

5	RAN2 to send an LS to RAN3 to inform about the RAN2 agreements on solution for network data logging

- keep event-triggered logging

- logging configuration is introduced as a new list of configurations under CSI-MeasConfig, based on TP1 in R2-2505860,

- Event evaluation for the event-triggered logging will be capturing within the existing A1/A2 events (in sub-clauses 5.5.4.2 and 5.5.4.3)

- For L1-related content for NW-side data collection, it is sufficient to collect the L1-RSRP and/or beam ID as agreed by RAN2

[POST131][038][AI PHY] LS to RAN1/RAN3 on nw-sided data collection (ZTE)

Intended outcome: agree to RAN1 and RAN3 LS capturing relevant RAN2 agreements

Deadline:  short

=> Approved R2-2506545 (LS out)

R2-2506545	LS to RAN1 and RAN3 on NW side data collection	RAN2	LS out	Rel-19	NR_AIML_air-Core	To:RAN1, RAN3

=> Approved

R2-2505860	TP approach (1) for email discussion [POST130][031][AI PHY] NW side data	Ericsson, ZTE Corporation	discussion

R2-2505861	TP approach (2) for email discussion [POST130][031][AI PHY] NW side data	Ericsson, ZTE Corporation	discussion

LS reply to SA5/RAN3: [for online discussion]

R2-2506079	Discussion on measurements for OAM-centric solution [S5-252842]	Huawei, HiSilicon, Ericsson, Nokia	discussion	Rel-19	NR_AIML_air-Core

RAN2 measurements 1 For network-side data collection for beam prediction, measurement reports include the following:. ⁻ Cell identity: CGI or PCI of the cell to which the measurement results are related. ⁻ Logged L1 radio measurement results including the beam identifiers associated to CSI-RS resources or SSBs (CSI-RS IDs or SSB IDs) and the corresponding measured L1-RSRPs. - [CB after OI discussion ]Information to indicate a gap that is longer than the configured logging periodicity in the logged measurements which can happen due to event-based logging. FFS the details. 2: The required L1 measurements (i.e. L1-RSRP of CSI-RS(s) or SSBs) are defined in TS 38.215 which is referenced in the definition of M1 measurement in TS 37.320. 3: RAN2 needs to ask SA5 how to transfer the measurement report with the above content and whether the M1 measurement defined in clause 5.4.1.1 of TS 37.320 can be used to transfer the measurement reports with the above content or not. 4: The gNB will configure the UE to log the above content via the NW-side data collection configuration, which is different from legacy immediate MDT configuration.

[POST131][036][AI PHY] Reply to SA5/RAN3 (Huawei)

Intended outcome: agree to LS

Deadline:  short

=> Approved in R2-2506543 (LS out)

R2-2506543	Reply LS on OAM-centric solution for NW-side data collection	RAN2	LS out	Rel-19	AIML_MGT_Ph2	To:SA5	Cc:RAN3

=> Approved

RRC Open Issues: Issues with Rapporteur proposal [online]

R2-2505778	RRC open issues for AIML for NR air interface	Ericsson	discussion (Moved from 8.1.1)

Proposal 13	(RRC-26) Multiplexing of legacy SON/MDT report and AIML logged data in the new SRB is not supported.

-	Samsung thinks that anyway the network can control the multiplexing

Noted

Agreements

- Multiplexing of legacy SON/MDT report and AIML logged data is not supported in the same UE information response message.  Up to the network to ensure that data is not requested at the same time

- The logging periodicity of a NW-side data collection configuration is configurable.

- No further indication/condition is specified (beyond already agreed ones) for the UE to inform source gNB about data availability before HO in Rel-19.

- The UE stores logged data for BM in a variable specific to L1 CSI related measurements.

- Only periodic CSI resources are used for NW sided data collection.  No need for new dynamic MAC CE mechanisms.

RRC-25/RRC-30: Dynamic Activation/Deactivation and Semi-Persistent Resources [for online discussion]

R2-2505675	Remaining issues of NW-sided data collection	Huawei, HiSilicon	discussion	Rel-19	NR_AIML_air-Core

Proposal 8: (RRC-25) (RRC-30) Only periodic CSI resources are used for NW sided data collection, i.e. neither aperiodic nor semi-persistent CSI resources are supported for this purpose.

Noted

R2-2505215	Consideration on NW side data collection	CATT	discussion	Rel-19	NR_AIML_air-Core

Proposal 2: (RRC-25) Dynamic activation/deactivation of data collection configurations for logging is not needed, as it can be implicitly achieved by the add/modify/release operation of the related data collection configurations.

Noted

R2-2505200	Open Issues on Network Side Data Collection 	Qualcomm Incorporated 	discussion	Rel-19

Proposal 1: To avoid the frequent reconfiguration for measurement resource reconfiguration, the network can configure the UE with one or more resource sets for data collection for network-side training.

Proposal 2: The network may dynamically (de)activate measurement on different resource sets (among the configured resource sets) based on the UE mobility and the network requirements.

Proposal 3: To assist the network with (de)activation of the appropriate RS for data collection, the UE may notify the network when the configured RS becomes undetectable, meaning the UE cannot measure the configured RS.

Noted

RRC-30: User consent [for online discussion]

R2-2505839	NW-side data collection	Ericsson	discussion

Proposal 8: (RRC-33) RAN2 does not discuss user consent related aspects for NW-side data collection in Rel-19.

Noted

R2-2506186	User consent for NW-side data collection	NTT DOCOMO, Apple, vivo, Xiaomi, OPPO	discussion

Proposal 1: NW-sided data collection for AI/ML model training requires user consent.

Proposal 2: re-use the existing user consent framework (defined for MDT) for NW-side data collection.

Noted

R2-2505687	Left issues for NW side data collection	Lenovo	discussion	Rel-19

Proposal 3: RAN2 sends LS to SA3/SA5 consulting if existing MDT user consent can be reused or AIML related user consent shall be provided for the gNB to initiate gNB-centric data collection.

Noted

Discussion

-	CMCC thinks that for MDT the only reason it is because of user location.  Apple explains that user consent is for all of MDT and not only for location.

Send an LS to SA3 indicating our work on nw-sided gNB and OAM centric data collection and content of collected data.   RAN2 discussed the need for user consent and would like to SA3 to take it into account and decide on the need for user consent for nw sided data collection.

[POST131][037][AI PHY] User Consent LS to SA3 (NTT Docomo)

Intended outcome: agree to LS and cc RAN3 ccSA5

Deadline:  short

=> Approved in R2-2506541 (LS out)

R2-2506541	LS on User consent for Data collection at the UE for NW-side model training	RAN2	LS out	Rel-19	NR_AIML_air-Core	To:SA3	Cc: RAN3, SA5

=> Approved

UE capabilities [for online discussion]

R2-2505150	Discussion on NW side data collection	Xiaomi	discussion

Proposal 18: (UECap-1) UE can support different data logging AS buffer- size across all use cases, which can be indicated by UE capability. The potential AS buffer size can be 128KB.

Proposal 20: (UECap-2) Periodic and event-based logging are part of basic logged data collection capability.

Proposal 21: (UECap-2) Define separate capability to indicate the supported measurement quantities for logged data collection.

Proposal 22: (UECap-2) Assistance information for NW side data collection is part of basic logged data collection capability.

Noted

R2-2505839	NW-side data collection	Ericsson	discussion

Proposal 12: (UE-Cap2) Introduce an optional generic UE capability to indicate supporting logged measurements for NW-side data collection, which implies the UE supports at least: the minimum AS layer memory size, periodic logging, reporting data availability upon reaching the full buffer size, and reporting data availability upon exceeding the configured buffer threshold.

Noted

RRC Open issues: Open issues that will be discussed first offline in order and then what’s left online

[AT131][004][AI PHY] NW sided OIs (InterDigital)

Intended outcome: agreeable proposals

Deadline:  Wednesday

R2-2506413	Summary of [AT131][AI PHY] NW side data collection open issues	InterDigital Inc. (Rapporteur)	discussion	Rel-19	NR_AIML_air-Core

Noted

Observation Companies have different views regarding how to capture the gap information in the log and whether if there is even a need for it.

Discussion

-	Ericsson doesn’t think 1 bit is enough and we would need some time information.

-	Nokia thinks absolute time stamp absolutely not necessary as the UE does’nt stay long in connected mode.   Qualcomm agrees with Nokia that we don’t need it.  Interdigital also agrees and we can group them together and use 1 bit indication is sufficient.

-	ZTE thinks that relative time is enough.

Agreements:

1	If LoggedDataCollectionAssistanceConfig is configured, then full buffer and low power indications are configured by default (i.e., no additional fields/bits required to configure them). Data threshold is (optionally) configured by including the threshold in the loggedDataCollectionAssistanceConfig.

2	Both the data collection configuration and the UAI configuration related to data collection are released when the UE transitions to IDLE/INACTIVE or initiates re-establishment (including RLF).

3	If the buffer is not full or the data threshold is configured and the amount of data is below the threshold, UE does not send data availability indication when it sends low power indication.

4	No additional handling of logged data to be specified (apart from the already agreed release during state transition and RLF, and release upon successful delivery).

5	No further discussion is needed on RRC issue 34 in re-19 as we have only one use case.

6	The UE will indicate the presence of a gap (i.e. there will be no indication on the length of gap or time instance, etc).  Rapporteur will suggest a way to implemented as part of the RRC review.

Not treated

RRC-19: Reporting assistance information related to logged measurements

R2-2505240	Remaining issues on NW side Data Collection	LG Electronics	discussion	Rel-19	NR_AIML_air-Core

Proposal 1. [RRC19] The otherConfig includes separate bits for low power, buffer full, and buffer threshold reached indications.

R2-2505504	Remaining issues on NW-sided data collection	Apple	discussion	Rel-19	NR_AIML_air-Core

Proposal 4 (open issue RRC-19): Full buffer indication is enabled by default whenever LoggedDataCollectionAssistanceConfig is configured (i.e. no additional configuration bit is needed).

R2-2505675	Remaining issues of NW-sided data collection	Huawei, HiSilicon	discussion	Rel-19	NR_AIML_air-Core

Proposal 2: (RRC-19) The indications reported by the UE as part of UAI for data collection purposes are controlled by the network in the following way:

- Full buffer indication is enabled by default when the UE is configured with

loggedDataCollectionAssistanceConfig

- Data threshold is enabled by including data threshold in the configuration

- Low power state indication is configurable with a dedicated parameter to enable it

RRC-21: Time related content of logged data

R2-2505839	NW-side data collection	Ericsson	discussion

Proposal 3 (RRC-21) The UE includes in the logged measurement report, an absolute time stamp (provided by the network at the time of configuring the UE with the NW-side data collection configuration), and for the first measurement entry and each measurement entry separated by a “gap” versus the immediate previous entry, a relative time stamp.

R2-2505075	Leftover Issue Discussion on NW side data collection	OPPO	discussion	Rel-19	NR_AIML_air-Core

Proposal 1: (RRC-21) Consecutive samples share the same record list, if there is a gap between two samples, another record list is used to include the consecutive samples after the gap. Optionally, the first sample of each record list is associated with a relative timestamp.

R2-2506118	Discussion on NW side data collection framework	ZTE Corporation	discussion	Rel-19	NR_AIML_air-Core

Proposal 1: For logged data attached with time information, the first logged data is attached with an absolute time stamp as long as UE starts the logging, and the following logged data is attached with relative time stamp until to the corresponding logging configuration is released.

RRC-28: Handling of configuration to report data availability and low power state during RRCReestablishment, transition to RRC_INACTIVE etc

R2-2505150	Discussion on NW side data collection	Xiaomi	discussion

Proposal 4: (RRC-28) Logging and UAI configuration are released upon IDLE and RLF.

Proposal 5: (RRC-28) Logging and UAI configuration are kept upon INACTIVE.

R2-2505346	Discussion on NW side data collection	Samsung	discussion	Rel-19	NR_AIML_air-Core

Proposal 3 (RRC-28). Upon RRE initiation (including RLF detection), UE discards any logged data.

Proposal 4 (RRC-28). Upon RRE initiation, UE releases 1) the configuration of NW-side data collection and 2) UAI configuration for NW-side data collection.

Proposal 5 (RRC-28). When going to RRC_IDLE, UE releases 1) the configuration of NW-side data collection and 2) UAI configuration for NW-side data collection (i.e., No spec change is needed).

Proposal 6 (RRC-28). When going to RRC_INACTIVE, UE does not store the configuration of NW-side data collection in UE Inactive AS Context.

Proposal 7 (RRC-28). When going to RRC_INACTIVE, UE first stores UAI configuration for NW-side data collection in UE Inactive AS Context and it is released upon initiation of RRC connection resume.

RRC-29: Data Availability Indication and Low Power State

R2-2505650	Discussion on open issues on NW side data collection	Sony	discussion	Rel-19	NR_AIML_air-Core

Proposal 3 (RRC-29): Data availability indication is not sent when indicating low power state.

R2-2506104	Data Collection for Training of NW-side AI/ML Models	Nokia	discussion	Rel-19	NR_AIML_air-Core

Proposal 4: (RRC-29) If the UE is in a low power state AND it has data in its buffer, even below the threshold, UE can still indicate both low power state and buffer threshold met indications. That is, no additional cause or flag is required.

R2-2505471	Further Discussion on the Remaining RRC Issues for Network-Side Data Collection 	MediaTek Inc.	discussion

Proposal 3: When the UE enters a low power state, it sends a low power state indication together with the buffer status to the network.

Proposal 4: The buffer status indicator (bufferStatus-r19) is extended to include additional values, i.e. belowThreshold and empty.

RRC-31: Release of Logged AI/ML Data

R2-2505511	Discussion on remaining issues on NW side data collection	CMCC	discussion	Rel-19	NR_AIML_air-Core

Proposal 4: In case the network does not retrieve logged AI/ML data, UE should store non-retrieved data for 48 hours after the release of data collection configuration. The UE will release all logged AI/ML data at switch off or de-registration, or receiving the explicit indication from the serving gNB.

R2-2506104	Data Collection for Training of NW-side AI/ML Models	Nokia	discussion	Rel-19	NR_AIML_air-Core

Proposal 6: (RRC-31) Data from the UE buffer will be released in the following scenarios:

- when data has been successfully transmitted using a UEInformationResponse message;

- the configuration for logging has been released and the flag to retain measurements after handover has not been set;

- the configuration for logging has been modified;

- All logged measurement configuration and the logged data shall be removed by the UE at switch off or detach.

R2-2505839	NW-side data collection	Ericsson	discussion

Proposal 7: (RRC-31) RAN2 does not specify further cases when the UE shall discard logged data for NW-side data collection, beyond already agreed cases.

RRC-34: Per use case vs common enhancements for NW-side data collection

R2-2505195	Remaining issues on NW side data collection	vivo	discussion	NR_AIML_air-Core

Proposal 6. [RRC-34] A common approach should be considered for enhancements for NW-side data collection, and the following enhancements are common for all use cases:

- AS layer memory;

- logged data availability;

- low-power state indication.

R2-2505075	Leftover Issue Discussion on NW side data collection	OPPO	discussion	Rel-19	NR_AIML_air-Core

Proposal 10: (RRC-34) AS layer memory and low power state indication should be defined per UE, i.e. common for all use cases, while data availability indicator should be sent per use case.

R2-2505687	Left issues for NW side data collection	Lenovo	discussion	Rel-19

Proposal 4: When UE indicates the data availability to NW, UE also indicates the list of data logging configuration ID for which data is available.

Proposal 5: When gNB requests logged data from UE via UEInformationRequest message, gNB can also include the interested data logging configuration ID.

Proposal 6: The cause value (i.e., buffer full, buffer threshold reached, low power state) of data availability indication remain generic for all data logging.

RRC-39: The naming of IEs related to NW-side data collection

R2-2505177	Discussion on UE-side data collection	LG Electronics Inc.	discussion	Rel-19	NR_AIML_air-Core

Proposal 10. [RRC39] To define field names and IE based on the content of the logged data rather than the specific use case

Noted

R2-2505675	Remaining issues of NW-sided data collection	Huawei, HiSilicon	discussion	Rel-19	NR_AIML_air-Core

Proposal 3: The names of the fields/IEs to request logged data from the UE and to report logged data or the availability of logged data to the NW should be use case specific.

Proposal 4: (RRC-39) Data availability triggering threshold should be configurable per data collection configuration.

Proposal 5: (RRC-39) The UE should include the corresponding configuration identifier (i.e. CSI-LoggedMeasurementConfigId) in the data available indication to allow the network to request specific data collected by the UE.

Proposal 6: (RRC-39) The network should be able to request the UE to send collected data for a specific data collection configuration, i.e.  UEInformationRequest for NW-side data collection should include one or more CSI-LoggedMeasurementConfigId(s).

Noted

RRC-36/51/52: How data is forwarded to OAM or source gNB after HO (RRC-36), data handling during inter-RAT HO (RRC-51), and how to handle collected data in CHO/LTM (RRC-52) [straight to offline]

R2-2505195	Remaining issues on NW side data collection	vivo	discussion	NR_AIML_air-Core

Proposal 7: [RRC-36] On how data is forwarded to OAM or the source gNB, the following is proposed:

-	How data is forwarded to OAM is out of RAN2 scope and should be referred to SA5;

-	Xn signaling procedure can be adopted on how data is forwarded to source gNB, and details should be discussed in RAN3;

-	RAN2 to LS to RAN3/SA5 to let them consider how data is forwarded to OAM or source gNB.

Noted

R2-2505150	Discussion on NW side data collection	Xiaomi	discussion

Proposal 16: (RRC-51) UE discards the logged data upon inter-RAT handover.

Noted

R2-2505504	Remaining issues on NW-sided data collection	Apple	discussion	Rel-19	NR_AIML_air-Core

Proposal 3 (open issue RRC-52): RAN2 confirm that the solution agreed in RAN2#130 is only applicable to regular HO. To enable NW control on retaining logged data at CHO/LTM, source cell makes decision and includes a list of 1-bit indication corresponding to each candidate cell configuration in RRCReconfiguration

Noted

Agreements

- To define field names and IE based on the content of the logged data rather than the specific use case

- Data forwarding to OAM or source gNB after HO is not in RAN2 scope and understands that other groups don’t have time to work on it.

- UE discards the logged data upon inter-RAT handover.

- RAN2 confirm that the solution agreed in RAN2#130 is applicable to regular HO and CHO (i.e. 1-bit indication corresponding to each candidate cell configuration in RRCReconfiguration is provided).

- Do not introduce an indication from the UE to NW about unsuitable data collection configurations in Rel-19

RRC-50: Suitability of data collection configuration (for both UE-side and network-side training

R2-2505150	Discussion on NW side data collection	Xiaomi	discussion

Proposal 15: (RRC-50) Do not introduce an indication from the UE to NW about unsuitable data collection configurations in Rel-19

Noted

R2-2505713	Discussion on NW side data collection	Spreadtrum, UNISOC	discussion	Rel-19

Proposal 8: When UE can’t perform data collection based on received configuration, an indication from UE to

Noted

Not treated

R2-2505873	Remaining issues for NW side data collection	Interdigital Inc.	discussion	Rel-19	NR_AIML_air-Core

R2-2505995	Discussion on NW-side data collection	NTT DOCOMO, INC.	discussion	Rel-19

R2-2506060	Discussion on NW side data collection	HONOR	discussion	Rel-19	NR_AIML_air-Core

R2-2506072	Remaining Issues on NW side data collection 	Kyocera	discussion

R2-2506103	Discussion on Data Collection for NW-side Model Training	Futurewei Technologies	discussion	Rel-19

R2-2506109	Discussion on NW-side data collection framework	Ericsson, Nokia, Huawei, HiSilicon, T-Mobile, BT Plc, Verizon, TIM, AT&T, Deutsche Telekom, LG Electronics	discussion

### 8.1.4	UE side data collection

Configuration details for Rel-19 data collection for UE-sided model can be discussed in contributions in 8.1.2.2.

Discuss any of the aspects identified in RANP WF

o	Study RAN aspects related to data transfer over UP

o	Discuss level of NG-RAN involvement in the control and configuration of UE side data collection.

o	Discuss NG-RAN involvement in the data transfer of UE side data collection (if any) (including visibility discussion).

o	Discuss aspects/solutions from RAN perspective that enable the data transfer to CN domain or OAM domain.

o	Discuss on the scalability aspects of CP

Including the outcome of [POST130][033][AI PHY] UE Side data collection (Ericsson)

This AI is downprioritized as it will be in scope of Rel-20.  If any contributionsa are submiited they should be focused on something critical needed to be included in the TR

Email discussion: outcome of [POST130][033][AI PHY] UE Side data collection:

R2-2505936	RAN2 inputs to TR 38.843 - first part	Ericsson	draftCR	Rel-19	38.843	18.0.0	F	FS_NR_AIML_air_Ph2

TP is agreed in R2-2506457 with Annex removed and last sentence above the annex and include this previously agreed TP in the LS to RAN1

R2-2505937	RAN2 inputs to TR 38.843 - second part	Ericsson	draftCR	Rel-19	38.843	18.0.0	F	FS_NR_AIML_air_Ph2

TP is agreed and will be included the LS to RAN1 to be included in the TR

[AT131][039][AI PHY] LS to RAN1 on TPs  (Ericsson)

Intended outcome: Notify RAN1 of agreed TPs, attach them and ask them to include in the TR.

Deadline:  Friday by email.

R2-2506474	RAN2 inputs to TR 38.843	RAN2	LS out	Rel-19	FS_NR_AIML_air_Ph2	To:RAN1

LS is approved

R2-2506088	Discussion on UE-side data collection	Huawei, HiSilicon, OPPO, China Telecom, CATT, China Unicom, ZTE, NTT DoCoMo	discussion	Rel-19	NR_AIML_air-Core

Not to be treated

standaridzed vs non-standaridzed data:

R2-2505472	Discussion on the Necessity of Non-Standardized Data	Mediatek Inc.,Nokia,Qualcomm Incorporated,InterDigital Inc.	discussion	R2-2503777

Proposal 1: Support non-standardized and partially standardized data contents in Solution 2 and/or Solution 3. The choice of solution will depend on the outcome of the data transfer study.

Proposal 2: Adopt the following principles to mitigate privacy risks and other principles are not excluded:

- User consent enforcement: Recommendation to adapt the existing 3GPP User consent mechanism for UE sided data collection.

- Data scope limitation: Restrict collection to non-user identifiable information, such as radio/sensor-generated metrics.

- MNO governance: Allow operators to enable/disable non-standardized data collection.

R2-2505177	Discussion on UE-side data collection	LG Electronics Inc.	discussion	Rel-19	NR_AIML_air-Core

Proposal 10. For Rel-19, RAN2 only focuses on standardized data, disregarding partial/non-standardized data.

R2-2505125	Analysis of solutions for UE side model data collection	NEC	discussion	Rel-19	NR_AIML_air-Core

R2-2506105	Discussion on Data Collection for UE-side Model Training	Futurewei Technologies	discussion	Rel-19

### 8.1.5	Model transfer/delivery

For RAN2#131 contributions can be submitted only by operators addressing requirements.   Other companies are encouraged to collaborate with operators.

Contributions will be treated in RAN2#131 only if time allows otherwise it is postponed for Rel-20.

Operator views on model transfer (If time allows)

R2-2505931	Operators views for the AI_ML model delivery options	BT Plc, T-Mobile, Deutsche Telekom, Turkcell, Orange, TIM, AT&T, Verizon, Docomo, Nokia, KPN	discussion	Rel-19

=> Revised in R2-2506169

R2-2506169	Operators views for the AI_ML model delivery options	BT Plc, T-Mobile, Deutsche Telekom, Turkcell, Orange, TIM, AT&T, Verizon, Docomo, Nokia, KPN, KDDI	discussion	Rel-19	R2-2505931

Proposal 1: Solution 4a, OTT without 3GPP impact, is no longer discussed in RAN2 as a standardised solution for transferring/delivering AI/ML models

Proposal 2: One-sided model transfer should be specified by 3GPP

Proposal 3: One-sided dataset/model transfer solution shall follow below principles:

•	A1 - Size: from RAN2 point of view, aim to support various sizes of dataset/model parameter transfer (FFS on model size);

•	A2 - Continuity: service continuity of model transfer/delivery during UE mobility needs to be supported;

•	A3 - Controllability: NW decides on if and when to transfer/delivery over the air interface;

•	A4 - Latency: relaxed latency requirement and infrequent update;

•	A5 - Visibility: dataset to be visible and understandable by MNO;

Proposal 4: Adopt additional requirements for both one-sided and two-sided model transfer/delivery.

•	Model transfer/delivery: traffic should be transferred at a different priority, e.g., lower than user traffic.

•	Differentiability: model transfer/delivery traffic should be differentiated from other user traffic.

•	Security: there should be a guarantee that models are transferred securely, in a NW-aware manner, such that untrusted models cannot be downloaded.

•	Addressability: Models need to be addressable such that the UE can request the transfer/delivery of a specific one

•	Controllability: the NW is in control of if and when to transfer/deliver a model

## 8.2	Ambient IoT

(Ambient_IoT_solutions, leading WG: RAN1; REL-19; WID: RP-250796)

Time budget: 2.5 TU

Tdoc Limitation: 3 tdocs

### 8.2.1	Organizational

LS, Rapporteur input, including workplan, etc.

Including outcome of [POST130][027][AIoT] MAC Running CR (Huawei) and [POST130][028][AIoT] 38.300 Running CR (CMCC)

Agreements

The AIoT WI is considered complete from RAN2 point of view

LS to RAN2

R2-2505006	Reply LS on paging ID length (C4-252466; contact: CICT Mobile)	CT4	LS in	Rel-19	Ambient_IoT_Solutions	To:RAN2	Cc:SA2, SA3, RAN3

- -	ZTE is concerned that CT4 didn’t try to restrict the size set. In RAN2 we are fighting over 1 or 2 bits and this has added few hundreds bits.  CATT thinks that the length is flexible and we have the length indicator.  ZTE thinks that the paging message has to support the maximum and we have a restriction of 1000 bits.  Qualcomm agrees with ZTE they even have a 8 bit paging type.  And also we have multiple paging IDs.

-	Xiaomi thinks that they tried but considering that SA3 is also asking for 100 bits.   Huawei thinks that the big overhead is the paging ID itself.

-	Futurewei thinks that compared to paging ID itself the length is small.

Noted

R2-2505040	Reply LS on D2R message size for inventory response (R3-253940; contact: Xiaomi)	RAN3	LS in	Rel-19	Ambient_IoT_Solutions	To:RAN2, SA2	Cc:SA3

Noted

R2-2505058	Reply LS on paging ID length (S2-2505793; contact: CATT)	SA2	LS in	Rel-19	AmbientIoT-ARC, Ambient_IoT_Solutions	To:RAN2, CT4, SA3	Cc:RAN3

Noted

R2-2505059	Reply LS on D2R message size for inventory response (S2-2505849; contact: InterDigital)	SA2	LS in	Rel-19	AmbientIoT-ARC	To:RAN2, RAN3

Noted

R2-2505064	LS on security parameter in paging request message (S3-252392; contact: Huawei)	SA3	LS in	Rel-19	FS_Ambient_IoT_Sec	To:RAN2	Cc:SA2

-	ZTE and CMCC thinks that there is a mode where this is not needed.  It would be nice to not include the 128 bits and wait for SA3.

-	CATT thinks it is feasible to include it but SA3 hasn’t concluded if this is mandatory or not.  We can assume it is optional.

Agreements

- RAN2 thinks it is feasible from a signalling perspective to add the 128 bits.  However, from RAN2 perspective the less overhead the better, so SA3 should avoid adding additional parameters if possible.

- Indicate to SA3 that RAN2 tries to minimize number of bits required.   Have a maximum size of 1000bits, and whatever they include has to fit in the 1000bits considering bits from all TSG.

- RAN2 will wait for SA3 conclusions in October on whether the “128bit random number in the paging request message” is always required or not.

- Reply to SA3

- -     RAN2 thinks it is feasible from a signalling perspective to add the 128 bits.  However, from RAN2 perspective the less overhead the better, so SA3 should avoid adding additional parameters if possible.

- -     Indicate to SA3 that RAN2 tries to minimize number of bits required.   Have a maximum size of 1000bits, and whatever they include has to fit in the 1000bits considering bits from all TSG.

-	Indicate space pressure from all the WG

[AT131][020][AIoT] Reply to SA3 (Huawei)

Intended outcome:

Deadline:  Thursday

R2-2506440	[Draft] Reply LS on security parameter in paging request message	Huawei, HiSilicon	LS out	Rel-19	FS_Ambient_IoT_solutions	To:SA3	Cc:RAN3, SA2

The LS is approved in R2-2506465

8R2-2506465	Reply LS on security parameter in paging request message	RAN2	LS out	Rel-19	FS_Ambient_IoT_solutions	To:SA3	Cc:RAN3, SA2

=> Approved

R2-2506418	LS on delayed A-IoT D2R NAS messages (C1-255165; contact: Huawei)	CT1	LS in	Rel-19	Ambient_IoT_Solutions	To:RAN2	Cc:SA2

Noted

LS with RAN2 in CC

R2-2505005	Reply LS on AIoT device identifier length (C4-252465; contact: Huawei)	CT4	LS in	Rel-19	AmbientIoT-CT	To:SA2	Cc:CT1, RAN2, RAN3, SA3, CT3

Noted

R2-2505033	Reply LS on the removal of service type information (R3-253800; contact: LGE)	RAN3	LS in	Rel-19	AmbientIoT-ARC, Ambient_IoT_Solutions	To:SA2, RAN2	Cc:RAN1

Noted

R2-2505054	Reply to LS on power and energy consumption budget for security features in A-IoT (RP-251886; contact: Huawei)	RAN	LS in	Rel-19	FS_Ambient_IoT_Sec, Ambient_IoT_Solutions-Core	To:SA3, RAN1	Cc:RAN2, SA

Noted

Rel19 CR

R2-2505520	A-IoT MAC running CR	Huawei, HiSilicon	draft TS	Rel-19	38.391	0.0.3	Ambient_IoT_Solutions

Endorse and used as a baseline for next revision

[POST131][021][AIoT] MAC spec (Huawei)

Intended outcome: agree to CR

Deadline:  Short

=> Agreed in R2-2506559 (v0.1.0)

R2-2505951	Introduction of Ambient IoT	CMCC	CR	Rel-19	38.300	18.6.0	1022	-	B	Ambient_IoT_Solutions

[POST131][022][AIoT] Stage 2 CR (CMCC)

Intended outcome: agree to CR with RAN3 CR merged

Deadline:  Short

=> Agreed in R2-2506490 (38.300)

R2-2506490	Introduction of Ambient IoT	CMCC	CR	Rel-19	38.300	18.6.0	1022	1	B	Ambient_IoT_Solutions

=> Agreed

Email Discussion Outputs

R2-2505521	Summary of A-IoT MAC open issues (outcome of [POST130][027][AIoT] MAC Running CR)	Huawei, HiSilicon	report	Rel-19

Noted

Discussions

Proposal 3 (Issue 1-5 Transaction ID in CFRA paging): RAN2 confirms the pervious RAN2 baseline that transaction ID is not included in paging message for CFRA. (23/25)

-	ZTE thinks that if we go this way the consequence is that we can’t use CFRA with command.  We have to use CBRA.  Qualcomm thinks that this is not properly designed as we can’t guarantee what happens down the road and it will create more problems in R20.  Offino also agrees with ZTE, having it mandatory is better but we can consider also having it optional could be an option.

-	Huawei explains that we can just clarify that for single device that reader can use CBRA.

-	Ericsson agrees with Qualcomm that forward compatibility should be taken into account but perhaps there are other ways to solve.

Proposals Add a 7-bit R2D TBS field (in unit of byte) after R2D message type indication in variable-length R2D messages (i.e., Paging message, Random ID Response message, R2D Upper Layer Data Transfer message, NACK Feedback message). (17/20)

-	CATT asks the need to include the TBS at all in RAN2.   How can RAN2 can decode TBS correctly before the CRC check.   Xiaomi indicates that you do it in parallel.

Transaction ID

Proposal 1-2 (Issue 1-2 Transaction ID): RAN2 to make quick decision on 4 bits (15/25) or 6 bits (12/25) of Transaction ID length.

-	ZTE thinks that 6 is minimum and the only argument is to save the 2 bits and we shouldn’t argue for 2 bits.  Xiaomi wants to confirm that we wouldn’t specify how it is generated.

AS ID

Proposal 5 (Issue 3-3 AS ID release): Explicit AS ID release message is not needed. (18/24). [If needed: New message can include no AS ID (to release all devices) or AS ID list (to release one or multiple devices).]

-	ZTE explains that we don’t have RRC connections concept and SA2 is referring to is context between reader and CN.   Xiaomi further explains that the reader will stop transmitting and the device will lose power.

-	Ericsson and Vivo have concerns.  Ericsson is concerned that the paging may come another day.   Xiaomi and Nokia further explains that the reader will stop transmitting and the device will lose power.

-	Mediatek thinks that there is a risk that the device keeps the AS ID as it changes readers, but it doesn’t break the system.   Nokia agrees with Mediatek.

Forward Compatibility Related Issues

Proposal 11 (Issue 4-5 forward compatibility):

-	Add a 1-bit version field in Msg2, NACK feedback message, R2D upper layer data transfer message, and Rel-19 devices drop the whole message when version field is set to 1. (The version field in the Paging message is handled the same way.)

-	Similar to Paging message, Access Trigger message can be extended to add more fields at the end of the message in further releases, and Rel-19 devices ignore the extension parts added in future release instead of dropping the whole message.

-	Qualcomm and Ericsson indicate that we don’t call these types of message R bits, as this is not how Rbits are used in legacy NR.

-	Ericsson thinks that we need a bit to indicate to the R19 UEs whether it should respond to that message or not.   ZTE thinks that in this Rel-19 this bit is set to zero.   Should we specify something on what happens with the bit is set to 1, we can’t specify that now.   We just need to agree how we extend this.    Mediatek thinks that we need to specify what happens with Rel-19 when it sees a bit to 1.   We can model it as an extension to the message type.

-	LG is concerned that with 1 bit version field we can’t introduce new format later on.

-	Qualcomm thinks that a version bit is not useful as we can just use a new message type.

-	Interdigital explains that we have both levels of flexibility, dropping completing the message (first part of proposal) and second possibility where we can re-use Rel-19 message with some extension.

-	Qualcomm thinks that we can just extend message type by 1 bit.  ZTE thinks that this allows extension per message, critical vs. non-critical.

Proposal 6 (Issue 3-5 D2R message type): RAN2 confirms D2R message type is not introduced in this release (Rationale: no valid requirement, as DOA coexist with R19 device is excluded in R20 WID). (20/25)

-	Apple thinks there is a benefit to use in R19, especially for zero message and that doesn’t carry any data.  So if we introduced a new message type then we don’t need to care about the next issue.  Huawei explains that it would require to add a byte.

-	Oppo explains that from RAN1 perspective no co-existing means not dealing with interference but from RAN2 perspective we shouldn’t preclude the new message type.  Xiaomi indicates that the note was added to avoid RAN2 discussions.   Huawei thinks that without RAN1 there is no point for RAN2 to discuss.

-	ZTE thinks that from a protocol perspective it would be beneficial for the reader to get the message type and this would avoid the reader having to process the messages in unexpected ways and remember the state.  Qualcomm, Offinno, ETRI and Nokia agrees, and even if DOA co-existance is excluded from deployment point of view there is nothing to exclude this from happening.   CCMC thinks that the reader would know what to expect.  ZTE thinks that it is not a good protocol design.

-	LG thinks that there are exceptional cases to be expected so have similar view with ZTE and Qualcomm.   Mediatek agrees with ZTE and has some concerns that we are building a brittle system.  If everything goes well by having state knowledge the reader can figure out. However we have made assumptions that may not be necessarily true in real deployments.

-	Vivo thinks we should keep it simple.

-	Interdigital thinks that we spend a lot of time ensuring forward compatibility for R2D and we should do the same for D2R.

-	Sony, Lenovo, agrees with Huawei.

-	Samsung asks if we can consider to make it extendable for future releases.    CATT agrees as for Rel-19 there is not need but for R20 we have a mechanism to extend for future releases.  Huawei thinks that we have ways to do this as we have extra bits on the SDU length.

-	Huawei explains that it doesn’t come for free as we have to add one byte.   Apple and ZTE thinks it is only 2 bits.

-	Mediatek and ZTE think that even if we have an extra byte it is not a critical problem as this UL message anyways typically carries data.

-	Oppo thinks that we anyways have to add 128 bits for security so adding one extra byte is not a critical problem.

-	Huawei, Lenovo,  and CMCC have concerns that we are adding overhead for no reason for Rel-19 without knowing the use cases of Rel-20.

Proposal 10 (Issue 2-3 trigger message alignment): Access Trigger message is bit-aligned (no padding bits). (16/21)

-	3 bit message

More Data Indication (MDI)

Proposal 8 (Issue 3-7 More data indication): For no data available case, RAN2 to quick decide either to set more data indication to "1" (9/23) or "0" (15/23). (Note: reader behavior is expected to be same either way).

-	Futurewei thinks that we have a legitimate case where we have a zero SDU case with nothing else expected.  And we have the usual zero SDU but data arrives later.   LG agrees as there can be integrity failure and in this case the device should indicate no more data indication.  Qualcomm thinks that even the NAS drops something it should create a response.   Futurewei thinks that if it doesn’t understand the message it will just drop it.

Agreements

1	RAN2 confirms how to generate transaction ID is left to reader (no spec impact)

2	8-bit length field (in unit of bit) is assumed to indicate the paging ID length, based on current SA2/CT4 conclusion.

3	RAN2 confirms the pervious RAN2 baseline that transaction ID is not included in paging message for CFRA.  Clarify that CBRA can be used by reader for single device.

4	No entry number is included in either Msg2 or NACK feedback message.  RAN2 understands that device can decode the entries one by one till message end, other implementations are not precluded (we will not capture this in the spec).

5	R2D TBS information is not included in the Access Trigger message.

6	Add a 7-bit R2D TBS field (in unit of byte) after R2D message type indication in variable-length R2D messages (i.e., Paging message, Random ID Response message, R2D Upper Layer Data Transfer message, NACK Feedback message).

7	6 bits for Transaction ID length.

8	Explicit AS ID release message is not needed.

9	For forward compatibility:

-	Paging message can be extended to add more fields at the end of the message in further releases, and Rel-19 devices ignore the extension parts added in future release instead of dropping the whole message, without extension indication. Future extension using the same message type is not supported for R2D messages other than paging message.

-	No version bit will be introduced

-	Remove the R-field in paging message from the running CR

-	Use 3-bit R2D message type.

10	A 2 bits D2R message type is introduced in this release.  For Rel-19 only one message type exists for D2R message.  RN16 doesn’t include message type as already agreed.

11	Access Trigger message is 3 bits and no padding bits are added (i.e. not byte aligned)

R2-2506441	Outcome of A-IoT forward compatibility offline	Huawei	discussion	Rel-19	FS_Ambient_IoT_solutions

Noted

Proposal 2b: FFS if to extend 3-bit R2D message type to 4 bits:

Option 1-1: Use 3-bit R2D message type. Use reserve values first if overhead is more critical. Can use a reserved value as future indication like eLCID, if needed. (Xiaomi, Huawei, ZTE, DCM, Ericsson, Lenovo, IDC, MTK, ETRI, vivo)

Option 1-2: Use 4-bit R2D message type. (Apple, SS, LG, QC, Nokia)

-	ZTE is concerned with adding extra bits in in access trigger.

### 8.2.2	A-IoT Paging

Contributions should focus on paging message content and format, including paging identifier details, transaction ID details, resource information details , end-of procedure,  etc.

Handling SA3 LS on Security [Monday]

R2-2505245	Discussion on paging procedure for Ambient IoT	OPPO	discussion	Rel-19	Ambient_IoT_Solutions

Proposal 1: Rely to SA3 that:

	It is feasible to include additional 128 bits security parameters in paging message.

	The maximum size of paging message is 1000 bits.

	It is difficult to calculate the maximum bits which can be used for security parameters, as the paging message is used to include both the Rel-19 paging fields and future extension fields. We should avoid sending too many bits of security parameters according to the current paging message size.

Visibility of paging ID

R2-2505679	Remaining issues on paging for Ambient IoT	Lenovo	discussion	Rel-19

Proposal 3 (Issue 1-7): RAN2 to confirm the working assumption that the paging identifier is transparent to the A-IoT MAC layer and can be carried by upper layer.

Noted

R2-2505091	Discussion on Paging for Ambient IoT	CATT	discussion	Rel-19	Ambient_IoT_Solutions

Proposal 3: (1-6) The paging ID is visible to the AIoT MAC layer.

Noted

The paging ID is visible to the reader.  No specification impact.

Encoding of number of access occasions

R2-2505121	Remaining open issues on A-IOT paging procedure	Xiaomi	discussion	Rel-19	Ambient_IoT_Solutions

Proposal 1a: The number of access occasions is introduced as linear value instead of exponential way.

Proposal 1b: If proposal 1a is not acceptable, the device should adjust the number of AOs to the smallest larger or equal multiple of X*NSFS before performing the AO selection

Noted

R2-2505196	Remaining issues on AIoT Paging	vivo	discussion	FS_Ambient_IoT_solutions

Proposal 1.	(Issue1-4) RAN2 to revisit the agreement and to introduce the number of resource sets (a set of resources following Paging message/Access trigger messages) instead of the number of access occasions in the paging message, via 10-bit linear encoding way.

Noted

R2-2505429	Remaining open issues on paging	ETRI	discussion	Rel-19

Proposal 4. The number of AO field has a 4-bit length and its value is interpreted using an exponential mapping.

Noted

Discussion

-	LG thinks we should keep the current way it is the in CR.  Qualcomm thinks that we have a problem with existing CR.

After offline

-	ZTE thinks that we should back to linear and 15 bits as 4 bit exponential doesn’t provide enough granularity.  Huawei agrees and we should either do linear or go back to previous meeting agreements.

-	Qualcomm thinks that we would many random generators if we go for linear.   Apple doesn’t agree.

Agreements

1	Keep current agreement.   The reader should provide enough access trigger to cover at least signalled AOs in current round, unless the reader choses to start the subsequent paging round.    Capture in stage 2 and rapporteur will work in the wording.

Additional Information in Access Trigger Message

R2-2505447	Discussion on Paging for Ambient IoT	Apple	discussion	Rel-19	Ambient_IoT_Solutions

Proposal 2 	The 5 LSB of “Access Slot number” is included in Access Trigger message.

Noted

R2-2505768	Discussion on the remaining issues on A-IoT paging	Samsung	discussion	Rel-19	Ambient_IoT_Solutions-Core

Proposal 1: RAN2 is kindly asked to discuss whether the configuration mismatch between reader and device exists due to the miss of the A-IoT paging message with the updated configurations (i.e., number of access occasions, D2R scheduling info. ).

Proposal 2: RAN2 is kindly asked to discuss whether and how to address the configuration mismatch issue, and, if needed, the following options can be considered:

	Option 1: do nothing

	Option 2: no configuration update among A-IoT paging message with the same Transaction ID

	Option 3: add configuration ID in A-IoT paging message and Access trigger message

Noted

R2-2505864	Remaining aspects of Ambient IoT Paging	Qualcomm Incorporated	discussion	Rel-19	Ambient_IoT_Solutions-Core

Proposal 2:	RAN2 confirms that R2D trigger message does not include slot number/count down number.

Noted

Discussions

-	Xiaomi thinks that the problem in apple and Samsung can agree but the problem is that it is quite late so we may not need to address this in R19.   Docomo agrees with qualcomm.   CMCC also thinks it is an optimization and we should keep it simple.

-	Samsung is concerned with interference problem if there is both UL and DL transmissions at the same time.  This will impact the efficiency of the whole system.

-	Apple thinks that with their solution it would solve Samsungs problem as well. And thinks that option 2 is not feasible.

-	ZTE asks what cannot be changed with option 2.   Samsung explains that X and Y shouldn’t be changed but number of AOs can change.   ZTE thinks that the reader shall be able to change the number of AOs and this can be done by either changing X and Y or changing number of access triggers.

Option 2: no configuration update for X or Y among A-IoT paging message with the same transaction ID

-	Vivo thinks that there is a problem with different transaction ID.  Samsung thinks option 3 solves that.

-	Huawei thinks that even if we do nothing the consequence is only for one device failure and then they will do re-access.  Xiaomi explains that this impacts the system.

-	ZTE agrees with the problem but thinks the solution is worst than the problem.    Adding bits to the trigger message is not a good solution.

Agreements

- RAN2 confirms that R2D trigger message does not include slot number/count down number in this release

- RAN2 acknowledges the problem, and it is up to reader implementation to avoid the mismatch between reader and device due to the miss of the A-IoT paging message with updated configurations in this release.  RAN2 will not add any explicit information in this release to address this problem

Not treated

R2-2505181	Discussion on Paging for A-IoT	Transsion Holdings	discussion	Rel-19

R2-2505263	Remaining issues on A-IoT Paging	Ofinno	discussion	Rel-19

R2-2505313	Paging consideration for forward compatibility of Device 1	Panasonic	discussion	Rel-19	Withdrawn

R2-2505375	open issues for paging	ZTE Corporation, Sanechips	discussion

R2-2505414	Forward Compatibility and Remaining Paging Details	InterDigital	discussion	Rel-19	Ambient_IoT_Solutions

R2-2505522	A-IoT paging	Huawei, HiSilicon	discussion	Rel-19

R2-2505565	Open issues on AIoT paging	Nokia	discussion

R2-2505613	Remaining issues on A-IoT paging	Sharp	discussion

R2-2505614	Discussion on security parameter in paging message	KT Corp.	discussion

R2-2505649	Discussions on ambient IoT paging  	ROBERT BOSCH GmbH	discussion	Rel-19

R2-2505651	Discussion on MAC Open issues on Paging	Sony	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2505727	Remaining issues on A-IoT paging	ITL	discussion

R2-2505852	Paging consideration for forward compatibility of Device 1	Panasonic	discussion	Rel-19

R2-2505909	A new issue on AIoT Paging	Futurewei	discussion	Rel-19	Ambient_IoT_Solutions

R2-2505935	Discussion on remaining issues on AIoT paging	NTT DOCOMO INC.	discussion	Rel-19

R2-2505950	Discussion on A-IoT paging	CMCC	discussion	Rel-19	Ambient_IoT_Solutions

R2-2506005	Discussion on A-IoT paging	LG Electronics Inc.	discussion	FS_Ambient_IoT_solutions

R2-2506061	Discussion on A-IoT paging	HONOR	discussion	Rel-19	Ambient_IoT_Solutions

R2-2506094	Discussion on DL messages for Ambient IoT UEs	Ericsson	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2506116	Discussion on Ambient-IoT paging	III	discussion

### 8.2.3	A-IoT Random Access

Contributions should focus on details of contention-based and contention-free access, including re-access for failure handling and MSG2 boundary, msg content/format, format (R2D trigger message,  Msg1, Msg2, NACK based feedback for re-access, etc.)

MSG2 Boundary and Frequency Index

R2-2505415	Remaining Random Access Issues	InterDigital Inc., Huawei, HiSilicon, Xiaomi, Fujitsu, Ofinno, ETRI, Lenovo, NEC, Ericsson, China Telecom, Mediatek	discussion	Rel-19	Ambient_IoT_Solutions

Proposal 1a:	Use option C for CBRA failure boundary: “The boundary is the reception of either the kth Access trigger message or the subsequent paging message”

Proposal 1b:	RAN2 discuss whether to use fixed value of K (e.g., K=2 or K=4) or a configurable value in option C.

Proposal 2: 	3-bit frequency index is included with each echoed random ID in MSG2.

Noted

R2-2505769	Discussion on the remaining issues on A-IoT RA procedure	Samsung	discussion	Rel-19	Ambient_IoT_Solutions-Core

Proposal 1: RAN2 is kindly asked to agree that the CBRA failure can be detected when the device receives the next Access Trigger message after Msg.1 transmission.

Proposal 2: RAN2 is kindly asked to agree that the frequency index in Msg. 2 is not needed for Msg.1 collision resolution.

Noted

Discussion

On boundary

-	Qualcomm, Vivo, thinks we increase complexity with option c without getting much value.

-	Vivo doesn’t want a configurable value.  Huawei and CMCC thinks we can make it fixed and complexity is not a bit problem.

-	Mediatek doesn’t see a real complexity problem as when you receive message 2 you always parse ms2 and the processing is the same whether it arrives in first or other windows.   Yes there is a counter that needs to be used but that can be done.  Qualcomm doesn’t think it is the same complexity, as if it get it in a window I’m not supposed to receive a msg2 I would kill the msg2.   Mediatek agrees what we would drop it, but the counter just extends the time you expect message 2.

-	ZTE thinks that this will increase collision probability as we increase K.  From reader perspective we would transmit message within the same trigger as there is no reason to keep it longer.   Huawei thinks that with RN16 we have very very low collision probability and even if increasing K we are increasing collision by 4, which is still very low.   Xiaomi agrees with Huawei.

-	Apple thinks that K is better for device and larger better for reader.  Suggests compromise to set it to 2.

-	ZTE would like a configurability to off or on with K=4.  Interdigital thinks that this configurability adds more complexity.

-	ZTE is concerned that now implementations that would do K=1 are penalized.

Frequency index

-	CMCC thinks it should be a bitmap.  Samsung thinks that we should first discuss the need for the frequency index.

-	Xiaomi wants the index even with k=1

-	MEdiatek the issue is for the pool of devices what is the probability of collision.  With 16 bits device ID and 64 devices you have a 3% collision probability for these devices.  .

R2-2506431	Comeback on Random Access and Access Occasion Number	InterDigital	discussion	Rel-19	Ambient_IoT_Solutions

Comeback agreements in 8.2.3

As a baseline, the boundary is the reception of either the kth Access trigger message or the subsequent paging message.  K is fixed to 4.   Reader implementation to send MSG2 immediately (before k) is not excluded.

-	ZTE thinks that the other implementation is essential and only way to agree is to make it configurable between 1 and 4.   Not only the failure case is problematic but also the success case as msg 2 can be repeated.   And we are finding more and more problem with K1.   CMCC thinks that ZTE comment is valid.   Qualcomm thinks that if K=4 the reader will delay the msg2 sending to the 4th window.    ZTE thinks that all other RACH parameters are configurable.

-	Vivo is concerned with K configurable.

	RAN2 to revisit the agreement and to introduce the number of resource sets (a set of resources following Paging message/Access trigger messages) instead of the number of access occasions in the paging message, via 4-bit exponential to indicate the number of Access trigger messages.  The maximum number of access occasions supported is limited to 2^15.

Agreements

- 3-bit frequency index is optionally included with each echoed random ID in MSG2.  We have 1 bit in MSG2 to indicate presence/absence of the frequency information for all included RN16s.

- The boundary is the reception of either the kth Access trigger message or the subsequent paging message. Reader implementation to send MSG2 immediately (before k) is allowed.    K can be configured to be either 1 or 4 in paging message.

Not treated

R2-2505092	Discussion on A-IoT random access	CATT	discussion	Rel-19	Ambient_IoT_Solutions

R2-2505122	Remaining open issues on access procedure for A-IOT	Xiaomi	discussion	Rel-19	Ambient_IoT_Solutions

R2-2505182	Discussion on Random Access for A-IoT	Transsion Holdings	discussion	Rel-19

R2-2505197	Remaining issues on AIoT Random Access	vivo	discussion	FS_Ambient_IoT_solutions

R2-2505264	Remaining issues on A-IoT Random Access	Ofinno	discussion	Rel-19

R2-2505376	open issues for random access	ZTE Corporation, Sanechips	discussion

R2-2505378	Discussions on ambient IoT random access  	ROBERT BOSCH GmbH	discussion	Rel-19

R2-2505416	On Byte Alignment of the Access Trigger Message	InterDigital, Apple, vivo	discussion	Rel-19	Ambient_IoT_Solutions

R2-2505448	Discussion on Random Access for Ambient IoT	Apple	discussion	Rel-19	Ambient_IoT_Solutions

R2-2505574	Discussion on remaining open issues on random access for A-IoT	OPPO	discussion	Rel-19	Ambient_IoT_Solutions

R2-2505576	Discussion on random access aspects of Ambient IoT	KT Corp.	discussion	Rel-19	Ambient_IoT_Solutions

R2-2505615	Msg2 monitoring window for A-IoT random access	Panasonic	discussion	Rel-19

R2-2505628	A-IoT random access procedure	Huawei, HiSilicon	discussion	Rel-19	Ambient_IoT_Solutions

R2-2505652	Random Access msg 2 monitoring configuration	Sony	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2505680	Remaining issues on random access for Ambient IoT	Lenovo	discussion	Rel-19

R2-2505709	Discussion on left issues of A-IoT random access	Spreadtrum, UNISOC	discussion	Rel-19

R2-2505728	Remaining issues on random access for A-IoT	ITL	discussion

R2-2505746	Handling of Msg2	Continental Automotive	discussion

R2-2505793	Discussion on UL multiple access	Ericsson	discussion	Rel-19	Ambient_IoT_Solutions

R2-2505818	Views on Random Access Aspects of Ambient IoT	Qualcomm Incorporated	discussion	Ambient_IoT_Solutions-Core

R2-2505910	Remaining issue on AIoT Random Access	Futurewei	discussion	Rel-19	Ambient_IoT_Solutions

R2-2505923	Open issues on random access for AIoT	Nokia	discussion	Ambient_IoT_Solutions

R2-2505969	Remaining open issues on random access for Ambient IoT	CMCC	discussion	Rel-19	Ambient_IoT_Solutions

R2-2505972	Remaining issues on random access in Ambient IoT 	Kyocera 	discussion	Rel-19

R2-2505981	Discussion on remaining issues on AIoT random access	NTT DOCOMO INC.	discussion	Rel-19

R2-2506004	Discussion on random access aspects for Ambient IoT	LG Electronics Inc.	discussion	FS_Ambient_IoT_solutions

R2-2506028	Discussion on Ambient IoT random access	ASUSTeK	discussion	Rel-19	Ambient_IoT_Solutions

R2-2506062	Discussion on A-IoT random access	HONOR	discussion	Rel-19	Ambient_IoT_Solutions

R2-2506167	Discussion on A-IoT Random Access	CEWiT	discussion

### 8.2.4	A-IoT Data Transmission and Other general aspects

Contributions should focus on MAC PDU/signaling general format (the content of paging, Msg1, Msg2 etc. should be discussed in above specific agendas), TBS size and byte alignment (based on RAN1 discussion), message details for segmentation for D2R, data not available case (pending CT1 inputs), segmentation for D2R, AS ID\

More Data Indication (MDI)

Not treated

R2-2505123	Remaining open issues on Data transmission	Xiaomi	discussion	Rel-19	Ambient_IoT_Solutions

Proposal 1: (Issue 3-7) RAN2 to agree that the more data indication is a mandatory field (without specification impact) and should be set to “1” for the case of zero SDU.

R2-2505924	Discussion on A-IoT data transmission	LG Electronics Inc.	discussion	Rel-19	Ambient_IoT_Solutions

Proposal 6. More Data Indication (MDI) field is set to 0 if upper layer data is not available for transmission.

R2-2505377	open issues for data transmission	ZTE Corporation, Sanechips	discussion

Proposal 2 (Issue 3-7):  The setting of “more data indication” can be left to implementation in case zero-sized upper layer SDU is included in the MAC PDU.

Follow-up discussion on CT1 LS

R2-2505523	A-IoT data transmission	Huawei, HiSilicon	discussion	Rel-19

Proposal 3:	(Issue 3-6) No further RAN2 standard action is needed, regardless the CT1 reply:

	In case CT1 concludes “successful execution” as the D2R response for write, implementation behaviors can be the following:

	Reader sends another R2D upper layer data message later (if D2R with “zero SDU” is received);

	Device implementation avoids the redundant write operation (by identifying the same command), and directly sends the actual D2R response.

	Otherwise, no further RAN2 action is needed either (i.e., the issue is addressed by CT1).

Noted

R2-2505913	Discussion on handling the delayed NAS response	Futurewei	discussion	Rel-19	Ambient_IoT_Solutions

Proposal 3. The reader, in response to 0 SDU and MDI=1 in the device’s MAC response, will send a follow-up R2D Upper Layer Data Transfer message at a later time to schedule another D2R Upper Layer Data Transfer message from the device. The follow-up R2D Upper Layer Data Transfer message includes the Received Data Size field with the Received Data Size field set to value 0, without including the original command.

Proposal 4. If the delayed NAS response becomes available at the MAC layer of the device, the device responds to the follow-up R2D Upper Layer Data Transfer message by sending a D2R Upper Layer Data Transfer message with 1 SDU (where the MDI is set based on the segmentation status); otherwise, if the delayed NAS response is still unavailable at the MAC layer of the device, the device sends a MAC response with 0 SDU and MDI=1 again.

Noted

R2-2505819	Data Transmission and Other General Aspects of Ambient IoT	Qualcomm Incorporated	discussion	Ambient_IoT_Solutions-Core

When there is no NAS response available to transmit at the D2R occasion, besides 0 SDU indication to Reader, A-IoT device sends to Reader an estimated time about when the NAS command response will be ready.

Noted

Discussion

-	Huawei, Mediatek and Xiaomi think that we can agree with not sending the original command.

-	Apple wonders how the reader knows when it should send the follow up scheduling to retrieve the data.   Mediatek indicates that later means later and it will be a re-iterative process.

-	Apple thinks that we are using the offset zero.  Interdigital explains that this is aligning the behaviour with segmentation.

-	CATT ask how gNB ends this re-iterative scheduling.    Xiaomi explains that this is reader implementation.   Mediatek explains that other layers and timers will handle this.

-	Qualcomm confirms what Futurewei was arguing on the fact the CT1 has cases where no response occurs.   Xiaomi would like to do some homework before discussing this this meeting.   ZTE thinks this is a problem created by NAS so it should be solved by them.   Qualcomm thinks that this creates a problem as the readers will keep sending scheduling in hopes that the UE will respond and this is a bad design.   Interdigital agrees we should address and we have a RAN2 solution where we can distinguish the cases between delayed response and no response.   MEdiatek thinks that we should get clarification from CT1.  We have a solution were zero SDU means delayed response, but doesn’t work for the case where there is no response at all.

-	Huawei, Samsung thinks we need some time to think and check.

-	Qualcomm thinks that we should focus on the whole system, like for example telling the reader what to expect, time can be set to infinity for this case and work for other cases.

Agreements on no data available due to delay in NAS

- The reader determines no data available case by SDU length 0.   As more data indication is mandatory, the device sets this bit to "0".

- The reader, in response to 0 SDU in the device’s MAC response, may send a follow-up R2D Upper Layer Data Transfer message at a later time to schedule another D2R Upper Layer Data Transfer message from the device.

- The follow-up R2D Upper Layer Data Transfer message includes the Received Data Size field with the Received Data Size field set to value 0, without including the original command.

- RAN2 would like to check if there is a case where NAS doesn’t provide a response at all.   If this case exists, RAN2 will discuss this issue.

AS ID Release Conditions

R2-2505829	Data transmission and other aspects in AIoT	Ericsson	discussion	Rel-19	Ambient_IoT_Solutions_Ph2-Core

Proposal 3	Upon reception of NACK message addressed to the device, its AS ID is released

Upon reception of NACK message addressed to the device, its AS ID is released

Noted

Byte Alignment

R2-2505523	A-IoT data transmission	Huawei, HiSilicon	discussion	Rel-19\

Noted

Proposal 6:	To ensure byte alignment for the variable size R2D message:

	Paging and Msg2 (Variable bit length): add one “fill field” in the end of the message (1~7bits).

	NACK feedback (AS ID entry(ies) self-aligns, message type is 3-bit fixed): add 5 R-bit after message type field.

	R2D upper layer data message (data SDU self-aligns, other fields are of fixed bits): add R bits after CI field to differentiate R-bit number when data SDU or received data size is present. Confirm MAC padding field and SDU length field are not needed.

-	Docomo and Qualcomm are concerned that we will not be able to multiplex multiple messages in the same PDU.

Agreements

To ensure byte alignment for the variable size R2D message:

- Paging and Msg2 (Variable bit length): add one “fill field” in the end of the message (1~7bits).

- NACK feedback (AS ID entry(ies) self-aligns, message type is 3-bit fixed): add R-bit(s) after message type field.

- R2D upper layer data message (data SDU self-aligns, other fields are of fixed bits): add R bits after CI field to differentiate R-bit number when data SDU or received data size is present. Confirm MAC padding field and SDU length field are not needed.

- R bit is set to zero in this release and ignored by the receiver.

- What’s included in the fill field is not specified, but device ignores the fill field.

- When a single D2R resource is signaled in R2D upper layer data transfer message , use 3-bit field to represent “Frequence Resource Indication” instead of the 8 bit bitmap.

Formatting of Scheduling Information

R2-2505449	Discussion on A-IoT MAC message formats and data transmission	Apple	discussion	Rel-19	Ambient_IoT_Solutions

Proposal 1 	Use a 9-bit field to jointly code “Bit Duration” and “ Frequence Resource Indication “ for D2R scheduling Information, if bitmap is used to indicate frequency resource.Proposal 2 	When a single D2R resource is signaled in R2D message, use 3-bit integer to represent “Frequence Resource Indication”.

Noted

R2-2505768	Discussion on the remaining issues on A-IoT paging	Samsung	discussion	Rel-19	Ambient_IoT_Solutions-Core

Proposal 4: RAN2 is kindly asked to use separate fields for bit duration and frequency resource indication (as the current running CR).

Noted

Discussions

-	Xiaomi, Vivo, Ericsson and LG thinks that if we have 9 bits it would add complication on interpretation of it.   Qualcomm thinks that from an engineering point of view 9bit is a very efficient and RAN1 has designed efficient signaling for everything else, so RAN2 should also strive for efficiency.

-	NEC also agreed with Apple and it comes for free.

-	Oppo thinks that we can use R bits for the future so agrees with Samsung.

-	Vivo thinks that this may cause us to add separate bits for CFRA and CBRA.

-	ZTE thinks proposal 2 should be supported and 9 bits reduces the signaling cost.

-	Huawei thinks that two different formats are making things complicated so we can chose one unified format for both CFRA and CBRA.  Apple and ZTE explains that the second proposal can be used for both dedicated and command messages.   Qualcomm thinks that CBRA paging is the bigger one with multiple resources so saving the two bits can save you a lot.  ZTE has a different view as for dedicate messages you are sending a lot of messages so it would bring a lot more gains.   Qualcomm explains that the reader has to provide 10000 resources.  Huawei agrees with ZTE that you have more gains with dedicated messages.

Energy Status Indication

R2-2506048	Ambient IoT data transmission and energy status indication	MediaTek Inc., Vodafone, Nokia, Samsung, NTT DOCOMO INC., InterDigital, T-Mobile USA	discussion	Rel-19	Ambient_IoT_Solutions-Core	R2-2506046

Proposal 1: Specify a mechanism for the device to indicate to the reader if the device can complete a requested command procedure when considering its energy status and radio conditions (taking into account both R2D and D2R directions, and with no requirement on the reader to take any specific action on the indication).

-	Xiaomi and Ericsson thinks that this is not in scope and anyways it is more useful for R20 where the UE has battery.   Offino thinks that it may be a little bit late as we are closing the WI and we can think of doing it as a TEI.

-	Mediatek sees value for passive devices as well as there are devices that are aware of their energy status.

Can consider it for Rel-20

Noted

Not treated

R2-2505093	Discussion on the A-IoT Data Transmission	CATT	discussion	Rel-19	Ambient_IoT_Solutions

R2-2505156	Left issues for Data transmission in A-IOT	Transsion Holdings	discussion	Rel-19

R2-2505198	Remaining issues on AIoT Data Transmission	vivo	discussion	FS_Ambient_IoT_solutions

R2-2505265	Remaining issues on A-IoT Data Transmission	Ofinno	discussion	Rel-19

R2-2505314	AS ID release upon receiving paging message	Panasonic	discussion	Rel-19	Withdrawn

R2-2505371	Discussions on ambient IoT data transmission 	ROBERT BOSCH GmbH	discussion	Rel-19

R2-2505417	Remaining Aspects on Data Transmission	InterDigital	discussion	Rel-19	Ambient_IoT_Solutions

R2-2505430	Remaining open issues on data transmission	ETRI	discussion	Rel-19

R2-2505561	Open issues in AIoT 	Nokia	discussion

R2-2505570	Ambient-IoT Data transmission	NEC	discussion	Rel-19	Ambient_IoT_Solutions-Core

R2-2505573	Discussion on left FFS issues on A-IoT Data Transmission	OPPO	discussion	Rel-19	Ambient_IoT_Solutions

R2-2505653	Energy level indication	Sony	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2505681	Remaining issues on data transmission for Ambient IoT	Lenovo	discussion	Rel-19

R2-2505742	AS ID release procedure for CFRA 	Continental Automotive	discussion

R2-2505770	Discussion on the remaining issues on A-IoT data	Samsung	discussion	Rel-19	Ambient_IoT_Solutions-Core

R2-2505851	AS ID release upon receiving paging message	Panasonic	discussion	Rel-19

R2-2505941	Discussion on the remaining issues of data transmission for A-IoT	CMCC	discussion	Rel-19	Ambient_IoT_Solutions

R2-2505973	Remaining issues on data transmission in Ambient IoT 	Kyocera 	discussion	Rel-19

R2-2506029	Discussion on Ambient IoT data transmission	ASUSTeK	discussion	Rel-19	Ambient_IoT_Solutions

R2-2506046	Ambient IoT data transmission and energy status indication	MediaTek Inc., Vodafone, Nokia, Samsung, NTT DOCOMO INC., InterDigital	discussion	Rel-19	Ambient_IoT_Solutions-Core

=> Revised in R2-2506048

R2-2506057	Discussion on remaining issues on AIoT data transfer	NTT DOCOMO INC.	discussion	Rel-19

R2-2506123	Remaining issues on A-IoT data transmission	Fraunhofer HHI, Fraunhofer IIS	discussion

## 8.3	AI/ML for Mobility

(FS_NR_AIML_Mob; leading WG: RAN2; REL-19; SID: RP-242393)

Time budget: 2 TUs

Tdoc Limitation: 4 tdocs

### 8.3.1	Organizational

LS, Rapporteur input, including workplan, etc.

Including outcome of [POST130][021][AI Mob] TR update (Oppo)

Agreements

The SI is considered completed from RAN2 point of view.

R2-2505161	Text proposal of TR 38.744 to capture spec impact_v15	OPPO	pCR	Rel-19	38.744	1.1.0	FS_NR_AIML_Mob

The TP is endorsed and will be used as baseline for further updates

[POST131][032][AI Mob] TR (Oppo)

Intended outcome: Agree to final TR to be submitted to plenary

Deadline:  Short

=> Agreed in R2-2506659 (TR 38.744)

R2-2506659	Study on Artificial Intelligence (AI)/Machine Learning (ML) for Mobility in NR	OPPO	draftTR	Rel-19	38.744	1.2.0	FS_NR_AIML_Mob

=> Agreed

R2-2505184	Text proposal of TR 38.744 on model complexity	OPPO	pCR	Rel-19	38.744	1.1.0	FS_NR_AIML_Mob

-	Huawei thinks that we can capture CDF as it is unclear what majority means

-	Apple would like to see a Note to capture that some simple models which perform well

-	Qualcomm would like to ensure that we don’t use this information to decide on UE capabilities.  Oppo confirms it is only informational.

Agree with intention to capture this information and further check offline how

Check whether and how to capture the simple AI model

R2-2505185	Text porposal of TR 38.744 on conclusion of AI mobility SID	OPPO	pCR	Rel-19	38.744	1.1.0	FS_NR_AIML_Mob

[AT131][033][AI Mob] Conclusions for TR (Oppo)

Intended outcome: Identify and agree to possible conclusions.

Deadline:  Thursday

R2-2506468	Summary of [AT131][033][AI Mob] Conclusions for TR (OPPO)	OPPO	discussion	Rel-19	FS_NR_AIML_Mob

The TP will be directly included in the TR and will take the changes agreed online into account.

### 8.3.2	UE sided model

Contributions should be submitted in 8.3.2.x.

#### 8.3.2.1	Functionality management for RRM measurement prediction

Including applicability procedure, inference configuration/reporting and performance monitoring for UE sided model for RRM measurement prediction

Supported Use cases:

R2-2505129	Recommendation on specification for L3 beam-level prediction	vivo, Qualcomm Incorporated, Huawei, HiSilicon, CATT, ZTE, Ericsson, CMCC, NTT DOCOMO, Interdigital	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 1: Capture in the TR that the concluded potential specification impact of RRM measurement prediction is applicable for both L3 cell-level and beam-level predictions unless otherwise explicitly described.

Proposal 2: From RAN2's perspective, for RRM measurement prediction of both UE- and network-sided models, support of L3 beam-level prediction is recommended for normative work.

-	Oppo indicates that RAN4 only considered cell-level. Apple wonders why this is not submitted in RAN4.   Huawei indicates that RAN4 is waiting for us, but there are no feasibility issues.

-	Mediatek is concerned about the UE sided feasibility as UE implementation complexity will be doubled.  But not for network sided.   Also we don’t have beam level simulations.

-	Ericsson thinks its not about just feasibility but it also a matter of the solution for interfrequency case working.   The current mobility would work as the network is not aware on which beam it should configure the UE.    Samsung and Apple thinks that even without this the network can configure the RA resources for all the beams, so nothing is broken.   Ericsson then we would need to implement two solutions.    Nokia thinks that this would be nice to have, even though nothing is broken.

-	Xiaomi that even if we allow beam level prediction it will be a UE capability so the network will have to handle UEs with both capabilities.

-	Vivo thinks that mobility will work without beam level but the performance of AI prediction will be reduced.

-	Samsung is concerned for network-sided temporal case A it would result in very large signaling delay.

Noted

Agreements on L3 beam-level prediction

For RRM measurement prediction, L3 beam-level prediction is feasible for network sided model.  CB after spec impact/overhead etc on the different cases.

For RRM measurement prediction, L3 beam-level prediction is feasible for UE sided model.  However, there are concerns on complexity and other WGs.

R2-2505151	Discussion on functionality management for RRM prediction	Xiaomi	discussion

Proposal 7: The temporal domain L3 beam-level measurement prediction can be considered, where future or latest L3 beam-level measurement result(s) can be predicted.

Proposal 8: The frequency domain L3 beam-level measurement prediction is not considered in Rel-19

Noted

Applicability determination/reporting:

R2-2505128	Discussion on functionality management for RRM measurement prediction	vivo	discussion	Rel-19	FS_NR_AIML_Mob

Noted

R2-2505440	UE-sided measurement prediction	Apple	discussion	Rel-19	FS_NR_AIML_Mob

Noted

Agreements 1. Confirm that as a baseline following agreements on applicability reporting in AI-based beam management are applicable for AI mobility: - UE may include “release configuration” flag in applicability reporting to indicate UEs preference to release a non-applicable configuration. - Introduce a flag in OtherConfig indicating whether applicability reporting via UAI is enabled or disabled. - When UE indicates that an inference configuration is not applicable, the gNB is expected to release the configuration (i.e., UE autonomous release is not supported). - The UE continues to perform the inference and reporting until the configuration is released. It is up to network implementation what to do with UE reported predicted values after UE indicates that an inference configuration is not applicable. - The UE shall report when an inference configuration becomes non-applicable. - How to handle RRC configuration in IDLE/INACTIVE/RLF, follow the legacy UE behaviour in TS 38.331 on whether to release or keep the RRC configuration. - Whether Option A and Option B can be configured in the same RRCReconfiguration message with the unified applicability report procedure. - RAN2 assumes applicability report for Option B (sets of inference related parameters) can be included in both RRCReconfigurationComplete and UAI (i.e., same as Option A). NOTE: these agreements will be aligned with AI PHY agreements at the end of this week and will not be re-discussed

Inference configuration/reporting:

R2-2505357	Discussion on functionality management for RRM measurement prediction	Samsung	discussion	Rel-19	FS_NR_AIML_Mob

Proposal. 6: For temporal domain Case A, NW can indicate the list of target cells for which the UE performs the prediction as part of inference configuration.

Proposal. 7: For the interpretation of “skipping pattern” in temporal domain Case B, RAN2 confirm that it refers to SSB configuration to indicate the timing of NW's SSB transmission—not timing of UE's SSB measurement/skipping.

-	ZTE thinks that it can be left to WI phase.  Asustek thinks that it is restricting the possible patterns.

-	Xiaomi thinks that this is the only way. Nokia, Apple, Ericcons and Qualcomm agrees.

-	CATT thinks it should be similar to what we did for the simulation.

Noted

R2-2505186	Discussion on functionality management for RRM measurement prediction v3	OPPO	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 10: For temporal domain case A, one or more instances of predicted measurement results in PW per one cell are reported in one measurementReport message

Proposal 11: For temporal domain case A, one or more instances of measurement results in OW per one cell are reported in one measurementReport message

Noted

R2-2506126	Discussion on functionality management for RRM measurement prediction	ZTE Corporation	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 13	For temporal domain Case B, the inference report includes the latest measurement results (regardless of actual results or predicted results).

Proposal 15	For frequency domain prediction, the inference report includes the latest measurement results of the predicted cell. (No change to the existing measurement report)

Noted

R2-2505216	Functionality management for RRM measurement prediction	CATT, Turkcell	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 2: For intra-frequency temporal domain case B, whether measurement result is actual or predicated should be indicated in the measurement results.

Noted

Agreements

- For the interpretation of “skipping pattern” in temporal domain Case B, RAN2 confirm that it refers to SSB configuration to indicate the timing of NW's SSB transmission—not timing of UE's SSB measurement/skipping.

- For temporal domain Case A, NW can indicate the list of target cells for which it expects results (if available) as part of inference configuration.  This list is optional.

- For temporal domain case A, one or more instances of predicted measurement results in PW per one cell are reported in one measurementReport message

- For temporal domain Case B, the inference report can include the latest measurement results (regardless of actual results or predicted results).

- For frequency domain prediction, the inference report can include the latest measurement results of the predicted cell. (No change to the existing measurement report)

Inference activation:

R2-2505460	Functionality management for RRM measurement prediction	Lenovo	discussion

Proposal 7: After receiving the full inference configuration for measurement prediction and reporting the applicable AI/ML functionalities, the inference can be triggered by UE immediately (e.g., for periodic configurations) or upon a certain event.

Noted

R2-2505473	Discussion on Functionality Management for RRM Measurement Prediction	MediaTek Inc.	discussion

Proposal 5: For RRM Temporal domain case B, Spatial domain prediction, and frequency domain prediction, when to perform inference is up to UE implementation. UE needs to meet the report requirements determined by RAN4.

Noted

R2-2505671	Functionality management for RRM measurement prediction	Huawei, HiSilicon	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 5: The exact time of triggering inference follows the inference configuration provided from the network, e.g. PW length, time instances to be reported etc.

Noted

Performance monitoring:

R2-2505186	Discussion on functionality management for RRM measurement prediction v3	OPPO	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 18: For UE side model, the metric is calculated per frequency for L3 cell level prediction i.e. intra-frequency cells should be calculated together.

Proposal 19: For temporal domain case A and case B, average L3 RSRP difference within PW is monitoring metric

Proposal 20: L3 RSRP difference should be further averaged within configured monitoring duration

Noted

R2-2505836	UE-side Model – Functionality Management for RRM Measurement Prediction	Ericsson	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 15	Network configures UE with performance monitoring window and/or percentage of prediction samples that UE should monitor in prediction window.

Proposal 18: UE reports performance monitoring outcome in periodic or event triggered basis, according to the performance monitoring report configuration configured by the network. FFS performance monitoring report triggering events.

Noted

R2-2505357	Discussion on functionality management for RRM measurement prediction	Samsung	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 24: For frequency domain prediction, RAN2 need to discuss how to calculate RSRP difference in WI phase (considering the case UE is not able to measure signals on multiple frequency bands simultaneously).

Noted

Monitoring window is it configurable or up to UE

-	Nokia thinks it based on case, for temporal can be up to UE but for inter-frequency it should be configurable as the UE needs measurement gaps.

-	Samsung thinks this is essential for measurement reduction.   Vivo also agrees especially when the monitoring window is dedicated.

-	Qualcomm thinks that monitoring window may not be needed and it can be up to UE implementation.

-	Apple thinks that maybe we may not even need to do performance monitoring if we satisfy the requirements.

A monitoring window, a window over which the performance monitoring metric can be calculated, can be configured for performance monitoring.  Up to WI on which cases (if any) this monitoring window is needed.

R2-2505114	Discussions on functionality management for RRM measuerment prediction	DOCOMO Beijing Labs	discussion

R2-2505151	Discussion on functionality management for RRM prediction	Xiaomi	discussion

R2-2505216	Functionality management for RRM measurement prediction	CATT, Turkcell	discussion	Rel-19	FS_NR_AIML_Mob

R2-2505337	Discussion on UE-side AIML model for RRM measurement prediction 	NEC	discussion	NR_AIML_Mob-Core

R2-2505569	Discussion on Functionality management for RRM measurement prediction for UE sided model	KT Corp.	discussion	FS_NR_AIML_Mob

R2-2505710	Discussion on functionality management for RRM measurement prediction	Spreadtrum, UNISOC	discussion	Rel-19

R2-2505874	Functionality management for UE sided model for RRM measurement prediction	Interdigital Inc.	discussion	Rel-19	FS_NR_AIML_Mob

R2-2505887	Functionality management for RRM measurement prediction for UE-side model	Qualcomm Incorporated	discussion	Rel-19	FS_NR_AIML_Mob

R2-2505964	Discussion on RRM measurement prediction with UE sided model	CMCC	discussion	Rel-19	FS_NR_AIML_Mob

R2-2505984	Functionality Management for AIML Mobility	Continental Automotive	discussion

R2-2506030	Discussion on functionality management for RRM measurement prediction	ASUSTeK	discussion	Rel-19	FS_NR_AIML_Mob

R2-2506135	Functionality management for RRM measurement prediction	Nokia	discussion	Rel-19	FS_NR_AIML_Mob

R2-2506177	Discussion on UE-sided Model Functionality Management	Sharp	discussion

#### 8.3.2.2	Functionality management for RRM measurement event prediction

Including applicability procedure, inference configuration/reporting and performance monitoring for UE sided model for RRM measurement event predictions

General (direct vs indirect):

R2-2505837	UE Sided model – Functionality Management for RRM Event Prediction	Ericsson	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 4	RAN2 first focus on designing signaling solution for the indirect event prediction. If direct event prediction comes for free, it can be left to UE implementation, otherwise it should be ruled out (to avoid different RRC solutions for different UE implementation).

Noted

R2-2505672	Functionality management for measurement event prediction	Huawei, HiSilicon	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 1: Only a single event prediction framework should be developed which will not distinguish indirect and direct prediction methods, i.e. method specific configurations/procedures should not be considered.

Noted

R2-2505358	Discussion on functionality management for RRM measurement event prediction	Samsung	discussion	Rel-19	FS_NR_AIML_Mob

Proposal. 8: For measurement event prediction, RAN2 support only indirect prediction in WI phase.

Noted

Agreements

Aim for a single framework for event prediction and performance monitoring.  From RAN2 point of view indirect event prediction, RSRP differences can be used as the performance metric for monitoring.  Capture in the TR that there is no consensus on the feasibility of performance monitoring of the direct event prediction and this would need to be resolved in WI phase before proceeding with specifying direct event prediction.

Applicability determination/reporting:

R2-2505130	Discussion on functionality management for RRM measurement event prediction	vivo	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 1.	For RRM measurement event prediction, the UE determines the applicability considering the inference configuration from network:

- For indirect measurement event prediction, the applicability determination is the same as RRM measurement prediction, and no need to consider some measurement event-specific parameters, e.g., threshold, hysteresis.

- For direct measurement event prediction, the applicability determination is based on measurement event-specific parameters, e.g., threshold, hysteresis.

-	Oppo thinks that we need at least one more bit to indicate that this is for event prediction.

-	Xiaomi thinks that this can be up to UE implementation.

What the UE requires to the determine applicability is similar to RRM prediction.

Noted

Inference configuration/reporting:

R2-2505875	Functionality management for UE sided model for RRM measurement event prediction	Interdigital Inc.	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 1: For indirect RRM measurement event prediction, the following are included in the inference configuration:

- The length of PW for the associated AIML model

- Event-related information (event type and event-related parameters)

Proposal 2: For direct RRM measurement event prediction, the following are included in the inference configuration:

- The length of PW (“occurrence window”)

- Event-related information (event type and event-related parameters)

- Probability threshold (to control predicted event reporting)

Proposal 3: For indirect RRM measurement event prediction, the event prediction report includes:

- Multiple RRM measurement samples (current measurement and predicted measurements in a detailed and/or summarized form)

- Time-related information about the predicted event (e.g., the time of occurrence)

Proposal 4: For direct RRM measurement event prediction, the event prediction report includes:

- Time-related information about the predicted event (e.g., occurrence window)

- Probability of occurrence for the event

-	Apple thinks that the differences are artificial.  Interdigital thinks that we can perhaps unify window definition but probability threshold cannot be unified.

-	Nokia thinks that probability threshold may not even be needed, and is dependent on RAN4.

-	Mediatek asks why multiple predicted RRM samples.

Agreements 1 For RRM measurement event prediction, the following are included in the inference configuration: - The length of PW for the associated AIML model - Event-related information (event type and event-related parameters) 2 For RRM measurement event prediction, the event prediction report can include: - RRM measurements (For normative phase to define what types of measurements) - Time-related information about the predicted event (e.g., the time/window of occurrence) 3 For measurement event determined, when the UE is configured with temporal domain case B, and frequency domain prediction, UE reports measurement event by following an approach similar legacy procedure. This can be achieved without spec impact for measurement reporting. FFS for normative phase if additional information needs to be added.

Noted

R2-2505187	Discussion on functionality management for measurement event prediction V3	OPPO	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 4: For measurement event prediction based on temporal domain case B, spatial domain and frequency domain prediction, UE need report predicted measurement event by following legacy procedure i.e. no spec impact is introduced.

-	Qualcomm ask how the network knows that there was a prediction.   Oppo thinks that the network knows what the UE is reporting as it configured the UE.    Vivo thinks that we can remove no spec impact.

Proposal 4x: For measurement event prediction based on temporal domain case A, @t0 UE can report timing information when corresponding TTT timer expires in future i.e. t1, measurement event in t1 and relevant measurement result in t0 and t1. How to treat such measurement report is up to network’s implementation.

Noted

Performance monitoring:

Not treated

R2-2505474	Discussion on Functionality Management for RRM measurement event prediction	MediaTek Inc.	discussion

Proposal 6: For indirect event prediction, RSRP differences can be used as the performance metric for monitoring.

R2-2505217	Functionality management for RRM measurement event prediction	CATT, Turkcell	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 9: For measurement event prediction, for performance monitoring, RAN2 agrees the following:

- For indirect measurement event prediction, RSRP differences/F1 score can be used as the

performance metric for monitoring;

- For direct measurement event prediction, F1 score can be used as the performance metric for monitoring.

R2-2506127	Discussion on functionality management for measurement event prediction	ZTE Corporation	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 12: RAN2 to confirm that F1 score and system level metric (e.g. HOF rate) can not be used as performance metric for both indirect and direct measurement event prediction.

R2-2505512	Discussion on functionality management for measurement event prediction	CMCC	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 4: For NW-side performance monitoring, the UE can report the following information to the network:

- For indirect prediction: actual measurement results, missed event detection, false event detection, true event time

- For direct prediction: missed event detection, false event detection

Proposal 5: For UE-side performance monitoring, the UE can report the following information to the network:

- For indirect prediction: RSRP differences, missed event detection, false event detection, time difference between true event time and predicted event time

- For direct prediction: missed event detection, false event detection

R2-2505441	Event prediction	Apple	discussion	Rel-19	FS_NR_AIML_Mob

R2-2505461	Functionality management for RRM measurement event prediction	Lenovo	discussion

R2-2505512	Discussion on functionality management for measurement event prediction	CMCC	discussion	Rel-19	FS_NR_AIML_Mob

R2-2505711	Discussion on functionality management for RRM measurement event prediction	Spreadtrum, UNISOC	discussion	Rel-19

R2-2505888	Functionality management for Measurement Event prediction for UE-side model	Qualcomm Incorporated	discussion	Rel-19	FS_NR_AIML_Mob

R2-2506006	Discussion on Functionality management for RRM measurement event prediction	NEC	discussion

R2-2506031	Discussion on functionality management for measurement event prediction	ASUSTeK	discussion	Rel-19	38.744	FS_NR_AIML_Mob

R2-2506134	Functionality management for measurement event prediction	Nokia	discussion	Rel-19	FS_NR_AIML_Mob

R2-2506180	Discussion on performance monitoring	Sharp	discussion

#### 8.3.2.3	Data collection

Aspects related to data collection for UE sided model for RRM measurement prediction and RRM measurement event prediction (excluding those aspects that are under study in AI/ML PHY)

Data collection procedure/configuration:

R2-2505513	Discussion on data collection for UE-sided model	CMCC	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 1: The following agreements on request/configuration for UE side data collection in AI/ML PHY are also applied for RRM measurement prediction and measurement event prediction for connected mode.

- The UE can request measurement configuration for data collection of AI/ML based beam management. The request can contain one or more of the following:

  - An indication on start/stop of data collection

  - Details of signaling are FFS. It is up to network what it configures at the end.

- Introduce UAI message for UE request of data collection measurement configuration. And it is up to UE implementation when to send the request.

- Data collection related configuration(s) and associated ID(s)(if needed) can be included in training data collection configuration.

- The network can provide or release the data collection configuration (at any point in time), with or without UE request.

- The following methods for network control of the initiation and configuration for data collection:

  - The network can decide when to start/stop the data collection and send configuration.

  - The network can configure whether UE is allowed to initiate request for data collection (e.g. start/stop indication).

Proposal 2: For UE side data collection, the RRM measurement framework is used to provide the data collection configuration to the UE.

-	Xiaomi thinks that preferred configuration may not be needed for mobility.  Interdigital agrees with Xioami.

Preferred configuration from a list of candidate configurations provided by NW.

Noted

R2-2505347	Discussion on data collection for AI/ML mobility	Samsung	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 2. For UE-side data collection, UE can perform measurement by re-using MOs configured for legacy RRM measurement.

Proposal 3. RAN2 to consider two options for UE-side data collection:

In Option 1,

- Step 1) NW configures a list of candidate MOs or frequencies or cells for data collection (e.g., via OtherConfig)

- Step 2) UE indicates its preferred MO(s) or frequencies or cells (e.g., via UEAssisntaceInformation message) among the received candidates for data collection.

- Step 3) NW configures MO(s) for measurement of UE-side data collection. Then, UE starts measuring according to the configuration and acquiring data for model training (e.g., cell level RSRP).

In Option 2,

- Step 1) NW allows transmission of UE’s preference without candidate configuration for data collection (e.g., via OtherConfig)

- Step 2) UE indicates its preferred frequencies or cells (e.g., via UEAssisntaceInformation message)

- Step 3) NW configures MO(s) for measurement of UE-side data collection. Then, UE starts measuring according to the configuration and acquiring data for model training (e.g., cell level RSRP).

Noted

R2-2505188	Discussion on data collection for UE sided model-V2	OPPO	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 5: Some assistant information from UE is necessary for gNB to configure data collection properly.

Proposal 7: Potential assistant information from UE for data collection could be:

| Potential assistant information | Applicable scenarios |
| --- | --- |
| Frequency range information | All scenarios (temporal domain, frequency domain, spatial domain) |
| Whether data collection is for frequency domain prediction | Frequency domain prediction |
| Whether L1 beam level measurement results need collected | RRM sub-case 2 of temporal domain case A |

Noted

Discussion

-	Samsung thinks that the UE can indicate the frequency as the network doesn’t know what frequency the UE needs for input.

-	Mediatek thinks that the network needs to provide the frequency information and not the UE.  Xiaomi thinks that you can just request measurement gap.   Nokia agrees that the UE needs to send some more information and there may be some delay issues.  We should consider the network giving a list of options to the UE and the UE can indicate.   Docomo agrees that it should be the network that provides which frequencies the UE can measure.    Ericsson thinks it makes sense as mobility is networks decisions.   ZTE, Huawei also thinks that the network should be the one that provides the frequency information.

-	Qualcomm thinks that the UE can measure for predicting on current frequency so it is up to the UE.

-	Xiaomi and Apple thinks that we should be allowed to just ask for a measurement gap without frequency.    ZTE doesn’t think this is not acceptable for the network side as this will impact legacy measurements.  The measurement gap depends on number of frequencies.   Also  how does the UE know what frequencies/cells belong to the same operator.

-	Huawei thinks we can use need for gap and frequency, but shouldn’t give preferred gap.  Ericsson that would also impact RAN4.

- Agreements 1 For request/configuration for UE side data collection, the following in AI/ML PHY agreements are also applied as a baseline for RRM measurement prediction and measurement event prediction for connected mode. - The UE can request measurement configuration for data collection of AI/ML based beam management. The request can contain one or more of the following: - An indication on start/stop of data collection - Details of signaling are FFS. It is up to network what it configures at the end. - Introduce UAI message for UE request of data collection measurement configuration. And it is up to UE implementation when to send the request. - Data collection related configuration(s) and associated ID(s)(if needed) can be included in training data collection configuration. - The network can provide or release the data collection configuration (at any point in time), with or without UE request. - The following methods for network control of the initiation and configuration for data collection: - The network can decide when to start/stop the data collection and send configuration. - The network can configure whether UE is allowed to initiate request for data collection (e.g. start/stop indication). NOTE this can be aligned with AI/ML PHY agreements at the end of this - For UE-side data collection, UE can perform measurement by re-using MOs configured for legacy RRM measurement. - The full list of candidate measurement configuration is not needed for AI mobility. Capture following options in the TR. Up to normative phase to determine solution. - Option 1 - Network can configure a set of candidate frequencies the UE can request. - The UE can indicate a preference for data collection within the set of candidate frequencies. - Option 2 - The UE can indicate preferred frequencies for data collection (under network control). FFS what mechanism to use. UP to normative phase if other information is required.

Content of collected data:

R2-2505131	Discussion on UE side data collection	vivo	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 5.	For UE-side data collection, there is no need for the network to configure the contents of collected data.

R2-2505218	Discussion on UE-sided data collection	CATT, Turkcell	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 3: The agreements about logging mechanism and content configuration achieved for network-sided data collection could be directly reused for UE-sided data collection:

- UE can be configured to log, at a certain logging periodicity, one or more of the following:

  - L3 cell level measurements;

  - L3 beam level measurements;

  - L1-filtered beam level measurements (if sub-case 1 and 3 is supported);

  - Cell ID (FFS CGI of serving cell.  If CGI is unavailable, or for neighboring cells the UE logs PCI-ARFCN as a fallback);

  - Time info (if as agreed by AI/ML and/or if needed).

- The UE can be configured with a L3 event for determining when logging is to be performed. When the event conditions are fulfilled, it performs the logging with the logging periodicity.

R2-2505188	Discussion on data collection for UE sided model-V2	OPPO	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 12: Measurement event should be logged if the target use case is measurement event prediction

Data collection in RRC_IDLE/INACTIVE:

R2-2505889	Data collection for UE-side model	Qualcomm Incorporated	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 3: For a UE-side model for AI/ML for mobility, the UE does not perform data collection in RRC_IDLE or RRC_INACTIVE.

Proposal 4: For a UE-side model for AI/ML for mobility, it is up to UE implementation whether to keep the collected data upon handover, upon experiencing RLF, and upon transitioning to RRC_IDLE or RRC_INACTIVE.

Noted

R2-2505383	Data collection for UE sided models	Ericsson	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 2	Data collection for UE sided models in RRC_IDLE/RRC_INACTIVE state is mainly up to UE implementation.

Proposal 3	RAN2 will discuss configuration of data collection in RRC_IDLE/RRC_INACTIVE in the work item phase.

Noted

Discussion

-	Xiaomi thinks this is up to UE implementation

-	Ericsson thinks that at least associated ID is required by the UE.  Qualcomm and Interdigital think you can get in connected mode.

-	Oppo thinks that this will impact power consumption and the network may need to be involved.

-	Nokia thinks that this should be considered and the UE spends most of its time in idle.

-	Mediatek doesn’t think this is technical useful but the UE can do it without specification impact.   The measurements are relaxed and those measurements are not very useful.

-	Huawei and Samsung thinks this can be up to UE implementation.

-	Ericsson asks why we have this connected mode then.  Qualcomm explains that the UE is not supposed to take these intensive measurements in idle mode but can do it if we are connected to the car for example.

-	Nokia thinks it is not that simple as the network doesn’t know where you will show up in the connected mode and whether your associated ID is still applicable.

UE can perform data collection in IDLE/INACTIVE mode without any specification impacts.

Not treated

R2-2505115	Discussions on data collection for UE sided model	DOCOMO Beijing Labs	discussion

R2-2505153	Discussion on data collection	Xiaomi	discussion

R2-2505442	UE-sided data collection	Apple	discussion	Rel-19	FS_NR_AIML_Mob

R2-2505475	Discussion on Data Collection for UE-sided Model	MediaTek Inc.	discussion	Withdrawn

R2-2505654	UE side Data collection for measurement/ event prediction	Sony	discussion	Rel-19	FS_NR_AIML_Mob

R2-2505673	Discussion on UE-side data collection	Huawei, HiSilicon	discussion	Rel-19	FS_NR_AIML_Mob

R2-2505876	Data collection for UE sided model	Interdigital Inc.	discussion	Rel-19	FS_NR_AIML_Mob

R2-2505915	UE-side data collection for AI Mobility	Lenovo	discussion

R2-2506128	Discussion on data collection for UE side model	ZTE Corporation	discussion	Rel-19	FS_NR_AIML_Mob

R2-2506136	Data collection for UE-sided model	Nokia	discussion	Rel-19	FS_NR_AIML_Mob

### 8.3.3	Network sided model

Including (sub)use cases to be supported, assistance information from the UE (e.g., measurements for inference or performance monitoring), and data collection.

Supported use cases:

R2-2505132	Discussion on NW-sided mode lin AI enhanced mobility	vivo	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 6.	All sub-use cases of NW-sided model are recommended for normative work, with the sub-use case type of NW-sided model is transparent to the UE.

Noted

R2-2505116	Discussions on NW-sided model	DOCOMO Beijing Labs	discussion

Proposal 3: Support the scenarios and sub-use cases in the Table.1 for Rel-20 specification.

Table.1 Supported scenarios and sub-use cases for Rel-20 specification

| Scenario |  | UE-sided model | NW-sided model |
| --- | --- | --- | --- |
| RRM prediction | Temporal domain Case A | Sub-use case 1,2,3,4 | Sub-use case 1,2,3,4 |
|  | Temporal domain Case B | Sub-use case 1,2,3,4 | Sub-use case 1,2,3,4 |
|  | Frequency domain | Sub-use case 1,2,3,4 | Sub-use case 1,2,3,4 |
|  | Spatial domain in beam dimension | -- | Sub-use case 1,3,4 |
| Event prediction |  | Indirect, Direct | -- |

Noted

R2-2505154	Discussion on NW sided model	Xiaomi	discussion

Proposal 1: NW sided model is not supported in temporal domain case B.

Proposal 2: L1 beam measurement result as model input, i.e. sub case 1, 3, 4 and 6, is not supported for NW sided model in RRM prediction and L3 beam prediction.

Noted

R2-2505359	Discussion on Network-sided model	Samsung	discussion	Rel-19	FS_NR_AIML_Mob

Proposal. 3: For cell-level temporal domain Case A, RAN2 support only subcase 2 for NW-sided model. I.e., UE can report only “cell-level” RRM measurement results at multiple time instances in one measurement report.

Noted

Discussoion

-	Apple, Qualcomm, Mediatek shares concerns with Xiaomi and Samsung as we never evaluated overhead and performance.

-	ZTE doesn’t thinks the overhead is a big problem as the UE won’t always configure the report only when in certain situations like cell edge.   Huawei and Ericsson agrees.

-	Huawei thinks the spec impact are minor and these are implementation issues and during implementation it will not configure the UE with something that breaks the system.

-	Oppo thinks that if we report for multiple beams that is a lot of overhead.   and points out that we agreed that there is no spec impact for other cases.  So we should agree with Samsung and only do the Case A.

-	Mediatek is concerned with implementation due to buffer size.

-	Samsung highlights that we already agreed that sub-case 2 outperforms sub-case 1 and 3.    ZTE thinks we need to beam level to enable CFRA.

-	Apple just thinks that these are sub optimal use cases that weren’t evaluated.  So companies are agreeing that you can do it but not to do further enhancements.

For network sided models for inference:

- For cell-level temporal domain Case A, sub-case 2 the following enhancement is considered UE can report “cell-level RRM measurement results at multiple time instances in one measurement report.

- For other cases there is no specification impact.   Can be discussed in WI phase whether any additional enhancements are needed and justified (i.e. multi-instances reporting of beam)

3	Study item can conclude that all scenarios and sub-cases are feasible.

Not treated

Reporting Information from UE for Inference Operation

R2-2506133	Considerations on network-sided model	Nokia, Nokia Shanghai Bell	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 10: Capture the overview of inference input to the network sided model in Table 2.2-1 in TR38.744.

Table 2.2-1: RRM measurement prediction per use case analysis for inference input

| Use case | Sub-use case | Temporal domain | Frequency domain | Spatial domain |
| --- | --- | --- | --- | --- |
| Cell-level prediction | 1 | Time-series of L1 filtered beam level measurements | L1 filtered beam level measurements of frequency 1 | Set B of L1 filtered beam level measurements |
|  | 2 | Time-series of L3 filtered cell level measurements | L3 filtered cell level measurements of frequency 1 | N/A |
|  | 3 | Time-series of L1 filtered beam level measurements | L1 filtered beam level measurements of frequency 1 | Set B of L1 filtered beam level measurements |
| Beam-level prediction | 4 | Time-series of L1 filtered beam level measurements | L1 filtered beam level measurements of frequency 1 | Set B of L1 filtered beam level measurements |
|  | 5 | Time-series of L3 filtered beam level measurements | L3 filtered beam level measurements of frequency 1 | N/A |
|  | 6 | Time-series of L1 filtered beam level measurements | L1 filtered beam level measurements of frequency 1 | Set B of L1 filtered beam level measurements |

Note: Frequency 1 denotes the carrier frequency at which measurements are taken for inference input, whereas the model output corresponds to measurements taken at Frequency 2.

R2-2505443	On network-sided models	Apple	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 1: for NW-sided inference, a UE should never be configured with extra measurements (in terms of periodicity, cells and frequencies to measure, etc) compared to what it would have been configured for legacy operation.

Proposal 2: UE should be aware whether the measurements configured are for network-sided inference or legacy operation.

R2-2505674	Discussion on NW-sided model support	Huawei, HiSilicon	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 4: There is no need for the UE awareness or preference indication on whether RRM measurement result is/can be used by the NW as input to NW-sided model based AIML algorithms.

Proposal 5: No assistance information from the UE is needed to support NW-side model inference other than L1/L3 RSRP beam/cell level measurement results.

R2-2505359	Discussion on Network-sided model	Samsung	discussion	Rel-19	FS_NR_AIML_Mob

Proposal. 1: For NW-side model inference, RAN2 should consider the ways to reduce the signalling overhead for the report of inference input (e.g., event-triggered reporting, adaptive reporting based on mobility status, …).

Performance Monitoring:

R2-2505965	Discussion on RRM measurement prediction with NW sided model	CMCC	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 3: At least one or more of the following should be considered for performance monitoring of NW-side model in RRM measurement prediction:

    - L3 cell level measurements

    - L3 beam level measurements

    - L1 beam level measurements

Proposal 4: The existing periodical RRM measurement reporting can be reused to report the actual measurement for performance monitoring of NW-side model.

R2-2505382	Network sided model	Ericsson	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 5	There is no need for UE (chipset) awareness of network side AI/ML related measurements.

Proposal 6	RAN2 abandons the discussion on UE awareness and leaves the user consent discussion to the relevant working groups (SA3) to discuss it on a need basis.

R2-2505877	Network sided model for AI/ML mobility	Interdigital Inc.	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 1: The network can request the UE for an on-demand measurement report for a given time duration (past or future time durations).

Proposal 4: A UE may support enhanced measurement logging/reporting for enabling LCM of a network sided AIML model (e.g., inference, performance monitoring, data collection for training, etc.,) without having AI/ML support on its side (e.g., no AI/ML mobility model/capability) or vice versa.

R2-2505443	On network-sided models	Apple	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 3: network must obtain user consent for using their device for performance monitoring of network-sided models.

Proposal 4: user must be aware when their device is used for performance monitoring of network-sided models.

Data Collection:

R2-2505189	Discussion on LCM procedures of network sided model	OPPO	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 9: For AI mobility, the following agreements also applies:

    - UAI related to buffer status or low power state is triggered only once when specific conditions are met (e.g., buffer full/threshold, and low power state). A prohibit timer is not necessary for UAI related to buffer status or low power state

    - No additional signaling from the UE is required when the low power issue is resolved

    - No additional signaling from the UE is required when the buffer full issue is resolved

R2-2505219	Discussion on Network sided model	CATT, Turkcell	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 4: UE can be configured to log and report the CGI of serving cell.  If CGI is unavailable, the UE shall log PCI-ARFCN as a fallback.

Proposal 6: The following agreements on starting/stopping logging for NW-side data collection in AI/ML PHY are also applied to RRM measurement prediction:

    - Data collection is controlled by the network. The UE will not autonomously stop when low power state is detected;

    - When UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging.

Proposal 7: The following agreements on sending of the collected data for NW-side data collection in AI/ML PHY are also applied to RRM measurement prediction:

    - As baseline, the UEInformationResponse contains one or more logged measurement entries in chronological order (i.e. starting from the oldest measurement entries stored in the UE memory), and an availability indication if there are further data available for transmission. Same principles as for logged MDT;

    - New SRB can be configured for NW-side data collection (with lower priority).

R2-2505190	Discussion on AI/ML performance monitoring for network-sided model	KT Corp.	discussion	Rel-19	FS_NR_AIML_Mob

R2-2505637	Discussion on NW-side model input	Sharp	discussion	Rel-19	Withdrawn

R2-2505695	Discussion on network sided model	Lenovo	discussion	Rel-19

R2-2505890	Network-side model	Qualcomm Incorporated	discussion	Rel-19	FS_NR_AIML_Mob

R2-2506007	Discussion on AIML mobility for Network Sided Model Performance	NEC	discussion	Rel-19	FS_NR_AIML_Mob

R2-2506129	Discussion on AI mobility for network side model	ZTE Corporation	discussion	Rel-19	FS_NR_AIML_Mob

R2-2506164	Discussion on NW-side model input	Sharp	discussion	Rel-19

## 8.4	Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

(NR_LPWUS-Core; leading WG: RAN1; REL-19; WID RP-251200)

Time budget: 1 TU

Tdoc Limitation: 3 tdocs

### 8.4.1	Organizational

LS, Rapporteur input, including workplan, Running CRs, email discussion summary, open issue list(s), etc.

LS

R2-2505020	Reply LS on LP-WUS in RRC_CONNECTED (R1-2504888; contact: NTT DOCOMO)	RAN1	LS in	Rel-19	NR_LPWUS	To:RAN2

Noted

Discussion

-	Ericsson think R1 mention MR and LR cannot operate simultaneously, but not sure what is the impact to R2 procedures. Apple think this was discussed in MAC running CR, and think this related to dual DRX group.

R2-2505025	Reply LS on LP-WUS UE RF (R1-2504943; contact: vivo)	RAN1	LS in	Rel-19	NR_LPWUS-Core	To:RAN4	Cc:RAN2

Noted

R2-2505028	LS on TP to TS38.300 for Rel-19 LP-WUS/WUR (R1-2505070; contact: vivo)	RAN1	LS in	Rel-19	NR_LPWUS-Core	To:RAN2

Noted

Discussion

-	Ericsson think there may some texts that we can simply to avoid overlapping, and think we can take into account R1’s texts.

R2-2505035	Reply LS on LP-WUS subgrouping progress (R3-253846; contact: NEC)	RAN3	LS in	Rel-19	NR_LPWUS-Core	To:RAN2

Noted

CRs

R2-2505234	Introduction of LP-WUS in TS 38.304	CATT	CR	Rel-19	38.304	18.4.0	0440	-	B	NR_LPWUS-Core

R2-2505392	Introduction of LP-WUS/WUR in RRC	vivo (Rapporteur)	CR	Rel-19	38.331	18.6.0	5416	-	B	NR_LPWUS-Core

R2-2505469	Introduction of LP-WUS in TS 37.340	ZTE Corporation, Sanechips	CR	Rel-19	37.340	18.6.0	0420	-	B	NR_LPWUS-Core

R2-2505476	Running MAC CR for LP-WUS	Apple (Rapporteur)	CR	Rel-19	38.321	18.6.0	2103	-	B	NR_LPWUS-Core

R2-2505863	Introduction of Low-Power Wake-Up Signal and Receiver for NR	Ericsson	CR	Rel-19	38.300	18.6.0	1015	-	B	NR_LPWUS-Core

The above 5 CRs are endorsed, and they will be taken as baseline for further updates and review

R2-2505670	Introduction of R19 LP-WUS UE Capabilities	Huawei, HiSilicon	draftCR	Rel-19	38.306	18.6.0	B	NR_LPWUS-Core

Endorsed

Post meeting email discussions for the CRs

[Post131][210][LPWUS] CR for TS 38.304 (CATT)

Intended outcome: Agree the CR for TS 38.304

Deadline:  Short

=> Agreed in R2-2506608 (38.304)

[Post131][211][LPWUS] CR for TS 38.331 (vivo)

Intended outcome: Agree the CR for TS 38.331

Deadline:  Short

=> Agreed in R2-2506583 (38.331)

[Post131][212][LPWUS] CR for TS 37.340 (ZTE)

Intended outcome: Agree the CR for TS 37.340

Deadline:  Short

=> Agreed in R2-2506607 (37.340)

[Post131][213][LPWUS] CR for TS 38.321 (Apple)

Intended outcome: Agree the CR for TS 38.321

Deadline:  Short

=> Agreed in R2-2506613 (38.321)

[Post131][214][LPWUS] CR for TS 38.300 (Ericsson)

Intended outcome: Agree the CR for TS 38.300

Deadline:  Short

=> Agreed in R2-2506609 (38.300)

[Post131][215][LPWUS] Draft CR for UE capability (Huawei)

Intended outcome: Endorse the draft CR for TS 38.306

Deadline:  Short

=> Not triggered since it was found there is no need for this discussion.

R2-2506608	Introduction of LP-WUS in TS 38.304	CATT	CR	Rel-19	38.304	18.4.0	0440	3	B	NR_LPWUS-Core

=> Agreed

R2-2506583	Introduction of LP-WUS/WUR in RRC	vivo (Rapporteur)	CR	Rel-19	38.331	18.6.0	5416	2	B	NR_LPWUS-Core

=> Agreed

R2-2506607	Introduction of LP-WUS in TS 37.340	ZTE Corporation, Sanechips	CR	Rel-19	37.340	18.6.0	0420	2	B	NR_LPWUS-Core

=> Agreed

R2-2506613	Introduction of LP-WUS in TS 38.321	Apple	CR	Rel-19	38.321	18.6.0	2103	2	B	NR_LPWUS-Core

=> Agreed

R2-2506609	Introduction of Low-Power Wake-Up Signal and Receiver for NR	Ericsson	CR	Rel-19	38.300	18.6.0	1015	1	B	NR_LPWUS-Core

=> Agreed

R2-2505380	Introduction of R19 LP-WUS UE Capabilities	Huawei, HiSilicon	CR	Rel-19	38.306	18.6.0	1325	-	B	NR_LPWUS-Core	Withdrawn

Email discussion summary

R2-2505235	Discussion of [Post130][211][LPWUS] Running CR for 38.304 (CATT)	CATT	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 1(11/2): In the running CR, UE supporting LP-WUS is used instead of LP-WUS UE.

Proposal 2(11/0): Nothing is needed on high priority frequency for serving cell measurement offloading or measurement relaxation with LP-WUS in 38.304 running CR if the corresponding higher priority frequency relaxation has been captured in RAN4 specification.

Proposal 3 (13/0): Same as LP-WUS monitoring, it is up to UE implementation to choose whether SSB measurement based or OOK LP-SS measurement based are used for RRM relaxation/offloading conditions if UE supports both measurement types.

Proposal 4: RAN2 discuss whether not to redefine aspects in TS 38.304 which are already defined in TS 38.213, i.e. the details regarding the locations, offsets, and UE behaviors related to LO (LP-WUS Occasion) monitoring.

In the running CR, ‘UE supporting LP-WUS’ is used instead of ‘LP-WUS UE’.

Nothing is needed on high priority frequency for serving cell measurement offloading or measurement relaxation with LP-WUS in 38.304 running CR if the corresponding higher priority frequency relaxation has been captured in RAN4 specification.

Discussions

P3

-	Nokia not sure why it is based on UE implementation, since these two types have different performance.

Same as LP-WUS monitoring, it is up to UE implementation to choose whether SSB measurement based or OOK LP-SS measurement based are used for RRM relaxation/offloading conditions if UE supports both measurement types.

P4

-	Ericsson think in general we should not repeat the same thing btw R1 and R2 specs.

-	CATT and vivo think for PEI we have similar texts so it is fine to keep them, even though there may be some similar texts also in R1 spec. Qualcomm, OPPO, Xiaomi agree and think this way it is clearer.

R2-2505393	Discussion summary and list of RRC open issue for LP-WUS WUR	vivo	discussion	Rel-19	NR_LPWUS-Core

Noted

Open issue RRC-6 (essential): the value range of ThresholdPLP and ThresholdQLP for LR measurement based threshold

Proposal 1: [8/13] RAN2 assumes the value range of ThresholdPLP and ThresholdQLP for LR measurement based threshold for entry/exit condition for LP-WUS monitoring and RRM relaxation/offloading is captured as below in RRC. It could be revised based on inputs from RAN1/RAN4, if any.

The IE ThresholdLP is used to indicate a measured RSRP threshold for LP-WUS. Actual value of threshold = field value * 2 [dBm].

ThresholdP-LP ::=               INTEGER (-80..0)

The IE ThresholdQ-LP is used to indicate a measured RSRQ threshold for LP-WUS. Actual value of threshold = field value [dB].

ThresholdQ-LP ::=               INTEGER (-34..0)

Open issue RRC-13 (essential): how to determine the cell quality for LR based measurement.

Proposal 2: [11/14] LR measurement based RX level and cell quality value should be derived by UE implementation in multi-beam operations.

Proposal 3: RAN2 will keep the current terminologies in RAN2 specification, i.e. LP-WUS, LP-SS, LO (LP-WUS Occasion), LR, and MR.

- 11/14 companies voted to keep the terminology: LP-WUS, LP-SS, LO (LP-WUS Occasion), and LR.

- 12/13 companies voted to keep the terminology: MR.

RAN2 assumes the following value range of ThresholdPLP and ThresholdQLP for LR measurement based threshold for entry/exit condition for LP-WUS monitoring and RRM relaxation/offloading

- The IE ThresholdLP is used to indicate a measured RSRP threshold for LP-WUS. Actual value of threshold = field value * 2 [dBm].

- ThresholdP-LP ::=               INTEGER (-80..0)

- The IE ThresholdQ-LP is used to indicate a measured RSRQ threshold for LP-WUS. Actual value of threshold = field value [dB].

- ThresholdQ-LP ::=               INTEGER (-34..0)

Discussions

P2

-	Ericsson think there is no need to introduce configuration for this, but want to have predictable behaviour, such as ‘using the strongest beam’. Nokia agree. LG E slightly prefer this way, and think it is better for NW to properly configure the thresholds.

-	QC want to leave this to UE implementation. Apple, CATT agree.

LR measurement based RX level and cell quality value should be derived by UE implementation in multi-beam operations. We assume this conclusion does not impact the cell reselection procedure.

Discussions

P3

-	Samsung agree if this is majority view in R2, but want to note that with this there is mis-alignment in R1/R2 wordings. Vivo think this is not critical issue.

-	Qualcomm think LR/MR are just names but not necessarily mandate any specific implementation choices.

RAN2 will keep the current terminologies in RAN2 specification, i.e. LP-WUS, LP-SS, LO (LP-WUS Occasion), LR, and MR.

R2-2505477	Report of [Post130][213][LPWUS] Running CR for TS 38.321 (Apple)	Apple (Rapporteur)	discussion	Rel-19	NR_LPWUS-Core

Noted

For open issue 1: Support of LP-WUS with dual DRX group

Proposal 1: Confirm the following working assumption to support LP-WUS with dual DRX group.

RAN2#130 progress

	Working assumption: LP-WUS can be configured on the PCell with secondary DRX. LP-WUS with secondary DRX is supported with option 1-1 and 1-2, i.e. the UE monitors LP-WUS before the on-duration occasion or periodically outside ActiveTime.  When LP-WUS is detected, then UE starts the drx-onDurationTimer (with option 1-1) or the lpwus-PDCCHMonitoringTimer (with option 1-2) in both DRX groups.

Proposal 2: If secondary DRX group is configured, UE monitors LP-WUS only when both DRX groups are not in DRX active time. (NOTE: One company has concern)

Proposal 2a: If secondary DRX group is configured, UE monitors LP-WUS only when both DRX groups are not in DRX active time. Regarding the RAN1 agreement on not supporting simultaneous LR and MR operation, further check with RAN1 whether the agreement is applicable to DC and CA, and whether it has any impact on per CG DRX operation.

Proposal 3: If secondary DRX group is configured, the lpwus-PDCCH-MonitoringTimer configuration for secondary DRX group is different from that for the default DRX group.

Proposal 3a: The lpwus-PDCCH-MonitoringTimer configuration for secondary DRX group is smaller than that for the default DRX group.

For open issue 2: UE operation for the potential collision

Proposal 4: Confirm the following RAN2#129bis working assumption for Option 1-1.

RAN2#129bis progress

	Working assumption for the case of potential collision (if any): In Option 1-1, when the UE is not able to monitor the LP-WUS occasion(s) the UE should start the drx-OnDurationTimer (as if LP-WUS was detected). FFS for Option 1-2.

Proposal 5: Agree the proposed LP-WUS TP (with the addition MUSIM gap case) to capture the UE operation in Option 1-1 for the collision and timing issue.

Proposal 6: For Option 1-2, NW configures UE whether to start the lpwus-PDCCH-MonitoringTimer in collision cases, i.e. when the UE is not able to monitor the LP-WUS occasion(s).

For open issue 3: MAC spec impact to support the LP-WUS in Cell DTX operation

Proposal 7: There is no MAC spec impact to reflect the LP-WUS operation in Cell DTX operation.

Proposal 9: RAN2 confirm that the available UL occasions (e.g. SR occasion, RACH occasion, CG occasion) are MR-ready occasions.

Confirm the following working assumption to support LP-WUS with dual DRX group.

- Working assumption: LP-WUS can be configured on the PCell with secondary DRX. LP-WUS with secondary DRX is supported with option 1-1 and 1-2, i.e. the UE monitors LP-WUS before the on-duration occasion or periodically outside ActiveTime.  When LP-WUS is detected, then UE starts the drx-onDurationTimer (with option 1-1) or the lpwus-PDCCHMonitoringTimer (with option 1-2) in both DRX groups.

If secondary DRX group is configured, the lpwus-PDCCH-MonitoringTimer configuration for secondary DRX group is different from that for the default DRX group.

Discussions

P2

-	Qualcomm think R1 conclusion is for single frequency band, and think typically we configure two DRX groups in FR1 and FR2, respectively. Qualcomm think the current P2 is not efficient in terms of power saving, and want to check with R1. Xiaomi have sympathy with Qualcomm view.

-	Ericsson think we should stick to general principle that UE monitor LPWUS outside active time, and think we should allow different possible UE implementations and think that should base on UE capability.

If secondary DRX group is configured, UE monitors LP-WUS only when both DRX groups are not in DRX active time.

RAN2 understand that the RAN1 agreement on not supporting simultaneous LR and MR operation is only applicable within one cell group (with or without secondary DRX group configuration). Send LS to RAN1 for confirmation.

The lpwus-PDCCH-MonitoringTimer configuration for secondary DRX group is smaller than or equal to that for the default DRX group.

Confirm the following RAN2#129bis working assumption for Option 1-1:

- In Option 1-1, when the UE is not able to monitor the LP-WUS occasion(s) the UE should start the drx-OnDurationTimer (as if LP-WUS was detected).

For Option 1-2, UE does not start the lpwus-PDCCH-MonitoringTimer in collision cases, i.e. when the UE is not able to monitor the LP-WUS occasion(s). Can discuss if critical issue identified with this mechanism.

Agree the addition of the MUSIM gap case, for the UE operation in Option 1-1 for the collision and timing issue.

There is no MAC spec impact to reflect the LP-WUS operation in Cell DTX operation. Can further check in maintenance phase.

RAN2 confirm that the available UL occasions (e.g. SR occasion, RACH occasion, CG occasion) are MR-ready. Can further check whether any spec change is needed.

[Post131][209][LPWUS] LS on not supporting simultaneous LR and MR operation (Apple)

Intended outcome: Prepare and approved the LS based on the corresponding agreements

Deadline:  Short

=> Approved in R2-2506550 (LS out)

R2-2506550	LS on not supporting simultaneous LR and MR operation	RAN2	LS out	Rel-19	NR_LPWUS-Core	To:RAN1	Cc:RAN4

=> Approved

Others

R2-2505478	Draft Reply LS on LP-WUS in RRC_CONNECTED	Apple	LS out	Rel-19	NR_LPWUS-Core	RAN1

Not treated

Agreements based on email summary discussions

In the running CR, ‘UE supporting LP-WUS’ is used instead of ‘LP-WUS UE’. Nothing is needed on high priority frequency for serving cell measurement offloading or measurement relaxation with LP-WUS in 38.304 running CR if the corresponding higher priority frequency relaxation has been captured in RAN4 specification. Same as LP-WUS monitoring, it is up to UE implementation to choose whether SSB measurement based or OOK LP-SS measurement based are used for RRM relaxation/offloading conditions if UE supports both measurement types. RAN2 assumes the following value range of ThresholdPLP and ThresholdQLP for LR measurement based threshold for entry/exit condition for LP-WUS monitoring and RRM relaxation/offloading - The IE ThresholdLP is used to indicate a measured RSRP threshold for LP-WUS. Actual value of threshold = field value * 2 [dBm]. - ThresholdP-LP ::= INTEGER (-80..0) - The IE ThresholdQ-LP is used to indicate a measured RSRQ threshold for LP-WUS. Actual value of threshold = field value [dB]. - ThresholdQ-LP ::= INTEGER (-34..0) LR measurement based RX level and cell quality value should be derived by UE implementation in multi-beam operations. We assume this conclusion does not impact the cell reselection procedure. RAN2 will keep the current terminologies in RAN2 specification, i.e. LP-WUS, LP-SS, LO (LP-WUS Occasion), LR, and MR. Confirm the following working assumption to support LP-WUS with dual DRX group. - Working assumption: LP-WUS can be configured on the PCell with secondary DRX. LP-WUS with secondary DRX is supported with option 1-1 and 1-2, i.e. the UE monitors LP-WUS before the on-duration occasion or periodically outside ActiveTime. When LP-WUS is detected, then UE starts the drx-onDurationTimer (with option 1-1) or the lpwus-PDCCHMonitoringTimer (with option 1-2) in both DRX groups. If secondary DRX group is configured, the lpwus-PDCCH-MonitoringTimer configuration for secondary DRX group is different from that for the default DRX group. If secondary DRX group is configured, UE monitors LP-WUS only when both DRX groups are not in DRX active time. RAN2 understand that the RAN1 agreement on not supporting simultaneous LR and MR operation is only applicable within one cell group (with or without secondary DRX group configuration). Send LS to RAN1 for confirmation. The lpwus-PDCCH-MonitoringTimer configuration for secondary DRX group is smaller than or equal to that for the default DRX group. Confirm the following RAN2#129bis working assumption for Option 1-1: - In Option 1-1, when the UE is not able to monitor the LP-WUS occasion(s) the UE should start the drx-OnDurationTimer (as if LP-WUS was detected). For Option 1-2, UE does not start the lpwus-PDCCH-MonitoringTimer in collision cases, i.e. when the UE is not able to monitor the LP-WUS occasion(s). Can discuss if critical issue identified with this mechanism. Agree the addition of the MUSIM gap case), for the UE operation in Option 1-1 for the collision and timing issue. There is no MAC spec impact to reflect the LP-WUS operation in Cell DTX operation. Can further check in maintenance phase. RAN2 confirm that the available UL occasions (e.g. SR occasion, RACH occasion, CG occasion) are MR-ready. Can further check whether any spec change is needed.

Completion of the WI

From RAN2 point of view, the Rel-19 work item NR_LPWUS-Core is completed.

### 8.4.2	Procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE

Procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping, and entry/exit condition for LP-WUS monitoring

RRC-12, whether/how to enable/disable LP-WUS

R2-2505381	Summary of [Post130][222][LPWUS] Potential solution to support enabling/disabling LP-WUS monitoring in IDLEI/NACTVE per UE (Huawei)	Huawei, HiSilicon	discussion	Rel-19

Noted

Proposal: (RRC-12) RAN2 to down-select from NAS signalling or RRC signalling to support enabling/disabling LP-WUS per UE.

R2-2506038	IDLE/Inactive LP-WUS disabling and enabling	Qualcomm Incorporated	discussion	NR_LPWUS-Core

Noted

Proposal 1	RAN2 selects one of the following solutions to disable or enable LP-WUS.

Option 4a: gNB sets proper offset between an LO and a reference PO/PF, UE determines whether to disable LP-WUS monitoring via comparing the offset and supported wake up delay.

Option 2a: The gNB can disable or enable LP-WUS using dedicated RRC message, e.g. RRC release message, and the enabling and disabling indication is only valid in current cell.

Discussion

-	Ericsson think there is majority support for the NAS base way. Vivo, IDT share this view. ZTE ok to go this way if there is majority. Apple think we do not need new NAS based solution.

-	Rapporteur think this feature is beneficial for deployment in the future.

-	CATT want to avoid the impact to the other WGs/TSGs. DCM share this view.

-	HW think both RRC and NAS based solution may have impact to other WG.

-	HW, Ericsson, InterDigital, vivo, NEC, OPPO, ZTE think NAS signalling based way has majority’s support. QC, CATT think not.

-	Xiaomi want to avoid impact to other WG. ZTE think the impact is not much.

-	WI Rapp think if we go with NAS signalling based way, it does not impact WI completion.

RAN2 assumes NAS signalling is introduced to support enabling/disabling LP-WUS per UE. Inform SA2, CT1 and RAN3 about this conclusion.

[AT131][203][LPWUS] Proposals for NAS signalling to support enabling/disabling LP-WUS per UE (Huawei)

Intended outcome: Summary with proposals in R2-2506245, draft LS in R2-2506246

Deadline: before Thursday CB.

R2-2506245	Summary of [AT131][203][LPWUS] Proposals for NAS signalling to support enabling/disabling LP-WUS per UE (Huawei)

Noted

RAN2 assumes that NAS signalling needs to be extended to enable/disable LP-WUS for a UE in IDLE and INACTIVE. Detail signalling is up to SA2, CT1.

RAN2 assumes that CN needs to inform gNB that LP-WUS is enabled/disabled for a UE in IDLE and INACTIVE. Details signalling is up to SA2, CT1, RAN3.

RAN2 assumes that without such NAS singalling, UE is allowed to use LPWUS in IDLE and INACTIVE.

R2-2506246	[Draft] LS on enabling/disabling LP-WUS per UE with NAS signalling

Noted

Add the ‘RAN2 assume that without such NAS singalling, UE is allowed to use LPWUS n IDLE and INACTIVE’ to the LS.

The LS is approved unseen in R2-2506261

38304-8: Whether LP-WUS is only used in the last used cell or in any cell

R2-2505605	Discussion on LP-WUS procedure and configuration	OPPO	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 3	LP-WUS can be used in any cell, i.e., don’t introduce lastUsedCellOnly for LP-WUS.

R2-2505856	LP-WUS in Idle and Inactive	Ericsson	discussion	Rel-19	NR_LPWUS-Core	R2-2504288

Noted

Proposal 1	Similar as with PEI LP-WUS can be configured to be used in lastUsedCellOnly or in any cell where LP-WUS is configured.

Discussions

-	ZTE support Ericsson proposal. Vodafone agree and think false alarm is a real issue. LG E agree.

-	Vivo think the false alarm issue is less serious for LPWUS since we now have more subgroups. Nokia agree. Nokia want to avoid also RAN3 impact. CATT agree.

-	Xiaomi think with the mechanism of enable/disabling this feature is not so critical. Vivo share this view.

-	LG E think we can just copy paste PEI mechanism.

-	Ericsson think impact to R3 is acceptable level.

-	vivo think we can close the WI even we do not have any conclusion.

-	Ericsson suggest to postpone the issue. Vivo think this does not impact the WI completion. Nokia think we can conclude.

LP-WUS can be used in any cell, i.e., don’t introduce lastUsedCellOnly for LP-WUS.

38304-9, Subgroup ID for LP-WUS outside CN PTW

R2-2505605	Discussion on LP-WUS procedure and configuration	OPPO	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 1	In RRC_INACTIVE state, for LP-WUS, when the UE uses the same i_s as for RRC_IDLE state, the UE shall use the same iPO LP-WUS as for RRC_IDLE state. Otherwise, the UE determines the iPO for LP-WUS for based on the i_s for RRC_INACTIVE state.

Proposal 2	In RRC_INACTIVE state with CN configured PTW, the SubgroupID for LP-WUS used outside CN PTW is the same as the SubgroupID used inside CN PTW.

Discussions

-	ZTE, Xiaomi, Ericsson support both proposals.

In RRC_INACTIVE state, for LP-WUS, when the UE uses the same i_s as for RRC_IDLE state, the UE shall use the same iPO LP-WUS as for RRC_IDLE state. Otherwise, the UE determines the iPO for LP-WUS for based on the i_s for RRC_INACTIVE state.

In RRC_INACTIVE state with CN configured PTW, the SubgroupID for LP-WUS used outside CN PTW is the same as the SubgroupID used inside CN PTW.

38304-10: FFS whether/how LP-WUS with SDT is supported

R2-2505479	Remaining issues of LP-WUS in RRC_IDLE/INACTIVE	Apple	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 6: Introduce the new UE capability to indicate whether UE supports the LPWUS when SDT is enabled.

Proposal 7: Introduce the new configuration in RRCRelease message to enable/disable LP-WUS when SDT is enabled.

R2-2505976	Remaining issues on LP-WUS in RRC IDLE or INACTIVE	LG Electronics Inc.	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 7	[OI 38304-10] Confirm that UE can initiate MO/MT-SDT while monitoring LP-WUS. No CR change is needed.

Discussions

-	ZTE, InterDigital, Xiaomi share the view from LG E.

-	Ericsson wonders in this case whether UE is transmitting small data and monitor LPWUS simultaneously.

-	Nokia think if SDT is triggered, MR is in operation. Nokia think one issue is when UE is monitoring LPWUS there is no RSRP measurement. Xiaomi agree with this issue.

-	InterDigital think we already excluded the P7 from Apple.

-	Xiaomi think if there is real concern about extra delay for UE measurement then one way is to disable the LPWUS.

-	vivo think intention of LG E proposal is fine, and suggest to update the wording ‘there is no impact to the SDT procedure’. Lenovo, QC agree.

-	Nokia think there is impact to the latency. LG E think it is not critical issue for delay tolerant services.

-	Ericsson think we need to specify that SDT is initiated then UE stops monitoring LP-WUS.

Confirm that SDT can be initiated while UE is monitoring LP-WUS, and there is no impact to the SDT procedure. Can check if any spec change is needed.

Other issues

R2-2505856	LP-WUS in Idle and Inactive	Ericsson	discussion	Rel-19	NR_LPWUS-Core	R2-2504288

Noted

Proposal 11	If the UE is not able to monitor the LP-WUS then the UE is required to monitor the following PO.

Discussions

-	LG E, Nokia, CATT support the proposal.

-	Xiaomi agree with the intention, and think we should consider the care when UE is doing cell reselection. CATT think the proposal covers different cases.

For the RRC-IDLE and RRC-INACTIVE, if the UE is not able to monitor the LP-WUS in all MO then the UE is required to monitor the following PEI and/or PO. Detailed changes to the spec can be further checked.

R2-2505394	Discussion on LP-WUS WUR in RRC_IDLE INACTIVE	vivo	discussion	Rel-19	NR_LPWUS-Core

Noted

Entry/exit condition for LP-WUS monitoring

Proposal 2: The conclusion on the entry/exit conditions for RRM relaxation/offloading for different LR types is applicable for the entry/exit conditions for LP-WUS monitoring, which is already captured in RRC and 38.304 running CR:

-	RAN2 assumes the entry/exit thresholds for LP-WUS monitoring for OFDM-based WUR measuring LP-SS only are the same as that for OOK-based WUR measuring LP-SS. It can be revisited based on RAN1/RAN4 process, if any. Network is allowed to provide either OOK based threshold or OFDM based WUR measuring SSB threshold or both.

-	RAN2 assumes for the entry/ exit conditions of LP-WUS monitoring: separate MR thresholds (according to RAN1 agreement)/LR thresholds can be configured for different types of LP WUR if a cell supports both types of LRs (can revisit based on RAN1 and RAN 4 progress, if any).

Discussions

-	HW think these needs RAN4 confirmation.

RAN2 assumes the entry/exit thresholds for LP-WUS monitoring for OFDM-based WUR measuring LP-SS only are the same as that for OOK-based WUR measuring LP-SS. Network is allowed to provide either OOK based threshold or OFDM based WUR measuring SSB threshold or both.

R2-2505629	Discussion on prioritizing the frequencies supporting LP-WUS	Huawei, HiSilicon, vivo, Nokia, Samsung, LG Electronics Inc., Apple, Ericsson, OPPO, Sharp, NEC	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 1: Support 1-bit flag for each inter-frequency in SIB. If the flag is set to true for a frequency and UE supports LP-WUS on this frequency, UE can consider this frequency to be the highest priority.

Discussions

-	Vodafone think we should not introduce this.

-	vivo think we need to conclude on this issue, and think it impact UE capability discussion.

-	QC think this has been discussed and would like not to reopen.

-	LG E think similar mechanism exists for many features so think this is important.

-	Nokia think there is no overload issue with the proposed solution.

UE capability

R2-2505394	Discussion on LP-WUS WUR in RRC_IDLE INACTIVE	vivo	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 1: The capability for supporting UE_ID based subgrouping is defined as one of the components of LP-WUS operation basic features based on OOK signal (62-1) and OFDM overlaid sequence (62-1a) in IDLE/INACTIVE.

The capability for supporting UE_ID based subgrouping is defined as one of the components of LP-WUS operation basic features based on OOK signal (62-1) and OFDM overlaid sequence (62-1a) in IDLE/INACTIVE.

R2-2505379	Further discussion on LP-WUS in RRC_IDLE/INACTIVE	Huawei, HiSilicon	discussion	Rel-19

Noted

R2-2505280	Remaining issues on LP-WUS paging monitoring	Xiaomi Communications, Huawei, HiSilicon, ZTE Corporation, Sanechips, Apple, Ericsson	discussion

Noted

Agreements on IDLE/INACTIVE procedures

On remaining issues RAN2 assumes NAS signalling is introduced to support enabling/disabling LP-WUS per UE. Inform SA2, CT1 and RAN3 about this conclusion. RAN2 assumes that NAS signalling needs to be extended to enable/disable LP-WUS for a UE in IDLE and INACTIVE. Detail signalling is up to SA2, CT1. RAN2 assumes that CN needs to inform gNB that LP-WUS is enabled/disabled for a UE in IDLE and INACTIVE. Details signalling is up to SA2, CT1, RAN3. RAN2 assumes that without such NAS singalling, UE is allowed to use LPWUS in IDLE and INACTIVE. The LS is approved unseen in R2-2506261 LP-WUS can be used in any cell, i.e., don’t introduce lastUsedCellOnly for LP-WUS. In RRC_INACTIVE state, for LP-WUS, when the UE uses the same i_s as for RRC_IDLE state, the UE shall use the same iPO LP-WUS as for RRC_IDLE state. Otherwise, the UE determines the iPO for LP-WUS for based on the i_s for RRC_INACTIVE state. In RRC_INACTIVE state with CN configured PTW, the SubgroupID for LP-WUS used outside CN PTW is the same as the SubgroupID used inside CN PTW. Confirm that SDT can be initiated while UE is monitoring LP-WUS, and there is no impact to the SDT procedure. Can check if any spec change is needed. For the RRC-IDLE and RRC-INACTIVE, if the UE is not able to monitor the LP-WUS in all MO then the UE is required to monitor the following PEI and/or PO. Detailed changes to the spec can be further checked. RAN2 assumes the entry/exit thresholds for LP-WUS monitoring for OFDM-based WUR measuring LP-SS only are the same as that for OOK-based WUR measuring LP-SS. Network is allowed to provide either OOK based threshold or OFDM based WUR measuring SSB threshold or both. On UE capability The capability for supporting UE_ID based subgrouping is defined as one of the components of LP-WUS operation basic features based on OOK signal (62-1) and OFDM overlaid sequence (62-1a) in IDLE/INACTIVE.

R2-2505236	Open issues on LP-WUS in IDLE and INACTIVE	CATT	discussion	Rel-19	NR_LPWUS-Core

R2-2505280	Remaining issues on LP-WUS paging monitoring	Xiaomi Communications, Huawei, HiSilicon, ZTE Corporation, Sanechips, Apple, Ericsson	discussion

R2-2505336	Discussion on LP-WUS in RRC_IDLE INACTIVE 	NEC	discussion	NR_LPWUS-Core

R2-2505379	Further discussion on LP-WUS in RRC_IDLE/INACTIVE	Huawei, HiSilicon	discussion	Rel-19

R2-2505381	Summary of [Post130][222][LPWUS] Potential solution to support enabling/disabling LP-WUS monitoring in IDLEI/NACTVE per UE (Huawei)	Huawei, HiSilicon	discussion	Rel-19

R2-2505394	Discussion on LP-WUS WUR in RRC_IDLE INACTIVE	vivo	discussion	Rel-19	NR_LPWUS-Core

R2-2505479	Remaining issues of LP-WUS in RRC_IDLE/INACTIVE	Apple	discussion	Rel-19	NR_LPWUS-Core

R2-2505529	Procedure and Configuration of LP-WUS in RRC Idle Inactive Mode	Samsung	discussion	Rel-19

R2-2505588	Remaining issues on LP-WUS in IDLE and INACTIVE	NTT DOCOMO INC.	discussion	Rel-19	NR_LPWUS-Core

R2-2505605	Discussion on LP-WUS procedure and configuration	OPPO	discussion	Rel-19	NR_LPWUS-Core

R2-2505629	Discussion on prioritizing the frequencies supporting LP-WUS	Huawei, HiSilicon, vivo, Nokia, Samsung, LG Electronics Inc., Apple, Ericsson, OPPO, Sharp, NEC	discussion	Rel-19	NR_LPWUS-Core

R2-2505655	Disabling/enabling LP-WUS in RRC Idle/Inactive mode	Sony	discussion	Rel-19	NR_LPWUS-Core

R2-2505682	Open issues on LP-WUS in RRC_IDLE/INACTIVE mode	Lenovo	discussion	Rel-19

R2-2505752	LP-WUS in IDLE and INACTIVE	Nokia	discussion	Rel-19	NR_LPWUS-Core

R2-2505779	Remaining issues in IDLE/INACTIVE procedure for LP-WUS	Tejas Network Limited	discussion	Rel-19

R2-2505856	LP-WUS in Idle and Inactive	Ericsson	discussion	Rel-19	NR_LPWUS-Core	R2-2504288

R2-2505906	Remaining issues on LP-WUS operation in RRC_IDLE/INACTIVE modes	InterDigital, Inc.	discussion	Rel-19	NR_LPWUS-Core

R2-2505968	Remaining issues of LP-WUS operation in IDLE/INACTIVE	CMCC	discussion	Rel-19	NR_LPWUS-Core

R2-2505976	Remaining issues on LP-WUS in RRC IDLE or INACTIVE	LG Electronics Inc.	discussion	Rel-19	NR_LPWUS-Core

R2-2505992	Procedure and configuration of LP-WUS for IDLE and INACTIVE mode	ZTE Corporation, Sanechips	discussion	Rel-19	NR_LPWUS-Core

R2-2506038	IDLE/Inactive LP-WUS disabling and enabling	Qualcomm Incorporated	discussion	NR_LPWUS-Core

### 8.4.3	RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE

RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions

RRC-7/38304-3 Whether separate exit condition is needed for Rel-19 RRM relaxation

R2-2505967	Remaining issues of RRM measurement relaxation and offloading in RRC_IDLE INACTIVE	CMCC	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 1: No separate exit condition is needed for serving cell RRM relaxation, and the exit condition can be defined as failing to meet the entry condition, i.e., MR or LR is below the configured threshold.

R2-2505907	Remaining issues on RRM measurement relaxation and offloading	InterDigital, Inc.	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 3: For relaxed case, support the exit condition based on LR measurement.

Discussions

-	Ericsson not sure what is the issue if the exit condition is based on LR.

-	LG E think if we go with CMCC proposal there is no issue. QC think there would be ping-pong issue. vivo do not see any ping-pong issue. OPPO, ZTE, Xiaomi, Lenovo agree with vivo.

-	CATT think if network only configure MR based condition then due to relaxed measurement the latency to quit the relaxation status is larger. HW, Apple agree with CATT. Vivo think this can be handled by NW implementation.

-	Xiaomi think even if we do relaxation the accuracy is not changed, so think there is no issue.

-	Samsung support the InterDigital proposal.

-	Ericsson think for full offloading we rely on LR.

-	Sony think the entry condition should be higher.

-	HW think there is no technical issue with separate condition. Ericsson think we should have separate entry and exist conditions.

The exit condition for RRM relaxation is defined as ‘not fulfilling the entry condition’.

RRC-10/38304-6, Whether UE low mobility criterion or stationary criterion should be considered for RRM relaxation/offloading.

R2-2506040	Open issues on LP-WUS RRM measurement relaxation and offloading	Qualcomm Incorporated	discussion	NR_LPWUS-Core

Noted

Proposal 5	RAN2 does not consider low mobility or stationary status to evaluate whether to enter or exit from Rel-19 RRM relaxation/offloading mode.

R2-2505530	RRM measurement relaxation and offloading in RRC Idle Inactive Mode	Samsung	discussion	Rel-19

Noted

Proposal 2: (RRC-10, 38304-6) For Rel19 MR serving/neighbour cell measurement relaxation for UEs capable of LP-WUS, UE not at cell edge and UE with low mobility are reused.

Proposal 3: (RRC-10, 38304-6) For Rel19 serving cell measurement (full) offloading for UEs capable of LP-WUS, UE not at cell edge and UE with low mobility are reused.

Discussions

-	Ericsson think we should avoid the case that a moving UE not measuring the serving cell for a very long time. Ericsson agree with P2 from Samsung but think the issue is only for neighbour cell measurement.

-	HW support QC proposal. vivo also agree. Vivo think UE implementation can handle the low/high mobility.  LG E share vivo’s understanding.

-	Lenovo support Samsung proposal and think we could reuse similar mechanism as Rel-16. OPPO agree and think the effort to extend to LR is limited. Ericsson, ZTE, DCM, Nokia, InterDigital also support.

-	ZTE think turning on and off the MR consume high power so especially for offloading case the low mob condition is meaningful.

-	Apple think we can consider mobility status is needed and let R4 decide whether any requirement is needed for that.

-	CATT think the case is different than legacy, and think NW can configure proper thresholds already.

-	Xiaomi do not want to extend the condition of low mob to LR.

-	Qulacomm think this is not considered in RAN4 so far, and think it is not clear how to handle different conditions if we now introduce more conditions.

RRC-15/38304-2, FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available.

R2-2505395	Discussion on RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	vivo	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 8: (RRC-15/38304-2) There is no impact on R16/R17 criterion of neighboring cell RRM relaxation regardless whether MR serving cell RRM measurement results are available or not, i.e., LR measurement won’t be used for R16/R17 RRM relaxation condition.

38304-12: FFS the impact on legacy low-mobility criteria for MR with LP-WUS.

R2-2505289	Remaining issues on RRM measurement relaxation for RRC_IDLE_INACTIVE	Xiaomi Communications	discussion

Noted

Proposal 4	[38304-12] When UE exits fully offload state, UE will reset the reference RSRP for “low mobility” criterion evaluation for MR if configured

R2-2505237	Open issues on RRM Relaxation and Offloading in IDLE and INACTIVE	CATT	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 5: RAN2 confirm the case that the UE (re)-starts the evaluation of R16 low mobility criterion after the UE exits fully offloading case is similar to the case that the UE (re)-starts the evaluation of R16 low mobility criterion after camping on a cell for a period. There is no specification impact for R16 low mobility criterion when UE exits using fully offloading.

[AT131][204][LPWUS] Proposals for RRC-10/38304-6, RRC-15/38304-2, 38304-12 (CATT)

Intended outcome: Proposals in R2-2506247 for RRC-10/38304-6, RRC-15/38304-2, 38304-12.

Deadline: Before CB

R2-2506247[AT131][204][LPWUS] Proposals for RRC-10/38304-6, RRC-15/38304-2, 38304-12 (CATT)

Noted

Discussions

P1

WI Rapporteur: in offline discussions there were different understanding on detailed mechanism if we introduce low mob criterion.

Chair: One company still has concern on Rel-19 UE cannot use low mobility criterion to save power.

RAN2 assumes UE low mobility criterion is not included for Rel-19 LP-WUS RRM relaxation/offloading mode.

LR measurement is not used for R16/R17 RRM relaxation condition.

R2-2505857	LP-WUS and RRM measurements	Ericsson	discussion	Rel-19	NR_LPWUS-Core	R2-2504289

Noted

Proposal 2	It is left to NW implementation to configure full or partial LP-WUS cell coverage, i.e. configuration of s-SearchThresholdP5 and s-SearchThresholdP6 is left to NW implementation.

Agreements on RRM measurement relaxation and offloading

The exit condition for RRM relaxation is defined as ‘not fulfilling the entry condition’. RAN2 assumes UE low mobility criterion is not included for Rel-19 LP-WUS RRM relaxation/offloading mode. LR measurement is not used for R16/R17 RRM relaxation condition.

R2-2505237	Open issues on RRM Relaxation and Offloading in IDLE and INACTIVE	CATT	discussion	Rel-19	NR_LPWUS-Core

R2-2505289	Remaining issues on RRM measurement relaxation for RRC_IDLE_INACTIVE	Xiaomi Communications	discussion

R2-2505480	Remaining issues of LP-WUS RRM Measurement	Apple	discussion	Rel-19	NR_LPWUS-Core

R2-2505530	RRM measurement relaxation and offloading in RRC Idle Inactive Mode	Samsung	discussion	Rel-19

R2-2505596	Remaining issues on RRM measurement relaxation and offloading	NTT DOCOMO INC.	discussion	Rel-19	NR_LPWUS-Core

R2-2505606	Discussion on the remaining issues on RRM measurement	OPPO	discussion	Rel-19	NR_LPWUS-Core

R2-2505683	Open issues on RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	Lenovo	discussion	Rel-19

R2-2505737	Further discussion on the criteria for RRM measurement relaxation and offloading	Huawei, HiSilicon	discussion	Rel-19	NR_LPWUS-Core

R2-2505753	RRM measurement relaxation in RRC_IDLE/INACTIVE	Nokia	discussion	Rel-19	NR_LPWUS-Core

R2-2505780	Remaining issues in LP-WUS based RRM relaxation and offloading 	Tejas Network Limited	discussion	Rel-19

R2-2505803	Remaining issues for LP-WUS RRM	ZTE Corporation, Sanechips	discussion	Rel-19	NR_LPWUS-Core

R2-2505857	LP-WUS and RRM measurements	Ericsson	discussion	Rel-19	NR_LPWUS-Core	R2-2504289

R2-2505907	Remaining issues on RRM measurement relaxation and offloading	InterDigital, Inc.	discussion	Rel-19	NR_LPWUS-Core

R2-2505967	Remaining issues of RRM measurement relaxation and offloading in RRC_IDLE INACTIVE	CMCC	discussion	Rel-19	NR_LPWUS-Core

R2-2505977	Remaining issues on measurement offloading and relaxation	LG Electronics Inc.	discussion	Rel-19	NR_LPWUS-Core

R2-2506040	Open issues on LP-WUS RRM measurement relaxation and offloading	Qualcomm Incorporated	discussion	NR_LPWUS-Core

### 8.4.4	Procedures for LP-WUS in RRC_CONNECTED

Procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring.

MAC-X1 The impact to the BWP switching mechanism

R2-2505630	Further discussion on LP-WUS for RRC_CONNECTED mode	Huawei, HiSilicon	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 6: (MAC-X1) The bwp-InactivityTimer can be stopped when UE monitors LP-WUS, in this case, the UE needs to (re)start the bwp-InactivityTimer when it receives the LP-WUS for PDCCH monitoring. Otherwise, there is no impact on bwp-InactivityTimer.

R2-2505684	Open issues on LP-WUS in RRC Connected mode	Lenovo	discussion	Rel-19

Noted

Proposal 6: (MAC-X1) UE does not start or re-start the bwp-InactivityTimer when receiving the LP-WUS.

Discussion

-	HW think R1 is discussing and think we can wait.

-	Xiaomi think we should avoid UE impact in BWP handling behaviour unless R1 decided so. Apple agree, and think from R2 point of view there is no special handling.

-	Ericsson wonders what the issue is if we do not specify anything. HW think it is possible that UE can only use LPWUS in some BWPs but not all.

-	Nokia, LG E, QC, InterDigial, CATT, Ericsson support Lenovo proposal.

-	vivo support HW proposal, and think if UE receives LPWUS then there may be data transmission following.

RAN2 assume UE does not start or re-start the bwp-InactivityTimer when receiving the LP-WUS.

MAC-X2 Whether to consider the multiple LP-WUS cycles

R2-2505396	Discussion on LP-WUS WUR in RRC_Connected	vivo	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 4: (MAC-X2) For Option 1-2, two LP-WUS cycles will be configured. RAN2 assumes the design of switching between long LP-WUS cycle and short LP-WUS cycle is same as the C-DRX mechanism, including the MAC CE control, time control.

R2-2505993	Procedure for LP-WUS in RRC_Connected state	ZTE Corporation, Sanechips	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 3(RRC-3): For Option 1-2, RAN2 confirms that one single LP-WUS cycle is defined to determine the LP-WUS monitoring occasion.

MAC-X3 UAI of the LP-WUS preference

R2-2505630	Further discussion on LP-WUS for RRC_CONNECTED mode	Huawei, HiSilicon	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 1: (MAC-X3) UE can send assistance information to the network indicating to disable the LP-WUS functionality or whether the LP-WUS can be enabled again.

Proposal 2: (MAC-X3) When to send the assistance information is up to UE implementation, without additional configuration to the UE.

Agreements on RRC connected procedure

RAN2 assume UE does not start or re-start the bwp-InactivityTimer when receiving the LP-WUS.

R2-2505108	Discussing on LP-WUS monitoring in Connected mode	Xiaomi	discussion	Rel-19	NR_LPWUS-Core

R2-2505238	Analysis on LP-WUS for RRC_CONNECTED	CATT	discussion	Rel-19	NR_LPWUS-Core

R2-2505396	Discussion on LP-WUS WUR in RRC_Connected	vivo	discussion	Rel-19	NR_LPWUS-Core

R2-2505463	Remaining issues on LP-WUS in RRC_CONNECTED	LG Electronics Inc.	discussion	Rel-19	NR_LPWUS-Core

R2-2505481	Remaining issues of LP-WUS in RRC_CONNECTED	Apple	discussion	Rel-19	NR_LPWUS-Core

R2-2505531	Procedures for LP-WUS in RRC Connected Mode	Samsung	discussion	Rel-19

R2-2505581	LP-WUS in RRC_CONNECTED	Nokia, Nokia Shanghai Bell	discussion	NR_LPWUS-Core

R2-2505597	Remaining issues on LP-WUS in RRC_CONNECTED	NTT DOCOMO INC.	discussion	Rel-19	NR_LPWUS-Core

R2-2505607	Discussion on LP-WUS in RRC_CONNECTED	OPPO	discussion	Rel-19	NR_LPWUS-Core

R2-2505630	Further discussion on LP-WUS for RRC_CONNECTED mode	Huawei, HiSilicon	discussion	Rel-19	NR_LPWUS-Core

R2-2505645	LP-WUS in CONNECTED mode	InterDigital	discussion	Rel-19	NR_LPWUS-Core

R2-2505684	Open issues on LP-WUS in RRC Connected mode	Lenovo	discussion	Rel-19

R2-2505782	Remaining issues in CONNECTED procedure for LP-WUS	Tejas Network Limited	discussion	Rel-19

R2-2505858	LP-WUS in Connected	Ericsson	discussion	Rel-19	NR_LPWUS-Core	R2-2504290

R2-2505942	Discussion on LP-WUS operation in CONNECTED mode	CMCC	discussion	Rel-19	NR_LPWUS-Core

R2-2505993	Procedure for LP-WUS in RRC_Connected state	ZTE Corporation, Sanechips	discussion	Rel-19	NR_LPWUS-Core

R2-2506039	Open issues on LP-WUS operation in CONNECTED state	Qualcomm Incorporated	discussion	NR_LPWUS-Core

## 8.5	Network Energy Saving Enh.

(Netw_Energy_NR_enh-Core; leading WG: RAN1; REL-19; WID: RP-242354)

Time budget: 1 TU

Tdoc Limitation: 3 tdocs

### 8.5.1	Organizational

Incoming LS, WI rapporteur inputs, CR rapporteur inputs (including post email discussion [POST130][107], [108], [109], [119], summary of identified stage-3 open issues that need online discussion and rapporteur’s suggestions if needed, details of UE capability discussion if needed).

WI is completed from RAN2 point of view.

R2-2505051	Reply LS on new servingCellMO of OD-SSB on SCell (R4-2508440; contact: Apple)	RAN4	LS in	Rel-19	Netw_Energy_NR_enh	To:RAN2	Cc:RAN1

Noted.

R2-2505564	Introduction of Network Energy Savings Enhancements	Huawei, HiSilicon	CR	Rel-19	38.300	18.6.0	1013	-	B	Netw_Energy_NR_enh-Core

[Session chair]: What about the following editor’s note?

“Editor’s note: FFS whether to merge OD-SSB and SSB adaptation into one section.”

Editor’s note needs to be removed in the revision.

Endorsed and considered as baseline for further discussion.

[POST131][108][NES] (Huawei)

Scope: Update NES 38.300 CR (including this meeting agreements also).

Intended outcome: 38.300 CR in R2-2506219 to be agreed.

Deadline: Short email discussion

=> Agreed in R2-2506615 (38.300)

R2-2506615	Introduction of Network Energy Savings Enhancements	Huawei, HiSilicon	CR	Rel-19	38.300	18.6.0	1013	2	B	Netw_Energy_NR_enh-Core

=> Agreed

R2-2505497	Summary report of [POST130][108][NES] 38.304 CR (Apple)	Apple (Rapporteur)	discussion	Rel-19	Netw_Energy_NR_enh-Core

Noted.

R2-2505496	Running 38.304 CR for network energy saving	Apple (Rapporteur)	CR	Rel-19	38.304	18.4.0	0442	-	B	Netw_Energy_NR_enh-Core

Endorsed and considered as baseline for further discussion.

[POST131][109][NES] (Apple)

Scope: Update NES 38.304 CR (including this meeting agreements also).

Intended outcome: 38.304 CR in R2-2506220 to be agreed.

Deadline: Short email discussion

=> Agreed in R2-2506561

R2-2506561	Introduction of Rel-19 network energy saving to TS 38.304	Apple (Rapporteur)	CR	Rel-19	38.304	18.4.0	0442	2	B	Netw_Energy_NR_enh-Core

=> Agreed

R2-2505708	Report of [POST130][107][NES] (Ericsson)	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core

[Ericsson]: Indicates there is one issue that CR rapporteur accepted during the email discussion, but realize we need more discussion. Will indicate that issue to session chair (including WI rapporteur), then we can the discussion back later. [Session chair]: In the email, Ericsson clarified that it was already included in the offline discussion (P5 in R2-2505507).

Noted.

R2-2505699	Introduction of enhancements for network energy efficiency 	Ericsson	CR	Rel-19	38.331	18.6.0	5428	-	B	Netw_Energy_NR_enh-Core

Endorsed and considered as baseline for further discussion.

[POST131][110][NES] (Ericsson)

Scope: Update NES 38.331 CR (including this meeting agreements also).

Intended outcome: 38.331 CR in R2-2506221 to be agreed.

Deadline: Short email discussion

=> Agreed in R2-2506221

R2-2506221	Introduction of enhancements for network energy efficiency 	Ericsson	CR	Rel-19	38.331	18.6.0	5428	1	B	Netw_Energy_NR_enh-Core

=> Agreed

R2-2505791	Report of [POST130][109][NES] Comments to 38.321 CR for NES	InterDigital	discussion	Rel-19	Netw_Energy_NR_enh-Core

Noted

R2-2505792	Introduction of network energy saving enhancements to TS 38.321	InterDigital	CR	Rel-19	38.321	18.6.0	2110	-	B	Netw_Energy_NR_enh-Core

Endorsed and considered as baseline for further discussion.

[POST131][111][NES] (InterDigital)

Scope: Update NES 38.321 CR (including this meeting agreements also).

Intended outcome: 38.321 CR in R2-2506222 to be agreed.

Deadline: Short email discussion

=> Agreed in R2-2506571 (38.321)

R2-2506571	Introduction of network energy saving enhancements to TS 38.321	InterDigital	CR	Rel-19	38.321	18.6.0	2110	2	B	Netw_Energy_NR_enh-Core

=> Agreed

R2-2505987	Introduction of UE capability for network energy saving enhancement in TS 38.306	ZTE Corporation, Sanechips	draftCR	Rel-18	38.306	18.6.0	F	Netw_Energy_NR_enh-Core

Endorsed

R2-2505988	Introduction of UE capability for network energy saving enhancement in TS 38.331	ZTE Corporation, Sanechips	draftCR	Rel-18	38.331	18.6.0	A	Netw_Energy_NR_enh-Core

Endorsed

[POST131][112][NES] (ZTE)

Scope: Update NES UE capability CRs (including this meeting agreements also).

Intended outcome: 38.331 CR in R2-2506223 and 38.306 CR in R2-2506224 to be endorsed.

Deadline: Short email discussion

=> Endorsed in:

- R2-2506223 (38.331)

- R2-2506224 (38.306)

R2-2506223	Introduction of UE capability for network energy saving enhancement in TS 38.331	ZTE Corporation, Sanechips	draftCR	Rel-18	38.331	18.6.0	B	Netw_Energy_NR_enh-Core

R2-2506224	Introduction of UE capability for network energy saving enhancement in TS 38.306	ZTE Corporation, Sanechips	draftCR	Rel-18	38.306	18.6.0	B	Netw_Energy_NR_enh-Core

[AT131][101][NES] (Apple)

Scope: Discuss and conclude the following stage-3 issues:

- Size of OD-SSB configuration index? P1 in R2-2506050, P1 in R2-2506112

- Whether RRC activation supports implicit deactivation? P1 of R2-2505338, P2 in R2-2506089

- Any other new PEI configuration? P3 in R2-2505500

- Information structure of paging adaptation: P2a and P2b in R2-2505255

- TP of SSB adaptation: P4 in R2-2506051, P1 in R2-2506095

- Other clarifications/corrections: P7 of R2-2505315, P4 and P7 in R2-2506089, P2 in R2-2505506, P6 in R2-2505097, P3 in R2-2505338, P5 in R2-2505507, P4 in R2-2505253, P1 in R2-2505255, and P4 in R2-2505170

Intended outcome: Discussion summary in R2-2506211.

Deadline: Phase-1: Email discussion (until 8/26 Tuesday noon), Phase-2: F2F discussion (8/26 Tuesday afternoon, offline meeting room will be announced) => Completed.

R2-2506211	Summary report of [AT131][101][NES] (Apple)	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

Agreeable proposals

Proposal 1 (13/15) maxNrofOD-SSB is set to 16.

Proposal 2 (13/15) RAN2 don’t pursue further optimization of MAC-CE format.

Proposal 3 (14/15) Following RAN1 conclusion, not support implicit deactivation with RRC-based activation. The following revisions to be reflected in RRC running CR:

•	od-ssb-ActivationStatus: Indicates the activation status of this OD-SSB pattern upon configuration. Only one OD-SSB pattern can be activated at one point of time.

•	od-ssb-nrofBurst: Indicates the number of OD-SSB bursts to be transmitted after OD-SSB is activated. Network only configures this field when od-ssb-ActivationStatus is absent.

Proposal 4 (7/13) The following Rel-19 PEI configurations with the same value range (except pei-FrameOffset-r19) as Rel-17 PEI configuration need to be introduced.

o	po-NumPerPEI-r19

o	payloadSizeDCI-2-7-r19

o	pei-FrameOffset-r19: extend to 32 radio frame

Proposal 5 (15/15) Adopt the following information structure of paging adaptation

•	If network supports paging adaptation, both pagingAdaptation-NS-r19 and pagingAdaptationNAndPagingFrameOffset are configured.

•	pagingAdaptation-NS-r19,  pagingAdaptationNAndPagingFrameOffset and  firstPDCCH-MonitoringOccasionOfPO are grouped together in one IE. pagingAdaptation-NS-r19,  pagingAdaptationNAndPagingFrameOffset are mandatory in this IE. firstPDCCH-MonitoringOccasionOfPO-r19 is optionally configured in the IE.

Proposal 8: Revise the type of od-ssb-nrofBurst as ENUMERATED {5, 10, 15, 20, 25, 30, 40, 50, 75, 100, 150, 200}.

Proposal 9: OD-SSB SMTC adaptation apply to both inter and intra frequency measurement, and it should be clear which SMTC to use for the UE. It is up to RRC CR Rapporteur how to capture it in running CR.

Proposal 10: On od-SSB-periodicity, remove default value of 5ms and change the presence to mandatory.

Proposal 11: For Case#2, if the PBCH block power or SSB sub-carrier spacing or absolute frequency for OD-SSB is not configured, the UE applies the same value configured for the always-on SSB. RRC running CR to reflect this agreement in the field description.

Proposal 12: Adopt the change suggested by Proposal 5 of R2-2505507.

Proposal 13: Adopt the following revised correction on OD-SIB1:

5.2.2.4.2x             Actions upon reception of SIBxx

1> if the UE has (re)selected to a cell providing OD-SIB1, the UE stores the configuration for SIB1 request for this cell and considers it valid while camping in this cell:

2>	store the SIBxx;

2>	SIB1 request configuration in the SIBxx is valid for acquiring OD-SIB1 of this cell in accordance with clause 5.2.3.3.x;

2>	SIB1 request configuration of another cell in this stored SIBxx is valid for acquiring OD-SIB during reselection to that cell, and after reselection to that cell if the stored SIBxx is a valid version for that cell in accordance with clause 5.2.2.2.1;

Proposal 14: If firstPDCCH-MonitoringOccasionOfPO-r19 is not configured, UE does not determine POs based on starting PDCCH monitoring occasion number. The starting PDCCH monitoring occasion number of (i_s + 1)th PO is equal to i_s * S*X.

Proposal 15: When pagingAdaptationPEI-Config is signaled in system information, pagingAdaptationFirstPDCCH-MonitoringOccasionOfPEI-O-r19 should be configured.

All proposals above (proposal 1 - 5, and 8 - 15) are agreed.

Need online discussion:

Proposal 6: Discuss whether to adopt the SSB adaptation TP (from Huawei or Ericsson) as baseline.

[Apple]: Propose first to handle the TP on OD-SSB, then we can comeback to proposal 6. [Apple]: Propose to have same way as OD-SSB. [Ericsson]: Prefer keeping the current running RRC way to avoid changes in procedure text in the last meeting. [Apple]: Huawei’s TP already provides the procedure text. Xiaomi can merge it. [Ericsson]: Prefer to handle TP in separate for this case. Either option can work. [Xiaomi]: Although either option works, prefer to have same way as OD-SSB.

Comeback:

R2-2506231	TP for SSB adaptation	Huawei	discussion	Rel-19	Netw_Energy_NR_enh-Core

[Huawei]: The new parts are text in 5.5.2.10 and adding SMTC in Measurement object. [Ericsson]: Wonder if we need also some UE procedure (e.g. like something captured in TP on OD-SSB). [Session chair]: It is up to RRC CR rapporteur to see the existing legacy section and analyse whether we need some UE procedure change also or not.

TP in R2-2506231 is baseline.

### 8.5.2	On-demand SSB SCell operation

Remaining essential open issues including stage-3 identified open issues if needed, etc.

1. Information structure of OD-SSB-Config (including where to include servingCellMO)?

R2-2505498	Remaining issues on on-demand SSB for SCell	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 3 (RRC open issue 3): Following RAN2#130 agreement, move new servingCellMO for OD-SSB under ServingCellConfig (i.e. servingCellMO is per SCell configuration rather than per OD-SSB configuration).

Proposal 4 (RRC open issue 3): Move SMTC for OD-SSB under MeasObjectNR and add procedure text.

Following RAN2#130 agreement, move new servingCellMO for OD-SSB under ServingCellConfig (i.e. servingCellMO is per SCell configuration rather than per OD-SSB configuration).

Move SMTC for OD-SSB under MeasObjectNR and add procedure text.

[AT131][105][NES] (Xiaomi)

Scope: Provide and discuss complete TP for option1 (including signalling and UE behaviour)

Intended outcome: TP in R2-2506215.

Deadline: TP will be treated in Wednesday session => Completed.

On proposal 3 and 4:

[Ericsson, Nokia]: If we agree with proposal 3, it needs more specification effort, e.g. SMTC list needs to be moved into measurement object and we should define an association between SMTC in the list and the corresponding OD-SSB configuration. Understand either option can work. [LG, Huawei, Xiaomi]: Support the proposal to keep the design of measurement configuration that has been for long time. [Session chair]: Agree with that either option can work. Suggest to follow majority companies’ preference.

- Option1: proposal 3 + proposal 4

  - Apple, Huawei, Xiaomi, Samsung, Google, Fujitsu, LG, Vivo, CATT

- Option2: keep the current information structure

  - Ericsson, Nokia, ZTE, OPPO, Qualcomm

Proposal 5 (RRC open issue 3): Move the following OD-SSB IEs which can’t be dynamically adapted via MAC-CE under SCellConfig (i.e. they can only be reconfigured by RRC rather than by MAC-CE)

•	od-ssb-sfn-Offset-r19

•	od-ssb-halfFrameIndex-r19

•	od-ssb-absoluteFrequency-r19

•	od-ssbSubcarrierSpacing-r19

•	od-ss-PBCH-BlockPower-r19

Agree with the intention of proposal 5.

How to implement it to RRC is left to RRC rapporteur.

On proposal 5:

[Ericsson]: Although it was not clear according to RAN1’s provided excel file, agree with intention. [LG, NTTDCM]: Understand od-ssb-sfn-Offset-r19 and od-ssb-halfFrameIndex-r19 can be also dynamically changed by MAC CE. [Apple]: It is not for OD-SSB. Understand there was similar discussion/decision for SSB adaptation. It is very clear what can be dynamically changed by MAC CE from RAN1 agreements.

R2-2506215	Text proposal for option 1	Xiaomi	discussion	Rel-19	Netw_Energy_NR_enh-Core

TP in R2-2506215 is baseline.

[Ericsson]: Proposed TP seems complicated and want to revert RAN2 agreement back. [Nokia]: We can first agree with three cases that Apple indicated during offline, then we may need further discussion how to implement the three cases. [Apple]: Understand Xiaomi’s TP already covers all three cases well. [Ericsson]: Understand this TP is also different compared to how to capture Rel-17 additional MO. [Xiaomi]: The difference comes from more cases for OD-SSB. [Session chair]: Suggest check companies’ opinions again after seeing the TP.

- Option1: Current running CR: Ericsson, Nokia, OPPO

- Option2: TP provided by Xiaomi: LG, Apple, Huawei, Xiaomi, Samsung

Comeback:

[Ericsson]: Wonder, when OD-SSB is activated, whether CSI measurement cannot be done. [LG]: Not understand the concern. The TP just specifies the relation between measurement and OD-SSB. [Nokia]: Understand the Ericsson’s concern is more RAN4 scope. [LG]: If concerned, we can just add legacy behaviours under the bullet “3> if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains an rsType set to ssb and ssb-ConfigMobility is configured in the measObject indicated by the servingCellMO-OD:”

TP is updated that measurement on CSI-RS is allowed as legacy.

Under “3>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains an rsType set to ssb and ssb-ConfigMobility is configured in the measObject indicated by the servingCellMO-OD:”, the following legacy UE behaviour is added.

4>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport and contains an rsType set to ssb:

5>	derive layer 3 filtered RSRP and RSRQ per beam for the serving cell based on SS/PBCH block, as described in 5.5.3.3a;

4>	derive serving cell measurement results based on SS/PBCH block, as described in 5.5.3.3;

3>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains an rsType set to csi-rs and CSI-RS-ResourceConfigMobility is configured in the measObject indicated by the servingCellMO:

4>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport and contains an rsType set to csi-rs:

5>	derive layer 3 filtered RSRP and RSRQ per beam for the serving cell based on CSI-RS, as described in 5.5.3.3a;

4>	derive serving cell measurement results based on CSI-RS, as described in 5.5.3.3;

“2> if the servingCellMO-OD is configured and OD-SSB transmission is activated:” is updated to “2> else if the servingCellMO-OD is configured and OD-SSB transmission is activated:”

2. Definition of intra-F measurement and inter-F measurement

R2-2505498	Remaining issues on on-demand SSB for SCell	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 6: RAN2 discuss and down-select between the following two options on how to define intra-frequency and inter-frequency measurement when OD-SSB and AO-SSB are in different frequencies:

•	Option 1: When the center frequency of either AO-SSB or OD-SSB of one SCell is same as that of neighbor cell SSB, it is intra-frequency measurement. Otherwise, it is inter-frequency measurement.

•	Option 2: When the center frequency of the applied servingCellMO of one SCell is same as that of neighbor cell SSB, it is intra-frequency measurement. Otherwise, it is inter-frequency measurement.

o	Note: which servingCellMO is applied follows RAN2#130 agreement.

[Ericsson]: There are some description for intra-F and inter-F in the ASN.1 form, we can rename that if it confuses us. [LG]: Understand the current intra-F and inter-F measurement definition is based on the SSB frequency location and there is no reason to change it from RAN2 point of view. Since it may be helpful to RAN4’s further discussion, it is good to capture RAN2 agreement/understanding.

RAN2 understand the current definition of intra-F measurement ad inter-F measurement is kept even for OD-SSB case.

Proposal 7: RAN2 discuss and down-select between the following two options on how to handle measurement gap when OD-SSB and AO-SSB are in different frequencies:

•	Option 1: if any of the UE configured BWPs (other than initial BWP) do not contain the frequency domain resources of both AO-SSB and OD-SSB, a measurement gap configuration is always provided.

•	Option 2: if any of the UE configured BWPs (other than initial BWP) do not contain the frequency domain resources of SSB in the applied servingCellMO, a measurement gap configuration is always provided.

o	Note: which servingCellMO is applied follows RAN2#130 agreement.

Noted.

R2-2505097	Remaining open issues on OD-SSB SCell operation	LG Electronics France	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2505113	Discussion on On-Demand SSB	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2505168	Consideration on on-demand SSB SCell operation	CATT	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2505254	On-demand SSB SCell Operation	Samsung	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2505276	Some details on OD-SSB for NES cell	Quectel	discussion

R2-2505315	Remaining open issues on OD-SSB	Xiaomi	discussion	Netw_Energy_NR_enh-Core

R2-2505338	Remaining issues of On-demand SSB SCell operation	vivo	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2505498	Remaining issues on on-demand SSB for SCell	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2505506	Remaining issues on on-demand SSB SCell operation	Fujitsu	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2505526	Discussion on on-demand SSB SCell operation	NTT DOCOMO, INC.	discussion	Rel-19

R2-2505566	Remaining issues on OD-SSB	Sharp	discussion

R2-2505790	Remaining issues for On demand SSB	InterDigital	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2505845	Discussion on remaining issues of on-demand SSB?	Qualcomm Incorporated	discussion

R2-2505943	Discussion on open issues of on-demand SSB for Scell	CMCC	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2505989	Remaining issues of on demand SSB SCell operation	ZTE Corporation, Sanechips	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2506050	Discussion on on-demand SSB SCell operation for NES	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2506089	On demand SSB handling	Nokia, Nokia Shanghai Bell	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2506112	Discussion on on-demand SSB for NES	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core

### 8.5.3	On-demand SIB1

Remaining essential open issues including whether to support SUL in OD-SIB1 cell, stage-3 identified open issues if needed, etc.

1. Whether to support SUL in OD-SIB1 cell?

R2-2505253	On-demand SIB1	Samsung	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1a: SIB 1 request is supported for both SUL and NUL.

Proposal 1b: RSRP threshold for UL carrier selection is included in SIB1 request configuration.

R2-2505990	Remaining issues of on demand SIB1	ZTE Corporation, Sanechips	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 2: Follow RAN1 conclusion on whether to support SUL for on demand SIB1 request configuration, i.e. SupplementaryUL is not included in the UL-WUS configuration. LS should be sent to RAN1 for confirmation if RAN2 make different agreement.

[Samsung]: Understand the intention of “SupplementaryUL is not included in the UL-WUS configuration” was not really regarding whether SUL is supported or not for OD-SIB1. It was about whether SUL was included or not in BWP configuration of OD-SIB1 REQ configuration. [Apple]: Understand there was no real technical discussion regarding whether SUL is supported or not in RAN1, just due to lack of time, RAN1 feature leader didn’t include SUL without actual discussion. Understands RAN2 can make decision. [Nokia]: Understand the RAN1 situation at that moment, but do we really have time to ask it to RAN1 to support SUL. Better to forget SUL due to lack of time. [Xiaomi]: Understand there is no actual RAN1 impact, even though RAN2 agrees to support SUL.

SIB 1 request is supported for both SUL and NUL.

RSRP threshold for UL carrier selection is included in SIB1 request configuration.

[AT131][106][NES] (Samsung)

Scope: Provide and discuss LS.

Intended outcome: LS in R2-2506216.

Deadline: Email approval until end of Wednesday => Handled online => Completed.

R2-2506216	LS on RAN2 agreement on SUL support for SIB1 request	LSout	To: RAN1	Rel-19	Netw_Energy_NR_enh-Core

Approved.

2. Clarification on si-BroadcastStatus

R2-2505562	Discussion on remaining issues of on-demand SIB1 operation for NES	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 5: Clarify in the normative text that a UE supporting OD-SIB1 considers the si-BroadcastStatus in the stored SIB1 as the latest one if a cell is providing OD-SIB1.

“Since the normative text already captures the UE behaviour, it is optional to include it in si-BroadcastStatus. In our view it can be captured in both places for clarity of the si-BroadcastStatus behaviour.”

[Ericsson]: Agree with intention, but wants to avoid “a cell is providing OD-SIB1”. [Samsung]: Instead, we can use the existing conditions, e.g. something like “if ssbCarrierOffset in MIB indicates no SIB1 and the UE has the corresponding OD-SIB1 REQ configuration”. [Ericsson]: Agree with Samsung.

Agree with the intention, but the existing conditions that has same meaning as “a cell is providing OD-SIB1” will be used. It is only applicable to RRC connected state. How to implement is up to RRC rapporteur.

Agree to include the similar description in the field description of si-BroadcastStatus.

3. Clarification on SIB1 update upon reception of short message

R2-2505339	Remaining issues of On-demand SIB1	vivo	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1: RAN2 to discuss how to understand the previous agreement ‘Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell.’:

-	Understanding 1: The UE assumes the NES cell will always transmits CD-SSB after sending SI change notification, the UE acquires SIB1 with the Coreset#0 and Searchspace#0 parameters in MIB as legacy;

-	Understanding 2: It is up to the NES cell to transmits CD-SSB or NCD-SSB after sending SI change notification. If the NES cell transmits CD-SSB, the UE acquires SIB1 with the Coreset#0 and Searchspace#0 parameters in MIB as legacy; and if the NES cell transmits NCD-SSB,

•	2a: the UE assumes the Coreset#0 and Searchspace#0 parameters in the stored OD-SIB1 configuration are still valid, and acquires the updated SIB1 with the stored Coreset#0 and Searchspace#0 parameters from the start of the of the next modification period;

•	2b: the UE assumes the Coreset#0 and Searchspace#0 parameters in the stored OD-SIB1 configuration are no longer valid, and performs cell reselection to other cells;

•	2c: the UE assumes the stored OD-SIB1 request configuration is still valid, and performs OD-SIB1 request as the current procedure in the running CR;

R2-2505169	Consideration on on-demand SIB1	CATT	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 2: Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from network directly without sending OD-SIB1 request in next MP, i.e., the network will switch to normal mode with legacy K_SSB value and transmit the updated SIB1 in next MP.

Proposal 3a: In emergency cases, it is up to network implementation to broadcast the SIB1 periodically based on the valid OD-SIB1 request configuration stored by UE, no spec impact will be introduced.

Proposal 3b: Upon receiving the etwsAndCmasIndication in a Short Message on the NES cell, the UE shall immediately re-acquire SIB1 based on the CORESET information contained in the OD-SIB1 request information, without sending an OD-SIB1 request. i.e., the UE will follow the legacy procedure.

[Samsung]: Agree with proposal 2, and think we need some change accordingly. [Ericsson]: Do not agree with proposal 2. [Session chair]: Based on the discussion, let’s list possible options.

- Option1: Upon sending SI change notification, NW configure Kssb < 24 or <12 and the UE (including legacy UEs) receives the updated SIB (legacy way)

- Option2a: Upon sending SI change notification, NW may configure/keep Kssb >24 or > 12, and the NES UE performs OD-SIB1 request procedure to acquire the updated SIB

- Option2b: Upon sending SI change notification, NW may configure/keep Kssb>24 or >12, but the NES UE does not perform OD-SIB1 request procedure, and with the additional UE behaviour change, the NES UE receives the updated SIB (legacy UE cannot receive it)

[Session chair]: Seems many companies prefer NW may configure any Kssb upon SI change notification, but NES UE still acquires the updated SIB1 without performing OD-SIB1 REQ procedure (option1 or option2b). Understand option2a is not aligned with RAN2 previous agreement.

NW may configure any Kssb upon SI change notification (including ETWS/CMAS), but NES UE still acquires the updated SIB1 without performing OD-SIB1 REQ procedure.

RRC CR rapporteur will provide the TP in R2-2506218 until Wednesday session.

R2-2506218	TP on SIB1 update upon reception of short message	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core

TP in R2-2506218 is baseline.

[Vivo]: In the concerned case, coreSetZero comes from OD-SIB1 REQ configuration, which is new. Think it may impact on RAN1 spec. We may need to send LS to RAN1. [Samsung, Apple]: This behaviour is specified in RAN2 spec. [Samsung]: Kssb is legacy value, and two cases (Kssb > 24/12, or not) are already specified in RAN1 spec. Do not see a need to send an LS to RAN1.

4. Co-existence with other legacy features

R2-2505110	Discussion on on-demand SIB1	Xiaomi	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 2: RAN2 to discuss following options for co-existence of on-demand SIB1 and SDT:

•	Option 0: Do nothing. Reselection to an OD-SIB1 cell is not supported during SDT procedure.

•	Option 1: UE stops SDT before performing OD-SIB1 procedure.

•	Option 2: UE performs OD-SIB1 when SDT is ongoing.

[Apple, Samsung, Huawei, ZTE, Qualcomm]: Agree with option 0. [Samsung]: Understand with option1, there is no spec impact.

For co-existence of on-demand SIB1 and SDT, do nothing.

R2-2505766	Discussion on on-demand SIB1 for NES	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 5: RAN2 to agree on addition of a new value for RA purpose to accommodate for the case where RA procedure was used for request for SIB1.

Noted.

[IDC]: Understand, based on the RACH configuration/partitioning, the network can know this is used for OD-SIB1 REQ. [Samsung]: Understand in general, feature specific MDT/SON will be discussed in the next release. We don’t need to discuss it now. [ZTE, Huawei, Xiaomi, NEC]: Agree with Samsung. [Xiaomi]: It can be discussed as TEI19/20.

R2-2505157	Discussion on on-demand SIB1	Transsion Holdings	discussion	Rel-19

Proposal 3: RAN2 is kindly asked to confirm whether the OD-SIB1 cell can be target cell of HO procedure.

Proposal 4: The SIB1 of OD-SIB1 cell includes OD-SIB1 indication (e.g., in CellAccessRelatedInfo)

[LG]: Support the proposal 3 only for OD-SIB capable UE. [Apple, Xiaomi, CATT, Huawei, Samsung]: Think OD-SIB1 cell can be target cell of HO, but do not think anything additional is required. [Ericsson]: Wonder if OD-SIB1 UE capability for RRC connected is required. [Session chair]: Do we need this capability for RRC connected UE? Seems companies do not think any OD-SIB1 UE capability for RRC connected is required.

OD-SIB1 cell can be target cell of HO but no spec change is required.

5. Others

R2-2505990	Remaining issues of on demand SIB1	ZTE Corporation, Sanechips	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 5: The OD-SIB1 capability is defined as optional with capability signaling to assist the dedicated reselection priority configuration at network side.

[NEC]: Support the proposal. [Huawei]: In RAN1 discussion, it was decided as optional without UE capability since it is applicable to RRC idle/inactive states. [Huawei, Nokia]: Ok with proposal 5. [Lenovo]: Understand the cells with OD-SIB1 REQ are more per cell level, not per frequency level. [Nokia]: It can be also frequency level.

The OD-SIB1 capability is defined as optional with capability signaling to assist the dedicated reselection priority configuration at network side.

[Ericsson]: Ok with the proposal. Wonder if we can also include this capability into paging container, so the network may skip sending paging to the cell that operates based on OD-SIB1 REQ and if the UE does not support this capability. [Session chair]: We didn’t bar the legacy UEs into NES cell. Any companies want to support putting this capability into paging container also? No company.

OD-SIB1 capability is not included into paging container.

R2-2505562	Discussion on remaining issues of on-demand SIB1 operation for NES	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1: Once a NES UE acquires Cell A’s SIB1 and knows that it provides SIBxx, it can acquire Cell A’s SIBxx to obtain WUS configurations of neighbouring cells, even if the UE type is barred in SIB1.

Noted.

[Vivo]: Understand the current specification does not stop that UE behaviour. We can leave it to UE implementation. [LG]: Consider over-optimization and also network can send it when it supports all types of the UE. [Xiaomi]: Do not think it is essential issue.

R2-2505110	Discussion on on-demand SIB1	Xiaomi	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2505157	Discussion on on-demand SIB1	Transsion Holdings	discussion	Rel-19

R2-2505169	Consideration on on-demand SIB1	CATT	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2505253	On-demand SIB1	Samsung	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2505339	Remaining issues of On-demand SIB1	vivo	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2505499	Remaining issues on on-demand SIB1	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2505507	Remaining issues on on-demand SIB1 procedure	Fujitsu	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2505509	Remaining open issues on OD-SIB1 request	Sharp	discussion	Rel-19

R2-2505527	Discussion on on-demand SIB1	NTT DOCOMO, INC.	discussion	Rel-19

R2-2505562	Discussion on remaining issues of on-demand SIB1 operation for NES	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2505644	Discussion on the remaining open issues for OD-SIB1	ITRI, Acer Incorporated	discussion	Netw_Energy_NR_enh-Core

R2-2505656	On-demand SIB1 request on SUL	Sony	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2505749	Consideration on on-demand SIB1	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2505766	Discussion on on-demand SIB1 for NES	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2505846	Discussion on remaining issues of on-demand SIB1	Qualcomm Incorporated	discussion

R2-2505944	Discussion on open issues of OD-SIB1	CMCC	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2505978	Remaining issues on OD-SIB1 operation	LG Electronics Inc.	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2505990	Remaining issues of on demand SIB1	ZTE Corporation, Sanechips	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2506008	Remaining essential open issues including whether to support SUL in OD-SIB1 cell.	NEC	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2506063	Discussion on on-demand SIB1	HONOR	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2506090	On demand SIB1 handling	Nokia, Nokia Shanghai Bell	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2506170	Discussion on Additional Aspects of OD-SIB1	CEWiT	discussion

### 8.5.4	Adaptation of common signal/channel transmissions

Remaining essential open issues including down-selection of solutions on how to disable RACH adaptation for MSG1 repetition, stage-3 identified open issues if needed, etc.

1. Detailed format of firstPDCCH-MonitoringOccasionOfPO-r19

R2-2505500	Remaining issues on common signal transmission adaptation	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1 (RRC open issue Q13/Samsung003/ LGE003/Apple003): As legacy, keep IE firstPDCCH-MonitoringOccasionOfPO-r19 with symbol level offset and without NW restriction on ssb-periodicityServingCell. Adopt the following ASN.1 format to support Ns=8 and N=T/32:

Proposal 2 (RRC open issue Q14/Samsung003/ LGE003/Apple003): As legacy, keep IE firstPDCCH-MonitoringOccasionOfPEI-O-r19 with symbol level offset. Adopt the following ASN.1 format to support up to 8 PEI-O per PF and 32 radio frames:

R2-2506095	Adaptation of common signal/channel transmissions for NES	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 6: For Rel-19 NES UEs, firstPDCCH-MonitoringOccasionOfPO-r19 is configured at slot level.

R2-2505340	Remaining issues of common signal adaptation	vivo	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1: For signaling overhead optimization of Rel-19 firstPDCCH-MonitoringOccasionOfPO, restrict the maximum value range of the following parameters not to exceed symbol amount corresponding to 16rf (radio frames) for different SCS corresponding to the Rel-19 nAndPagingFrameOffset (i.e. N):

-	Rel-19 firstPDCCH-MonitoringOccasionOfPO;

-	Rel-19 firstPDCCH-MonitoringOccasionOfPEI-O;

R2-2506091	Adaptation of common signals	Nokia, Nokia Shanghai Bell	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 8: RAN2 to discuss how to efficiently transmit firstPDCCH-MonitoringOccasionOfPO for the 4 additional POs of a PF e.g. by reducing granularity or delta to previous PO signaling.

[Samsung]: firstPDCCH-MonitoringOccasionOfPO is not simple number of symbols or slots. It is logical number of paging occasions. [LG, Huawei, CATT, OPPO, Qualcomm, Samsung]: Support Apple’s proposal. [Vivo]: Ok, can accept Apple’s proposal. [Samsung]: From Apple’s proposal, let’s remove “offset” not to give any confusion.

As legacy, keep IE firstPDCCH-MonitoringOccasionOfPO-r19 with symbol level and without NW restriction on ssb-periodicityServingCell. Adopt the following ASN.1 format to support Ns=8 and N=T/32.

As legacy, keep IE firstPDCCH-MonitoringOccasionOfPEI-O-r19 with symbol level. Adopt the following ASN.1 format to support up to 8 PEI-O per PF and 32 radio frames:

2. Whether to support paging adaptation in non-initial BWP for redcap UE?

R2-2506051	Discussion on adaptation of common signals/channels transmissions	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1: firstPDCCHMonitoringOccasionOfPO-r19 is introduced both PCCH-Config and PDCCH-ConfigCommon.

Proposal 2: firstPDCCH-MonitoringOccasionOfPEI-O-r19 is only introduced in PDCCH-ConfigCommon, not in PCCH-Config.

R2-2505275	Discussion on adaptation of common signal channel transmission	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 4: R2 not work on paging adaptation co-existence with (e)RedCap at Q3, i.e., remove the IE pagingAdaptationFirstPDCCH-MonitoringOccasionOfPO-r19 implemented in PDCCH-ConfigCommon in the current RRC running CR for now, and discuss the cross-WI issues (if any) in the maintenance phase.

[Apple]: After checking, there is no RAN1/2 spec impact to support (e)RedCap. Also note the current running CR already supports (e)RedCap. [Vivo]: Agree with Apple. Understand if we exclude only (e)RedCap, it needs more specification efforts.

firstPDCCHMonitoringOccasionOfPO-r19 is introduced both PCCH-Config and PDCCH-ConfigCommon.

firstPDCCH-MonitoringOccasionOfPEI-O-r19 is only introduced in PDCCH-ConfigCommon, not in PCCH-Config.

3. Enhancement for the combination of paging adaptation and RACH adaptation

R2-2506097	Joint utilization of paging and RA resource adaptations 	Ericsson, Nokia, Nokia Shanghai Bell, InterDigital, NEC	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1	Extend the paging message with a per-UE indicator that signals the availability of activated adaptive RA resources for the corresponding UE.

Proposal 2	Adopt the TP below in the running 38.331 CR.

[Samsung, Huawei, Apple, LG, Lenovo, Qualcomm, OPPO, CATT, Vivo, Fujitsu]: Do not think a real need of it. [Xiaomi]: Support the proposal since understand it does not impact other WGs. [Apple]: Understand paging adaptation is used when the load is low, then do not agree with the scenario. Also understand there may be cross-WG issues. Paging adaptation is handled by CU and RACH adaptation is handled by DU, then we may need some information exchange between CU and DU, impact to RAN3. [LG]: Understand the gain from the proposal is marginal since the responding UE is random. [Lenovo]: Lenovo submitted a contribution in the past to show the paging log information in big cities. Paging was very few in each DRX cycle. Do not see the need of this optimization. [Ericsson]: Agree that it cannot be accurate, but think to certain extension, it will be helpful for distribution. For inter-WG issues, we may rely on network implementation. [Vivo]: Network doesn’t know where the idle/inactive UEs locate.

We don’t consider any optimization for the combination of paging adaptation and RACH adaptation.

4. Additional RO configuration with CE exclusion

R2-2505789	Remaining issues for time domain adaptation of common signalling and channels	InterDigital	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1: RAN2 to select between option 1b and 1c based on whether there is a need for the network to mix CE and non-CE RA in RACH-ConfigCommon. Consider TP in Annex if option 1b is agreed.

[Nokia]: Prefer option 1b. [Session chair]: All understand either option works and considering option1b can give some network flexibility, is option1b ok? Seems so.

Option 1b (a same featureCombinationPreamblesList-r17 applies to both legacy and additional RACH, and R2 keep the conclusion, allow mixing of CE feature and non-CE feature in the same RACH-ConfigCommon, and clarify in the MAC spec that if CE is applicable for random access procedure and RACH-ConfigCommon associated with selected random access resource set includes additional RO, UE does not use additional RO configured in this RACH-ConfigCommon.) is agreed.

TP in R2-2505789 is baseline for MAC update.

R2-2505099	Remaining issues on common signal and channel transmissions adaptation	LG Electronics Inc.	discussion	Rel-19

R2-2505170	Adaptation of Common signal channel transmissions	CATT	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2505255	Adaptation of common signal channel transmissions	Samsung	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2505275	Discussion on adaptation of common signal channel transmission	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2505316	Remaining open issues on common channel adaptation	Xiaomi	discussion	Netw_Energy_NR_enh-Core

R2-2505340	Remaining issues of common signal adaptation	vivo	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2505500	Remaining issues on common signal transmission adaptation	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2505508	Remaining issues on adaptation of common signal/channel	Fujitsu	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2505528	Discussion on adaptation of common signal and channel	NTT DOCOMO, INC.	discussion	Rel-19

R2-2505789	Remaining issues for time domain adaptation of common signalling and channels	InterDigital	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2505847	Discussion on remaining issues for RACH adaptation 	Qualcomm Incorporated	discussion

R2-2505991	Adaptation of common signal/channel transmissions	ZTE Corporation, Sanechips	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2506051	Discussion on adaptation of common signals/channels transmissions	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2506064	Discussion on adaptation of common signal/channel transmissions	HONOR	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2506091	Adaptation of common signals	Nokia, Nokia Shanghai Bell	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2506095	Adaptation of common signal/channel transmissions for NES	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2506097	Joint utilization of paging and RA resource adaptations 	Ericsson, Nokia, Nokia Shanghai Bell, InterDigital, NEC	discussion	Rel-19	Netw_Energy_NR_enh-Core

## 8.6	Mobility Enhancement Ph4

(NR_Mob_Ph4-Core; leading WG: RAN2; REL-19; WID: RP-242356)

Time budget: 2 TU

Tdoc Limitation: 3 tdocs

### 8.6.1	Organizational

Incoming LS, WI rapporteur inputs, CR rapporteur inputs (including post email discussion [POST130][110], [111], [120], summary of identified stage-3 open issues that need online discussion and rapporteur’s suggestions if needed).

WI is completed from RAN2 point of view.

R2-2505012	LS on frequency location of CSI-RS resources for CSI acquisition in LTM (R1-2504828; contact: Fujitsu)	RAN1	LS in	Rel-19	NR_Mob_Ph4-Core	To:RAN4	Cc:RAN2

Noted.

R2-2505065	Reply LS on security handling for inter-CU LTM in non-DC cases (S3-252398; contact: Huawei)	SA3	LS in	Rel-19	NR_Mob_Ph4-Core	To:RAN3	Cc:RAN2

Noted.

R2-2505453	Introduction of NR mobility enhancements Phase 4 in TS 38.300	Apple Inc	CR	Rel-19	38.300	18.6.0	1011	-	B	NR_Mob_Ph4-Core

=> Revised in R2-2506195

R2-2506195	Introduction of NR mobility enhancements Phase 4 in TS 38.300	Apple Inc	CR	Rel-19	38.300	18.6.0	1011	1	B	NR_Mob_Ph4-Core	R2-2505453

Revised in R2-2506415.

[Nokia, ZTE, Qualcomm]: Prefer removing “Beam characteristics could be different depending on whether they are configured for L3 or L1 mobility.” from NOTE 1. [Ericsson]: Ok to remove it, but in that case, the figure is only applicable to L3 measurement model. [Ericsson]: Understand, once the UE detects a cell, L1 beam(s) can be different (e.g. narrower/wider) than a L1 beam used for L3 measurement purpose. [Apple]: Want to have some offline to handle this issue.

Comeback:

R2-2506415	Introduction of NR mobility enhancements Phase 4 in TS 38.300	Apple Inc	CR	Rel-19	38.300	18.6.0	1011	2	B	NR_Mob_Ph4-Core	R2-2505453

Endorsed and considered as baseline for further discussion.

[POST131][113][MOB] (Apple)

Scope: Update MOB 38.300 CR (including this meeting agreements also).

Intended outcome: 38.300 CR in R2-2506225 to be agreed.

Deadline: Short email discussion

=> Agreed in R2-2506614

R2-2506614	Introduction of NR mobility enhancements Phase 4 in TS 38.300	Apple Inc (Rapporteur)	CR	Rel-19	38.300	18.6.0	1011	4	B	NR_Mob_Ph4-Core

=> Agreed

R2-2505291	Introduction of NR mobility enhancements Phase 4 in TS 37.340	China Telecom	CR	Rel-19	37.340	18.6.0	0419	-	B	NR_Mob_Ph4-Core

Endorsed and considered as baseline for further discussion.

[POST131][114][MOB] (China Telecom)

Scope: Update MOB 37.340 CR (including this meeting agreements also).

Intended outcome: 37.340 CR in R2-2506226 to be agreed.

Deadline: Short email discussion

=> Agreed in R2-2506616

R2-2506616	Introduction of NR mobility enhancements Phase 4 in TS 37.340	China Telecom, CATT	CR	Rel-19	37.340	18.6.0	0419	2	B	NR_Mob_Ph4-Core

=> Agreed

R2-2505164	Report of [POST130][120][MOB]	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1: Define a per UE capability for the support of all the LTM events.

Agreed.

Proposal 2: No need to define a separate UE capability for the support of the following functions in L1 event triggered measurement reporting,

-	The new MR MAC CE.

-	The Truncated MR MAC CE.

-	The event triggered periodic MR MAC CE reporting

-	The reportOnLeave for LTM

-	The current beam reporting in the MR MAC CE

Agreed.

Proposal 3: Discuss whether to define a per UE capability for reporting the beam not satisfying the event condition in the MR MAC CE.

No UE capability for reporting the beam not satisfying the event condition in the MR MAC CE.

[Nokia]: Do not understand what additional UE capability means to support it. [Ericsson]: Even when network sets this configuration, the actual reporting depends on the availability of measurement. [Samsung, CATT]: Ok to not having this capability.

Proposal 4: When a UE indicates support for both conditional LTM and ltm-RACH-LessCG-r18, and at least one of cltm-EarlyTA-Indication-r19 and ue-TA-Measurement-r18, it implies that the UE supports RACH-less conditional LTM with a configured grant. This revises the previous agreement.

Agreed.

Proposal 5: For the new UE capability “cltm-ExecutionConditionL3-r19” and ”cltm-ExecutionConditionL1-r19”, Except for NTN bands, UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands, all TDD-FR2-1 bands and all TDD-FR2-2 bands respectively. For NTN case, the decision is up to NR NTN WI.

[Ericsson]: Prefer to have per UE capability since anyway same value should be set for all bands. [ZTE]: Per UE capability is ok, but we need differentiated bits between FR1 and FR2, probably between TDD and FDD. Also, think we need to define whether this capability needs to be set based on the serving cell or target cell. [MediaTek]: Sounds no real difference then, why not stick to the original proposal. [Samsung]: Wonder if cltm-ExecutionConditionL1-r19 means measurement related capability or not. Understand it’s not about measurement related one, then we don’t need per band differentiation. [Qualcomm]: Do not see the need of differentiation of FR1/FR2 and TDD/FDD in this release phase. [Ericsson]: Agree with Samsung and still think per UE capability should be enough.

[AT131][107][MOB] (CATT)

Scope: Discuss and decide the following questions.

- Difference between per UE capability and per band capability with a restriction to set same value for all bands?

- “cltm-ExecutionConditionL3-r19” and ”cltm-ExecutionConditionL1-r19” per UE capability or per band capability with the restriction?

- Do we need differentiation for intra-F C-LTM and inter-F C-LTM? Note LTM capability has different ones for intra-F and inter-F cases.

Intended outcome: Discussion summary in R2-2506217.

Deadline: Offline meeting room will be announced by CATT. Comeback either in Wednesday session or CB session. => Completed.

Proposal 6: Discuss whether to define a separate UE capability for CLTM fast recovery.

Treated based on company contribution.

Proposal 7: Discuss whether to define a per UE capability for inter-CU LTM recovery.

A per UE capability for inter-CU LTM recovery.

R2-2506217	Report of [AT131][107][MOB](CATT)	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1a: FRx or xDD differentiation is not needed for the UE capability“cltm-ExecutionConditionL3-r19”and “cltm-ExecutionConditionL1-r19”.

Proposal 1b: “cltm-ExecutionConditionL3-r19”and “cltm-ExecutionConditionL1-r19” are defined as per UE capabilities without FRx or xDD differentiation. Whether this capability is actually supported on given band depends on whether the L1/L3 measurement is supported on the corresponding band.

Proposal 2: No need to differentiate between intra-F CLTM and inter-F CLTM.

Proposal 1a, 1b and 2 are agreed.

R2-2505162	Draft 306 running CR for UE capability for Mob Ph4	CATT	draftCR	Rel-19	38.306	18.6.0	NR_Mob_Ph4-Core

Endorsed and considered as baseline for further discussion.

R2-2505163	Draft 331 running CR for UE capability for Mob Ph4	CATT	draftCR	Rel-19	38.331	18.6.0	NR_Mob_Ph4-Core

Endorsed and considered as baseline for further discussion.

[POST131][115][MOB] (CATT)

Scope: Update MOB UE capability CRs (including this meeting agreements also).

Intended outcome: 38.331 CR in R2-2506227 and 38.306 CR in R2-2506228 to be endorsed.

Deadline: Short email discussion

=> Endorsed in:

- R2-2506227 (38.331)

- R2-2506228 (38.306)

R2-2506227	Draft 331 running CR for UE capability for Mob Ph4	CATT	draftCR	Rel-19	38.331	18.6.0	NR_Mob_Ph4-Core

Endorsed

R2-2506228	Draft 306 running CR for UE capability for Mob Ph4	CATT	draftCR	Rel-19	38.306	18.6.0	NR_Mob_Ph4-Core

Endorsed

R2-2505398	Discussion summary and list of MAC open issue for Mob Ph4	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

Open issue MAC-20 (essential): How to design the MAC CE to activate/deactivate the SP CSI-RS resource for LTM CSI acquisition for candiate cells based on RAN1 LS

Proposal 1a: (12/14) Common SP CSI-RS Resource Set Activation/Deactivation MAC CE is used for both RRM measurement and early CSI acquisition purpose, as captured in the current MAC running CR.

Agreed.

Proposal 1b: RAN2 to discuss whether to ask RAN1 for confirmation:

-	whether CSI-RS resource set configured for early CSI acquisition can contain CSI-RSs from multiple candidates. If so, the UE is only required to measure the RSs of target cell and ignore other RSs.

-	whether the CSI-RS resources related to one LTM-CSI-ResourceConfigId can be applied to both RRM and CSI acquisition.

[Ericsson]: Understand there is no need to ask RAN1. Network configures CSI-RS resource set configuration in correct way anyway. [ZTE]: Want to support a case that “CSI-RS resource set configured for early CSI acquisition can contain CSI-RSs from multiple candidates. If so, the UE is only required to measure the RSs of target cell and ignore other RSs.” [LG]: Why not allow the UE performing measurement on other candidates than target cell after the reception of LTM CSC if capable? We do not need any restriction. [Samsung]: Agree with LG. [Vivo]: Even though UE performs measurement on other candidates for early CSI acquisition, it is not useful since early CSI acquisition is for one-shot for the target.

If a CSI-RS resource set configured for early CSI acquisition contains CSI-RSs from multiple candidates, the UE is expected to measure the RSs of target cell after reception of LTM CSC.

Proposal 2: (14/14) SP CSI-RS Resource Set Activation/Deactivation MAC CE could be also used to activate and deactivate the configured semi-persistent CSI-IM resource sets of a LTM candidate cell, as captured in the current MAC running CR.

Agreed.

[Ericsson]: RAN1 decided that IM resource and NZP CSI resource cannot be configured (in a given CSI-RS resource set) together. [Vivo]: Good to check with own RAN1 delegate if it is true.

Comeback:

[Vivo]: After checking further, IM resource and NZP CSI resource cannot be configured for a CSI-RS resource configuration id. [Samsung]: Do not understand it. In legacy, it was already allowed. [Apple]: Understand Samsung’s concern, but it was RAN1 decision. [CATT]: Prefer to re-design the MAC CE to enable to include two CSI-RS resource configuration id. [Ericsson]: Do not see a need. Nothing is broken. Two MAC CEs can be sent.

IM resource and NZP CSI resource cannot be configured for a CSI-RS resource configuration id.

Comeback session:

[Vivo]: Based on further offline discussion, all companies are ok with change of MAC CE format to handle both a NZP CSI resource configuration and IM resource configuration.

SP-CSI MAC CE includes up to two CSI-RS resource configuration ids.

Detailed change is left to MAC CR rapporteur.

Open issue MAC-21: Whether indicate the selected RS index to the lower layer for RACH-less CLTM.

Proposal 3: (14/14) MAC indicates the selected RS index to the lower layer for RACH-less CLTM, as captured in the current MAC running CR.

Agreed.

Open issue MAC-22: whether multiple MR should be triggered and whether only one or multiple L1 measurement report MAC CE should be generated in case multiple beams associated with an LTM report configuration satisfies triggering condition for TTT.

Proposal 4: (14/14) RAN2 confirms when multiple MR(s) associated with an LTM report configuration is triggered, only one L1 measurement report MAC CE will be generated, as captured in the current MAC running CR.

Agreed.

Open issue MAC-23: whether support CG resource associated with CSI-RS.

Proposal 5: (14/14) RAN2 assumes CG resource associated with CSI-RS is not supported.

Agreed.

Other open issues:

Proposal 6: RAN2 to consider the above open issues related to MAC for mobility enhancement in section 2.2. Rapp: MAC-27 to MAC-31 are all marked as “Not Essential” as they should not impact the completion of this WI, and are proposed by single company.

Noted.

R2-2505397	Introduction of NR mobility enhancements Phase 4 in MAC	vivo (Rapporteur)	CR	Rel-19	38.321	18.6.0	2098	-	B	NR_Mob_Ph4-Core

Endorsed and considered as baseline for further discussion.

[POST131][116][MOB] (Vivo)

Scope: Update MOB 38.321 CR (including this meeting agreements also).

Intended outcome: 38.321 CR in R2-2506229 to be agreed.

Deadline: Short email discussion

=> Agreed in R2-2506229

R2-2506229	Introduction of NR mobility enhancements Phase 4 in MAC	vivo (Rapporteur)	CR	Rel-19	38.321	18.6.0	2098	1	B	NR_Mob_Ph4-Core

=> Agreed

R2-2505815	Introduction of RRC changes for mobility enhancements phase 4	Ericsson	CR	Rel-19	38.331	18.6.0	5443	-	B	NR_Mob_Ph4-Core

Endorsed and considered as baseline for further discussion.

[POST131][117][MOB] (Ericsson)

Scope: Update MOB 38.331 CR (including this meeting agreements also).

Intended outcome: 38.331 CR in R2-2506230 to be agreed.

Deadline: Short email discussion

=> Agreed in R2-2506567

R2-2506567	Introduction of RRC changes for mobility enhancements phase 4	Ericsson	CR	Rel-19	38.331	18.6.0	5443	2	B	NR_Mob_Ph4-Core

=> Agreed

R2-2505816	List of open issues for mobility ph4	Ericsson	discussion	Rel-19	NR_Mob_Ph4-Core

Noted.

[AT131][102][MOB] (Ericsson)

Scope: Discuss and conclude RRC-2, RRC-3, RRC-4, RRC-5, RRC-6, RRC-7, RRC-8 and RRC-10 (with the consideration of RRC CR rapporteur’s inputs). Also including P2a and P2b in R2-2505165 and P3 in R2-2505893.

Intended outcome: Discussion summary in R2-2506212.

Deadline: 1000 – 1100 (8/27 Wednesday, main session room) => Completed.

[AT131][103][MOB] (Vivo)

Scope: Discuss and conclude MAC-26, MAC-27, MAC-28, MAC-29, MAC-30, MAC-31, and P2 in R2-2505926.

Intended outcome: Discussion summary in R2-2506213.

Deadline: 1000 – 1100 (8/26 Tuesday, main session room) => Completed.

R2-2506213	Discussion report on [AT131][103][MOB] MAC open issues (vivo)	Vivo	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal: RAN2 to agree the below offline agreement:

Offline agreement for Event triggered L1 measurement:

- No impact on event triggered L1 MR procedure when beam failure is detected.

- RAN2 will keep the current definition of Type II in MAC, i.e. any beam.

Offline agreement for conditional intra-CU LTM:

- During CLTM is ongoing, after the first transmission, if TAT timer expires while RACH-less LTM is ongoing, UE always fall back to RACH-based LTM regardless whether UE based TA is valid or not.

- Stop the running ltm-Candidate-TimeAlignmentTimer for the corresponding candidate cell and release the stored TA value for the candidate cell if the corresponding CLTM candidate configuration is released.

- If RACH-less CLTM fallback to RACH-based CLTM due to PTAG expiration, RAN2 assumes it’s up to UE implementation to obtain the MAC PDU for 1st UL transmission. Captured as a Note in the MAC spec.

- For MAC-31, wait for RAN1. it is not essential issue, and does not impact the WI completion.

All proposals above are agreed.

[Lenovo]: If there is 2TA in a target, when fall-back RACH-based LTM is used, which TA is used? [Samsung]: It is not for Rel-19 LTM issue. When RACH-based LTM is used, 2TA is not considered.

R2-2506212	List of open issues for mobility phase4	Ericsson	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1: In case of MCG failure recovery, network reconfigure the UE in case it wishes the UE to restart the evaluation of the LTM cell switch execution condtion.

Proposal 2: UE understands whether to send an MR or trigger a conditional LTM cell switch procedure based on the presence of the reporting content configuration. The related ASN.1 field is changed from mandatory to optional.

Proposal 3: The UE shall not perform secondary key change when intra-MN LTM is performed (take TP in R2-2505583 as baseline).

Proposal 4: The reference configuration can be signalled to UE via the ltm-Config and ltm-ConfigNRDC (current restrictions in RRC are removed).

Proposal 5: RRC-9, RRC-10, RRC-11, RRC-12 is handled during the post email discussion.

Proposal 6: RRC-5, RRC-7, and RRC-8 are not pursued.

All proposals above (proposal 1,2,3,4,5, and 6) are agreed.

[Ericsson]: If companies that raised RRC-10/11/12 provides a draft TP to RRC CR rapporteur, it will be good.

### 8.6.2	Inter-CU LTM

Remaining essential open issues including handling of target cell’s SP CSI-RS resources after cell switch, stage-3 identified open issues if needed, etc.

1. Handling of target cell’s SP CSI-RS resource after cell switch

R2-2505277	Remaining issues of inter-CU LTM	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1: (MAC-25) SP CSI-RS resource for target cell is deactivated after cell switch.

R2-2505117	Discussion on L1 event-triggered measurement reporting	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1: (MAC-25) The SP CSI-RS resource of the target cell remains activated after cell switch if the SP CSI-RS was activated before cell switch.

SP CSI-RS resource for target cell is deactivated after CSI reporting in the first UL transmission. How to capture it in MAC is up to MAC CR rapporteur.

R2-2505717	Final View on Measurement Reporting Enhancements for Rel-19 LTM	Nokia	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 2: For L1-RSRP measurement, the UE deactivates SP CSI-RS resources of all candidate cells, including the target cell, after the LTM cell switch.

Proposal 3: For early CSI acquisition, the UE deactivates SP CSI-RS resources of all candidate cells except the target cell, after the LTM cell switch.

Noted.

2. Inter-CU LTM Recovery to the target cell

R2-2505277	Remaining issues of inter-CU LTM	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 2: For inter-CU LTM failure recovery, even if the selected cell is the target cell, there is no security issue, and no specification change is needed in RRC CR for the case.

R2-2505893	Inter-CU LTM	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 2: If inter-CU LTM cell switch triggered by the enhanced LTM CSC MAC CE fails, LTM recovery is configured, and the selected cell is the same as the target cell, the UE performs RRC re-establishment.

[Huawei]: Think RRC reconfiguration complete message may be different at recovery. [Qualcomm]: Do not think RRC reconfiguration complete message can be different. [IDC]: Can other RRC message be sent even before RRC reconfiguration complete at recovery? [Nokia]: Guess not. [ZTE, CATT]: Support Xiaomi proposal and understand similar situation happens at CHO failure and we allow recovery for the same serving cell. [IDC]: After the second thought, support Xiaomi proposal.

For inter-CU LTM failure recovery, even if the selected cell is the target cell, there is no security issue, and no specification change is needed in RRC CR for the case.

3. Inter-CU LTM Recovery after L3 HO/CHO failure

R2-2505399	Discussion on inter-CU LTM	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 6: After L3 HO or CHO failure, the UE cannot perform inter-CU LTM fast recovery.

[Ericsson]: Assume there is no spec impact since it’s same as Rel-18 LTM. [Vivo]: We may have no spec impact, but first we need a decision on the issue.

After L3 HO or CHO failure, the UE cannot perform inter-CU LTM fast recovery.

4. Clarification for ltm-config and ltm-configNRDC

R2-2506139	Discussion on inter-CU LTM	ZTE Corporation, Sanechips	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 2: RAN2 to confirm following principles for LTM:

	MCG LTM configuration is always provided within ltm-Config in an MN RRC message;

	Intra-SN or inter-SN SCG LTM with MCG configuration is provided within ltm-ConfigNRDC in an MN RRC message;

	Intra-SN SCG LTM without MCG configuration is provided within ltm-Config in an SN RRC message;

	For one UE, if inter-SN and intra-SN candidate cells are configured simultaneously, SCG LTM candidate configurations can only be provided within the ltm-ConfigNR-DC in MN RRC message(s).

Agreed.

Proposal 3: RAN2 to discuss where and how to capture the principles above, i.e. applicable scenarios of ltm-Config-r18 and ltm-ConfigNRDC-r19, in the current spec:

	Option 1: capture them in the RRC spec (see the TP to TS 38.331 above);

	Option 2: capture them in the stage-2 spec (see the TP to TS 37.340 above).

[Huawei]: Suggest not to use stage-3 IE names in stage-2 spec.

RRC CR will capture the principles.

5. UE capability for inter-CU LTM recovery

R2-2505158	UE capability for fast recovery for inter-CU LTM	MediaTek Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1: New UE capability is defined to indicate the UE support for Rel-19 fast recovery procedure.

Noted.

R2-2505158	UE capability for fast recovery for inter-CU LTM	MediaTek Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505165	Discussion on Inter-CU LTM	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505277	Remaining issues of inter-CU LTM	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505311	Remaining issues on inter-CU LTM	ETRI	discussion	Rel-19

R2-2505399	Discussion on inter-CU LTM	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505455	Remaining issues of Inter-CU LTM	Samsung	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505517	Discussion on open issues for inter-CU LTM	OPPO	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505546	Discussion on inter-CU LTM	Qualcomm Incorporated	discussion

R2-2505583	Remaining issues on inter-CU LTM	LG Electronics Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505657	LTM CG Resource consumption for the target cells	Sony	discussion	Rel-19	NR_Mob_Ph4

R2-2505730	Remaining issues of Inter-CU LTM	Rakuten Mobile, Inc	discussion	Rel-19

R2-2505786	Discussion on open issues of inter-CU LTM	Ofinno	discussion	Rel-19

R2-2505869	How to include the NCC value in the Enhanced Cell Switch Command MAC CE	Ericsson	discussion	NR_Mob_Ph4-Core

R2-2505870	On remaining open issues for Inter-CU LTM and DC-LTM	Nokia	discussion

R2-2505893	Inter-CU LTM	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2506022	Remaining issues and solutions on inter-CU LTM	Sharp	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2506113	Discussion on SP CSI-RS for target cell	NTT DOCOMO, INC.	discussion	Rel-19

R2-2506139	Discussion on inter-CU LTM	ZTE Corporation, Sanechips	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2506158	Discussion on inter-CU LTM	DENSO CORPORATION	discussion	Rel-19	NR_Mob_Ph4-Core

### 8.6.3	L1 event triggered measurement reporting

Remaining essential open issues includingpost email discussion [POST130][117] and conclusion on the coexistence with mTRP, stage-3 identified open issues if needed, etc.

1. MAC-8

R2-2505719	Report from [POST130][117][MOB] (Nokia)	Nokia	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1: When mTRP is configured in the serving cell the UE uses the best beam (in terms of RSRP) of the two “current beams” for LTM event evaluation.

[MediaTek]: Ok if it does not bring additional UE requirements to compare two beams. [Samsung]: Understand the UE needs to measure both beams, which may provide more complexities. [Nokia]: During email discussion, think no company really expects additional UE procedure/requirements to derive the best beam. [Samsung]: Is proposal 1 applicable to single DCI based mTRP or multi-DCI based mTRP? [Apple]: Prefer leaving it to UE implementation considering all additional complexities.

When mTRP is configured in the serving cell the UE uses the best beam of the two “current beams” for LTM event evaluation, and it’s up to UE implementation to how to derive the best beam.

Proposal 2: RAN2 to discuss if the NW needs to know the exact TRP and thus MR MAC CE shall indicate the TCI state associated with the reported RSRP value for the serving/current beam.

No additional indication in MR MAC CE is needed.

Proposal 3: The coexistence between event-triggered L1 measurement reporting and mTRP for the source cell is supported in Release 19 LTM.

[Samsung]: Understand proposal 3 is applicable to C-LTM also if P3 is agreed. [Nokia]: Do not see any problem to apply it to C-LTM also.

The coexistence between event-triggered L1 measurement reporting and mTRP for the source cell is supported in Release 19 LTM.

[AT131][104][MOB] (Nokia)

Scope: Prepare an agreeable TP on mTRP.

Intended outcome: TP in R2-2506214.

Deadline: Comeback in 8/27 Wednesday session => Completed.

R2-2506214	Text Proposal for multi-TRP in Rel-19 LTM		Nokia	discussion	Rel-19	NR_Mob_Ph4-Core

“best beam of the two current beams” is changed to “best beam of the current beams”

TP for stage-2 and MAC are baseline.

[MediaTek]: We may need to send a LS to RAN1 to inform RAN2 agreement. [ZTE, Apple]: Do not prefer sending a LS to RAN1. RAN2 agreement can be directly considered in RAN1 if need. [Apple]: Want to reword “best beam of the two current beams” to “best beam of the current beams”

R2-2505159	Remaining issues on event triggered L1 MR	MediaTek Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1: Option b) (up to the UE select the serving beams for L1 event evaluation) is adopted for event evaluation for LTM, if the coexistence with mTRP is agreed.

Noted.

2. MAC-24: Truncated MR MAC CE

R2-2506149	Remaining Issues of L1 Event Triggered Measurement Report	Samsung	discussion

Proposal 4: RAN2 to discuss below two options for reporting current beam measurements in LTM2

a.	UE reports the current beam measurements irrespective of reportCurrentBeam.

b.	Network always configures reportCurrentBeam in the LTM-EventTriggeredReportContent.

[ZTE]: Prefer option b. Simpler. [Nokia]: Prefer option a. [Session chair]: Where is current beam measured result located in MR for LTM2? [Vivo]: To the current running CR, it is located in the end as other LTM events. [LG]: Prefer option b with the consideration of where current beam measured result is located in MR for LTM2. [Apple, Ericsson]: Share the same understanding with Vivo. Understand either option works, but we need to consider which option brings less specification impact. In that point of view, support option b.

For LTM2, network always configures reportCurrentBeam in the LTM-EventTriggeredReportContent.

R2-2505518	Open issues for L1 event triggered measurement reporting	OPPO	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 5: Confirm the following working assumption: If network configures to include the current serving beam in MR, the UE always includes the current serving beam in the truncated MR MAC CE. In this case, minimum size of truncated MR MAC CE is {at least one triggered beam + the current serving beam}. If grant is not enough for that, the UE does not assemble a truncated MR MAC CE.

R2-2506140	Discussion on L1 event triggered measurement reporting	ZTE Corporation, Sanechips	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 3: (MAC-24) RAN2 to confirm the working assumption with some modification: If network configures to include the current serving beam in MR, the UE always includes the current serving beam in the truncated MR MAC CE. In this case, for Event LTM 3/4/5, minimum size of truncated MR MAC CE is {at least one triggered beam + the current serving beam}. While, for Event LTM2, minimum size of truncated MR MAC CE is the current serving beam. If grant is not enough for that, the UE does not assemble a truncated MR MAC CE.

Serving beam measured result is only included at the end of a (truncated) MR MAC CE for LTM2.

[Nokia, Ofinno, Lenovo]: Prefer, if the grant is received only for triggered beam even when the network configures current serving beam reporting, UE still is able to send truncated MR MAC CE including only triggered beam. [Session chair]: Check how many companies are ok with previous working assumption? Many.

Confirm the following working assumption: If network configures to include the current serving beam in MR, the UE always includes the current serving beam in the truncated MR MAC CE. In this case, minimum size of truncated MR MAC CE is {at least one triggered beam + the current serving beam}. If grant is not enough for that, the UE does not assemble a truncated MR MAC CE.

3. UE capability for reporting type 11 beam(s)

R2-2505159	Remaining issues on event triggered L1 MR	MediaTek Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 3: A per UE capability is introduced to report the beam not satisfying the event condition in the MR MAC CE.

Noted.

R2-2505117	Discussion on L1 event-triggered measurement reporting	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505135	Remaining issues of L1 event triggered measurement reporting	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505159	Remaining issues on event triggered L1 MR	MediaTek Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505166	L1 event triggered measurement reporting	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505180	Discussion on L1 event triggered measurement reporting	Transsion Holdings	discussion	Rel-19

R2-2505348	Discussions on L1 event triggered measurement reporting	Fujitsu	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505400	Discussion on LTM measurement event evaluation and reporting	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505456	Remaining Issues of L1 Event Triggered Measurement Report	Samsung	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505482	Remaining issues of LTM measurement	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505518	Open issues for L1 event triggered measurement reporting	OPPO	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505544	Discussion on L1 event-triggered measurement reporting	Qualcomm Incorporated	discussion

R2-2505548	Discussion on L1 event triggered measurement reporting for LTM	KDDI Corporation	discussion	Rel-19

R2-2505717	Final View on Measurement Reporting Enhancements for Rel-19 LTM	Nokia	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505719	Report from [POST130][117][MOB] (Nokia)	Nokia	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505731	Remaining issues of L1 event triggered measurement reporting 	Rakuten Mobile, Inc	discussion	Rel-19

R2-2505764	Discussion on triggering of MR MAC CE for leaving beam 	LG Electronics Inc.	discussion	NR_Mob_Ph4-Core

R2-2505787	Remaining issues for L1 event triggered measurement report	Ofinno	discussion	Rel-19

R2-2505868	Handling of SP CSI-RS resources of target cell after LTM cell switch	Ericsson	discussion	NR_Mob_Ph4-Core

R2-2505926	Leftover for L1 measurement and report	Lenovo	discussion	NR_Mob_Ph4-Core

R2-2505966	Remaining issues of L1 event triggered measurement reporting	CMCC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505974	Remaining issues of event-triggered L1 measurement reporting for LTM 	Kyocera 	discussion	Rel-19

R2-2506023	Discussion on issues for supporting L1 event triggered measurement reporting	Sharp	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2506065	Discussion on measurement event evaluation and report	HONOR	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2506140	Discussion on L1 event triggered measurement reporting	ZTE Corporation, Sanechips	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2506149	Remaining Issues of L1 Event Triggered Measurement Report	Samsung	discussion

### 8.6.4	Conditional intra-CU LTM

Remaining essential open issues including stage-3 identified open issues if needed, etc.

1. How to handle if the number of stored TAs exceeds the UE capability?

R2-2505278	Remaining issues of conditional LTM	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 7: When the number of CLTM TAs that the UE stores is up to cltm-EarlyTA-Indication-r19, to adding a new CLTM TA to be maintained by the UE, the following options can be considered.

•	Option 1: (Network-based method) Before sending the LTM Candidate Timing Advance Command to add a new CLTM TA, the network can remove one CLTM TA by setting the TAC field to “FFF” in LTM Candidate Timing Advance Command MAC CE.

•	Option 2: (UE-based method) Upon receiving the LTM Candidate Timing Advance Command MAC CE for adding a new CLTM TA, the UE shall remove the oldest CLTM TA and stop associated CLTM TAT.

[Ericsson]: It may not be easy network to track TAs all the time. If provided TA is beyond UE capability, it’s ok to leave it to UE implementation to decide which one is released. [Lenovo, NEC, Huawei, MediaTek]: Agree with Ericsson. And simpler. [ZTE, Vivo]: Prefer option 1. Note in RAN3, it was not decided whether the source forwards all remaining TAs for candidates to the target once C-LTM is completed.

For C-LTM, if the UE receives more TAs beyond its capability, it’s up to UE implementation to decide which one is released.

2. RRC-1

R2-2505360	Issue in Fast LTM Recovery after Conditional LTM Failure	Fujitsu, NTT DOCOMO, InterDigital	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1: RAN2 to address the issue about the SN gap in fast LTM recovery after CLTM failure.

Proposal 2: RAN2 to choose the solution from the following three options.

1)	Place a bit in PDCP header of RRCReconfigurationComplete message which indicates whether the ongoing LTM is CLTM or fast LTM recovery.

2)	Attach a new MAC CE to RRCReconfigurationComplete message which indicates whether the ongoing LTM is CLTM or fast LTM recovery.

3)	Define a new RRC parameter which configures whether the UE performs fast LTM recovery after CLTM failure.

R2-2505167	Discussion on Conditional Intra-CU LTM	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 3: It’s up to NW implementation to avoid the latency caused by the PDCP SN gap due to CLTM fast recovery.

[OPPO, Samsung, Ericsson, ZTE, Lenovo, Apple, Nokia, LG]: Support CATT proposal, i.e. leave it to network implementation to handle this issue. [NTTDCM]: Not sure if network can handle it well. [NTTDCM]: Ok with relying on network implementation, but prefer reflecting it as note in stage 2.

It’s up to NW implementation to avoid the latency caused by the PDCP SN gap due to CLTM fast recovery.

It will be added as a note into stage-2 spec. How to capture it is left to stage-2 spec CR rapporteur.

3. MAC-14

R2-2505483	Remaining issues of conditional LTM	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1: If UE receives the LTM CS command triggered by NW, UE shall follow the R18 behaviour to initiate LTM CS in the target cell regardless of whether the UE has the available C-TA value.

R2-2505312	Remaining issues on conditional LTM	ETRI	discussion	Rel-19

Proposal 3: When the UE receives an (Enhanced) LTM cell switch MAC CE with value “FFF” for the TA value requesting a cell switch to a CLTM candidate cell, the UE performs a RACH-less LTM cell switch with the indicated cell if it has a valid TA (i.e., CLTM TAT timer for the target cell is still running or the UE is configured with UE-based TA measurement). Otherwise, it performs a RACH-based LTM cell switch procedure.

[Huawei]: Do not understand why not network just sends the corresponding TA? Why in that case, the network will set “FFF”? [ZTE]: The concerned scenario is not for initial LTM case, it is for LTM after C-LTM switch. [Samsung, MediaTek, Nokia]: After C-LTM switch happened, if the new target decides for LTM, since the target doesn’t know which TA is valid for how long, target just set FFF, but if the UE maintains a valid TA for the candidate, it can be simply reused. See some benefit. [Qualcomm, Huawei, CATT, OPPO, LG, Ericsson]: Support Apple proposal, it just follows Rel-18 principle. [Ericsson]: Understand RAN3 will do their job, i.e. source forwards remaining TAs to the target upon C-LTM completion. Then we don’t need further optimization. [ZTE]: Ok with Apple proposal, but want to send a LS to RAN3 to ask their job. [Apple]: Prefer not to send a LS to RAN3, instead, we can capture RAN2 agreement/understanding in the session minutes.

If UE receives the LTM CS command triggered by NW, UE shall follow the R18 behaviour to initiate LTM CS in the target cell regardless of whether the UE has the available C-TA value.

RAN2 understands RAN3 handles the remaining TA information exchange from the source to the target.

4. UE capability for inter-CU LTM recovery

R2-2505278	Remaining issues of conditional LTM	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 9: No need to define a separate UE capability for CLTM fast recovery (i.e., reuse the UE capability for Rel-18 intra-CU LTM fast recovery).

Agreed.

5. Co-existence with HO/CHO/DAPS

R2-2505945	Discussion on open issue of conditional LTM	CMCC	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 4: CLTM can coexist with network triggered L3 HO, CHO and network triggered PScell change, while the coexistence of CLTM and DAPS HO is not supported.

Agreed.

R2-2505894	Intra-CU conditional LTM	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 4: C-LTM supports all the MCG LTM DC scenarios that are supported in Rel-18. No impact to TS 38.331 or TS 38.321 is expected.

Noted.

[NEC, Apple, Samsung]: Think it is not aligned with WID scopes that RAN decided.

R2-2505098	Discussion on Conditional Intra CU LTM	Lekha Wireless Solutions	discussion	Rel-19

R2-2505104	Discussion on remaining open issues of conditional intra-CU LTM	Transsion Holdings	discussion

R2-2505160	Remaining issues in conditional LTM	MediaTek Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505167	Discussion on Conditional Intra-CU LTM	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505278	Remaining issues of conditional LTM	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505312	Remaining issues on conditional LTM	ETRI	discussion	Rel-19

R2-2505349	Discussion on MAC open issues for Conditional intra-CU LTM	Fujitsu	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505360	Issue in Fast LTM Recovery after Conditional LTM Failure	Fujitsu, NTT DOCOMO, InterDigital	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505401	Discussion on conditional LTM	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505483	Remaining issues of conditional LTM	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505519	Open issues for conditional LTM	OPPO	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505545	Discussion on conditional intra-CU LTM	Qualcomm Incorporated	discussion

R2-2505584	Remaining issues on CLTM	LG Electronics Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505620	Remaining Open issues for CLTM	NEC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505641	Discussion on early TA for conditional LTM	ITRI	discussion	NR_Mob_Ph4-Core

R2-2505696	Remaining issues for CLTM	Lenovo	discussion	Rel-19

R2-2505729	Remaining issues of Conditional LTM	Rakuten Mobile, Inc	discussion	Rel-19

R2-2505733	Introduction of Hybrid CHO/LTM Handover Mechanism	Jio Platforms	discussion	Rel-19

R2-2505788	Discussion on remaining issues of CLTM	Ofinno	discussion	Rel-19

R2-2505867	L2 Reset Coordination for Conditional LTM	Ericsson	discussion	NR_Mob_Ph4-Core

R2-2505894	Intra-CU conditional LTM	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505945	Discussion on open issue of conditional LTM	CMCC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2506010	Remaining issues of Conditional intra-CU LTM 	Kyocera 	discussion

R2-2506024	Discussion on issues for supporting conditional LTM	Sharp	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2506032	Discussion on remaining issue for C-LTM	ASUSTeK	discussion	Rel-19	38.321	NR_Mob_Ph4-Core

R2-2506066	Discussion on conditional LTM	HONOR	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2506132	Considerations on conditional LTM	Nokia	discussion	Rel-19	NR_Mob_Ph4

R2-2506141	Discussion on conditional intra-CU LTM	ZTE Corporation, Sanechips	discussion	Rel-19	NR_Mob_Ph4-Core

## 8.7	XR Enhancements Ph3

(NR_XR_Ph3-Core; leading WG: RAN2; REL-19; WID: RP-250107)

Time budget: 2 TU

Tdoc Limitation: 3 tdocs

### 8.7.1	Organizational

LS, rapporteur input, workplan, running CRs, open issues lists etc.

Incoming LS

R2-2505039	LS on uplink rate control (R3-253927; contact: Nokia)	RAN3	LS in	Rel-19	NR_XR_Ph3-Core	To:RAN2

Noted

R2-2505047	LS on UE assistance information (R4-2508312; contact: Nokia)	RAN4	LS in	Rel-19	NR_XR_Ph3-Core	To:RAN2	Cc:RAN1

Noted

R2-2505061	Reply LS on RTP retransmission (S2-2505975; contact: InterDigital)	SA2	LS in	Rel-19	5G_RTP_Ph2, XRM_Ph2	To:SA4	Cc:RAN2

Noted

Running CRs

R2-2505069	Introduction of XR enhancements	Qualcomm Incorporated	CR	Rel-19	38.321	18.6.0	2102	-	B	NR_XR_Ph3-Core

R2-2505119	Introduction of R19 XR enhancements for RRC spec	Huawei, HiSilicon	CR	Rel-19	38.331	18.6.0	5395	-	B	NR_XR_Ph3-Core

R2-2505136	Draft 38.306 CR for Rel-19 XR UE capabilities	Xiaomi	draftCR	Rel-19	38.306	18.6.0	B	NR_XR_Ph3-Core

R2-2505137	Draft 38.331 CR for Rel-19 XR UE capabilities	Xiaomi	draftCR	Rel-19	38.331	18.6.0	B	NR_XR_Ph3-Core

R2-2505279	Introduction of XR Enhancements Phase 3	Nokia	CR	Rel-19	38.300	18.6.0	1007	-	B	NR_XR_Ph3-Core

R2-2505402	Introduction of R19 XR enhancements for RLC spec.	vivo	CR	Rel-19	38.322	18.2.0	0065	-	B	NR_XR_Ph3-Core

R2-2505438	Introduction of R19 XR enhancements for PDCP spec.	LG Electronics Inc.	CR	Rel-19	38.323	18.5.0	0149	-	B	NR_XR_Ph3-Core

All above are endorsed

[POST131][504][XR] Final 38.300 CR (Nokia)

Scope: Produce a final CR for R19 XR and merge with RAN3 CR

Intended outcome: CR for agreement in R2-2506335

Deadline:  Tuesday

=> Agreed in R2-2506335 (38.300)

R2-2506335	Introduction of XR Enhancements Phase 3	Nokia	CR	Rel-19	38.300	18.6.0	1007	1	B	NR_XR_Ph3-Core

=> Agreed

[POST131][505][XR] Final 38.331 CR (Huawei)

Scope: Produce a final CR for R19 XR

Intended outcome: CR for agreement in R2-2506336

Deadline:  Short

=> Agreed in R2-2506585 (38.331)

R2-2506585	Introduction of R19 XR enhancements for RRC spec	Huawei, HiSilicon	CR	Rel-19	38.331	18.6.0	5395	2	B	NR_XR_Ph3-Core

=> Agreed

[POST131][506][XR] Final 38.323 CR (LGE)

Scope: Produce a final CR for R19 XR

Intended outcome: CR for agreement in R2-2506337

Deadline:  Short

=> Agreed in R2-2506337 (38.323)

R2-2506337	Introduction of R19 XR enhancements for PDCP spec.	LG Electronics Inc.	CR	Rel-19	38.323	18.5.0	0149	2	B	NR_XR_Ph3-Core

=> Agreed

[POST131][507][XR] Final 38.322 CR (vivo)

Scope: Produce a final CR for R19 XR

Intended outcome: CR for agreement in R2-2506338

Deadline:  Short

=> Agreed in R2-2506338 (38.322)

R2-2506338	Introduction of R19 XR enhancements for RLC spec.	vivo	CR	Rel-19	38.322	18.2.0	0065	1	B	NR_XR_Ph3-Core

=> Agreed

[POST131][508][XR] Final 38.321 CR (Qualcomm)

Scope: Produce a final CR for R19 XR

Intended outcome: CR for agreement in R2-2506339

Deadline:  Short

=> Agreed in R2-2506349 (38.321)

R2-2506349	Introduction of XR enhancements Phase 3	Qualcomm Incorporated	Rel-19	38.321	18.6.0	2102	2	B	NR_XR_Ph3-Core

=> Coversheet revised by MCC in R2-2506619 (Wrong revision number)

R2-2506619	Introduction of XR enhancements Phase 3	Qualcomm Incorporated	Rel-19	38.321	18.6.0	2102	3	B	NR_XR_Ph3-Core

=> Agreed

[POST131][509][XR] Final UE capability CRs (Xiaomi)

Scope: Produce final draft CRs for R19 XR UE capabilities for merging into UE capabilities mega CR

Intended outcome: Draft CRs for endorsement in R2-2506340 (38.331) and R2-2506341 (38.306)

Deadline:  Sept. 4th

=> Endorsed in:

- R2-2506341 (38.306)

- R2-2505137 (38.331)

R2-2506341	Draft 38.306 CR for Rel-19 XR UE capabilities	Xiaomi	draftCR	Rel-19	38.306	18.6.0	B	NR_XR_Ph3-Core

=> Endorsed

R2-2505137	Draft 38.331 CR for Rel-19 XR UE capabilities	Xiaomi	draftCR	Rel-19	38.331	18.6.0	B	NR_XR_Ph3-Core

=> Endorsed

R2-2506460	Introduction of R19 XR enhancements for PDCP spec.	LG Electronics Inc.	CR	Rel-19	38.323	18.5.0	0149	1	B	NR_XR_Ph3-Core

This version can be used as a baseline for post-meeting e-mail discussion

Open issue lists

R2-2505070	List of open issues in MAC	Qualcomm Incorporated	discussion	Rel-19	NR_XR_Ph3-Core

R2-2505138	Open issues of Rel-19 XR UE capabilities	Xiaomi	discussion	Rel-19	NR_XR_Ph3-Core

R2-2505403	Summary of RLC open issue list for R19 XR	vivo	discussion	Rel-19	NR_XR_Ph3-Core

R2-2505439	Summary of [POST130][507][XR] PDCP running CR and open issues (LGE)	LG Electronics Inc. (Rapporteur)	discussion	Rel-19	NR_XR_Ph3-Core

All above are noted

Open issues discussed based on company Tdocs

R2-2505120	Summary of [POST130][506][XR] RRC running CR (Huawei)	Huawei, HiSilicon	discussion	Rel-19	NR_XR_Ph3-Core

The following proposals should be potentially easy to agree

Proposal2: Prohibit timer configuration for UAI for gap cancellation ratio preference is released at the initiation of RRC re-establishment or RRC resume procedure or at the cell selection during RRC re-establishment. (14 out of 14)

Propsoal3: The prohibit timer for the preference for gap occasion cancellation ratio is (14 out of 14)

	started when UAI carrying the field gapOccasionCancelRatio is transmitted

	stopped when releasing the GapOccasionPreferenceReportConfig when

	connection reestablishment/resume procedure is initiated or cell reselection happens during reestablishment

	GapOccasionPreferenceReportConfig is set to release

Proposal4: the following candidate values { s0, s0dot5, s1, s2, s5, s10, s20, s30,s60, s90, s120, s300, s600, spare3, spare2, spare1} can be reused for the prohibit timer for preference of gap occasion cancellation ratio. (14 out of 14)

Proposal5: For UL data rate query, the value of prohibit timer is the same for all flows. (10 out of 14)

Proposal6a: The candidate values for the UL available data rate query prohibit timer can be { s0, s0dot4, s1dot6, s0dot8, s3, s6, s12, s30} (14 out of 14)

The following proposal need further discussion:

Proposal1: For UAI for reporting preference for gap cancellation ratio, when multiple gap configurations are provided, a single timer is maintained for all the gap configurations. (9 out of 14)

Proposal6b: RAN2 to further discuss whether the two values { s0dot1, s0dot2}should be added to the candidate values for ul data rate query prohibit timer.

Proposal7: mg-CancellationDCI-0-3/1-3 is configured per BWP. (6 out of 14)

Open issue list

The following potential issues could be addressed from RRC point of view

RRC-1: FFS whether UAI for gap cancellation ratio preference can be reported to SN

Discussion on P1:

- QCM think per MG configuration timer makes more sense as these usually are for different frequencies which have different mobility characteristics.

- Xiaomi share the view with Qualcomm. There will be a few MG configs anyway, so finer granularity should be OK.

- Nokia thinks that multiple timers may cause UE to send UAI more often. Network can adjust the timer accordingly.

DISCUSSION on P6b:

- QCM sees value in having additional values.

DISCUSSION on P7:

- Huawei thinks that if we configure this per BWP, then it may be complex UE behavior. Should be per serving cell.

- QCM thinks if the issue for multi-carrier scheduling, then they do not see the difference between per BWP and per serving cell.

- Ofinno thinks the main difference is where we configure this. Wonders if we would have to align other formats.

- Huawei would not like to re-discuss other formats, they can stay per BWP. It will be simpler form configuration point of view if it is per serving cell.

- OPPO thinks that in case of per BWP configuration we need a new DCI format.

- QCM thinks this should be discussed in RAN1.

- Nokia thinks this is mainly signalling optimization, so we can stick to per BWP.

- Nokia asks why this is under PDSCH and not PDCCH configuration? Huawei clarifies that this follows legacy configuration.

- Samsung also thinks we should ask RAN1 to decide.

DISCUSSION on RRC-1:

- Ericsson thinks DC is not in the scope, so why do we need this proposal?

- Huawei thinks we can add a clarification in RRC that UE should not send this to SN.

- Xiaomi thinks R4 LS referred only to EN-DC and NE-DC, but NR-DC might be in the scope.

- Nokia does not think any special handling is needed.

- Lenovo wonders whether SN can send skipping DCI?

- Ericsson does not think we need anything as this will only be configured by MN.

Agreements on measurement gap skipping and RRC open issues - Prohibit timer configuration for UAI for gap cancellation ratio preference is released at the initiation of RRC re-establishment or RRC resume procedure or at the cell selection during RRC re-establishment. (14 out of 14) - The prohibit timer for the preference for gap occasion cancellation ratio is (14 out of 14) - started when UAI carrying the field gapOccasionCancelRatio is transmitted - stopped when releasing the GapOccasionPreferenceReportConfig when - connection reestablishment/resume procedure is initiated or cell reselection happens during reestablishment - GapOccasionPreferenceReportConfig is set to release - the following candidate values { s0, s0dot5, s1, s2, s5, s10, s20, s30,s60, s90, s120, s300, s600, spare3, spare2, spare1} can be reused for the prohibit timer for preference of gap occasion cancellation ratio. (14 out of 14) - For UL data rate query, the value of prohibit timer is the same for all flows. (10 out of 14) - The candidate values for the UL available data rate query prohibit timer can be { s0, s0dot4, s1dot6, s0dot8, s3, s6, s12, s30} (14 out of 14) - For UAI for reporting preference for gap cancellation ratio, when multiple gap configurations are provided, a single timer is maintained for all the gap configurations. (9 out of 14) - two values { s0dot1, s0dot2} should be added to the candidate values for ul data rate query prohibit timer. - mg-CancellationDCI-0-3/1-3 is configured per BWP

[AT131][501][XR] LS to RAN1 on measurement gap skipping (Huawei)

Scope: LS as per agreements

Intended outcome: Agreeable LS

Deadline:  Wednesday 2025-08-27, 11:00

- After offline Huawei clarifies that companies checked with their R1 colleagues and they confirmed RAN2 can make a decision. No LS is needed and a proposal is to make the configuration per BWP.

- For the NR-DC issue, Huawei thinks that RAN4 did not make explicit decision on NR-DC. Huawei proposes to try to capture potential impact on RRC during RRC post-meeting e-mail discussion and companies can check whether this is OK.

Rapporteur inputs / work plan

R2-2505329	Rapporteur Inputs	Nokia, Qualcomm (Rapporteurs)	discussion	Rel-19	NR_XR_Ph3-Core

WI completion

From RAN2 point of view the work item is complete

### 8.7.2	Multi-modality support

No contributions are expected for this AI

### 8.7.3	RRM measurement gaps/restrictions related enhancements

Remaining issues for this AI are expected to be covered by [POST130][506][XR] RRC running CR (Huawei) and no contributions are expected.

R2-2505658	UE Assistance Information (UAI) for recommended gap cancellation ratio	Sony	discussion	Rel-19	NR_XR_Ph3

R2-2505975	Discussion on UAI and Measurement Gaps	ETRI	discussion	Rel-19

### 8.7.4	Scheduling enhancements

Remaining open issues related to LCP and DSR enhancements.

MAC-1 (DSR cancellation with no delay-critical data)

R2-2505260	Scheduling Enhancements for XR	Ofinno	discussion	Rel-19

Proposal 1	[MAC-1] For the cancellation of a pending DSR, if at least one LCG is configured with dsr-ReportingThresList (i.e., Multiple Entry DSR MAC CE is enabled), a delay-reporting PDCP SDU is considered to be associated with the DSR (if it is associated with the LCH which triggered the DSR and it has not been transmitted in any MAC PDU).

R2-2505071	Discussion on DSR enhancements	Qualcomm Incorporated	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 1.	[MAC-1] Keep the current agreement that a pending DSR is canceled once the UE no longer has delay critical SDUs or all delay critical SDUs have been reported. No additional conditions are needed.

DISCUSSION:

- Huawei thinks we agreed not to change triggering and cancellation conditions, so we should not re-discuss.

- Ericsson also thinks this is NW decision how to set the thresholds.

- LGE agrees with Huawei. We should have consistent behavior for triggering and cancellation. If we change, we may need to discuss additional cases in future.

- Xiaomi thinks we can reuse current conditions.

- Nokia slightly prefers Ofinno way. Nokia thinks in a way condition is not change because we can change the definition of “associated data”.

- Lenovo agrees with LGE and Huawei.

[MAC-1] Keep the current agreement that a pending DSR is canceled once the UE no longer has delay critical SDUs or all delay critical SDUs have been reported. No additional conditions are needed.

MAC-2 (BSR cancellation)

R2-2505260	Scheduling Enhancements for XR	Ofinno	discussion	Rel-19

Proposal 2	[MAC-2] Cancellation of pending BSR is decoupled from DSR (i.e., no specification impact)

R2-2505678	Discussion on open issues for scheduling enhancements	Samsung	discussion	Rel-19

[MAC-2] Proposal 5: All the pending BSRs should be cancelled if all the data eligible for inclusion in a BSR MAC CE is reported in a DSR MAC CE.

R2-2505071	Discussion on DSR enhancements	Qualcomm Incorporated	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 2.  [MAC-2] It is up to UE implementation whether to cancel a BSR when all the buffer status up to the last event that triggered the BSR is reported by a DSR MAC CE.

DISCUSSION:

- Ericsson is concerned about the uncertainty of the network when sometimes you get only DSR and sometimes both. So this cannot be up to UE implementation. It is an optimization with small gain.

- QCM thinks the gNB can tell from padding bits, there is no confusion.

- Nokia thinks the situation is different than in Rel-18, because DSR has more information now. Supports having this.

- LGE understands Ericsson’s concern and also thinks there is no new reason to couple BSR and DSR.

- CATT supports Samsung’s proposal and agrees with Nokia’s point that the situation is different.

- Xiaomi agrees with LGE. If needed, it should be a new capability.

- CMCC also tend to decouple DSR and BSR to limit UE complexity.

- Huawei also thinks Rel-19 is different because we now have delay reporting data, so support Samsung proposal.

[MAC-2] Cancellation of pending BSR is decoupled from DSR (i.e., no specification impact)

MAC-6 (DSR cancellation in DC)

R2-2505290	Remaining issues on scheduling enhancement	Xiaomi Communications	discussion

Proposal 4   [MAC-6] RAN2 to confirm the previous agreement: “if MAC PDU is sent in one MAC entity, then the other MAC entity will see that there is no PDCP SDU associated with DSR and will cancel the DSR.” No spec impacts.

R2-2505756	Remaining issues on LCP and DSR enhancements	Huawei, HiSilicon	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 2:	[MAC-6] In Dual Connectivity case, for completeness in the spec and aligning with the understanding from the RAN2 agreement ‘MAC entity will see that there is no PDCP SDU associated with DSR and will cancel the DSR’, add in procedure text or as a note that ‘a MAC entity can cancel the pending DSR if the volume of delay-critical data associated with the DSR is zero even though no MAC PDU including PDCP SDUs associated with the DSR was transmitted’.

[MAC-6] RAN2 to confirm the previous agreement: “if MAC PDU is sent in one MAC entity, then the other MAC entity will see that there is no PDCP SDU associated with DSR and will cancel the DSR.” No spec impacts.

UE capability-1 (R18 and R19 DSR)

R2-2505290	Remaining issues on scheduling enhancement	Xiaomi Communications	discussion

Proposal 5  [UE capability-01] A UE supporting Rel-19 enhance DRS shall also indicate support of delayStatusReport-r18.

R2-2505274	Discussion on scheduling enhancement for XR	OPPO	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 2	[UE Cap-1] From UE capability signalling perspective, no need to have the pre-requisite for the capability of Rel-19 DSR.

DISCUSSION:

- Sharp agrees with OPPO, Rel-19 is covered by Rel-18.

- Ofinno agrees no need to couple.

- LGE thinks the UE supporting Rel-19 will support Rel-18 but we do not have to couple.

- QCM the dependency is only needed if there is dependency in the procedures, but this is not the case here.

[UE Cap-1] From UE capability signalling perspective, no need to have the pre-requisite for the capability of Rel-19 DSR.

RLC-13 (Delay-reporting data visibility in RLC)

R2-2505273	Removing Non-delay-reporting RLC SDU from RLC specification	Sharp, Ericsson	discussion

Proposal 1	(RLC-13) If dsr-ReportNonDelayCriticalData is configured, PDCP indicates non-delay reporting PDCP PDUs associated with the i:th dsr-ReportingThreshold as the delay-reporting RLC SDUs associated with the i-th dsr-ReportingThreshold.

Proposal 2	(RLC-13) Remove all the description related to non-delay reporting RLC SDU from RLC specification.

R2-2505372	Scheduling enhancements for XR	ZTE Corporation, Sanechips	discussion

Proposal 1a(RLC-13): The delay-reporting or non-delay-reporting information is visible to RLC.

Proposal 1b(RLC-13): For both DSR and LCP procedure, the interaction between various protocol layers can be left to UE implementation.

DISCUSSION:

- LGE thinks this is not only the matter of case, but if it is not based on the indication from PDCP, then there can be miscalculation of data volume for DSR. There is no non-delay reporting indication from PDCP currently which causes issues.

- Xiaomi thinks what LGE mentions is a different issue. Xiaomi thinks there are different cases, depending on whether the data has been submitted to lower layers by RLC. So some concept of delay critical data in RLC should be kept.

- Ericsson agrees with the intention to have an indication from PDCP to RLC.

- Nokia thinks what is missing the knowledge of non-delay reporting data. Nokia thinks PDCP can indicate RLC what to include in data volume.

- Apple agrees with Nokia.

[RLC-13] PDCP should indicate to RLC what needs to be included in the data volume calculation in RLC for each threshold

[RLC-13] We can remove definition of delay reporting and non-delay reporting data from RLC

PDCP-1 (Delay-reporting PDCP SDU text placement)

R2-2505372	Scheduling enhancements for XR	ZTE Corporation, Sanechips	discussion

Proposal 5 (PDCP-1): The text “and are not considered as delay-reporting PDCP data volume associated with any of the k:th dsr-ReportingThreshold where k < i” should be moved to the definition section for both delay-reporting PDCP SDU and non-delay-reporting PDCP SDU.

R2-2505458	Remaining issues on scheduling enhancement for XR	LG Electronics Inc.	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 6. [PDCP-1] Keep the current definition of delay-reporting PDCP SDU and non-delay-reporting PDCP SDU.

New identified issue

R2-2506331	[PDCP-1][RLC-13] Evaluation of non-delay-reporting PDCP SDU, Nokia, Nokia Shanghai Bell, LG Electronics

[AT131][503][XR] Cover remaining PDCP/RLC issues for DSR (Nokia)

Scope: Cover remaining PDCP/RLC issues, including PDCP-1, RLC-13 and new issue from R2-2506331

Intended outcome: Report with proposals

Deadline:  Report ready for Thursday CB session

R2-2506332	[AT131][503][XR] Cover remaining PDCP/RLC issues for DSR (Nokia)	Nokia, Nokia Shanghai Bell

[RLC-13]: RAN2 will start from option 2 from R2-2506332 and will check during the running CR review.

The text proposal in R2-2506331 is adopted.

[PDCP-1]: Keep the current definition of delay-reporting PDCP SDU and non-delay-reporting PDCP SDU.

Agreements on scheduling enhancements - [MAC-1] Keep the current agreement that a pending DSR is canceled once the UE no longer has delay critical SDUs or all delay critical SDUs have been reported. No additional conditions are needed. - [MAC-2] Cancellation of pending BSR is decoupled from DSR (i.e., no specification impact) - [MAC-6] RAN2 to confirm the previous agreement: “if MAC PDU is sent in one MAC entity, then the other MAC entity will see that there is no PDCP SDU associated with DSR and will cancel the DSR.” No spec impacts. - [UE Cap-1] From UE capability signalling perspective, no need to have the pre-requisite for the capability of Rel-19 DSR. - [RLC-13] PDCP should indicate to RLC what needs to be included in the data volume calculation in RLC for each threshold - [RLC-13] We can remove definition of delay reporting and non-delay reporting data from RLC - [RLC-13]: RAN2 will start from option 2 from R2-2506332 and will check during the running CR review. - The text proposal in R2-2506331 is adopted. - [PDCP-1]: Keep the current definition of delay-reporting PDCP SDU and non-delay-reporting PDCP SDU.

R2-2505171	Consideration on Scheduling Enhancement	CATT	discussion	Rel-19	NR_XR_Ph3-Core

R2-2505272	Leftover issues on scheduling enhancements	Sharp	discussion	Rel-19	NR_XR_Ph3-Core

R2-2505372	Scheduling enhancements for XR	ZTE Corporation, Sanechips	discussion

R2-2505404	Remaining issues on DSR enhancements for XR	vivo	discussion	Rel-19	NR_XR_Ph3-Core

R2-2505444	Remaining Issues of DSR Enhancements for Rel-19 XR	Apple	discussion	Rel-19	NR_XR_Ph3-Core

R2-2505547	Scheduling Enhancements for XR	Nokia, Nokia Shanghai Bell	discussion	Rel-19	NR_XR_Ph3-Core

R2-2505619	Remaining Issues on DSR enhancements	ETRI	discussion	Rel-19

R2-2505638	Remaining issues on LCP and DSR	NEC	discussion	Rel-19	NR_XR_Ph3-Core

R2-2505646	Remaining open issues of DSR enhancements	InterDigital	discussion	Rel-19	NR_XR_Ph3-Core

R2-2505970	Remaining open issues on scheduling enhancement for XR	CMCC	discussion	Rel-19	NR_XR_Ph3-Core

R2-2506067	Discussion on DSR enhancements	HONOR	discussion	Rel-19	NR_XR_Ph3-Core

R2-2506115	Discussion on XR DSR enhancements	III	discussion

R2-2506148	Discussion on DSR enhancements	Ericsson	discussion	Rel-19	NR_XR_Ph3-Core

### 8.7.5	RLC enhancements

Remaining open issues related to RLC enhancements.

RLC-10 (Config restrictions for stopReTxObsoleteSDU and DL t-RxDiscard)

R2-2505172	Remaining issues on XR-specific RLC Enhancement	CATT	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 2: (RLC-10) Add RRC configuration guidance for the stopReTxDiscardedSDU and t-RxDiscard, i.e., stopReTxDiscardedSDU and t-RxDiscard are configured together.

Proposal 3: (RLC-10) Besides RRC configuration guidance, capture in stage-2 that this is a “combined” approach for unnecessary retransmission avoidance.

R2-2506001	Discussion on RLC enhancements	DENSO CORPORATION	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 1:	(RLC-10) There is no need to introduce limitations or associations for the configuration of UL stopReTxObsoleteSDU and DL t-RxDiscard. This can be left to network implementation.

DISCUSSION:

- OPPO thinks QOS requirements for DL and UL can be different, it can be up to NW implementation.

- Ofinno has no strong view, but CATT’s P2 is not correct.

- Xiaomi agree with DENSO.

[RLC-10] There is no need to introduce limitations or associations for the configuration of UL stopReTxObsoleteSDU and DL t-RxDiscard. This can be left to network implementation.

RLC-11 (No SDU to transmit the poll with when tPollRetransmit expires)

R2-2506001	Discussion on RLC enhancements	DENSO CORPORATION	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 2:	(RLC-11) The window stalling issue with polling retransmission should be addressed, i.e., UE needs to retransmit a poll with a SDU upon the expiry of t-PollRetransmit, even if the SDU has been indicated as discarded.

Proposal 3:	(RLC-11) For the window stalling issue with polling retransmission, RAN2 can keep the current Rel-18 specification related to expiry of t-PollRetransmit unchanged.

R2-2505445	Remaining Issues of RLC-AM Enhancements for Rel-19 XR	Apple	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 1: (RLC-11) When all RLC SDUs with SNs up to POLL_SN are already positively/negatively acknowledged or discarded by PDCP, the transmitter should stop and reset the running t-PollRetransmit.

DISCUSSION:

- Ericsson, Samsung, QCM, Lenovo, Ofinno agrees with Apple.

- Ofinno does not think the window stalling issue can be avoided by the network.

- LGE thinks that even without changes it works and supports DENSO proposal. Apple thinks this is silly UE behavior, because the timer will run for no reason. Apple thinks we can say “UE may stop the timer” to allow for different UE behaviours.

- Huawei think Apple’s proposal is OK.

- Sharp thinks window stalling can happen and DENSO’s proposal is OK for this.

- vivo thinks current spec works, so prefer no change.

[RLC-11] When all RLC SDUs with SNs up to POLL_SN are already positively/negatively acknowledged or discarded by PDCP, the transmitter may stop and reset the running t-PollRetransmit. Capture this as a note.

RLC-11 (No SDU to transmit the poll with when remaining time based polling is triggered)

R2-2505705	Clarification on RLC AM	NEC, Fujitsu	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 3: [RLC-11] In case “there are only SDUs buffered whose transmissions have been stopped due to discard indication from PDCP, there is no SDU to retransmit the poll with”, RAN2 confirm no need to poll or retransmit the poll for any acknowledgement (no spec change).

R2-2506068	Discussion on RLC enhancements	HONOR	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 1: (RLC-11) If t-PollRetransmit is not running, upon notification of a transmission opportunity by lower layer, the RLC TX shall consider an SDU for retransmission when an indication of remaining-time-based RLC polling is received but there is no SDUs to transmit.

DISCUSSION:

- Lenovo thinks the situation is the same as for the previous discussion, so the above agreement is sufificent.

- LGE thinks HONOR’s proposal cannot happen as polling cannot be triggered if there is no such case. Ofinno agrees.

- Huawei thinks the scenarios are different.

- Vivo thinks this depends on the UE behavior, i.e. if the timer is running if the SDU is already submitted to lower layers.

- Nokia asks if an understanding now is that it will never get stuck, because all implementations will stop the discard timer?

- Ericsson thinks it can only get stuck in case data was discarded or sent. So proper implementation is to stop the timer.

[RLC-11] In case “there are only SDUs buffered whose transmissions have been stopped due to discard indication from PDCP, there is no SDU to retransmit the poll with”, RAN2 confirm no need to poll or retransmit the poll for any acknowledgement (no spec change).

RLC-12 (Impact of discard on PDCP SN gap report)

R2-2505344	Discussion on RLC re-transmission related enhancements	OPPO	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 3	(RLC-12) No special handling is needed in R19 for PDCP SN gap report during UE mobility.

R2-2505328	RLC Enhancements	Nokia, Nokia Shanghai Bell	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 7: RAN2 agree that unnecessary starting of t-Reordering at PDCP-mobility target RAN node because of PDCP-SN-gap information missing in the RAN is a problem to be solved in Rel.19.

Proposal 8: as part of PDCP entity re-establishment, for AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink, the transmitting PDCP entity shall re-transmit any previously (prior to the PDCP entity re-establishment) transmitted PDCP SN Gap report(s) for which the successful delivery has not been confirmed by lower layers.

R2-2505586	remaining open issues for RLC enhancements	Lenovo	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 4. (RLC-12) If RAN2 agrees to introduce a solution for Rel-19, the UE should trigger a PDCP SN gap report when it receives a PDCP status report in which an SDU indicated in a previous SN gap report is negatively acknowledged.

DISCUSSION:

- Samsung indicates a solution was agreed by RAN3 already for this issue.

- LGE does not think there is a problem as PDCP SR is not used for retransmission, only ACKed SDUs matter. We can only have some unnecessary retransmissions.

- QCM thinks Lenovo’s proposal is good enhancement for a general scenario.

- Ericsson thinks the proposals put additional requirements on the UEs.

- Nokia thinks RAN3 did not address the case when the loss happens before HO. Nokia agrees there is some new requirement on the UE, but this is normal case.

- Xiaomi agrees with OPPO we do not need a new solution. RAN3 solution addresses most cases and what is left is corner case. Sharp agrees. Sharp thinks also NW implementation can minimize the issue.

- Nokia thinks discarding will now happen more often, we can simplify to say that the UE just repeats the report which minimizes the UE impact.

- Huawei thinks no enhancement is needed.

(RLC-12) No special handling is needed in R19 for PDCP SN gap report during UE mobility. It can be left to UE implementation whether to re-send the gap report after HO.

RLC-14 (Whether to discard RLC SDUs if they have been submitted to lower layers already)

R2-2505344	Discussion on RLC re-transmission related enhancements	OPPO	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 4	(RLC-14) The transmitting side of the AM RLC entity can discard the RLC SDU or SDU segment(s) that have been submitted to lower layers if stopReTxDiscardedSDU is configured and discard indication is received from upper layer.

R2-2505804	Remaining open issues on RLC enhancements for XR	LG Electronics Inc.	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 5. (RLC-14) The RLC entity does not discard the RLC SDU or RLC SDU segment as in legacy, although the RLC SDU or RLC SDU segment indicated by the discard indication from upper layers has been submitted to the lower layers, i.e., no RLC specification change required.

DISCUSSION:

- Samsung agrees with LGE, no change is needed.

- Apple thinks this is UE implementation issue, no spec change needed.

- Lenovo does not want to change, discarding should be based on receiving an SR.

- NEC does not think it is up to UE implementation. If we change nothing, the UE cannot discard. NEC supports OPPO proposal.

- Ofinno thinks we can still rely on SR.

- Xiaomi supports OPPO’s view, it is important from buffer management point of view. There is no reason to keep these SDUs.

- Sharp indicates the buffer needs to fit the whole window size anyway, so discarding is not critical.

- LGE thinks this is an optimization and it will only save short time.

(RLC-14) The RLC entity does not discard the RLC SDU or RLC SDU segment as in legacy, although the RLC SDU or RLC SDU segment indicated by the discard indication from upper layers has been submitted to the lower layers, i.e., no RLC specification change required.

PDCP-2 (PDCP SN gap report modifications)

R2-2505804	Remaining open issues on RLC enhancements for XR	LG Electronics Inc.	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 6. (PDCP-2) PDCP SN gap report triggering condition is not updated considering Rel-19 RLC AM enhancement.

R2-2505373	RLC enhancements for XR	ZTE Corporation, Sanechips	discussion

Proposal 1(PDCP-2): If stopReTxDiscardedSDU is configured, PDCP SN gap report can be triggered even when the transmitting AM RLC entity has submitted the discarded RLC SDU or a segment thereof to the lower layers.

DISCUSSION:

- Sharp agrees with ZTE, this is a relevant issue.

- Ericsson agrees with LGE. Asks what will now happen if we have this SN gap report additionally. It is better not to change.

- ZTE thinks the question is what happens if lower layers discard the SDU. LGE explains that in this case we rely on reordering timer. LGE agrees there is an issue but it is very minor.

- Samsung thinks this can happen frequently with the latest enhancements, the change is very small.

- Lenovo ACKs the issue and SN gap report was to avoid the delay of reordering timer.

- Xiaomi is OK with ZTE’s proposal.

- Ericsson thinks we are unnecessarily mixing operations in different layers.

- Nokia liked the proposal, but thinks we need to be consistent, so we should not address this.

- Samsung does not think the scenario is the same, here we are not speaking of HO scenario, it is a general case.

(PDCP-2) PDCP SN gap report triggering condition is not updated considering Rel-19 RLC AM enhancement.

UE capability-4 (Tx and Rx side capabilities coupling)

R2-2505882	Remaining Open Issues on RLC Enhancements	Ericsson	discussion	Rel-19

Proposal 1	(UE capability-04) UE supporting txRLC-StopReTxDiscardedSDU-r19 need not indicate support for rxRLC-Discard-r19.

R2-2505172	Remaining issues on XR-specific RLC Enhancement	CATT	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 6: (UE capability-04) a UE supporting txRLC-StopReTxDiscardedSDU-r19 shall also indicate support of rxRLC-Discard-r19.

(UE capability-04) UE supporting txRLC-StopReTxDiscardedSDU-r19 need not indicate support for rxRLC-Discard-r19.

Other RLC CR issues

R2-2505271	Leftover issues on RLC enhancements	Sharp	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 8	The RLC CR clarifies that a discard indication shall cancel pending considerations for retransmission.

- Apple has some sympathy, but is also afraid this may require us to make much more changes. Either we do nothing or cover all cases. LGE has a similar view.

- Ofinno thinks current specs is clear enough.

- Sharp think currently is not clear and would be good to specify something somewhere.

Try to clarify in RLC specs that “RLC SDU segment(s) for transmission or retransmission” covers RLC SDU segment(s) which have been already considered or pending for transmission/retransmission.

R2-2505405	Discussion on RLC enhancement for XR	vivo	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 8: The pending remaining time based RLC retransmission should be cancelled when the RLC entity receives a positively ACK for the corresponding RLC SDU, RAN2 to discuss whether there are any specification impacts.

- Lenovo suggests to capture that UE will cancel the retransmission and we can try to have it covered together with the previous agreements.

- Ericsson agrees with the intention, but we do not have to distinguish retransmission types. No change is needed.

Can try to clarify together with the previous agreement.

R2-2505586	remaining open issues for RLC enhancements	Lenovo	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 3: RAN2 to discuss capturing an additional note in the spec to prevent the discrepancy in the information provided in a status report when the duration of the new timer and t-Reassembly are set to the same value.

- Ericsson thinks we can leave this up to UE implementation.

- LGE agrees with the intention, but smart implementation can do this anyway.

- Vivo thinks it is corner case and can be up to UE implementation. Can capture a note.

- Samsung thinks this is a corner case.

It is up to UE implementation to prevent the discrepancy in the information provided in a status report when the duration of the new timer and t-Reassembly are set to the same value. No spec change.

Agreements on RLC enhancements - [RLC-10] There is no need to introduce limitations or associations for the configuration of UL stopReTxObsoleteSDU and DL t-RxDiscard. This can be left to network implementation. - [RLC-11] When all RLC SDUs with SNs up to POLL_SN are already positively/negatively acknowledged or discarded by PDCP, the transmitter may stop and reset the running t-PollRetransmit. Capture this as a note. - [RLC-11] In case “there are only SDUs buffered whose transmissions have been stopped due to discard indication from PDCP, there is no SDU to retransmit the poll with”, RAN2 confirm no need to poll or retransmit the poll for any acknowledgement (no spec change). - (RLC-12) No special handling is needed in R19 for PDCP SN gap report during UE mobility. It can be left to UE implementation whether to re-send the gap report after HO. - (RLC-14) The RLC entity does not discard the RLC SDU or RLC SDU segment as in legacy, although the RLC SDU or RLC SDU segment indicated by the discard indication from upper layers has been submitted to the lower layers, i.e., no RLC specification change required. - (PDCP-2) PDCP SN gap report triggering condition is not updated considering Rel-19 RLC AM enhancement. - (UE capability-04) UE supporting txRLC-StopReTxDiscardedSDU-r19 need not indicate support for rxRLC-Discard-r19. - Try to clarify in RLC specs that “RLC SDU segment(s) for transmission or retransmission” covers RLC SDU segment(s) which have been already considered or pending for transmission/retransmission. - Can try to clarify together with the previous agreement. [That: “The pending remaining time based RLC retransmission should be cancelled when the RLC entity receives a positively ACK for the corresponding RLC SDU”] - It is up to UE implementation to prevent the discrepancy in the information provided in a status report when the duration of the new timer and t-Reassembly are set to the same value. No spec change.

Enhancements, if time allows

R2-2505072	Discussion on RLC enhancements	Qualcomm Incorporated	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 4.	The transmitter includes an extra indication in the RLC PDU header whether the included poll is an enhanced poll (i.e. triggered based on remaining time).

Proposal 5.	Upon receiving an enhanced poll, the receiver sends a status report immediately, ignoring t-StatusProhibit; or apply a shorter t-StatusProhibit for the report.

R2-2505659	Timely retransmissions for RLC AM	Sony, Canon	discussion	Rel-19	NR_XR_Ph3

Proposal 1: After Rx side receives polling information, the Rx side should bypass/ignore the t-Reassembly timer for any SDU or segments of SDU if the t-Reassembly timer is running when generating status report.

Proposal 2: If proposal 1 is agreed, for any SDU or segments of SDU with bypassed t-Reassembly timer or still stuck in the lower layer retransmissions should be reported as UNKNOWN status in the status report (SR) by indicating/ redefining the corresponding reserved bit as 1 (R = 1).

R2-2505328	RLC Enhancements	Nokia, Nokia Shanghai Bell	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 9: to minimize autonomous retransmissions, allow the network to send frequent ACKs, by introducing a possibility to configure the UE not to consider SDUs for retransmission based on received NACKs (to eradicate the problem of premature NACKs).

R2-2505139	RLC AM retransmission enhancements	Xiaomi	discussion	Rel-19	NR_XR_Ph3-Core

R2-2505261	RLC Enhancements for XR	Ofinno	discussion	Rel-19

R2-2505271	Leftover issues on RLC enhancements	Sharp	discussion	Rel-19	NR_XR_Ph3-Core

R2-2505505	Discussion on RLC AM enhancements	Huawei, HiSilicon	discussion	Rel-19	NR_XR_Ph3-Core

R2-2505643	Discussion on RLC Enhancements for Unnecessary Retransmissions Avoidance	ITRI	discussion	NR_XR_Ph3-Core

R2-2505647	Discussion on RLC enhancements	InterDigital	discussion	Rel-19	NR_XR_Ph3-Core

R2-2505677	Discussion on open issues for RLC enhancements	Samsung	discussion	Rel-19

R2-2505955	Discussion on the open issue of RLC enhancements	CMCC	discussion	Rel-19	NR_XR_Ph3-Core

R2-2506189	On remaining issue on RLC enhancements	NTT DOCOMO INC..	discussion	Rel-19

### 8.7.6	XR rate control

Remaining open issues related to XR rate control.

UE capability-5 (Maximum number of QoS flows that a UE supports for rate control)

R2-2505406	Discussion on XR rate control	vivo	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 1	(Capability-05) RAN2 to define the maximum number of allowed QoS flows for rate control as an AS UE capability, with the value up to 16.

R2-2505558	Discussion on UL rate control for Rel-19 XR	Samsung	discussion	Rel-19	NR_XR_Ph3-Core

[UE capability-05] Proposal 6: RAN2 to not consider UE AS capability on the maximum number of QoS flows subject to rate control.

DISCUSSION:

- QCM thinks a capability is needed as different types of UEs can have different capabilities, e.g. AR glasses. Ofinno agrees.

- Huawei sees two issues: maximum number of flows and whether capability is needed. Huawei thinks 8 flows is enough, so can be handled with single capability.

- Ofinno thinks less flows also means smaller MAC CE.

- Nokia agrees with Huawei, there is no need for many flows and we can agree a number per UE.

- Xiaomi also thinks we need a capability.

- Sharp thinks that the number of flows does not have much impact on UE processing, one single value is enough.

- ZTE also does not think the complexity issue, the UE can support a small number of flows, then the NW will only control a small number. We just need a single maximum value.

- QCM indicates that some application like cloud gaming require more than, e.g. 8 flows. We can further discuss the limitation for UE to send queries.

- ZTE still does not see a complexity issue. ZTE thinks we do not have to control a lot flows, 8 is more than enough.

- OPPO thinks that flow adaptation impacts reconfiguration of codec, so up to 4 without capability is OK.

- IDT asks why we need a maximum value if we don’t have a capability.

- Huawei also thinks we need a maximum value for RRC configuration. Currently the CR captures 8.

- Ericsson thinks that signalling should support up to 64. If we restrict, then it has impact on CN.

- Futurewei also thinks we do not need a high number. We only need to rate-adapt those flows which contribute to the traffic.

- Nokia thinks that we can just agree no capability is needed. One thing is RRC configuration and the other thing is limitation in MAC CE format. ZTE agrees.

- Chair: It seems whether/what is the maximum number flows may depend on MAC CE format, so we come back to this, if needed, after discussing MAC CE format.

No capability to indicate the number of rate-adaptable flows is introduced.

MAC-3, MAC-4 (QoS flow – explicit or implicit)

R2-2505648	Discussion on UL congestion signaling	InterDigital	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 1: Use the LCID + QFI (6 + 6 bits) combination to identify a specific QoS flow for UL rate control.

R2-2505446	Remaining Issues of UL Rate Control for Rel-19 XR	Apple	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 1: (MAC-3) For identifier of QoS flows in the UL Rate Control MAC CE, RAN2 should focus on implicit indication of QFI.

R2-2505350	Discussions on XR rate control	Fujitsu	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 1: (MAC-3) ID for QoS flows in UL Rate Control MAC CE is the index of the QoS flow.

DISCUSSION on explicit vs implicit QFI indication:

- Nokia thinks that explicit ID is only beneficial in case of single QFI indication, for more than this this implicit is more beneficial.

- QCM agrees with Apple and Nokia, this has less overhead.

- OPPO thinks implicit ccuases issues during reconfiguration/remapping.

- Xiaomi thinks this depends on the number of flows, implicit makes more sense as we do not need many flows to be controlled.

- Vivo thinks implicit is more signalling overhead friendly.

- ZTE agrees bitmap is better provided that we limit the maximum number of controllable flows.

- Lenovo assumes a limited number is OK, but still thinks explicit is simpler.

- LGE thinks explicit is simpler to interpret by the UE.

- Huawei indicates that we agreed multiple flows in one MAC CE, so prefers implicit to reduce overhead.

- Ofinno supports implicit way for the overhead reasons.

- CATT supports explicit manner as overhead for DL is an issue. Thinks that implicit requires more discussions.

- Sharp thinks both options work, but implicit is more efficient.

- NEC support explicit.

- Samsung thinks implicit is more efficient.

- Fujitsu thinks explicit is not needed as we can use RRC ID.

Show of hands on explicit vs implicit QFI indication:

    - Implicit: 15

    - Explicit: 8

We go with implicit way

MAC-3, MAC-4 (DRB ID – explicit or implicit)

R2-2505446	Remaining Issues of UL Rate Control for Rel-19 XR	Apple	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 2: (MAC-3) The QoS flows in the UL Rate Control MAC CE can be identified via a DRB ID along with a bitmap of QoS flows mapped to the indicated DRB.

R2-2505350	Discussions on XR rate control	Fujitsu	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 2: (MAC-3) The mapping between the index in the rate control MAC CE and the QFI and the PDU session ID can be pre-defined, e.g., based on the ascending order of PDU session ID and QFI.

R2-2506191	On remaining issue on XR UL rate control	NTT DOCOMO INC..	discussion	Rel-19

Proposal 1.	(MAC-04) We should adopt the implicit signaling, i.e., a list of combinations of a QFI and a DRB ID is configured by RRC reconfiguration in advance, and then a bitmap in the MAC CE which corresponds to the list preconfigured in RRC, indicates one or multiple QoS flows.

DISCUSSION on explicit vs implicit DRB indication:

- QCM agrees with Fujitsu. We can have an identifier configured.

- Ofinno does not like explicit indication. It can increase overhead if flows are mapped to different DRBs.

- Ericsson thinks that explicit DRB is not beneficial having implicit QFI. RRC identifier would solve the issue.

DISCUSSION on whether we have a mapping rule or RRC ID:

- Nokia thinks mapping rule is sufficient.

- ZTE thinks that what works better depends on the maximum number.

The mapping between the index in the rate control MAC CE and the QFI and the PDU session ID/DRB ID can be pre-defined/configured

Offline to discuss:

    - Whether we have a maximum number of flows rate-adaptable with MAC CE

    - Whether we introduce an identifier in RRC for mapping between PDU/DRB+Qos flow ID and an identifier used in MAC CE, or we use a mapping based on order of PDSU session and QF ID, including whether DRB ID or PDU session ID should be used for mapping

    - Any other issues for MAC CE format

[AT131][502][XR] MAC CE for XR rate (LGE)

Scope: Discuss the details of MAC CE format for XR rate control

Intended outcome: Report with agreeable proposals

Deadline: Report ready for Thursday CB session

R2-2506417	Report of [AT131][502][XR] MAC CE for XR rate (LGE)	LG Electronics Inc.

A maximum number of QoS flows rate-adaptable with MAC CE is 16.

A 16-bit bitmap is included in the MAC CE in order to identify a specific QoS flow for UL rate control.

Predefined mapping rule based on the order of PDU session ID + Qos flow ID is used for the bitmap. (i.e., it can be ordered based on the ascending order of PDU session ID and Qos flow ID)

The Rate Query MAC CE has the same format as the Rate Control MAC CE.

An understanding in RAN2 is that these agreements do not limit how many QoS flows can be configured/indicated as rate-adaptable by the CN.

MAC-3, MAC-4 (Detailed design)

R2-2505446	Remaining Issues of UL Rate Control for Rel-19 XR	Apple	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 3: (MAC-4) The MAC CE for UL rate control/query should include the following:

•	One DRB ID

•	One bitmap indication of concerned QoS flow IDs that are mapped to the indicated DRB

•	A list of recommended/preferred rates corresponding to the concerned QoS flow IDs

R2-2505350	Discussions on XR rate control	Fujitsu	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 3: (MAC-4) In the rate control MAC CE, a bitmap indicates the index (indices) of the QoS flows for rate control/query.

Proposal 3a: (MAC-4) The length of the bitmap is 8 bits.

Proposal 3b: (MAC-4) Each bit in the bitmap indicates whether the bit rate field for the corresponding QoS flow is present or not.

R2-2505648	Discussion on UL congestion signaling	InterDigital	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 2: For a DRB, only one LCID indication within the UL rate control MAC CE is required which can follow number of QFIs for QoS flows for which the rate control information is provided for the given DRB.

Proposal 3: [MAC-4] The new UL Rate Control MAC CE can provide UL rate control information for multiple QoS flows within a single DRB.

Proposal 4: [MAC-4] Use an extension bit to indicate a further pair of QFI+Bit Rate fields follows in the UL Rate Control MAC CE.

MAC-5 (Handling of triggered UL rate queries)

R2-2505578	Concluding XR rate control for Rel-19	Nokia, Nokia Shanghai Bell	discussion	NR_XR_Ph3-Core

Proposal 4 (MAC-5): The UL rate control MAC CE can be transmitted when available UL-SCH resources can accommodate the UL rate control MAC CE including all the pending queries plus its subheader.

R2-2505406	Discussion on XR rate control	vivo	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 4	(MAC-05) UL Rate Control MAC CE is transmitted if the available UL-SCH resources can accommodate at least one of the pending queries.

Proposal 5	(MAC-05) UE can decide the included pending rate control queries in UL Rate Control MAC CE, in two ways:

-	Way-1: totally up to UE implementation;

-	Way-2: UE can decide how many pending rate control queries to include based on the amount of UL-resurces, and then select the queries in the decreasing priority order of LCH, mapped from the corresponding QoS flows.

DISCUSSION:

- QCM prefers to leave this up to UE implementation. There is not always the need to adapt at the same time. UE can decide which queries to include. Query MAC CE has low priority so UE may have problem always transmitting all queries at once.

- Ofinno supports vivo proposal, but has another option for choosing the flows.

- Apple thinks we agreed that we should follow legacy, so UE can decide which flows to include.

- LGE thinks there is some impact on specs.

- Lenovo that form specifications point of view we can follow Nopkia’s proposal, but UE decides when to trigger a query.

- Sharp thinks it is not a big problem to delay transmission of the MAC CE.

- CMCC indicates that we have already implicit way, so Nokia’s proposal is preferred.

- LGE thinks XR is different than legacy MAC CE, because XR is high bitrate service.

- Ofinno has concerns on Nokia’s proposal as this MAC CE is very low priority.

- Huawei agrees with LGE and Lenovo. Huawei does not want truncated MAC CE, but we can leave up to UE what to include.

- QCM does not think rate query is up to UE implementation as this comes from higher layer.

No truncated rate query MAC CE is introduced.

UL Rate Control MAC CE can be transmitted if the available UL-SCH resources can accommodate at least one of the pending queries. If not all can be accommodated, it is up to UE implementation which ones to include. We can capture this as a NOTE in MAC, if possible.

Agreements on XR rate control - No capability to indicate the number of rate-adaptable flows is introduced. - We go with implicit way [for QoS flow indication] - The mapping between the index in the rate control MAC CE and the QFI and the PDU session ID/DRB ID can be pre-defined/configured - A maximum number of QoS flows rate-adaptable with MAC CE is 16. - A 16-bit bitmap is included in the MAC CE in order to identify a specific QoS flow for UL rate control. - Predefined mapping rule based on the order of PDU session ID + Qos flow ID is used for the bitmap. (i.e., it can be ordered based on the ascending order of PDU session ID and Qos flow ID) - The Rate Query MAC CE has the same format as the Rate Control MAC CE. - An understanding in RAN2 is that these agreements do not limit how many QoS flows can be configured/indicated as rate-adaptable by the CN. - No truncated rate query MAC CE is introduced. - UL Rate Control MAC CE can be transmitted if the available UL-SCH resources can accommodate at least one of the pending queries. If not all can be accommodated, it is up to UE implementation which ones to include. We can capture this as a NOTE in MAC, if possible.

Other issues, if time allows

R2-2505578	Concluding XR rate control for Rel-19	Nokia, Nokia Shanghai Bell	discussion	NR_XR_Ph3-Core

Proposal 5: For XR rate control, RAN2 discusses whether the prohibit timer starts or restarts when the UE receives the uplink rate control MAC CE from the gNB.

The proposal is not pursued

DISCUSSION:

- QCM does not like the proposal from UE perspective. It will prevent the UE from sending a query when it receives rate adaptation request from NW that it does not like.

- Nokia thinks UE should not send query if the NW is in congestion. QCM thinks that it is only one potential request.

- Apple thinks we agreed already to based on legacy and prefers not to enhance.

- Ofinno thinks the overhead is small anyway.

- Xiaomi also does not think it is needed.

- Vivo supports the proposal from Nokia.

- LGE indicates rate control query is lowest priority anyway.

R2-2505883	Remaining Issues on XR Rate Control	Ericsson	discussion	Rel-19

Proposal 1	Discuss the issue on the CN being unaware of UE’s XR rate control capability when determining the rate adaptability of the QoS flow.

Proposal 2	Send an LS to SA2 to discuss the issue about awareness of UE’s XR rate control capability in the CN.

- ZTE indicates that radio UE capabilities are sent to CN and stored there.

- Ericsson clarifies there is only storing, CN does not read them.

- ZTE thinks that AMF can read capabilities by implementation.

- Lenovo thinks that CN needs to know UE capabilities for many purposes and the CN knows already.

- OPPO has sympathy for Ericsson concern.

- Nokia think the information is in CN and is visible already.

RAN2 understands this can be discussed directly in other WGs, e.g. SA2

R2-2505073	Discussion on XR rate control	Qualcomm Incorporated	discussion	Rel-19	NR_XR_Ph3-Core

R2-2505118	Discussion on XR rate control	Huawei, HiSilicon	discussion	Rel-19	NR_XR_Ph3-Core

R2-2505140	XR rate control	Xiaomi	discussion	Rel-19	NR_XR_Ph3-Core

R2-2505173	Discussion on XR Rate Control	CATT	discussion	Rel-19	NR_XR_Ph3-Core

R2-2505262	UL Rate Control for XR	Ofinno	discussion	Rel-19

R2-2505374	XR Rate control details	ZTE Corporation, Sanechips	discussion

R2-2505587	Remaining open issues for XR Rate Control	Lenovo	discussion	Rel-19	NR_XR_Ph3-Core

R2-2505639	Uplink rate control for XR	NEC	discussion	Rel-19	NR_XR_Ph3-Core

R2-2505648	Discussion on UL congestion signaling	InterDigital	discussion	Rel-19	NR_XR_Ph3-Core

R2-2505750	Discussion on XR Rate Control	OPPO	discussion	Rel-19	NR_XR_Ph3-Core

R2-2505805	Remaining open issues on rate control signaling for XR	LG Electronics Inc.	discussion	Rel-19	NR_XR_Ph3-Core

R2-2505883	Remaining Issues on XR Rate Control	Ericsson	discussion	Rel-19

R2-2505971	Remaining open issues on rate control for XR	CMCC	discussion	Rel-19	NR_XR_Ph3-Core

R2-2506069	Discussion on XR rate control	HONOR	discussion	Rel-19	NR_XR_Ph3-Core

R2-2506130	Remaining issues of XR rate control	ETRI	discussion	Rel-19

R2-2506191	On remaining issue on XR UL rate control	NTT DOCOMO INC..	discussion	Rel-19

## 8.8	NTN for NR Ph3

(NR_NTN_Ph3-Core; leading WG: RAN2; REL-19; WID: RP-243300)

LTE_TN_NR_NTN_mob, leading WG: RAN2, Rel-19 WID: RP-240924)

Time budget: 2 TU

Tdoc Limitation: 3 tdocs

R2-2505744	Clarify SMTC2-LP offset assumption for NTN (align with SMTC/SMTC4list)	Jio Platforms	CR	Rel-19	38.331	18.6.0	5435	-	D	NR_NTN_Ph3-Core

=> Withdrawn

### 8.8.1	Organizational

LS, Rapporteur input, including workplan, running CRs, open issues lists, etc.

Rapporteur inputs do not count towards the tdoc limitation.

Including the outcome of the following email discussion:

[Post130][301][R19 NR NTN] Stage2 CR (Thales)

[Post130][302][R19 NR NTN] RRC CR (Ericsson)

[Post130][303][R19 NR NTN] 38.304 CR (ZTE)

[Post130][304][R19 NR NTN] capability CR (Apple)

For the LTE_TN_NR_NTN_mob WI, including endorsed draft CRs from the WI spec rapporteurs.

Incoming LSs

R2-2505023	LS on NR-NTN TP for TS 38.300 (NR_NTN_Ph3; contact: Thales)	RAN1	LS in	Rel-19	NR_NTN_Ph3	To:RAN2

Noted

R2-2505024	LS on Msg4 PDSCH repetition (R1-2504936; contact: Thales)	RAN1	LS in	Rel-19	NR_NTN_Ph3	To:RAN2

Noted

R2-2505050	Reply LS on SMTC enhancements (R4-2508433; contact: Xiaomi)	RAN4	LS in	Rel-19	NR_NTN_Ph3-Core	To:RAN2

- We can check offline whether we need to send another LS to RAN4 to explain RAN2 intentions about configuring more than 4 (i.e 6) SMTCs

Noted

R2-2505067	Reply on Inclusion of NTN intended service area in the Service Announcement (S4-251099; contact: Ericsson)	SA4	LS in	Rel-19	NR_NTN_Ph3-Core	To:RAN2	Cc:SA2

Noted

NR_NTN_Ph3-Core

- Stage 2 CR

R2-2506171	Stage 2 Running CR for NR NTN phase 3	THALES (Rapporteur)	CR	Rel-19	38.300	18.6.0	1023	-	B	NR_NTN_Ph3-Core

-	Thales thinks there are some editorials to be fixed

-	Ericsson that the current definition of ISA in the Stage 2 CR is different from the one in RAN3 and SA2

We can inform RAN3 and SA2 that RAN2 already decided that an ISA can span (parts of) multiple NTN cells and ask them to update their definition accordingly

Draft an LS to RAN3 and SA2 in R2-2506279

CR is endorsed as a baseline for further discussion

R2-2506279	Draft LS on the definition of Intended Service Area	Thales	LS out	Rel-19	NR_NTN_Ph3-Core	To:RAN3, SA2

-	Thales indicates that after further checking the other groups specification there seems to be no issue so no LS in needed

Withdrawn

- 38.331 CR

R2-2505828	Introduction of NTN Phase 3 enhancements	Ericsson	draftCR	Rel-19	38.331	18.6.0	B	NR_NTN_Ph3-Core

CR is endorsed as a baseline for further discussion

R2-2505825	Remaining RRC open issues for NR NTN Rel-19	Ericsson	discussion	Rel-19	NR_NTN_Ph3-Core

[Proposals for easy agreement]

SMTC5 list can be configured independently. Delta signalling with SMTC4 list can be supported to avoid unnecessary overhead (5/5)

-	Nokia agrees with the proposal. Vivo agrees

The maximum number of 6 configured SMTCs include the SMTC of the serving cell (5/5)

-	HW thinks the 6 configured SMTCs should only be for the neighbour cells

-	Xiaomi suggests to go for 7. Apple/CMCC/Ericsson agree

The maximum number configured SMTCs for idle/inactive is 7 and it also includes the SMTC of the serving cell (This updates a previous decision to have a maximum of 6 STMCs).

Existing mechanisms are sufficient to associated the ISA with relevant neighbour frequencies to perform MBS frequency prioritization (3/4).

We don’t introduce new AS mechanisms to associate the ISA with relevant neighbour frequencies to perform MBS frequency prioritization

[Proposals for further discussion]

FFS Stage 3 impact of the limit of maximum 2 SMTC periodicities per frequency layer and the configuration of 6 SMTC offsets per frequency. Companies are invited to bring concrete text proposals.

FFS whether multiple SMTC periodicities are needed in frequency layers different from the serving cell frequency (e.g., inter-frequency neighbours) and clarify in which inter-frequency scenario(s) this enhancement is beneficial.

SMTC enhancements in connected mode for NTN DL coverage enhancements are pending RAN2 confirmation of the working assumption: “We introduce a mechanism to assist the NW to configure the SMTCs in connected mode”. It is FFS the choice between Alt 1 or Alt2.

FFS to align RAN2 specifications and use the Ellipsoid Point with uncertainty Circle from TS 37.355 to define the circle shape of the ISA in the new SIBXX.

FFS on the terminology misalignment between ‘broadcast service area’ vs. ‘intended service area’ and the potential impact on the MII procedure.

R2-2505827	Inclusion of the ISA in the Service Announcement	Ericsson	discussion	Rel-19	NR_NTN_Ph3-Core

Observation 1	SA4 has confirmed the inclusion of the Intended Service Area defined with circles and polygon shapes in the Service Announcement.

Observation 2	RAN2 and SA4 have chosen different representations of the Intended Service Area in System Information and Service Announcement, respectively.

Proposal 1	It is up to UE implementation how to use the ISA definition in SIB/USD to skip MCCH acquisition when it is outside the ISA of the MBS broadcast service(s) it is interested in. Take the text proposals to TS 38.331 and TS 38.300 as baseline.

-`	Samsung indicates that in USD the used term is Target Service Area

It is up to UE implementation how to use the ISA definition in SIB/USD to skip MCCH acquisition when it is outside the ISA of the MBS broadcast service(s) it is interested in (can further check the wording for the change in 38.300 and 38.331)

Proposal 2	The Intended Service Area in the Service Announcement can be used for frequency prioritization in MBS NTN. Introduce the following NOTE in Stage 2 or TS 38.304.

-`	Fujitsu thinks that also for this we need to take into account that in USD the used term is Target Service Area

-	Nokia thinks we could also have some normative text

-	Thales thinks the Note is not needed

The Intended Service Area in the Service Announcement can be used for frequency prioritization in MBS NTN. Introduce a corresponding statement in TS 38.304 (FFS on the actual wording and whether it’s normative text or a note)

Proposal 3	Use the Ellipsoid Point with uncertainty Circle from TS 37.355 to define the circle shape of the ISA in the new SIBXX.

-	ZTE prefers to keep the existing signalling. HW agrees

-	Ericsson thinks that if we don’t align some NW entity will have to perform a mapping

Can be further discussed in the ASN.1 review

Agreements:

1.	The maximum number configured SMTCs for idle/inactive is 7 and it also includes the SMTC of the serving cell (This updates a previous decision to have a maximum of 6 STMCs).

2.	We don’t introduce new AS mechanisms to associate the ISA with relevant neighbour frequencies to perform MBS frequency prioritization

3.	It is up to UE implementation how to use the ISA definition in SIB/USD to skip MCCH acquisition when it is outside the ISA of the MBS broadcast service(s) it is interested in (can further check the wording for the change in 38.300 and 38.331)

4.	The Intended Service Area in the Service Announcement can be used for frequency prioritization in MBS NTN. Introduce a corresponding statement in TS 38.304 (FFS on the actual wording and whether it’s normative text or a note)

- 38.304 CR

R2-2505281	Introduction of Rel1-9 NR NTN in 38.304	ZTE Corporation, Sanechips	CR	Rel-19	38.304	18.4.0	0441	-	B	NR_NTN_Ph3-Core

CR is endorsed as a baseline for further discussion

- UE capability CRs

R2-2505489	Draft CR for Rel-19 NR NTN UE capabilities	Apple	draftCR	Rel-19	38.331	18.6.0	B	NR_NTN_Ph3-Core

CR is endorsed as a baseline for further discussion

R2-2505490	Draft CR for Rel-19 NR NTN UE capabilities	Apple	draftCR	Rel-19	38.306	18.6.0	B	NR_NTN_Ph3-Core

-	Apple/QC thinks we should limit the capability to support STMC enhancements to FR1. Ericsson agrees

We introduce a capability to support STMC enhancements to FR1 (FFS on FR2 depending on RAN4 progress)

CR is endorsed as a baseline for further discussion

Agreements:

1.	We introduce a capability to support STMC enhancements to FR1 (FFS on FR2 depending on RAN4 progress)

- K-Mac extension

R2-2506175	k-Mac extension for NR NTN	THALES, Samsung	CR	Rel-19	38.331	18.6.0	5463	-	F	NR_NTN_Ph3-Core

-	QC barring thinks it’s not a good solution and does not support this. Nokia agrees

-	Xiaomi thinks this is not a good way to introduce k-Mac extension

-	Samsung thinks that if we do it this is the only way to do it

-	Thales thinks this could be used for greenfield deployments

Postponed

[Post131][301][R19 NR NTN] Stage2 CR (Thales)

Scope: finalize the running Stage 2 CR

Intended outcome: Agreed CR

Deadline: short

=> Agreed in R2-2506592 (38.300)

[Post131][302][R19 NR NTN] RRC CR (Ericsson)

Scope: finalize the running RRC CR

Intended outcome: Agreed CR

Deadline: short

=> Agreed in R2-2506522 (38.331)

[Post131][303][R19 NR NTN] 38.304 CR (ZTE)

Scope: finalize the running 38.304 CR

Intended outcome: Agreed CR

Deadline: short

=> Agreed in R2-2506547 (38.304)

[Post131][304][R19 NR NTN] capability CR (Apple)

Scope: finalize the running capability CR

Intended outcome: Endorsed CR

Deadline: short

=> Endorsed in:

- R2-2506531 (38.331)

- R2-2506532 (38.306)

[Post131][315][R19 NR NTN] MAC CR (Interdigital)

Scope: create the MAC CR

Intended outcome: Agreed CR

Deadline: short

=> Agreed in R2-2506560 (38.321)

R2-2506562	Introduction of NR NTN phase 3	THALES (Rapporteur)	CR	Rel-19	38.300	18.6.0	1023	2	B	NR_NTN_Ph3-Core

=> Agreed

R2-2506522	Introduction of NTN Phase 3 enhancements	Ericsson	CR	Rel-19	38.331	18.6.0	5481	-	B	NR_NTN_Ph3-Core

=> Agreed

R2-2506547	Introduction of Rel-19 NR NTN	ZTE Corporation, Sanechips	CR	Rel-19	38.304	18.4.0	0441	1	B	NR_NTN_Ph3-Core

=> Agreed

R2-2506531	Draft CR for Rel-19 NR NTN UE capabilities	Apple	draftCR	Rel-19	38.331	18.6.0	B	NR_NTN_Ph3-Core

=> Endorsed

R2-2506532	Draft CR for Rel-19 NR NTN UE capabilities	Apple	draftCR	Rel-19	38.306	18.6.0	B	NR_NTN_Ph3-Core

=> Endorsed

R2-2506560	Introduction of NTN Phase 3 enhancements	InterDigital	CR	Rel-19	8.321	18.6.0	2121	-	B	NR_NTN_Ph3-Core

=> Agreed

From RAN2 point of view the WI on NR_NTN_Ph3 is completed.

- NTN less than 5MHz

R2-2505282	Introduction of less than 5MHz in NTN	ZTE Corporation, Xiaomi, Sanechips	CR	Rel-19	38.331	18.6.0	5389	1	B	NR_IoT_NTN_req_test_enh	R2-2504779

Agreed

R2-2505283	Introduce UE capability signalling for NTN less than 5MHz	ZTE Corporation, Xiaomi, Sanechips	CR	Rel-19	38.306	18.6.0	1307	2	B	NR_IoT_NTN_req_test_enh	R2-2504774

Agreed

LTE_TN_NR_NTN_mob

- Stage2 CR

R2-2506137	Introduction of stage 2 for LTE TN to NR NTN idle mode mobility	Samsung	CR	Rel-19	36.300	18.5.0	1412	6	B	LTE_TN_NR_NTN_mob	R2-2504096

CR is endorsed as a baseline for further discussion

- 36.331 CR

R2-2506144	Introduction of LTE TN to NR NTN IDLE mode mobility	CATT	CR	Rel-19	36.331	18.6.0	5065	6	B	LTE_TN_NR_NTN_mob-Core	R2-2504530

-	Ericsson thinks that in SIB33 there is a definition of ephemeris applicable for R19 but we also need a definition for R18

-	Xiaomi thinks there is an issue with smtc in the CarrierInfoNR-r19

CR is endorsed as a baseline for further discussion, e.g. to cover the comments above

- 36.306 CR

R2-2505389	Introduction of LTE TN to NR NTN Mobility UE Capability	vivo	CR	Rel-19	36.306	18.5.0	1918	-	B	LTE_TN_NR_NTN_mob-Core

CR is endorsed as a baseline for further discussion

[Post131][316][LTE NR NTN mob] Stage2 CR (Samsung)

Scope: finalize the running Stage 2 CR

Intended outcome: Agreed CR

Deadline: short

=> Agreed in R2-2506553 (36.300)

[Post131][317][LTE NR NTN mob] RRC CR (CATT)

Scope: finalize the running RRC CR

Intended outcome: Agreed CR

Deadline: short

=> Agreed in R2-2506535 (36.331)

[Post131][318][LTE NR NTN mob] capability CR (vivo)

Scope: finalize the running capability CR

Intended outcome: Agreed CR

Deadline: short

=> Agreed in R2-2506549 (36.306)

R2-2506553	Introduction of LTE TN to NR NTN idle mode mobility	Samsung	CR	Rel-19	36.300	18.5.0	1412	7	B	LTE_TN_NR_NTN_mob

=> Agreed

R2-2506535	Introduction of LTE TN to NR NTN IDLE mode mobility	CATT	CR	Rel-19	36.331	18.6.0	5065	7	B	LTE_TN_NR_NTN_mob-Core

=> Agreed

R2-2506549	Introduction of LTE TN to NR NTN Mobility UE Capability	vivo	CR	Rel-19	36.306	18.5.0	1918	1	B	LTE_TN_NR_NTN_mob-Core

=> Agreed

From RAN2 point of view the WI on LTE_TN_NR_NTN_mob is completed

R2-2505147	Introduction of stage 2 for LTE TN to NR NTN idle mode mobility	Samsung	CR	Rel-19	36.300	18.5.0	1412	5	B	LTE_TN_NR_NTN_mob	R2-2504096

Withdrawn

R2-2505233	Introduction of LTE TN to NR NTN IDLE mode mobility	CATT	CR	Rel-19	36.331	18.6.0	5141	-	B	LTE_TN_NR_NTN_mob

Withdrawn

### 8.8.2	Downlink coverage enhancements

Contributions should focus on remaining open issues related to RAN2 aspects of DL coverage enhancements due to extended SIB periodicity (up to 160ms),  e.g. possible SMTC impacts (while no contributions are expected on cell level / beam level DTX/DRX mechanism).

- SMTC enhancements

  - Two different SMTC periodicities (with different offsets) for SMTCs

  - Multiple SMTC periodicities for inter-frequency neighbour cells

  - Signalling details

R2-2506052	Discussion on DL coverage enhancements	Huawei, HiSilicon	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1: Support configuring two different SMTC periodicities (with different offsets) for SMTCs in one frequency layer for Idle/Inactive and Connected mode UEs.

-	QC thinks we should not support this in RAN2 based on the impact on the workload in RAN4. CATT agrees. Samsung also agrees

-	Ericsson is not so sure about the extra benefit for the UE if we support multiple SMTC periodicities

-	vivo thinks we already discussed the need to support this and should not repeat the discussion. Xiaomi agrees

-	Fujitsu thinks that we can confirm our agreement and if RAN4 does not conclude their work on requirement we can finally limit the configuration

RAN2 assumes that if we support configuring two different SMTC periodicities (with different offsets) for SMTCs in one frequency layer:

    - This is mainly for connected mode.

    - This will imply a dedicated UE capability

    - If RAN4 will not finish their requirements in R19 we can introduce some statement in the specs in the future to limit the configuration to one periodicity only

Continue the discussion in offline 303 on whether we support configuring two different SMTC periodicities (with different offsets) for SMTCs in one frequency, based on the RAN2 assumptions

Proposal 2: Multiple SMTC periodicities are needed for both intra-frequency neighbour cells and inter-frequency neighbour cells.

-	QC would like to limit this to intra-frequency

Can come back to this in the ASN.1 review

Proposal 5: For SSB and CORESET multiplexing pattern 1, SIB1 repetition transmission period is 160ms if SSB periodicity is 160ms.

-	Ericsson indicates that there is no consensus in RAN1 that this is needed

R2-2505421	Open issues on Downlink Coverage Enhancement	Samsung	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1: Introduce smtc5list, which is a list including up to 5 elements of SSB-MTC5, and each element SSB-MTC5 contains a list of PCI, an offset, and a periodicity (if agreed 2 SMTC periodicities). If a parameter (PCI list, or periodicity or offset) of an element SSB-MTC5 in smtc5list is absent, the corresponding parameter of the element in the same position in smtc4list is applied.

To be discussed in the CR review.

Proposal 2: If both smtc4list and smtc5list are configured, smtc5list is applied if supported and smtc4list is ignored.

To be discussed in the CR review.

R2-2505532	Discussion on beam hopping with multiple SMTC offsets	Qualcomm Incorporated	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 6	RAN2 first need to decide whether more than 4 SMTCs configuration is applicable for inter-frequency measurements.

Proposal 7	If P6 is agreed, a new UE capability to support more than 4 SMTCs configuration for inter-frequency measurements is introduced and an LS is sent to RAN4 for any issue/concern.

Proposal 8	Multiple SMTC periodicities are not introduced to be used in combination with more than 4 SMTCs per frequency layer.

[AT131][303][R19 NR NTN] Two SMTC periodicities (Xiaomi)

Scope: continue the discussion on whether to support configuring two different SMTC periodicities (with different offsets) for SMTCs in one frequency, based on the RAN2 assumptions

Intended outcome: summary of the offline discussion (in R2-2506280)

Offline time: Wednesday morning coffee break in BO1

Deadline for offline discussion summary:  Wednesday 2025-08-27 14:00

R2-2506280	Report of [AT131][303][R19 NR NTN] Two SMTC periodicity	Xiaomi	discussion	NR_NTN_Ph3-Core

Proposal 1: RAN2 supports to configure two different SMTC periodicities (with different offsets) for SMTCs per frequency layer for idle/inactive/connected mode, and UE capability will be introduced for this purpose. If RAN4 cannot finish the corresponding RRM requirement design, we can introduce some statement in the specs in the future to limit the configuration to one periodicity only.

-	Ericsson still thinks this is not needed, at least not for idle/inactive. Xiaomi thinks there is no real additional workload in RAN4 if idle/inactive is also supported

RAN2 supports to configure two different SMTC periodicities (with different offsets) for SMTCs per frequency layer for idle/inactive/connected mode, and UE capability will be introduced for this purpose (FFS if per UE or per band). If RAN4 cannot finish the corresponding RRM requirement design, we can introduce some statement in the specs in the future to limit the configuration to one periodicity only.

Send an LS to RAN4 on latest RAN2 agreements about SMTC enhancements

Agreements:

1.	RAN2 supports to configure two different SMTC periodicities (with different offsets) for SMTCs per frequency layer for idle/inactive/connected mode, and UE capability will be introduced for this purpose (FFS if per UE or per band). If RAN4 cannot finish the corresponding RRM requirement design, we can introduce some statement in the specs in the future to limit the configuration to one periodicity only.

  - Further details of location-based SMTC selection

  - SSB-index based SMTC selection in idle/inactive mode

  - WA on UE assistance for SMTC selection in connected mode

R2-2506052	Discussion on DL coverage enhancements	Huawei, HiSilicon	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 3: Support SSB-based SMTC for UEs in Idle/Inactive mode.

Proposal 4: Confirm the working assumption and support Alt1: UE provide the closest N reference locations/neighbour cells to network. Adopt the TP in the Annex.

R2-2505636	Discussion on Remaining Issues of DL coverage enhancement	Beijing Xiaomi Mobile Software	discussion	Rel-19

Proposal 4: [RRC-3] RAN2 agrees not to confirm the WA on assisting NW to configure SMTCs in RRC connected mode.

Proposal 5: [RRC-not listed] RAN2 agrees to broadcast a distance threshold together with the reference location of neighbor cells for location based SMTC selection in idle/inactive mode.

Observation 1: If SSBs are transmitted with different satellite beams, serving cell SSB index based SMTC selection can save UE power consumption as it doesn't require location acquisition.

Proposal 6: [RRC-not listed] RAN2 agrees to support serving cell SSB index based SMTC selection.

Proposal 7: [RRC-not listed] For serving cell SSB index based solution, a bitmap of SMTCs per SSB is broadcast in SIB.

R2-2505532	Discussion on beam hopping with multiple SMTC offsets	Qualcomm Incorporated	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1	Location-based SMTC enhancement is also supported in moving cell scenario. The existing movingReferenceLocation is reused for this purpose (i.e., network can update the movingReferenceLocation with epoch time without notifying UE).

Proposal 2	Location-based SMTC is sufficient to solve issue of selecting four SMTCs both in connected mode and idle mode measurements and there is no need to introduce second solution to solve the same issue in Rel-19.

Proposal 3	If there is no scheduling restriction from configuration of more than 4 SMTCs based on UE’s capability, the UE does not report to network which SMTCs are used.

Proposal 4	If there is scheduling restriction from configuration of more than 4 SMTCs based on UE’s capability, extend RRCReconfigurationComplete message to indicate which SMTCs the UE has selected.

Proposal 5	Introduce a UE capability signaling to indicate whether the UE supports configuration of more than 4 SMTCs for intra-frequency measurements.

R2-2505925	Remaining issues on Downlink coverage enhancements	Nokia, Nokia Shanghai Bell	discussion	NR_NTN_Ph3-Core

- SMTC selection in RRC connected

Proposal 5: RAN2 to confirm WA to provide SMTC configuration assistance in RRC_CONNECTED

Proposal 6: RAN2 go for Alt 1 i.e. allowing the gNB to configure the UE to report the x most suitable SMTC configurations. FFS whether it is up to UE implementation how to determine the information.

Proposal 7: the closest N reference locations could be based on the RRC_IDLE/RRC_INACTIVE information i.e. by UE reporting the index of the reference location.

Proposal 8: UE assistance information should be the index of the legacy neighbour cell list of the closest N neighbours

R2-2506153	DL coverage enhancements	Ericsson	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 2	RAN2 to confirm the WA (to introduce a mechanism that assists the network to configure SMTCs in RRC_CONNECTED).

Observation 8	Alt. 1 overcomes many of the drawbacks of an approach relying on legacy means to select SMTC(s), except for the requirement for an additional RRC reconfiguration for final configuration of the selected SMTC(s).

Observation 9	The requirement to send two RRC reconfiguration messages for each UE entering RRC_CONNECTED poses a scalability issue, especially given the large number of UEs and the scarceness of radio resources in NTN cells.

Observation 10	Alt. 1b is a variant of Alt. 1 in which the UE reports the relevant (e.g., N closest) reference location as identified in RRC_IDLE/INACTIVE during RRC connection establishment/resume.

Observation 11	Alt. 1b effectively eliminates the need for additional signalling messages, minimizes the signalling overhead, and harmonizes the features for RRC_IDLE/INACTIVE and RRC_CONNECTED modes.

-	Samsung and Toyota think we can also avoid confirming the WA

-	Ericsson thinks assistance information from the UE is beneficial. ZTE agrees

-	Oppo thinks the SMTC selection could change after this procedure

-	vivo is not sure we need this enhancement

Continue the discussion on Alt1b and Alt 2 in offline 304

Observation 12	Alt. 2 solves the issue of SMTC selection in RRC_CONNECTED as good as Alt. 1 but with less signalling overhead in terms of number of RRCReconfiguration messages, thus solving the otherwise arising scalability challenge.

Proposal 3	A UE selects the most relevant SMTC(s) based on location information included in the measurement configurations and reports its selection to the network.

Proposal 4	In RRC_CONNECTED, the network can configure the UE to select a certain number of SMTCs, e.g., K, among the configured SMTCs.

R2-2506014	Discussion on Downlink Coverage Enhancements	CSCN, ZTE Corporation, Sanechips, Huawei, HiSilicon, CATT	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1: RAN2 should introduce an SSB-based solution that uses the selected beam which UE camps on to map neighbour cell SMTCs for SMTC selection enhancement in RRC_Idle/Inactive mode.

Proposal 2: RAN2 should introduce a mechanism to assist the NW to configure SMTCs based on UAI in RRC_CONNECTED mode.

[AT131][304][R19 NR NTN] UE assisted SMTC selection in connected (Ericsson)

Scope: continue the discussion on Alt1b and Alt2 for UE assisted SMTC selection in connected mode

Intended outcome: summary of the offline discussion (in R2-2506281)

Offline time: Wednesday afternoon coffee break in BO1

Deadline for offline discussion summary:  Wednesday 2025-08-27 18:00

Scope: continue the discussion on the remaining FFS: content of the UE assistance information (e.g. index/indices of preferred SMTCs or closest/best reference location(s)) and whether in connected mode the NW can only configure up to 4 STMCs

Intended outcome: summary of the offline discussion (in R2-2506291)

Offline time: Thursday 2025-08-28 15:00:15:30 in BO3

Deadline for offline discussion summary:  Friday 2025-08-29 08:00

R2-2506281	Report of [AT131][304][R19 NR NTN] UE assisted SMTC selection in connected	Ericsson	discussion	NR_NTN_Ph3-Core

Proposal 1	It is beneficial to provide assistance information to configure the most relevant SMTCs for the area where the UE is located.

RAN2 reconfirms the understanding that if the NW configures more than 4 STMCs the UE in idle/inactive the UE selects up to 4 STMCs (or less, depending on the UE capability) based on location and only considers them for measurements

In connected mode the NW will only configure up to 4 STMC.

UE Assistance Information message will include information to the NW to (re)configure the most relevant SMTCs for the area where the UE is located

Continue in offline 304

Proposal 2	Based on the selected solution, it is FFS whether there is SA3 (or other working group) impact.

Proposal 3	UE Assistance Information can be used by the UE to provide future updates on closest SMTC based on the previously received information.

-	Nokia thinks we could just remove “future” from p3 and then we have a full proposal

-	Xiaomi still thinks different options are not clear

-	Samsung is not sure there is strong support for this enhancement

-	QC can accept either original option 1 or 2, but not 1a

-	ZTE supports the enhancements and supports Alt1 but in any case we need to fix the wording of p3

-	Samsung can compromise on the solution using UAI to convey the information on the closest references

R2-2506291	Report of [AT131][304][R19 NR NTN] UE assisted SMTC selection in connected – second round	Ericsson	discussion	NR_NTN_Ph3-Core

Proposal 1         We adopt Alt 1 based on companies’ preference. (6/10)

Proposal 2         The procedure is as shown in the flowchart of Alt. 1 in Figure 1:

Step 1: The network configures the UE with a list of reference locations and a parameter N, thereby configuring the UE to report the N closest reference locations. If the list of reference locations is not provided, the list of reference locations provided in system information is reused.

Step 2: The UE reports an indication of the N closest reference locations via UE assistance information, e.g. in form of a bitmap, where each bit maps to a reference location.

FFS if the UE can report the N closest reference locations via the RRCReconfigurationComplete message.

Step 3: The network configures the relevant SMTC(s). The maximum number of configured SMTCs is according to UE capabilities (same as legacy).

Step 4/5: The UE provides an update on the set of the N closest reference locations, if changed, via UE assistance information. Upon receiving an update, the network reconfigures the relevant SMTC(s), if changed.

The procedure for UE assisted SMTC configuration in connected mode is follows:

- -	For a UE indicating the relevant capability (FFS the name of the capability and it this is merged with some other capability) the network configures the UE via reconfiguration message with a list of reference locations and a parameter N, thereby configuring the UE to report the N closest reference locations (FFS whether also a distance threshold is indicated). If the list of reference locations is not provided, the list of reference locations provided in system information is reused.

- -	The UE reports an indication of the N closest reference locations via UE assistance information, e.g. bitmap or list of indices of the locations.

- -	The UE can report the N closest reference locations via the RRCReconfigurationComplete message.

- -	The network configures the relevant SMTC(s). The maximum number of configured SMTCs is according to legacy UE capability in terms of SMTC support (same as legacy).

- -	The UE provides an update on the set of the N closest reference locations, if changed, via UE assistance information. Upon receiving an update, the network reconfigures the relevant SMTC(s), if changed.

Agreements:

1.	RAN2 reconfirms the understanding that if the NW configures more than 4 STMCs the UE in idle/inactive the UE selects up to 4 STMCs (or less, depending on the UE capability) based on location and only considers them for measurements

2	In connected mode the NW will only configure up to 4 STMC.

3.	UE Assistance Information message will include information to the NW to (re)configure the most relevant SMTCs for the area where the UE is located

4.	The procedure for UE assisted SMTC configuration in connected mode is follows:

-	For a UE indicating the relevant capability (FFS the name of the capability and it this is merged with some other capability) the network configures the UE via reconfiguration message with a list of reference locations and a parameter N, thereby configuring the UE to report the N closest reference locations (FFS whether also a distance threshold is indicated). If the list of reference locations is not provided, the list of reference locations provided in system information is reused.

-	The UE reports an indication of the N closest reference locations via UE assistance information, e.g. bitmap or list of indices of the locations.

-	The UE can report the N closest reference locations via the RRCReconfigurationComplete message.

-	The network configures the relevant SMTC(s). The maximum number of configured SMTCs is according to legacy UE capability in terms of SMTC support (same as legacy).

-	The UE provides an update on the set of the N closest reference locations, if changed, via UE assistance information. Upon receiving an update, the network reconfigures the relevant SMTC(s), if changed.

R2-2506290	LS on additional agreements on SMTC enhancements	Xiaomi	LS out	Rel-19	NR_NTN_Ph3-Core	To:RAN4

Add the agreements on keeping the maximum number of configured SMTC in Connected to 4 and on the capability for FR1

Remove Draft

Revised in R2-2506296

R2-2506296	LS on additional agreements on SMTC enhancements	Xiaomi	LS out	Rel-19	NR_NTN_Ph3-Core	To:RAN4

Remove RAN2

Revised in R2-2506298

R2-2506298	LS on additional agreements on SMTC enhancements	Xiaomi	LS out	Rel-19	NR_NTN_Ph3-Core	To:RAN4

Approved

- Msg4 PDSCH repetition

R2-2505285	Consideration on Msg4 PDSCH repetition	ZTE Corporation,  Sanechips	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1: LCID in table 6.2.1-2C when LX field is set to 1 is used for UE to report capability/request of Msg4 PDSCH repetition.

Agreed

Proposal 2: No further conditions are introduced for the UE to report its capability/request for Msg4 PDSCH repetition.

-	Apple thinks we can follow the solution for other similar enhancements

Agreed (we can come back to this if we receive other indications from RAN1)

Proposal 3: RAN2 assumes combination of below features are possible and will define a signalling to support any combination of below features using LCID values in Table 6.2.1-2c:

  - Msg4 PDSCH repetition

  - Msg4 HARQ-ACK PUCCH repetition

  - (e)RedCap

-	Ericsson wonders if it’s ok to limit the possibility to introduce other features in the future

Agreed

The combination of Msg4 PDSCH repetition, Msg4 HARQ-ACK PUCCH repetition and (e)RedCap shall be confirmed by RAN1.

Proposal 4: If P1-P3 is agreed, RAN2 discusses and agrees TP attached in the annex.

R2-2505492	Msg4 PDSCH repetition in NTN	Apple	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1: Network configures one RSRP threshold for Msg4 PDSCH repetition in SIB19.

Proposal 2: As baseline, introduces extra 6 LCID code points when the LX field is set to 1, for CCCH of size 48bits/64bits for Msg4 PDSCH repetition of normal UE(s), RedCap UE(s), and eRedCap UE(s).

Proposal 3: UE indicates its capability and/or request for Msg4 PDSCH repetition by transmitting Msg3 with those extra LCID code points if UE is capable of Msg4 PDSCH repetition and the measured DL reference signal strength is lower than configured RSRP threshold.

Proposal 4: RAN2 to discuss about joint indication to request for both Rel-18 UL coverage enhancement of PUCCH repetition for Msg4 HARQ-ACK and Rel-19 DL coverage enhancement of Msg4 PDSCH repetition.

Agreements:

1.	LCID in table 6.2.1-2C when LX field is set to 1 is used for UE to report capability/request of Msg4 PDSCH repetition.

2.	No further conditions are introduced for the UE to report its capability/request for Msg4 PDSCH repetition (we can come back to this if we receive other indications from RAN1)

3.	RAN2 assumes combination of below features are possible and will define a signalling to support any combination of below features using LCID values in Table 6.2.1-2c:

-	Msg4 PDSCH repetition

-	Msg4 HARQ-ACK PUCCH repetition

-	(e)RedCap

R2-2505078	Remaining Issues on SMTC Enhancements for NTN	vivo	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2505079	Remaining Issues on Repetition Enhancements for NTN	vivo	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2505225	Discussion on link level enhancement	CATT	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2505226	Discussion on potential SMTC enhancements	CATT	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2505284	Consideration on remaining issues on SMTC enhancements	ZTE Corporation,  Sanechips	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2505285	Consideration on Msg4 PDSCH repetition	ZTE Corporation,  Sanechips	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2505293	Remaining consideration on NR NTN downlink coverage enhancements	DENSO CORPORATION	discussion	NR_NTN_Ph3-Core

R2-2505351	Discussions on downlink coverage enhancement	Fujitsu	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2505421	Open issues on Downlink Coverage Enhancement	Samsung	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2505491	SMTC enhancement in NTN	Apple	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2505492	Msg4 PDSCH repetition in NTN	Apple	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2505532	Discussion on beam hopping with multiple SMTC offsets	Qualcomm Incorporated	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2505533	Msg4 PDSCH repetition capability indication	Qualcomm Incorporated	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2505608	Discussion on DL coverage enhancement for NTN	OPPO	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2505635	Msg3 indication on support of Msg4 PDSCH repetition	Nokia, Nokia Shanghai Bell	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2505636	Discussion on Remaining Issues of DL coverage enhancement	Beijing Xiaomi Mobile Software	discussion	Rel-19

R2-2505642	Discussions on supporting multiple SMTC periodicities for inter-frequency neighbour cells	ITRI	discussion	NR_NTN_Ph3-Core

R2-2505688	Some remaining issues for DL-CE in NTN	Lenovo	discussion	Rel-19

R2-2505707	Clarification on downlink coverage enhancement	NEC	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2505925	Remaining issues on Downlink coverage enhancements	Nokia, Nokia Shanghai Bell	discussion	NR_NTN_Ph3-Core

R2-2505953	Discussion on DL coverage enhancements due to extended SSB periodicity	CMCC	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2505985	Downlink coverage enhancement for NTN	InterDigital, Inc.	discussion	Rel-19

R2-2506013	Open Issues for NR NTN DL Coverage Enhancements in Rel-19	ETRI	discussion	Rel-19

R2-2506014	Discussion on Downlink Coverage Enhancements	CSCN, ZTE Corporation, Sanechips, Huawei, HiSilicon, CATT	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2506018	Discussion on Downlink Coverage Enhancements	Sharp	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2506052	Discussion on DL coverage enhancements	Huawei, HiSilicon	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2506150	Details on DL CE in NR NTN	NERCDTV	discussion	Rel-19

R2-2506153	DL coverage enhancements	Ericsson	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2506154	Discussion on Reply LS on SMTC enhancements	Ericsson	discussion	Rel-19	NR_NTN_Ph3-Core

### 8.8.3	Uplink Capacity/Throughput Enhancement

Contributions can be submitted on the possible RAN2 aspects of the agreements reached in RAN1.

R2-2505922	On OCC applicability to RACH-less handovers	Samsung	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1: RAN2 to confirm OCC is applicable for RACH-less handovers by suitable network implementation.

-	QC is ok with this

Can be discussed in the post email discussion / in the next meeting

Proposal 2: RAN2 to consider documenting OCC being applicable for RACH-less handovers in Stage 2 as seen in Annex A.

R2-2505436	Discussion on Uplink Capacity Enhancements	Huawei, HiSilicon	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2505956	Remaining issues on uplink capacity and throughput enhancement for NR NTN	CMCC	discussion	Rel-19	NR_NTN_Ph3-Core

### 8.8.4	Support of Broadcast service

Contributions should address the remaining open issues related to the signaling of the intended service area of a broadcast service.

- Known open issues

R2-2505352	Discussions on supporting broadcast service	Fujitsu, Ericsson	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1: RAN2 to confirm that, with respect to the NTN MBS service, the term ‘broadcast service area’ refers to the ‘intended service area’. Add a note in Stage 2.

-	Xiaomi thinks there is no need to cover this in Stage 2. Oppo agrees.

-	Thales/Nokia don’t think this clarification is needed

RAN2 understands that for MII reporting when an ‘intended service area’ is signalled the term ‘broadcast service area’ refers to the ‘intended service area’ (no spec impact)

Proposal 2: A UE may initiate MII upon entering or leaving the ISA. No specification change is required.

RAN2 understands that a UE may initiate MII upon entering or leaving the ISA (no spec impact)

R2-2505080	Remaining Issues on MBS Broadcast Provision in NTN	vivo	discussion	Rel-19	NR_NTN_Ph3-Core

- Target service area in Service Announcement:

Proposal 1: RAN2 confirms that there is no need to align the MBS ISA signaling in SIBxx with that in Service Announcement.

Proposal 2: It is up to UE implementation to use the targeted service area list in the USD to avoid acquiring the MCCH when the UE is outside the MBS service area of the MBS broadcast service.

Proposal 3: From RAN2 perspective, targeted service area list in the USD is not used for other purposes.

- Signalling details:

Proposal 4: If a broadcast session is intended for reception within the entire cell area, the network does not configure the intended area ID(s) associated with that service.

Proposal 5: RAN2 confirms that if no intended area ID is explicitly indicated in MCCH for an MBS broadcast service the UE is interested in, the UE considers the service is allowed for reception within the entire cell area, with legacy behavior applicable.

RAN2 confirms that if no intended area ID is explicitly indicated in MCCH for an MBS broadcast service the UE is interested in, the UE considers the service is applicable for reception within the entire cell area, with legacy behavior applicable (FFS whether we capture this in the spec)

- MII:

Proposal 6: RRC_CONNECTED UE may initiate the MBS Interest Indication reporting procedure when UE is entering or leaving the intended service area.

Proposal 7: The existing term “broadcast service area” in sub-clause 5.9.4.2 of TS 38.331 refers to “intended service area” when the cell provides SIBxx. Clarifying this with a NOTE in the RRC specification.

R2-2505716	Remaining Issues for MBS in NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1: RAN2 does not consider including ISA in Service Announcement, irrespective of the SA4 conclusions on targeted service area in Service Announcement.

Proposal 2: There is no need to explicitly define that the broadcast service area from a physics perspective may be different than the intended service area.

Proposal 3: RAN2 does not specify any new behaviour with respect to MBS Interest Indication when Rel-19 NTN capable UE enters or leaves the MBS broadcast service area.

- Other issues / optimizations

R2-2505822	Support for broadcast services in NR NTN	Ericsson	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1	Broadcast future neighbour cells providing the same MBS service(s) as the serving cell.

Proposal 2	In MBS NTN, a UE is informed whether the cell replacing the serving cell provides an MBS service with identical MTCH configuration parameters.

Can be discussed in the next meeting

R2-2505895	Discussion on MBS broadcast over NTN	Huawei, HiSilicon	discussion	Rel-19	NR_NTN_Ph3-Core

- ISA or ISAs

Proposal 1: Clarify whether "ISA" refers to the whole area in which an MBS broadcast service is to be received or to one polygon/circle defined in SIBxx of one cell, and update the TS 38.331 CR accordingly.

- Skip MCCH acquisition

Proposal 4a: SIBx indicates (e.g., 1-bit) whether the cell may broadcast MBS broadcast services relevant in the whole cell coverage.

Proposal 4b: If SIBx does not indicate that the cell may transmit MBS broadcast services relevant in the whole cell coverage, a UE interested in receiving one or more MBS broadcast service but not in any ISA of the cell may skip reading MCCH.

Proposal 5a: The UE that has acquired MCCH and is out of the ISAs of the MBS service of interest shall monitor MCCH change notification and re-acquire MCCH again.

Proposal 5b: Discuss how to reduce unnecessary MCCH re-acquisition for the UE that has acquired MCCH and is out of the ISAs of the MBS services of interest, such as:

alt. 1)		upon MCCH change notification, the UE only re-acquire MCCH when the bit indicating "service start" is set ;

alt. 2)	upon MCCH change notification, the UE only re-acquires MCCH when a new bit indicating "change of ISAs of ongoing MBS service" is set;

alt. 3)	the UE only monitors MCCH change notification every x MCCH modification periods, where x is provided to the UE e.g., in SIBx.

Moved here from 8.8

R2-2505747	Clarify UE use of SIBXX (ISA) to gate MCCH acquisition for MBS broadcast in NTN	Jio Platforms Limited	CR	Rel-19	38.300	18.6.0	1014	-	B	NR_NTN_Ph3-Core

Postponed

R2-2506054	ISA-aided frequency (de)prioritisation for MBS broadcast in NTN (RRC_IDLE/INACTIVE)	Jio Platforms Limited	CR	Rel-19	38.304	18.4.0	0445	-	B	NR_NTN_Ph3-Core

Postponed

Agreements:

1.	RAN2 understands that for MII reporting when an ‘intended service area’ is signalled the term ‘broadcast service area’ refers to the ‘intended service area’ (no spec impact)

2.	RAN2 understands that a UE may initiate MII upon entering or leaving the ISA (no spec impact)

3.	RAN2 confirms that if no intended area ID is explicitly indicated in MCCH for an MBS broadcast service the UE is interested in, the UE considers the service is applicable for reception within the entire cell area, with legacy behavior applicable (FFS whether we capture this in the spec)

R2-2505080	Remaining Issues on MBS Broadcast Provision in NTN	vivo	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2505149	Remaining issues on the support of broadcast service in NTN	ETRI	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2505227	Further discussion on support of broadcast service in NR NTN	CATT	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2505286	Consideration on remaining issues on broadcast service enhancements	ZTE Corporation,  Sanechips	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2505352	Discussions on supporting broadcast service	Fujitsu, Ericsson	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2505422	Open issues on Broadcast service area	Samsung	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2505572	Discussion on providing MBS service area in NTN network	OPPO	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2505669	The remaning issue of MBS in NTN	China Telecommunications	discussion

R2-2505689	Some remaining issues for MBS and ETWS broadcast	Lenovo	discussion	Rel-19

R2-2505716	Remaining Issues for MBS in NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2505767	Remaining open issues for MBS service continuity over NTN	Continental Automotive	discussion

R2-2505822	Support for broadcast services in NR NTN	Ericsson	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2505895	Discussion on MBS broadcast over NTN	Huawei, HiSilicon	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2505957	Remaining issues on broadcast service for NR NTN	CMCC	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2505979	Discussion on the remaining issues on the MBS	Xiaomi	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2506017	Remaining issues on intended service area	Sharp	discussion	Rel-19	NR_NTN_Ph3-Core

Moved here from 8.8

R2-2505748	ISA-aided frequency (de)prioritisation for MBS broadcast in NTN	Jio Platforms	CR	Rel-19	38.304	18.4.0	0443	-	B	NR_NTN_Ph3-Core

Withdrawn

### 8.8.5	Support of regenerative payload

Contributions, if any, should focus on the needed updates for Stage 2 description and on whether any other existing essential features (not considered so far) would be affected - and potentially need any modifications - in a regenerative payload architecture.

R2-2505706	Stage 2 updates for regenerative payload	NEC	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1: clarify UE supports mobility between gNBs operating with transparent and regenerative NTN payloads in section 16.14.3.2

Proposal 2: clarify in section 16.14.3.2.3 that satellite switch with re-synchronization is only for transparent mode

Can be discussed in the post meeting email discussion

R2-2505660	Satellite switch with re-sync in regenerative payload	Sony	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2505879	Remaining issues on support of regenerative payload	ETRI, Korea University	discussion	Rel-19	NR_NTN_Ph3-Core

### 8.8.6	LTE to NR NTN mobility

Contributions, if any, should focus on any possible missing aspects for the support of idle mode mobility between LTE and NR NTN.

R2-2505980	Discussion on the smtc in the CarrierInfoNR-r19	Xiaomi	discussion	Rel-19	NR_NTN_Ph3-Core

- The smtc-r19 is absent in the CarrierInfoNR-r19

Proposal 1: The following options can be considered for UE to determine SMTC when the smtc-r19 is not present in the CarrierInfoNR-r19.

  - Option 1: UE uses the SMTC configured in the SIB24 when the smtc-r19 is not present in the CarrierInfoNR-r19.

  - Option 2: Add the sentence ‘In this release, the smtc is mandatory present in the CarrierInfoNR-r19’ in the field description.

-	Oppo prefers option 2. ZTE and Samsung agree and thinks this apply also to the corresponding TEI19

-	vivo wonders if this is needed and we could live this to NW implementation. Ericsson and HW agree. CATT agrees

Can come back in the next meeting

- UE adjusts the SMTC according to the actual propagation delay

Proposal 2: For the smtc-r19 in the CarrierInfoNR-r19, the UE should adjust the offset according to the actual propagation delay

-	Oppo/ZTE support.

-	Ericsson thinks we should have this discussion in general not only for NTN

-	After the discussion ZTE could agree this is not essential

Can come back in the next meeting

R2-2505609	Discussion on dedicated priority via RRCConnectionRelease	OPPO	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1	Similar to redirection, include the satellite ID information in the FreqPriorityNR IE in RRCConnectedRelease

message when E-UTRA provide the dedicated priority for NR NTN frequency.

-	QC supports this

Can come back in the next meeting

R2-2505986	Discussion on LTE to NR NTN mobility	InterDigital, Inc.	discussion	Rel-19

Proposal 1:	The satellite ID list is present for each entry of FreqPriorityListNR-r15

## 8.9	IoT NTN Ph3

(IoT_NTN_Ph3-Core; leading WG: RAN2; REL-19; WID: RP-243278)

Time budget: 1 TU

Tdoc Limitation: 3 tdocs

### 8.9.1	Organizational

LS, Rapporteur input, including workplan, running CRs, open issues lists, etc.

Rapporteur inputs do not count towards the tdoc limitation.

Including the outcome of the following email discussion:

[Post130][305][R19 IoT NTN] Stage2 CR (Ericsson)

[Post130][306][R19 IoT NTN] RRC CR (Huawei)

[Post130][307][R19 IoT NTN] MAC CR (Mediatek)

[Post130][308][R19 IoT NTN] 36.304 CR (Nokia)

[Post130][309][R19 IoT NTN] capability CR (Qualcomm)

Incoming LSs

R2-2505004	Reply LS on S&F mode indications to NAS (C1-254119; contact: CICT Mobile)	CT1	LS in	Rel-19	5GSAT_Ph3_ARCH, IoT_NTN_Ph3-Core	To:RAN2	Cc:SA2

Noted

R2-2505021	Reply LS on CB Msg3 EDT for IoT NTN Ph3 (R1-2504905; contact: MediaTek)	RAN1	LS in	Rel-19	IoT_NTN_Ph3	To:RAN2

Noted

R2-2505026	Reply LS on CB Msg3 EDT for IoT NTN Ph3 (R1-2504959; contact: MediaTek)	RAN1	LS in	Rel-19	IoT_NTN_Ph3	To:RAN2

Noted

R2-2505056	Reply LS on stage 1 requirements for the support for PWS over satellite NGRAN in Rel-17 (S2-2505538; contact: Samsung)	SA2	LS in	Rel-19	IoT_NTN_Ph3-Core	To:RAN2, CT1	Cc:SA3, RAN3, SA1

-	QC highlights that SA2 agreement is that support is applicable only for NB-IoT RAT over NTN.

Noted

Stage 2 CR

R2-2505145	Introduction of IoT NTN phase 3	Ericsson	CR	Rel-19	36.300	18.5.0	1425	-	B	IoT_NTN_Ph3-Core

Endorsed as a baseline for further discussion

R2-2506185	IoT NTN phase 3 open issues in Stage 2	Ericsson	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 1	If the CB-Msg4 is successfully decoded and it contains a HARQ ACK resource allocation for a UE, the UE shall always send HARQ ACK.

Proposal 2	If the CB-Msg4 is successfully decoded and it contains a matching UE contention resolution identity and a C-RNTI, if UE is later scheduled it follows the configured HARQ feedback enabled/disabled.

Proposal 3	RAN2 to confirm the current modelling in stage 2 where CB-Msg3-EDT is a variant of existing EDT procedures.

-	QC thinks we could have separate figures for CB-msg3-EDT

We confirm the current modelling in stage 2 where CB-Msg3-EDT is a variant of existing EDT procedures, but add separate figures for CB-Msg3-EDT

Proposal 4	RAN2 to discuss whether there is a need for further description of PWS in stage 2.

-	vivo thinks we could add examples about the cases where PWS may not be supported

Can further discuss this in the final CR review

36.331 CR

R2-2505246	Introduction of IoT NTN Ph3	Huawei, HiSilicon	CR	Rel-19	36.331	18.6.0	5137	-	B	IoT_NTN_Ph3-Core

Endorsed as a baseline for further discussion

R2-2505247	RRC open issue list for IoT NTN	Huawei, HiSilicon	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 0: The value ranges for all the RRC parameters can be further checked during the next CR review.

Agreed

Proposal 1: (RRC-2) RAN2 to discuss whether to allow the UE to skip reading SIB1-NB to shorten the latency of PWS acquisition.

Proposal 2: (RRC-3) RAN2 to discuss whether to allow UE to receive and assemble PWS segments from different cells during mobility based on proponent’s TP.

Proposal 3: (RRC-4) RAN2 to discuss whether the information of current working mode should be forwarded to upper layers after SI acquisition.

Agreements:

1. The value ranges for all the RRC parameters can be further checked during the next CR review.

36.321 CR

R2-2505201	Introduction of IoT NTN enhancements phase 3	MediaTek Inc.	CR	Rel-19	36.321	18.4.0	1591	-	B	IoT_NTN_Ph3-Core	R2-2504525

Endorsed as a baseline for further discussion

R2-2505555	Remaining MAC open issues in IoT NTN	MediaTek Inc.	discussion	Rel-19	IoT_NTN_Ph3-Core	R2-2504526

- Potentially easy agreements

Proposal 5: When the CB-Msg3ResponseTimer expires and the maximum number of re-attempts has been reached, the MAC indicates failure of CB-Msg3 transmission to the upper layer (i.e. RRC) [11/11].

Agreed

Proposal 8: CB-Msg3 response window is modeled as a timer in MAC [12/12].

Agreed

Proposal 9: If the PDCCH is successfully decoded before CB-Msg3ResponseTimer expires, but the corresponding PDSCH is successfully decoded after the timer has expired, the MAC PDU is treated the same as the one successfully decoded before the timer expires. [11/11]

Agreed

- Need mode discussion

Proposal 1: RAN2 to confirm the working assumption on CB-RNTI formula [10/11].

Proposal 2: For the agreed CB-RNTI formula from P1, RAN2 to discuss

•	Whether to define Msg3_W_index to floor(start SFN_id of Tx window / minimum Tx window length or periodicity].

•	Whether to define Y = 16.

•	Whether starting value X is optionally configurable.

Proposal 3: Power ramping in CB-Msg3-EDT is NOT supported [9/11].

Proposal 4: RAN2 to choose between 3ms or 0ms processing time when determine the Msg4 monitoring starts.

Proposal 6: RAN2 to discuss the RRC behavior when receiving the CB-Msg3 EDT failure indication from MAC.

Proposal 7: RAN2 to further discuss whether to support multiple TBS.

•	Option 1 [4/11] – Support multiple TBS per CE level

•	Option 2 [7/11] – One TBS per CE level is enough

o	Note that 3 companies think R1 input is needed

Proposal 12: RAN2 to discuss whether to support CQI reporting in anchor carrier and/or non-anchor in CB-Msg3-EDT procedure. If yes, RAN2 to discuss how does the UE prepare DL channel quality measurement result in CB-Msg3 without Msg1/Msg2.

Agreements:

1.	When the CB-Msg3ResponseTimer expires and the maximum number of re-attempts has been reached, the MAC indicates failure of CB-Msg3 transmission to the upper layer (i.e. RRC).

2.	CB-Msg3 response window is modeled as a timer in MAC.

3.	If the PDCCH is successfully decoded before CB-Msg3ResponseTimer expires, but the corresponding PDSCH is successfully decoded after the timer has expired, the MAC PDU is treated the same as the one successfully decoded before the timer expires.

36.304 CR

R2-2505249	Running CR for IoT-NTN Rel-19 Idle mode procedures	Nokia Solutions & Networks (I)	CR	Rel-19	36.304	18.4.0	0882	-	B	IoT_NTN_Ph3-Core

Endorsed as a baseline for further discussion

R2-2505872	Rapporteur Summary TS36.304 Open Issues	Nokia , Nokia Shanghai Bells	discussion

Proposal 1:  Introduce acceptable cell support for NB-IoT in Rel-19.

Proposal  1A: Following are the specification changes needed to reflect the support for acceptable cell feature for NB-IoT capable of PWS.

•	Figure in 5.2.2-2 to be updated.

•	Changes to 5.2.8a to clarify the PWS support of NB-IoT for acceptable cell.

Proposal 1B :

The Text proposal in Annexure A is considered as basis for introducing changes for acceptable cell support for NB-IoT

Proposal 2 :  Majority of the responses indicated that working assumption can be confirmed. However no convergence on whether to have specification changes for idle mode operation based on this parameter. RAN2 to discuss the expected UE behavior corresponding to the working assumption on introducing SF mode for neighbour cell.

Proposal 3:  RAN2 confirms that it is up to up to UE implementation on how to handle the S&F Monitoring list as specified in TS23.401. No RAN2 SPEC change expected.

Proposal 4:  Paging monitoring relaxation for cell operating in SF mode without any stored DL data can be considered for Rel-19 based on assessment of required spec changes. Required spec changes are evaluated based on text proposal from interested companies on the above issue.

UE capability CRs

R2-2505540	UE capability Running CR for Rel-19 IoT NTN	Qualcomm Inc.	CR	Rel-19	36.306	18.5.0	1912	1	B	IoT_NTN_Ph3-Core	R2-2504321

Endorsed as a baseline for further discussion

R2-2505541	UE capability draft RRC CR for Rel-19 IoT NTN	Qualcomm Incorporated	draftCR	Rel-19	36.331	18.6.0	IoT_NTN_Ph3-Core

Endorsed as a baseline for further discussion

R2-2505542	Open issues on Rel-19 IoT NTN UE capabilities	Qualcomm Incorporated	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 1	nextHopChainingCount-r15 in RRC Release message is reused for CB-Msg3 EDT using UP solution [8/8].

Agreed (with likely no spec impact)

Proposal 2	RAN2 decide whether to introduce a new optional UE capability without signaling for the support of MT CB-Msg3 EDT for UEs supporting MO CB-Msg3 EDT [5/8].

Proposal 3	From RAN2 perspective, PWS can be supported in NB-IoT Terrestrial Network. Inform SA1 that requirements for PWS support in NB-IoT terrestrial networks should be added.

From RAN2 perspective, PWS can be supported in NB-IoT Terrestrial Network. Inform SA1 (cc SA2) to consider whether requirements for PWS support in NB-IoT terrestrial networks should be added (FFS about support for acceptable cells)

Proposal 4	No UE radio capability signaling is introduced to indicate whether UE supports S&F mode operation [5/8].

Agreements:

1.	nextHopChainingCount-r15 in RRC Release message is reused for CB-Msg3 EDT using UP solution (likely no spec impact)

2.	From RAN2 perspective, PWS can be supported in NB-IoT Terrestrial Network. Inform SA1 (cc SA2) to consider whether requirements for PWS support in NB-IoT terrestrial networks should be added (FFS about support for acceptable cells)

R2-2506293	Draft LS PWS support in NB-IoT terrestrial networks	Qualcomm	LS out	Rel-19	NR_NTN_Ph3-Core	To:SA1	Cc:SA2

Include also the agreement that geofencing is not supported for NB-IoT in TN.

Remove Draft

Revised in R2-2506297

R2-2506297	LS PWS support in NB-IoT terrestrial networks	Qualcomm	LS out	Rel-19	NR_NTN_Ph3-Core	To:SA1	Cc:SA2

Approved

k-Mac extension

R2-2506174	k-Mac extension for IoT NTN	THALES, Samsung	CR	Rel-19	36.331	18.6.0	5153	-	F	IoT_NTN_Ph3-Core

Can come back in the next meeting

[Post131][305][R19 IoT NTN] Stage2 CR (Ericsson)

Scope: finalize the running Stage 2 CR

Intended outcome: Agreed CR

Deadline: short

=> Agreed in R2-2506494 (36.300)

[Post131][306][R19 IoT NTN] RRC CR (Huawei)

Scope: finalize the running RRC CR

Intended outcome: Agreed CR

Deadline: short

=> Agreed in R2-2506573 (36.331)

[Post131][307][R19 IoT NTN] MAC CR (Mediatek)

Scope: finalize the running MAC CR

Intended outcome: Agreed CR

Deadline: short

=> Agreed in R2-2506493 (36.321)

[Post131][308][R19 IoT NTN] 36.304 CR (Nokia)

Scope: finalize the running 36.304 CR

Intended outcome: Agreed CR

Deadline: short

=> Agreed in R2-2506556 (36.304)

[Post131][309][R19 IoT NTN] capability CR (Qualcomm)

Scope: finalize the running capability CR

Intended outcome: Agreed CR

Deadline: short

=> Agreed in R2-2506572 (36.306)

=> Endorsed in R2-2506538 (36.331)

R2-2506494	Introduction of IoT NTN phase 3	Ericsson	CR	Rel-19	36.300	18.5.0	1425	1	B	IoT_NTN_Ph3-Core

=> Agreed

R2-2506573	Introduction of IoT NTN Ph3	Huawei, HiSilicon	CR	Rel-19	36.331	18.6.0	5137	2	B	IoT_NTN_Ph3-Core

=> Agreed

R2-2506493	Introduction of IoT NTN enhancements phase 3	MediaTek Inc.	CR	Rel-19	36.321	18.4.0	1591	1	B	IoT_NTN_Ph3-Core

=> Agreed

R2-2506556	Introduction of IoT NTN Enhancements	Nokia, Nokia Shanghai Bell	CR	Rel-19	36.304	18.4.0	0882	1	B	IoT_NTN_Ph3-Core

=> Agreed

R2-2506572	UE capability for Rel-19 IoT NTN	Qualcomm Inc.	CR	Rel-19	36.306	18.5.0	1912	3	B	IoT_NTN_Ph3-Core

=> Agreed

R2-2506538	UE capability draft RRC CR for Rel-19 IoT NTN	Qualcomm Incorporated	draftCR	Rel-19	36.331	18.6.0	IoT_NTN_Ph3-Core

=> Endorsed

From RAN2 point of view the WI on IoT_NTN_Ph3 is completed

### 8.9.2	Support of Store & Forward

Contributions should focus on remaining open issues related to possible impacts to the radio interface.

R2-2505370	Leftover issues on the satellite S&F operation	Google	discussion	Rel-19	IoT_NTN_Ph3-Core

- (RRC-4) Whether to forward mode information to the upper layer

Observation 1	(RRC-4) The UE NAS needs to know from the RRC layer the current operation mode of the serving satellite, in order to decide whether to indicate the S&F capability in an Attach or in a TAU procedure.

Proposal 1	(RRC-4) UE forwards the S&F mode indication (i.e., sf-OperationMode) to upper layers upon acquiring it from SIB1(-NB). RAN2 to capture this UE behavior in 3GPP TS 36.331, sub-clause 5.2.2.7.

-	ZTE is fine with the intention but thinks we can add another information (i.e. transition time)

-	Sateliot thinks this is required for NAS.

The UE forwards the S&F mode indication (e.g. sf-OperationMode) to upper layers upon acquiring it from SIB1(-NB). RAN2 to capture this UE behavior in 3GPP TS 36.331, sub-clause 5.2.2.7.

regarding the indication to NAS of the transition time:

-	HW would like to keep the previous RAN2 agreement, also because this might be used at the application layer even if not needed at NAS layer. Vivo agrees HW.

-	Ericsson thinks we should revert the agreement based on the LS received by CT1. Xiaomi/Oppo agree with Ericsson.

-	QC thinks no indication to NAS is needed

-	Nokia doesn’t see the need to send the transition time to NS but they are ok to keep this

-	Toyota thinks we should not revert the agreement.

-	Also Apple would like to keep the agreement. ZTE agrees

- The impact of t-ModeSwitching on the UE AS

Observation 2	As the S&F mode indication (i.e., sf-OperationMode) and the legacy barring indications can only change at the boundaries of SI modification periods, the S&F mode transition time (i.e., t-ModeSwitching) signaled in SIB31 allows UEs to know more precisely when the serving cell changes its operational mode.

Proposal 2	Non-S&F-capable UE can utilize the transition time from the normal mode to the S&F mode to determine whether to establish/reestablish/resume an RRC connection (no spec impact).

Proposal 3	Non-S&F-capable UE can utilize the transition time from the normal mode to the S&F mode to determine when to trigger the measurement for cell reselection.

Proposal 4	If P3 is agreed, RAN2 to adopt the TP in Annex into the current 36.304 running CR.

- S&F mode indications for neighbour cells

Proposal 5	The S&F mode indication (i.e., sf-OperationMode) and the S&F mode transition time (i.e., t-ModeSwitching) of the neighbor satellite are signaled in SIB33 per neighbor satellite.

-	Sateliot would like to confirm the WA and supports the proposal

-	QC supports this but thinks transition time could be optional (and thinks UE behaviour could left to UE implementation)

-	Panasonic thinks this should be per satellite ID

-	vivo thinks it’s not clear how this would be used by the UE. MTK agrees

-	Ericsson supports this and thinks we should clarify the UE behaviour in terms of cell reselection. IDC agrees

-	ZTE supports this

-	Nokia thinks the information would be useful at the application layer

The S&F mode indication (i.e., sf-OperationMode) and the S&F mode transition time (i.e., t-ModeSwitching) of the neighbor satellite are signaled in SIB33 per neighbor satellite.

-	Sateliot thinks the same indication should be included in SIB32

Proposal 6	Non-S&F-capable UE deprioritizes the neighbor cells operating in the S&F mode by decreasing the cell rankings of these neighbor cells by a common delta value.

A Rel-19 UE may deprioritize the neighbor cells operating in the S&F mode (FFS on the detailed specification impact, i.e. whether a note is sufficient or some normative text. To be considered in the post meeting CR review)

-	IDC thinks we could also allow the NW to signal some offset instead

R2-2505228	Discussion on cell reselection enhancement based on the S&F monitoring list	CATT, Samsung, Google, Huawei, Thales, Sateliot	discussion	Rel-19	IoT_NTN_Ph3-Core

Observation 1: SA2 agreed that the S&F Monitoring list is used by the UE assist with saving power and determining which satellites to receive MT data/signalling.

Observation 2: As specified in the latest TS 23.401, SA2 intends to leave some room for the UE to decide which S&F satellite to access, with the consequence also specified that if the UE access a satellite that is not belonging to the S&F Monitoring List there is increased probability that it will not be able to complete the NAS procedure.

Observation 3: Following the current cell (re)selection procedure, the UE shall (re)select to/camp on a cell that meets related RSRP/RSRQ conditions and the NW configured priority. This leaves no room for UE implementation to further consider the S&F Monitoring list as specified in SA2 Spec, resulting in increasing UE access failure as highlighted by SA2 (in case of an access attempt to a satellite not in the S&F Monitoring list).

Proposal 1: RAN2 agrees that the UE may not consider the cell operating in S&F mode as a suitable cell for reselection, if it is not a cell provided by the satellites included in the S&F Monitoring list.

-	Xiaomi thinks it could be risky to prevent reselection to a normal cell

-	Ericsson does not agree with this. Nokia agrees and thinks this information is about pending NAS procedures and it should not affect cell reselection. ZTE also agrees.

-	HW thinks we can discuss AS impacts for this and sees some benefit to consider this information

-	CATT thinks we should clarify some AS impacts due to this. IDC agrees

-	Sateliot thinks we should at least pass the Satellite ID information to the NAS level. ZTE could agrees. IDC thinks this could work but it would be less efficient. Nokia is ok with this, but cell reselection should not be prevented

-	QC thinks we should not send information to NAS but indicate in AS that the use of this information is up to UE implementation

-	Ericsson has some sympathy for sending the information but thinks this should be requested by CT1

We send an LS to CT1 asking whether the Satellite ID information broadcast in SIB should be sent to NAS

Draft an LS in R2-2508285

Proposal 1a: RAN2 adopts the TP provided in the Annex A as the baseline, if Proposal 1 is agreeable.

Proposal 2: RAN2 discusses whether any new assistance information needs to be introduced into system information for the S&F operation (e.g. mapping between satellite ID(s) and {frequency(ies), PCI(s)}), if Proposal 1 is agreeable.

R2-2505229	Discussion on relaxation of IDLE mode task based on the S&F monitoring list	CATT, Samsung, Google, Thales	discussion	Rel-19	IoT_NTN_Ph3-Core

Observation 1: SA2 agreed that the S&F monitoring list is used by the UE assist with saving power and determining which satellites to receive MT data/signalling.

Proposal 1: When a S&F monitoring list is configured by upper layers, the UE may stop the IDLE mode tasks (e.g. cell (re)selection, paging monitoring, etc.) if the UE determines that it is out of coverage of all target satellite(s) indicated by the list. Endorse the TP in Annex.

Proposal 2: Clarify that the satellite IDs within the S&F monitoring list refers to the satelliteId with long-term valid ephemeris in SIB32.

Can come back in the next meeting

R2-2506152	On Satellite ID aspects for S&F Satellite operation	Sateliot, Thales, Novamint, CATT, Samsung, Ericsson, Nordic	discussion	Rel-19	R2-2504617

- Alignment of Satellite IDs:

Observation 1: AS-based and NAS-based satellite IDs are of the same type.

Observation 2: A UE that has been provided with a “S&F Monitoring List”, which includes a list of satelliteIDs, is expected to determine the satelliteID value of NTN cells operating in S&F from the satelliteID information broadcast in the SIBs.

Proposal 1: RAN2 is respectively requested to add a stage-2 description in Rel-18 specs to clarify that the satellite identifier value used in different SIB messages to identify the satellite assistance information of one certain satellite should be the same.

Proposal 2: RAN2 is respectfully requested to add a stage-2 description in Rel-19 specs to state that the satellite identifier values used at the AS-level (e.g., satellite identifiers included in SIB messages) and the NAS-level (e.g. satellite identifiers included in the S&F Monitoring list) are sent consistently, i.e. with the same value, when they refer to the same satellite (related TP provided in the Annex)

-	ZTE thinks we should not discuss NAS aspects in AS specs

-	Xiaomi supports the proposal, we should indicate that this should be ensured by NW implementation and inform other relevant groups

We clarify the stage-2 description in 36.300 to state that the satellite identifier values used at the AS-level (e.g., satellite identifiers included in SIB messages) and the NAS-level (e.g. satellite identifiers included in the S&F Monitoring list) are set consistently, i.e. with the same value, when they refer to the same satellite (FFS on the actual wording)

Inform CT1 and SA2 about this in the LS out

- Scope and uniqueness of Satellite IDs:

Proposal 3: RAN2 is respectfully requested to continue discussion on the scope and uniqueness of satellite IDs, considering options such as:

1)	A satellite identifier value should uniquely identify a satellite to the UE within a PLMN

2)	Quasi(?)-Global uniqueness based on an identifier such as PLMN

3)	Cell-specific uniqueness

4)	Up to network/UE implementation (e.g. the satellite Id does not have to be unique for the whole life of the network, but maybe only for a sufficiently long period of time ensuring that an assistance information got from SIB by the UE has expired and so, not be used)

Proposal 4: RAN2 is respectfully requested to capture outcome of P3 in Stage 2.

Proposal 5: If uniqueness is established to be quasi(?)-global and the value range of existing defined satellite IDs (satelliteId-r17/r18) is not enough, then introduce new satelliteId-r19.

Proposal 6: RAN2 discuss informing other WGs for alignment.

Continue the discussion offline 305

R2-2505105	Discussion on Store and Forward operation	Xiaomi	discussion	Rel-19	IoT_NTN_Ph3-Core

- Architecture option broadcasting

Proposal 3: RAN2 to consider indicating S&F architecture option (Split MME or Full CN) in SIB.

Proposal 4: UE does not initiate user plane establishment and user plane data transfer in the cell indicating split MME architecture.

Can come back in the next meeting

R2-2505437	Further consideration on Store and Forward	Huawei, HiSilicon, China Telecom	discussion	Rel-19	IoT_NTN_Ph3-Core

Paging in S&F satellite operation

Proposal 1:An indication is introduced in the System information to indicate that the UE could skip monitoring paging and initiating RRC connection for MT data reception.

Continue the discussion offline 305

[AT131][305][R19 IoT NTN] S&F open issues (Sateliot)

Scope: continue the discussion on remaining proposals from R2-2506152 and p1 and R2-2505437. Also draft the LS to CT1, SA2

Intended outcome: summary of the offline discussion (in R2-2506288). Draft LS in R2-2506285

Offline time: FFS

Deadline for offline discussion summary:  Friday 2025-08-29 18:00

R2-2506288	Report of [AT131][305][R19 IoT NTN] S&F open issues	Sateliot	discussion	IoT_NTN_Ph3-Core

Proposal 1: Given that there is no immediate need for having to increase the size of a satellite ID and define any new satellite ID, RAN2 is respectfully requested not to pursue this.

We don’t increase the size of satellite ID and or define any new satellite ID as part of this Rel-19 WI

Proposal 2: Given that it is not clear how P1 of [4] can optimize paging, RAN2 is respectfully requested not to pursue this.

-	HW thinks paging optimization is essential and easy to support. Nokia agrees. Ericsson see the benefit

-	QC thinks this is not easy and we cannot even discuss this alone without involvement of other groups.

-	Samsung is not sure how this would work

-	Toyota thinks this is useful

-	ZTE thinks in any case the UE would still have to monitoring system information. HW is not sure

-	Ericson thinks that SI update would not be trigger

Can come back in the next meeting

Agreements:

1.	The UE forwards the S&F mode indication (e.g. sf-OperationMode) to upper layers upon acquiring it from SIB1(-NB). RAN2 to capture this UE behavior in 3GPP TS 36.331, sub-clause 5.2.2.7.

2.	The S&F mode indication (i.e., sf-OperationMode) and the S&F mode transition time (i.e., t-ModeSwitching) of the neighbor satellite are signaled in SIB33 per neighbor satellite.

3.	A Rel-19 UE may deprioritize the neighbor cells operating in the S&F mode (FFS on the detailed specification impact, i.e. whether a note is sufficient or some normative text. To be considered in the post meeting CR review)

4.	We clarify the stage-2 description in 36.300 to state that the satellite identifier values used at the AS-level (e.g., satellite identifiers included in SIB messages) and the NAS-level (e.g. satellite identifiers included in the S&F Monitoring list) are set consistently, i.e. with the same value, when they refer to the same satellite (FFS on the actual wording)

5.	We don’t increase the size of satellite ID and or define any new satellite ID as part of this Rel-19 WI

R2-2506285	LS on Satellite ID for S&F	Sateliot	LS out	Rel-19	IoT_NTN_Ph3-Core To:CT1	Cc:SA2

Approved

R2-2505081	Remaining Issues on S&F Operation	vivo	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2505146	Store & Forward: Remaining Neighbour Cell Issues	PANASONIC	discussion

R2-2505178	Discussion on support

of Store&Forward	Transsion Holdings	discussion	Rel-19

R2-2505230	Discussion on leftover issues of S&F operation	CATT	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2505257	Remaining issues for S&F operation	ZTE Corporation, Sanechips	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2505294	Remaining consideration on Store & Forward operation	DENSO CORPORATION	discussion	IoT_NTN_Ph3-Core

R2-2505437	Further consideration on Store and Forward	Huawei, HiSilicon, China Telecom	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2505494	Remaining issues in S&F operation	Apple	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2505550	Discussion on Store & Forward satellite operation	OPPO	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2505567	RAN2 impact on S&F mode	MediaTek Inc.	discussion	IoT_NTN_Ph3-Core	R2-2504527

R2-2505690	Some remaining issues for S&F operation mode and transition time	Lenovo	discussion	Rel-19

R2-2505798	Discussion on Paging and Mode Switching	Toyota ITC	discussion	Rel-19	IoT_NTN_Ph3-Core	R2-2504097

R2-2505823	Support for store and forward in IoT NTN	Ericsson	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2505871	Open issues for SF operation	Nokia , Nokia Shanghai Bells	discussion

R2-2505878	Remaining issues on Store and Forward satellite operation	ETRI, Korea University	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2505916	Open issues on Store and Forward operation	Samsung	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2505928	Remaining issues for Store & Forward satellite operation	SHARP Corporation	discussion

R2-2505962	Discussion on Store and Forward remaining issues	CMCC	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2506033	Discussion on usage of time information for S&F	ASUSTeK	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2506070	Discussion on the Store and Forward satellite operation	HONOR	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2506151	On cell (re)selection and idle mode task relaxation for S&F Satellite operation	Sateliot, Thales, Novamint	discussion	Rel-19	R2-2504617

R2-2506156	Store and Forward open issues	Interdigital, Inc.	discussion	Rel-19	IoT_NTN_Ph3-Core

### 8.9.3	Uplink Capacity Enhancement

Contributions should focus on remaining open issues related to the enhancements to reduce the necessary uplink and downlink signaling to complete an EDT transaction (Msg3 transmission without msg1/RAR; efficient delivery of msg4 / RRCEarlyDataComplete).

- Working Assumption confirmation on CB-RNTI formula

including

  - Whether to define Msg3_W_index to floor(start SFN_id of Tx window / minimum Tx window length or periodicity].

  - Whether to define Y = 16.

  - Whether starting value X is optionally configurable.

(RAN2#130 Working Assumption:

The formula for RNTI for mMsg4 monitoring is:

RNTI=X + Msg3_W_index modulo (Y) + Y*CE_level + 3*Y*carrier_id.

•	X is the starting RNTI for Msg4 reception, which can be defined by RAN2 e.g. X=2401 for eMTC or 4097 for NB-IoT,

•	Msg3_W_index is the index of Msg3 transmission window within a periodicity of 1024 SFNs and index 0 corresponds to the Msg3 transmission window starts at the SFN defined by IE startSFN-r19,

•	Y is ceil (Msg4_WS/Msg3_WP),

•	CE_level is the CE level, 0 <= CE_level < 3

•	carrier_id is the index of the UL carrier of the CB-Msg3 resources, anchor carrier has index 0,

0 <= carrier_id < 16

Can come back to check if the NW can also simply configure RNTI = X)

R2-2506416	Discussion on CB-RNTI	Mediatek	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal: The CB-RNTI is calculated as below:

•	For eMTC

o	2401 + floor (start SFN_id of Tx window/2)

•	For NB-IoT

o	4097 + floor (start SFN_id of Tx window/4) + 256*carrier_id

o	carrier_id is the index of the UL carrier associated with the selected UL grants (0 <= carrier_id < 16). The carrier_id of the anchor carrier is 0.

-	QC thinks we should confirm the WA and fix what is needed (Y with a fixed value). We should also keep the CE level

-	Ericsson is generally ok with this proposal but not sure about the carried ID

Continue in offline 302

R2-2505082	Remaining Issues on CB-Msg3 EDT Mechanism	vivo	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 5 (MAC-2): RAN2 confirms that the network configuration of RNTI = X is not supported.

Proposal 6 (MAC-2): For CB-RNTI, define Msg3_W_index to floor(start SFN_id of Tx window / minimum Tx window length or periodicity).

Proposal 7 (MAC-2): For CB-RNTI, RAN2 confirms that there is no need to define Y=16. The configurable value of Msg3_WP is designed taking the valid range of CB-RNTI into account.

Proposal 8 (MAC-2): For CB-RNTI, RAN2 confirms that X is a hard-coded value.

R2-2505551	Discussion on CB-Msg3 EDT and Msg4 enhancement	OPPO	discussion	Rel-19

Proposal 1	The WA on CB-RNTI calculation formula can be confirmed.

Proposal 2	Define Msg3_W_index to floor (start SFN_id of Tx window / Tx window periodicity) where the Tx window periodicity configured for the selected CE level and carrier is used.

R2-2505231	Discussion on open issues for CB-Msg3 EDT	CATT	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 2 (MAC-2): For the agreed CB-RNTI formula:

-	Replace the Msg3_W_index with floor(SFN_id / [minimum Tx window periodicity]), where SFN_id is the index of the first radio frame of the selected CB-Msg3 transmission window.

-	Replace Y (i.e., ceil(Msg4_WS/Msg3_WP)) as a fixed value; the exact value can be determined when the candidate lengths of CB-Msg3 response timer length and CB-Msg3 transmission window periodicity are finalized.

-	Stick to fixed X value in the CB-RNTI formula, i.e., 2401 for eMTC or 4097 for NB-IoT.

R2-2505258	Remaining issues for CB-msg3-EDT	ZTE Corporation, Sanechips	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 1c: It’s no need to introduce any factor (e.g., modulo (Y)) to adjust the result of [floor (start SFN_id of Tx window/minimum Tx window periodicity)]. So the final formula can be like:

RNTI=X + [floor (start SFN_id of Tx window/minimum Tx window periodicity)] + Y*CE_level + 3*Y*carrier_id

R2-2506184	UL capacity enhancements for IoT NTN	Ericsson	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 6	A first CB-Msg3 window (M = 0), starting in startH-SFN and startSFN, is associated with a CB-RNTI equal to startCB-RNTI.

Proposal 7	For the Mth CB Msg3 window before or after the first CB-Msg3 window, the associated CB-RNTI is equal to:

CB-RNTI = startCB-RNTI + (M modulo (nrofCB-RNTI))

- Whether to support OCC

R2-2505536	Support of OCC	Qualcomm Incorporated, European Space Agency, German Aerospace Center	discussion	Rel-19	IoT_NTN_Ph3-Core

Observation 1.	The procedure for Msg3 transmission and Msg4 reception defined for non-OCC based CB-Msg3 EDT is applicable for OCC-based CB-Msg3 EDT. Only additional work is to define signalling support for OCC configuration.

Observation 2.	Even a power imbalance of 10 dB results in only a performance degradation that is minimal i.e., 0.25 dB for NPUSCH with OCC.

Proposal 1	Confirm that OCC-based CB-Msg3 EDT is supported.

-	Toyota agrees OCC should support OCC in Rel-19

R2-2505571	Discussion on CB-Msg3 procedure	MediaTek Inc.	discussion	IoT_NTN_Ph3-Core	R2-2504528

“There is still a significant amount of unfinished normative work, such as the OCC resource design, details on the RSRP threshold for transmitting CB-Msg3 using OCC, how the UE chooses OCC-based and OCC-free CB-Msg3 resource, how the UE selects the OCC code, how the UE transmits replicas using OCC, whether an extension of the CB-Msg4 monitoring window is needed, UE capability, etc. It is unlikely that all of these can be completed in a single RAN2 meeting. Therefore, we suggest that the OCC for CB-Msg3 is not supported in Release 19.”

Proposal 1: OCC for CB-Msg3 is NOT supported in Release 19.

-	Ericsson agrees we should not support OCC in this release and RAN1 might still have to work on this. Oppo agrees. Nokia also agrees

-	Session chair thinks that rather than the possible performance degradation and the need for further RAN1 work, a possible implication of supporting OCC for CB-Mg3-EDT is the need for further resource partitioning

-	QC and ZTE thinks that if we don’t take an agreement to introduce OCC now we can continue the work and then come back to add OCC support later, e.g. via TEi19

We don’t further work on the inclusion of OCC support for CB-Msg3-EDT as part of IoT_NTN_Ph3-Core in Rel19.

-	Session chair thinks that supporting companies might come back and suggest to introduce OCC support for CB-Msg3-EDT at a later point in time

- Power ramping

R2-2505632	Remaining issues on UL capacity enhancement for IoT NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-19	IoT_NTN_Ph3-Core

Observation 2: The CB-Msg3 UL transmission power calculation is not clearly defined in TS36.213 (i.e., which j should be followed for CB-Msg3 UL transmission power calculation).

Observation 3: In legacy Msg3 transmission, UE can not only perform Msg3 power ramping but also increase its CE level to improve the Msg3 transmission successful rate.

Observation 4: The UE has no means to improve its CB-Msg3 transmission reliablity if both the CB-Msg3 power ramping and the CE level increase are not supported.

Observation 5: There is no clear conclusion in RAN1 whether the CB-Msg3 power ramping should be supported because they have no time for performance evaluation. RAN2 can make the final decision considering the CB-Msg3 CE level increase is forbidden.

Proposal 3: Power ramping between CB-Msg3 windows is supported.

Propo

sal 4: The same principle as for legacy Msg3 power ramping (j=2) should be followed to limit the specification impact.

-	vivo thinks that without RAN1 work we cannot decide on power ramping

-	ESA thinks that if we can use existing procedures for power ramping we should do it

-	Samsung thinks we have agreed to avoid increasing the CE level assuming that there would be power ramping.

-	IDC is not sure we should introduce power ramping without RAN1

-	HW thinks power ramping is not essential

-	ZTE is ok to discuss a possible simple power ramping scheme

We try to define a simple power ramping scheme reusing existing mechanisms and then we inform RAN1 inviting them to respond if they find a problem

Continue in offline 302

From R2-2505231, R2-2505736, R2-2505551, R2-2505958:

Proposal: Power ramping is not supported for CB-msg3-EDT in Rel-19

-	QC agrees that power ramping might not be needed

- processing time

R2-2506184	UL capacity enhancements for IoT NTN	Ericsson	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 8	The NW processing time shall be considered for the start of Msg4 monitoring.

Observation 4	In legacy eMTC and NB-IoT, there is always 3 ms processing time for the eNB between a (N)PUSCH transmission and (N)PDCCH monitoring.

Proposal 9	The UE shall start PDCCH monitoring for CB-Msg4 earliest UE-eNB RTT + 3 ms after the end of the CB-Msg3 window.

R2-2505632	Remaining issues on UL capacity enhancement for IoT NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-19	IoT_NTN_Ph3-Core

“In TS36.321, the start time of the Msg3 response window were defined in both the legacy RACH and PUR, in which 4 ms were considered in PUR while it is not added in RACH contention resolution.

For CB-Msg3, we tend to keep the specification simple and align the UE behaviour with the contention resolution timer as defined in legacy RACH procedure. Furthermore, the processing time may not be needed in order to start the transmission of CB-Msg4 earlier since the network can process the Msg3 transmission and prepare for Msg4 transmission well in advance for those UEs who transmit the Msg3 at the early part of CB-Msg3 transmission window.”

Proposal 5: When the Msg4 monitoring window starts at the end of CB-Msg3-EDT transmission window plus UE-eNB RTT, there is no need to consider additional delays for NW/UE processing time.

-	Xiaomi and MTK would like to keep things simple and support the proposal

-	Ericsson can also accept this

Agreed

- RRC behavior when receiving the CB-Msg3 EDT failure indication from MAC.

R2-2505493	Remaining issues in CB-Msg3	Apple	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 3: In Section 5.3.3.1b of RRC spec, add a note that when both CB-Msg3-EDT and legacy EDT conditions are met, which one to select is up to UE.

We leave to UE implementation to decide in which order the conditions to trigger the different procedures are met.

Proposal 4: RAN2 to down select between Option 1 and Option 2, to support fallback from CB-Msg3-EDT to RACH based EDT.

•	Option 1: RRC procedure is not re-initiated. Same Msg3 is transmitted for RACH based EDT.

•	Option 2: RRC procedure is re-initiated (T300 is re-started), and how to avoid data loss is up to UE implementation.

When the RRC layer receives a CB-Msg3 EDT failure indication from the MAC layer the RRC procedure is re-initiated (how to avoid data loss is up to UE implementation). Which procedure (e.g. EDT, 4-step RACH, CB-Msg3-EDT) is initiated is up to UE implementation

Proposal 4a: If Option 1 is taken, the following procedures could be considered.

0) MAC notifies CB-Msg3-EDT failure to upper layer;

1) RRC layer continues with RACH based EDT condition checking and if RACH based EDT condition is met:

- RRC message is not re-generated;

- T300 keeps running (not re-started);

2) RRC layer informs MAC layer for EDT based RACH together with an indication “not flush Msg3”

3) MAC layer initiates EDT based RACH, with the same Msg3.

-	Ericsson thinks for EDT we did not specify how this is done and we could leave this to UE implementation. CATT and Oppo agree

-	MTK thinks option 1 is too complex and we could leave this to UE implementation.

-	Samsung wonders what happens to T300 and maybe one solution is to specify option 2

-	HW thinks that option 2 is simpler

-	Xiaomi can accept option 2

Proposal 5: Add one more condition for CB-Msg3-EDT condition checking that “CB-Msg3-EDT fallback indication has not been received from lower layers for this establishment or resumption procedure”.

R2-2505632	Remaining issues on UL capacity enhancement for IoT NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 6: When the RRC layer receives a CB-Msg3 EDT failure indication from the MAC layer, it may let the UE fall back to legacy 4-step RACH or EDT. The UE shall follow the legacy mechanism to determine whether to trigger EDT.

Proposal 7: The Msg3 buffer shall not be flushed when the UE falls back from the CB-Msg3 EDT procedure to the legacy 4-step RACH or EDT.

Proposal 8: The RRC layer shall provide a CB-Msg3 fallback indication to the MAC layer.

- Whether to support multiple TBS.(multiple vs one TBS per CE level)

R2-2505632	Remaining issues on UL capacity enhancement for IoT NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 9: Multiple TBSs and TBS selection are supported for CB-Msg3 EDT.

-	vivo thinks in any case we cannot decide this in RAN2 without RAN1. Xiaomi agrees

-	Samsung thinks this is not a critical enhancement

We don’t support multiple TBSs per CE level for CB-Msg3 EDT

Proposal 10: RAN2 to downselect below two options:

•	Option1: NW can configure multiple CB-Msg3 TBSs for a CE level (to avoid RAN1 impact)

•	Option2: UE is allowed to select TBS smaller than the configured CB-Msg3 TBS for the corresponding CE level.

R2-2505537	CB-Msg3-EDT and Msg4 multicast	Qualcomm Incorporated	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 5	Support multiple-TBSs option same as that of Rel-15 EDT.

Proposal 6	Support smaller TBS size of 144 bits for CB-Msg3 EDT.

We introduce one additional value smaller than the smallest possible TBS size at the moment (328bits), i.e. 144bits (discuss further details when drafting the LS to RAN1).

- whether to support CQI reporting in anchor carrier and/or non-anchor in CB-Msg3-EDT procedure. If yes, RAN2 to discuss how does the UE prepare DL channel quality measurement result in CB-Msg3 without Msg1/Msg2.

R2-2505917	On open issues for CB-Msg3-EDT	Samsung	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 5: RAN2 to confirm CQI reporting (in RRCEarlyData and DCQR and AS RAI MAC CE) in CB-Msg3 for CB-Msg3-EDT (for CP/UP eMTC and NB-IoT) is supported for anchor carrier.

-	MTK thinks this CQI reporting is not very useful in this procedure and we can avoid supporting this. Nokia agrees

-	QC thinks this could be supported at least for anchor carrier

-	Xiaomi wonders if also for the anchor carrier we need to check with RAN1

-	vivo thinks we need to also ask RAN4

-	HW is reluctant to introduce this if it impacts other WGs

-	Samsung thinks that in any case we need to spec changes to say that CQI reporting is not include in this case

We clarify that CQI reporting is not included in CB-Msg3 for CB-Msg3-EDT for non-anchor carrier but it can be included for anchor carrier (send LS to RAN4)

- Further details on resource configuration

R2-2505082	Remaining Issues on CB-Msg3 EDT Mechanism	vivo	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 1: RAN2 confirms that both multi-PRB allocation and sub-PRB allocation are supported for CB-Msg3.

Proposal 2: RAN2 confirms that both single-tone and multi-tone are supported for CB-Msg3, and intends to reuse the parameter npusch-MCS-r16 for CB-Msg3.

Proposal 3: RAN2 confirms only one narrowband is configured for MPDCCH monitoring, as the running CR implementation.

Proposal 4: RAN2 sends an LS to RAN1 to inform them of Proposal 1 and Proposal 2.

Proposal 9 (RRC-1.4): CB-Msg3 transmission window length and periodicity are configured in multiples of the CB-Msg3 PUSCH periodicity (e.g., value ranging from 1..32).

R2-2505632	Remaining issues on UL capacity enhancement for IoT NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 14: To support capacity enhancement in line with the objective of the WI, the MPDCCH narrowband configuration for CB-Msg4 monitoring shall be defined as a set, following the legacy RACH procedure.

R2-2505258	Remaining issues for CB-msg3-EDT	ZTE Corporation, Sanechips	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 3: It’s suggested to use value 3 as the number of frequency-domain resources set for both eMTC and NB-IoT.

Proposal 4a: It’s suggested to define the length of CB-msg3 transmission window in units of (N)PUSCH resources periodicity.

Proposal 4b: It’s suggested to introduce a multiplication factor for parameter of replicas, e.g., K as the configurable parameter for the length of CB-msg3 transmission window. The final value of the length of CB-msg3 transmission window is the result of [(the number of replicas) multiplied by the factor of K], that is:

Length of CB-msg3 transmission window = [(the number of replicas) * K] * (N)PUSCH resources periodicity

Proposal 4c: The number of K can be configured by the NW with the maximum value of 4.

Proposal 4d: It’s suggested to also define the periodicity of CB-msg3 transmission window as a value in units of (N)PUSCH resources periodicity.

R2-2506184	UL capacity enhancements for IoT NTN	Ericsson	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 5	The periodicity of (N)PUSCH resources for CB-Msg3 is not larger than H-SFN duration.

- On CB-Msg3-EDT completion

R2-2505369	Leftover issues on CB-Msg3-EDT	Google	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 1	Upon receiving a matching Contention Resolution Identity and a C-RNTI in a CB-Msg4, the UE does not consider the CB-Msg3-EDT procedure has ended and continues monitoring PDCCH for receiving an RRC message responding to the RRCEarlyDataRequest.

-	Ericsson thinks this is how it is currently described in the running CR

Upon receiving a matching Contention Resolution Identity and a C-RNTI in a CB-Msg4 without an RRC message, the UE considers UL transmission as successful but the UE does not consider the CB-Msg3-EDT procedure has ended and continues monitoring PDCCH.

Proposal 2	Upon receiving a CB-Msg4 including a matching Contention Resolution Identity without including an RRC message or a C-RNTI, the UE terminates the CB-Msg3-EDT procedure and keeps in RRC_IDLE.

Upon receiving a CB-Msg4 including a matching Contention Resolution Identity without including both an RRC message and a C-RNTI, the UE behaves as if it received an empty EarlyDataComplete message, terminates the CB-Msg3-EDT procedure and keeps in RRC_IDLE.

Proposal 3	The UE indicates to the network, whether it is able to terminate a CB-Msg3-EDT procedure upon receiving a CB-Msg4 containing no RRC message, as one of its capabilities.

Proposal 4	The UE can indicate in CB-Msg3, whether DL data following the UL data in CB-Msg3 is expected or not.

-	ZTE wonders if this is needed

-	IDC highlights that we specified the same behavior in PUR

-	QC thinks there are NAS mechanisms for this

Based on NW indication, it shall be possible for the UE to indicate during CB-Msg3-EDT procedure whether DL data following the UL data in CB-Msg3 is expected or not. RAI will be reused for this.

R2-2505917	On open issues for CB-Msg3-EDT	Samsung	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 3: RRC procedures need to capture CRID as complete response for the CB-Msg3 EDT in CP solution.

Proposal 4: For introducing enhancements from CB-Msg3 EDT to 4-step EDT, the only viable feature would be receiving CRID as complete response.

R2-2506157	Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete	Interdigital, Inc.	discussion	Rel-19	IoT_NTN_Ph3-Core

Observation 1: PUR feature can terminate the EDT procedure without using RRC, by using Layer 1 ACK or Timing advance MAC CE, if eNB is aware that there is no pending downlink data or signalling.

Proposal 1: RAN2 to discuss how eNB knows that there is no pending downlink data from the application layer.

Observation 2: With the PUR feature it is possible for the UE to indicate in PURConfigurationRequest whether it expects a downlink response by RRC.

Proposal 2: Introduce “rrc-ACK”, to RRCEarlyDataRequest to indicate whether an RRC Response Message is preferred by the UE for CB-Msg3, similar to PURConfigurationRequest

Proposal 3: Contention Resolution MAC CE includes a new flag indicating whether to terminate only the uplink transmissions, or to terminate the entire procedure and return to RRC_IDLE.

- Whether to support NW-indicated fallback

R2-2505632	Remaining issues on UL capacity enhancement for IoT NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-19	IoT_NTN_Ph3-Core

Observation 7: When the CB-Msg3 resource is overloaded, repeated CB-Msg3 transmission attempts may result in continuous failures. It would be beneficial for the network to fall back UEs to the 4-step RACH procedure to balance the load.

Proposal 12: RAN2 to discuss whether to support the network-indicated fallback to legacy RACH or EDT.

-	Ericsson wonders how the NW knows when to send this message so this might not be very useful

-	vivo also thinks the NW could rather change the resource configuration

-	HW thinks the NW could also set the BI for this. IDC is not sure this would solve the problem

-	Xiaomi thinks we could have separate access control for CB-msg3 EDT. Nokia thinks this would also be possible

-	Ericsson thinks there are already ways to control the load

We don’t introduce support for network-indicated fallback to legacy RACH or EDT.

Proposal 13: The reserved value for MAC subheader type indication can be used for NW-indicated fallback.

R2-2505537	CB-Msg3-EDT and Msg4 multicast	Qualcomm Incorporated	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 8:	If MAC subheader field T = 11 and there is indication that no UL grant is present, then no control element is present and the UE exits the CB-Msg3 EDT to fallback to either 4 step RACH or Rel-15 EDT.

Proposal 9: If MAC subheader field T = 11 and there is indication that UL grant is present, then control element includes the UL grant field, the location (time and frequency indicator) of transmitted Msg3 as in RAPID, timing advance and a temporary C-RNTI. Based on UL grant TBS, the UE decides whether the Msg3 includes UL data or not (i.e., update Msg3 or not).

- Some other proposals

R2-2505917	On open issues for CB-Msg3-EDT	Samsung	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 2: RAN2 to discuss options on TA report for CB-Msg3 EDT.

1.	TA report is not included for CB-Msg3-EDT – RAN2 clarifies the RRC procedures.

2.	TA report is included in the CB-Msg3-EDT according to the RRC procedures – RAN2 clarifies the ta-Report field description

3.	New configuration to configure TA report for CB-Msg3-EDT

R2-2506184	UL capacity enhancements for IoT NTN	Ericsson	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 2	The UE shall use the cell specific Koffset for the CB-Msg3-EDT procedure, regardless of if it has a UE specific Koffset or not.

Proposal 3	The MAC entity shall discard any Differential Koffset at the start of a CB-Msg3-EDT procedure.

R2-2505106	Discussion on uplink capacity enhancements for IoT NTN	Xiaomi	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 5: RAN2 to discuss whether to have separate access control for CB-msg3 EDT.

[AT131][302][R19 IoT NTN] UL enhancements (Mediatek)

Scope: continue the discussion on the CB-RNTI formula and on the details of the power ramping.

Intended outcome: summary of the offline discussion (in R2-2506278)

Offline time: Wednesday 12:00-12:30 in BO3

Deadline for offline discussion summary:  Thursday 2025-08-28 08:00

Scope: continue the discussion on selected proposals for discussion in 8.9.3 that could not be treated online

Intended outcome: summary of the offline discussion (in R2-2506292)

Offline time: Thursday 2025-08-28 15:30:16:00 in BO3

Deadline for offline discussion summary:  Friday 2025-08-29 08:00

R2-2506278	Report of [AT131][302][R19 IoT NTN] UL enhancements	Mediatek	discussion	IoT_NTN_Ph3-Core

CB-RNTI

Proposal 1: RAN2 to select one of below options to calculate CB-RNTI

•	Option 1

o	For eMTC

	floor (start SFN_id of Tx window/2) + 2401

o	For NB-IoT

	floor (start SFN_id of Tx window/4) + 256*carrier_id + 4097

	carrier_id is the index of the UL carrier associated with the selected UL grants (0 <= carrier_id < 16). The carrier_id of the anchor carrier is 0.

o	Note: To ensure 20ms (for eMTC) and 40ms (for NB-IoT) is the minimum value in transmission window periodicity configuration.

•	Option 5

o	For eMTC

	floor (start SFN_id of Tx window/WP) modulo (32) + 32*CE_level + 2401

	the CE_level is the selected enhanced coverage level (0 <= CE_level < 2)

o	For NB-IoT

	floor (start SFN_id of Tx window/WP) modulo (32) + 32*CE_level + 96*carrier_id + 4097

	the CE_level is the selected enhanced coverage level (0 <= CE_level < 3)

	the carrier_id is the index of the UL carrier associated with the selected UL grants (0 <= carrier_id < 16). The carrier_id of the anchor carrier is 0.

o	WP is the configured CB-MSG3 transmission window periodicity in unit of 10ms.

-	Ericsson wonders whether we can remove CE_level or make it configurable

-	QC thinks CE level is needed

-	vivo thinks that CE level is not needed

We go for option 5

Power Ramping

Proposal 2: RAN2 to define two new RRC parameters for CB-Msg3 power ramping (i.e., powerRampingStep and cb-Msg3-InitialReceivedTargetPower), with the same value ranges as those defined for legacy Msg3 power ramping.

Agreed

Proposal 3: The UE applies power ramping when the CB-Msg3ResponseTimer has expired and the UE proceeds to the next CB-msg3 transmission, by reusing the CB_MSG3_TRANSMISSION_COUNTER_CE as defined in the MAC running CR.

Agreed

Proposal 4: The CB-Msg3 received target power can be calculated as below:

•	the CB-MSG3_RECEIVED_TARGET_POWER is set to cb-Msg3-InitialReceivedTargetPower + (CB_MSG3_TRANSMISSION_COUNTER_CE – 1) * powerRampingStep;

Agreed

We inform RAN1 that RAN2 agreed to support power ramping for both NB-IoT and eMTC copying RAN2 agreements asking RAN1 to check the specification impact for RAN1 specs

Draft an LS to RAN1 in R2-2506284

R2-2506292	Report of [AT131][302][R19 IoT NTN] UL enhancements – second round	Mediatek	discussion	IoT_NTN_Ph3-Core

- Potential easy

Proposal 1: RAN2 confirms that both multi-PRB allocation and sub-PRB allocation are supported for CB-Msg3.

Agreed

Proposal 2: RAN2 confirms that both single-tone and multi-tone are supported for CB-Msg3, and intends to reuse the parameter npusch-MCS-r16 for CB-Msg3.

Agreed

Proposal 3: RAN2 sends an LS to RAN1 to inform them of Proposal 1 and Proposal 2.

Agreed

Proposal 6: The UE shall use the cell specific Koffset for the CB-Msg3-EDT procedure, regardless of if it has a UE specific Koffset or not.

Agreed (no spec impact)

- Need Further discussion:

Proposal 4: CB-Msg3 transmission window length [and periodicity] is configured in multiples of the CB-Msg3 PUSCH periodicity.

Continue the discussion in the post email discussion

Proposal 5: RAN2 to discuss whether to configure the MPDCCH narrowband as a set or a single value. Including R2 conclusion in the LS to RAN1.

We configure 2 MPDCCH narrowbands as a set (inform RAN1)

Proposal 7: For TA report in CB-Msg3 EDT, RAN2 to select one of below options.

Continue the discussion in the post email discussion

Option 1 - TA report is not included for CB-Msg3-EDT – RAN2 clarifies the RRC procedures. [4/8]

Option 2 - TA report is included in the CB-Msg3-EDT according to the RRC procedures – RAN2 clarifies the ta-Report field description. [4/8]

- Missing proposal during offline:

Proposal 8: The periodicity of (N)PUSCH resources for CB-Msg3 is not larger than H-SFN duration.

Agreed

Agreements:

1.	We don’t further work on the inclusion of OCC support for CB-Msg3-EDT as part of IoT_NTN_Ph3-Core in Rel19.

2.	We try to define a simple power ramping scheme reusing existing mechanisms and then we inform RAN1 inviting them to respond if they find a problem

3.	When the Msg4 monitoring window starts at the end of CB-Msg3-EDT transmission window plus UE-eNB RTT, there is no need to consider additional delays for NW/UE processing time.

4.	We leave to UE implementation to decide in which order the conditions to trigger the different procedures are met.

5.	When the RRC layer receives a CB-Msg3 EDT failure indication from the MAC layer the RRC procedure is re-initiated (how to avoid data loss is up to UE implementation). Which procedure (e.g. EDT, 4-step RACH, CB-Msg3-EDT) is initiated is up to UE implementation

6.	We don’t support multiple TBSs per CE level for CB-Msg3 EDT

7.	We introduce one additional value smaller than the smallest possible TBS size at the moment (328bits), i.e. 144bits (discuss further details when drafting the LS to RAN1).

8.	We clarify that CQI reporting is not included in CB-Msg3 for CB-Msg3-EDT for non-anchor carrier but it can be included for anchor carrier (send LS to RAN4)

9.	Upon receiving a matching Contention Resolution Identity and a C-RNTI in a CB-Msg4 without an RRC message, the UE considers UL transmission as successful but the UE does not consider the CB-Msg3-EDT procedure has ended and continues monitoring PDCCH.

10.	Upon receiving a CB-Msg4 including a matching Contention Resolution Identity without including both an RRC message and a C-RNTI, the UE behaves as if it received an empty EarlyDataComplete message, terminates the CB-Msg3-EDT procedure and keeps in RRC_IDLE.

11.	Based on NW indication, it shall be possible for the UE to indicate during CB-Msg3-EDT procedure whether DL data following the UL data in CB-Msg3 is expected or not. RAI will be reused for this.

12.	We don’t introduce support for network-indicated fallback to legacy RACH or EDT

13.	CB-RNTI formula:

For eMTC

-	floor (start SFN_id of Tx window/WP) modulo (32) + 32*CE_level + 2401

-	the CE_level is the selected enhanced coverage level (0 <= CE_level < 2)

For NB-IoT

-	floor (start SFN_id of Tx window/WP) modulo (32) + 32*CE_level + 96*carrier_id + 4097

-	the CE_level is the selected enhanced coverage level (0 <= CE_level < 3)

-	the carrier_id is the index of the UL carrier associated with the selected UL grants (0 <= carrier_id < 16). The carrier_id of the anchor carrier is 0.

WP is the configured CB-MSG3 transmission window periodicity in unit of 10ms.

14. RAN2 to define two new RRC parameters for CB-Msg3 power ramping (i.e., powerRampingStep and cb-Msg3-InitialReceivedTargetPower), with the same value ranges as those defined for legacy Msg3 power ramping.

15. The UE applies power ramping when the CB-Msg3ResponseTimer has expired and the UE proceeds to the next CB-msg3 transmission, by reusing the CB_MSG3_TRANSMISSION_COUNTER_CE as defined in the MAC running CR.

16.	The CB-Msg3 received target power can be calculated as below:

the CB-MSG3_RECEIVED_TARGET_POWER is set to cb-Msg3-InitialReceivedTargetPower + (CB_MSG3_TRANSMISSION_COUNTER_CE – 1) * powerRampingStep;

17.	RAN2 confirms that both multi-PRB allocation and sub-PRB allocation are supported for CB-Msg3 (inform RAN1)

18.	RAN2 confirms that both single-tone and multi-tone are supported for CB-Msg3, and intends to reuse the parameter npusch-MCS-r16 for CB-Msg3 (inform RAN1)

19. The UE shall use the cell specific Koffset for the CB-Msg3-EDT procedure, regardless of if it has a UE specific Koffset or not (not spec impact)

20.	We configure 2 MPDCCH narrowbands as a set (inform RAN1)

21. The periodicity of (N)PUSCH resources for CB-Msg3 is not larger than H-SFN duration

R2-2506284	LS on power ramping and RRC configuration for CB-Msg3-EDT	Nokia	LS out	Rel-19	IoT_NTN_Ph3-Core To:RAN1

To be discussed in [Post131][319]

[Post131][319][R19 IoT NTN] LS on CB-Msg3-EDT (Nokia)

Scope: Draft the LS on additional agreements for CB-Msg3-EDT

Intended outcome: Approved LS (in R2-2506284)

Deadline: short

=> Approved in R2-2506284 (LS out)

R2-2506299	LS on CQI reporting for CB-Msg3-EDT	Samsung	LS out	Rel-19	IoT_NTN_Ph3-Core To:RAN4	Cc:RAN1

To be discussed in [Post131][320]

[Post131][320][R19 IoT NTN] LS on CQI reporting (Samsung)

Scope: Draft the LS on CQI reporting for CB-Msg3-EDT

Intended outcome: Approved LS (in R2-2506299)

Deadline: short

=> Approved in R2-2506299 (LS out)

R2-2505082	Remaining Issues on CB-Msg3 EDT Mechanism	vivo	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2505106	Discussion on uplink capacity enhancements for IoT NTN	Xiaomi	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2505179	Discussion on uplink capacity enhancement	Transsion Holdings	discussion	Rel-19

R2-2505231	Discussion on open issues for CB-Msg3 EDT	CATT	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2505258	Remaining issues for CB-msg3-EDT	ZTE Corporation, Sanechips	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2505369	Leftover issues on CB-Msg3-EDT	Google	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2505493	Remaining issues in CB-Msg3	Apple	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2505536	Support of OCC	Qualcomm Incorporated, European Space Agency, German Aerospace Center	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2505537	CB-Msg3-EDT and Msg4 multicast	Qualcomm Incorporated	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2505551	Discussion on CB-Msg3 EDT and Msg4 enhancement	OPPO	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2505571	Discussion on CB-Msg3 procedure	MediaTek Inc.	discussion	IoT_NTN_Ph3-Core	R2-2504528

R2-2505632	Remaining issues on UL capacity enhancement for IoT NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2505691	EDT for uplink capacity enhancement in NTN	Lenovo	discussion	Rel-19

R2-2505736	Further consideration on UL capacity enhancement	Huawei, HiSilicon	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2505917	On open issues for CB-Msg3-EDT	Samsung	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2505958	Discussion on remaining issues of uplink capacity enhancement for IoT-NTN	CMCC	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2506157	Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete	Interdigital, Inc.	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2506184	UL capacity enhancements for IoT NTN	Ericsson	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2506168	Support of OCC with CB UL in IoT NTN	Aalyria, Qualcomm Incorporated	discussion	Rel-19

Withdrawn

### 8.9.4	Support of PWS

Contributions should focus on remaining open issues related to the introduction of support for broadcast of PWS messages for NB-IoT, re-using the LTE mechanisms.

R2-2505563	Remaining issues on PWS support for NB-IoT	Huawei, HiSilicon, China Telecom	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 1: (RRC-3) A NB-IoT NTN UE is allowed to assemble PWS segments from different cells within an eNB based on a NW indication.

Proposal 2: An eMTC NTN UE is allowed to assemble PWS segments from different cells within an eNB based on a NW indication.

Proposal 3: (RRC-2) The NW can choose to send the scheduling information of PWS SIBs in advance and indicate whether the PWS SIBs are being broadcast or not similar to NR on-demand SI si-BroadcastStatus indication.

-	QC supports this

-	ZTE is not sure how the network would schedule information in this case. Ericsson/Nokia agreed.

We don’t pursue this enhancement as part of this WI

R2-2505633	On support of inter-cell PWS reception for NB-IoT NTN	Nokia, Nokia Shanghai Bell, Google, Huawei	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 1: RAN2 to confirm support for continued reception of PWS message segments from different cells provided by the same eNB (inter-cell, intra-eNB case).

-	MTK thinks this is not needed. QC agrees and thinks the UE would miss segments during the interruption

-	Oppo thinks this would introduce a new principle of transmitting system info of one cell from other cells.

-	Xiaomi and vivo also don’t support this

-	HW thinks that without this we are not really supporting PWS

-	Ericsson also thinks this is essential

Proposal 2: Introduce a new indication in SIB for inter-cell PWS segmentation reception.

Observation 3: The UE must be able to determine the eNB identity of the current cell to facilitate inter-cell intra-eNB PWS segmentation reception.

Observation 4: The TS 38.331 specification includes gNB-ID-Length that defines which bits in CellIdentity provide the gNB ID.

Proposal 3: Specify eNB-ID-Length in SIB31-NB to indicate the number of bits used for providing the eNB ID in the CellIdentity.

Proposal 4: RAN2 to discuss the TP to support continued reception of PWS message segments in inter-cell intra-eNB scenario.

We don’t support for continued reception of PWS message segments from different cells provided by the same eNB (inter-cell, intra-eNB case) as part of this WI.

R2-2505552	Discussion on PWS for NB-IoT	OPPO	discussion	Rel-19	IoT_NTN_Ph3-Core

•	Option 1: Introduce an acceptable cell category for NB-IoT

•	Option 2: Only simply specify that “a PWS capable NB-IoT UE may camp on an NB-IoT cell for PWS reception, without requiring the cell to be a part of UEs PLMNs”

Proposal 1	Introduce an acceptable cell category for NB-IoT (at least for NTN).

-	Samsung supports option 1

-	regarding the UE capability support Xiaomi/Oppo think this can be linked to the capability to support PWS. QC thinks it could be a separate capability

Introduce an acceptable cell category for NB-IoT (at least for NTN) (FFS if a separate capability is needed for this)

Agreements:

1.	We don’t pursue the enhancement whereby the NW can choose to send the scheduling information of PWS SIBs in advance and indicate whether the PWS SIBs are being broadcast or not as part of this WI

2.	We don’t support for continued reception of PWS message segments from different cells provided by the same eNB (inter-cell, intra-eNB case) as part of this WI.

3.	Introduce an acceptable cell category for NB-IoT (at least for NTN) (FFS if a separate capability is needed for this)

R2-2505083	Remaining Issues on PWS Support for NB-IoT	vivo	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2505107	PWS support for NB-IoT over NTN	Xiaomi	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2505259	Remaining issues for PWS support	ZTE Corporation, Sanechips	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2505538	Discussion on PWS in NB-IoT NTN	Qualcomm Incorporated	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2505568	Remaining open issues of PWS for NB-IoT	MediaTek Inc.	discussion	IoT_NTN_Ph3-Core

R2-2505692	Further considerations on PWS broadcast support in IoT NTN	Lenovo	discussion	Rel-19

R2-2505824	Enhancements to support PWS in NB-IoT NTN	Ericsson	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2505918	Open issues on PWS for NB-IoT NTN	Samsung	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2505959	Remaining issues on support of PWS	CMCC	discussion	Rel-19	IoT_NTN_Ph3-Core

## 8.10	SON/MDT Ph4

(NR_ENDC_SON_MDT_Ph4-Core; leading WG: RAN3; REL-19; WID: RP-234038)

Time budget: 0.5 TU

Tdoc Limitation: 2 tdocs

### 8.10.1	Organizational

LS, Rapporteur input, including workplan, etc.

LSs

R2-2505038	LS on support of MRO for S-CPAC (R3-253886; contact: Samsung)	RAN3	LS in	Rel-19	NR_ENDC_SON_MDT_Ph4-Core	To:RAN2

“RAN3 therefore asks RAN2 to consider if the description of previousPSCellId and timeSCGFailure in SCGFailureInformation needs to be updated to cover the S-CPAC scenario”

-	Samsung highlights that this is already considered in the running CR

Noted

R2-2505041	Reply to LS on SON for LTM (R3-253944; contact: Samsung)	RAN3	LS in	Rel-19	NR_ENDC_SON_MDT_Ph4-Core	To:RAN2

Noted

Running CRs, etc.

R2-2505206	Summary of [Post130][603][SONMDT] Running capability CR (CATT)	CATT	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: an optional UE capability with signalling is defined for RLF report for MCG LTM.

Proposal 2: an optional UE capability without signalling is defined for SHR/RA report for MCG LTM.

Proposal 3: an optional UE capability with signalling is defined for RLF report for CHO with candidate SCG.

Proposal 4: an optional UE capability without signalling is defined for SHR/SPR/SCGFailureInformation for CHO with candidate SCG.

Proposal 5: an optional UE capability without signalling is defined for RLF report for time/location based CHO.

Proposal 6: an optional UE capability with signalling is defined for geographic area scope checking for logged MDT.

Proposal 7: an optional UE capability without signalling is defined for RA-report for SDT.

Proposal 8: an optional UE capability without signalling is defined for MHI Enhancement for SCG Deactivation/Activation.

Proposal 9: an optional UE capability without signalling is defined for MRO for EN-DC SCG failure in LTE specification.

Proposal 10: A UE which supports RLF-Report for conditional handover with time-based and location-based trigger conditions also supports RLF report for conditional handover.

Proposal 11: A UE which supports RLF-Report for conditional handover with candidate SCG also supports RLF report for conditional handover.

Discussion:

P1 and P3:

-	Qualcomm wonders why UE should report this capa? CATT think this is the same as RLF for DAPS and CHO, so they want to align. Samsung think we should have the indications.

Agreements - An optional UE capability with signalling is defined for RLF report for MCG LTM. - an optional UE capability without signalling is defined for SHR/RA report for MCG LTM. - an optional UE capability with signalling is defined for RLF report for CHO with candidate SCG. - an optional UE capability without signalling is defined for SHR/SPR/SCGFailureInformation for CHO with candidate SCG. - an optional UE capability without signalling is defined for RLF report for time/location based CHO. - an optional UE capability with signalling is defined for geographic area scope checking for logged MDT. - an optional UE capability without signalling is defined for RA-report for SDT. - an optional UE capability without signalling is defined for MHI Enhancement for SCG Deactivation/Activation. - an optional UE capability without signalling is defined for MRO for EN-DC SCG failure in LTE specification. - A UE which supports RLF-Report for conditional handover with time-based and location-based trigger conditions also supports RLF report for conditional handover. - A UE which supports RLF-Report for conditional handover with candidate SCG also supports RLF report for conditional handover.

R2-2505207	Introduction of SONMDT UE Capabilities	CATT	draftCR	Rel-19	38.331	18.6.0	B	NR_ENDC_SON_MDT_Ph4-Core

Endorsed and to be updated

R2-2505208	Introduction of SONMDT UE Capabilities	CATT	draftCR	Rel-19	38.306	18.6.0	B	NR_ENDC_SON_MDT_Ph4-Core

Endorsed and to be updated

R2-2505209	Introduction of SONMDT UE Capabilities	CATT	CR	Rel-19	36.306	18.5.0	1915	-	B	NR_ENDC_SON_MDT_Ph4-Core

Agreed

[Post131][601][SONMDT] SONMDT capability CRs (CATT)

Scope:

    - Produce agreeable CRs

Intended outcome:

    - Agreed CR(s) in R2-2506377 and R2-2506378

Deadline:

    - Short

=> Endorsed in:

- R2-2506377 (38.331)

- R2-2506378 (38.306)

R2-2506377	Introduction of SONMDT UE Capabilities	CATT	draftCR	Rel-19	38.331	18.6.0	B	NR_ENDC_SON_MDT_Ph4-Core

=> Endorsed

R2-2506378	Introduction of SONMDT UE Capabilities	CATT	draftCR	Rel-19	38.306	18.6.0	B	NR_ENDC_SON_MDT_Ph4-Core

=> Endorsed

R2-2506402	Introduction of SONMDT features	Ericsson, ZTE	CR	Rel-19	38.331	18.6.0	5446	1	B	NR_ENDC_SON_MDT_Ph4-Core

Endorsed and to be updated

[Post131][602][SONMDT] SONMDT NR RRC CR (Ericsson)

Scope:

    - Produce agreeable CRs

Intended outcome:

    - Agreed CR(s) in R2-2506376

Deadline:

    - Short

=> Agreed in R2-2506612 (38.331)

R2-2506612	Introduction of SONMDT features	Ericsson, ZTE	CR	Rel-19	38.331	18.6.0	5446	4	B	NR_ENDC_SON_MDT_Ph4-Core

=> Agreed

R2-2505833	Open Issues for NR RRC SONMDT features	Ericsson, ZTE	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1	[Open issue RRC-1] For correlation of SHR and SPR generated for a CHO with Candidate SCG, RAN2 discuss and agree to one of the following options, Companies supporting each option provide a text proposal illustrating the implementation.

a.	Correlation indication in RRC_Complete message (a flag beside availability indication)

b.	Correlation indication inside the SHR and SPR

c.	Logging C-RNTI of PCell and timeSinceSPR in SPR

d.	The duplicated information in correlated SHR and SPR is sufficient. no further information is needed for correlation.

Proposal 2	[Open issue RRC-2] For the scenario of CHO with candidate SCG with complementary CHO only configuration, companies are invited to submit contribution with a text proposal addressing the issue and their proposed solution.

Proposal 3	[Open issue RRC-3] concerning logging highest ranked NSAG info associated with cell, freq etc.) companies are invited to submit their contribution addressing this issue with a text proposal reflecting their solution.

Proposal 4	[Open issue RRC-4] If the UE is unable to obtain its location, the UE follows legacy procedure as in logged MDT in terrestrial network.

Proposal 5	[Open issue RRC-5] Granularity of distanceFromReference1, distanceFromReference2 is in steps of 50m, the actual distance will be rounded down to the nearest step value. The maximum value is 65535, meaning 65535 or any larger distance.

Proposal 6	[Open issue RRC-6] RAN2 agree that RAN3 based solution in logging LTM candidate cells can be applicable/used in both RLF and SHR scenarios. RAN2 remove logging LTM candidate cell flag from SHR procedure.

Proposal 7	[Open issue RRC-7] If time allows RAN2 discuss duplicated CHO with Candidate SCG information (ChoWithCandidateSCGInfo) in the RLF and SCGFailureInformation.

Proposal 8	[Open issue RRC-8] If time allows RAN2 discuss logging traffic information in MHI.

Proposal 9	[Open issue RRC-9] If time allows RAN2 discuss logging CAPC execution conditions in RLF-report e.g., in a cpacConfig IE (similar to choConfig IE).

-	Ericsson suggests agreeing the easily agreeable. Take the rest based on papers

Discussion:

P4:

-	Qualcomm think this is pending RAN3 discussion, they think that if the UE cannot get its position the UE should stop. Ericsson think that the legacy behaviour is that the UE checks the PLMN and this should be fine without UE knowing its location, this is how the running CR is written. Nokia is OK with the proposal

Agreements - Granularity of distanceFromReference1, distanceFromReference2 is in steps of 50m, the actual distance will be rounded down to the nearest step value. The maximum value is 65535, meaning 65535 or any larger distance. - The RAN3 based solution in logging LTM candidate cells can be applicable/used in both RLF and SHR scenarios. RAN2 remove logging LTM candidate cell flag from SHR procedure.

[AT131][607][SONMDT] Area config without having location (Ericsson)

Scope:

    - Discuss offline an agreeable way forward regarding UE not knowing its location

Intended outcome:

    - Agreeable way forward which can be implemented in the running CR.

Deadline:

    - Wednesday 17:00. The intention is to agree over email.

-	Ericsson reports after the offline that the following is agreeable.

UE suspends logging, if it cannot obtain location information.

R2-2506081	Introduction of R19 SONMDT features in TS 36.331	Huawei, HiSilicon	CR	Rel-19	36.331	18.6.0	5150	-	B	NR_ENDC_SON_MDT_Ph4-Core

Endorsed and to be updated

[Post131][603][SONMDT] SONMDT LTE RRC CR (Huawei)

Scope:

    - Produce agreeable CRs

Intended outcome:

    - Agreed CR(s) in R2-2506379

Deadline:

    - Short

=> Agreed in R2-2506564 (36.331)

R2-2506564	Introduction of R19 SONMDT features in TS 36.331	Huawei, HiSilicon	CR	Rel-19	36.331	18.6.0	5150	2	B	NR_ENDC_SON_MDT_Ph4-Core

=> Agreed

Withdrawn/old revisions

R2-2505832	Introduction of SONMDT features	Ericsson, ZTE	CR	Rel-19	38.331	18.6.0	5446	-	B	NR_ENDC_SON_MDT_Ph4-Core

### 8.10.2	MRO enhancements for Rel-18 mobility features

LTM has 1st priority. CHO with candidate SCGs has 2nd priority

Subsequent CPAC is paused until if/when we get a RAN3 LS on the subject

RRC-1

R2-2505210	MRO Enhancements for CHO with Candidate SCGs	CATT	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: (RRC-1) Correlation of the SHR and SPR is needed in case of both reports being generated in one CHO with candidate SCGs execution.

Proposal 2: (RRC-1) To enable the correlation between SHR and SPR reports, enhance SPR to include the C-RNTI assigned by the target PCell of the CHO with candidate SCGs and timeSinceSPR (time elapsed since the execution of the CHO with candidate SCGs to the retrieval of the SPR).

Proposal 3: (RRC-1) To enable the correlation between SHR and SPR reports, enhance timeSinceSHR in SHR to support CHO with candidate SCGs case.

Proposal 4: (RRC-1) Adopt the TP in Annex if P2 and P3 are agreed.

R2-2505295	[Open issue RRC-1] Correlation of SHR and SPR	vivo	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: [Open issue RRC-1] The network can correlate SHR and SPR generated for the same CHO with Candidate SCG based on the duplicated information in correlated SHR and SPR by implementation.

Discussion:

-	LG have another proposal of how to correlate, but if majority prefers they are OK with the CATT proposal.

-	Nokia think the CRNTI requires the NW to look through all the report, so on top of the CRNTI they want a flag inside the UE report.

-	Huawei and ZTE only wants the C-RNTI. Ericsson think the C-RNTI is already used to correlate other reports so want to use that also here. But Ericsson don’t want the timing info.

-	Samsung support vivo. Qualcomm also think vivo approach is good enough and there is a lot of info in the report which the NW can use to correlate.

-	vivo think the C-RNTI approach has flaws since the NW might reuse a C-RNTI for other UEs.

-	Qualcomm thinks the NW should know which UE it got the reports from. CATT thinks the NW might just receive one of the reports.

Agreements - We add the C-RNTI for the PCell as a correlation indication for SHR and SPR reports. UE adds this unconditionally, and no other correlation info will be added.

RRC-2

R2-2505940	MRO enhancements for CHO with candidate SCGs	CMCC, Huawei, HiSilicon, CATT, Ericsson	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: UE reports the information of fulfilled condition(s), i.e., CHO, or CPAC, or neither, when CHO only configuration is received, in RLF report and/or SHR report.

Proposal 2: UE reports following information in RLF report and SHR report:

－	Identifier of candidate PCell(s) which met the CHO execution conditions when CHO only configuration is received

－	Identifier of candidate PSCell(s) which met the CPAC execution conditions when CHO only configuration is received

R2-2506142	Discussion on Rel-19 SONMDT open issues	Xiaomi	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 2: (RRC-2) RAN2 do not need to do optimization for the scenario of CHO with candidate SCG with complementary CHO only configuration.

Proposal 3: (RRC-2) RAN2 needs to consider to extend firstFulfilledConfig IE in 38.331 running CR to support “neither” in case of both CHO and CPAC conditions are not fulfilled.

Discussion:

-	vivo suggested some alternative solution. Huawei thinks its late in the release to consider new scenarios, and only want to consider the proposal in the CMCC paper now.

-	LG supports Xiaomis proposal as we shouldn’t optimize at this point. Xiaomi think we shouldn’t do any optimization since its late now.

-	Nokia think we should only consider the scenario when the NW sends both CHO with CPAC and the CHO-only at the same time. Ericsson thinks the scenario CMCC has brought up is valid and its important to understand if a CHO-only config is useful or not and if not useful the NW can avoid wasting resources on configuring it.

Agreements - UE reports the information of fulfilled condition(s), i.e., CHO, or CPAC, or neither, when CHO only configuration is received, in RLF report and/or SHR report. - UE reports following information in RLF report and SHR report: － Identifier of candidate PCell(s) which met the CHO execution conditions when CHO only configuration is received － Identifier of candidate PSCell(s) which met the CPAC execution conditions when CHO only configuration is received This is an optional UE feature, TBD with or without signalling

RRC-9 and T310

R2-2505724	MRO enhancements for Rel-18 mobility features (RRC-1, RRC-2, RRC-9)	Nokia	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 3.1: CHO configuration should also be logged in case of SCG failure, i.e. choConfig should be also included in the SCGFailureInformation message.

Proposal 3.2: RAN2 agrees to log CPAC configuration and dual event information in RLF report and SCGFailureInformation via additional new IEs similar to ones used for CHO.

Proposal 4: RAN2 to agree that there is a need to distinguish if the T310 started before or after fulfilment of the first execution conditions for CHO with Candidate SCG(s) and an indication about this in SHR and SPR.

Noted

Discussion:

P3.1:

-	LG do not support the proposal, the use of the info is limited since its unlikely that the SN is the issue for a failed CHO. Nokia think a CHO-only config is not mandatory.

Misc

R2-2505211	MRO Enhancements for LTM	CATT	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: MRO enhancement for SCG LTM is not considered in Rel-19 SON/MDT.

Proposal 2: The legacy SHR trigger conditions based on T310 threshold and T304 threshold can be applied to LTM.

Proposal 3: Include L3 measurement results of serving cell, target cell and other LTM candidate cell(s) in SHR for LTM MRO.

Proposal 4: A new raPurpose in RACH report is introduced to indicate the network-based early TA acquisition.

Proposal 5: RAN2 to discuss how to log the RACH information of network-based early TA acquisition to avoid the impact on the legacy RACH information logging (e.g., RACH information for handover):

-	Option 1: one entry in RACH report to log the RACH information of all network-based early TA acquisition before one LTM execution;

-	Option 2: one entry in RACH report to log the RACH information of all network-based early TA acquisition before LTM configuration release;

-	Option 3: log the RACH information of network-based early TA acquisition per candidate cell in one or multiple entries.

Proposal 6: To address the issue of frequent overriding RLF report/SHR for subsequent LTM, recording multiple reports can be considered as a candidate solution.

Proposal 7: Indication information is needed in RLF report/SHR to indicate whether it’s L1 measurement based LTM or L3 measurement based LTM.

Discussion:

P4/P5:

-	AsusTek prefers Option 3. Ericsson wonders what the purpose is. ZTE also do not understand the reason for logging the Early UL sync RAs. Qualcomm agree with ZTE and Ericsson. Ericsson think the UE shall not need to log anything w.r.t. early UL sync RAs.

P6:

-	Qualcomm does not agree and think the problem is not necessarily to solve. Samsung agrees.

P7:

-	Qualcomm think the UE should only log L1 measurements for L1 LTM. ZTE thinks the info will be known implicitly.

Agreements - MRO enhancement for SCG LTM is not considered in Rel-19 SON/MDT. - The legacy SHR trigger conditions based on T310 threshold and T304 threshold can be applied to LTM. - Include L3 measurement results of serving cell, target cell and other LTM candidate cell(s) in SHR for LTM MRO. - UE does not log information regarding RA procedures performed for the purpose of early UL sync. - UE includes L1 measurements only for L1 based LTM, as in current running CR.

choWithCandidateSCGInfoList

R2-2505754	Correlation Indication and other issues of CHO with Candidate SCG(s) 	Samsung	discussion

Proposal 2: Update the RRC Spec so that the pCellId and psCellId are logged in choWithCandidateSCGInfoList for SCGFailureInformation only for the candidates with fulfilled execution conditions as below.

1>	for each entry of condReconfigList in the MCG VarConditionalReconfig including both condExecutionCond and condExecutionCondPSCell,where all triggering events of any of condExecutionCond and condExecutionCondPSCell are fulfilled include an entry in choWithCandidateSCGInfoList and set the values as follows

Noted, if critical we can revisit this

RACH-less LTM

R2-2506041	Open issues on MRO enhancements for mobility	Qualcomm Incorporated	discussion	NR_ENDC_SON_MDT_Ph4-Core

Proposal 2	UE indicates to gNB in SHR if RACH-less LTM is attempted or not before LTM recovery success.

Proposal 3	UE indicates to gNB in RLF Report if RACH-less LTM is attempted or not when RACH-less LTM is not successful.¨

Discussion:

P2:

-	Samsung and Ericsson think its known implicitly.

Noted

previousPSCellId

R2-2506082	Discussion on MRO for mobility	Huawei, HiSilicon	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: It is proposed to add a clarification to the field description of previousPSCellId, i.e. it also indicates the cell that is the source PSCell associated to the last execution of RRCReconfiguration message including reconfigurationWithSync for the SCG.

Discussion:

P1:

-	Ericsson think we have discussed this field description many times and we should be careful not to break legacy behaviour.

Agreements - We intend to add a clarification to the field description of previousPSCellId, i.e. it also indicates the cell that is the source PSCell associated to the last execution of RRCReconfiguration message including reconfigurationWithSync for the SCG. This can be reconsidered if we find issues.

RRC-7

R2-2505631	Remaining open issues for MOB MRO	LG Electronics Inc.	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 5:	(RRC-7)	The UE is allowed to avoid duplicate CHO+CPAC logging across RLF report and SCG failure information.

Proposal 6:	(RRC-7)	Similar to Open issue RRC-1, to keep the procedures simple and efficient, add a NOTE to the relevant procedure text to suppress duplication logging, as described in the TP in Annex.

Noted

R2-2505685	Discussion on MRO enhancements for mobility	Lenovo	discussion	Rel-19

RA report enhancements for LTM

Proposal 1: RACH information for early TA acquisition can be included in RA report.

Proposal 2: One entry can be used to log both RACH information related with early TA acquisition and RACH information related with RACH based access (e.g. LTM failure recovery or RRC re-establishment, or RACH-based LTM for the case that network does not send the early TA value to the UE due to the early TA is outdated) in RA report.

R2-2505755	MRO for CHO with Candidate SCG(s) and CHO only configuration 	Samsung	discussion

Proposal 1: [Open issue RRC-2] Include choConfig and triggeredEvent for both CHO with candidate SCG(s) and CHO only configuration in RLF report.

R2-2505801	Discussion on open issues for MRO	ZTE Corporation, Sanechips	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

R2-2505834	MRO for CHO with candidate SCG	Ericsson	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

### 8.10.3	Other

RACH optimization for SDT focus on RSRP and data volume in SON reports, and existing failure causes.

MHI Enhancement for SCG Deactivation/Activation.

SON/MDT for Slicing

SON/MDT for NTN

RRC-3

R2-2505835	Discussion on SON-MDT enhancements for Slicing and NTN	Ericsson	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1	[Open issue RRC-3] RAN2 logs the cell reselection failure information in the next/first MDT sample logged after the detected cell reselection failure.

Proposal 2	[Open issue RRC-3] RAN2 include the following information along with NSAG information in the logged MDT report a) Cell information (if available, it may not be available if the UE goes to any cell selection state) b) location where UE fails to access a cell according to NSAG priority

Proposal 3	Set 8 as the Maximum number of geographical areas that the UE can be configured as implemented in the RRC running CR.

R2-2506083	Discussion on SONMDT for others	Huawei, HiSilicon	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal: (RRC-3) It is sufficient to reply on the following UE behavior (from the previous RAN2#130 agreement):

If the UE supports slice-based cell reselection does not find any suitable cell in the frequencies corresponding to the highest ranked NSAG, the UE logs the highest ranked NSAG.

And then the possible RAN2 impacts are:

(1) add one flag into LogMeasInfo-r16, i.e. NSAG-ID-r17 for the highest ranked NSAG information

(2) update the procedural text by adding this flag

Discussion:

P1/P2 (Ericsson):

-	Samsung thinks the Huawei proposal is simopler. ZTE think the Ericsson solution is more complete since it has cell info, but location info is out of scope. Nokia agrees with the Huawei approach but also want the NSAG ID. Ericsson think the cell info is needed to know where the UE were. Nokia thinks the UE reports neighbour cells and from this the operator can know which cell is the problematic cell. ZTE think there can be a time gap between the failure and the lack of NSAG, so we need to tell where the failure happened.

Agreements - RAN2 includes the following information along with NSAG information in the logged MDT report: Cell information of the cell that the UE was in when it was not able to find its highest prio NSAG (if available, it may not be available if the UE goes to any cell selection state) - Set 8 as the Maximum number of geographical areas that the UE can be configured as implemented in the RRC running CR.

RRC-8

R2-2505802	Discussion on other leftover issues	ZTE Corporation, Sanechips	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 2	(RRC-8) UE includes traffic characteristics (e.g., average data rate during activation period) in the MHI together with the SCG activation/deactivation information.

Discussion:

P2:

-	Qualcomm does not agree with this.

Noted

NTN

R2-2506042	Open issues on NTN SONMDT	Qualcomm Incorporated	discussion	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1	The legacy area scope should not be configured when Rel-19 area scope is configured.

-	Huawei supports P1. Ericsson think we can discuss this in the comeback to have a unified solution.

Agreements - RAN2 intends to specify that the legacy area scope cannot be configured when Rel-19 area scope is configured.

R2-2505725	SON/MDT enhancements for network slicing (RRC-3), MHI and NTN	Nokia	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 2: It is proposed to include additional assistance information related to the condition(s) for SCG activation/deactivation, in MHI such as DL and/or UL volume per second which was transmitted or received by UE during the SCG activation time for the given PSCell.

Proposal 3.1: RAN2 to consider enhancing RLF Report and SHR for DG based RACH-less access for NTN with the addition of the following parameters:

1)	PDCCH Monitoring Time Until Success

2)	No of PUSCH Attempts Until Success

Proposal 3.2: Define a new SHR trigger (based on PDCCH monitoring time and/or number of PUSCH attempts) for the UE to log the enhancements measurements specific to NTN RACH-less access.

Discussion:

P2:

-	Qualcomm thinks the NW is in control and the UE is not. Samsung agrees with QC.

P3.1:

-	Qualcomm does not support this since its hard to get L1 statistics. This has been discussed before also.

Noted

R2-2506143	Discussion on other aspects of Rel-19 SONMDT	Xiaomi	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: (RRC-4) If the UE is unable to obtain its location, the UE stops logging MDT for NTN.

## 8.11	Evolution of NR duplex operation: Sub-band full duplex (SBFD)

(NR_duplex_evo-Core; leading WG: RAN1; REL-19; WID: RP-251874)

Time budget: 0.5 TU

Tdoc Limitation: 2 tdocs

### 8.11.1	Organizational

Incoming LS, Rapporteur input, including workplan, running CRs, email discussion summary, open issue list(s), etc..

LSin

R2-2505015	Reply LS on simultaneous configuration of SBFD and DC (R1-2504858; contact: Xiaomi)	RAN1	LS in	Rel-19	NR_duplex_evo-Core	To:RAN2	Cc:RAN3, RAN4

Noted

R2-2505030	LS on TP for TS38.300 on Rel-19 SBFD (R1-2505081; contact: Huawei)	RAN1	LS in	Rel-19	NR_duplex_evo-Core	To:RAN2	Cc:RAN3

Noted

CRs

R2-2505088	Introduction of SBFD in TS 38300	CATT	CR	Rel-19	38.300	18.6.0	1008	-	B	NR_duplex_evo-Core	R2-2503422

R2-2505363	Introduction of Rel-19 Evolution of NR duplex operation (SBFD)	Huawei, HiSilicon	CR	Rel-19	38.331	18.6.0	5414	-	B	NR_duplex_evo-Core

R2-2505575	Introduction of Rel-19 Evolution of NR duplex operation (SBFD) for MAC spec	Samsung	CR	Rel-19	38.321	18.6.0	2106	-	B	NR_duplex_evo-Core

The above 3 CRs are endorsed, and they will be taken as baseline for further updates and review

R2-2505549	Introduction of Rel-19 Evolution of NR duplex operation (SBFD) for MAC spec	Samsung	CR	Rel-19	38.321	18.6.0	2105	-	B	NR_duplex_evo-Core	Withdrawn

Post meeting email discussions for the CRs

[Post131][219][SBFD] CR for TS 38.300 (CATT)

Intended outcome: Agree the CR for TS 38.300

Deadline:  Short

=> Agreed in R2-2506604 (38.300)

[Post131][220][SBFD] CR for TS 38.331 (Huawei)

Intended outcome: Agree the CR for TS 38.321

Deadline:  Short

=> Agreed in R2-2506605 (38.331)

[Post131][221][SBFD] CR for TS 38.321 (Samsung)

Intended outcome: Agree the CR for TS 38.321

Deadline:  Short

=> Agreed in R2-2506606 (38.321)

R2-2506605	Introduction of Rel-19 Evolution of NR duplex operation (SBFD)	CATT	CR	Rel-19	38.300	18.6.0	1008	1	B	NR_duplex_evo-Core

=> Agreed

R2-2506605	Introduction of Rel-19 Evolution of NR duplex operation (SBFD)	Huawei, HiSilicon	CR	Rel-19	38.331	18.6.0	5414	2	B	NR_duplex_evo-Core

=> Agreed

R2-2506606	Introduction of Rel-19 Evolution of NR duplex operation (SBFD) for MAC spec	Samsung	CR	Rel-19	38.321	18.6.0	2106	2	B	NR_duplex_evo-Core

=> Agreed

Email discussion summary

R2-2505364	Summary of [Post130][216][SBFD] Running CR for 38.331	Huawei, HiSilicon	discussion	Rel-19	NR_duplex_evo-Core

Noted

[Proposals for easy agreement]:

[Proposal for RRC-2] Not to support that a further different SSB RSRP threshold is indicated/configured for an SSB or a group of SSBs. [13/13]

[Proposal for RRC-3] Not to pursue the further optimization of parameter signalling of SBFD RACH configuration. [11/12]

[Proposal for RRC-6] (Only) support RACH-based LTM cell change in SBFD symbols [10/11]. Add RO type indication in LTM cell switch command MAC CE.

[Proposals for discussion]:

[Proposal for RRC-1] For the network indicating RO type, use 1 bit signalling (as in the current RRC running CR) [9/13].

Not to support that a further different SSB RSRP threshold is indicated/configured for an SSB or a group of SSBs.

Not to pursue the further optimization of parameter signalling of SBFD RACH configuration.

Discussions

RRC-6

-	Samsung object this proposal. Samsung think this should be discussed in LTM session.

-	InterDigital think there is majority so we can agree, think we can discuss in SBFD because it is co-existence with Rel-18 LTM. Nokia, LG E, ZTE, CATT agree.

-	ZTE think at least for intra-DU case we should be able to agree. QC agree with ZTE, think if we cannot conclude here it is not easy to agree in LTM session either. Ericsson, CATT agree.

-	Samsung also has concern because there is RAN3 impact, and it is the last meeting for the WI. ZTE think if we focus on intra-DU, then there is not additional R3 impact.

-	Samsung wonder if we agree which WI should handle the necessary MAC spec change.

Support co-existence of SBFD with intra-DU LTM. Whether to support the co-existence between SBFD and other LTM cases is not discussed in the Rel-19 SBFD WI.

Discussions

RRC-1

-	Nokia think we do not need the proposal, and think 1 bit is not sufficient to handle the load balancing well.

-	HW think we do not need to re-open because we already agree. Ericsson, CATT, Xiaomi, InterDigital, Sharp, ZTE, Samsung, LG E agree.

-	Sony think it is open issue, and agree with Nokia load balancing need to be considered.

-	CATT think other solutions are not discussed and there is no time for that.

-	Apple, Nokia think we either remove this 1 bit or we use 2 bits. HW think NW can configure the bit when necessary. ZTE agree with HW.

-	InterDigital think if we have more bits we need more time to discuss what is the UE behaviour.

-	Sony think RSRP based threshold is not sufficient because all UEs see the same threshold.

-	LG E want to avoid additional UE complexity.

-	Charter think we need to capture the agreement clearly.

For the network indicating RO type, use 1 bit signalling (as in the current RRC running CR)

R2-2505560	Summary of the SBFD open issues in MAC	Samsung	discussion	Rel-19	NR_duplex_evo-Core

Noted

[Proposals for easy agreement without contributions]

[Proposal 1 for MAC-1] In RO type switching, for the other RO type, UE can select the set of Random Access resources associated with the same feature or feature combination, and with higher Msg1 repetition number, if the set with the same Msg1 repetition number is not available.

If [Proposal 1 for MAC-1] is agreed:

[Proposal 2 for MAC-1] In RO type switching, when UE has to select a set of Random Access resources with higher Msg1 repetition number for the other RO type, if there are multiple sets with multiple higher Msg1 repetition numbers available, UE selects the set with next higher Msg1 repetition number.

Discussions

MAC-1

-	Nokia think UE does not have to go to higher repetition number, and think it can check the RSRP to determine which number is selected.

-	LG E, Ericsson, ZTE agree with the proposal.

-	ZTE additional RSRP checking might leave to other issues.

In RO type switching, for the other RO type, UE can select the set of Random Access resources associated with the same feature or feature combination, and with higher Msg1 repetition number, if the set with the same Msg1 repetition number is not available.

In RO type switching, when UE has to select a set of Random Access resources with higher Msg1 repetition number for the other RO type, if there are multiple sets with multiple higher Msg1 repetition numbers available, UE selects the set with next higher Msg1 repetition number.

Agreements on the remaining open issues

Not to support that a further different SSB RSRP threshold is indicated/configured for an SSB or a group of SSBs. Not to pursue the further optimization of parameter signalling of SBFD RACH configuration. Support co-existence of SBFD with intra-DU LTM. Whether to support the co-existence between SBFD and other LTM cases is not discussed in the Rel-19 SBFD WI. For the network indicating RO type, use 1 bit signalling (as in the current RRC running CR) In RO type switching, for the other RO type, UE can select the set of Random Access resources associated with the same feature or feature combination, and with higher Msg1 repetition number, if the set with the same Msg1 repetition number is not available. In RO type switching, when UE has to select a set of Random Access resources with higher Msg1 repetition number for the other RO type, if there are multiple sets with multiple higher Msg1 repetition numbers available, UE selects the set with next higher Msg1 repetition number.

Completion of the WI

From RAN2 point of view, the Rel-19 work item NR_duplex_evo-Core is completed.

### 8.11.2	Random access in SBFD

RAN2 impacts to support SBFD operation to support random access in SBFD symbols by UEs in RRC _CONNECTED mode and RRC_IDLE/INACTIVE mode.

Open issue MAC 2-1 and MAC 2-2

R2-2505365	Discussion on MAC open issues for random access in SBFD	Huawei, HiSilicon	discussion	Rel-19	NR_duplex_evo-Core

Noted

[Proposal for MAC-2-1 and MAC-2-2]: The condition of “when the RACH resources for the same RO type is provided for CBRA” is always satisfied, hence no need to make any additional MAC specification changes (from the current MAC running CR).

R2-2505590	Discussion on random access procedure in SBFD	ZTE Corporation	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 1: Support the case that CFRA indicates SBFD RO, but the corresponding common RACH resource on the same BWP does not indicate any SBFD RACH resource.

Proposal 2: If a CFRA case indicates to use SBFD RO, when selecting set for the CFRA, UE should:

	If common RACH resource of the same BWP provides SBFD ROs, UE selects set within SBFD RACH resources;

	If common RACH resource of the same BWP does not provide SBFD ROs, UE selects set within the legacy RACH resources.

Discussions

-	ZTE is ok to go with majority view.

-	OPPO wonder whether this proposal applies to both RACH config option 1 and option 2. ZTE and LG E think this is obvious.

RAN2 assume that when CFRA indicates SBFD RO, the RACH resources for the same RO type is provided for CBRA. FFS if any spec changes is needed.

Open issue MAC 3-1 and MAC 3-2

R2-2505459	Remaining issues on Random Access procedure for SBFD	LG Electronics Inc.	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 3. [MAC-3-1] No need to clarify on how to derive the SBFD RO locations for CFRA.

R2-2505559	Discussions on the open issues for Random Access	Samsung	discussion	Rel-19	NR_duplex_evo-Core

Noted

[MAC-3] Proposal 2: To resolve the ambiguity on SBFD RO location derivation for CFRA, RAN2 to consider one of the following two workable options, where Option 1 is more preferable from the spec impact perspective:

-	Option 1: Introduce a restriction that network can indicate RO type as SBFD RO for CFRA, only if an SBFD RACH configuration is configured for CBRA, and apply the same configuration for CFRA SBFD RO location derivation.

-	Option 2: Introduce new RRC signalling to indicate which SBFD RACH configuration should be applied for SBFD RO location derivation for CFRA.

Discussions

-	ZTE think that with the previous agreement we do not need to capture more agreements for MAC 3-1/3-2. Ericsson agree.

Open issue MAC 4-1 and MAC 4-2

R2-2505495	Open issues for RACH in SBFD	Apple	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 5: For RACH Configuration Option 2, all related parameters should be re-initialized as RAN1 intention is to have independent configurations for SBFD and non-SBFD RACH. For RACH Configuration Option 1, at least sbfd-RACHSingleConfig-preambleReceivedTargetPower is re-initialized.

R2-2505559	Discussions on the open issues for Random Access	Samsung	discussion	Rel-19	NR_duplex_evo-Core

Noted

[MAC-4] Proposal 3: Parameter initialization should be performed after RO type switching.

[MAC-4] Proposal 4: preambleTransMax should be excluded from the parameter initialization.

R2-2505141	Discussion on RACH in SBFD	Xiaomi	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 3: (MAC-4) Parameter initialization is needed after RO type switching in the case of RACH configuration Option 2. At least PREAMBLE_POWER_RAMPING_STEP and SCALING_FACTOR_BI are initialized. PREAMBLE_POWER_RAMPING_COUNTER should be excluded from the initialization.

Discussions

-	Sharp think it is reasonable to initialize the configured parameters, and agree with Xiaomi the UE variables should be excluded. Charter agree.

On preambleTransMax

-	ZTE agree with Samsung that this should be excluded.

-	Apple, OPPO, Nokia think it should be re-initialized after RO type switching.

-	ZTE and Ericsson have concern for the case when the ‘2nd’ preambleTransMax could be smaller than the ‘1st’ value. LG E think NW implementation can handle it.

UE variables

-	ZTE and Apple think the UE variables for power ramping are initialized.

For RACH configuration Option 2, all the RRC configured parameters are re-initialized after RO type switching.

For RACH Configuration Option 1, sbfd-RACHSingleConfig-preambleReceivedTargetPower is re-initialized after RO type switching.

Can discuss in the RRC CR review the configuration restriction (if needed) for preambleTransMax

For RACH configuration Option 2, PREAMBLE_POWER_RAMPING_STEP and SCALING_FACTOR_BI are re-initialized after RO type switching.

For both RACH configuration Option 1 and RACH configuration Option 2,  PREAMBLE_POWER_RAMPING_COUNTER is not re-initialized after RO type switching.

Can further check the other UE variables in the CR review.

Open issue MAC 5

R2-2505243	Power ramping issue on the RO type fallback	OPPO, ZTE Corporation, Sharp, NEC, NTT DOCOMO INC., Qualcomm Incorporated, CATT	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal: For the RO type fallback between legacy RO and additional RO, a power offset given by the difference between the two quantities of preamble power ramping steps is added.

R2-2505666	SBFD RA remaining aspects	Ericsson	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 4	Not support introducing a power offset to compensate the power ramping difference between the two RO types (in SBFD RACH configuration Option 2) for RO type switching.

Discussions

-	Samsung, Xiaomi, ZTE support OPPO proposal.

-	Xiaomi think there is similar mechanism for 2step and 4step fallback. DCM, CATT, Apple agree.

-	Nokia support Ericsson proposal, and think for SBFD it is obvious UE use different power for different RO types, which is different than the case of 2s/4s RACH.

[AT131][208][SBFD] Proposals for Power ramping issue on the RO type fallback (OPPO)

Intended outcome: Summary with proposals in R2-2506255

Deadline: before CB.

R2-2506255	Report of [AT131][208][SBFD] Proposals for Power ramping issue on the RO type fallback (OPPO)

Noted

Proposal: For the RO type fallback between legacy RO and additional RO for RACH configuration option 2, a power offset given by the difference between the two quantities of preamble power ramping steps is added.

For the RO type fallback between legacy RO and additional RO for RACH configuration option 2, a power offset given by the difference between the two values of preamble power ramping steps is added. Exact change to the MAC spec can be further discussed in the CR review.

Chair: other issues can be discussed in CB session if time allows

R2-2505459	Remaining issues on Random Access procedure for SBFD	LG Electronics Inc.	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 6. Only for RACH configuration Option 1, reuse the rsrp-ThresholdMsg1-RepetitionNum2/4/8 to determine Msg1 repetition number in SBFD RO if sbfd-RSRP-ThresholdMsg1-RepetitionNum2/4/8 is not configured.

Proposal 7. For RACH configuration Option 2, determine Msg1 repetition number only by sbfd-RSRP-ThresholdMsg1-RepetitionNum2/4/8 (i.e., not reuse rsrp-ThresholdMsg1-RepetitionNum2/4/8).

Only for RACH configuration Option 1, reuse the rsrp-ThresholdMsg1-RepetitionNum2/4/8 to determine Msg1 repetition number in SBFD RO if sbfd-RSRP-ThresholdMsg1-RepetitionNum2/4/8 is not configured.

For RACH configuration Option 2, determine Msg1 repetition number only by sbfd-RSRP-ThresholdMsg1-RepetitionNum2/4/8 (i.e., not reuse rsrp-ThresholdMsg1-RepetitionNum2/4/8).

Agreements on RACH

RAN2 assume that when CFRA indicates SBFD RO, the RACH resources for the same RO type is provided for CBRA. FFS if any spec changes is needed. For RACH configuration Option 2, all the RRC configured parameters are re-initialized after RO type switching. For RACH Configuration Option 1, sbfd-RACHSingleConfig-preambleReceivedTargetPower is re-initialized after RO type switching. Can discuss in the RRC CR review the configuration restriction (if needed) for preambleTransMax For RACH configuration Option 2, PREAMBLE_POWER_RAMPING_STEP and SCALING_FACTOR_BI are re-initialized after RO type switching. For both RACH configuration Option 1 and RACH configuration Option 2, PREAMBLE_POWER_RAMPING_COUNTER is not re-initialized after RO type switching. Can further check the other UE variables in the CR review. For the RO type fallback between legacy RO and additional RO for RACH configuration option 2, a power offset given by the difference between the two values of preamble power ramping steps is added. Exact change to the MAC spec can be further discussed in the CR review. Only for RACH configuration Option 1, reuse the rsrp-ThresholdMsg1-RepetitionNum2/4/8 to determine Msg1 repetition number in SBFD RO if sbfd-RSRP-ThresholdMsg1-RepetitionNum2/4/8 is not configured. For RACH configuration Option 2, determine Msg1 repetition number only by sbfd-RSRP-ThresholdMsg1-RepetitionNum2/4/8 (i.e., not reuse rsrp-ThresholdMsg1-RepetitionNum2/4/8).

R2-2505089	Leftover Issues on Random Access in SBFD	CATT	discussion	Rel-19	NR_duplex_evo-Core

R2-2505126	Remaining issues of RA for SBFD	NEC	discussion	Rel-19	NR_duplex_evo-Core

R2-2505141	Discussion on RACH in SBFD	Xiaomi	discussion	Rel-19	NR_duplex_evo-Core

R2-2505243	Power ramping issue on the RO type fallback	OPPO, ZTE Corporation, Sharp, NEC, NTT DOCOMO INC., Qualcomm Incorporated, CATT	discussion	Rel-19	NR_duplex_evo-Core

R2-2505244	Clarification on the CFRA for SBFD RO	OPPO	discussion	Rel-19	NR_duplex_evo-Core

R2-2505365	Discussion on MAC open issues for random access in SBFD	Huawei, HiSilicon	discussion	Rel-19	NR_duplex_evo-Core

R2-2505459	Remaining issues on Random Access procedure for SBFD	LG Electronics Inc.	discussion	Rel-19	NR_duplex_evo-Core

R2-2505495	Open issues for RACH in SBFD	Apple	discussion	Rel-19	NR_duplex_evo-Core

R2-2505559	Discussions on the open issues for Random Access	Samsung	discussion	Rel-19	NR_duplex_evo-Core

R2-2505590	Discussion on random access procedure in SBFD	ZTE Corporation	discussion	Rel-19	NR_duplex_evo-Core

R2-2505591	Discussion on the co-existence of SBFD and LTM	ZTE Corporation, OPPO, Interdigital, LG, Apple, Charter, Nokia	discussion	Rel-19	NR_duplex_evo-Core

R2-2505661	Remaining issues for Random Access in SBFD Operation	Sony	discussion	Rel-19	NR_duplex_evo-Core

R2-2505666	SBFD RA remaining aspects	Ericsson	discussion	Rel-19	NR_duplex_evo-Core

R2-2505751	Random Access Operation of SBFD	Nokia	discussion	Rel-19	NR_duplex_evo-Core

R2-2505820	Views on random access for SBFD	Qualcomm Incorporated	discussion	NR_duplex_evo-Core

R2-2505904	Remaining issues on RACH aspect in SBFD	InterDigital, Inc.	discussion	Rel-19	NR_duplex_evo-Core

R2-2505929	Discussion on SBFD RA open issues	Sharp	discussion	Rel-19	NR_duplex_evo-Core

R2-2505952	Discussion on random access in SBFD	CMCC	discussion	Rel-19	NR_duplex_evo-Core

R2-2505982	Remaining issues on RACH procedure for SBFD	vivo	discussion	Rel-19	NR_duplex_evo-Core

### 8.11.3	Other aspects

Other RAN2 impacts with SBFD if not covered by the previous agenda items.

RRC-7

R2-2505667	DC and CSI-RS measurements in SBFD	Ericsson	discussion	Rel-19	NR_duplex_evo-Core

Noted

Proposal 1	RAN2 to conclude that

a.	Whether SBFD and DC can be supported is up to RAN4 decision

b.	The issue doesn’t block the completion of the WI in RAN2

c.	Further spec changes in RAN2 can be triggered based on RAN4 decision.

Discussions

-	InterDigital think from R2 point of view we can support SBFD + DC. QC agree.

-	Samsung, CATT, Xiaomi agree with the proposal.

-	QC think we can send LS to RAN4. LG E, Xiaomi agree.

-	CATT, HW think R4 is discussing so no need to send LS.

RAN2 evaluated the specification impact to support simultaneous configuration of SBFD and DC, and concluded the following: From RAN2 point of view there may be limited specification impact (i.e., stage-2 impact but no need to introduce stage-3 spec impact). Send LS to RAN4 and RAN1 (cc RAN3) to inform this conclusion.

[Post131][223][SBFD] LS on simultaneous configuration of SBFD and DC (ZTE)

Intended outcome: Approve the LS

Deadline:  Short

=> Approved in R2-2506492 (LS out)

R2-2506492	Reply LS on simultaneous configuration of SBFD and DC	RAN2	LS out	Rel-19	NR_duplex_evo-Core	To:RAN1, RAN4	Cc:RAN3

=> Approved

Agreements on other aspects

RAN2 evaluated the specification impact to support simultaneous configuration of SBFD and DC, and concluded the following: From RAN2 point of view there may be limited specification impact (i.e., stage-2 impact but no need to introduce stage-3 spec impact). Send LS to RAN4 and RAN1 (cc RAN3) to inform this conclusion.

Chair: other issues can be discussed in CB session if time allows

R2-2505090	Leftover Issues on other aspects in SBFD	CATT	discussion	Rel-19	NR_duplex_evo-Core

R2-2505142	Other aspects of SBFD	Xiaomi	discussion	Rel-19	NR_duplex_evo-Core

R2-2505366	Discussion on other aspects of SBFD	Huawei, HiSilicon	discussion	Rel-19	NR_duplex_evo-Core

R2-2505592	Discussion on multi-carrier and measurements in SBFD	ZTE Corporation	discussion	Rel-19	NR_duplex_evo-Core

R2-2505667	DC and CSI-RS measurements in SBFD	Ericsson	discussion	Rel-19	NR_duplex_evo-Core

R2-2505821	Other aspects of SBFD	Qualcomm Incorporated	discussion	NR_duplex_evo-Core

R2-2505905	Discussion on other aspect in SBFD	InterDigital, Inc.	discussion	Rel-19	NR_duplex_evo-Core

R2-2505930	Discussion on SBFD other open issue	Sharp	discussion	Rel-19	NR_duplex_evo-Core

R2-2505983	SBFD other aspects	vivo	discussion	Rel-19	NR_duplex_evo-Core

R2-2506092	Other aspects of SBFD	Nokia	discussion	Rel-19	NR_duplex_evo-Core	Withdrawn

R2-2506131	Other Aspects of SBFD	Nokia 	discussion	Rel-19	NR_duplex_evo-Core

=> Revised in R2-2506166

R2-2506166	Other Aspects of SBFD	Nokia	discussion	Rel-19	NR_duplex_evo-Core	R2-2506131

## 8.12	NR MIMO Phase 5

(NR_MIMO_Ph5-Core; leading WG: RAN1; REL-19; WID: RP-242394)

Time budget: 0.5 TU

Tdoc Limitation: 2 tdocs

### 8.12.1	Organizational

LSs and rapporteur input, including workplan, running CRs, email discussion summary, open issue list(s), etc.

LSin

R2-2505027	LS on Draft CR on TS38.300 for Rel-19 MIMO (R1-2505008; contact: Samsung)	RAN1	LS in	Rel-19	NR_MIMO_Ph5	To:RAN2

Noted

CRs

R2-2505423	Introduction of MIMO	Samsung (Rapporteur)	CR	Rel-19	38.321	18.6.0	2100	-	B	NR_MIMO_Ph5-Core

R2-2505806	Introduction of MIMO Phase 5	Ericsson	CR	Rel-19	38.331	18.6.0	5441	-	B	NR_MIMO_Ph5-Core

The above 2 CRs are endorsed, and they will be taken as baseline for further updates and review

R2-2505949	Running CR for Rel-19 MIMO Phase 5	CMCC	CR	Rel-19	38.300	18.6.0	1021	-	B	NR_MIMO_Ph5-Core

Revised in R2-2506244

Discussion

-	ZTE think the new texts on pathloss reference is the existing behaviour and no need to capture in spec, and think if we capture it, the right place should be stage 3.

-	OPPO think they are not reflected in stage 3 and think it is ok to put it in stage 2. LG E, CMCC agree.

-	Samsung think 6.x should be merged to mTRP section, no need for separate section.

R2-2506244 Introduction of Rel-19 MIMO Phase 5	CMCC	CR	Rel-19	38.300	18.6.0	1021	1	B	NR_MIMO_Ph5-Core

Endorsed

Post meeting email discussions for the CRs

[Post131][216][MIMO_Ph5] CR for TS 38.300 (CMCC)

Intended outcome: Agree the CR for TS 38.300

Deadline:  Short

=> Agreed in R2-2506546 (38.300)

[Post131][217][MIMO_Ph5] CR for TS 38.321 (Samsung)

Intended outcome: Agree the CR for TS 38.321

Deadline:  Short

=> Agreed in R2-2506539 (38.321)

[Post131][218][MIMO_Ph5] CR for TS 38.331 (Ericsson)

Intended outcome: Agree the CR for TS 38.331

Deadline:  Short

=> Agreed in R2-2506587 (38.331)

R2-2506546	Introduction of Rel-19 MIMO Phase 5	CMCC	CR	Rel-19	38.300	18.6.0	1021	2	B	NR_MIMO_Ph5-Core

=> Agreed

R2-2506539	Introduction of MIMO	Samsung	CR	Rel-19	38.321	18.6.0	2100	1	B	NR_MIMO_Ph5-Core

=> Agreed

R2-2506587	Introduction of MIMO Phase 5	Ericsson	CR	Rel-19	38.331	18.6.0	5441	2	B	NR_MIMO_Ph5-Core

=> Agreed

Work plan

R2-2505948	Work Plan for Rel-19 on NR MIMO Phase 5	CMCC, Samsung, MediaTek	Work Plan	Rel-19	NR_MIMO_Ph5-Core

Noted

Email discussion summary

R2-2505424	Report of MAC open issues for MIMO	Samsung	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal for easy agreement:

Proposal 1 (12/12): If the PUCCH of a UEI report configuration is pointed to a SCell whose TAT of the single sTAG is expired, this PUCCH for the SCell is released by RRC. If the type-1 CG of a UEI report configuration is pointed to a SCell whose TAT of the single sTAG is expired, this type-1 CG for the SCell is cleared as a configured UL grant. There is no MAC specification impact.

Proposal 4 (12/12): Regardless of whether the MAC entity is monitoring PDCCH or not on the Serving Cells in a DRX group, the MAC entity transmits mode-A UE-initiated CSI reporting on PUCCH and PUSCH on the Serving Cells in the DRX group when such is expected.

Proposal 5 (8/11): UE does not transmit PUCCH/PUSCH for mode-B if either PUCCH or PUSCH (first valid type-1 CG occasion) is outside DRX Active Time.

Proposal 6 (11/12): For mode-A UEI report, regarding monitoring PDCCH for DG in cell DTX, no enhancement is needed.

Proposal 7 (12/12): Regarding UEI report in cell DRX:

-	If the PUSCH for mode-A UEI report is scheduled by NW, UE shall transmit regardless of cell DRX, no MAC spec. impact.

-	UE does not transmit mode-A UEI report PUCCH outside cell DRX Active Period.

-	UE does not transmit PUCCH/PUSCH for mode-B UEI report if either the PUCCH or PUSCH (first valid type-1 CG occasion) for a report is outside cell DRX Active Period.

Proposal 8 (12/12): sDCI mTRP 2TA (in case of no PL offset) is supported for legacy Rel-18 LTM cell switch with no MAC specification impact.

If the PUCCH of a UEI report configuration is pointed to a SCell whose TAT of the single sTAG is expired, this PUCCH for the SCell is released by RRC. If the type-1 CG of a UEI report configuration is pointed to a SCell whose TAT of the single sTAG is expired, this type-1 CG for the SCell is cleared as a configured UL grant. There is no MAC specification impact

Regardless of whether the MAC entity is monitoring PDCCH or not on the Serving Cells in a DRX group, the MAC entity transmits mode-A UE-initiated CSI reporting on PUCCH and PUSCH on the Serving Cells in the DRX group when such report is expected.

Discussions

P5

-	Apple has concern since UE does not know whether the next PUSCH opportunity is in or outside of DRX active time. CATT share this concern. Ofinno think this is up to UE implementation.

-	 LG E think the current P5 aligns with the existing behaviour for semi-persistent CSI, so no issue. Nokia agree.

UE does not transmit PUCCH and PUSCH for mode-B if either PUCCH or PUSCH (first valid type-1 CG occasion) is outside DRX Active Time.

UE does not transmit PUCCH and PUSCH for mode-B if either PUCCH or PUSCH (first valid type-1 CG occasion) is outside DRX Active Time, and if the PUCCH has been transmitted, UE does not transmit the PUSCH if type-1 CG occasion is outside DRX Active time. Inform RAN1 about this conclusion.

For mode-A UEI report, regarding monitoring PDCCH for DG in cell DTX, no enhancement is needed.

Regarding UEI report in cell DRX:

- -	If the PUSCH for mode-A UEI report is scheduled by NW, UE shall transmit regardless of cell DRX, no MAC spec. impact.

- -	UE does not transmit mode-A UEI report PUCCH outside cell DRX Active Period.

- -	UE does not transmit PUCCH/PUSCH for mode-B UEI report if either the PUCCH or PUSCH (first valid type-1 CG occasion) for a report is outside cell DRX Active Period.

sDCI mTRP 2TA (in case of no PL offset) is supported for legacy Rel-18 LTM cell switch with no MAC specification impact.

R2-2505807	Open issues for MIMO on 38.331	Ericsson	discussion

Noted

Agreements on the remaining open issues

If the PUCCH of a UEI report configuration is pointed to a SCell whose TAT of the single sTAG is expired, this PUCCH for the SCell is released by RRC. If the type-1 CG of a UEI report configuration is pointed to a SCell whose TAT of the single sTAG is expired, this type-1 CG for the SCell is cleared as a configured UL grant. There is no MAC specification impact Regardless of whether the MAC entity is monitoring PDCCH or not on the Serving Cells in a DRX group, the MAC entity transmits mode-A UE-initiated CSI reporting on PUCCH and PUSCH on the Serving Cells in the DRX group when such report is expected. UE does not transmit PUCCH and PUSCH for mode-B if either PUCCH or PUSCH (first valid type-1 CG occasion) is outside DRX Active Time, and if the PUCCH has been transmitted, UE does not transmit the PUSCH if type-1 CG occasion is outside DRX Active time. Inform RAN1 about this conclusion. For mode-A UEI report, regarding monitoring PDCCH for DG in cell DTX, no enhancement is needed. Regarding UEI report in cell DRX: - - If the PUSCH for mode-A UEI report is scheduled by NW, UE shall transmit regardless of cell DRX, no MAC spec. impact. - - UE does not transmit mode-A UEI report PUCCH outside cell DRX Active Period. - - UE does not transmit PUCCH/PUSCH for mode-B UEI report if either the PUCCH or PUSCH (first valid type-1 CG occasion) for a report is outside cell DRX Active Period. sDCI mTRP 2TA (in case of no PL offset) is supported for legacy Rel-18 LTM cell switch with no MAC specification impact.

Completion of the WI

From RAN2 point of view, the Rel-19 work item NR_MIMO_Ph5-Core is completed.

### 8.12.2	Asymmetric DL sTRP/UL mTRP

Remaining issues for asymmetric DL sTRP/UL mTRP

On RRC parameter for Rel-19 2TA

R2-2505425	RRC parameters for two-TA operation	Samsung	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 1: RAN2 clarifies that the parameter that enables Rel-19 two-TA configuration (singleDCI-MultiTRP-2TA) is applied for both Rel-19 intra-cell 2TA and inter-cell 2TA for which mDCI mTRP is not configured. Update the field description to capture this and informs RAN1 by LS.

Discussion

-	Nokia think it is useful to inform R1.

RAN2 clarifies that the parameter that enables Rel-19 two-TA configuration (singleDCI-MultiTRP-2TA) is applied for both Rel-19 intra-cell 2TA without mDCI mTRP (including asymmetric TRP and symmetric TRP) and inter-cell 2TA without mDCI mTRP (i.e., ICBM). The corresponding parameter name and its field description will be updated to capture this agreement, and the agreement will be informed to RAN1 by LS.

[Post131][222]MIMO_Ph5] LS on two-TA configuration scenarios  (Samsung)

Intended outcome: Approve the LS

Deadline:  Short

=> Approved in R2-2506540 (LS out)

R2-2506540	LS on two-TA configuration and UE-initiated CSI reporting	RAN2	LS out	Rel-19	NR_MIMO_Ph5	To:RAN1

=> Approved

R2-2505361	Discussion on Asymmetric DL sTRP UL mTRP	CATT	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 1a (RRC, Issue-4): RAN2 to confirm the current RRC parameter singleDCI-MultiTRP-2TA can enable UE to configure either intra-cell or inter-cell sDCI mTRP with two-TA. No further RRC spec impact.

Proposal 1b (RRC, Issue-4): LS to RAN1 that RAN2 introduced a new RRC parameter singleDCI-MultiTRP-2TA per UL BWP to enable the UE configured with intra-cell or inter-cell sDCI mTRP with two-TA. Adopt Annex 1 for the draft LS.

R2-2505585	Discussion on open issues for asymmetric DL sTRPUL mTRP	vivo	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 1:	(RRC-x) Two RRC parameter for Rel-19 sDCI-mTRP 2TA, e.g. singleDCI-MultiTRP-IntraCell2TA, and singleDCI-MultiTRP-InterCell2TA, are introduced in RAN2 to enable intra-cell and inter-cell 2TA, respectively.

Chair: other issues can be discussed in CB session if time allows

Agreements on Asymmetric DL sTRP/UL mTRP

RAN2 clarifies that the parameter that enables Rel-19 two-TA configuration (singleDCI-MultiTRP-2TA) is applied for both Rel-19 intra-cell 2TA without mDCI mTRP (including asymmetric TRP and symmetric TRP) and inter-cell 2TA without mDCI mTRP (i.e., ICBM). The corresponding parameter name and its field description will be updated to capture this agreement, and the agreement will be informed to RAN1 by LS.

R2-2505242	Clarification on the coexistence between LTM and UL-only TRP	OPPO	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2505267	Enhancements for Asymmetric DL sTRP and UL mTRP	Ofinno	discussion	Rel-19

R2-2505361	Discussion on Asymmetric DL sTRP UL mTRP	CATT	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2505425	RRC parameters for two-TA operation	Samsung	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2505585	Discussion on open issues for asymmetric DL sTRPUL mTRP	vivo	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2505862	Asymmetric DL/UL mTRP impact from MIMO ph. 5	Ericsson	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2505891	Remaining issues on Asymmetric DL sTRP/UL mTRP	Huawei, HiSilicon	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2505901	Remaining issues on asymmetric DL sTRP/UL mTRP	Nokia	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2505946	Discussion on Asymmetric DL sTRP/UL mTRP	CMCC	discussion	Rel-19	NR_MIMO_Ph5-Core

### 8.12.3	Others

Remaining issues for UE-initiated reporting, and other issues if not covered by the previous agenda items.

Open issue for UE-initiated beam report related to TAT expiry

R2-2505464	Discussion on UEI beam reporting impact	LG Electronics Inc.	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 1. When UEIBR is initiated but TAT expires, UE does not initiate RACH as in legacy.

Proposal 2. When UEIBR is initiated but TAT expires, MAC indicates to cancel the triggered UEIBR to PHY layer.

R2-2506035	Discussion on MAC open issues for MIMO phase 5	ASUSTeK	discussion	Rel-19	38.321	NR_MIMO_Ph5-Core

Noted

Proposal 1:	RAN2 to confirm that based on the current MAC spec, the UE releases PUCCH resources and clears type-1 CG PUSCH resources configured in a UEI report configuration of a SCell if the TAT of the sTAG of the SCell is expired, regardless of the Cell on which the PUCCH and PUSCH resources are configured.

Proposal 2:	The UE initiates RACH when a UEI report is triggered but there is no PUCCH or type-1 CG for the triggered UEI report due to the associated TAT being expired, and UE indicates the cause of RACH via at least one of the following options:

-	Option 1: explicit indication via a MAC CE in Msg3

-	Option 2: implicit indication via a dedicated preamble configured for UEI report-initiated RACH.

R2-2505850	Discussion on remaining issues for UE initiated beam report 	Qualcomm Incorporated	discussion

Noted

Proposal 2. For mode-A UEI report, if TAT is expired after transmitting PUCCH and before PUSCH for a triggered report, no enhancement is needed..

Discussions

-	Ericsson think no need for further enhancement.

-	Ofinno think we could use LCID to indicate so the impact to R2 is limited.

-	Nokia agree with P2 from AsusTek and think O1 is better. Nokia fine with P2 from Qualcomm.

-	LG E has concern on sending outdated CSI report to the network.

-	Nokia wonders with LG E P1, what is the UE behaviour from L1 point of view.

For both mode-A and mode-B UEI beam report,

When UEIBR is initiated but TAT expires, UE does not initiate RACH.

If TAT is expired after transmitting PUCCH and before PUSCH for a triggered report, no enhancement is needed.

Other proposals

R2-2505268	Enhancements for UE-initiated Beam Reporting	Ofinno	discussion	Rel-19

Noted

Proposal 1: Explicitly specify the RAN2 agreement: For Rel-15 UL skipping (skipUplinkTxDynamic is configured), same principle as legacy aperiodic CSI applies for multiplexing UCI of mode-A DG-based UE-initiated report in PUSCH.

Discussions

P1

-	ZTE do not agree with P1.

-	Qualcomm think R15 does not consider aperiodic CSI, and think we do not need to revisit.

-	CATT think P1 is acceptable.

R2-2505998	Consideration on the Remaining MAC Issues of UEIBM	ZTE Corporation	discussion	Rel-19	NR_MIMO_Ph5-Core

Noted

Proposal 1: RAN2 to confirm whether to further specify the broken R15 UL skipping function for the UEIBM.

Proposal 1a: No need to further specify the broken R15 UL skipping function for the UEIBM.

NW does not configure r15 UL skipping skipUplinkTxDynamic for a cell group if UEI-CSI reporting is configured for any cell of the cell group, this is captured in RRC skipUplinkTxDynamic field description, no MAC change for r15 UL skipping.

R2-2505808	Remaining aspects from other NR MIMO Ph.5 objectives	Ericsson	discussion

Noted

Proposal 2	Keep UEI report parameters within CSI-ReportConfig. No specification impact.

Proposal 3	referenceAntennaPort is defined with values 1, 2 and 4.

Discussions

P1

-	CATT, Samsung agree, and think this align with R1 understanding.

-	ZTE has concern on signalling efficiency. Ericsson think this is not critical issue to enhance.

Keep UEI report parameters within CSI-ReportConfig. No specification impact.

P3

-	Samsung think we should include 8.

-	ZTE think all values from 1 to 8 are possible.

referenceAntennaPort is introduced in RRC.

R2-2506035	Discussion on MAC open issues for MIMO phase 5	ASUSTeK	discussion	Rel-19	38.321	NR_MIMO_Ph5-Core

Noted

Proposal 1:	RAN2 to confirm that based on the current MAC spec, the UE releases PUCCH resources and clears type-1 CG PUSCH resources configured in a UEI report configuration of a SCell if the TAT of the sTAG of the SCell is expired, regardless of the Cell on which the PUCCH and PUSCH resources are configured.

Agreements on other issues

For both mode-A and mode-B UEI beam report, When UEIBR is initiated but TAT expires, UE does not initiate RACH. If TAT is expired after transmitting PUCCH and before PUSCH for a triggered report, no enhancement is needed. NW does not configure r15 UL skipping skipUplinkTxDynamic for a cell group if UEI-CSI reporting is configured for any cell of the cell group, this is captured in RRC skipUplinkTxDynamic field description, no MAC change for r15 UL skipping. Keep UEI report parameters within CSI-ReportConfig. No specification impact. referenceAntennaPort is introduced in RRC.

Chair: other issues can be discussed in CB session if time allows

R2-2505241	Discussion on the remaining issues of UE-initiated beam report	OPPO	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2505268	Enhancements for UE-initiated Beam Reporting	Ofinno	discussion	Rel-19

R2-2505362	Discussion on UE initiated beam reporting	CATT	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2505407	Discussion on MAC and RRC open issues for UEI BMR	vivo	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2505426	Open issues on UE-initiated CSI Reporting	Samsung	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2505464	Discussion on UEI beam reporting impact	LG Electronics Inc.	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2505484	Remaining issues of UE initiated beam reporting	Apple	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2505694	Remaining issues on UEI report	Lenovo	discussion	Rel-19

R2-2505808	Remaining aspects from other NR MIMO Ph.5 objectives	Ericsson	discussion

R2-2505850	Discussion on remaining issues for UE initiated beam report 	Qualcomm Incorporated	discussion

R2-2505892	UE-initiated/event-driven beam management	Huawei, HiSilicon	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2505902	Other MIMO issues	Nokia	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2505947	Discussion on other issues of NR MIMO Phase 5	CMCC	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2505998	Consideration on the Remaining MAC Issues of UEIBM	ZTE Corporation	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2505999	Consideration on the Remaining Asn.1 Issues	ZTE Corporation	discussion	Rel-19	NR_MIMO_Ph5-Core

R2-2506034	Discussion on RRC impacts for MIMO phase 5	ASUSTeK	discussion	Rel-19	38.331	NR_MIMO_Ph5-Core

R2-2506035	Discussion on MAC open issues for MIMO phase 5	ASUSTeK	discussion	Rel-19	38.321	NR_MIMO_Ph5-Core

## 8.13	NR sidelink multi-hop relay

(NR_SL_relay_multihop; leading WG: RAN2; REL-19; WID: RP-250188)

Time budget: 1 TU

Tdoc Limitation: 3 tdocs

### 8.13.1	Organizational

LSs and rapporteur input, including workplan, etc.

Including outcomes of email discussions on running CRs

Running CRs and open issue lists

R2-2505353	Introduction of NR sidelink multi-hop relay in TS 38.351	OPPO	CR	Rel-19	38.351	18.4.0	0041	-	B	NR_SL_relay_multihop-Core

Endorsed (for further changes this meeting)

R2-2505354	SRAP open issues for NR sidelink multi-hop relay	OPPO	other	Rel-19	NR_SL_relay_multihop-Core

Noted

[Post131][411][Relay] Rel-19 relay 38.351 CR (OPPO)

Scope: Update the CR in R2-2505353 in accordance with decisions of RAN2#131.

Intended outcome: Agreed CR in R2-2506323

Deadline: Short (for RP)

=> Agreed in R2-2506566 (38.351)

R2-2506566	Introduction of NR sidelink multi-hop relay in TS 38.351	OPPO	CR	Rel-19	38.351	18.4.0	0041	2	B	NR_SL_relay_multihop-Core

=> Agreed

R2-2505427	Introduction of NR Sidelink Multi-hop Relay	InterDigital France R&D, SAS	CR	Rel-19	38.321	18.6.0	2101	-	B	NR_SL_relay_multihop-Core

Endorsed (for further changes this meeting)

[Post131][412][Relay] Rel-19 relay 38.321 CR (InterDigital)

Scope: Update the CR in R2-2505427 in accordance with decisions of RAN2#131.

Intended outcome: Agreed CR in R2-2506324

Deadline: Short (for RP)

=> Agreed in R2-2506626 (38.321)

R2-2506626	Introduction of NR Sidelink Multi-hop Relay	InterDigital (Rapporteur)	CR	Rel-19	38.321	18.6.0	2101	3	B	NR_SL_relay_multihop-Core

=> Agreed

R2-2506603	Introduction of NR Sidelink Multi-hop Relay	InterDigital (Rapporteur)	CR	Rel-19	38.321	18.6.0	2101	2	B	NR_SL_relay_multihop-Core

=> Agreed but then coversheet revised by MCC (wrong tdoc number)

R2-2506626	Introduction of NR Sidelink Multi-hop Relay	InterDigital (Rapporteur)	CR	Rel-19	38.321	18.6.0	2101	3	B	NR_SL_relay_multihop-Core

=> Agreed

R2-2505621	Introduction of NR sidelink multi-hop U2N relay in TS 38.300	LG Electronics Inc.	draftCR	Rel-19	38.300	18.6.0	NR_SL_relay_multihop

Endorsed (for further changes this meeting)

[Post131][413][Relay] Rel-19 relay 38.300 CR (LG)

Scope: Update the CR in R2-2505621 in accordance with decisions of RAN2#131.

Intended outcome: Agreed CR in R2-2506325

Deadline: Short (for RP)

=> Agreed in R2-2506568 (38.300)

R2-2506568	Introduction of NR sidelink multi-hop U2N relay	LG Electronics Inc.	CR	Rel-19	38.300	18.6.0	1028	2	NR_SL_relay_multihop

=> Agreed

R2-2505714	Introduction of NR sidelink multi-hop relay in TS 38.331	Huawei, HiSilicon	CR	Rel-19	38.331	18.6.0	5429	-	B	NR_SL_relay_multihop-Core

Endorsed (for further changes this meeting)

[Post131][414][Relay] Rel-19 relay 38.331 CR (Huawei)

Scope: Update the CR in R2-2505714 in accordance with decisions of RAN2#131.

Intended outcome: Agreed CR in R2-2506326

Deadline: Short (for RP)

=> Agreed in R2-2506625 (38.331)

R2-2506625	Introduction of NR sidelink multi-hop relay	Huawei, HiSilicon	CR	Rel-19	38.331	18.6.0	5429	3	B	NR_SL_relay_multihop-Core

=> Agreed

R2-2506326	Introduction of NR sidelink multi-hop relay in TS 38.331	Huawei, HiSilicon	CR	Rel-19	38.331	18.6.0	5429	1	B	NR_SL_relay_multihop-Core

R2-2506574	Introduction of NR sidelink multi-hop relay	Huawei, HiSilicon, CATT	CR	Rel-19	38.331	18.6.0	5429	2	B	NR_SL_relay_multihop-Core

=> Agreed but then coversheet revised by MCC in R2-2506625 (wrong revision number)

R2-2506625	Introduction of NR sidelink multi-hop relay	Huawei, HiSilicon, CATT	CR	Rel-19	38.331	18.6.0	5429	3	B	NR_SL_relay_multihop-Core

=> Agreed

R2-2505432	Open issues for Multi hop Sidelink Relay in TS 38.331	Huawei, HiSilicon	discussion	Rel-19	38.331	NR_SL_relay_multihop-Core

Noted

Proposal 1: Introduce sl-L2U2N-MH-Relay for indicating the support of NR sidelink Layer-2 U2N multi hop relay operation in SIB 12. Status: Closed.

Proposal 2: Introduce capabilities for R19 e.g.  relayUE-MH-Operation-L2-r19 and remoteUE-MH-Operation-L2-r19 in SidelinkParameters IE. Status: Closed.

Discussion:

Huawei confirm that these issues were closed last meeting.

Proposal 3: Update TS 38.331, Section 3.1, to include the following clarification in the definitions of Intermediate U2N Relay UE and First Relay UE:

“An Intermediate U2N Relay UE first establishes a connection to the network as a U2N Remote UE, before beginning to relay traffic for connected U2N Remote UEs.”

Discussion:

OPPO agree with this understanding but think the clarification could be captured only in stage 2.  Huawei think stage 3 would then not have the behaviour described clearly.

LG think this is related to the control  procedures and may need text beyond just the definitions.

Apple agree with OPPO that this is more suitable for stage 2, and they are not sure how it would be reflected in stage 3 since there is no “beginning relay traffic” procedure, only the first RRC message.  Ericsson agree and also think we should say “in this release”.

Huawei think it would be OK to capture in stage 2.

Apple want to clarify that this is for user traffic, not discovery, so it does not close the open issue on the PC5 link before discovery.

Agreement:

Update definitions in TS 38.300 to include the following clarification in the definitions of Intermediate U2N Relay UE and First Relay UE:

“In this release, an [Intermediate/First] U2N Relay UE first establishes a connection to the network as a U2N Remote UE, before beginning to relay traffic for connected U2N Remote UEs.”

Proposal 4: Confirm that the Remote UE can request the SFN-DFN offset from the connected parent Relay UE and that the SFN-DFN offset provided at the L2 U2N Relay UE or the L2 Last U2N Relay UE can be forwarded by intermediate U2N Relay UEs within the multihop relay chain, as proposed in the current running CR.

Discussion:

CATT think this was discussed in Rel-18 as a TEI, and they would like to treat this in the same way.

Qualcomm agree with CATT and think the relay session is not the right place to discuss this; they are not sure it will work well in the multihop setting.

Samsung share the same concern and have a concern about if the delay to forward the offset spans an SFN boundary.

Proposal 5 – RAN 2 should discuss if there are any parameter not visible to AS layer that could change and would affect reselection and force the remote UE to be notified.

Proposal 6 – RAN2 to confirm that the Intermediate Relay UE sets the indication type to a specific value,  relayUE-RelayReselection, to clearly signal that the notification is triggered by its own reselection action, which follows a notification of AS failure received from its parent relay UE.

Discussion:

Lenovo think if the intermediate relay UE is in coverage, it may perform cell selection instead.  LG note that it is up to UE implementation whether to keep or release the PC5 link, and if the intermediate relay UE keeps the link with the parent relay UE, the new cause value is not appropriate.

InterDigital think a new cause value is needed so that the remote UE knows if it needs to take a recovery action.  They agree with Lenovo’s comment and think we could have a single cause value for cell and relay reselection.

Apple think we could have one new cause value for reselection and upstream RLF, and suppress the indication if the parameters are not changed.

Huawei agree with Lenovo about cell reselection, and we should avoid double notification cases.

Samsung think there is some relation to how to transmit the notification for multi-path relay, and we could see contributions.

Proposal 7 – RAN2 to discuss if we need to further fine tune the timers value calculation for the multihop scenario.

Proposal 8 – RAN2 should clarify in TS 38.300 or TS 38.331 that, for Scenario C and Scenario D, the Remote UE shall report only Relay UEs in RRC_CONNECTED state for path switching, based on the Relay State indication received in the discovery message RRC container.

Discussion:

OPPO understand the network can select and there is no need for the reporting restriction.  LG agree.

CATT also agree and think we do not need the resitrction.

Lenovo also think the network can handle it.

Huawei wonder what happens if we have idle/inactive UEs and the UE does not report any connected UE; presumably the gNB would take no action, and the intention here is to guarantee that the gNB can do something.

Samsung think we can reduce the reporting from remote UEs and this is the main benefit; they support the proposal.

Ericsson agree with Lenovo and think we could leave it to UE implementation if anything.

InterDigital think the proposal makes sense and we should think about extending to more hops in the future, where unnecessary measurement reports would be a worse problem; they also think we should consider future inter-gNB cases.

Kyocera agree with Samsung and InterDigital.

Qualcomm think we concluded last time that how to use it was up to UE implementation.

Chair wonders what the gNB will do with idle/inactive UEs.  OPPO understand the gNB can page an inactive relay into RRC_CONNECTED; they think the key point is that the gNB selects the target path and so we should not have UE filtering.

LG think the proposal is not needed and the gNB can determine; they also think that OPPO’s comment is valid and the gNB can page some UEs into RRC_CONNECTED.

Samsung note that the report is triggered by some event, and we should try to reduce the possible failure cases.

Xiaomi think we should at least prioritise the connected UEs, but if there are no connected UEs the network may be able to do something; they suggest that we could include the state in the measurement report.

InterDigital somewhat agree with Samsung that the reports can be useless to the network if it is too late to do anything; they think Xiaomi’s suggestion could be a way forward.

Apple have a similar view to InterDigital.

OPPO do not see why the state is needed, since the network already knows it.  Kyocera agree that it is not necessary, and they agree also that the gNB may need to page a relay UE.

Ericsson think an indication for RRC_IDLE would be enough.

Proposal 9 – RAN2 confirms that the existing note, stating that the Remote UE may prioritize selecting/reselecting a suitable Relay UE that is in the RRC_CONNECTED state based on RRC state information included in the Discovery Message container, is retained in the running CR.

Discussion:

Qualcomm think it is up to implementation and we do not need to capture anything in the spec.

OPPO agree with Qualcomm and note that latency depends on other characteristics besides the RRC state; they would like to leave it clearly fully up to UE implementation.

Lenovo also agree with Qualcomm, and they wonder about prioritisation between a suitable relay and a suitable cell.

InterDigital think the intention is not to dictate the implementation but indicate what the UE could do with the information; they agree there are other cases but think the current note is good.

Toyota note that “may” still means it is up to UE implementation, and we could also note that the UE may choose to minimise the number of hops.

Qualcomm think stage 2 would be suitable and we could list different information from the discovery message as examples.

Ericsson think the note is a bit misleading since what can be considered depends on many things, and they see this note as somewhat breaking the layer boundary.  Toyota think we should keep the list of examples to benefit the UE implementation.  Ericsson think it is already clear what is in the discovery message.

Huawei do not see how the note is misleading, and the inter-layer aspect is about information that has to be shared anyway.

LG are not sure that the state information is essential in discovery, and they understand that upstream relay UEs could be in any state if the first relay UE is not connected, so the latency is not predictable; they also think the states can change.

ZTE think the SA2 spec indicates that every relay UE will generate its own AS container, so it will show only the state of the first relay UE.

Huawei think we could include multiple states, but if the first relay is connected then all the upstream relays are connected, and such a path would be preferable for the remote; they understand that if the state changes, the discovery messages will be updated.

Apple think adding extra states would have SA2/CT1 impact.

Qualcomm think ZTE are mistaken and the container is forwarded down from the last relay UE.

Kyocera understand that in model B, the last relay UE selects the path, so it needs to know the downstream states.

Agreements:

RAN2 confirm that the selection of a suitable relay is up to UE implementation.

Capture in a revision of the existing note in RRC running CR (relay reselection section 5.8.15.3) that the UE may prioritise relays based on any information available in the discovery message for the applicable model.

Working assumption on having RRC_CONNECTED state indication for the announcing/responding UE (only) in the discovery AS container is confirmed.  No SA2/CT1 impact is expected.

R2-2505796	Introduction of multi-hop U2N relay in TS 38.323	Ericsson	CR	Rel-19	38.323	18.5.0	0150	-	B	NR_SL_relay_multihop

Endorsed (for further changes this meeting)

[Post131][415][Relay] Rel-19 relay 38.323 CR (Ericsson)

Scope: Update the CR in R2-2505796 in accordance with decisions of RAN2#131.

Intended outcome: Agreed CR in R2-2506327

Deadline: Short (for RP)

=> Agreed in R2-2506635 (38.323)

R2-2506327	Introduction of multi-hop U2N relay in TS 38.323	Ericsson	CR	Rel-19	38.323	18.5.0	0150	1	B	NR_SL_relay_multihop

=> Agreed, but then coversheet revised by MCC: CR number has only three digits (150)

R2-2506635	Introduction of multi-hop U2N relay in TS 38.323	Ericsson	CR	Rel-19	38.323	18.5.0	0150	2	B	NR_SL_relay_multihop

=> Agreed

R2-2506047	Introduction of multi-hop U2N relay in TS 38.304	MediaTek Inc.	CR	Rel-19	38.304	18.4.0	0444	-	B	NR_SL_relay_multihop-Core

Endorsed (for further changes this meeting)

[Post131][416][Relay] Rel-19 relay 38.304 CR (MediaTek)

Scope: Update the CR in R2-2506047 in accordance with decisions of RAN2#131.

Intended outcome: Agreed CR in R2-2506328

Deadline: Short (for RP)

=> Agreed in R2-2506328 (38.304)

R2-2506328	Introduction of multi-hop U2N relay in TS 38.304	MediaTek Inc.	CR	Rel-19	38.304	18.4.0	0444	1	B	NR_SL_relay_multihop-Core

=> Agreed

UE capabilities

R2-2505771	Summary on [Post130][401][Relay] Rel-19 relay capability	Samsung	discussion	Rel-19	NR_SL_relay_multihop-Core

Noted

Proposal 1: FFS on capability differentiation between last relay UE and intermediate relay UE. If the differentiation is needed, RAN2 is kindly asked to discuss how to define two capabilities based on the following two options:

	Option 1: define two new capabilities, LastRelayUE-MH-Operation-L2, and IntermediateRelayUE-MH-Operation-L2, where the remoteUE-MH-Operation-L2 is the prerequisite of  IntermediateRelayUE-MH-Operation-L2;

	Option 2: keep relayUE-MH-Operation-L2 for both last relay UE and intermediate relay UE, while for intermediate relay UE, the remoteUE-MH-Operation-L2 also indicates.

Discussion:

Huawei think we cannot have UEs without Uu interface.

Apple think we do not need to differentiate the capabilities, so we may not need either option.  They think it is harmful to public safety use cases if relays cannot switch roles when needed.

OPPO understand the capabilities do not imply that an intermediate could not switch to a last, but a UE may not implement Uu SRAP and then could not be a last relay UE.  They see the differentiation as being the Uu SRAP for the last relay and remote UE functionality for the intermediate.

AT&T do not see the need for differentiation; they understand that a relay UE should implement both.

Qualcomm think we should support the case of a fixed last relay UE that is never called on to become an intermediate.

Ericsson agree with AT&T.  Kyocera also support no differentiation.  FirstNet also agree.

Samsung agree with Qualcomm’s comment and think differentiation gives more deployment flexibility.

LG think fragmentation is bad and we should not have differentiation.

Xiaomi think there should be differentiation.  Qualcomm wonder what harm there is for the network in differentiation.

AT&T think it is more important to have flexibility in deployment with relays switching roles when needed.

OPPO do not think having different capabilities restricts this scenario; it just means that UEs would be needed that supported both roles.

Samsung think option 2 may be a middle ground.

Proposal 2: In Rel-19, the new band combination capability for multi-hop relay discovery is not needed.

Proposal 3: In Rel-19, the new path switch capability for multi-hop relay is not needed.

Proposal 4: In Rel-19, the L3 relay capability for multi-hop relay is not needed.

Agreements:

RAN2 confirm that UEs without Uu interface support are not in scope of this WI.

In Rel-19, the new band combination capability for multi-hop relay discovery is not needed.

In Rel-19, the new path switch capability for multi-hop relay is not needed.

In Rel-19, the L3 relay capability for multi-hop relay is not needed.

Keep relayUE-MH-Operation-L2 for both last relay UE and intermediate relay UE, while at least for intermediate relay UE, the remoteUE-MH-Operation-L2 is also indicated.  Whether remote operation is a prerequisite for relay operation can be discussed in maintenance.

R2-2505772	Introduction of multi-hop sidelink relay capability	Samsung	CR	Rel-19	38.306	18.6.0	1334	-	B	NR_SL_relay_multihop-Core

Endorsed (for further changes this meeting)

[Post131][417][Relay] Rel-19 relay capability CRs (Samsung)

Scope: Update the CR in R2-2505772 in accordance with decisions of RAN2#131 and develop a complementary CR to 38.331.

Intended outcome: Endorsed CRs in R2-2506329 (38.306) and R2-2506330 (38.331)

Deadline: Very short (for merge into mega CRs)

=> Endorsed in:

- R2-2506329 (38.306)

- R2-2506330 (38.331)

R2-2506329	Introduction of multi-hop sidelink relay capability	Samsung	CR	Rel-19	38.306	18.6.0	1334	1	B	NR_SL_relay_multihop-Core

=> Endorsed

R2-2506330	Introduction of multi-hop sidelink relay capability	Samsung	CR	Rel-19	38.331	18.6.0	5478	-	B	NR_SL_relay_multihop-Core

=> Endorsed

Agreement:

Rel-19 relay WI is complete from RAN2 point of view (pending closure of the CR email discussions).

Withdrawn/Not available

R2-2505431	Introduction of NR sidelink multi-hop relay in TS 38.331	Huawei, HiSilicon	draftCR	Rel-19	38.331	18.6.0	F	NR_SL_relay_multihop-Core	Withdrawn

Withdrawn

### 8.13.2	Relay discovery and (re)selection

Enhancements to relay dscovery and (re)selection to support one additional hop relay (remote UE  first relay UE  last relay UE  gNB). Extensibility to a second additional hop in this WI is considered as a design criterion.

RRC-5/RRC-12: notification and cause code

Discussed jointly

R2-2505085	(RRC-5/11/12) Notification message handling for intermediate relay UE	vivo	discussion	Rel-19

Noted

Proposal 1: (RRC-5/12) When an intermediate relay UE receives a NotificationMessageSidelink message from the parent relay, it forward a copy of the original notification (with the same cause) to its child UEs.

Proposal 2: (RRC-5/12) When intermediate relay UE detects PC5 RLF, it should set the cause value to ‘relayUE-PC5-RLF’ to its child UE.

R2-2505418	Open Issues on Discovery and Relay (Re)Selection for Multi-hop U2N Relays	InterDigital	discussion	Rel-19	NR_SL_relay_multihop

Noted

Proposal 5:	In case of notification message triggered by an error (e.g., RLF) the intermediate relay UE can send cell/relay reselection cause value to the remote UE in RRC_IDLE/INACTIVE if it successfully performs a recovery action. Otherwise, it sends relayUE-PC5-RLF cause value if PC5 RLF is detected by itself or the same cause value received in the notification message from the parent node.

Proposal 6:	In case of notification message triggered by an error (e.g., RLF), the intermediate relay UE sends relayUE-PC5-RLF cause value if PC5 RLF is detected by itself or the same cause value as received by the parent node in the notification message to an RRC_CONNECTED remote UE.

Discussion:

OPPO generally agree that the intermediate relay should generate its own notification message, but they are not sure on the details of the InterDigital proposals; they think forwarding the notification may cause double triggering.

Kyocera also agree with InterDigital’s approach and think the timing of the notification needs to be considered.

vivo are not clear about the double trigger; they understand that the notification will be sent after reselection happens, and they do not see that generating a new notification automatically suppresses the double cases.

Huawei are not sure how forwarding can avoid double notification, because the original event and the reselection both happen.

LG think the intermediate relay UE can decide whether to keep the PC5 link with the parent, and it should forward the notification if it keeps the link but generate its own if it releases the link; they think this will avoid double notification cases.

Samsung understand that the intention of the notification is to indicate the UE’s own link status, not that of the parent, so they think it is unnecessary to send a copy.

Ericsson think the main thing is to let the remote UE know the upstream status, and either approach can work as long as this information is there.

InterDigital agree with Ericsson; we need to agree that if we use the legacy behaviour, a failure at the last relay will affect all the relays downstream, and it eliminates any possibility of recovery.

ZTE think either cause value will cause the same remote UE behaviour, so they would like to keep a single cause value.

Apple think there is a need to differentiate because reselection and RLF are two different scenarios, and the intermediate relay can generate a notification based on its own status.

OPPO understand vivo’s intention is that the notification would happen only on the subsequent reselection but not on the reception of the upstream notification itself, and this avoids double notification.

Cause value:

OPPO understand that the RLF cause value can be used for both cases because the behaviour will be the same: It should always check the following discovery message to know if a potential relay is suitable, so it cannot rely on the cause value.

Lenovo think there are different cases depending on the state of the relay UE; the connected relay will always perform re-establishment, and they think we could use the PC5-RLF cause value for all cases.

Agreements:

Notification by an intermediate relay UE at least in idle/inactive, when caused by an upstream reselection/RLF/link release, occurs upon the intermediate relay UE’s handling after receiving the notification or release message (e.g., relay reselection or cell selection), but not triggered by the reception of the upstream notification itself.  RRC_CONNECTED case to be considered as part of service continuity.

In case of notification message triggered by an error (e.g., RLF), for intermediate relay UE in idle/inactive, the intermediate relay UE can send cell/relay reselection cause value to the remote UE in RRC_IDLE/INACTIVE if it successfully performs a recovery action. If recovery action is not successful or if it opts to release the upstream PC5 link, it sends relayUE-PC5-RLF cause value if PC5 RLF is detected by itself or the same cause value received in the notification message from the parent node.

RRC-11: notifications in idle/inactive

R2-2505773	Discussion on remaining issues of relay discovery and (re)selection	Samsung	discussion	Rel-19	NR_SL_relay_multihop-Core

Noted

Proposal 5: RAN2 is kindly asked agree that the intermediate relay UE in RRC_IDLE/INACTIVE can omit the notification message transmission if the relay reselection does not cause the change of the hop count or serving cell.

Discussion:

Apple support the proposal.  LG support it as well, but they note that the hop count is in the upper layer.

InterDigital think the serving cell is obvious, and for the hop count they acknowledge that upper layers are responsible for selection based on hop count, they think we can still have a single mechanism to inform the child UE.

OPPO understand that we cannot assume the hop count and other upper-layer parameters are available without checking with SA2; they think other parameters like the user info would need to be considered as well.

ZTE can accept the proposal; on OPPO’s concern, they think we cannot capture all upper-layer parameter behaviour, and they agree that there are some interactions but it can be left to UE implementation.

Apple think the hop count is available for sure without needing to ask SA2, and we can coordinate parameters between layers by implementation.

Ericsson are not sure if the hop count will be visible to AS layer, but they are OK with the serving cell.

Qualcomm agree that the notification is only sent when AS-layer aspects change, but maybe we don’t need to consider upper-layer parameters like the hop count.  Chair wonders if this would mean we had cases of the notification being suppressed when the hop count changes; Qualcomm understand that this information could be propagated through discovery messages instead of the notification.

LG agree with Qualcomm that a changed hop count or path information will be reflected in discovery and the upper layer can handle it.

Kyocera support the proposal and think the relay UE should be able to determine if the hop count has changed upstream, so it can decide whether to send the notification.

InterDigital think whether to send the notification is an AS layer decision, and that functionality can depend on upper-layer parameters as done before in previous releases.

OPPO understand we agreed and notified SA2 that if the reselection is triggered by AS conditions, it will be handled by AS-layer signalling; they understand that LG and Qualcomm are suggesting we would revert that agreement and have an AS trigger handled by upper layers.

Huawei think we are optimizing; they are not sure it is critical to do this.

Samsung think we could focus on the serving cell as an AS layer parameter and leave the rest to upper layers.

OPPO think the upper layer does not know the situation since there will be no upper layer signalling, and they think there will be no unnecessary interruptions since the remote UE can decide by implementation whether to keep or release the link.

Samsung are not sure it is an optimization, and they think we faced a similar situation with the recovery case and decided that the notification can be avoided; they think we should reduce the signalling on each hop.  LG have the same understanding and think it is unnecessary to trigger relay reselection by the remote UE in these cases.

Ericsson wonder if we could leave it to intermediate relay UE implementation.

InterDigital think it cannot be entirely implementation, because if the serving cell changes there must be a notification.

Apple think we already agreed that this can be done and the question is which parameters are visible to AS layer, so they think we could omit the hop count and only have serving cell.

Lenovo note that we should consider cell selection as well.

Kyocera note that the hop count could increase and the notification would still be omitted.  OPPO also wonder about this scenario.

InterDigital think reselection can be triggered with no discovery message sent because the PC5 link is still of good quality.  Kyocera understand that the intermediate relay UE would look at the discovery signalling from the candidate upstream relay.  LG think this is not a concern in model B, and in model A the announcement message will change because the path has changed.

OPPO would like to understand the specification impact; would it be a NOTE?  InterDigital understand that by default, any reselection will trigger the notification, but if the intermediate relay does not change the serving cell, it is allowed to suppress the notification; however, this does not speak to the hop count.  InterDigital are OK with the agreement.  Apple think it should be normative text.

Agreement:

The intermediate relay UE in RRC_IDLE/INACTIVE may omit sending the notification message if the relay reselection or cell selection (from multihop to direct path for the intermediate relay) does not cause the change of the serving cell.  This does not preclude the implementation considering other parameters such as hop count if they are made visible to AS layer by implementation.  Capture this in a NOTE in the stage 3 spec.

RRC-6: model B without PC5 link

R2-2505174	Discussion on Multi-hop Discovery and (Re)selection	CATT	discussion	Rel-19	NR_SL_relay_multihop-Core

Noted

Proposal 2: (RRC-6) RAN2 confirms that the case for the last relay UE to check AS conditions before responding to intermediate relay UE’s discovery model B request is valid and the corresponding FFS can be deleted without further handling.

Discussion:

LG think the SA2 spec says the intermediate relay UE can respond if it already has a PC5 link with the parent, and in this case the intermediate relay UE has to check PC5-RSRP.  CATT think this is correct but not the same scenario, where we focus on the last relay UE.  LG suggest that we have to discuss the intermediate relay UE case as well.  CATT think we should focus on the RRC open issue.

OPPO can accept the proposal.

Proposal 3: In case the Last Relay UE needs to decide whether to send a discovery response message to the Intermediate Relay UE, RAN2 introduces a separate SD-RSRP threshold similar as the threshold used by the Intermediate Relay UE to determine whether to forward the solicitation message be applied.

Discussion:

OPPO think we could reuse the threshold configured for the intermediate relay.  CATT can follow majority view on this point.

Agreements:

RAN2 confirms that the case for the last relay UE to check AS conditions before responding to intermediate relay UE’s discovery model B request is valid and the corresponding FFS can be deleted without further handling.

In case the Last Relay UE needs to decide whether to send a discovery response message to the Intermediate Relay UE, it uses the same SD-RSRP threshold as the Intermediate Relay UE to determine whether to forward the solicitation message.

WA on state information in discovery message [resolved by earlier discussion]

R2-2505616	Discussion on discovery and relay reselection for multi-hop U2N relay	LG Electronics Inc.	discussion	Rel-19	NR_SL_relay_multihop

R2-2505844	Relay reselection and discovery under multihop relay 	Kyocera	discussion

Potential SA2 interaction (both documents moved from AI 8.13.3)

R2-2505932	Feasibility of Including RRC State Information through RRC Container in Discovery Messages for L2 Multi-hop U2N Relay	NIST	discussion	Rel-19	NR_SL_relay_multihop-Core

Noted

Proposal: RAN2 is kindly asked to discuss the three options for the intermediate relay to handle the RRC container in the discovery messages, taking into account the difference between Model A and Model B based solutions as pointed out in the Observation. Inform SA2 if needed

Option 1: No change to the RRC container received from the parent node, just forward.

Option 2: Add its own RRC state indication to the RRC container received.

Option 3: Replaces the RRC state indication in the RRC container received with its own RRC state indication.

Discussion:

NIST consider that we agreed the container will contain the state of only one node, but we may need to clarify if it is the last relay or the first relay, and this is not clearly specified in SA2; the intermediate relay could keep the same RRC container or modify it, and they think we should clarify for SA2.

Chair understood we agreed option 3.  Huawei have the same understanding.

ZTE think options 1 and 3 are both feasible in terms of the implementation details, because the UE can omit regenerating the container if it has the same state as the parent; they do not see that we need to notify SA2.

Huawei understand from SA2 colleagues that each intermediate relay UE can have its own RRC state and include it in the container.

Kyocera think we are talking about the state, not just the RRC_CONNECTED indication.

Agreement:

The RRC_CONNECTED indicator in the discovery message RRC container reflects the state of the UE that sends the discovery message.  The UE modifies the container if necessary to include this information, but this does not preclude a UE from forwarding the container unchanged if it has the same RRC_CONNECTED status as the UE from which it received the container.  Detailed spec impact can be discussed in the running CR implementation.

R2-2506044	Ensure L2 Multi-hop U2N relay operation conditions	Qualcomm Incorporated	discussion	NR_SL_relay_multihop-Core

Noted

Proposal 1	If the intermediate Relay UE has established PC5 connection for one RSC associated with one last Relay UE, the intermediate Relay UE does not transmit other RSC(s) in any discovery message.

Proposal 2	RAN2 discusses the two options for the case whether a UE has established connection via direct path can be act as an intermediate Relay UE.

Option 1: The intermediate Relay UE performs path switch from direct path to indirect path when receiving relay request from the Remote UE/child UE.

Option 2: The Relay UE does not forward discovery message received from a relay UE, that means the UE cannot be an intermediate Relay UE.

Proposal 3	Sends LS to SA2 to on the above two proposals.

Discussion:

Apple are not sure P1 is correct; they understand that the last relay may support multiple RSC in the discovery message towards the same node.

LG are not sure the intermediate relay UE uses just one RSC; they think once the intermediate relay UE has a PC5 link with one upstream UE, it cannot establish one towards another upstream UE.

OPPO support P1 to avoid cross-path topologies and think an LS does need to go to SA2.  On Apple’s point, they understand that from UE perspective it does not know whether the different RSCs correspond to different UEs.  Apple think the L2ID might be the same.

Qualcomm understand that different RSCs must have different L2IDs and different PC5 links, so they think multiple RSCs cannot be supported.

Huawei support the proposal.

OPPO think we could notify SA2 of the topology restriction and leave the details to them to implement.

Apple think neither option has SA2 impact.  Qualcomm think if we have a restriction on discovery transmission when the UE has a direct path, there is SA2 impact.

Qualcomm think if we interpret option 2 of P2, we would still need to notify SA2.  OPPO understand that a UE that has a direct path will not act as an intermediate relay due to existing agreements.  Qualcomm are concerned about the case where the UE has a direct path but does not satisfy the criteria for a last relay UE.

Samsung think the relay UE can reselect if it needs to.

OPPO understand that a connected UE is under network control for mobility, and for idle/inactive it is not clear what it would mean for a UE to have a direct path.  Qualcomm think the network does not know if it needs to switch the UE to an indirect path.

Agreement:

LS to SA2 to notify them that RAN2 excludes cross-path topologies for L2 and that a UE should not attempt to function as an intermediate relay UE towards two different parent nodes.  Leave to SA2 the possible spec impact to reflect this.

[Post131][407][Relay] LS to SA2 on cross-path topologies (Qualcomm)

Scope: Draft an LS to SA2 to notify them that RAN2 excludes cross-path topologies for L2 and that a UE should not attempt to function as an intermediate relay UE towards two different parent nodes.  Action is just to take into account (no reply expected).

Intended outcome: Approved LS

Deadline: Short (not for RP)

=> Approved in R2-2506569 (LS out)

R2-2506569	LS on topologies for L2 based multi-hop U2N relay	RAN2	LS out	Rel-19	NR_SL_relay_multihop-Core	To:SA2

=> Approved

Other contributions

R2-2505100	Discussion on Relay discovery and (re)selection	ZTE Corporation, Sanechips	discussion	Rel-19	NR_SL_relay_multihop

R2-2505341	Discovery and relay (re)selection for multi-hop U2N relay	OPPO	discussion	Rel-19	NR_SL_relay_multihop-Core

R2-2505433	Relay discovery and (re)selection for multi-hop Relay	Huawei, HiSilicon	discussion	Rel-19	NR_SL_relay_multihop-Core

R2-2505450	Discusison on Remaining issue on relay discovery and reselection	Apple	discussion	Rel-19	NR_SL_relay_multihop

R2-2505662	Multi-hop relay selection/re-selection	Sony	discussion	Rel-19	NR_SL_relay_multihop-Core

R2-2505697	Discussion on notification message	Lenovo	discussion	Rel-19

R2-2505732	Discovery (Model-B) forwarding thresholds for multi-hop U2N relay	Jio Platforms Limited	CR	Rel-19	38.331	18.6.0	5431	-	B	NR_SL_relay_multihop

R2-2505795	Discussion on relay discovery and relay (re)selection	Ericsson	discussion	Rel-19	NR_SL_relay_multihop

R2-2506019	Remaining issues and solutions on Relay discovery and (re)selection for multi-hop relay	Sharp	discussion	Rel-19	NR_SL_relay_multihop-Core

R2-2506036	Discussion on remaining issues on notification message	ASUSTeK	discussion	Rel-19	NR_SL_relay_multihop

R2-2506043	Open issues on relay discovery and (re)selection	Qualcomm Incorporated	discussion	NR_SL_relay_multihop-Core

R2-2506165	Relay (re)selection in multi-hop Relay	TOYOTA Info Technology Center	discussion	Rel-19

### 8.13.3	Control Plane Procedures and SRAP impact

Contributions should focus on control plane procedures and can include SRAP impact and QoS handling to support additional hops.

RRC-10: SFN-DFN offset

R2-2505698	Passing the SFN-DFN offset in multi-hop scenario	Lenovo	discussion	Rel-19

Noted

Proposal 1(RRC-10): support passing the SFN-DFN offset at the L2 U2N Relay UE or at the L2 Last U2N Relay UE in a multi-hop scenario.

Proposal 2(RRC-10): the intermediate relay UE sets sfn-DFN-OffsetSupported to supported once the intermediate relay UE obtains the SFN-DFN offset from the connected parent relay UE if the previous setting is absent.

R2-2505127	Remaining issues for Multi-hop Relay	NEC	discussion	Rel-19	NR_SL_relay_multihop

Noted

Proposal-1[RRC-10]: When the Remote UE requests the SFN-DFN offset from the connected parent Relay UE, the SFN-DFN offset provided at the L2 U2N Relay UE or the L2 Last U2N Relay UE should be compensated by the timing offset between the UEs when it is forwarded by intermediate U2N Relay UEs within the multi-hop relay chain.

Agreement:

For this WI, we do not specify forwarding of the SFN-DFN offset.  It can be addressed under TEI with the input of positioning experts.  RRC-10 can be closed.

RRC-13: timers

R2-2505434	Control plane procedures for multi-hop relay	Huawei, HiSilicon	discussion	Rel-19	NR_SL_relay_multihop-Core

Noted

Proposal 5: Extend the T300, T301 and T319 timer for multi-hop U2N relay operation in accordance with the hop count for connection establishment/ re-establishment and resume procedure by either

•	Multiplying the UE-TimersAndConstants with the hop count ( eg t300 * hop count) that includes only the Uu hop count; or

•	Introducing PC5-specific timers (t300-PC5/t301-PC5/t319-PC5) and calculating the timer value as t300 + t300-PC5*n, where n is the number of PC5 hops and one Uu hop is included.

Discussion:

Chair wonders about the meaning of “only the Uu hop count”.  Huawei clarify that it means legacy operation would use hop count 1.

InterDigital think the second option may be a bit overspecified; they would prefer to use the same timers, and they note that the Rel-17 timers already consider two hops, so multiplication looks like the wrong computation.

Samsung would prefer option 2.  ZTE agree and think this resolves concerns with the previous agreement.

CATT support option 1 to limit the spec impact.

OPPO understand the current running CR works and as long as the timer is long enough there should be no problem; they think no option is 100% perfect but they would prefer keeping the current CR.

vivo agree with OPPO and think the running CR can work, but they think it can be discussed in ASN.1 review.

LG agree with InterDigital.

Qualcomm and Lenovo agree with OPPO.

Ericsson think option 1 is simpler.

InterDigital think we could exclude option 2, and we could determine during ASN.1 review if the multiplication is performed on the Rel-17 timers or the legacy timers.

Samsung understand that the UE normally just applies the timer value from the network, but here it would be somewhat inferring the timer by itself and it changes the range of the timer; they have some concern about UE impact.  Huawei think this is analogous to what we did in Rel-17 and the range we have is good enough.

Samsung’s concern is that the timers may not have a well-defined maximum time.

Agreements:

Exclude the option of defining new multihop timers for T300/T301/T319.

No change to the running CR now in this respect; whether to multiply the Rel-17 timers or the legacy Uu-only timers can be confirmed in ASN.1 review.  RRC-13 can be closed.

Paging issues

R2-2505175	Discussion on the Control Plane Procedures	CATT	discussion	Rel-19	NR_SL_relay_multihop-Core

Noted

Proposal 1: Once the intermediate relay UE enters into RRC_CONNECTED mode, it should release the paging-related information in the parent for its child UEs if there is any.

Proposal 2:  When Last Relay UE in RRC CONNECTED and L2 Remote UE(s) or Intermediate Relay UE(s) in RRC_IDLE/RRC_INACTIVE, the Last Relay UE can monitor PO on behalf of each RRC_IDLE/RRC_INACTIVE intermediate relay UE or remote UE connected to it via multi-hop if the active DL BWP of Last Relay UE is configured with common CORESET and common search space.

Discussion:

Qualcomm understand this means the intermediate relay UE can monitor paging for the child, but they are not sure the coverage situation will support this well; frequent changes of coverage may mean the monitoring changes all the time, so they would prefer relying only on the last relay UE.

InterDigital are fine with the proposals and think they align with what we agreed for SI.  Qualcomm think paging is different because it is not for the intermediate itself but for the child UE.

Samsung understand P1 implies that once the intermediate goes to connected, it will stop forwarding paging even if it is already stored.

LG understand that for system information, the intermediate can forward information that it does not need for itself, but here it may not be essential to have the intermediate do the monitoring.

Apple think the intermediate cannot know what node will receive the paging for its child.  They think it can be left to relay implementation whether it releases the information or not.

Agreement:

Management of paging monitoring information at the relay UEs can be further checked in maintenance; no update to the running CR in this respect now.

Multi-hop QoS

R2-2506020	Remaining issues and solutions on C-plane procedure for multi-hop relay	Sharp	discussion	Rel-19	NR_SL_relay_multihop-Core

Noted

Proposal 6. QoS related optimization should be considered since very low E2E latency is required for some use cases.

Proposal 7. The principles of resource allocation follow the legacy mechanism, i.e., last relay can be configured with mode 1 resource allocation, and remote UE and other relay UEs are configured with mode 2 resource allocation.

Proposal 8. UE reports hop-by-hop link quality to gNB for path selection and split QoS configuration.

Discussion:

Samsung wonder about the specification impact; they think it is network implementation except for proposal 8.

Huawei think it is not really essential since the network can have information about the path from the individual UEs.

Apple have some uncertainty about the P8 impact since each UE reports its own PC5 conditions; they see that this information is already known to the gNB.

LG think P7 is the same as legacy operation; on P8, they agree with Apple.  Lenovo also agree with Apple.

Chair wonders if P7 has spec impact for us.

RLC channel configuration

R2-2505451	Discusison on Remaining UP and CP issues for multi-hop U2N relay	Apple	discussion	Rel-19	NR_SL_relay_multihop

Proposal 1 	RAN2 discuss whether each SL-RLC Channel configuration in sl-ConfigDedicatedNR for Intermediate relay UE is to be established in all N+1 PC5-RRC connections (N child node, 1 parent node). If not, what would be the correct UE behaviour in Rel-19.

R2-2505343	Control plane procedures of multi-hop U2N relay	OPPO	discussion	Rel-19	NR_SL_relay_multihop-Core

Noted

Proposal 5	Similar to SIB request, a paging information list is be introduced for the intermediate relay UE to carry the paging information from the downstream remote UEs as in the TP above.

Discussion:

Apple support the proposal to avoid sending a lot of individual requests.

Qualcomm think the UE implementation can decide whether to batch the signalling together.

ZTE wonder if the relay UE will hold paging requests for some time to build the list.  Apple think there are scenarios where a lot of requests come at the same time, e.g., for cancellation after link failure.  OPPO have the same understanding as Apple and intend to align with what we have for SIB requests.

Huawei wonder if the intermediate prefers sending the individual requests, if there is any point in the list.  Apple think it is most useful in the release case to save overhead.

Agreement:

Similar to SIB request, a paging information list is introduced for the intermediate relay UE to carry the paging information from the downstream remote UEs.  Signalling details can be checked in the ASN.1 review as usual.

SRAP-1: reflective bearer mapping

R2-2505102	Discussion on Reflective Bearer mapping of SL relay	ZTE Corporation, Sanechips, Ericsson, Apple, Nokia	discussion	Rel-19	NR_SL_relay_multihop

Noted

Proposal 1.	(SRAP-1)Re-use legacy remote UE add/mod/release list to configure indirect remote UE’s SRAP configuration in intermediate relay UE.

Proposal 2.	(SRAP-1)RAN2 to downselect the two options:

Option 1 - reflective bearer mapping is mandatory supported for indirectly connected child, no new field is needed to be included(TP in 4.2.1).

Option 2 - reflective bearer mapping is optional, we need to include a new field sl-DLNextHop-L2Identity-r19 in SRAP configuration, based on absence or presence of the field, it is indicated that the intermediate relay UE shall apply reflective bearer mapping or not(TP in 4.1 and 4.2.2).

[AT131][406][Relay] TP on reflective bearer mapping (ZTE)

Scope: F2F offline to check the TP from R2-2505102 for correctness/completeness.

Intended outcome: Report to Wednesday relay session in R2-2506312

Schedule: Tuesday 1030-1100 in Brk3

Deadline: Tuesday 2025-08-27 1900 IST

R2-2506312	Discussion on TP for Reflective Bearer mapping	ZTE Corporation	discussion	Rel-19	NR_SL_relay_multihop

Noted

Discussion:

OPPO have two concerns: RRC configuration (network includes the SRAP and RLC channel configurations, and the UE can apply them immediately; but under the proposal, the relay UE would have to wait to apply the configuration until the first uplink message), and error handling (relay UE needs to check that the received packet is from the correct ingress link, and this cannot be done in the reflective model, so an error will send all the following packets to the wrong egress link and the relay has no way to recover).

Apple think the RLC channel configuration is independent of the SRAP configuration anyway, and we do not have a requirement for them to be configured at the same time; on the error handling point, they do not have a big concern and think if the first uplink traffic has the wrong ID, it will not be handled by the gNB and there will be no traffic responding, so no error to detect in the relay UE.

Qualcomm have additional concerns; they think that reusing the legacy remote UE IDs to provide configurations for the other child UEs means that the relay UE knows the remote UE, and with the reflective model it only knows its directly connected UEs.

ZTE consider that on RLC channel configuration, the configuration from the network is not restricted to be applied together; on the error handling concern, they understand that OPPO assume the network configuration is reliable, but the network configuration is also based on the first packet, so the same problem could occur there; and on the third issue raised by Qualcomm, they understand there is no requirement to follow the legacy single-hop behaviour.

OPPO think the error case is about the mapping between the L2ID and the local ID, and the network relies on the reported L2ID; the difference is whether the routing table is based on the network configuration or on the first received packet.

OPPO understand that the network provides the local ID based on the L2ID, which should be reliable.  Apple think in the error case mentioned by OPPO, it can result in an erroneous entry, but it will never be used and will not present a real problem.

Ericsson think we already have the UE remembering the request for system information, and if the concern is right that this information is not reliable, we need to back out the previous agreement.

Qualcomm think we always trust the gNB configuration; if the first uplink packet is wrong, then the whole configuration will be wrong.  Apple think the uplink packet will have the wrong local ID.

ZTE think the first packet will be the RRC setup request, and the gNB can detect if the ID is wrong in this message and it will not respond to the problematic UE.

OPPO are not sure how the network detects the error.  Apple understand that the network allocates the local ID and can see if it is used by the wrong L2ID.  OPPO think the network cannot tell since the L2ID is not carried.

ZTE think this is not an issue for the reflective bearer mapping alone.  OPPO think in legacy operation, the SUI contains the L2ID and SRB0 is forwarded based on the allocated local ID, so the relay is following the mapping provided by the network, as opposed to the mapping being derived by the relay UE.

Apple think the network can recognise if the local ID comes from a different egress link.  ZTE think the message in an erroneous case may not be appropriate for the first RRC message, and the network can detect this.

OPPO are not sure how the network knows which link it came on, since for the network there is only the Uu link.

[Chair’s note: Discussion continues after further informal offline time]

ZTE understand after further discussion that the issue can be solved by other means, e.g., gNB implementation, but it is difficult to reach consensus.  They suggest that it could be an optional feature and vendors who do not consider it reliable will not support it.

OPPO think error cases can happen and we need to handle them; the baseline solution handles this situation correctly, but the reflective model does not detect it, and they see impact to the real UE’s RRC connection (blocked by the fake UE); they also do not see concrete gains over the baseline solution.

Apple think the error is in a very short time window, and it comes from an intermediate relay rather than a remote UE; the offending UE has passed authorisation, so we might not expect that it is malicious.

Samsung think we have spent a lot of time without reaching consensus, and they think we could leave it open to adopt this in maintenance if we can see solutions to solve the error case.

Ericsson are uncertain of the scenario, since the legacy security will prevent the connection being set up towards the wrong UE.

LG think the current CR is workable, and they wonder what the benefit of the reflective mapping is and/or what the problem with the current solution is.

Huawei think making it optional would be complicated because it requires considering the whole chain, where some UEs may support it and others not.  Apple think this can be addressed by signalling capabilities.

Qualcomm also see that the current CR works, and it is not clear why we talk about an additional feature; they share OPPO’s concern that the error handling will be compromised.

ZTE point out that the baseline solution was adopted exactly as a baseline, not a final agreement, to allow progress, but we have been considering the reflective solution.  They think the reflective solution reduces spec impact.

Show of hands: 4 companies (CATT, OPPO, Qualcomm, LG) cannot accept the reflective solution even if it is optional; no company thinks the system does not work without it.

ZTE think the first uplink packet is always unreliable.  OPPO consider that the mapping associated with it is reliable, i.e., the routing table will be right; the contents of the packet need to be checked by security as usual.

OPPO emphasise that nothing is broken without this feature.

Apple recall that we agreed to draft the running CR on the baseline solution, while keeping the reflective discussion open; they think the error case is not very serious and may not be different from the single-hop scenario, but there may be too much attachment to keeping the existing solution.

Qualcomm understand the previous agreement differently: They think we intended to adopt the reflective solution only if the baseline was broken, and here we are seeing that the baseline is not broken and the reflective solution has a risk.

ZTE think we started with the baseline partly because it had larger spec impact and needed time to implement and review; they note that in the L3 relay this may be a problem as well, and if SA3 can accept it for L3 they should be able to accept it for L2.

Qualcomm think security/reliability is one reason to specify L2 rather than L3.

Agreement:

Reflective bearer mapping is not adopted; maintenance of the SRAP configuration follows the currently implemented running CR.  SRAP-1 can be closed.

Fast failover proposal

R2-2505726	Fast failover via pre-configured egress candidate list for multi-hop L2 U2N relay	Jio Platforms Limited	CR	Rel-19	38.331	18.6.0	5430	-	B	NR_SL_relay_multihop

Other contributions

R2-2505086	(SRAP-1/RRC-13) Discussion on SRAP configuration and timer extension in multi-hop relay	vivo	discussion	Rel-19

R2-2505101	Discussion on paging for multi-hop relay	ZTE Corporation, Sanechips	discussion	Rel-19	NR_SL_relay_multihop

R2-2505342	SRAP configuration for multi-hop U2N Relay	OPPO	discussion	Rel-19	NR_SL_relay_multihop-Core

R2-2505419	Remaining Issues on Control Plane for Multi-Hop U2N Relays	InterDigital	discussion	Rel-19	NR_SL_relay_multihop

R2-2505617	Remaining issues on control plane procedure for SL relay	KT Corp.	discussion

R2-2505618	Discussion on the control plane procedure for multi-hop U2N relay	LG Electronics Inc.	discussion	Rel-19	NR_SL_relay_multihop

R2-2505759	Discussion on control plane procedures for multi-hop SL Relay	ZTE Corporation, Sanechips	discussion	Rel-19	NR_SL_relay_multihop

R2-2505774	Discussion on remaining issues of MH SL relay control plane procedures	Samsung	discussion	Rel-19	NR_SL_relay_multihop-Core

R2-2505775	Outstanding issues related to MH SRAP design	Samsung R&D Institute UK	discussion

R2-2505794	Discussion on control plane procedures	Ericsson	discussion	Rel-19	NR_SL_relay_multihop

R2-2505927	On SFN DFN offset and time sensitive applications	Nokia	discussion	NR_SL_relay_multihop

R2-2506037	Missing Intermediate U2N Relay UE behaviours upon sidelink radio link failure	ASUSTeK	discussion	Rel-19	38.331	NR_SL_relay_multihop

Revised in R2-2506199

R2-2506199	Missing Intermediate U2N Relay UE behaviours upon sidelink radio link failure	ASUSTeK	discussion	Rel-19	38.331	NR_SL_relay_multihop

### 8.13.4	Service continuity

First priority scenarios: (A) intra-gNB multi-hop indirect to direct path switch, (B) intra-gNB multi-hpo indirect to single-hop indirect path switch.  Second priority scenarios: (C) intra-gNB direct to multi-hop indirect path switch, (D) intra-gNB single-hop indirect to multi-hop indirect path switch.

RRC-14: reporting of target relays

R2-2505797	Service Continuity for Multi-Hop Relays	Ericsson	discussion	Rel-19	NR_SL_relay_multihop

Noted

Proposal 1	To discuss whether the agreements on RRC state of (first)intermediate/last Relay UE need to be captured in spec or can be up to gNB’s implementation to handle.

Discussion:

Huawei think we should capture something in the spec.  CATT think we have the principle in the stage 2, and how to guarantee it is up to gNB implementation.  Huawei agree it might be better in stage 2.

LG confirm that there is corresponding text in the stage 2.  Ericsson did not find it but can check offline.

Agreement:

The principle of path switch to a connected relay UE is documented in stage 2.  Companies can check offline if the current CR captures it adequately, and the wording can be fine-tuned in CR implementation.  RRC-14 can be closed.

Inter-gNB cases

Discussed jointly

R2-2505435	Discussion on service continuity for Multi-hop Relay	Huawei, HiSilicon	discussion	Rel-19	NR_SL_relay_multihop-Core

Noted

Proposal 5: Either the serving gNB shall only allow inter-gNB path switching from indirect (single-hop) to indirect (single-hop) paths for Rel-18 UEs, or RAN2 shall consider adding an indication in the measurement report to specify whether the candidate Relay UE is on a single-hop or multi-hop target path, or to provide the hop count information.

R2-2506021	Remaining issues and solutions on service continuity for multi-hop relay	Sharp	discussion	Rel-19	NR_SL_relay_multihop-Core

Noted

Proposal 5. The cellIdentity in sl-MeasResultRelay without enhancement can be used for decision whether the candidate first relay UE is connecting with the same gNB.

Discussion:

Huawei understand that the cell ID is not enough information to distinguish.

Samsung agree that the cell ID may not be adequate for the UE, but they think the network side can solve it and will know if the target is multihop or not.

OPPO think SA2 have already agreed that single-hop and multihop will use different RSCs, so a UE targeting multihop will not discover a single-hop relay.  Huawei think the UE cannot understand if the target is inter-gNB and the RSC does not help with this; in the inter-gNB case, the source gNB will not know if the target is single-hop or multihop.

Qualcomm have some sympathy with Huawei’s scenario, also considering that the single-hop target may not be connected.

Samsung think the target gNB will reject a handover for a target multihop path.

ZTE the UE can recognise the inter-gNB case if the gNB ID is indicated.

Ericsson think this is a bit of an optimisation and the target will reject.  Huawei think this requires extra signalling on Xn for the failed handovers.

Qualcomm think if a remote UE reports an idle candidate, the gNB does not know if it is single-hop or multihop, so the indication is useful for this case as well.  Samsung wonder what happens from the network side in this scenario.  Qualcomm understand that the gNB can select the idle target for single-hop cases only.

Lenovo think the RAN3 spec may not provide a suitable cause value for the multihop rejection on Xn, so the Huawei proposal makes sense.  OPPO understand that the rejection is still possible, and for the idle case mentioned by Qualcomm we can rely on the network.

Samsung are not sure the path switch to an idle relay can be allowed without somehow triggering paging before the path switch.  Qualcomm understand that we do allow idle targets in single-hop.  LG wonder how the target gNB can page the target knowing only the L2ID.

LG agree that the target should be in connected mode for multihop, and they think the gNB can handle it, but they are OK to discuss further in maintenance if something is broken.

Samsung note that it is a new issue and there could be RAN3 impact, and we should check in maintenance.

CATT agree with LG and Samsung that this is not a blocking issue.

Agreement:

No enhancement is added now to allow indicating the hop count of a candidate target relay UE to the gNB.  It can be discussed in maintenance if something is broken with the case where the target relay UE is in idle/inactive.

Path switch for relay UE

R2-2506045	Service continuity discussion	Qualcomm Incorporated	discussion	NR_SL_relay_multihop-Core

Noted

Proposal 2	The intermediate Relay UE can report measurement report UE to gNB for path switching to direct path or indirect path acting as a Remote.

Proposal 3	When the intermediate Relay UE performs path switching to direct path or indirect path, the intermediate Relay UE informs the Remote UE via the child intermediate relay UE with release cause 'RRC connection failure'.

Proposal 4	When the Remote UE receives notification from the relay UE, the Remote UE should go back to IDLE.

Discussion:

LG think the gNB has the context of the UEs, so it is different from group handover; they do not think the notification is needed.

InterDigital think the situation is similar to single-hop, where when the relay is handed over there is a notification to the remote UE.

Lenovo think P2 is legacy operation as long as the target indirect path is single-hop.  They think on P3, the legacy cause value could be reused, and on P4, the remote UE could re-establish instead of going to idle.

Ericsson have the same understanding as Lenovo and think P2 is a bit confusing: There is no restriction in the spec that would state otherwise.  They also think P3 is legacy behaviour except for the new cause value, which they are not sure is necessary; on P4, they think relay reselection could also be triggered as already captured in the spec.

LG think it is different from the single-hop case because the path switch for us will always be intra-gNB; they do not see it as group handover and think service continuity will still be possible.

OPPO think the notification is needed and the cause value should be handover; they think it is necessary to avoid group path switch even in the intra-gNB case, and the gNB implementation can choose to switch the remote UE(s) first.

Huawei think we should stick to the legacy behaviour.

Kyocera have some sympathy for LG’s view; they think re-establishment will usually bring the remote UE back to the same intermediate relay, so the notification seems a bit redundant.

Ericsson think we may be overengineering, and if the notification results in coming back to the same relay UE, that is just the outcome that happens.  They would like to stick to the notification behaviour.

Samsung and Lenovo think there is no specification impact if we keep the legacy behaviour.

Kyocera think this is not a group handover case.

LG still think it is an unnecessary disruption.

Lenovo think there are a lot of notification use cases that need to be considered.  InterDigital note that we are talking about the connected cases, so reselection does not apply; they think it is OK to forward what is sent to the child node.

Samsung wonder how the intermediate relay UE can know if its handover is intra- or inter-gNB.

Huawei think we should not over-optimize this case.

Agreements:

When the intermediate Relay UE receives a path switch command, the intermediate Relay UE sends a notification downstream using the handover cause value.  Handling of the notification is in accordance with the existing CR and legacy behaviour.

When the RRC_CONNECTED intermediate relay UE does re-establishment as a remote UE, the intermediate Relay UE sends a notification downstream, with the cause value to be confirmed in CR implementation.

Other contributions

R2-2505087	(RRC-14) Reporting of Target Relay UEs for scenario C and D	vivo	discussion	Rel-19

R2-2505176	Intra-gNB Service Continuity for Multi-hop U2N Relay	CATT	discussion	Rel-19	NR_SL_relay_multihop-Core

## 8.14	Additional topological enhancements

(NR_WAB_5GFemto; leading WG: RAN3; REL-19; WID RP-243009)

Time budget: 0 TU

Tdoc Limitation: 0 tdocs

Work on this WI will only be triggered by LS from RAN3 so work on this WI is not expected to start RAN2#127bis or RAN2#128.

No contributions expected for this meeting

R2-2505031	Reply LS on NR Femto node shared by PLMN and PNI-NPN (R3-252337; contact: LGE)	RAN3	LS in	Rel-19	5G_Femto, NR_WAB_5GFemto-Core, eNPN	To:SA2	Cc:RAN2

R2-2505055	Reply LS on FS_VMR_Ph2 solution impacts to RAN (Additional ULI) (S2-2504110; contact: Qualcomm)	SA2	LS in	Rel-19	VMR_Ph2	To:RAN3	Cc:RAN2

## 8.15	NavIC L1 SPS A-GNSS support

(LCS_NAVIC_L1_SPS_NR_LTE-Core; leading WG: RAN2; REL-19; WID RP-251552

Time budget: 0 TU

Tdoc Limitation: 1 tdoc

R2-2505776	Missing Capabilities for SSR Orbit/Clock Corrections Set2	Qualcomm Incorporated	discussion

Noted

R2-2505720	Introduction of NavIC L1 SPS A-GNSS in NR Stage 2 specification	Ericsson, Reliance Jio, ISRO, MediaTek Inc., CEWiT, Huawei	CR	Rel-19	38.305	18.6.0	0179	2	B	LCS_NAVIC_L1_SPS_NR_LTE-Core	R2-2504298

Agreed

R2-2505721	Introduction of NavIC L1 SPS A-GNSS in LTE Stage 2 specification	Ericsson, Reliance Jio, ISRO, MediaTek Inc., CEWiT, Huawei	CR	Rel-19	36.305	18.0.0	0120	2	B	LCS_NAVIC_L1_SPS_NR_LTE-Core	R2-2504299

Agreed

R2-2505722	Introduction of NavIC L1 SPS A-GNSS in LPP	Ericsson, Reliance Jio, ISRO, MediaTek Inc., CEWiT, Huawei	CR	Rel-19	37.355	18.5.0	0532	5	B	LCS_NAVIC_L1_SPS_NR_LTE-Core	R2-2504893

Revised in R2-2506430

R2-2506430	Introduction of NavIC L1 SPS A-GNSS in LPP	Ericsson, Reliance Jio, ISRO, MediaTek Inc., CEWiT, Huawei	CR	Rel-19	37.355	18.5.0	0532	6	B	LCS_NAVIC_L1_SPS_NR_LTE-Core	R2-2505722

Agreed

Agreement:

Rel-19 NavIC WI is complete from RAN2 point of view.

## 8.16	BDS B2b in A-GNSS

LCS_BDS_B2b_LTE_NR; leading WG: RAN2; REL-19; WID RP-250767)

Time budget: 0 TU

Tdoc Limitation: 1 tdoc

R2-2505094	Introduction of B2b signal in BDS system in A-GNSS	CAICT, CATT, CMCC, China Telecome, China Unicom, Ericsson, Huawei, HiSilicon, Lenovo, OPPO, vivo, Xiaomi, ZTE, MediaTek Inc, Qualcomm Incorporated	CR	Rel-19	37.355	18.5.0	0545	2	B	LCS_BDS_B2b_LTE_NR-Core	R2-2501435

Agreed

R2-2505095	Introduction of BDS B2b in A-GNSS	CAICT, CATT, CMCC, China Telecome, China Unicom, Ericsson, Huawei, HiSilicon, Lenovo, OPPO, vivo, Xiaomi, ZTE	CR	Rel-19	36.305	18.0.0	0121	1	B	LCS_BDS_B2b_LTE_NR-Core	R2-2410158

Revised in R2-2606314

R2-2506314	Introduction of BDS B2b in A-GNSS	CAICT, CATT, CMCC, China Telecome, China Unicom, Ericsson, Huawei, HiSilicon, Lenovo, OPPO, vivo, Xiaomi, ZTE, MediaTek Inc., Qualcomm Incorporated	CR	Rel-19	36.305	18.0.0	0121	2	B	LCS_BDS_B2b_LTE_NR-Core	R2-2415095

Agreed

R2-2505096	Introduction of BDS B2b in A-GNSS for TS 38305	CAICT, CATT, CMCC, China Telecome, China Unicom, Ericsson, Huawei, HiSilicon, Lenovo, OPPO, vivo, Xiaomi, ZTE	CR	Rel-19	38.305	18.6.0	0180	1	B	LCS_BDS_B2b_LTE_NR-Core	R2-2410159

Revised in R2-2506315

R2-2506315	Introduction of BDS B2b in A-GNSS for TS 38305	CAICT, CATT, CMCC, China Telecome, China Unicom, Ericsson, Huawei, HiSilicon, Lenovo, OPPO, vivo, Xiaomi, ZTE, MediaTek Inc., Qualcomm Incorporated	CR	Rel-19	38.305	18.6.0	0180	2	B	LCS_BDS_B2b_LTE_NR-Core	R2-2410159

Agreed

Agreement:

The Rel-19 BDS B2b WI is complete from RAN2 point of view.

## 8.17	IoT-NTN TDD mode

(IoT_NTN_TDD; leading WG: RAN1; REL-19; WID RP-243293)

Time budget: 0.5 TU

Tdoc Limitation: 1 tdoc

Including the outcome of the following email discussion:

[Post130][310][IoT NTN TDD] Stage2 CR (Iridium)

[Post130][311][IoT NTN TDD] RRC CR (Huawei)

[Post130][312][IoT NTN TDD] MAC CR (Toyota)

[Post130][313][IoT NTN TDD] 36.304 CR (Xiaomi)

[Post130][314][IoT NTN TDD] capability CR (Samsung)

Incoming LS

R2-2505018	LS on TP for 36.300 for IOT NTN TDD mode (R1-2504883; contact: Qualcomm)	RAN1	LS in	Rel-19	IoT_NTN_TDD-Core	To:RAN2

Noted

Stage 2 CR

R2-2505256	Stage 2 CR for Introduction of IoT NTN TDD mode	Iridium Satellite LLC	CR	Rel-19	36.300	18.5.0	1426	-	B	IoT_NTN_TDD-Core

-	Iridium indicates that RAN1 agreed some changes to the TP and they are sending a follow-up LS

Endorsed as a baseline for further discussion (including the discussion on the updated TP from RAN1)

36.331 CR

R2-2505248	Introduction of IoT NTN TDD mode	Huawei, HiSilicon	CR	Rel-19	36.331	18.6.0	5138	-	B	IoT_NTN_TDD-Core

Endorsed as a baseline for further discussion

36.321 CR

R2-2505250	Introduction of IoT NTN TDD mode	TOYOTA Info Technology Center	CR	Rel-19	36.321	18.4.0	1592	-	B	IoT_NTN_TDD-Core

Endorsed as a baseline for further discussion

36.304 CR

R2-2505111	Introduction of IoT NTN TDD mode	Xiaomi	CR	Rel-19	36.304	18.4.0	0883	-	B	IoT_NTN_TDD-Core

Endorsed as a baseline for further discussion

36.306 CR

R2-2505385	Report of [Post130][314][IoT NTN TDD] capability CR	Samsung	discussion

Proposal 1: (CAP-1) On the general IoT NTN TDD mode capability, RAN2 to decide on type of capability:

- Conditionally mandatory (as in draft CR)

- Optional capability without signalling

IoT NTN TDD mode capability is conditionally mandatory (as in the draft CR) if the UE supports the relevant TDD band

R2-2505148	Introduction of capabilities for IoT NTN TDD	Samsung	CR	Rel-19	36.306	18.5.0	1914	-	B	IoT_NTN_TDD-Core

Endorsed as a baseline for further discussion

- list of neighbour cells operating in TDD mode

R2-2505539	Discussion on new NB-IoT NTN TDD mode	Qualcomm Incorporated	discussion	Rel-19	IoT_NTN_TDD

Observation 1.	As NB-IoT does not support frequency priority, the UE may select TDD mode cell over FDD cell.

Proposal 4	Introduce a new list of neighbor cells operating in TDD mode for measurements and cell reselection.

-	Xiaomi wonders if we need such new specific list. ZTE agrees

-	Nokia thinks that something is needed

-	Iridium thinks this is not needed, and not even a time offset

-	HW thinks that at least the time offset might be needed

R2-2506176	Remaining issues for IoT-NTN TDD mode	THALES	discussion	Rel-19	IoT_NTN_TDD

Proposal 1: No need to introduce a new list of neighbour cells operating in TDD mode in Rel-19

No need to introduce a new list of neighbour cells operating in TDD mode in Rel-19

- Neighbor cell synchronization and measurement

R2-2505109	Discussion on support of IoT-NTN TDD mode	Xiaomi	discussion	Rel-19	IoT_NTN_TDD

Neighbor cell measurement

Proposal 1: RAN2 assume that for IoT NTN TDD mode, serving cell and neighbor cells are not always synchronized.

RAN2 assume that for IoT NTN TDD mode, serving cell and neighbor cells are not always synchronized.

Proposal 2: TDD radio frame offset between serving cell and neighbor cell(s) is provided to the UE for neighbor cell measurement.

-	Iridium thinks this is not needed

-	Thales thinks this is an optimization not necessary for the system to work

-	vivo thinks this was also discussed in RAN1 and there is no agreement this is needed

-	Xiaomi thinks that power consumption is a big concern in this case and then it’s critical to support this. Nokia agrees

-	HW thinks this can be an optional IE

-	QC thinks the UE should not be required to read MIB of neighbour cells just to measure cells

TDD radio frame offset between serving cell and neighbor cell(s) can be provided via SIB33 to the UE for neighbor cell measurement.

-	Xiaomi thinks we should inform other groups of this decision. Nokia, Iridium and Toyota think this is not needed

Proposal 3: If proposal 1 and 2 are agreeable, RAN2 can send LS to RAN1 for confirmation.

R2-2505539	Discussion on new NB-IoT NTN TDD mode	Qualcomm Incorporated	discussion	Rel-19	IoT_NTN_TDD

Proposal 5	Discuss how to provide the UEs with timing information of neighbor cell that is operating in NB-IoT TDD mode for measurements.

R2-2505634	Remaining issues on support of TDD mode for IoT-NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-19	IoT_NTN_TDD

Proposal 2: Severing cell to provide assistance information on the active periods of the neighbour cells configured for measurements. RAN2 to discuss the TS 36.331 TP.

R2-2505919	On open issues for IoT NTN TDD	Samsung	discussion	Rel-19	IoT_NTN_TDD

Proposal 3: RAN2 to wait for LS from RAN4 on providing UE with active periods of neighbouring cells.

R2-2505738	Remaining issues of IoT NTN TDD	Huawei, HiSilicon	discussion	Rel-19	IoT_NTN_TDD

Impact on Measurement

Proposal 1: RAN2 to confirm that inter-frequency neighboring cell measurement can be performed during the GAP configured outside the serving cell's available UL and DL subframes to avoid interfering the normal data transmission during the serving cell’s available UL and DL subframes.

- Misc open issues

R2-2505144	Final aspects on loT NTN TDD mode	Iridium Satellite LLC	discussion	Rel-19	IoT_NTN_TDD

Observation 1: There is no performance benefit in distributing the start paging subframes within D=8 subframes.

Proposal 1: There is no benefit in introducing a new idle mode DRX cycle length for IoT NTN TDD mode.

Agreed

Proposal 2: There is no specification impact for RAN2 (36.304) related to paging scheduling.

-	QC thinks we should at least say that paging might be postponed to the next available subframe. Xiaomi thinks this is already clear in RAN1 spec

Agreed

Proposal 3: There is no specification change required in the existing SIB1 scheduling mechanism.

Agreed

Proposal 4: No specification changes are required for providing neighbor cell information for cell reselection.

Proposal 5: SIB33-NB should be updated to provide extended k-mac value up to 1023ms for the neighbor cells

-	HW clarifies this is already captured in RRC

R2-2506176	Remaining issues for IoT-NTN TDD mode	THALES	discussion	Rel-19	IoT_NTN_TDD

Proposal 3: Preamble format 0 and 1 and NPRACH repetitions < 64 and >= 64 are supported for IoT NTN TDD mode, based on FDD procedures. Consider the table 5.1.4-1 modifications for TS 36.321 below:

Table 5.1.4-1: Subframes between preamble transmission and RA Response Window in NB-IoT

| TDD/FDD/IoT NTN TDD mode | Preamble format | Number of NPRACH repetitions | X |
| --- | --- | --- | --- |
| FDD/IoT NTN TDD | 0 or 1 | >= 64 | 41 |
| FDD/IoT NTN TDD | 0 or 1 | < 64 | 4 |
| FDD | 2 | >= 16 | 41 |
| FDD | 2 | < 16 | 4 |
| TDD | Any | Any | 4 |

-	Toyota confirms this is already captured in the MAC CR

R2-2505919	On open issues for IoT NTN TDD	Samsung	discussion	Rel-19	IoT_NTN_TDD

Proposal 1: Change the value range of k-Mac-r19 to 513...1024.

Not agreed

Proposal 2: RAN2 does not specify any enhancements related to GNSS measurement gaps for IoT NTN TDD.

R2-2505287	Remaining issues for IoT NTN TDD	ZTE Corporation,  Sanechips	discussion	Rel-19	IoT_NTN_TDD

Proposal 2: The repetitions of SC-PTM message transmission falling on the non-D subframes are postponed to the next valid D subframe within the transmission Period.

Can come back if we see a need to clarify the situation for SC-PTM

R2-2505539	Discussion on new NB-IoT NTN TDD mode	Qualcomm Incorporated	discussion	Rel-19	IoT_NTN_TDD

Proposal 1	Clarify that the explicit epoch time is associated with the SFN where SIB1 was originally scheduled to be transmitted.

-	Nokia thinks the NW can ensure there is no problem. Iridium agrees

-	Xiaomi doesn’t think a clarification is needed

RAN2 assumes the network will take into account the invalid subframes when setting the explicit EpochTime

Proposal 2	To avoid paging delay due to collision of postponed paging transmission with the scheduled DL transmission, UE is configured whether to monitor paging in next valid DL frames of next SFN X and SFN X+1 (if no paging in SFN X).

Proposal 3	For SPS interval, the values of semiPersistSchedIntervalUL-r15 are interpreted as new values that are multiple of 90ms.

R2-2505634	Remaining issues on support of TDD mode for IoT-NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-19	IoT_NTN_TDD

Proposal 1: The start of the RAR window and Contention Resolution timer shall be aligned with the start of the DL occasions, when the UE-eNB RTT is shorter than the UL-DL gap. RAN2 to discuss the TP implementation.

Proposal 3: RAN2 to discuss reusing the AS operation from GNSS measurement gap for RRC Connected UE working in NB-IoT TDD NTN, except the neighbor cell measurement which is possible outside the serving cell’s UL-DL pair. The TP for TS 36.300 can be a starting point.

Proposal 4: RAN2 to discuss handling of the PUR & SPS postponement including postponement to another valid UL subframe to avoid UL collision between different UEs.

- Rel-19 features applicable to IoT NTN TDD

R2-2506176	Remaining issues for IoT-NTN TDD mode	THALES	discussion	Rel-19	IoT_NTN_TDD

Proposal 4: OCC defined for IoT NTN Rel-19 can be supported for IoT NTN TDD mode (No spec impact)

RAN2 assumes that OCC defined for IoT NTN Rel-19 can be supported for IoT NTN TDD mode but ask RAN1 for confirmation

Proposal 5: Support the CB-Msg3 and CB-Msg4 for IoT NTN TDD mode, with one of the adaptations below:

Option a) Add configurations that are aligned with the 90 ms periodicity for npusch-Periodicity and windowPeriodicity-NB

Option b) Support postponement of CB-Msg3-EDT occasions to the next valid D frame within the transmission window when the number of replicas > 1

-	QC thinks we could consider option A

We aim at introducing the needed changes (if any) to support CB-Msg3 and CB-Msg4 for IoT NTN TDD mode during the ASN.1 review, including at least

    - Adding configurations that are aligned with the 90 ms periodicity for npusch-Periodicity and windowPeriodicity-NB

- (FFS other needed enhancements based on the progress of the CB-Msg3-EDT discussion this week)

Proposal 6: IoT NTN PWS Rel-19 can be supported in IoT NTN TDD (no spec impact)

Agreed

[Post131][321][IoT NTN TDD] LS to OCC support (Huawei)

Scope: Draft the LS to RAN1 on RAN2 agreement regarding OCC support for IoT NTN TDD

Intended outcome: Approved LS (in R2-2506300)

Deadline: short

=> Approved in R2-2506300 (LS out)

R2-2506300	LS on OCC for IoT-NTN TDD mode	RAN2	LS out	Rel-19	IoT_NTN_TDD-Core	To:RAN1

=> Approved

- Open issue CAP-1

R2-2505232	Discussion on support of NB-IoT NTN TDD	CATT	discussion	Rel-19	IoT_NTN_TDD

Proposal 2: (CAP-1) IoT NTN TDD mode capability can be optional capability without signaling, and a statement can be included in capability specification to declare that UE supporting band n249 must support IoT NTN TDD feature group.

R2-2505287	Remaining issues for IoT NTN TDD	ZTE Corporation,  Sanechips	discussion	Rel-19	IoT_NTN_TDD

Proposal 1: Define the type of UE capability as conditionally mandatory method (i.e. as in the TS36.306 running CR).

- On early implementation of the IoT-NTN TDD mode

R2-2505701	On early implementation of the IoT-NTN TDD mode	Nordic Semiconductor, Iridium, Thales	discussion	Rel-19	IoT_NTN_TDD

Observation-1: The UE capability of the IoT-NTN TDD mode is implicitly associated with the used TDD band.

Observation-2: The network capability of the IoT-NTN TDD mode is implicitly associated with the used TDD band.

Observation-3: There is no UE IoT-NTN TDD mode capability or band support ambiguity from the network point of view, since supporting UEs are only being served on specific IoT-NTN TDD mode bands.

Observation-4: The Rel-19 IoT-NTN TDD mode can be implemented on top of Rel-17.

Proposal-1: Document in 3GPP TS36.331 Annex G that Rel-19 IoT-NTN TDD mode is early implementable feature starting from Rel-17 for a UE supporting a IoT-NTN TDD mode band.

-	Iridium thinks that this feature is tied to a band and then it should be possible to have this early implementable from Rel-17

-	QC thinks that early implementation for a single CR is one thing, but for a whole feature/WI is not so common so has some doubts about this.

-	Samsung reminds we also agreed to support Cb-Msg3-EDT for IoT NTN TDD, and this is definitely not a Rel-17 feature

-	vivo thinks RAN2 cannot decide alone on this

Agreements:

1.	IoT NTN TDD mode capability is conditionally mandatory (as in the draft CR) if the UE supports the relevant TDD band

2.	No need to introduce a new list of neighbour cells operating in TDD mode in Rel-19

3.	RAN2 assume that for IoT NTN TDD mode, serving cell and neighbor cells are not always synchronized.

4.	There is no benefit in introducing a new idle mode DRX cycle length for IoT NTN TDD mode.

5.	There is no specification impact for RAN2 (36.304) related to paging scheduling.

6.	There is no specification change required in the existing SIB1 scheduling mechanism.

7.	RAN2 assumes the network will take into account the invalid subframes when setting the explicit EpochTime

8.	RAN2 assumes that OCC defined for IoT NTN Rel-19 can be supported for IoT NTN TDD mode but ask RAN1 for confirmation

9.	We aim at introducing the needed changes (if any) to support CB-Msg3 and CB-Msg4 for IoT NTN TDD mode during the ASN.1 review, including at least

Adding configurations that are aligned with the 90 ms periodicity for npusch-Periodicity and windowPeriodicity-NB

(FFS other needed enhancements based on the progress of the CB-Msg3-EDT discussion this week)

10.	IoT NTN PWS Rel-19 can be supported in IoT NTN TDD (no spec impact)

11.	TDD radio frame offset between serving cell and neighbor cell(s) can be provided via SIB33 to the UE for neighbor cell measurement.

[Post131][310][IoT NTN TDD] Stage2 CR (Iridium)

Scope: finalize the running Stage 2 CR

Intended outcome: Agreed CR

Deadline: short

=> Agreed in R2-2506497 (36.300)

[Post131][311][IoT NTN TDD] RRC CR (Huawei)

Scope: finalize the running RRC CR

Intended outcome: Agreed CR

Deadline: short

=> Agreed in R2-2506558 (36.331)

[Post131][312][IoT NTN TDD] MAC CR (Toyota)

Scope: finalize the running MAC CR

Intended outcome: Agreed CR

Deadline: short

=> Agreed in R2-2506523 (36.321)

[Post131][313][IoT NTN TDD] 36.304 CR (Xiaomi)

Scope: finalize the running 36.304 CR

Intended outcome: Agreed CR

Deadline: short

=> Agreed in R2-2506542 (36.304)

[Post131][314][IoT NTN TDD] capability CR (Samsung)

Scope: finalize the running capability CR

Intended outcome: Agreed CR

Deadline: short

=> Agreed in R2-2506554 (36.306)

R2-2506497	Introduction of IoT NTN TDD mode	Iridium Satellite LLC	CR	Rel-19	36.300	18.5.0	1426	1	B	IoT_NTN_TDD-Core

=> Agreed

R2-2506558	Introduction of IoT NTN TDD mode	Huawei, HiSilicon	CR	Rel-19	36.331	18.6.0	5138	1	B	IoT_NTN_TDD-Core

=> Agreed

R2-2506523	Introduction of IoT NTN TDD mode	TOYOTA ITC	CR	Rel-19	36.321	18.4.0	1592	1	B	IoT_NTN_TDD-Core

=> Agreed

R2-2506542	Introduction of IoT-NTN TDD mode	Xiaomi	CR	Rel-19	36.304	18.4.0	0883	1	B	IoT_NTN_TDD-Core

=> Agreed

R2-2506554	Introduction of capabilities for IoT NTN TDD	Samsung	CR	Rel-19	36.306	18.5.0	1914	1	B	IoT_NTN_TDD-Core

=> Agreed

From RAN2 point of view the IoT_NTN_TDD WI is completed.

R2-2505109	Discussion on support of IoT-NTN TDD mode	Xiaomi	discussion	Rel-19	IoT_NTN_TDD

R2-2505144	Final aspects on loT NTN TDD mode	Iridium Satellite LLC	discussion	Rel-19	IoT_NTN_TDD

R2-2505232	Discussion on support of NB-IoT NTN TDD	CATT	discussion	Rel-19	IoT_NTN_TDD

R2-2505287	Remaining issues for IoT NTN TDD	ZTE Corporation,  Sanechips	discussion	Rel-19	IoT_NTN_TDD

R2-2505539	Discussion on new NB-IoT NTN TDD mode	Qualcomm Incorporated	discussion	Rel-19	IoT_NTN_TDD

R2-2505553	Discussion on IoT NTN TDD mode	OPPO	discussion	Rel-19	IoT_NTN_TDD

R2-2505634	Remaining issues on support of TDD mode for IoT-NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-19	IoT_NTN_TDD

R2-2505701	On early implementation of the IoT-NTN TDD mode	Nordic Semiconductor, Iridium, Thales	discussion	Rel-19	IoT_NTN_TDD

R2-2505738	Remaining issues of IoT NTN TDD	Huawei, HiSilicon	discussion	Rel-19	IoT_NTN_TDD

R2-2505919	On open issues for IoT NTN TDD	Samsung	discussion	Rel-19	IoT_NTN_TDD

R2-2505960	Remaining issues on support of IoT-NTN TDD mode	CMCC	discussion	Rel-19	IoT_NTN_TDD

R2-2506176	Remaining issues for IoT-NTN TDD mode	THALES	discussion	Rel-19	IoT_NTN_TDD

R2-2505143	DraftCR 36300 IoT NTN TDD	Iridium Satellite LLC	draftCR	Rel-19	36.300	18.5.0	B	IoT_NTN_TDD-Core

Withdrawn

## 8.18	LTE-based 5G Broadcast

(LTE_terr_bcast_Ph2; leading WG: RAN1; REL-19; WID RP-250794)

Time budget: 0.25 TU

Tdoc Limitation: 1 tdoc

### 8.18.1	Organizational

Incoming LS, rapporteur input, running CRs, etc.

Incoming LS

R2-2505022	LS on RAN2 aspects for LTE-based 5G Broadcast Phase 2 (R1-2504922; contact: EBU)	RAN1	LS in	Rel-19	LTE_terr_bcast_Ph2	To:RAN2

Noted

Running CRs

R2-2505411	Introduction of LTE-based 5G Broadcast Phase 2	Qualcomm Incorporated, EBU	CR	Rel-19	36.331	18.6.0	5143	-	B	LTE_terr_bcast_Ph2-Core

R2-2505556	Introduction of LTE-based 5G Broadcast Phase 2	Samsung	CR	Rel-19	36.321	18.4.0	1593	-	B	LTE_terr_bcast_Ph2-Core

R2-2505740	Introduction of LTE-based 5G Broadcast Phase 2	Huawei, HiSilicon	CR	Rel-19	36.306	18.5.0	1920	-	B	LTE_terr_bcast_Ph2

R2-2505741	Introduction of LTE-based 5G Broadcast Phase 2	Huawei, HiSilicon, Qualcomm Incorporated	CR	Rel-19	36.331	18.6.0	5144	-	B	LTE_terr_bcast_Ph2

R2-2505799	Introduction of LTE-based 5G Broadcast Phase 2	ZTE Corporation, Sanechips	CR	Rel-19	36.300	18.5.0	1428	-	B	LTE_terr_bcast_Ph2-Core

The above running CRs are endorsed

RRC-4: RRC capability CR will be merged into overall RRC CR

- Lenovo asks if we need separate RRC capability CR.

- QCM clarifies that we can merge with overall RRC CR.

[POST131][511][LTE Broadcast] Final 36.331 CR (Qualcomm)

Scope: Produce a final CR for LTE Broadcast

Intended outcome: CR for agreement in R2-2506344

Deadline:  Short

=> Agreed in R2-2506344 (36.331)

R2-2506344	Introduction of LTE-based 5G Broadcast Phase 2	Qualcomm Incorporated, EBU	CR	Rel-19	36.331	18.6.0	5143	1	B	LTE_terr_bcast_Ph2-Core

=> Agreed

[POST131][512][LTE Broadcast] Final 38.321 CR (Samsung)

Scope: Produce a final CR for LTE Broadcast

Intended outcome: CR for agreement in R2-2506345

Deadline:  Short

=> Agreed in R2-2506348 (36.321)

R2-2506348	Introduction of LTE-based 5G Broadcast Phase 2	Samsung	CR	Rel-19	36.321	18.4.0	1593	2	B	LTE_terr_bcast_Ph2-Core

=> Agreed

[POST131][513][LTE Broadcast] Final 38.306 CR (Huawei)

Scope: Produce a final CR for LTE Broadcast

Intended outcome: CR for agreement in R2-2506346

Deadline:  Short

=> Agreed in R2-2506346 (36.306)

R2-2506346	Introduction of LTE-based 5G Broadcast Phase 2	Huawei, HiSilicon	CR	Rel-19	36.306	18.5.0	1920	1	B	LTE_terr_bcast_Ph2

=> Agreed

[POST131][514][LTE Broadcast] Final 38.300 CR (ZTE)

Scope: Produce a final CR for LTE Broadcast

Intended outcome: CR for agreement in R2-2506347

Deadline:  Short

=> Agreed in R2-2506347 (36.300)

R2-2506347	Introduction of LTE-based 5G Broadcast Phase 2	ZTE Corporation, Sanechips	CR	Rel-19	36.300	18.5.0	1428	1	B	LTE_terr_bcast_Ph2-Core

=> Agreed

Open issue lists

R2-2505412	[POST130][510][LTE Broadcast] RRC Open Issues	Qualcomm Incorporated, EBU	report	Rel-19	LTE_terr_bcast_Ph2-Core

R2-2505554	[POST130][511][LTE Broadcast] MAC Open Issues	Samsung	report

Two documents above are noted

Open issues discussed based on company Tdocs

WI completion:

From RAN2 point of view WI can be closed

### 8.18.2	Other

RAN2 signalling impacts to support time-frequency interleavers.

RRC open issues

R2-2505413	Views on RRC and MAC Open Issues	Qualcomm Incorporated	discussion	Rel-19	LTE_terr_bcast_Ph2-Core

Proposal 3: For open issue RRC-1: Introduce Rel-19 specific MBS Interest Indication IE (MBMSInterestIndication-v19xy-IEs) taking the TP from R2-2505413 as baseline.

Proposal 4: For open issue RRC-2: No RAN2 spec impact.

Proposal 5: For open issue RRC-3: Wait for RAN1 further update, if any, and finalize during RRC CR review.

On P3:

- Samsung indicates there was some agreement in RAN1 already which we can capture.

On P2:

- ZTE has different understanding. ZTE thinks it should be captured, e.g. as captured in stage-2 CR currently.

For open issue RRC-1: Introduce Rel-19 specific MBS Interest Indication IE (MBMSInterestIndication-v19xy-IEs) taking the TP from R2-2505413 as baseline. We will consider RAN1 agreements. Consider also whether procedural changes, e.g. as in R2-2505557 are needed.

RRC-2: Capture the RAN1 agreement in stage-2 running CR.

For open issue RRC-3: Wait for RAN1 further update, if any, and finalize during RRC CR review.

R2-2505557	Way forward on remaining issues for RRC and MAC	Samsung	discussion	Rel-19

[RRC-1] Proposal 3: RAN2 to capture the procedural and signalling impact for the Rel-19 IE of MBMS Interest Indication. (Adopt TP2)

[RRC-2] Proposal 4: RAN2 specification need not capture anything for time-interleaved PMCH transmission (as configured) exceeding UE’s soft buffer size or supported TBS and consider RAN1 agreement as informative.

[RRC-3] Proposal 5: RAN2 need to wait for RAN1 progress about cyclic shift parameter for PMCH. Maintain the current placeholder in RRC specification.

MAC-1 (HARQ handling)

R2-2505413	Views on RRC and MAC Open Issues	Qualcomm Incorporated	discussion	Rel-19	LTE_terr_bcast_Ph2-Core

Proposal 1: For open issue MAC-1: Incorporate the rapporteur’s proposed changes #1 and #2 from the email discussion report R2-2505554.

R2-2505557	Way forward on remaining issues for RRC and MAC	Samsung	discussion	Rel-19

Proposal 1: RAN2 to specify the HARQ handling for Rel-19 time-interleaved MCH transmission in MAC specification. (Adopt TP 1)

- ZTE think this is over-specification. We did not specify this for MBS previously. ZTE thinks even without this it works. QCM agrees – the change in the figure would be sufficient.

- Samsung thinks this is a new mechanism, so this is why it is different.

MAC-1: RAN2 to specify the HARQ handling for Rel-19 time-interleaved MCH transmission in MAC specification. (Adopt TP 1 as baseline, can attempt to simplify)

R2-2505739	Discussion on time-frequency interleavers for MBMS	Huawei, HiSilicon	discussion	Rel-19	LTE_terr_bcast_Ph2

Proposal 2: No need to introduce HARQ handling specific to interleaving MBMS.

R2-2505800	Open issues in MAC layer on supporting TFI	ZTE Corporation, Sanechips	discussion	Rel-19	LTE_terr_bcast_Ph2

Proposal 1	Do not specify HARQ handling in case of time interleaving for LTE eMBMS.

MAC-2 (Extended MSI)

R2-2505413	Views on RRC and MAC Open Issues	Qualcomm Incorporated	discussion	Rel-19	LTE_terr_bcast_Ph2-Core.

Proposal 2: For open issue MAC-2: Mirror the same changes from 6.1.3.7 to 6.1.3.7a in MAC CR.

R2-2505557	Way forward on remaining issues for RRC and MAC	Samsung	discussion	Rel-19

[MAC-2] Proposal 2: To support implementation flexibility, reflect the MSI description changes also to extended MSI description.

R2-2505800	Open issues in MAC layer on supporting TFI	ZTE Corporation, Sanechips	discussion	Rel-19	LTE_terr_bcast_Ph2

Proposal 2	Do not specify Extended MSI MAC CE enhancements in case of time interleaving for LTE eMBMS.

- ZTE thinks that extended MSI is for different use case than dedicated MBMS.

- QCM indicates that in RRC CONNECTED the UEs can also receive time-interleaved service.

- Samsung thinks we do not have to restrict network flexibility.

- Huawei has no strong view, but tend to think it is OK to capture.

MAC-2: Mirror the same changes from 6.1.3.7 to 6.1.3.7a in MAC CR.

MAC – other

R2-2505739	Discussion on time-frequency interleavers for MBMS	Huawei, HiSilicon	discussion	Rel-19	LTE_terr_bcast_Ph2

Proposal 1: Following RAN1 agreement, RAN2 should capture in MAC that the number of subframes used for time-interleaving derived from ‘stop MTCH (x+1)’ is expected to be an integer multiple of MxN.

- QCM thought it is obvious from procedures, but after discussing offline is OK to capture.

Following RAN1 agreement, RAN2 should clarify in MAC that the number of subframes used for time-interleaving derived from ‘stop MTCH (x+1)’ is expected to be an integer multiple of MxN.

Other issues

R2-2505739	Discussion on time-frequency interleavers for MBMS	Huawei, HiSilicon	discussion	Rel-19	LTE_terr_bcast_Ph2

Proposal 3: A separate frequency list is introduced in SIB15 to indicate provision of MBMS with interleaving, following similar principle as legacy.

Noted, not needed

- QCM does not see a clear use case where the issue happens. There are no legacy UEs to handle.

- Samsung shares view with QCM. ZTE agrees.

Agreements on RRC open issues - RRC-4: RRC capability CR will be merged into overall RRC CR - For open issue RRC-1: Introduce Rel-19 specific MBS Interest Indication IE (MBMSInterestIndication-v19xy-IEs) taking the TP from R2-2505413 as baseline. We will consider RAN1 agreements. Consider also whether procedural changes, e.g. as in R2-2505557 are needed. - RRC-2: Capture the RAN1 agreement in stage-2 running CR. - For open issue RRC-3: Wait for RAN1 further update, if any, and finalize during RRC CR review. Agreements on MAC open issues - MAC-1: RAN2 to specify the HARQ handling for Rel-19 time-interleaved MCH transmission in MAC specification. (Adopt TP 1 as baseline, can attempt to simplify) - MAC-2: Mirror the same changes from 6.1.3.7 to 6.1.3.7a in MAC CR. - Following RAN1 agreement, RAN2 should clarify in MAC that the number of subframes used for time-interleaving derived from ‘stop MTCH (x+1)’ is expected to be an integer multiple of MxN.

## 8.19	TEI19

Time budget: 1 TU

Tdoc Limitation: 1 tdoc for new proposals and 1 tdoc for old proposals for RAN2-led.

1 additional tdoc for primary co-sourcing company on top of the limit is allowed for co-sourced contribution with 4 or more companies.

Companies are encouraged to submit co-sourced contributions, which will have priority for discussion in RAN2#130

### 8.19.1	RAN2-led

[UAV]

R2-2505220	Discussion on TEI19 UAV mobility enhancements	CATT, NTT DOCOMO, LG Electronics Inc., Kyocera, LGU+, China Telecom, NEC, SK Telecom, Qualcomm Incorporated, CMCC, Ericsson, Nokia	discussion	Rel-19	TEI19

handover.

Proposal 1: One indication is introduced in SIB2 to inform whether the serving frequency is UAV dedicated frequency, and another per frequency indication is introduced in SIB4 to inform it is a UAV dedicated frequency.

Proposal 2: UAV UE may consider the UAV dedicated frequency to be the highest priority for cell reselection. Optionally, an altitude range can be configured by network and only when a UAV UE is within this altitude range, it may prioritize UAV frequency for cell reselection.

Proposal 3: for the idle/inactive UE, ssb-ToMeasureAltitudeBasedList is configured in SIB2 and SIB4 (for each frequency).

Proposal 4: If ssb-ToMeasureAltitudeBasedList is configured in SIB2 or SIB4, when the idle/inactive UE is within an altitude range indicated by altitudeRange, it may prioritize the measurements on the SSB(s) configured in ssb-ToMeasure-r18 (if present). Otherwise (i.e., the idle/inactive UE is within an altitude range indicated by altitudeRange and ssb-ToMeasure-r18 is absent) it measures on all SS-blocks. When the idle/inactive UE is outside all the altitude ranges indicated by altitudeRange (if any), ssb-ToMeasure (without suffix) applies.

-	Docomo supports this as there is internal demand for cell reselection enhancement.

Proposal 5: Define the following altitude-based CHO trigger events for UAV

-	condEvenA3H1, condEventA3H2, condEvenA5H1, condEventA5H2

On conditional handover

-	Samsung doesn’t think CHO enhancements is necessary.  Interdigital thinks the spec changes are small and it does have some benefits.  Samsung thinks that whats important is the technical benefit.  The network can provide the command on time as the communication on time.   Interidigital thinks that the fact the network knows where the UAV is an andvantage for using CHO.    Nokia thinks that there is a use case..

-	Qualcomm agrees it may not strictly necessary but it is beneficial.

-	Huawei doesn’t see the need but will go with the majority.

Cell reselection and CHO enhancement will be address in TEI19

Agreements

1	One indication is introduced in SIB2 to inform whether the serving frequency is UAV dedicated frequency, and another per frequency indication is introduced in SIB4 to inform it is a UAV dedicated frequency.

2	UAV UE may consider the UAV dedicated frequency to be the highest priority for cell reselection. Optionally, an altitude range can be configured by network and only when a UAV UE is within this altitude range, it may prioritize UAV frequency for cell reselection.

3	for the idle/inactive UE, ssb-ToMeasureAltitudeBasedList is configured in SIB2 and SIB4 (for each frequency).

4	If ssb-ToMeasureAltitudeBasedList is configured in SIB2 or SIB4, when the idle/inactive UE is within an altitude range indicated by altitudeRange, it may prioritize the measurements on the SSB(s) configured in ssb-ToMeasure-r18 (if present). Otherwise (i.e., the idle/inactive UE is within an altitude range indicated by altitudeRange and ssb-ToMeasure-r18 is absent) it measures on all SS-blocks. When the idle/inactive UE is outside all the altitude ranges indicated by altitudeRange (if any), ssb-ToMeasure (without suffix) applies.

5	Define the following altitude-based CHO trigger events for UAV

-	condEvenA3H1, condEventA3H2, condEvenA5H1, condEventA5H2

R2-2505221	Introduction of UAV mobility enhancements [UAV_Mobility]	CATT, NTT DOCOMO, LG Electronics Inc., Kyocera, LGU+, China Telecom, NEC, SK Telecom, Qualcomm Incorporated, Ericsson, Nokia	CR	Rel-19	38.300	18.6.0	1004	-	B	TEI19

R2-2505222	Introduction of UAV mobility enhancements [UAV_Mobility]	CATT, NTT DOCOMO, LG Electronics Inc., Kyocera, LGU+, China Telecom, NEC, SK Telecom, Qualcomm Incorporated, Ericsson, Nokia	CR	Rel-19	38.304	18.4.0	0439	-	B	TEI19

R2-2505223	Introduction of UAV mobility enhancements [UAV_Mobility]	CATT, NTT DOCOMO, LG Electronics Inc., Kyocera, LGU+, China Telecom, NEC, SK Telecom, Qualcomm Incorporated, Ericsson, Nokia	CR	Rel-19	38.306	18.6.0	1319	-	B	TEI19

R2-2505224	Introduction of UAV mobility enhancements [UAV_Mobility]	CATT, NTT DOCOMO, LG Electronics Inc., Kyocera, LGU+, China Telecom, NEC, SK Telecom, Qualcomm Incorporated, Ericsson, Nokia	CR	Rel-19	38.331	18.6.0	5399	-	B	TEI19

[AT131][012][TEI19] UAV CRs (CATT)

Intended outcome: Agree to UAV CRs by email

Deadline:  Thursday

R2-2506461	Introduction of UAV mobility enhancements [UAV_Mobility]	CATT, NTT DOCOMO, LG Electronics Inc., Kyocera, LGU+, China Telecom, NEC, SK Telecom, Qualcomm Incorporated, Ericsson, Nokia, InterDigital, vivo, CMCC	CR	Rel-19	38.300	18.6.0	1004	1	B	TEI19	R2-2505221

The CR is agreed

R2-2506462	Introduction of UAV mobility enhancements [UAV_Mobility]	CATT, NTT DOCOMO, LG Electronics Inc., Kyocera, LGU+, China Telecom, NEC, SK Telecom, Qualcomm Incorporated, Ericsson, Nokia, InterDigital, vivo, CMCC	CR	Rel-19	38.304	18.4.0	0439	1	B	TEI19	R2-2505222

The CR is agreed

R2-2506463	Introduction of UAV mobility enhancements [UAV_Mobility]	CATT, NTT DOCOMO, LG Electronics Inc., Kyocera, LGU+, China Telecom, NEC, SK Telecom, Qualcomm Incorporated, Ericsson, Nokia, InterDigital, vivo, CMCC	CR	Rel-19	38.306	18.6.0	1319	1	B	TEI19	R2-2505223

The CR is agreed

R2-2506464	Introduction of UAV mobility enhancements [UAV_Mobility]	CATT, NTT DOCOMO, LG Electronics Inc., Kyocera, LGU+, China Telecom, NEC, SK Telecom, Qualcomm Incorporated, Ericsson, Nokia, InterDigital, vivo, CMCC	CR	Rel-19	38.331	18.6.0	5399	1	B	TEI19	R2-2505224

The CR is agreed

[Per band capability for paging]

R2-2505454	Introduction of band specific capability for paging [Per_Band_Paging_Cap]	Huawei, Nokia, Xiaomi, Ericsson	CR	Rel-19	38.331	18.6.0	5417	-	B	TEI19

The CR is agreed

[Early CSI acquisition]

R2-2505598	Support early CSI acquisition for L3 Handover	Huawei, HiSilicon, China Unicom, Sony, Turkcell, NTT Docomo INC., Meta, Ericsson, Reliance Jio, Vodafone, ZTE Corporation, BT Plc., Deutsche Telekom, Vivo, LG Electronics Inc., Xiaomi, NEC	discussion	Rel-19

Proposal 1: RAN2 adapts the following framework for early CSI acquisition and CSI reporting for L3 handovers:

1)	CSI-RS configuration provided in handover command (details as in Section 2.3.2 and 5.1).

2)	Measures CSI-RS upon reception of handover command.

3)	CSI reporting re-uses LTM solution framework.

Proposal 2: If RAN2 agrees to support early CSI acquisition for L3 handover, send an LS to RAN1 informing RAN2 agreement and potential TP to RAN1.

Observation 2: As UE does not hold the transmission of the 1st PUSCH after receiving handover command, there is no impact to RAN4.

Noted

R2-2505604	Support early CSI acquisition for L3 handover [EarlyCSI_L3HO]	Huawei, HiSilicon, China Unicom, Sony, Turkcell, NTT Docomo INC., Meta, Ericsson, Reliance Jio, Vodafone, ZTE Corporation, BT Plc., Deutsche Telekom, Vivo, LG Electronics Inc., Xiaomi, NEC	CR	Rel-19	38.331	18.6.0	5426	-	B	TEI19

-	Qualcomm indicates that the field description is normally captured in RAN1.   Ericsson agrees and we need to ensure that RAN1 does the work properly.

-	MEdiatek thinks that this is a RAN1 feature so RAN1 should discuss first.   Huawei doesn’t think this is a RAN1 issue.

-	Nokia is supportive concerned about RAN4 and RAN1 issues that need to be first resolved for LTM.  If that is resolved then it doesn’t mean that it can automatically be re-used.    Huawei thinks that the framework is agreed and that is enough for us to move forward.   There is no RAN4 impacts.

-	Vivo thinks it can be simple and we just bring a CR in RAN1.

-	Apple doesn’t think we can agree on what are RAN1/RAN4.

-	ZTE agrees there are minor RAN1 impacts but we don’t need RAN1 to evaluate the feasibility.  The only difference is how you get it.

-	Samsung doesn’t agree with this issue and it should be triggered by RAN1.  Qualcomm also think this is RAN1 domain.

Prepare offline a RAN2 CR (how it would like).  Wait for some RAN1/RAN4 progress on LTM related issues and identify whether we can assume that those solutions can be used.

Prepare an LS to RAN1 and cc RAN4 to indicate intention on RAN2 solution, provide the RAN2 CR (how it would look like).  Provide the identified impacts to RAN1 and ask if this is ok.

[AT131][011][TEI19] Early CSI  (Huawei)

- Intended outcome:  Agree to RAN1 LS by email (R2-2506504)

- Deadline:  Friday

R2-2506449	Report of [AT131][011][TEI19] Early CSI (Huawei)	Huawei, HiSilicon	discussion	Rel-19	TEI19

R2-2506452	[Draft] LS on early CSI acquisition for L3 handover	Huawei, HiSilicon	LS out	Rel-19	TEI19	To:RAN1	Cc:RAN4

Indicate that RAN2 has assumed no RAN4 work

Revised in R2-2506504

R2-2506504	LS on early CSI acquisition for L3 handover	RAN2	LS out	Rel-19	TEI19	To:RAN1	Cc:RAN4

LS is approved

R2-2506450	Support early CSI acquisition for L3 handover [EarlyCSI_L3HO]	Huawei, HiSilicon, China Unicom, Sony, Turkcell, NTT Docomo INC., Meta, Ericsson, Reliance Jio, Vodafone, ZTE Corporation, BT Plc., Deutsche Telekom, Vivo, LG Electronics Inc., Xiaomi, NEC	CR	Rel-19	38.331	18.6.0	5426	1	B	TEI19	R2-2505604

The CR is technically correct

RAN2 assumes there is no RAN4 work

The CR is postponed pending RAN1 confirmation and will be discussed next meeting

Not treated

R2-2506451	Introduction of UE capability to support early CSI acquisition for L3 handover [EarlyCSI_L3HO]	Huawei, HiSilicon, China Unicom, Sony, Turkcell, NTT Docomo INC., Meta, Ericsson, Reliance Jio, Vodafone, ZTE Corporation, BT Plc., Deutsche Telekom, Vivo, LG Electronics Inc., Xiaomi, NEC	CR	Rel-19	38.306	18.6.0	1333	1	B	TEI19	R2-2505612

R2-2505612	Introduction of UE capability to support early CSI acquisition for L3 handover [EarlyCSI_L3HO]	Huawei, HiSilicon, China Unicom, Sony, Turkcell, NTT Docomo INC., Meta, Ericsson, Reliance Jio, Vodafone, ZTE Corporation, BT Plc., Deutsche Telekom, Vivo, LG Electronics Inc., Xiaomi, NEC	CR	Rel-19	38.306	18.6.0	1333	-	B	TEI19

[LTM]

R2-2505663	LTM CG Resource consumption for the target cells	Sony	discussion	Rel-19	TEI19

Proposal 1: RAN2 to discuss how minimize the configured grant resource consumption for the candidate target cells.

-	Apple thinks that this is up to the network.  If there is anything for the network to do, that is more RAN3.   Xiaomi has the same understanding as Apple and this was not pursued in the running CR and this should be up to network implementation.

-	Ericsson thinks that the intention is not to use the CG until the cell switch.   This is not of a TEI but rather a clarification in R18.

-	Vivo has similar view as Sony.   If we leave it to network implementation it may cause HO delay.

-	Qualcomm thinks we should focus on UE action, the UE can’t use the CG resource until the switch.  It should be already clear, but if needed we can clarify in chair notes for Rel-18.

-	ZTE thinks that the UE will only apply the configuration after the LTM command so this problem won’t happen.  Nokia, Lenovo, Interdigital agrees with ZTE it is not a RAN2 topic and the resources won’t be used anyways until the received command.

Proposal 2: CG resources of the target cell should be activated only after the serving cell receives L1 measurement report from the UE and the serving cell makes the decision to issue the LTM command to the UE for the target cell. The activation message should be sent from the serving cell to the target cell only.

Proposal 3: Target cell should activate CG resources after receiving CELL SWITCH NOTIFICATION message from the source cell.

Proposal 4: Add a note in the spec that it is up to network implementation the order of step 14 and 15 where step 15 can be earlier than step 14.

Not supported as TEI

Noted

R2-2505817	Introducing SR resources in LTM cell switch MAC CE [LTM_enh_SR]	Ericsson, Telia Company, Verizon, T-Mobile USA, NTT Docomo, Turkcell, InterDigital, Charter Communications, MediaTek Inc., BT Plc., Vodafone, Continental Automotive, Rakuten Mobile, China Mobile, Nokia, Qualcomm Incorporated, AT&T, LG Electronics	discussion	Rel-19	TEI19

=> Revised in R2-2506196

R2-2506196	Introducing SR resources in LTM cell switch MAC CE [LTM_enh_SR]	Ericsson, Telia Company, Verizon, T-Mobile USA, NTT Docomo, Turkcell, InterDigital, Charter Communications, MediaTek Inc., BT Plc., Vodafone, Continental Automotive, Rakuten Mobile, China Mobile, Nokia, Qualcomm Incorporated, AT&T, LG Electronics	discussion	Rel-19	TEI19

-	Samsung thinks that this is not needed.  In LTM we have multiple solution and we can use CG.   Ericsson thinks that it is true but CG is very costly.  Network doesn’t know which candidate will be used.   With pre-scheduling we can waste resources as we have to send the grant multiple time.

-	Xiaomi also not sure this is needed as it is network implementation.  Ericsson thinks that Xiaomi is not a network vendor and the reality is not as simple.

-	Lenovo thinks that there is no delay.   Ericsson explains that the DU doesn’t know the UE implementation.  There is no RAN4 requirements when the UE starts monitoring the PDCCH.

-	CATT doesn’t think this enhancement is necessary.

-	Huawei isn’t interested in the problem but understand that there may be some network implementation considerations.    If we want to do this we need to specify more things for UE behaviour.

-	Sony has some sympathy and support.

Agree with the intention of the network indicating which SR resources to use within a DU.   Review and CRs in the next meeting

[POST131][009][TEI19]  LTM Cell Switch (Ericsson)

Intended outcome: Review updated CRs after plenary

Deadline:  Long

Mobility state parameters

R2-2506114	Discussion on MobilityStateParameters in RRC_CONNECTED	KDDI Corporation (TTC)	discussion

Proposal 1:  RAN2 should discuss whether to introduce the same functionality as LTE in NR, which changes the values of mobility-related parameters (e.g., timeToTrigger) based on the UE's mobility state and delivers mobilityStateParameters in RRCReconfiguration.

-	Apple points out that we had this discussion in NR and we agreed it was not needed.   KDDI thinks that after we implemented NR we noticed it was mistake.   Huawei thinks that it make sense so we should go back to discussion to understand why it was not introduced.

-	Ericsson doesn’t think the number of HO is a good indication of mobility state.

-	ZTE thinks from implementation perspective this would be useful in the field especially for high speed scenarios.

-	Qualcomm remembers that the counting is not very reliable, but the scenario that KDDI is bringing may be a relevant scenario and should be considered.

Consider the enhancements and discuss possible solution/way to address it

Proposal 2:　RAN2 should discuss the release from which to introduce the functionality.

Proposal 3:　RAN2 should discuss additional candidate solutions for NR Mobility State evaluation method.

Noted

R2-2505693	Discussion on PL offset in the case of LTM	Lenovo	discussion	Rel-19

Proposal 1: Support sDCI mTRP 2TA with PL offset for Rel-18LTM cell switch.

Proposal 2: To support the cell of asymmetric DL sTRP and UL mTRP with PL offset as a target cell in the case of LTM cell switch, the PL offset value should be indicated in the LTM cell switch command.

-	Oppo thinks this was discussed in MIMO session last meeting.  Vivo agrees we haven’t concluded this.

-	Ericsson thinks we can do this as part of review of MIMO, as MIMO isn’t concluded yet.  We can wait until MIMO.

Discussion is postponed and should take place in MIMO first.  In October during ASN.1 review cross-WI discussion we may discuss whether the feature can be extended to LTM

Noted

[SR to RA fallback]

R2-2505884	Scheduling Request (SR) to RACH Fallback Enhancements	Ericsson	discussion	Rel-19

Not supported in R19

Noted

[PDCCH_Usage]

R2-2505963	Introducing PDCCH CCE Usage for L2M [PDCCH_Usage]	CMCC, Huawei, Ericsson, ZTE, CATT, Nokia	CR	Rel-19	38.314	18.0.0	0035	2	B	TEI19	R2-2504664

The CR is agreed

[Cell DTX/DRX]

R2-2506049	Discussion on UE assistance information for cell DTX/DRX	Huawei, HiSilicon, China Telecom, NTT DOCOMO, Deutsche Telekom, KT Corporation, LG Uplus, Orange, Turkcell, SK Telecom, China Unicom, CMCC, Sharp, TCL, HONOR, ZTE Corporation, Sanechips, Google, Fraunhofer IIS, Fraunhofer HHI, CEWiT, CAICT, Apple, OPPO, CATT	discussion	Rel-19	TEI19

=> Revised in R2-2506198

R2-2506198	Discussion on UE assistance information for cell DTX/DRX	Huawei, HiSilicon, China Telecom, NTT DOCOMO, Deutsche Telekom, KT Corporation, LG Uplus, Orange, Turkcell, SK Telecom, China Unicom, CMCC, Sharp, TCL, HONOR, ZTE Corporation, Sanechips, Google, Fraunhofer IIS, Fraunhofer HHI, CEWiT, CAICT, Apple, OPPO, CATT, TIM	discussion	Rel-19	TEI19

Proposal 1: UE indicates to the network that the configured cell DTX/DRX does not suit UE services. RAN2 to down-select from the following:

Option 1: Introduce a 1-bit indication in UAI, indicating the UE prefers not to operate under the current cell DTX/DRX configuration;

-	Option 1-a: 1-bit indication as above, with an optional recommendation from UE of cell DTX/DRX configuration;

Option 2: Clarify in the spec that DRX-Preference-r16 can also be used by the network for cell DTX/DRX configuration.

-	Docomo supports this proposal as this helps energy consumption from network perspective.

-	Nokia doesn’t share the enthusiasm as we can use normal QoE procedure to let the network control UE behaviour.   Huawei explains that this is for OTT services that the network doesn’t understand what type of service it is and that no one pays for it.

-	Xiaomi agrees with intention but DRX preference report is sufficient.  Huawei thinks that this is option 2, but we need to clarify so after dtx/drx is configured and it is sending a preference it is also sending a preference for dtx/drx.  LG shares the view with Xiaomi.

-	ZTE agrees and prefers option 1, and using DRX preference without clarification it will be confusing to the network.

-	Ericsson thinks that I can report unhappy but it doesn’t mean the network will make the UE happy, so this can go for a while.  Huawei assumes that there is always a coverage cell and if it wants it can handover these unhappy UEs that cell.

-	BT doesn’t see why we need to use this.  Huawei explains that it is for OTT services.

-	Vivo thinks that option 1 doesn’t exactly work so option 2 would work better.

-	Ericsson thinks that the DRX preference for r16 would work implicitly.  Huawei is explaining that we just need to clarify for implementers that this can also be used and also only the long cycle would need to be sent.

UE indicates to the network that the configured cell DTX/DRX does not suit UE services. Clarify in the spec that DRX-Preference-r16 can also be used by the network for cell DTX/DRX configuration.

Don’t need to capture how the NW will use this indication

[POST131][019][TEI19] NES UE Caps CR (Huawei)

Intended outcome: update and agree to CRs by email

Deadline:  short

=> Agreed in:

- R2-2506580 (38.331)

- R2-2506582 (38.306)

R2-2506454	Introduction of UE assistance information for cell DTX/DRX [UAI-cellDTRX]	Huawei, HiSilicon	CR	Rel-19	38.331	18.6.0	5474	-	B	TEI19

-	Xiaomi is concerned that this feature may not function well may not be able to differentiate for which purpose the UE reported this preference and may not be able to properly configure the UE.

Clarify in a NOTE: If DRX preference contains only long DRX cycle and shorter than configured CELL DTX/DRX cycle it implies preference for CELL DTX/DRX.

Optional with capability signaling.  Huawei will provide CR in POST email discussion.

Revised in R2-2506482

R2-2506482	Introduction of UE assistance information for cell DTX/DRX [UAI-cellDTRX]	Huawei, HiSilicon	CR	Rel-19	38.331	18.6.0	5474	1	B	TEI19

=> Revised in R2-2506580

R2-2506580	Introduction of UE assistance information for cell DTX/DRX [UAI_cellDTRX]	Huawei, HiSilicon	CR	Rel-19	38.331	18.6.0	5474	2	B	TEI19

The CR is agreed

R2-2506489	Introduction of UE assistance information for cell DTX/DRX [UAI-cellDTRX]	Huawei, HiSilicon	CR	Rel-19	38.306	18.6.0	1352	-	B	TEI19

=> Revised in R2-2506580

R2-2506582	Introduction of UE assistance information for cell DTX/DRX [UAI_cellDTRX]	Huawei, HiSilicon	CR	Rel-19	38.306	18.6.0	1352	1	B	TEI19

The CR is agreed

To be treated in positioning session

Equal integer ambiguity level assistance data (postponed from RAN2#130)

R2-2505840	UE request for equalIntegerAmbiguityLevel assistance data [GNSS-EqualIntegerAmbiguity]	AT&T, Ericsson, Huawei, CATT, Samsung, ZTE Corporation, Nokia, Deutsche Telekom	CR	Rel-19	37.355	18.5.0	0557	1	B	TEI19	R2-2504306

Revised in R2-2506318

R2-2506318	UE request for equalIntegerAmbiguityLevel assistance data [GNSS-EqualIntegerAmbiguity]	AT&T, Ericsson, Huawei, CATT, Samsung, ZTE Corporation, Nokia, Deutsche Telekom	CR	Rel-19	37.355	18.5.0	0557	2	B	TEI19	R2-2504306

Agreed

[Periodic AD]

R2-2505321	Introduction of periodic AD for NR integrity service alert [IntegrityPeriodicAD]	Huawei, HiSilicon, Ericsson, Vivo	CR	Rel-19	37.355	18.5.0	0558	-	B	TEI19

Revised in R2-2506316

R2-2506316	Correction on the periodic AD of NR integrity service alert [IntegrityPeriodicAD]	Huawei, HiSilicon, Ericsson, Vivo	CR	Rel-19	37.355	18.5.0	0558	1	B	TEI19	R2-2505321

Revised (post-meeting email, tdoc number needed)

Discussion:

Lenovo think there are some detailed issues (suffixes, ASN.1 extension brackets, etc.).

Qualcomm agree there are some errors, connected to the applicability of the common IEs and how the capability is introduced (should be just a capability for the procedure).

Huawei would like to understand if we need additional capabilities for DL-TDOA according to Qualcomm’s comment.  Qualcomm understand that for DL-TDOA the capability is already there.

[Post131][408][POS] LPP CR on periodic integrity assistance data (Huawei)

Scope: Revise the CR in R2-2506316 in accordance with the comments raised during the meeting and check the details.

Intended outcome: Agreed CR

Deadline: Short (for RP)

R2-2505322	Introduction of periodic delivery of NR integrity service alert [IntegrityPeriodicAD]	Huawei, HiSilicon, Ericsson, VIVO	CR	Rel-19	38.305	18.6.0	0192	-	B	TEI19

Revised in R2-2505676

R2-2505676	Correction on the periodic AD of NR integrity service alert [IntegrityPeriodicAD]	Huawei, HiSilicon, Ericsson, Vivo	CR	Rel-19	38.305	18.6.0	0192	1	B	TEI19	R2-2505322

Title to be changed to match R2-2505322

Coversheet to include cross-reference to LPP CR

Agreed with these changes as R2-2506319

R2-2506319	Introduction of periodic delivery of NR integrity service alert [IntegrityPeriodicAD]	Huawei, HiSilicon, Ericsson, Vivo	CR	Rel-19	38.305	18.6.0	0192	2	B	TEI19

=> Agreed

Discussion:

Qualcomm note that the title is inconsistent.

CATT think the coversheet should include cross-reference to the LPP CR.

To be treated in NTN breakout session

[LTE operation via satellite]

R2-2505452	Indication to the UE on the LTE operation via satellite	Apple	discussion	Rel-19	TEI19

EUTRAN to NB-IoT NTN mobility

R2-2505084	Introduction of E-UTRAN to NB-IoT NTN Mobility UE Capability [EUTRAN-to-NBIoTNTN]	vivo, Samsung, Google, THALES, MediaTek Inc., Aalyria	CR	Rel-19	36.306	18.5.0	1917	-	B	TEI19

Agreed

R2-2505367	Introduction of NB-IoT satellite information in E-UTRAN [EUTRAN-to-NBIoTNTN]	Google, Samsung, vivo, THALES, MediaTek Inc., Aalyria	CR	Rel-19	36.331	18.6.0	5140	-	B	TEI19

Agreed

R2-2505368	Introduction of NB-IoT satellite information in E-UTRAN [EUTRAN-to-NBIoTNTN]	Google, Samsung, vivo, THALES, MediaTek Inc., Aalyria	CR	Rel-19	36.300	18.5.0	1427	-	B	TEI19

Agreed

Redirection from NR TN to NR NTN (and from IoT TN to IoT NTN)

R2-2505920	Draft CR Introduction of redirection from NR TN to NR NTN to 38.331	Samsung	draftCR	Rel-19	38.331	18.6.0	B	TEI19

R2-2505921	Draft CR Introduction of redirection from NR TN to NR NTN to 38.306	Samsung	draftCR	Rel-19	38.306	18.6.0	B	TEI19

R2-2505954	Discussion on redirection from TN to IoT NTN and NR NTN	CMCC	discussion	Rel-19	TEI19.

All contributions to be discussed first in offline 301

Proposals are endorsed. Actual CRs to be drafted in R2-2506426, R2-2506426 and R2-2506428, R2-2506429

R2-2506426	Introduction of redirection from NR TN to NR NTN to 38.331 [NR-TN-NTN-redir]	Samsung	CR	Rel-19	38.331	18.6.0	5429	-	B	TEI19

Revised in R2-2506433

R2-2506433	Introduction of redirection from NR TN to NR NTN to 38.331 [NR-TN-NTN-redir]	Samsung	CR	Rel-19	38.331	18.6.0	5429	1	B	TEI19

Agreed, but then coversheet revised by MCC in R2-2506652: hyphens in TEI identifier

R2-2506427	Introduction of redirection from NR TN to NR NTN to 38.306 [NR-TN-NTN-redir]	Samsung	CR	Rel-19	38.306	18.6.0	1348	-	B	TEI19

Agreed, but then coversheet revised by MCC in R2-2506650: hyphens in TEI identifier

R2-2506518	Introduction of TN to NTN redirection for IoT to 36.331 [IoT-TN-NTN-redir]	Samsung, Ericsson, Google	CR	Rel-19	36.331	18.6.0	5156	1	B	TEI19

Agreed, but then coversheet revised by MCC in R2-2506653: hyphens in TEI identifier

R2-2506429	Introduction of TN to NTN redirection for IoT to 36.306 [IoT-TN-NTN-redir]	Samsung, Ericsson, Google	CR	Rel-19	36.306	18.5.0	1925	-	B	TEI19

Agreed, but then coversheet revised by MCC in R2-2506651: hyphens in TEI identifier

R2-2506650	Introduction of redirection from NR TN to NR NTN to 38.306 [NR_TN_NTN_redir]	Samsung	CR	Rel-19	38.306	18.6.0	1348	1	B	TEI19

Agreed

R2-2506651	Introduction of TN to NTN redirection for IoT to 36.306 [NR_TN_NTN_redir]	Samsung, Ericsson, Google	CR	Rel-19	36.306	18.5.0	1925	1	B	TEI19

Agreed

R2-2506652	Introduction of redirection from NR TN to NR NTN to 38.331 [NR_TN_NTN_redir]	Samsung	CR	Rel-19	38.331	18.6.0	5429	2	B	TEI19

Agreed

R2-2506653	Introduction of TN to NTN redirection for IoT to 36.331 [NR_TN_NTN_redir]	Samsung, Ericsson, Google	CR	Rel-19	36.331	18.6.0	5156	2	B	TEI19

Agreed

NB-IoT NTN to NR NTN Cell Selection

R2-2506098	NB-IoT NTN to NR NTN Cell Selection	EchoStar, Qualcomm, Aalyria, Terrestar, Skylo, Sateliot	CR	Rel-19	36.331	18.6.0	5151	-	B	IoT_NTN_enh-Core

R2-2506099	Asisstance for inter-RAT cell-selection from NB-IoT NTN to NR-NTN	EchoStar, Qualcomm, Aalyria, Terrestar, Skylo, Sateliot	CR	Rel-19	36.306	18.5.0	1923	-	B	IoT_NTN_enh-Core

R2-2506100	NR-NTN to NB-IoT NTN Cell Selection	EchoStar, Qualcomm, Aalyria, Terrestar, Skylo	CR	Rel-19	38.331	18.6.0	5455	-	B	NR_NTN_enh-Core

R2-2506101	Asisstance for inter-RAT cell-selection from NR NTN to NB-IoT NTN	EchoStar, Qualcomm, Aalyria, Terrestar, Skylo, Sateliot	CR	Rel-19	38.306	18.6.0	1340	-	B	NR_NTN_enh-Core

All contributions to be discussed first in offline 301

-	Samsung thinks the concerns with this are related to the use cases (even if they think it could make sense). But the CRs need to be fixed in any case

-	Echostar thinks there is a use case and standardization support is needed to convince the ecosystem to work on this. Aalyria agrees (and this applies also to the proposal on redirection)

-	QC also thinks this would be beneficial and supports the proposal

-	Viasat also supports this as this is a very valid use case

-	Xiaomi thinks it’s too early to support this and we can reconsider this after the takeoff of NR NTN products. Proponents could come back with a TEI20 proposal

-	Ericsson thinks that this could be already supported by UE implementation based on existing specs (saying that the UE could use stored information for cell selection)

-	Skylo also thinks this is a valid use case. Skylo is also not sure about the possibility to use stored information, e.g. in case of moving satellites

-	Ericsson thinks there is also a problem to exchange the needed information between eNbs and gNBs for this

Can come back with this proposal

Redirection for NB-IoT NTN to NR-NTN

R2-2506178	Introduction of RRC Release with Redirection for NB-IoT NTN to NR-NTN	Aalyria	discussion	Rel-19	IoT_NTN_Ph3, IoT_NTN_Ph3-Core

R2-2506179	CR to TS 36.331 - Introduction of RRC Release with Redirection for NB-IoT NTN to NR-NTN	Aalyria	CR	Rel-19	36.331	18.6.0	5154	-	B	IoT_NTN_enh-Core, TEI19

All contributions to be discussed first in offline 301

-	ZTE thinks we should avoid at least this proposal for now as really not urgent

Can come back with this proposal

Satellite Switch with Resynchronization support for IoT NTN

R2-2505933	TEI19] Introduction of Satellite Switch with Resynchronization support for IoT NTN	Skylo Technologies, Lockheed Martin, EchoStar, Sateliot	discussion	Rel-19

R2-2505934	SatSwitchwithReSync support for NB-IoT NTN	Skylo Technologies, Lockheed Martin, EchoStar, Sateliot	CR	Rel-19	36.300	18.5.0	1429	-	B	IoT_NTN_Ph3-Core

R2-2505938	SatSwitchwithReSync support for IoT NTN	Skylo Technologies, Lockheed Martin, EchoStar, Sateliot	CR	Rel-19	36.321	18.4.0	1594	-	B	IoT_NTN_Ph3-Core

R2-2505939	SatSwitchwithReSync support for IoT NTN	Skylo Technologies, Lockheed Martin, EchoStar, Sateliot	CR	Rel-19	36.331	18.6.0	5149	-	B	IoT_NTN_Ph3-Core

All contributions to be discussed first in offline 301

-	Viasat, Echostar, ESA, Aalyria also support this proposal

-	HW also see some benefits to support this

-	Xiaomi thinks that also for hard switch there is RAN4 impact

-	Aalyria thinks hard switch can also be supported without RAN4 work

-	MTK thinks this is the last meeting for Rel-19 and it’s the first time we see this proposal. We could consider it for the next release

-	Ericsson thinks NB-IoT does not support connected mode mobility and they would like this to stay like that. Also this would be needed for connections that last for a long time and this is not the case for typical IoT scenarios

-	Session chair thinks this could also be considered as an update for the R20 IoT NTN WID

RAN2 thinks this could be further considered in Rel-20

Other

R2-2506015	Discussion on measurement events for inter-frequency scenarios with overlapping coverage in NR NTN	CSCN, ZTE Corporation, Sanechips, Huawei, HiSilicon, CATT	discussion	Rel-19	TEI19

We don’t pursue this enhancement in TEI19

[AT131][301][TEI19] NTN related TEI19 (Samsung)

Scope: collect views on the NTN related TEI19 proposals (apart from [EUTRAN-to-NBIoTNTN] which is already endorsed), with the intention to prioritize the most important ones / merge / simplify the proposals

Intended outcome: summary of the offline discussion (in R2-2506271)

Offline time: Monday 2025-08-25 afternoon coffee break in room offline 1 (Amaryllis+Petunia)

Deadline for offline discussion summary:  Wednesday 2025-08-27 14:00

R2-2506271	Report of [AT131][301][TEI19] NTN related TEI19	Samsung	discussion	TEI19

[Easy agreements]

Proposal 1: For NR TN to NR NTN: Optional explicit ephemeris in redirection message is introduced if SIB19 is not broadcast. If explicit ephemeris is not present, the UE uses the neighbour cell ephemeris in SIB19 for which the carrier frequency matches.

Agreed

Proposal 2: Agree the CRs in R2-2506426/R2-2506427.

Proposal 3: On minor open issues:

-   Regarding the SMTC in RRCRelease, we can follow solution as will be discussed for LTE TN to NR NTN.

-   Any discussion on redirection behaviour in 38.304 can be discussed in maintenance.

Proposal 4: For IoT TN to IoT NTN redirection agree CRs in R2-2506428/R2-2506429.

[More discussion]

Proposal 5: (3 support/5 negative) Discuss downprioritizing enhancements for new measurement event (single referencelocation and threshold) for TEI19.

Proposal 6: (4 support/7 negative) Discuss downprioritizing enhancements for support for NB-IoT NTN<->NR NTN cell selection in TEI19.

Proposal 7: (4 support/4 negative) Discuss downprioritizing enhancements for satellite switch with resync for TEI19.

Proposal 8: Downprioritize enhancements for redirection from NB-IoT NTN to NR NTN for TEI19.

R2-2505452	Indication to the UE on the LTE operation via satellite	Apple	discussion	Rel-19	TEI19

Proposal 1: Introduce an optional IE in LTE SIB1 to indicate to the UE about the LTE NTN operation by the NW.

Proposal 2: Introduce the IE from Rel-19 but make it release independent.

Proposal 3: RAN2 to agree that the access stratum simply forwards the SIB1 indication to the upper layers when the UE reads this from LTE SIB1.

-	TIM comments (via email): our understanding is that the problem highlighted in the paper is one of the reasons why NTN supporters of 3GPP NTN solutions triggered this kind of work in 3GPP RAN groups to have fully 3GPP based and compliant solutions. This [proposal] is harming 3GPP NTN ecosystem in general, aiming to pre NTN proprietary solutions. As already commented in RAN2 and in plenary, 3GPP is going to die soon if we continue to put 3GPP stamp on proprietary solutions

-	Aalyria thinks the correct approach to solve this is via PLMN selection and also wonders why not relying on NR NTN if in any case we need to change something in LTE

-	Thales is a bit surprised by this attempt to address this issue in RAN2/RAN4 while this should be addressed via a WI proposal in RAN plenary.

-	CMCC thinks this could be solved by CT1 via a NAS solution

-	Xiaomi thinks that a specific PLMN could solve this problem

-	Viasat thinks this is going to further fragment the market and it’s not the right way to go

-	Samsung agrees with TIM comment about putting a 3GPP stamp on proprietary solutions and in any case the decision for this should not be made here. ESA also agrees on this

R2-2505390	NTN-NR to NB-IoT Cell Selection	EchoStar, Qualcomm, Aalyria, Terrestar, Skylo, Sateliot	CR	Rel-19	38.331	18.6.0	5415	-	B	NR_NTN_enh-Core

Withdrawn

R2-2505391	Asisstance for inter-RAT cell-selection from NR NTN to NB-IoT NTN	EchoStar, Qualcomm, Aalyria, Terrestar, Skylo, Sateliot	CR	Rel-19	38.306	18.6.0	1326	-	B	NR_NTN_enh-Core

Withdrawn

R2-2505409	NB-IoT NTN to NR NTN Cell Selection	EchoStar, Qualcomm, Aalyria, Terrestar, Skylo, Sateliot	CR	Rel-19	36.331	18.6.0	5142	-	B	IoT_NTN_enh-Core

Withdrawn

R2-2505410	Asisstance for inter-RAT cell-selection from NB-IoT NTN to NR-NTN	EchoStar, Qualcomm, Aalyria, Terrestar, Skylo, Sateliot	CR	Rel-19	36.306	18.5.0	1919	-	B	IoT_NTN_enh-Core

Withdrawn

### 8.19.2	Other WG-led

Positioning breakout session

SRS frequency hopping ofr non-RedCap UE (CRs endorsed at RAN2#130)

R2-2505032	Reply LS on non-RedCap UE UL SRS frequency hopping for positioning (R3-253749; contact: ZTE)	RAN3	LS in	Rel-19	TEI19	To:RAN1, RAN2

Noted

R2-2505594	Introduction of UE capability for SRS frequency hopping for non-RedCap UE in 37355 [Pos_SRSHop]	ZTE Corporation	CR	Rel-19	37.355	18.5.0	0553	1	B	TEI19	R2-2503877

“Other specs affected” to be updated on coversheet: include RRC CR, untick “N” box, replace tdoc number with CR number

Refer to LPP capability instead of RRC capability in field description of posSRS-TxFH-RRC-Inactive-NonRedCap

“Inactive-NonRedCap” should be “InactiveNonRedCap” in all IE/field names

Agreed with these changes as R2-2506320

Discussion:

Lenovo think there are a few minor issue in “Other specs affected” on the coversheet and in the field description with a dependency on RRC signalling (could refer to the same field in LPP instead).

R2-2505595	Introduction on the SRS frequency hopping for non-RedCap UE in 38331 [Pos_SRSHop]	ZTE Corporation, Ericsson	CR	Rel-19	38.331	18.6.0	5290	2	B	TEI19	R2-2503878

“Other specs affected” to be updated on coversheet: include LPP CR, untick “N” box, replace tdoc number with CR number

Second sentence in “Reason for change” to be updated to include also the restriction for FR2

Agreed with these changes as R2-2506321

R2-2505063	Response to Reply LS on paging capability loss issue (S2-2506087; contact: Vodafone)	SA2	LS in	Rel-19	NR_LPWUS-Core, TEI19	To:RAN2, RAN3	Cc:RAN, CT1

- -	Vivo and Xiaomi think that there was misunderstanding from SA2.  There is no intention for all networks to upgrade to Rel-19.

-	Nokia also agrees there was a misunderstanding.

-	CATT thinks that RAN3 will discuss this issue so we can wait for RAN3.    Qualcomm agrees.

Noted

[AT131][013][TEI19] Paging capability (Vivo)

Intended outcome: Conclude on need to have response to SA2, after RAN3 conclusion, and agree what RAN2 to indicate to SA2 (e.g. including misunderstanding).

Deadline:  Thursday

R2-2506016	Discussion on SA2 reply LS on paging capability loss issue	vivo	discussion	Rel-19	NR_LPWUS-Core, TEI19

Proposal 1: Reply LS to SA2 to clarify the RAN2 discussion situation:

-	The “container” is newly introduced from Rel-19. All the gNBs before Rel-19 cannot support this “container”, which means that there may be paging capability miss issue.

o	RAN2 common understanding is to rely on network implementation to handle it (e.g. upgraded to support the ASN.1 from Rel-19) or not to address it.

-	Regarding the network from Rel-19, if operators want to support the related features in one area, e.g. LP-WUS, new version of gNBs from Rel-19 in this area should support this “container”; while for old version of gNB before Rel-19 (including Rel-19 gNB not supporting Rel-19 “container”) in this area:

o	if they are not upgraded to support Rel-19 “container”, there may be paging capability miss issue in this area only when the UE camps on this kind of gNB during the initial registration or TAU procedure and reports capability via this gNB.

o	if they are upgraded to support Rel-19 “container” (by implementation, only need to be upgraded to support the ASN.1 from Rel-19), there will no paging capability miss issue.

Proposal 2: RAN2 to consider the draft reply LS in Annex.

Noted

R2-2506503	[Draft] Reply LS on paging capability loss issue	vivo	LS out	Rel-19	NR_LPWUS-Core, TEI	To:SA2	Cc:RAN3

Wait to respond to SA2 after we have the full solution

Noted

TXSwitch_R19

R2-2505333	Introduction of 3Tx UL switching [TxSwitch_R19]	MediaTek Inc., Ericsson, T-Mobile USA	CR	Rel-19	38.331	18.6.0	5411	-	B	TEI19

R2-2505334	Introduction of 3Tx UL switching [TxSwitch_R19]	MediaTek Inc., Ericsson, T-Mobile USA	CR	Rel-19	38.331	18.6.0	5412	-	B	TEI19	Withdrawn

R2-2505335	Introduction of 3Tx UL switching [TxSwitch_R19]	MediaTek Inc., Ericsson, T-Mobile USA	CR	Rel-19	38.306	18.6.0	1324	-	B	TEI19	Withdrawn

R2-2505579	Introduction of 3Tx UL switching [TxSwitch_R19]	MediaTek Inc., Ericsson, T-Mobile USA	draftCR	Rel-19	38.331	18.6.0	B	TEI19

R2-2505580	Introduction of 3Tx UL switching [TxSwitch_R19]	MediaTek Inc., Ericsson, T-Mobile USA	draftCR	Rel-19	38.306	18.6.0	B	TEI19

[POST131][042][TEI19] TXSwitch (Mediatek)

Intended outcome: agree and endorse CRs

Deadline:  short and UE capability part Thursday

=> Agreed in:

- R2-2506589 (38.331, CR)

=> Endorsed in:

- R2-2506590 (38.331, draftCR)

- R2-2506591 (38.306, draftCR)

R2-2506589	Introduction of 3Tx UL switching [TxSwitch_R19]	MediaTek Inc., Ericsson, T-Mobile USA	CR	Rel-19	38.331	18.6.0	5411	2	B	TEI19

=> Agreed

R2-2506590	Introduction of 3Tx UL switching [TxSwitch_R19]	MediaTek Inc., Ericsson, T-Mobile USA	draftCR	Rel-19	38.331	18.6.0	B	TEI19

=> Endorsed

R2-2506591	Introduction of 3Tx UL switching [TxSwitch_R19]	MediaTek Inc., Ericsson, T-Mobile USA	draftCR	Rel-19	38.306	18.6.0	B	TEI19

=> Endorsed

R2-2506190	Support for MINT in EPS (MINT_Ph2)	Google	discussion	Rel-19

-	Lenovo indicates that CT1 is only starting work this meeting so we should wait for progress and wait for their LS.   Qualcomm agrees with Lenovo.

Postponed waiting for CT1 progress

R2-2506192	Introduction of MINT in EPS	Google	CR	Rel-19	36.331	18.6.0	5155	-	B	TEI19

R2-2506193	Introduction of MINT in EPS	Google	CR	Rel-19	36.306	18.5.0	1924	-	B	TEI19

R2-2506194	Introduction of MINT in EPS	Google	CR	Rel-19	36.300	18.5.0	1431	-	B	TEI19

R2-2505723	ProSe support in NPN [ProSe_NPN]	Nokia, ZTECorporation, Sanechips, LGE, Philips	CR	Rel-19	38.300	18.6.0	0957	1	B	TEI19	R2-2500362

The CR is agreed, but then coversheet revised by MCC: wrong meeting number

R2-2506629	ProSe support in NPN [ProSe_NPN]	Nokia, ZTECorporation, Sanechips, LGE, Philips	CR	Rel-19	38.300	18.6.0	0957	2	B	TEI19

=> Agreed

R2-2505758	ProSe support in NPN [ProSe_NPN]	ZTE Corporation, Sanechips, Nokia, LGE, Philips	CR	Rel-19	38.331	18.6.0	5209	1	B	TEI19	R2-2500422

The CR is agreed

R2-2505133	SR triggered SSSG switching [SRTrig_SSSGSwitch]	Ericsson, Qualcomm Incorporated	CR	Rel-19	38.331	18.6.0	5396	-	B	TEI19

-	Huawei is concerned that if we changed the parameters then RAN1 has to update this specs.  Ericsson thinks that we shouldn’t add parameters that already exists and RAN1 should update their specs.   Xiaomi thinks that it looks reasonable.

- => Add RAN1 CRs in other specs impacted

- => Qualcomm will take a CR to RAN1 directly to fix the misalignment

- => The CR is agreed in R2-2506412

R2-2506412	SR triggered SSSG switching [SRTrig_SSSGSwitch]	Ericsson, Qualcomm Incorporated	CR	Rel-19	38.331	18.6.0	5396	1	B	TEI19

=> Agreed

R2-2505134	32 HARQ process numbers for TN in FR1 and FR2-1 [TN32HARQ]	Ericsson, ZTE	CR	Rel-19	38.331	18.6.0	5397	-	B	TEI19

R2-2505332	Introduction of 32 HARQ processes to TN [TN32HARQ]	Huawei, HiSilicon	CR	Rel-19	38.331	18.6.0	5410	-	B	TEI19

Discussions

-	Huawei thinks that RAN1 agreed to separate parameters to not confuse with the legacy IEs that were introduced for NTN.  Ericsson thinks that they aren’t only for NTN.

-	CATT agrees with Huawei’s approach.

[AT131][031][TEI19] 32HARQ CR (Huawei)

Intended outcome: agree to CR by email

Deadline:  Thursday

R2-2506471	Introduction of 32 HARQ processes to TN [TN32HARQ]	Huawei, HiSilicon, Ericsson, ZTE Corporation	CR	Rel-19	38.331	18.6.0	5410	1	B	TEI19	R2-2505332

The CR is agreed

To be treated in LTE MBS breakout session

R2-2505330	Introduction of CAS muting in LTE-based 5G broadcast [5GB_CASMuting]	Huawei, HiSilicon, Qualcomm Incorporated, EBU	CR	Rel-19	36.306	18.5.0	1916	-	B	TEI19

The CR is endorsed

Handle in the post-meeting e-mail discussion whether we should add a capability dependency

[POST131][510][TEI19] UE capability CR for 5GB_CASMuting (Huawei)

Scope: Discuss whether we should add a capability dependency

Intended outcome: Decision and revised draft CR for endorsement in R2-2506343, if needed

Deadline:  Very short

=> Agreed in R2-2506343 (36.306)

- Lenovo wonders if the capability should capture that UE also should support Rel-14 feMBMS.

- QCM thinks this should refer to a new capability which we introduce as part of LTE based 5G broadcast WI.

R2-2506343	Introduction of CAS muting in LTE-based 5G broadcast [5GB_CASMuting]	Huawei, HiSilicon, Qualcomm Incorporated, EBU	CR	Rel-19	36.306	18.5.0	1916	1	B	TEI19

=> Agreed

R2-2505331	Introduction of CAS muting in LTE-based 5G broadcast [5GB_CASMuting]	Huawei, HiSilicon, Qualcomm Incorporated, EBU	CR	Rel-19	36.331	18.6.0	5139	-	B	TEI19

The CR is revised in R2-2506334

R2-2506334	Introduction of CAS muting in LTE-based 5G broadcast [5GB_CASMuting]	Huawei, HiSilicon, Qualcomm Incorporated, EBU	CR	Rel-19	36.331	18.6.0	5139	1	B	TEI19

The CR is revised in R2-2506342

R2-2506342	Introduction of CAS muting in LTE-based 5G broadcast [5GB_CASMuting]	Huawei, HiSilicon, Qualcomm Incorporated, EBU	CR	Rel-19	36.331	18.6.0	5139	2	B	TEI19

The CR is agreed

## 8.20	NR Others

Tdoc limit: 2

Specific items may be allocated to a breakout session for treatment.

Impacts from Other RAN WGs and TSGs that has no separate TU budget in RAN2. LS ins for Rel-19 specific WIs/SIs that has no RAN WI.

Additional tdocs on top of limit can be allowed for co-sourced contribution with 3 or more companies

### 8.20.1	RAN4

Signallig for 7Mhz Ch BW

R2-2505046	Reply LS to RAN2 on Signalling for 7 MHz Channel Bandwidth (R4-2508088; contact: T-Mobile)	RAN4	LS in	Rel-19	NR_FR1_7MHz_BW-Core	To:RAN2	Cc:RAN3

Noted

R2-2505903	Discussion on 7 MHz channel bandwidth capabilities	Nokia	discussion	Rel-19	NR_FR1_7MHz_BW-Core

Noted

R2-2506000	Consideration on the 7M Channel Bandwidth Reporting	ZTE Corporation	discussion	Rel-19	NR_FR1_7MHz_BW-Core

Noted

Discussions

-	Qualcomm think this is from Rel-19.

Allocate fifth leftmost bit in channelBWs-DL-v1590 / channelBWs-UL-v1590 to 7 MHz.

Support 7 MHz maximum/minimum channel bandwidth capabilities based on supportedBandwidthDL/UL and supportedMinBandwidthDL/UL from Rel-18

The changes are from R18 and allow early implementation from the R15.

[AT131][202][NR_Others] CRs for Signallig for 7Mhz Ch BW (T-Mobile USA, Ericsson)

Intended outcome: agreeable CR in R2-2506242/ R2-2506243 for Signalling for 7Mhz Ch BW

Deadline: before Friday CB.

R2-2506242	Introduction of 7MHz channel bandwidth	T-Mobile USA, Ericsson	CR	Rel-18	38.331	18.6.0	5308	2	B	NR_FR1_7MHz_BW-Core

The CR is agreed, but then coversheet revised by MCC: Wrong Rel-19 WI code for an earlier release CR

R2-2506243	Introduction of 7MHz channel bandwidth T-Mobile USA, Ericsson	CR	Rel-18	38.306	18.6.0	1258	2	B	NR_FR1_7MHz_BW-Core

The CR is agreed, but then coversheet revised by MCC: Wrong Rel-19 WI code for an earlier release CR

R2-2506647	Introduction of 7MHz channel bandwidth [FR1_7MHz_BW]	T-Mobile USA, Ericsson	CR	Rel-18	38.331	18.6.0	5308	3	B	TEI18

=> Agreed

R2-2506648	Introduction of 7MHz channel bandwidth [FR1_7MHz_BW]	T-Mobile USA, Ericsson	CR	Rel-18	38.306	18.6.0	1258	3	B	TEI18

=> Agreed

R2-2505384	Introduction of 7MHz channel bandwidth	T-Mobile USA, Ericsson	CR	Rel-18	38.331	18.6.0	5308	1	A	TEI18, NR_FR1_7MHz_BW-Core	R2-2502572

R2-2505386	Introduction of 7MHz channel bandwidth	Ericsson, T-Mobile	CR	Rel-17	38.331	17.13.0	5307	1	B	TEI17, NR_FR1_7MHz_BW-Core	R2-2502571

R2-2505387	Introduction of 7MHz channel bandwidth	Ericsson, T-Mobile	CR	Rel-18	38.306	18.6.0	1258	1	A	TEI18, NR_FR1_7MHz_BW-Core	R2-2502570

R2-2505388	Introduction of 7MHz channel bandwidth	Ericsson, T-Mobile	CR	Rel-17	38.306	17.13.0	1257	1	B	TEI17, NR_FR1_7MHz_BW-Core	R2-2502569

UE Signaling design for NR ATG enh

R2-2505048	LS on UE signaling design for NR ATG enh (R4-2508329; contact: CMCC)	RAN4	LS in	Rel-19	NR_ATG_enh	To:RAN2

Noted

R2-2505761	Discussion on ATG LS	Ericsson	discussion	Rel-19	NR_ATG_enh-Core

Noted

Proposal 1	RAN2 to discuss whether to place the new parameter in servingcellConfig or measObjectNR.

R2-2505961	Discussion on RAN4 LS on UE signaling design for NR ATG enh	CMCC	discussion	Rel-19	NR_ATG_enh

Noted

Proposal 1: It is proposed to introduce one bit indication (e.g. neighCellMeasofScc) in MeasObjectNR IE, and if it is configured as disabled, it indicates not to allow the UE to perform the neighbour cell measurements on the frequency configured by servingcellMO for SCC.

Proposal 2: If P1 is supported, suggest RAN2 to agree the TP in annex.

Discussions

-	Qualcomm wonders whether the ‘disabled’ configuration is needed or not.

RRC signaling is introduced to indicate whether an ATG UE supporting antennaArrayType-r18  on the SCC shall perform the neighbour cell measurements or not on the frequency configured by servingcellMO for SCC.

[AT131][201][NR_Others] UE Signaling design for NR ATG enh (CMCC)

Intended outcome: agreeable CR in R2-2506241 for UE Signaling design for NR ATG enh.

Deadline: before Friday CB.

R2-2506241	Introduction of Secondary Cell Measurement Skipping for NR ATG	CMCC, Ericsson	Rel-19	38.331	18.6.0	5477	-	B	NR_ATG_enh-Core

R2-2506500	Introduction of Secondary Cell Measurement Skipping for NR ATG	CMCC, Ericsson	Rel-19	38.300	18.6.0	1027	-	B	NR_ATG_enh-Core

Content of the two CRs are agreeable.

Revise the style and format issues in post meeting email discussions.

R2-2506501	Introduction of Secondary Cell Measurement Skipping for NR ATG	CMCC, Ericsson	Rel-19	38.306	18.6.0	1351	-	B	NR_ATG_enh-Core

Content of the CR is agreeable

Revise the format issue and prepare draftCR for endorsement in post meeting email discussions

[Post131][224][NR_Others] CRs for NR ATG enh (CMCC)

Intended outcome: Agree the CRs for TS38.331 and TS38.300, endorse the CR for TS38.306

Deadline:  Short

=> Agreed in:

- R2-2506514 (38.331)

- R2-2506515 (38.300)

=> Endorsed in R2-2506516 (38.306)

R2-2506514	Introduction of Secondary Cell Measurement Skipping for NR ATG	CMCC, Ericsson	CR	Rel-19	38.331	18.6.0	5477	1	B	NR_ATG_enh-Core

=> Agreed

R2-2506515	Introduction of Secondary Cell Measurement Skipping for NR ATG	CMCC, Ericsson	CR	Rel-19	38.300	18.6.0	1027	1	B	NR_ATG_enh-Core

=> Agreed

R2-2506516	Introduction of Secondary Cell Measurement Skipping for NR ATG	CMCC, Ericsson	draftCR	Rel-19	38.306	18.6.0	B	NR_ATG_enh-Core

=> Endorsed

On Rx BSF opt.

R2-2505205	Introduction of Rx BSF optimization for NR RRM Ph5	CATT, Ericsson, Apple, ZTE Corporation	draftCR	Rel-19	38.331	18.6.0	B	NR_RRM_Ph5-Core

Noted

[Post131][225][NR_Others] On Rx BSF optimization (CATT)

Intended outcome: Discussion summary on the UAI issue, and endorse the CR for TS38.331

Deadline:  Long

R2-2506093	Fast Beam Sweeping Factor	Nokia	discussion	Rel-19	NR_RRM_Ph5-Core

Noted

Discussions

-	CATT explains we expect more parameters from R4, so suggest we endorse for now and update based on further R4 input.

-	Nokia point out that R4 already have some agreement on Timer/UAI, and think we can already capture. HW fine with Nokia proposal.

-	HW think in CATT CR, the new procedure text can be reflected in the field description. LG E agree.

-	CATT think we may get more input from R4 during the meeting week, so we can discuss using post meeting email to take into account everything.

On CSSF optimization

R2-2505485	Introduction of CSSF optimization for NR RRM Ph5 (Alt1)	Apple	CR	Rel-19	38.331	18.6.0	5419	-	B	NR_RRM_Ph5-Core

Revised in R2-2506475

R2-2505486	Introduction of CSSF optimization for NR RRM Ph5 (Alt2)	Apple	CR	Rel-19	38.331	18.6.0	5420	-	B	NR_RRM_Ph5-Core

Not pursued

R2-2506432	Reply LS on CSSF optimization for NR RRM Phase 5

Noted

R2-2506475	Introduction of CSSF optimization for NR RRM Ph5	Apple	CR	Rel-19	38.331	18.6.0	5419	1	B	NR_RRM_Ph5-Core

The CR is agreed.

UE capability on 6 DL MIMO layers

R2-2505303	Discussion on DL MIMO layer capability for 6Rx UE	Xiaomi, Intel Corporation, Oppo, Ericsson, Nokia, Qualcomm Incorporated, ZTE, Sanechips, CATT, T-mobile USA, CHTTL	discussion	Rel-19	NR_ENDC_RF_Ph4	R2-2503446

Noted

Proposal 1: Introduce a new capability for maximum 6 DL MIMO layer as optional capability.

Observation 1: 8 DL MIMO layer can only be supported via 8 Rx receiver, which is only applicable for FWA. Generic signaling design in RAN2 specifications is used to indicate 8 DL MIMO layer in maxNumberMIMO-LayersPDSCH without differentiating FWA and handheld UE.

Proposal 2: RAN2 specification will not capture 6 DL MIMO layer is only applicable for FWA.

R2-2505610	Introduction of 6 DL MIMO layer	Xiaomi, Intel Corporation, Oppo, Ericsson, Nokia, Qualcomm Incorporated, ZTE, Sanechips, CATT, T-mobile USA, CHTTL	draftCR	Rel-19	38.306	18.6.0	B	NR_ENDC_RF_Ph4	R2-2503447

R2-2505611	Introduction of 6 DL MIMO layer	Xiaomi, Intel Corporation, Oppo, Ericsson, Nokia, Qualcomm Incorporated, ZTE, Sanechips, CATT, T-mobile USA, CHTTL	draftCR	Rel-19	38.331	18.6.0	B	NR_ENDC_RF_Ph4	R2-2503448

R2-2506187	Introduction of UE capability on 6 DL MIMO layers	Huawei, HiSilicon, Samsung, MediaTek Inc., Apple	draftCR	Rel-19	38.331	18.6.0	B	NR_ENDC_RF_Ph4

R2-2506188	Introduction of UE capability on 6 DL MIMO layers	Huawei, HiSilicon, Samsung, MediaTek Inc., Apple	draftCR	Rel-19	38.306	18.6.0	B	NR_ENDC_RF_Ph4

Discussions

-	HW think R4 does not specify that ‘6 DL MIMO layer is only applicable for FWA.  Xiaomi think R4 is still discussing.

-	T-mobile think the signalling design should be independent from the device type.

-	T-mobile suggest we technically endorse both set of CRs and it is up to the plenary to decide.

[CB] Friday in the main session

R2-2505304	Introduction of 6 DL MIMO layer	Xiaomi	CR	Rel-19	38.306	18.6.0	1320	-	B	NR_ENDC_RF_Ph4	R2-2503447	Withdrawn

R2-2505305	Introduction of 6 DL MIMO layer	Xiaomi	CR	Rel-19	38.331	18.6.0	5402	-	B	NR_ENDC_RF_Ph4	R2-2503448	Withdrawn

R2-2506110	Introduction of UE capability on 6 DL MIMO layers	Huawei, HiSilicon, Samsung, MediaTek Inc., Apple	CR	Rel-19	38.331	18.6.0	5458	-	B	NR_ENDC_RF_Ph4	Withdrawn

R2-2506111	Introduction of UE capability on 6 DL MIMO layers	Huawei, HiSilicon, Samsung, MediaTek Inc., Apple	CR	Rel-19	38.306	18.6.0	1341	-	B	NR_ENDC_RF_Ph4	Withdrawn

Simultaneous Rx-Tx capability for TDD-SDL band combination

R2-2505622	Corrections on simultaneous Rx-Tx capability for TDD-SDL band combination	Huawei, HiSilicon, Ericsson	CR	Rel-15	38.306	15.28.0	1310	1	F	LTE_NR_R19_Simult_RxTx	R2-2504734

R2-2505623	Corrections on simultaneous Rx-Tx capability for TDD-SDL band combination	Huawei, HiSilicon, Ericsson	CR	Rel-16	38.306	16.21.0	1311	1	A	LTE_NR_R19_Simult_RxTx	R2-2504735

R2-2505624	Corrections on simultaneous Rx-Tx capability for TDD-SDL band combination	Huawei, HiSilicon, Ericsson	CR	Rel-17	38.306	17.13.0	1312	1	A	LTE_NR_R19_Simult_RxTx	R2-2504736

R2-2505625	Corrections on simultaneous Rx-Tx capability for TDD-SDL band combination	Huawei, HiSilicon, Ericsson	CR	Rel-18	38.306	18.6.0	1313	1	A	LTE_NR_R19_Simult_RxTx	R2-2504737

=> The 4 CRs above are agreed, but then coversheet revised by MCC: wrong Rel-19 WI code for an earlier release CR

R2-2506639	Corrections on simultaneous Rx-Tx capability for TDD-SDL band combination	Huawei, HiSilicon, Ericsson	CR	Rel-15	38.306	15.28.0	1310	2	F	TEI15

R2-2506640	Corrections on simultaneous Rx-Tx capability for TDD-SDL band combination	Huawei, HiSilicon, Ericsson	CR	Rel-16	38.306	16.21.0	1311	2	A	TEI15

R2-2506641	Corrections on simultaneous Rx-Tx capability for TDD-SDL band combination	Huawei, HiSilicon, Ericsson	CR	Rel-17	38.306	17.13.0	1312	2	A	TEI15

R2-2506642	Corrections on simultaneous Rx-Tx capability for TDD-SDL band combination	Huawei, HiSilicon, Ericsson	CR	Rel-18	38.306	18.6.0	1313	2	A	TEI15

=> The 4 CRs are agreed

Signaling support for intra-band non-collocated EN-DC/NR-CA

R2-2506002	Introduction of signaling support for intra-band non-collocated EN-DC/NR-CA deployment Phase 2: new receiver type(s)	KDDI, OPPO, Apple, Ericsson, Huawei, HiSilicon, ZTE, Qualcomm Incorporated, Samsung	draftCR	Rel-19	38.331	18.6.0	NonCol_intraB_ENDC_NR_CA_Ph2-Core

R2-2506003	Introduction of signaling support for intra-band non-collocated EN-DC/NR-CA deployment Phase 2: new receiver type(s)	KDDI, OPPO, Apple, Ericsson, Huawei, HiSilicon, ZTE, Qualcomm Incorporated, Samsung	draftCR	Rel-19	38.306	18.6.0	NonCol_intraB_ENDC_NR_CA_Ph2-Core

R2-2506009	Introduction of signaling support for intra-band non-collocated EN-DC/NR-CA deployment Phase 2: new receiver type(s)	KDDI, OPPO, Apple, Ericsson, Huawei, HiSilicon, ZTE, Qualcomm Incorporated, Samsung	draftCR	Rel-19	38.331	18.6.0	NonCol_intraB_ENDC_NR_CA_Ph2-Core

Discussions

-	KDDI explains the need to send LS to RAN4, regarding reporting the capability for the super BC. HW not sure about the benefit for reporting the super BC, but fine to send LS.

-	QC think it should be allowed to report for super BC, and not sure if RAN4 can decide on this. Ericsson share the view from QC, think we should decide in RAN2.

-	MediaTek think super BC should be clearly specified in RAN4 spec. And, MediaTek think for intraBandNR-CA-non-collocated-r19 we should refer to the corresponding R4 spec in the 306 description part.

-	Nokia think the concept of ‘super BC’ is not so clear.

-	QC fine to send LS, and think the issue with super BC is for both type 2 and type 4.

-	Apple wonders what is our question to RAN4, e.g., is it about whether we allow UE reporting the super BC or the exact BC.

R2-2506002 and R2-2506009 are endorsed.

R2-2506003 is revised in R2-2506254.

R2-2506254	Introduction of signaling support for intra-band non-collocated EN-DC/NR-CA deployment Phase 2: new receiver type(s)	KDDI, OPPO, Apple, Ericsson, Huawei, HiSilicon, ZTE, Qualcomm Incorporated, Samsung	draftCR	Rel-19	38.306	18.6.0	NonCol_intraB_ENDC_NR_CA_Ph2-Core

The CR is endorsed

[AT131][205][NR_Others] Draft LS to RAN4 about Signaling support for intra-band non-collocated EN-DC/NR-CA (KDDI)

Intended outcome: draft LS to include questions on capability report for super BC in R2-2506248.

Deadline: before CB.

R2-2506256	Introduction of signaling support for intra-band non-collocated EN-DC/NR-CA deployment Phase 2: new receiver type(s)	KDDI, OPPO, Apple, Ericsson, Huawei, HiSilicon, ZTE, Qualcomm Incorporated, Samsung	CR	Rel-19	38.331	18.6.0	5479	-	B	NonCol_intraB_ENDC_NR_CA_Ph2-Core

The CR is agreed.

R2-2506248	Draft LS to RAN4 on Type 4 for non-collocated deployment

Noted, can discuss the LS in the next meeting

R2-2506472	LS on RRC signalling for power domain enhancement	RAN4	LS in	Rel-19	NR_ENDC_RF_Ph4-Core	To:RAN2

Noted

### 8.20.2	Other WGs

On low NR band CA via switching

R2-2505017	LS on Low NR band carrier aggregation via switching (R1-2504869; conact: Apple)	RAN1	LS in	Rel-19	NR_LBCA_Sw	To:RAN2, RAN4

Noted

R2-2505487	Introduction of low NR band carrier aggregation via switching	Apple, Telus, Nokia	CR	Rel-19	38.331	18.6.0	5421	-	B	NR_LBCA_Sw

Revised in R2-2506502

R2-2505488	Introduction of low NR band carrier aggregation via switching	Apple, Telus, Nokia (Rapporteur)	CR	Rel-19	38.300	18.6.0	1012	-	B	NR_LBCA_Sw

Revised in R2-2506250

Discussions

On scenario

-	Nokia think we start from CA.

-	QC think this is only for CA, and wonders whether this does not include other band/band pairs since it is switching btw bands. QC think we may check with R1/R4. ZTE think this is stilling being discussed in RAN4.

-	Apple, ZTE and HW think we focus on 2CC cases for now. Nokia think the R4 spec has the band combinations and it is clear.

Stage 2

-	ZTE think we do not need ‘RF chain’ in the text, want to change to ‘switching of patterns’.

-	Nokia

RRC

-	MediaTek should we should capture the R1 agreement for the Pcell to Scell case in the filed description.

-	ZTE think we should drop ‘It is expected that at maximum there is [X] switches within [Y] slots.’ Because R1 is still discussing.

-	Nokia think we use SetupRelease to include the new parameters.

[AT131][206][NR_Others] CRs for low NR band CA, discussion on capability (Apple)

Intended outcome: Updated CRs for low NR band CA in R2-2506249 and R2-2506250, proposals on UE capability R2-2506251.

Deadline: before CB.

R2-2506502	Introduction of low NR band carrier aggregation via switching	Apple, Telus, Nokia	CR	Rel-19	38.331	18.6.0	5421	2	B	NR_LBCA_Sw

Endorsed, will update the exact value in post meeting email discussion

R2-2506250	Introduction of low NR band carrier aggregation via switching	Apple, Telus, Nokia (Rapporteur)	CR	Rel-19	38.300	18.6.0	1012	1	B	NR_LBCA_Sw

The CR is agreed.

[Post131][226][NR_Others] CR for low NR band CA via switching (Apple)

Intended outcome: Agree the CR for TS38.331

Deadline:  Short

=> Agreed in R2-2506533 (38.331)

R2-2506533	Introduction of low NR band carrier aggregation via switching	Apple, Telus, Nokia	CR	Rel-19	38.331	18.6.0	5421	3	B	NR_LBCA_Sw

=> Agreed

Multi-carrier enh.

R2-2505016	LS on TS38.300 TP for Multi-carrier enhancements in Rel-19 (R1-2504861; contact: Lenovo)	RAN1	LS in	Rel-19	NR_MC_enh2	To:RAN2

Noted

R2-2505251	Stage 2 CR for Rel-19 Multi-carrier enhancements	Lenovo	CR	Rel-19	38.300	18.6.0	1005	-	B	NR_MC_enh2

Revised in R2-2506252

R2-2505252	Introduction of Rel-19 Multi-carrier enhancements	Lenovo	CR	Rel-19	38.331	18.6.0	5400	-	B	NR_MC_enh2

Revised in R2-2506253

[AT131][207][NR_Others] CRs for MC enh (Lenovo)

Intended outcome: Updated CRs for MC enh in R2-2506252, R2-2506253.

Deadline: before CB.

R2-2506252	Stage 2 CR for Rel-19 Multi-carrier enhancements	Lenovo	CR	Rel-19	38.300	18.6.0	1005	1	B	NR_MC_enh2

The CR is agreed.

R2-2506253	Introduction of Rel-19 Multi-carrier enhancements	Lenovo	CR	Rel-19	38.331	18.6.0	5400	1	B	NR_MC_enh2

The CR is agreed.

Number of UEs in RRC_INACTIVE state with data transmission

R2-2505292	Introduction of number of UEs in RRC_INACTIVE state with data transmission	China Telecom, Huawei, HiSilicon, ZTE Corporation, Sanechips, CATT, Ericsson, Nokia	CR	Rel-19	38.314	18.0.0	0034	4	B	PM_KPI_5G_Ph4	R2-2504742

The CR is agreed.

The following contributions will be handled in other session.

R2-2505068	LS on the RAN simulation assumptions for ULBC (S4-251584; contact: Qualcomm)	SA4	LS in	Rel-20	FS_ULBC	To:RAN1, RAN2, RAN4, SA2, CT1	Cc:SA1

R2-2505077	Discussion on SA4 LS regarding RAN Simulation Assumptions for ULBC	vivo	discussion	Rel-20	FS_ULBC

R2-2505428	Reply LS on the RAN simulation assumptions for ULBC	Qualcomm Technologies Ireland	LS out	Rel-20	FS_ULBC	SA4	SA2, CT1, RAN1

R2-2505601	Response to SA4 LS on the RAN simulation assumptions for ULBC	ZTE Corporation, Sanechips	discussion	Rel-20	FS_ULBC

R2-2506125	Draft Reply LS on the RAN Simulation Assumptions for ULBC	vivo	LS out	Rel-20	FS_ULBC	SA4, SA2	RAN1, CT1

# 9	Breakout session reports

No documents shall be submitted to this AI or its sub-AIs. It is only for at-meeting-generated contents.

## 9.1	Session on V2X/SL, R19 NES and MOB

R2-2506201	Report from session on R18 SL, R18/19 MOB, and R19 NES	Vice Chairman (Samsung)	Report

Approved

## 9.2	Session on Rel-18 MIMO, Rel-19 MIMO, LPWUS, SBFD, NR Others

R2-2506202	Report from session on Rel-18 MIMO, Rel-19 MIMO, LPWUS, SBFD, NR Others	Vice Chairman (CATT)	Report

Approved

6MIMO layer

- two CRs have been produced one for general and one for explicit

-	Xiaomi explains that we received an LS from RAN4 and thought that we should make it general similar to other.

-	Samsung thinks that we should just capture the agreements and send both.

-	Tmobile thinks we have a cross WG issue.

-	Huawei thinks that there is a clear agreement that the capability is specific for this capability.

-	Qualcomm thinks that this was RAN4 issue.  Apple thinks that it is clear that this is specific and not general.

RAN2 will provide two sets of technically endorsed CRs and it is up to the plenary to decide

The CRs that capture general capability are technically endorsed in R2-2506259 and R2-2506260

The CRs that capture specific capability are technically endorsed in R2-2506466 and R2-2506467

R2-2506472	LS on RRC signalling for power domain enhancement	RAN4	LS in	Rel-19	NR_ENDC_RF_Ph4-Core	To:RAN2

=> Noted

[POST131][044][NR Others] Power domain enh (Huawei)

Intended outcome: Prepare CR, review CR and agree if agreeable by companies.

Deadline:  Short

=> Agreed in R2-2506529 (38.331)

R2-2506529	Introduction of extension ratio configuration for MPR reduction	Huawei, HiSilicon	CR	Rel-19	38.331	18,6,0	5482	-	B	NR_ENDC_RF_Ph4-Core

=> Agreed

## 9.3	Session on NR NTN and IoT NTN

R2-2506203	Report from session on NR NTN and IoT NTN	Session chair (ZTE)	Report

Approved

## 9.4	Session on positioning and sidelink relay

R2-2506204	Report from session on positioning and sidelink relay	Session chair (MediaTek)	Report

Approved

## 9.5	Session on R19 XR and LTE-based 5G Broadcast

R2-2506205	Report from session on R19 XR and LTE Broadcast	Session chair (Huawei)	Report

Approved

## 9.6	Session on maintenance and SON/MDT

R2-2506206	Report from session on maintenance and SON/MDT	Session chair (Ericsson)	Report

=> Revised in R2-2506399

R2-2506399	Report from session on maintenance and SON/MDT	Session chair (Ericsson)	Report

Approved

# Closing of the meeting

The meeting was closed by the chair at 13:10 Indian Time on Friday, 29th of August.

# Annex A:	List of participants

RAN2#131 participants list is attached to this report.

Total number of participants: 371

# Annex B:	List of Tdocs

The list of tdocs from RAN2#131 is attached to this report.

Total of 1617 tdoc numbers were allocated of which 1585 tdocs were made available.

# Annex C:	Incoming liaison statements

| TDoc | Title | Source | Status | Rel | Related WIs | To | Cc | Original LS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R2-2505004 | Reply LS on S&F mode indications to NAS (C1-254119; contact: CICT Mobile) | CT1 | noted | Rel-19 | 5GSAT_Ph3_ARCH, IoT_NTN_Ph3-Core | RAN2 | SA2 | C1-254119 |
| R2-2505005 | Reply LS on AIoT device identifier length (C4-252465; contact: Huawei) | CT4 | noted | Rel-19 | AmbientIoT-CT | SA2 | CT1, RAN2, RAN3, SA3, CT3 | C4-252465 |
| R2-2505006 | Reply LS on paging ID length (C4-252466; contact: CICT Mobile) | CT4 | noted | Rel-19 | Ambient_IoT_Solutions | RAN2 | SA2, SA3, RAN3 | C4-252466 |
| R2-2505007 | LS on draft-ietf-6man-deprecate-router-alert, “Deprecation of the IPv6 Router Alert Option for New Protocols” (contact: Huawei) | IETF IPv6 Maintenance | noted |  |  | RAN2, SA2, CT1, CT3 CT4 |  |  |
| R2-2505008 | LS on Rel-19 higher layers parameters list Post RAN1#120bis (R1-2503154; contact: Ericsson) | RAN1 | noted | Rel-19 | NR_MIMO_Ph5, NR_duplex_evo, Netw_Energy_NR_enh, NR_LPWUS, NR_XR_Ph3, NR_Mob_Ph4, NR_MC_enh2, TEI19 | RAN2, RAN3 | RAN4 | R1-2503154 |
| R2-2505009 | LS on Rel-19 higher layers parameters list Post RAN1#121 (R1-2503242; contact: Ericsson) | RAN1 | noted | Rel-19 | NR_AIML_air, NR_MIMO_Ph5, NR_duplex_evo, Netw_Energy_NR_enh, NR_LPWUS, NR_Mob_Ph4, NR_XR_Ph3, NR_NTN_Ph3, IoT_NTN_Ph3, IoT_NTN_TDD, NR_MC_enh2, NR_LBCA_Sw, LTE_terr_bcast_Ph2, TEI19 | RAN2, RAN3 | RAN4 | R1-2503242 |
| R2-2505010 | LS on updated Rel-19 RAN1 UE features lists for NR after RAN1#121 (R1-2504675; contact: NTT DOCOMO, AT&T) | RAN1 | noted | Rel-19 | NR_AIML_air, NR_MIMO_Ph5, NR_duplex_evo, Netw_Energy_NR_enh, NR_LPWUS, NR_Mob_Ph4, NR_NTN_Ph3, NR_MC_enh2, TEI19 | RAN2 | RAN4 | R1-2504675 |
| R2-2505011 | LS on updated Rel-19 RAN1 UE features lists for LTE after RAN1#121 (R1-2504678; contact: NTT DOCOMO, AT&T) | RAN1 | noted | Rel-19 | IoT_NTN_Ph3, IoT_NTN_TDD, LTE_terr_bcast_Ph2 | RAN2 | RAN4 | R1-2504678 |
| R2-2505012 | LS on frequency location of CSI-RS resources for CSI acquisition in LTM (R1-2504828; contact: Fujitsu) | RAN1 | noted | Rel-19 | NR_Mob_Ph4-Core | RAN4 | RAN2 | R1-2504828 |
| R2-2505013 | LS on maximum transmission power for STxMP (R1-2504839; contact: Huawei) | RAN1 | noted | Rel-18 | NR_MIMO_evo_DL_UL-Core | RAN4 | RAN2 | R1-2504839 |
| R2-2505014 | Reply LS on startSFN for positioning SRS frequency hopping (R1-2504854; contact: ZTE) | RAN1 | noted | Rel-18 | NR_pos_enh2-Core | RAN2 |  | R1-2504854 |
| R2-2505015 | Reply LS on simultaneous configuration of SBFD and DC (R1-2504858; contact: Xiaomi) | RAN1 | noted | Rel-19 | NR_duplex_evo-Core | RAN2 | RAN3, RAN4 | R1-2504858 |
| R2-2505016 | LS on TS38.300 TP for Multi-carrier enhancements in Rel-19 (R1-2504861; contact: Lenovo) | RAN1 | noted | Rel-19 | NR_MC_enh2 | RAN2 |  | R1-2504861 |
| R2-2505017 | LS on Low NR band carrier aggregation via switching (R1-2504869; conact: Apple) | RAN1 | noted | Rel-19 | NR_LBCA_Sw | RAN2, RAN4 |  | R1-2504869 |
| R2-2505018 | LS on TP for 36.300 for IOT NTN TDD mode (R1-2504883; contact: Qualcomm) | RAN1 | noted | Rel-19 | IoT_NTN_TDD-Core | RAN2 |  | R1-2504883 |
| R2-2505019 | Reply LS on differentiation of sDCI based mTRP and sTRP (R1-2504885; contact: CATT) | RAN1 | noted | Rel-18 | NR_MIMO_evo_DL_UL-Core | RAN2 |  | R1-2504885 |
| R2-2505020 | Reply LS on LP-WUS in RRC_CONNECTED (R1-2504888; contact: NTT DOCOMO) | RAN1 | noted | Rel-19 | NR_LPWUS | RAN2 |  | R1-2504888 |
| R2-2505021 | Reply LS on CB Msg3 EDT for IoT NTN Ph3 (R1-2504905; contact: MediaTek) | RAN1 | noted | Rel-19 | IoT_NTN_Ph3 | RAN2 |  | R1-2504905 |
| R2-2505022 | LS on RAN2 aspects for LTE-based 5G Broadcast Phase 2 (R1-2504922; contact: EBU) | RAN1 | noted | Rel-19 | LTE_terr_bcast_Ph2 | RAN2 |  | R1-2504922 |
| R2-2505023 | LS on NR-NTN TP for TS 38.300 (NR_NTN_Ph3; contact: Thales) | RAN1 | noted | Rel-19 | NR_NTN_Ph3 | RAN2 |  | R1-2504934 |
| R2-2505024 | LS on Msg4 PDSCH repetition (R1-2504936; contact: Thales) | RAN1 | noted | Rel-19 | NR_NTN_Ph3 | RAN2 |  | R1-2504936 |
| R2-2505025 | Reply LS on LP-WUS UE RF (R1-2504943; contact: vivo) | RAN1 | noted | Rel-19 | NR_LPWUS-Core | RAN4 | RAN2 | R1-2504943 |
| R2-2505026 | Reply LS on CB Msg3 EDT for IoT NTN Ph3 (R1-2504959; contact: MediaTek) | RAN1 | noted | Rel-19 | IoT_NTN_Ph3 | RAN2 |  | R1-2504959 |
| R2-2505027 | LS on Draft CR on TS38.300 for Rel-19 MIMO (R1-2505008; contact: Samsung) | RAN1 | noted | Rel-19 | NR_MIMO_Ph5 | RAN2 |  | R1-2505008 |
| R2-2505028 | LS on TP to TS38.300 for Rel-19 LP-WUS/WUR (R1-2505070; contact: vivo) | RAN1 | noted | Rel-19 | NR_LPWUS-Core | RAN2 |  | R1-2505070 |
| R2-2505029 | LS on Rel-19 AI/ML positioning higher layer parameters list Post RAN1#121 (R1-2505073; contact: Ericsson) | RAN1 | noted | Rel-19 | NR_AIML_air | RAN2, RAN3 | RAN4 | R1-2505073 |
| R2-2505030 | LS on TP for TS38.300 on Rel-19 SBFD (R1-2505081; contact: Huawei) | RAN1 | noted | Rel-19 | NR_duplex_evo-Core | RAN2 | RAN3 | R1-2505081 |
| R2-2505031 | Reply LS on NR Femto node shared by PLMN and PNI-NPN (R3-252337; contact: LGE) | RAN3 | available | Rel-19 | 5G_Femto, NR_WAB_5GFemto-Core, eNPN | SA2 | RAN2 | R3-252337 |
| R2-2505032 | Reply LS on non-RedCap UE UL SRS frequency hopping for positioning (R3-253749; contact: ZTE) | RAN3 | noted | Rel-19 | TEI19 | RAN1, RAN2 |  | R3-253749 |
| R2-2505033 | Reply LS on the removal of service type information (R3-253800; contact: LGE) | RAN3 | noted | Rel-19 | AmbientIoT-ARC, Ambient_IoT_Solutions | SA2, RAN2 | RAN1 | R3-253800 |
| R2-2505034 | Reply LS on per-UE UE performance metrics (R3-253816; contact: Nokia) | RAN3 | noted | Rel-18 | NR_AIML_NGRAN-Core | SA5 | RAN2 | R3-253816 |
| R2-2505035 | Reply LS on LP-WUS subgrouping progress (R3-253846; contact: NEC) | RAN3 | noted | Rel-19 | NR_LPWUS-Core | RAN2 |  | R3-253846 |
| R2-2505036 | Reply LS on OAM-centric solution for NW-side data collection (S5-252842; contact: Huawei) | SA5 | noted | Rel-19 | AIML_MGT_Ph2 | RAN2 | RAN3 | S5-252842 |
| R2-2505037 | Reply LS on stage 1 requirements to support PWS over satellite NG-RAN in Rel-17 (R3-253867; contact: Ericsson) | RAN3 | noted | Rel-17 | NR_NTN_solutions-Core, LTE_NBIOT_eMTC_NTN-Core | SA1, CT1, RAN2 | SA2, CT4, RAN, SA | R3-253867 |
| R2-2505038 | LS on support of MRO for S-CPAC (R3-253886; contact: Samsung) | RAN3 | noted | Rel-19 | NR_ENDC_SON_MDT_Ph4-Core | RAN2 |  | R3-253886 |
| R2-2505039 | LS on uplink rate control (R3-253927; contact: Nokia) | RAN3 | noted | Rel-19 | NR_XR_Ph3-Core | RAN2 |  | R3-253927 |
| R2-2505040 | Reply LS on D2R message size for inventory response (R3-253940; contact: Xiaomi) | RAN3 | noted | Rel-19 | Ambient_IoT_Solutions | RAN2, SA2 | SA3 | R3-253940 |
| R2-2505041 | Reply to LS on SON for LTM (R3-253944; contact: Samsung) | RAN3 | noted | Rel-19 | NR_ENDC_SON_MDT_Ph4-Core | RAN2 |  | R3-253944 |
| R2-2505042 | Reply LS on signalling feasibility of dataset and parameter sharing (R3-253961; contact: Xiaomi) | RAN3 | noted | Rel-19 | NR_AIML_air-Core | RAN2, RAN1 | SA2, SA5, SA3 | R3-253961 |
| R2-2505043 | Reply LS on OAM-centric solution for NW-side data collection (R3-253962; contact: ZTE) | RAN3 | noted | Rel-19 | NR_AIML_air-Core | RAN2 | SA5 | R3-253962 |
| R2-2505044 | LS on Rel-19 RAN4 UE feature list for NR (version 1) (R4-2508078; contact: CMCC) | RAN4 | noted | Rel-19 | NR_ENDC_RF_Ph4, NonCol_intraB_ENDC_NR_CA_Ph2, NR_ATG_enh, NR_RRM_Ph5, Netw_Energy_NR_enh, NR_LPWUS, NR_Mob_Ph4, NR_XR_Ph3, NR_FR1_lessthan_5MHz_BW_Ph2, NR_LBCA_Sw, NR_FR1_7MHz_BW | RAN2 | RAN1 | R4-2508078 |
| R2-2505045 | LS on AI/ML functionality activation (R4-2508085; contact: CMCC) | RAN4 | noted | Rel-19 | NR_AIML_air-Core | RAN2 | RAN1 | R4-2508085 |
| R2-2505046 | Reply LS to RAN2 on Signalling for 7 MHz Channel Bandwidth (R4-2508088; contact: T-Mobile) | RAN4 | noted | Rel-19 | NR_FR1_7MHz_BW-Core | RAN2 | RAN3 | R4-2508088 |
| R2-2505047 | LS on UE assistance information (R4-2508312; contact: Nokia) | RAN4 | noted | Rel-19 | NR_XR_Ph3-Core | RAN2 | RAN1 | R4-2508312 |
| R2-2505048 | LS on UE signaling design for NR ATG enh (R4-2508329; contact: CMCC) | RAN4 | noted | Rel-19 | NR_ATG_enh | RAN2 |  | R4-2508329 |
| R2-2505049 | LS on mandatory gaps capability (R4-2508379; contact: Ericsson) | RAN4 | noted | Rel-16 | NR_RRM_enh-Core | RAN2 |  | R4-2508379 |
| R2-2505050 | Reply LS on SMTC enhancements (R4-2508433; contact: Xiaomi) | RAN4 | noted | Rel-19 | NR_NTN_Ph3-Core | RAN2 |  | R4-2508433 |
| R2-2505051 | Reply LS on new servingCellMO of OD-SSB on SCell (R4-2508440; contact: Apple) | RAN4 | noted | Rel-19 | Netw_Energy_NR_enh | RAN2 | RAN1 | R4-2508440 |
| R2-2505052 | LS to RAN2 on MUSIM gap and Measurement gap (R4-2508447; contact: R4-2508447) | RAN4 | noted | Rel-18 | NR_DualTxRx_MUSIM-Core | RAN2 |  | R4-2508447 |
| R2-2505053 | Reply LS from RAN on removal of support of PWS over satellite NG-RAN in Rel-17 and 18 (RP-251859; contact: Aalyria) | RAN | noted | Rel-17 | NR_NTN_solutions-Core, LTE_NBIOT_eMTC_NTN-Core | CT1, SA1, CT, SA, RAN3, RAN2 | SA2, CT4 | RP-251859 |
| R2-2505054 | Reply to LS on power and energy consumption budget for security features in A-IoT (RP-251886; contact: Huawei) | RAN | noted | Rel-19 | FS_Ambient_IoT_Sec, Ambient_IoT_Solutions-Core | SA3, RAN1 | RAN2, SA | RP-251886 |
| R2-2505055 | Reply LS on FS_VMR_Ph2 solution impacts to RAN (Additional ULI) (S2-2504110; contact: Qualcomm) | SA2 | available | Rel-19 | VMR_Ph2 | RAN3 | RAN2 | S2-2504110 |
| R2-2505056 | Reply LS on stage 1 requirements for the support for PWS over satellite NGRAN in Rel-17 (S2-2505538; contact: Samsung) | SA2 | noted | Rel-19 | IoT_NTN_Ph3-Core | RAN2, CT1 | SA3, RAN3, SA1 | S2-2505538 |
| R2-2505057 | Reply LS on AI/ML UE sided data collection (S2-2505713; contact: vivo, Samsung) | SA2 | noted | Rel-20 | NR_AIML_air, FS_AIML_CN_Ph2 | RAN | SA, SA3, SA5, RAN1, RAN2 | S2-2505713 |
| R2-2505058 | Reply LS on paging ID length (S2-2505793; contact: CATT) | SA2 | noted | Rel-19 | AmbientIoT-ARC, Ambient_IoT_Solutions | RAN2, CT4, SA3 | RAN3 | S2-2505793 |
| R2-2505059 | Reply LS on D2R message size for inventory response (S2-2505849; contact: InterDigital) | SA2 | noted | Rel-19 | AmbientIoT-ARC | RAN2, RAN3 |  | S2-2505849 |
| R2-2505060 | Reply LS on emergency call back and paging (S2-2505938; contact: Qualcomm) | SA2 | noted | Rel-17 | NR_newRAT-Core, NR_redcap-Core | RAN3 | RAN2, CT1, RAN | S2-2505938 |
| R2-2505061 | Reply LS on RTP retransmission (S2-2505975; contact: InterDigital) | SA2 | noted | Rel-19 | 5G_RTP_Ph2, XRM_Ph2 | SA4 | RAN2 | S2-2505975 |
| R2-2505062 | Reply LS on Handling of UE Radio Capability for Paging (S2-2506082; contact: Ericsson) | SA2 | noted | Rel-17 | NR_newRAT-Core, 5GS_Ph1, TEI17 | RAN3 | RAN2 | S2-2506082 |
| R2-2505063 | Response to Reply LS on paging capability loss issue (S2-2506087; contact: Vodafone) | SA2 | noted | Rel-19 | NR_LPWUS-Core, TEI19 | RAN2, RAN3 | RAN, CT1 | S2-2506087 |
| R2-2505064 | LS on security parameter in paging request message (S3-252392; contact: Huawei) | SA3 | noted | Rel-19 | FS_Ambient_IoT_Sec | RAN2 | SA2 | S3-252392 |
| R2-2505065 | Reply LS on security handling for inter-CU LTM in non-DC cases (S3-252398; contact: Huawei) | SA3 | noted | Rel-19 | NR_Mob_Ph4-Core | RAN3 | RAN2 | S3-252398 |
| R2-2505066 | LS to RAN3 on MBS Communication Service Type (S4-251096; contact: Nokia) | SA4 | noted | Rel-18 | NR_QoE_enh-Core | RAN3 | RAN2, SA5 | S4-251096 |
| R2-2505067 | Reply on Inclusion of NTN intended service area in the Service Announcement (S4-251099; contact: Ericsson) | SA4 | noted | Rel-19 | NR_NTN_Ph3-Core | RAN2 | SA2 | S4-251099 |
| R2-2505068 | LS on the RAN simulation assumptions for ULBC (S4-251584; contact: Qualcomm) | SA4 | noted | Rel-20 | FS_ULBC | RAN1, RAN2, RAN4, SA2, CT1 | SA1 | S4-251584 |
| R2-2506418 | LS on delayed A-IoT D2R NAS messages (C1-255165; contact: Huawei) | CT1 | noted | Rel-19 | Ambient_IoT_Solutions | RAN2 | SA2 | C1-255165 |
| R2-2506432 | Reply LS on CSSF optimization for NR RRM Phase 5 (R4-2512161; contact: Apple) | RAN4 | noted | Rel-18 | NR_RRM_Ph5-Core | RAN2 |  | R4-2512161 |
| R2-2506472 | LS on RRC signalling for power domain enhancement (R4-2511759; contact: Huawei) | RAN4 | noted | Rel-19 | NR_ENDC_RF_Ph4-Core | RAN2 |  | R4-2511759 |

68 incoming LS, of which 66 LS were noted. The remaining non-treated LSin will be treated in RAN2#131bis.

# Annex D:	Outgoing liaison statements

| TDoc | Title | Rel | Related WIs | To | Cc |
| --- | --- | --- | --- | --- | --- |
| R2-2506216 | LS on RAN2 agreement on SUL support for SIB1 request | Rel-19 | Netw_Energy_NR_enh-Core | RAN1 |  |
| R2-2506261 | LS on enabling/disabling LP-WUS per UE with NAS signalling | Rel-19 | NR_LPWUS-Core | SA2, CT1, RAN3 |  |
| R2-2506284 | LS on power ramping and RRC configuration for CB-Msg3-EDT | Rel-19 | IoT_NTN_Ph3-Core | RAN1 |  |
| R2-2506285 | LS on S&F mode operation | Rel-19 | IoT_NTN_Ph3-Core | CT1 | SA2 |
| R2-2506297 | LS PWS support in NB-IoT terrestrial networks | Rel-19 | NR_NTN_Ph3-Core | SA1 | SA2 |
| R2-2506298 | LS on additional agreements on SMTC enhancements | Rel-19 | NR_NTN_Ph3-Core | RAN4 |  |
| R2-2506299 | LS on CQI reporting for CB-Msg3-EDT | Rel-19 | IoT_NTN_Ph3-Core | RAN4 | RAN1 |
| R2-2506300 | LS on OCC for IoT-NTN TDD mode | Rel-19 | IoT_NTN_TDD-Core | RAN1 |  |
| R2-2506465 | Reply LS on security parameter in paging request message | Rel-19 | FS_Ambient_IoT_solutions | SA3 | RAN3, SA2 |
| R2-2506470 | LS on candidate data collection | Rel-19 | NR_AIML_air-Core | RAN1 | RAN4 |
| R2-2506474 | RAN2 inputs to TR 38.843 | Rel-19 | FS_NR_AIML_air_Ph2 | RAN1 |  |
| R2-2506492 | LS on simultaneous configuration of SBFD and DC | Rel-19 | NR_duplex_evo-Core | RAN1, RAN4 | RAN3 |
| R2-2506504 | LS on early CSI acquisition for L3 handover | Rel-19 | TEI19 | RAN1 | RAN4 |
| R2-2506519 | LS on when RRC layer submits periodic CSI inference configuration to lower layer | Rel-19 | NR_AIML_air-Core | RAN1 |  |
| R2-2506520 | Reply LS on AI/ML functionality activation | Rel-19 | NR_AIML_air-Core | RAN4 | RAN1 |
| R2-2506540 | LS on two-TA configuration and UE-initiated CSI reporting | Rel-19 | NR_MIMO_Ph5 | RAN1 |  |
| R2-2506541 | LS on User consent for Data collection at the UE for NW-side model training | Rel-19 | NR_AIML_air-Core | SA3 | RAN3, SA5 |
| R2-2506543 | Reply LS on OAM-centric solution for NW-side data collection | Rel-19 | AIML_MGT_Ph2 | SA5 | RAN3 |
| R2-2506545 | LS on NW side data collection | Rel-19 | NR_AIML_air-Core | RAN1, RAN3 |  |
| R2-2506550 | LS on not supporting simultaneous LR and MR operation | Rel-19 | NR_LPWUS-Core | RAN1 | RAN4 |
| R2-2506565 | LS on per band and per BC capability | Rel-19 | NR_MIMO_Ph5, NR_AIML_air | RAN1 | RAN4 |
| R2-2506569 | LS on topologies for L2 based multi-hop U2N relay | Rel-19 | NR_SL_relay_multihop-Core | SA2 |  |
| R2-2506658 | LS on the implicit indication of TRP location coordinates via "Associated ID" | Rel-19 | NR_AIML_air-Core | RAN1 |  |

23 outgoing LS.

# Annex E:	List of agreed CRs

| TDoc | Title | Source | Rel | Spec | Related WIs | CR | Rev | Cat |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R2-2505084 | Introduction of E-UTRAN to NB-IoT NTN Mobility UE Capability [EUTRAN-to-NBIoTNTN] | vivo, Samsung, Google, THALES, MediaTek Inc., Aalyria | Rel-19 | 36.306 | TEI19 | 1917 |  | B |
| R2-2505094 | Introduction of B2b signal in BDS system in A-GNSS | CAICT, CATT, CMCC, China Telecome, China Unicom, Ericsson, Huawei, HiSilicon, Lenovo, OPPO, vivo, Xiaomi, ZTE, MediaTek Inc, Qualcomm Incorporated | Rel-19 | 37.355 | LCS_BDS_B2b_LTE_NR-Core | 0545 | 2 | B |
| R2-2505155 | Correction on NR Sidelink measurements based on SL-PRS | vivo, Ericsson | Rel-18 | 38.331 | NR_pos_enh2-Core | 5398 |  | F |
| R2-2505209 | Introduction of SONMDT UE Capabilities | CATT | Rel-19 | 36.306 | NR_ENDC_SON_MDT_Ph4-Core | 1915 |  | B |
| R2-2505270 | SSB position restrictions for less-than-5MHz SCells | Qualcomm Incorporated | Rel-19 | 38.331 | NR_FR1_lessthan_5MHz_BW_Ph2-Core | 5249 | 3 | B |
| R2-2505283 | Introduce UE capability signalling for NTN less than 5MHz | ZTE Corporation, Xiaomi, Sanechips | Rel-19 | 38.306 | NR_IoT_NTN_req_test_enh | 1307 | 2 | B |
| R2-2505292 | Introduction of number of UEs in RRC_INACTIVE state with data transmission | China Telecom, Huawei, HiSilicon, ZTE Corporation, Sanechips, CATT, Ericsson, Nokia | Rel-19 | 38.314 | PM_KPI_5G_Ph4 | 0034 | 4 | B |
| R2-2505367 | Introduction of NB-IoT satellite information in E-UTRAN [EUTRAN-to-NBIoTNTN] | Google, Samsung, vivo, THALES, MediaTek Inc., Aalyria | Rel-19 | 36.331 | TEI19 | 5140 |  | B |
| R2-2505368 | Introduction of NB-IoT satellite information in E-UTRAN [EUTRAN-to-NBIoTNTN] | Google, Samsung, vivo, THALES, MediaTek Inc., Aalyria | Rel-19 | 36.300 | TEI19 | 1427 |  | B |
| R2-2505454 | Introduction of band specific capability for paging [Per_Band_Paging_Cap] | Huawei, Nokia, Xiaomi, Ericsson | Rel-19 | 38.331 | TEI19 | 5417 |  | B |
| R2-2505600 | Miscellaneous corrections on Aggregated SP Positioning SRS Activation/Deactivation MAC CE | Lenovo | Rel-18 | 38.321 | NR_pos_enh2-Core | 2107 |  | F |
| R2-2505626 | Correction on DCP considering MUSIM gaps | Huawei, HiSilicon | Rel-17 | 38.321 | LTE_NR_MUSIM-Core, NR_UE_pow_sav-Core | 2108 |  | F |
| R2-2505627 | Correction on DCP considering MUSIM gaps | Huawei, HiSilicon | Rel-18 | 38.321 | LTE_NR_MUSIM-Core, NR_UE_pow_sav-Core | 2109 |  | A |
| R2-2505720 | Introduction of NavIC L1 SPS A-GNSS in NR Stage 2 specification | Ericsson, Reliance Jio, ISRO, MediaTek Inc., CEWiT, Huawei | Rel-19 | 38.305 | LCS_NAVIC_L1_SPS_NR_LTE-Core | 0179 | 2 | B |
| R2-2505721 | Introduction of NavIC L1 SPS A-GNSS in LTE Stage 2 specification | Ericsson, Reliance Jio, ISRO, MediaTek Inc., CEWiT, Huawei | Rel-19 | 36.305 | LCS_NAVIC_L1_SPS_NR_LTE-Core | 0120 | 2 | B |
| R2-2505758 | ProSe support in NPN [ProSe_NPN] | ZTE Corporation, Sanechips, Nokia, LGE, Philips | Rel-19 | 38.331 | TEI19 | 5209 | 1 | B |
| R2-2505812 | Correction to scellWithoutSSB | Ericsson | Rel-18 | 38.306 | Netw_Energy_NR-Core | 1338 |  | F |
| R2-2505963 | Introducing PDCCH CCE Usage for L2M [PDCCH_Usage] | CMCC, Huawei, Ericsson, ZTE, CATT, Nokia | Rel-19 | 38.314 | TEI19 | 0035 | 2 | B |
| R2-2506012 | Correction on Power Ramping for early TA | Samsung | Rel-18 | 38.321 | NR_Mob_enh2-Core | 2111 |  | F |
| R2-2506077 | Correction on procedual text for successPSCell-Config | Huawei, HiSilicon, CATT, CMCC, Samsung, Ericsson | Rel-18 | 38.331 | NR_ENDC_SON_MDT_enh2-Core | 5454 |  | F |
| R2-2506086 | Introduction of ANR reporting of HSDN cells [ANR_HSDN] | Huawei, HiSilicon, CMCC, China Unicom, China Telecom, CATT, NTT DoCoMo, Samsung | Rel-19 | 36.331 | TEI19 | 5110 | 3 | B |
| R2-2506087 | Introduction of ANR reporting of HSDN cells [ANR_HSDN] | Huawei, HiSilicon, CMCC, China Unicom, China Telecom, CATT, NTT DoCoMo, Samsung | Rel-19 | 36.306 | TEI19 | 1911 | 2 | B |
| R2-2506107 | Correction to startPreambleForThisPartition and numberOfPreamblesPerSSB-ForThisPartition in RA-report | Sharp | Rel-18 | 38.331 | NR_ENDC_SON_MDT_enh2-Core | 5456 |  | F |
| R2-2506160 | Correction to reportQuantity | Ericsson | Rel-18 | 38.331 | NR_MIMO_evo_DL_UL-Core | 5462 |  | F |
| R2-2506197 | Introduction of ANR reporting of HSDN cells [ANR_HSDN] | Huawei, HiSilicon, CMCC, China Unicom, China Telecom, CATT, NTT DoCoMo, Samsung | Rel-19 | 38.306 | TEI19 | 1264 | 3 | B |
| R2-2506221 | Introduction of enhancements for network energy efficiency | Ericsson | Rel-19 | 38.331 | Netw_Energy_NR_enh-Core | 5428 | 1 | B |
| R2-2506229 | Introduction of NR mobility enhancements Phase 4 in MAC | vivo (Rapporteur) | Rel-19 | 38.321 | NR_Mob_Ph4-Core | 2098 | 1 | B |
| R2-2506232 | Clarification on PDCCH order triggered early RACH | ZTE Corporation | Rel-18 | 38.321 | NR_Mob_enh2-Core | 2115 | 2 | F |
| R2-2506250 | Introduction of low NR band carrier aggregation via switching | Apple, Telus, Nokia (Rapporteur) | Rel-19 | 38.300 | NR_LBCA_Sw | 1012 | 1 | B |
| R2-2506252 | Stage 2 CR for Rel-19 Multi-carrier enhancements | Lenovo | Rel-19 | 38.300 | NR_MC_enh2 | 1005 | 1 | B |
| R2-2506253 | Introduction of Rel-19 Multi-carrier enhancements | Lenovo | Rel-19 | 38.331 | NR_MC_enh2 | 5400 | 1 | B |
| R2-2506256 | Introduction of signaling support for intra-band non-collocated EN-DC/NR-CA deployment Phase 2: new receiver type(s) | KDDI, OPPO, Apple, Ericsson, Huawei, HiSilicon, ZTE, Qualcomm Incorporated, Samsung | Rel-19 | 38.331 | NonCol_intraB_ENDC_NR_CA_Ph2-Core | 5479 |  | B |
| R2-2506274 | Clarification on reference point for hard satellite switch with resynchronization | Qualcomm Inc., Huawei, HiSilicon, Xiaomi | Rel-18 | 38.331 | NR_NTN_enh-Core | 5372 | 2 | F |
| R2-2506275 | Corrections on IoT NTN SIB33 | Samsung | Rel-18 | 36.331 | IoT_NTN_enh-Core | 5121 | 2 | F |
| R2-2506277 | Clarification of satellite identifiers | THALES, Samsung, Sateliot, Nordic Semiconductor ASA, Ericsson | Rel-18 | 36.331 | IoT_NTN_enh-Core | 5152 | 1 | F |
| R2-2506295 | Clarification of satellite identifiers | THALES, Samsung, Sateliot, Nordic Semiconductor ASA, Novamint, CATT, Ericsson | Rel-18 | 36.300 | IoT_NTN_enh-Core | 1430 | 2 | F |
| R2-2506304 | Correction on SP Positioning SRS frequency hopping | Huawei, HiSilicon, Ericsson, Ofinno | Rel-18 | 38.321 | NR_pos_enh2-Core | 2120 |  | F |
| R2-2506306 | Miscellaneous corrections on NR positioning enhancements | Lenovo | Rel-18 | 38.331 | NR_pos_enh2-Core | 5425 | 1 | F |
| R2-2506307 | Correction for timing Reporting Granularity Factor | Ericsson | Rel-18 | 37.355 | NR_pos_enh2-Core | 0560 | 1 | F |
| R2-2506308 | Correction regarding SL-PRS Resource Request | ASUSTeK | Rel-18 | 38.321 | NR_pos_enh2-Core | 2112 | 1 | F |
| R2-2506311 | Correction to PDCP duplication description for L2 MP using SL relay or N3C indirect path | Huawei, HiSilicon, Nokia (Rapporteur), CMCC, China Telecom | Rel-18 | 38.300 | NR_SL_relay_enh-Core | 0989 | 2 | F |
| R2-2506313 | Corrections on the startSFN of the UTW | ZTE Corporation, Ericsson, vivo, Qualcomm, Samsung, Nokia, Lenovo | Rel-18 | 38.331 | NR_pos_enh2-Core | 5464 | 1 | F |
| R2-2506314 | Introduction of BDS B2b in A-GNSS | CAICT, CATT, CMCC, China Telecome, China Unicom, Ericsson, Huawei, HiSilicon, Lenovo, OPPO, vivo, Xiaomi, ZTE, MediaTek Inc, Qualcomm Incorporated | Rel-19 | 36.305 | LCS_BDS_B2b_LTE_NR-Core | 0121 | 2 | B |
| R2-2506315 | Introduction of BDS B2b in A-GNSS for TS 38305 | CAICT, CATT, CMCC, China Telecome, China Unicom, Ericsson, Huawei, HiSilicon, Lenovo, OPPO, vivo, Xiaomi, ZTE, MediaTek Inc, Qualcomm Incorporated | Rel-19 | 38.305 | LCS_BDS_B2b_LTE_NR-Core | 0180 | 2 | B |
| R2-2506318 | UE request for equalIntegerAmbiguityLevel assistance data [GNSS-EqualIntegerAmbiguity] | AT&T, Ericsson, Huawei, CATT, Samsung, ZTE Corporation, Nokia, Deutsche Telekom | Rel-19 | 37.355 | TEI19 | 0557 | 2 | B |
| R2-2506319 | Introduction of periodic delivery of NR integrity service alert [IntegrityPeriodicAD] | Huawei, HiSilicon, Ericsson, Vivo | Rel-19 | 38.305 | TEI19 | 0192 | 2 | B |
| R2-2506320 | Introduction of UE capability for SRS frequency hopping for non-RedCap UE in 37355 [Pos_SRSHop] | ZTE Corporation | Rel-19 | 37.355 | TEI19 | 0553 | 2 | B |
| R2-2506321 | Introduction on the SRS frequency hopping for non-RedCap UE in 38331 [Pos_SRSHop] | ZTE Corporation, Ericsson | Rel-19 | 38.331 | TEI19 | 5290 | 3 | B |
| R2-2506328 | Introduction of multi-hop U2N relay in TS 38.304 | MediaTek Inc. | Rel-19 | 38.304 | NR_SL_relay_multihop-Core | 0444 | 1 | B |
| R2-2506335 | Introduction of XR Enhancements Phase 3 | Nokia | Rel-19 | 38.300 | NR_XR_Ph3-Core | 1007 | 1 | B |
| R2-2506337 | Introduction of R19 XR enhancements for PDCP spec. | LG Electronics Inc. | Rel-19 | 38.323 | NR_XR_Ph3-Core | 0149 | 2 | B |
| R2-2506338 | Introduction of R19 XR enhancements for RLC spec. | vivo | Rel-19 | 38.322 | NR_XR_Ph3-Core | 0065 | 1 | B |
| R2-2506342 | Introduction of CAS muting in LTE-based 5G broadcast [5GB_CASMuting] | Huawei, HiSilicon, Qualcomm Incorporated, EBU | Rel-19 | 36.331 | TEI19 | 5139 | 2 | B |
| R2-2506343 | Introduction of CAS muting in LTE-based 5G broadcast [5GB_CASMuting] | Huawei, HiSilicon, Qualcomm Incorporated, EBU | Rel-19 | 36.306 | TEI19 | 1916 | 1 | B |
| R2-2506344 | Introduction of LTE-based 5G Broadcast Phase 2 | Qualcomm Incorporated, EBU | Rel-19 | 36.331 | LTE_terr_bcast_Ph2-Core | 5143 | 1 | B |
| R2-2506346 | Introduction of LTE-based 5G Broadcast Phase 2 | Huawei, HiSilicon, Qualcomm Incorporated | Rel-19 | 36.306 | LTE_terr_bcast_Ph2-Core | 1920 | 1 | B |
| R2-2506347 | Introduction of LTE-based 5G Broadcast Phase 2 | ZTE Corporation, Sanechips | Rel-19 | 36.300 | LTE_terr_bcast_Ph2-Core | 1428 | 1 | B |
| R2-2506348 | Introduction of LTE-based 5G Broadcast Phase 2 | Samsung | Rel-19 | 36.321 | LTE_terr_bcast_Ph2-Core | 1593 | 2 | B |
| R2-2506351 | Correction on LTE RLF report logging | CATT | Rel-16 | 36.331 | LTE_feMob-Core | 5134 | 1 | F |
| R2-2506352 | Correction on LTE RLF report logging | CATT | Rel-17 | 36.331 | LTE_feMob-Core | 5135 | 1 | A |
| R2-2506353 | Correction on LTE RLF report logging | CATT | Rel-18 | 36.331 | LTE_feMob-Core | 5136 | 1 | A |
| R2-2506354 | Correction on MBMS Interest Indication | Samsung, Nokia, Ericsson, Qualcomm Incorporated, ZTE | Rel-18 | 36.331 | LTE_terr_bcast-Core, TEI18 | 5125 | 2 | F |
| R2-2506355 | Correction on previousPCellId in RLF report | Huawei, HiSilicon, CATT, CMCC | Rel-16 | 38.331 | NR_SON_MDT-Core | 5451 | 1 | F |
| R2-2506356 | Correction on previousPCellId in RLF report | Huawei, HiSilicon, CATT, CMCC | Rel-17 | 38.331 | NR_SON_MDT-Core | 5452 | 1 | A |
| R2-2506357 | Correction on previousPCellId in RLF report | Huawei, HiSilicon, CATT, CMCC | Rel-18 | 38.331 | NR_SON_MDT-Core | 5453 | 1 | A |
| R2-2506359 | Correction on intraF-NeighMeasForSCellWithoutSSB capability | ZTE Corporation, Sanechips, Nokia, Huawei, HiSilicon | Rel-15 | 38.306 | NR_newRAT-Core | 1327 | 1 | F |
| R2-2506360 | Correction on intraF-NeighMeasForSCellWithoutSSB capability | ZTE Corporation, Sanechips, Nokia, Huawei, HiSilicon | Rel-16 | 38.306 | NR_newRAT-Core | 1328 | 1 | A |
| R2-2506361 | Correction on intraF-NeighMeasForSCellWithoutSSB capability | ZTE Corporation, Sanechips, Nokia, Huawei, HiSilicon | Rel-17 | 38.306 | NR_newRAT-Core | 1329 | 1 | A |
| R2-2506363 | Correction on configured grant power control in unified TCI framework | Ofinno | Rel-17 | 38.331 | NR_FeMIMO-Core | 5439 | 1 | F |
| R2-2506364 | Correction on configured grant power control in unified TCI framework | Ofinno | Rel-18 | 38.331 | NR_FeMIMO-Core | 5440 | 1 | A |
| R2-2506374 | Correction on UL spatial multiplexing | ASUSTeK | Rel-18 | 36.321 | LTE_LATRED_L2-Core, TEI18 | 1596 | 1 | F |
| R2-2506375 | Correction on the dlRSRPAboveThreshold in RA-report | Xiaomi | Rel-18 | 38.331 | NR_ENDC_SON_MDT_enh2-Core | 5466 | 1 | F |
| R2-2506403 | Restriction on RAT utilization | Apple, Vodafone, InterDigital, Ericsson | Rel-19 | 36.300 | ECRATU | 1432 |  | B |
| R2-2506404 | Restriction on RAT utilization | Apple, Vodafone, InterDigital, Ericsson | Rel-19 | 38.300 | ECRATU | 1024 |  | B |
| R2-2506408 | Introduction of Multi-path Relay Enhancement [N3C_M_Relay] | CMCC, ZTE, MediaTek, vivo, Huawei, CATT, Meta, Nokia, Nokia Shanghai Bell, Xiaomi, Spreadtrum, UNISOC | Rel-19 | 38.300 | TEI19 | 0991 | 2 | B |
| R2-2506409 | Introduction of Multi-path Relay Enhancement [N3C_M_Relay] | CMCC, ZTE, MediaTek, vivo, Huawei, CATT, Meta, Nokia, Nokia Shanghai Bell, Xiaomi, Spreadtrum, UNISOC | Rel-19 | 38.331 | TEI19 | 5373 | 2 | B |
| R2-2506410 | Introduction of Multi-path Relay Enhancement [N3C_M_Relay] | CMCC, ZTE, MediaTek, vivo, Huawei, CATT, Meta, Nokia, Nokia Shanghai Bell, Xiaomi, Spreadtrum, UNISOC | Rel-19 | 38.306 | TEI19 | 1296 | 3 | B |
| R2-2506411 | Correction to multi-Rx preference reporting | Huawei, HiSilicon, Apple | Rel-18 | 38.331 | NR_FR2_multiRX_DL-Core | 5449 | 1 | F |
| R2-2506412 | SR triggered SSSG switching [SRTrig_SSSGSwitch] | Ericsson, Qualcomm Incorporated | Rel-19 | 38.331 | TEI19 | 5396 | 1 | B |
| R2-2506419 | Correction on the configuration of the joint TCI for multi-cell scheduling | Ofinno | Rel-18 | 38.331 | NR_MC_enh-Core | 5401 | 1 | F |
| R2-2506424 | Restriction on RAT utilization | Apple, Vodafone, InterDigital, Ericsson, Nokia, OPPO, Samsung | Rel-19 | 36.304 | ECRATU | 0884 | 1 | B |
| R2-2506425 | Restriction on RAT utilization | Apple, Vodafone, InterDigital, Ericsson, Nokia, OPPO, Samsung | Rel-19 | 38.304 | ECRATU | 0446 | 1 | B |
| R2-2506430 | Introduction of NavIC L1 SPS A-GNSS in LPP | Ericsson, Reliance Jio, ISRO, MediaTek Inc., CEWiT, Huawei | Rel-19 | 37.355 | LCS_NAVIC_L1_SPS_NR_LTE-Core | 0532 | 6 | B |
| R2-2506435 | Correction to F field in MAC subheader for DL-SCH and UL-SCH | NTT DOCOMO INC., Huawei, HiSilicon, MediaTek Inc., Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, Samsung, ZTE | Rel-15 | 38.321 | NR_newRAT-Core | 2116 | 1 | F |
| R2-2506437 | Correction to F field in MAC subheader for DL-SCH and UL-SCH | NTT DOCOMO INC., Huawei, HiSilicon, MediaTek Inc., Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, Samsung, ZTE | Rel-17 | 38.321 | NR_newRAT-Core | 2118 | 1 | A |
| R2-2506438 | Correction to F field in MAC subheader for DL-SCH and UL-SCH | NTT DOCOMO INC., Huawei, HiSilicon, MediaTek Inc., Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, Samsung, ZTE | Rel-18 | 38.321 | NR_newRAT-Core | 2119 | 1 | A |
| R2-2506442 | Correction on Presence condition of ssbFrequency in servingCellMO for SSB-less SCell | ZTE Corporation, Sanechips, Huawei, HiSilicon, Nokia | Rel-15 | 38.331 | NR_newRAT-Core | 5470 |  | F |
| R2-2506443 | Correction on Presence condition of ssbFrequency in servingCellMO for SSB-less SCell | ZTE Corporation, Sanechips, Huawei, HiSilicon, Nokia | Rel-16 | 38.331 | NR_newRAT-Core | 5471 |  | A |
| R2-2506444 | Correction on Presence condition of ssbFrequency in servingCellMO for SSB-less SCell | ZTE Corporation, Sanechips, Huawei, HiSilicon, Nokia | Rel-17 | 38.331 | NR_newRAT-Core | 5472 |  | A |
| R2-2506445 | Correction on Presence condition of ssbFrequency in servingCellMO for SSB-less SCell | ZTE Corporation, Sanechips, Huawei, HiSilicon, Nokia | Rel-18 | 38.331 | NR_newRAT-Core, Netw_Energy_NR-Core | 5473 |  | A |
| R2-2506446 | Clarification to musim-AffectedBandsList | ZTE Corporation | Rel-18 | 38.331 | NR_DualTxRx_MUSIM-Core | 5447 | 1 | F |
| R2-2506459 | Correction to the description of PDU set QoS parameter | Huawei, HiSilicon, Nokia (Rapporteur) | Rel-18 | 38.300 | NR_XR_enh-Core | 1010 | 1 | F |
| R2-2506461 | Introduction of UAV mobility enhancements [UAV_Mobility] | CATT, NTT DOCOMO, LG Electronics Inc., Kyocera, LGU+, China Telecom, NEC, SK Telecom, Qualcomm Incorporated, Ericsson, Nokia, InterDigital, vivo, CMCC | Rel-19 | 38.300 | TEI19 | 1004 | 1 | B |
| R2-2506462 | Introduction of UAV mobility enhancements [UAV_Mobility] | CATT, NTT DOCOMO, LG Electronics Inc., Kyocera, LGU+, China Telecom, NEC, SK Telecom, Qualcomm Incorporated, Ericsson, Nokia, InterDigital, vivo, CMCC | Rel-19 | 38.304 | TEI19 | 0439 | 1 | B |
| R2-2506463 | Introduction of UAV mobility enhancements [UAV_Mobility] | CATT, NTT DOCOMO, LG Electronics Inc., Kyocera, LGU+, China Telecom, NEC, SK Telecom, Qualcomm Incorporated, Ericsson, Nokia, InterDigital, vivo, CMCC | Rel-19 | 38.306 | TEI19 | 1319 | 1 | B |
| R2-2506464 | Introduction of UAV mobility enhancements [UAV_Mobility] | CATT, NTT DOCOMO, LG Electronics Inc., Kyocera, LGU+, China Telecom, NEC, SK Telecom, Qualcomm Incorporated, Ericsson, Nokia, InterDigital, vivo, CMCC | Rel-19 | 38.331 | TEI19 | 5399 | 1 | B |
| R2-2506469 | Corrections on Rel-18 UE capability descriptions, including [HARQ-ACK MUX on PUSCH] | Xiaomi | Rel-18 | 38.306 | NR_MIMO_evo_DL_UL, NR_Mob_enh2, Netw_Energy_NR, NR_XR_enh, NR_NTN_enh, NR_SL_enh2, TEI18 | 1322 | 1 | F |
| R2-2506471 | Introduction of 32 HARQ processes to TN [TN32HARQ] | Huawei, HiSilicon, Ericsson, ZTE Corporation | Rel-19 | 38.331 | TEI19 | 5410 | 1 | B |
| R2-2506475 | Introduction of CSSF optimization for NR RRM Ph5 | Apple | Rel-19 | 38.331 | NR_RRM_Ph5-Core | 5419 | 1 | B |
| R2-2506476 | Correction to F field in MAC subheader for DL-SCH and UL-SCH | NTT DOCOMO INC., Huawei, HiSilicon, MediaTek Inc., Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, Samsung, ZTE | Rel-16 | 38.321 | NR_newRAT-Core | 2117 | 2 | A |
| R2-2506477 | Introduction of periodic AD for NR integrity service alert [IntegrityPeriodicAD] | Huawei, HiSilicon, Ericsson, Vivo | Rel-19 | 37.355 | TEI19 | 0558 | 2 | B |
| R2-2506478 | Correction on DRX Command handling | Samsung | Rel-17 | 38.321 | NR_MBS-Core | 2113 | 1 | F |
| R2-2506479 | Correction on DCP considering MUSIM gaps | Huawei, HiSilicon, Nokia (Rapporteur), vivo | Rel-17 | 38.300 | LTE_NR_MUSIM-Core, NR_UE_pow_sav-Core | 1025 | 1 | F |
| R2-2506480 | Correction on DCP considering MUSIM gaps | Huawei, HiSilicon, Nokia (Rapporteur), vivo | Rel-18 | 38.300 | LTE_NR_MUSIM-Core, NR_UE_pow_sav-Core | 1026 | 1 | A |
| R2-2506481 | Correction on DRX Command handling | Samsung | Rel-18 | 38.321 | NR_MBS-Core | 2114 | 1 | A |
| R2-2506483 | UE capability for support of event A4 based CHO for ATG | Qualcomm Inc., Samsung | Rel-18 | 38.306 | NR_ATG-Core | 1330 | 2 | F |
| R2-2506484 | UE capability for support of event A4 based CHO for ATG | Qualcomm Inc., Samsung | Rel-18 | 38.331 | NR_ATG-Core | 5468 | 1 | F |
| R2-2506490 | Introduction of Ambient IoT | CMCC | Rel-19 | 38.300 | Ambient_IoT_Solutions | 1022 | 1 | B |
| R2-2506493 | Introduction of IoT NTN enhancements phase 3 | MediaTek Inc. | Rel-19 | 36.321 | IoT_NTN_Ph3-Core | 1591 | 1 | B |
| R2-2506494 | Introduction of IoT NTN phase 3 | Ericsson | Rel-19 | 36.300 | IoT_NTN_Ph3-Core | 1425 | 1 | B |
| R2-2506497 | Introduction of IoT NTN TDD mode | Iridium Satellite LLC | Rel-19 | 36.300 | IoT_NTN_TDD-Core | 1426 | 1 | B |
| R2-2506498 | Introduction of AI/ML for NR Air interface feature in TS 38.300 | vivo | Rel-19 | 38.300 | NR_AIML_air-Core | 1006 | 1 | B |
| R2-2506506 | Clarification on emergency call back and UE RRC state | R3 (Ericsson, Nokia, Huawei, ZTE, CATT, Samsung, Qualcomm Inc, NTT DOCOMO) | Rel-17 | 38.300 | NR_redcap-Core | 1029 |  | F |
| R2-2506507 | Clarification on emergency call back and UE RRC state | R3 (Ericsson, Nokia, Huawei, ZTE, CATT, Samsung, Qualcomm Inc, NTT DOCOMO) | Rel-18 | 38.300 | NR_redcap-Core | 1030 |  | A |
| R2-2506508 | Clarification on BL CE UE in RRC_INACTIVE | R3 (Ericsson, Nokia, Huawei, ZTE, CATT, Samsung, Qualcomm Inc, NTT DOCOMO) | Rel-17 | 36.300 | NB_IOTenh4_LTE_eMTC6 | 1433 |  | F |
| R2-2506509 | Clarification on BL CE UE in RRC_INACTIVE | R3 (Ericsson, Nokia, Huawei, ZTE, CATT, Samsung, Qualcomm Inc, NTT DOCOMO) | Rel-18 | 36.300 | NB_IOTenh4_LTE_eMTC6 | 1434 |  | A |
| R2-2506510 | Support Aerial UE Flight Information Reporting | R3 (CATT, ZTE, Ericsson, CMCC, Nokia, China Telecom, Huawei, LG Electronics) | Rel-19 | 38.300 | UAS_Ph3 | 1031 |  | B |
| R2-2506511 | Recovery of N3mb path failure for unicast transport of multicast session | R3 (ZTE Corporation, Nokia, Huawei, CATT) | Rel-19 | 38.300 | 5MBS_Ph2, TEI19 | 1032 |  | F |
| R2-2506512 | E-CID measurement enhancement [ECID_enh1] | R3 (Ericsson, ZTE) | Rel-19 | 38.305 | TEI19 | 0195 |  | B |
| R2-2506514 | Introduction of Secondary Cell Measurement Skipping for NR ATG | CMCC, Ericsson | Rel-19 | 38.331 | NR_ATG_enh-Core | 5477 | 1 | B |
| R2-2506515 | Introduction of Secondary Cell Measurement Skipping for NR ATG | CMCC, Ericsson | Rel-19 | 38.300 | NR_ATG_enh-Core | 1027 | 1 | B |
| R2-2506517 | Introduction of less than 5MHz in NTN | ZTE Corporation, Xiaomi, Sanechips | Rel-19 | 38.331 | NR_IoT_NTN_req_test_enh | 5389 | 2 | B |
| R2-2506522 | Introduction of NTN Phase 3 enhancements | Ericsson | Rel-19 | 38.331 | NR_NTN_Ph3-Core | 5481 |  | B |
| R2-2506523 | Introduction of IoT NTN TDD mode | Toyota ITC | Rel-19 | 36.321 | IoT_NTN_TDD-Core | 1592 | 1 | B |
| R2-2506529 | Introduction of extension ratio configuration for MPR reduction | Huawei, HiSilicon | Rel-19 | 38.331 | NR_ENDC_RF_Ph4-Core | 5482 |  | B |
| R2-2506533 | Introduction of low NR band carrier aggregation via switching | Apple, Telus, Nokia | Rel-19 | 38.331 | NR_LBCA_Sw | 5421 | 3 | B |
| R2-2506535 | Introduction of LTE TN to NR NTN IDLE mode mobility | CATT | Rel-19 | 36.331 | LTE_TN_NR_NTN_mob-Core | 5065 | 7 | B |
| R2-2506539 | Introduction of MIMO | Samsung | Rel-19 | 38.321 | NR_MIMO_Ph5-Core | 2100 | 1 | B |
| R2-2506542 | Introduction of IoT-NTN TDD mode | Xiaomi | Rel-19 | 36.304 | IoT_NTN_TDD-Core | 0883 | 1 | B |
| R2-2506546 | Introduction of Rel-19 MIMO Phase 5 | CMCC | Rel-19 | 38.300 | NR_MIMO_Ph5-Core | 1021 | 2 | B |
| R2-2506547 | Introduction of Rel-19 NR NTN | ZTE Corporation, Sanechips | Rel-19 | 38.304 | NR_NTN_Ph3-Core | 0441 | 1 | B |
| R2-2506549 | Introduction of LTE TN to NR NTN Mobility UE Capability | vivo | Rel-19 | 36.306 | LTE_TN_NR_NTN_mob-Core | 1918 | 1 | B |
| R2-2506553 | Introduction of LTE TN to NR NTN idle mode mobility | Samsung | Rel-19 | 36.300 | LTE_TN_NR_NTN_mob | 1412 | 7 | B |
| R2-2506554 | Introduction of capabilities for IoT NTN TDD | Samsung | Rel-19 | 36.306 | IoT_NTN_TDD-Core | 1914 | 1 | B |
| R2-2506556 | Introduction of IoT NTN Enhancements | Nokia, Nokia Shanghai Bell | Rel-19 | 36.304 | IoT_NTN_Ph3-Core | 0882 | 1 | B |
| R2-2506558 | Introduction of IoT NTN TDD mode | Huawei, HiSilicon | Rel-19 | 36.331 | IoT_NTN_TDD-Core | 5138 | 1 | B |
| R2-2506560 | Introduction of NTN Phase 3 enhancements | InterDigital | Rel-19 | 38.321 | NR_NTN_Ph3-Core | 2121 |  | B |
| R2-2506561 | Introduction of Rel-19 network energy saving to TS 38.304 | Apple (Rapporteur) | Rel-19 | 38.304 | Netw_Energy_NR_enh-Core | 0442 | 2 | B |
| R2-2506562 | Introduction of AI/ML for Air Interface to TS 38.321 | Apple (Rapporteur) | Rel-19 | 38.321 | NR_AIML_air-Core | 2104 | 2 | B |
| R2-2506564 | Introduction of R19 SONMDT features in TS 36.331 | Huawei, HiSilicon | Rel-19 | 36.331 | NR_ENDC_SON_MDT_Ph4-Core | 5150 | 2 | B |
| R2-2506566 | Introduction of NR sidelink multi-hop relay in TS 38.351 | OPPO | Rel-19 | 38.351 | NR_SL_relay_multihop-Core | 0041 | 2 | B |
| R2-2506567 | Introduction of RRC changes for mobility enhancements phase 4 | Ericsson, Huawei | Rel-19 | 38.331 | NR_Mob_Ph4-Core | 5443 | 2 | B |
| R2-2506568 | Introduction of NR sidelink multi-hop U2N Relay | LG Electronics (Rapporteur) | Rel-19 | 38.300 | NR_SL_relay_multihop-Core | 1028 | 2 | B |
| R2-2506571 | Introduction of network energy saving enhancements to TS 38.321 | InterDigital | Rel-19 | 38.321 | Netw_Energy_NR_enh-Core | 2110 | 2 | B |
| R2-2506572 | UE capability for Rel-19 IoT NTN | Qualcomm Inc. | Rel-19 | 36.306 | IoT_NTN_Ph3-Core | 1912 | 3 | B |
| R2-2506573 | Introduction of IoT NTN Ph3 | Huawei, HiSilicon | Rel-19 | 36.331 | IoT_NTN_Ph3-Core | 5137 | 2 | B |
| R2-2506579 | Correction to delivery of posSIB segments by dedicated signalling in RRC_CONNECTED | Huawei, HiSilicon, Ericsson, Qualcomm | Rel-18 | 38.331 | NR_pos-Core, TEI18 | 5409 | 4 | F |
| R2-2506580 | Introduction of UE assistance information for cell DTX/DRX [UAI_cellDTRX] | Huawei, HiSilicon | Rel-19 | 38.331 | TEI19 | 5474 | 2 | B |
| R2-2506582 | Introduction of UE assistance information for cell DTX/DRX [UAI_cellDTRX] | Huawei, HiSilicon | Rel-19 | 38.306 | TEI19 | 1352 | 1 | B |
| R2-2506583 | Introduction of LP-WUS/WUR in RRC | vivo (Rapporteur) | Rel-19 | 38.331 | NR_LPWUS-Core | 5416 | 2 | B |
| R2-2506584 | Introduction of AI/ML Positioning Accuracy Enhancements | Qualcomm Incorporated (Rapporteur) | Rel-19 | 37.355 | NR_AIML_air-Core | 0559 | 2 | B |
| R2-2506585 | Introduction of R19 XR enhancements for RRC spec | Huawei, HiSilicon | Rel-19 | 38.331 | NR_XR_Ph3-Core | 5395 | 2 | B |
| R2-2506586 | Introduction of AI for Air interface feature in TS 37.320 | Huawei, HiSilicon, Ericsson, Nokia | Rel-19 | 37.320 | NR_AIML_air-Core | 0143 | 3 | B |
| R2-2506587 | Introduction of MIMO Phase 5 | Ericsson | Rel-19 | 38.331 | NR_MIMO_Ph5-Core | 5441 | 2 | B |
| R2-2506589 | Introduction of 3Tx UL switching [TxSwitch_R19] | MediaTek Inc., Ericsson, T-Mobile USA | Rel-19 | 38.331 | TEI19 | 5411 | 2 | B |
| R2-2506592 | Introduction of NR NTN phase 3 | THALES (Rapporteur) | Rel-19 | 38.300 | NR_NTN_Ph3-Core | 1023 | 2 | B |
| R2-2506593 | Support of enhancements on AI/ML for NG-RAN | R3 (NEC, ZTE, Qualcomm, Samsung, CMCC, Ericsson, CATT, Ofinno, Huawei, Nokia, Jio Platforms) | Rel-19 | 38.300 | NR_AIML_NGRAN_enh-Core | 1033 |  | B |
| R2-2506595 | Addition of SON enhancements | R3 (CMCC, ZTE, Nokia, Ericsson) | Rel-19 | 37.340 | NR_ENDC_SON_MDT_Ph4-Core | 0421 |  | B |
| R2-2506596 | Addition of SON enhancements | R3 (China Unicom, Nokia, ZTE, Ericsson) | Rel-19 | 38.300 | NR_ENDC_SON_MDT_Ph4-Core | 1034 |  | B |
| R2-2506597 | Introduction of NR Femto Architecture and Protocol Aspects | R3 (Ericsson, Nokia, TMO US, AT&T, Verizon Wireless, BT, Charter, Huawei, LG Electronics, Samsung, Lenovo, Baicells, ZTE, NEC, CATT, Qualcomm) | Rel-19 | 38.300 | NR_WAB_5GFemto-Core | 1035 |  | B |
| R2-2506598 | Support of XR enhancements | R3 (ZTE Corporation, Nokia, Nokia Shanghai Bell, Qualcomm Inc, Lenovo, Huawei, Ericsson, CATT, CMCC, Samsung, Xiaomi, NEC, China Telecom, Ofinno) | Rel-19 | 37.340 | NR_XR_Ph3-Core | 0422 |  | B |
| R2-2506599 | AIML for NR-DC | R3 (Huawei, Lenovo, CATT, ZTE Corporation, Samsung, Ericsson, Nokia, Huawei, Ofinno, FiberCop, LG Electronics, Jio Platforms) | Rel-19 | 37.340 | NR_AIML_NGRAN_enh-Core | 0423 |  | B |
| R2-2506600 | Addition of SON enhancements | R3 (Lenovo, Ericsson) | Rel-19 | 36.300 | NR_ENDC_SON_MDT_Ph4-Core | 1435 |  | B |
| R2-2506601 | Support of Location Service Involving WAB-Nodes | R3 (ZTE Corporation, Nokia, Nokia Shanghai Bell, Ericsson, Qualcomm, Lenovo, CATT, Samsung, Huawei, China Telecom) | Rel-19 | 38.305 | NR_WAB_5GFemto-Core | 0196 |  | B |
| R2-2506602 | Support for Wireless Access Backhaul | R3 (Qualcomm, Ericsson, CATT, ZTE, Nokia, Nokia Shanghai Bell, NTTDOCOMO, Samsung, LG Electronics, Huawei, China Telecom, NTT DOCOMO, Airbus, NEC) | Rel-19 | 38.300 | NR_WAB_5GFemto-Core | 1036 |  | B |
| R2-2506604 | Introduction of Rel-19 Evolution of NR duplex operation (SBFD) | CATT | Rel-19 | 38.300 | NR_duplex_evo-Core | 1008 | 1 | B |
| R2-2506605 | Introduction of Rel-19 Evolution of NR duplex operation (SBFD) | Huawei, HiSilicon | Rel-19 | 38.331 | NR_duplex_evo-Core | 5414 | 2 | B |
| R2-2506606 | Introduction of Rel-19 Evolution of NR duplex operation (SBFD) for MAC spec | Samsung | Rel-19 | 38.321 | NR_duplex_evo-Core | 2106 | 2 | B |
| R2-2506607 | Introduction of LP-WUS in TS 37.340 | ZTE Corporation, Sanechips | Rel-19 | 37.340 | NR_LPWUS-Core | 0420 | 2 | B |
| R2-2506608 | Introduction of LP-WUS in TS 38.304 | CATT | Rel-19 | 38.304 | NR_LPWUS-Core | 0440 | 3 | B |
| R2-2506609 | Introduction of Low-Power Wake-Up Signal and Receiver for NR | Ericsson | Rel-19 | 38.300 | NR_LPWUS-Core | 1015 | 1 | B |
| R2-2506612 | Introduction of SONMDT features | Ericsson, ZTE | Rel-19 | 38.331 | NR_ENDC_SON_MDT_Ph4-Core | 5446 | 4 | B |
| R2-2506613 | Introduction of LP-WUS in TS 38.321 | Apple | Rel-19 | 38.321 | NR_LPWUS-Core | 2103 | 2 | B |
| R2-2506614 | Introduction of NR mobility enhancements Phase 4 in TS 38.300 | Apple Inc (Rapporteur) | Rel-19 | 38.300 | NR_Mob_Ph4-Core | 1011 | 4 | B |
| R2-2506615 | Introduction of Network Energy Savings Enhancements | Huawei, HiSilicon | Rel-19 | 38.300 | Netw_Energy_NR_enh-Core | 1013 | 2 | B |
| R2-2506616 | Introduction of NR mobility enhancements Phase 4 in TS 37.340 | China Telecom, CATT | Rel-19 | 37.340 | NR_Mob_Ph4-Core | 0419 | 2 | B |
| R2-2506617 | Introduction of AIML Positioning | CATT | Rel-19 | 38.305 | NR_AIML_air-Core | 0190 | 2 | B |
| R2-2506618 | MDT enhancement for NTN | R3 (CMCC, ZTE, Nokia, Ericsson) | Rel-19 | 37.320 | NR_ENDC_SON_MDT_Ph4-Core | 0144 | 1 | B |
| R2-2506619 | Introduction of XR enhancements Phase 3 | Qualcomm Incorporated | Rel-19 | 38.321 | NR_XR_Ph3-Core | 2102 | 3 | B |
| R2-2506620 | Correction on inter-RAT FR2 measurement capabilities | Nokia, Qualcomm Incorporated | Rel-17 | 36.331 | NR_MG_enh-Core | 5147 | 2 | F |
| R2-2506621 | Correction on inter-RAT FR2 measurement capabilities | Nokia, Qualcomm Incorporated | Rel-18 | 36.331 | NR_MG_enh-Core | 5148 | 2 | A |
| R2-2506622 | Correction on inter-RAT FR2 measurement capabilities | Nokia, Qualcomm Incorporated | Rel-17 | 36.306 | NR_MG_enh-Core | 1921 | 2 | F |
| R2-2506623 | Correction on inter-RAT FR2 measurement capabilities | Nokia, Qualcomm Incorporated | Rel-18 | 36.306 | NR_MG_enh-Core | 1922 | 2 | A |
| R2-2506624 | Introduction of ANR reporting of HSDN cells [ANR_HSDN] | Huawei, HiSilicon, CMCC, China Unicom, China Telecom, CATT, NTT DoCoMo, Samsung | Rel-19 | 38.331 | TEI19 | 5318 | 7 | B |
| R2-2506625 | Introduction of NR sidelink multi-hop relay | Huawei, HiSilicon, CATT | Rel-19 | 38.331 | NR_SL_relay_multihop-Core | 5429 | 3 | B |
| R2-2506626 | Introduction of NR Sidelink Multi-hop Relay | InterDigital (Rapporteur) | Rel-19 | 38.321 | NR_SL_relay_multihop-Core | 2101 | 3 | B |
| R2-2506627 | Introduction of Rel-19 UE capability, including [TN32HARQ], [Pos_SRSHop], [SRTrig_SSSGSwitch], [SRSCS_ULTxSwitch], [SimCSI_count], [SimCSI_countNES], [TxSwitch_R19] | Xiaomi | Rel-19 | 38.306 | NR_AIML_air, NR_Mob_Ph4, NR_MIMO_Ph5, NR_duplex_evo, Netw_Energy_NR_enh, NR_LPWUS, NR_XR_Ph3, NR_NTN_Ph3, NR_ENDC_RF_Ph4, NR_ATG_enh, NR_RRM_Ph5, NonCol_intraB_ENDC_NR_CA_Ph2, NR_SL_relay_multihop, NR_ENDC_SON_MDT_Ph4, TEI19 | 1321 | 2 | B |
| R2-2506628 | Introduction of Rel-19 UE capability, including [TN32HARQ], [Pos_SRSHop], [SRTrig_SSSGSwitch], [SRSCS_ULTxSwitch], [SimCSI_count], [SimCSI_countNES], [TxSwitch_R19] | Xiaomi | Rel-19 | 38.331 | NR_AIML_air, NR_Mob_Ph4, NR_MIMO_Ph5, NR_duplex_evo, Netw_Energy_NR_enh, NR_LPWUS, NR_XR_Ph3, NR_NTN_Ph3, NR_ENDC_RF_Ph4, NR_ATG_enh, NR_RRM_Ph5, NonCol_intraB_ENDC_NR_CA_Ph2, NR_SL_relay_multihop, NR_ENDC_SON_MDT_Ph4, TEI19 | 5403 | 2 | B |
| R2-2506629 | ProSe support in NPN [ProSe_NPN] | Nokia, ZTECorporation, Sanechips, LGE, Philips | Rel-19 | 38.300 | TEI19 | 0957 | 2 | B |
| R2-2506630 | Clarification on UTC time offset in IoT NTN | ZTE Corporation, Sanechips | Rel-18 | 36.331 | LTE_NBIOT_eMTC_NTN-Core, TEI18 | 5146 | 3 | F |
| R2-2506631 | Corrections on eventD1 and condEventD1/T1 | Samsung, Huawei, HiSilicon, Ericsson, Nokia, Xiaomi | Rel-17 | 38.331 | NR_NTN_solutions | 5365 | 5 | F |
| R2-2506632 | Corrections on eventD1/D2 and condEventD1/D2/T1 | Samsung, Huawei, HiSilicon, Ericsson, Nokia, Xiaomi | Rel-18 | 38.331 | NR_NTN_solutions, NR_NTN_enh-Core | 5366 | 5 | F |
| R2-2506633 | Correction on SP Positioning SRS frequency hopping | Huawei, HiSilicon, Ericsson, Ofinno | Rel-18 | 38.331 | NR_pos_enh2-Core | 5467 | 1 | F |
| R2-2506634 | Correction on field description of sl-CapabilityInformationSidelink for U2U Relay | OPPO | Rel-18 | 38.331 | NR_SL_relay_enh-Core | 5413 | 2 | F |
| R2-2506635 | Introduction of multi-hop U2N relay in TS 38.323 | Ericsson | Rel-19 | 38.323 | NR_SL_relay_multihop | 0150 | 2 | B |
| R2-2506636 | Clarification on RRC procedure delay for BWP switching | Samsung | Rel-18 | 38.331 | NR_newRAT-Core, TEI18 | 5436 | 2 | F |
| R2-2506637 | Correction on uplink power control for Type-1 CG-PUSCH [PL RS Type 1 CG] | Ofinno | Rel-18 | 38.331 | TEI18 | 5438 | 2 | F |
| R2-2506638 | Introduction of AIML for NR air interface | Ericsson | Rel-19 | 38.331 | NR_AIML_air-Core | 5437 | 3 | B |
| R2-2506639 | Corrections on simultaneous Rx-Tx capability for TDD-SDL band combination | Huawei, HiSilicon, Ericsson | Rel-15 | 38.306 | TEI15 | 1310 | 2 | F |
| R2-2506640 | Corrections on simultaneous Rx-Tx capability for TDD-SDL band combination | Huawei, HiSilicon, Ericsson | Rel-16 | 38.306 | TEI15 | 1311 | 2 | A |
| R2-2506641 | Corrections on simultaneous Rx-Tx capability for TDD-SDL band combination | Huawei, HiSilicon, Ericsson | Rel-17 | 38.306 | TEI15 | 1312 | 2 | A |
| R2-2506642 | Corrections on simultaneous Rx-Tx capability for TDD-SDL band combination | Huawei, HiSilicon, Ericsson | Rel-18 | 38.306 | TEI15 | 1313 | 2 | A |
| R2-2506643 | Clarifications on the applicability of independent gap UE capability | Qualcomm Incorporated | Rel-15 | 38.306 | TEI15 | 1346 | 1 | F |
| R2-2506644 | Clarifications on the applicability of independent gap UE capability | Qualcomm Incorporated | Rel-16 | 38.306 | TEI15 | 1347 | 1 | A |
| R2-2506645 | Clarifications on the applicability of independent gap UE capability | Qualcomm Incorporated | Rel-17 | 38.306 | TEI15 | 1331 | 2 | A |
| R2-2506646 | Clarifications on the applicability of independent gap UE capability | Qualcomm Incorporated | Rel-18 | 38.306 | TEI15 | 1332 | 2 | A |
| R2-2506647 | Introduction of 7MHz channel bandwidth [FR1_7MHz_BW] | T-Mobile USA, Ericsson | Rel-18 | 38.331 | TEI18 | 5308 | 3 | B |
| R2-2506648 | Introduction of 7MHz channel bandwidth [FR1_7MHz_BW] | T-Mobile USA, Ericsson | Rel-18 | 38.306 | TEI18 | 1258 | 3 | B |
| R2-2506649 | Correction on intraF-NeighMeasForSCellWithoutSSB capability | ZTE Corporation, Sanechips, Nokia, Huawei, HiSilicon | Rel-18 | 38.306 | NR_newRAT-Core | 1345 | 1 | A |
| R2-2506650 | Introduction of redirection from NR TN to NR NTN to 38.306 [NR_TN_NTN_redir] | Samsung | Rel-19 | 38.306 | TEI19 | 1348 | 1 | B |
| R2-2506651 | Introduction of IoT TN to NTN redirection to 36.306 [IoT_TN_NTN_redir] | Samsung, Ericsson, Google | Rel-19 | 36.306 | TEI19 | 1925 | 1 | B |
| R2-2506652 | Introduction of redirection from NR TN to NR NTN to 38.331 [NR_TN_NTN_redir] | Samsung | Rel-19 | 38.331 | TEI19 | 5469 | 2 | B |
| R2-2506653 | Introduction of IoT TN to NTN redirection [IoT_TN_NTN_redir] | Samsung, Ericsson, Google | Rel-19 | 36.331 | TEI19 | 5156 | 2 | B |
| R2-2506654 | Introduction of control parameters for on-demand posSIB request [OdPosSIB_Req] | Huawei, HiSilicon, Ericsson, Samsung | Rel-19 | 38.305 | TEI19 | 0191 | 2 | B |
| R2-2506655 | Introduction of control parameters for on-demand posSIB request [OdPosSIB_Req] | Huawei, HiSilicon, Ericsson, Samsung | Rel-19 | 38.331 | TEI19 | 5406 | 3 | B |
| R2-2506656 | Introduction of control parameters for on-demand posSIB request [OdPosSIB_Req] | Huawei, HiSilicon, Ericsson, Samsung | Rel-19 | 38.306 | TEI19 | 1323 | 3 | B |
| R2-2506657 | Introduction of control parameters for on-demand posSIB request [OdPosSIB_Req] | Huawei, HiSilicon, Ericsson, Samsung | Rel-19 | 38.300 | TEI19 | 1009 | 3 | B |

217 Agreed CRs.

# Annex F:	List of email discussions during the meeting

Discussions during the RAN2#130 meeting:

List of AT meeting email discussions

Unofficial offline - LPP-21 Implicit TRP Location Coordinates

Unofficial offline-[MIMO] Stage-2 CR

[AT131][002][AI PHY] LCM BM OI (Ericsson)

[AT131][003][AI PHY] Functionality activation (Apple)

[AT131][004][AI PHY] NW sided OIs (InterDigital)

[AT131][006][UE caps] Per band

[AT131][007][ASN.1] Cross WI interactions (Ericsson)

[AT131][008][UP] F field in MAC (NTT Docomo)

[AT131][011][TEI19] Early CSI  (Huawei)

[AT131][012][TEI19] UAV CRs (CATT)

[AT131][013][TEI19] Paging capability (Vivo)

[AT131][014][R17 UP] DCP stage 2 CR (Huawei)

[AT131][015][R18 NES] CRs (ZTE)

[AT131][016][MUSIM] Offline (ZTE)

[AT131][017][R18 XR] PDU set QoS parameter CR (Huawei)

[AT131][018][R18 Others] MC CR (Ofinno)

[AT131][019][TEI19] NES (Huawei)

[AT131][020][AIoT] Reply to SA3 (Huawei)

[AT131][022][R18 UE caps] CR  (Xiaomi)

[AT131][028][AI PHY] UE capabilities (Xiaomi)

[AT131][029][AI PHY] LS to RAN1 (Xiaomi)

[AT131][030][AI PHY] Multicell v.s Single cell associate ID (Samsung)

[AT131][031][TEI19] 32HARQ CR (Huawei)

[AT131][033][AI Mob] Conclusions for TR (OPPO)

[AT131][039][AI PHY] LS to RAN1 on TPs (Ericsson)

[AT131][040][R17 UP] DRX command handling (Samsung)

[AT131][041][R18] Event A4-based CHO (Qualcomm)

[AT131][101][NES] (Apple) Stage 3 corrections of Rel-19 NES

[AT131][102][MOB] (Ericsson)

[AT131][103][MOB] (Vivo)

[AT131][104][MOB] TP fo multi-TRP (Nokia)

[AT131][105][NES] (Xiaomi)

[AT131][106][NES] (Samsung)

[AT131][107][MOB](CATT)

[AT131][200] Organizational – Rel-18 MIMO, Rel-19 MIMO, LPWUS, SBFD, NR Others (RAN2 VC)

[AT131][201][NR_Others] CR for UE Signaling design for NR ATG enh (CMCC)

[AT131][202][NR_Others] CRs for Signallig for 7Mhz Ch BW (T-Mobile, USA, Ericsson)

[AT131][203][LPWUS] enabling_disabling LP-WUS (Huawei)

[AT131][204][LPWUS] Proposals for RRC-10 38304-6, RRC-15 38304-2, 38304-12 (CATT)

[AT131][205][NR_Others] Draft LS to RAN4 for non-collocated EN-DC NR-CA (KDDI)

[AT131][206][NR_Others] CRs for low NR band CA, discussion on capability (Apple)

[AT131][207][NR_Others] CRs for MC enh (Lenovo)

[AT131][208][SBFD] Proposals for Power ramping issue on the RO type fallback (OPPO)

[AT131][300] Organizational – NR-NTN and IoT-NTN session

[AT131][301][TEI19] NTN related TEI19 (Samsung)

[AT131][302][R19 IoT NTN] UL enhancements (Mediatek)

[AT131][303][R19 NR NTN] Two SMTC periodicities (Xiaomi)

[AT131][304][R19 NR NTN] UE assisted SMTC selection in connected (Ericsson)

[AT131][305][R19 IoT NTN] S&F Open Issue (Sateliot)

[AT131][401][POS] CRs on posSIB segments in dedicated signalling (Huawei)

[AT131][402][POS] CR on startSFN (ZTE)

[AT131][403][POS] SP-SRSp with frequency hopping (Huawei)

[AT131][404][Relay] SI reception for multi-path relay (LG)

[AT131][405][Relay] PDCP duplication for multi-path (Huawei)

[AT131][406][Relay] TP on reflective bearer mapping (ZTE)

[AT131][500][XR] Organizational – Session on R19 XR and LTE Broadcast (Session chair)

[AT131][501][XR] LS to RAN1 on measurement gap skipping (Huawei)

[AT131][502][XR] MAC CE for XR rate (LGE)

[AT131][503][XR] Cover remaining PDCP and RLC issues for DSR (Nokia)

[AT131][600] Organizational – Maintenance and SON/MDT (Ericsson)

[AT131][601][Maint] Spatial Relation Info list extension (ZTE)

[AT131][602][Maint] Correction on previousPCellId in RLF report (Huawei)

[AT131][603][Maint] Clarification on RRC procedure delay for BWP switching (Samsung)

[AT131][604][Maint] Correction on intraF-NeighMeasForSCellWithoutSSB (ZTE)

[AT131][605][Maint] Correction on SCGFailureInformation (Ericsson)

[AT131][606][Maint] Inter-RAT FR2 meas caps (Nokia)

[AT131][607][SONMDT] Area config without having location (Ericsson)

[AT131][608][SONMDT] Correction on the dlRSRPAboveThreshold in RA-report (Xiaomi)

68 at-meeting offline discussions.

# Annex G:	Post-meeting email discussions after RAN2#131

Guidelines for email discussions:

General guidelines for SHORT email discussions, to be concluded approved endorsed at current meeting.

- Aim to have the final version of the agreed documents provided by the rapporteur at or shortly after the deadline.

- Please provide comments on the first version of the document in good time before the deadline. This allows the rapporteur to make an update addressing all companies' comments and there still be time for a quick round of comments on the update.

- If you have provided comments in the discussion then please indicate to the rapporteur if you are ok with the update provided (preferably via reflector). This avoids the rapporteur having to wait before they can conclude that their update is acceptable to you.

- Rapporteurs, if not already available, please request your tdoc number from the Secretary when you initiate your email discussion and then provide the final version as soon as you are confident that it is agreeable. You do not need to wait for a reminder from chair, session chair or Secretary before sending the final version.

- To avoid any confusion, Secretary, chair, or session chair will send an email to confirm the final status of the document.

For emails discussion to the next meeting (long):

- Rapporteurs, feel free to set an intermediate deadline for companies to provide initial comments, so that the conclusions and proposals can be prepared and distributed before the final deadline.

- Participants, please respect any intermediate deadline indicated by the rapporteur, and preferably provide your feedback as soon as possible.

Deadlines

Sept. 5th 	1000 UTC	Deadline short email discussions (unless otherwise indicated)

Sept 26th	1000 UTC	Deadline long email discussions

Oct 3rd	1000 UTC	Submission Deadline RAN2#131bis

Weekends are inactive periods.

It is recommended to not send emails or update files on the server during inactive periods while It is not strictly prohibited. Rapporteurs may kick-off discussions during inactive period. However, no intermediate deadlines and no interactive discussion, no decision making may occur during the inactive period. It shall be possible for a delegate to stay away from reflector and 3GPP server during the inactive period, and still fully participate. Rapporteur announcements during the inactive period, if any, or other updates, can be taken into account after the inactive period.

Please request R2-131 TDoc numbers for the following email discussions from MCC if not already allocated. Approval / endorsement will be declared at or shortly after the deadline.

[POST131][002][Organizational] Post email discussions

Intended outcome: share email discussion list

[POST131][003][AI PHY] Functionality activation (Apple)

Intended outcome: LS to RAN4  and LS to RAN1

Deadline:  short

=> Approved in:

R2-2506519: LS to RAN1

R2-2506520: LS to RAN4

[POST131][005][UE caps] UE capability CRs (Xiaomi)

Intended outcome: Agree to Mega CRs, 38.331 and 38.306

Deadline:  CR capturing RAN1/4 feature list – Thursday, Sept. 4

CR merging all RAN2 endorsed CRs as well - Tuesday, Sept. 9

=> Agreed in:

- R2-2506610 (38.306)

- R2-2506611 (38.331)

[POST131][006][UE caps] Per band/BC  (Samsung)

Intended outcome: agree to LS

Deadline:  short

=> Approved in R2-2506565 (LS out)

[POST131][019][TEI19] NES UE Caps CR (Huawei)

Intended outcome: update and agree to CRs by email

Deadline:  short

=> Agreed in:

- R2-2506580 (38.331)

- R2-2506582 (38.306)

[POST131][021][AIoT] MAC spec (Huawei)

Intended outcome: agree to TR

Deadline:  Short

=> Agreed in R2-2506559 (v0.1.0)

[POST131][022][AIoT] Stage 2 CR (CMCC)

Intended outcome: agree to CR with RAN3 CR merged

Deadline:  Short or Tuesday

=> Agreed in R2-2506490 (38.300)

[POST131][023][AI PHY] 37.320 (Huawei)

Intended outcome: Agree to final CR

Deadline:  short

=> Agreed in R2-2506586 (37.320)

[POST131][024][AI PHY] 37.355 (Qualcomm)

Intended outcome: Agree to final CR and then trigger LS to RAN1 on questions related to LPP21

Deadline:  short for CR and 2 weeks after for LS

=> Agreed in R2-2506584 (37.355)

=> Approved in R2-2506658 (LSout)

[POST131][025][AI PHY] 38.300 (Vivo)

Intended outcome: Agree to final CR and merge RAN3 CR

Deadline:  Tuesday

=> Agreed in R2-2506498 (38.300)

[POST131][026][AI PHY] 38.305 (CATT)

Intended outcome: Agree to final CR and merge RAN3 CR

Deadline:  Tuesday

=> Agreed in R2-2506617 (38.305)

[POST131][027][AI PHY] 38.331 (Ericsson)

Intended outcome: Agree to final CR

Deadline:  short

=> Agreed in R2-2506530 (38.331)

[POST131][028][AI PHY] 38.321 (Apple)

Intended outcome: Agree to final CR

Deadline:  short

=> Agreed in R2-2506562 (38.321)

[POST131][032][AI Mob] TR (Oppo)

Intended outcome: Agree to final TR to be submitted to plenary

Deadline:  Short

=> Agreed in R2-2506659 (TR 38.744)

[POST131][036][AI PHY] Reply to SA5/RAN3 (Huawei)

Intended outcome: agree to LS

Deadline:  short

=> Approved in R2-2506543 (LS out)

[POST131][037][AI PHY] User Consent LS to SA3 (NTT Docomo)

Intended outcome: agree to LS and cc RAN3 ccSA5

Deadline:  short

=> Approved in R2-2506541 (LS out)

[POST131][038][AI PHY] LS to RAN1/RAN3 on nw-sided data collection (ZTE)

Intended outcome: agree to RAN1 and RAN3 LS capturing relevant RAN2 agreements

Deadline:  short

=> Approved R2-2506545 (LS out)

[POST131][042][TEI19] TXSwitch  (Mediatek)

Intended outcome: agree and endorse CRs

Deadline:  short and UE capability part Thursday

=> Agreed in:

- R2-2506589 (38.331, CR)

=> Endorsed in:

- R2-2506590 (38.331, draftCR)

- R2-2506591 (38.306, draftCR)

[POST131][043][AI PHY] UE capabilities (Xiaomi)

Intended outcome: align on remaining open issues and agreeable proposals

Deadline:  Sept. 4th

=> Endorsed in:

R2-2506524 (38.306)

- R2-2506525 (38.331)

[POST131][044][NR Others] Power domain enh (Huawei)

Intended outcome: Prepare CR, review CR and agree if agreeable by companies.

Deadline:  Short

=> Agreed in R2-2506529 (38.331)

[POST131][108][NES] (Huawei)

Scope: Update NES 38.300 CR (including this meeting agreements also) with RAN3 CR merged.

Intended outcome: 38.300 CR in R2-2506219 to be agreed.

Deadline: Tuesday

=> Agreed in R2-2506615 (38.300)

[POST131][109][NES] (Apple)

Scope: Update NES 38.304 CR (including this meeting agreements also).

Intended outcome: 38.304 CR in R2-2506220 to be agreed.

Deadline: Short email discussion

=> Agreed in R2-2506561 (38.304)

[POST131][110][NES] (Ericsson)

Scope: Update NES 38.331 CR (including this meeting agreements also).

Intended outcome: 38.331 CR in R2-2506221 to be agreed.

Deadline: Short email discussion

=> Agreed in R2-2506221 (38.331)

[POST131][111][NES] (InterDigital)

Scope: Update NES 38.321 CR (including this meeting agreements also).

Intended outcome: 38.321 CR in R2-2506222 to be agreed.

Deadline: Short email discussion

=> Agreed in R2-2506571 (38.321)

[POST131][112][NES] (ZTE)

Scope: Update NES UE capability CRs (including this meeting agreements also).

Intended outcome: 38.331 CR in R2-2506223 and 38.306 CR in R2-2506224 to be endorsed.

Deadline: Sept 6th

=> Endorsed in:

- R2-2506223 (38.331)

- R2-2506224 (38.306)

[POST131][113][MOB] (Apple)

Scope: Update MOB 38.300 CR (including this meeting agreements also) and merge endorsed RAN3 CR.

Intended outcome: 38.300 CR in R2-2506225 to be agreed.

Deadline: Short email discussion

=> Agreed in R2-2506614 (38.300)

[POST131][114][MOB] (China Telecom)

Scope: Update MOB 37.340 CR (including this meeting agreements also) and merge endorsed RAN3 CR.

Intended outcome: 37.340 CR in R2-2506226 to be agreed.

Deadline: Tuesday

=> Agreed in R2-2506616 (37.340)

[POST131][115][MOB] (CATT)

Scope: Update MOB UE capability CRs (including this meeting agreements also).

Intended outcome: 38.331 CR in R2-2506227 and 38.306 CR in R2-2506228 to be endorsed.

Deadline: Sept. 4th

=> Endorsed in:

- R2-2506227 (38.331)

- R2-2506228 (38.306)

[POST131][116][MOB] (Vivo)

Scope: Update MOB 38.321 CR (including this meeting agreements also).

Intended outcome: 38.321 CR in R2-2506229 to be agreed.

Deadline: Short email discussion

=> Agreed in R2-2506229 (38.321)

[POST131][117][MOB] (Ericsson)

Scope: Update MOB 38.331 CR (including this meeting agreements also).

Intended outcome: 38.331 CR in R2-2506230 to be agreed.

Deadline: Short email discussion

=> Agreed in R2-2506567 (38.331)

[Post131][209][LPWUS] LS on not supporting simultaneous LR and MR operation (Apple)

Intended outcome: Prepare and approved the LS based on the corresponding agreements

Deadline:  Short

=> Approved in R2-2506550 (LS out)

[Post131][210][LPWUS] CR for TS 38.304 (CATT)

Intended outcome: Agree the CR for TS 38.304

Deadline:  Short

=> Agreed in R2-2506608 (38.304)

[Post131][211][LPWUS] CR for TS 38.331 (vivo)

Intended outcome: Agree the CR for TS 38.331

Deadline:  Short

=> Agreed in R2-2506583 (38.331)

[Post131][212][LPWUS] CR for TS 37.340 (ZTE)

Intended outcome: Agree the CR for TS 37.340

Deadline:  Short

=> Agreed in R2-2506607 (37.340)

[Post131][213][LPWUS] CR for TS 38.321 (Apple)

Intended outcome: Agree the CR for TS 38.321

Deadline:  Short

=> Agreed in R2-2506613 (38.321)

[Post131][214][LPWUS] CR for TS 38.300 (Ericsson)

Intended outcome: Agree the CR for TS 38.300 and merge RAN3 CR

Deadline:  Tuesday

=> Agreed in R2-2506609 (38.300)

[Post131][215][LPWUS] Draft CR for UE capability (Huawei)

Intended outcome: Endorse the draft CR for TS 38.306

Deadline:  Sept. 4th

=> Not triggered since it was found there is no need for this discussion.

[Post131][216][MIMO_Ph5] CR for TS 38.300 (CMCC)

Intended outcome: Agree the CR for TS 38.300

Deadline:  Short

=> Agreed in R2-2506546 (38.300)

[Post131][217][MIMO_Ph5] CR for TS 38.321 (Samsung)

Intended outcome: Agree the CR for TS 38.321

Deadline:  Short

=> Agreed in R2-2506539 (38.321)

[Post131][218][MIMO_Ph5] CR for TS 38.331 (Ericsson)

Intended outcome: Agree the CR for TS 38.331

Deadline:  Short

=> Agreed in R2-2506587 (38.331)

[Post131][219][SBFD] CR for TS 38.300 (CATT)

Intended outcome: Agree the CR for TS 38.300 with RAN3 CR merged

Deadline:  Tuesday

=> Agreed in R2-2506604 (38.300)

[Post131][220][SBFD] CR for TS 38.331 (Huawei)

Intended outcome: Agree the CR for TS 38.321

Deadline:  Short

=> Agreed in R2-2506605 (38.331)

[Post131][221][SBFD] CR for TS 38.321 (Samsung)

Intended outcome: Agree the CR for TS 38.321

Deadline:  Short

=> Agreed in R2-2506606 (38.321)

[Post131][222]MIMO_Ph5] LS on two-TA configuration scenarios  (Samsung)

Intended outcome: Approve the LS

Deadline:  Short

=> Approved in R2-2506540 (LS out)

[Post131][223][SBFD] LS on simultaneous configuration of SBFD and DC (ZTE)

Intended outcome: Approve the LS

Deadline:  Short

=> Approved in R2-2506492 (LS out)

[Post131][224][NR_Others] CRs for NR ATG enh (CMCC)

Intended outcome: Agree the CRs for TS38.331 and TS38.300, endorse the CR for TS38.306

Deadline:  short and for 38.306 Sept. 4th

=> Agreed in:

- R2-2506514 (38.331)

- R2-2506515 (38.300)

=> Endorsed in R2-2506516 (38.306)

[Post131][226][NR_Others] CR for low NR band CA via switching (Apple)

Intended outcome: Agree the CR for TS38.331

Deadline: Short

=> Agreed in R2-2506533 (38.331)

[Post131][301][R19 NR NTN] Stage2 CR (Thales)

Scope: finalize the running Stage 2 CR and merge with RAN3 CR

Intended outcome: Agreed CR

Deadline: short

=> Agreed in R2-2506592 (38.300)

[Post131][302][R19 NR NTN] RRC CR (Ericsson)

Scope: finalize the running RRC CR

Intended outcome: Agreed CR

Deadline: short

=> Agreed in R2-2506522 (38.331)

[Post131][303][R19 NR NTN] 38.304 CR (ZTE)

Scope: finalize the running 38.304 CR

Intended outcome: Agreed CR

Deadline: short

=> Agreed in R2-2506547 (38.304)

[Post131][304][R19 NR NTN] capability CR (Apple)

Scope: finalize the running capability CR

Intended outcome: Endorsed CRs

Deadline: Sept. 4th

=> Endorsed in:

- R2-2506531 (38.331)

- R2-2506532 (38.306)

[Post131][305][R19 IoT NTN] Stage2 CR (Ericsson)

Scope: finalize the running Stage 2 CR and combined RAN3 CR

Intended outcome: Agreed CR

Deadline: Tuesday

=> Agreed in R2-2506494 (36.300)

[Post131][306][R19 IoT NTN] RRC CR (Huawei)

Scope: finalize the running RRC CR

Intended outcome: Agreed CR

Deadline: short

=> Agreed in R2-2506573 (36.331)

[Post131][307][R19 IoT NTN] MAC CR (Mediatek)

Scope: finalize the running MAC CR

Intended outcome: Agreed CR

Deadline: short

=> Agreed in R2-2506493 (36.321)

[Post131][308][R19 IoT NTN] 36.304 CR (Nokia)

Scope: finalize the running 36.304 CR

Intended outcome: Agreed CR

Deadline: short

=> Agreed in R2-2506556 (36.304)

[Post131][309][R19 IoT NTN] capability CR (Qualcomm)

Scope: finalize the running capability CR

Intended outcome: Agreed CR

Deadline: short

=> Agreed in R2-2506572 (36.306)

=> Endorsed in R2-2506538 (36.331)

[Post131][310][IoT NTN TDD] Stage2 CR (Iridium)

Scope: finalize the running Stage 2 CR

Intended outcome: Agreed CR

Deadline: short

=> Agreed in R2-2506497 (36.300)

[Post131][311][IoT NTN TDD] RRC CR (Huawei)

Scope: finalize the running RRC CR

Intended outcome: Agreed CR

Deadline: short

=> Agreed in R2-2506558 (36.331)

[Post131][312][IoT NTN TDD] MAC CR (Toyota)

Scope: finalize the running MAC CR

Intended outcome: Agreed CR

Deadline: short

=> Agreed in R2-2506523 (36.321)

[Post131][313][IoT NTN TDD] 36.304 CR (Xiaomi)

Scope: finalize the running 36.304 CR

Intended outcome: Agreed CR

Deadline: short

=> Agreed in R2-2506542 (36.304)

[Post131][314][IoT NTN TDD] capability CR (Samsung)

Scope: finalize the running capability CR

Intended outcome: Agreed CR

Deadline: short

=> Agreed in R2-2506554 (36.306)

[Post131][315][R19 NR NTN] MAC CR (Interdigital)

Scope: create the MAC CR

Intended outcome: Agreed CR

Deadline: short

=> Agreed in R2-2506560 (38.321)

[Post131][316][LTE NR NTN mob] Stage2 CR (Samsung)

Scope: finalize the running Stage 2 CR

Intended outcome: Agreed CR

Deadline: short

=> Agreed in R2-2506553 (36.300)

[Post131][317][LTE NR NTN mob] RRC CR (CATT)

Scope: finalize the running RRC CR

Intended outcome: Agreed CR

Deadline: short

=> Agreed in R2-2506535 (36.331)

[Post131][318][LTE NR NTN mob] capability CR (vivo)

Scope: finalize the running capability CR

Intended outcome: Agreed CR

Deadline: short

=> Agreed in R2-2506549 (36.306)

[Post131][319][R19 IoT NTN] LS on CB-Msg3-EDT (Nokia)

Scope: Draft the LS on additional agreements for CB-Msg3-EDT

Intended outcome: Approved LS (in R2-2506284)

Deadline: short

=> Approved in R2-2506284 (LS out)

[Post131][320][R19 IoT NTN] LS on CQI reporting (Samsung)

Scope: Draft the LS on CQI reporting for CB-Msg3-EDT

Intended outcome: Approved LS (in R2-2506299)

Deadline: short

=> Approved in R2-2506299 (LS out)

[Post131][321][IoT NTN TDD] LS to OCC support (Huawei)

Scope: Draft the LS to RAN1 on RAN2 agreement regarding OCC support for IoT NTN TDD

Intended outcome: Approved LS (in R2-2506300)

Deadline: short

=> Approved in R2-2506300 (LS out)

[Post131][407][Relay] LS to SA2 on cross-path topologies (Qualcomm)

Scope: Draft an LS to SA2 to notify them that RAN2 excludes cross-path topologies for L2 and that a UE should not attempt to function as an intermediate relay UE towards two different parent nodes.  Action is just to take into account (no reply expected).

Intended outcome: Approved LS

Deadline: Short (not for RP)

=> Approved in R2-2506569 (LS out)

[Post131][408][POS] LPP CR on periodic integrity assistance data (Huawei)

Scope: Revise the CR in R2-2506316 in accordance with the comments raised during the meeting and check the details.

Intended outcome: Agreed CR

Deadline: Short (for RP)

=> Agreed in R2-2506477 (37.355)

[Post131][409][POS] Control parameters for on-demand posSIB request (Huawei)

Scope: Revise the CR in R2-2505317 in accordance with the decision of RAN2#131 to reduce the reporting to “start” and “stop”, and confirm the accompanying CRs in R2-2505318 / R2-2505319 / R2-2505320

Intended outcome: Agreed CRs in R2-2506322 (38.331) and original tdoc numbers (other specs)

Deadline: Short (for RP)

=> Agreed in:

- R2-2506528 (38.305)

- R2-2506576 (38.331)

- R2-2506577 (38.306)

- R2-2506578 (38.300)

[Post131][411][Relay] Rel-19 relay 38.351 CR (OPPO)

Scope: Update the CR in R2-2505353 in accordance with decisions of RAN2#131.

Intended outcome: Agreed CR in R2-2506323

Deadline: Short (for RP)

=> Agreed in R2-2506566 (38.351)

[Post131][412][Relay] Rel-19 relay 38.321 CR (InterDigital)

Scope: Update the CR in R2-2505427 in accordance with decisions of RAN2#131.

Intended outcome: Agreed CR in R2-2506324

Deadline: Short (for RP)

=> Agreed in R2-2506603 (38.321)

[Post131][413][Relay] Rel-19 relay 38.300 CR (LG)

Scope: Update the CR in R2-2505621 in accordance with decisions of RAN2#131.

Intended outcome: Agreed CR in R2-2506325

Deadline: Short (for RP)

=> Agreed in R2-2506568 (38.300)

[Post131][414][Relay] Rel-19 relay 38.331 CR (Huawei)

Scope: Update the CR in R2-2505714 in accordance with decisions of RAN2#131.

Intended outcome: Agreed CR in R2-2506326

Deadline: Short (for RP)

=> Agreed in R2-2506574 (38.331)

[Post131][415][Relay] Rel-19 relay 38.323 CR (Ericsson)

Scope: Update the CR in R2-2505796 in accordance with decisions of RAN2#131.

Intended outcome: Agreed CR in R2-2506327

Deadline: Short (for RP)

=> Agreed in R2-2506327 (38.323)

[Post131][416][Relay] Rel-19 relay 38.304 CR (MediaTek)

Scope: Update the CR in R2-2506047 in accordance with decisions of RAN2#131.

Intended outcome: Agreed CR in R2-2506328

Deadline: Short (for RP)

=> Agreed in R2-2506328 (38.304)

[Post131][417][Relay] Rel-19 relay capability CRs (Samsung)

Scope: Update the CR in R2-2505772 in accordance with decisions of RAN2#131 and develop a complementary CR to 38.331.

Intended outcome: Endorsed CRs in R2-2506329 (38.306) and R2-2506330 (38.331)

Deadline: Thursday, Sept. 4th

=> Endorsed in:

- R2-2506329 (38.306)

- R2-2506330 (38.331)

[Post131][418][POS] Rel-18 CR on multiple posSIB segments in dedicated signalling (Huawei)

Scope: Update the CR in R2-2503603 to a category F CR reflecting the guidance in the chair notes from RAN2#131.

Intended outcome: Agreed CR in R2-2506458

Deadline: Short (for RP)

=> Agreed in R2-2506579 (38.331)

[POST131][504][XR] Final 38.300 CR (Nokia)

Scope: Produce a final CR for R19 XR and merge with RAN3 CR

Intended outcome: CR for agreement in R2-2506335

Deadline:  Tuesday

=> Agreed in R2-2506335 (38.300)

[POST131][505][XR] Final 38.331 CR (Huawei)

Scope: Produce a final CR for R19 XR

Intended outcome: CR for agreement in R2-2506336

Deadline:  Short

=> Agreed in R2-2506585 (38.331)

[POST131][506][XR] Final 38.323 CR (LGE)

Scope: Produce a final CR for R19 XR

Intended outcome: CR for agreement in R2-2506337

Deadline:  Short

=> Agreed in R2-2506337 (38.323)

[POST131][507][XR] Final 38.322 CR (vivo)

Scope: Produce a final CR for R19 XR

Intended outcome: CR for agreement in R2-2506338

Deadline:  Short

=> Agreed in R2-2506338 (38.322)

[POST131][508][XR] Final 38.321 CR (Qualcomm)

Scope: Produce a final CR for R19 XR

Intended outcome: CR for agreement in R2-2506339

Deadline:  Short

=> Agreed in R2-2506349 (38.321), but then coversheet revised by MCC into R2-2506619

[POST131][509][XR] Final UE capability CRs (Xiaomi)

Scope: Produce final draft CRs for R19 XR UE capabilities for merging into UE capabilities mega CR

Intended outcome: Draft CRs for endorsement in R2-2506340 (38.331) and R2-2506341 (38.306)

Deadline:  Sept. 4th

=> Endorsed in:

- R2-2506341 (38.306)

- R2-2505137 (38.331)

[POST131][510][TEI19] UE capability CR for 5GB_CASMuting (Huawei)

Scope: Discuss whether we should add a capability dependency

Intended outcome: Decision and revised draft CR for agreement in R2-2506343, if needed

Deadline:  short

=> Agreed in R2-2506343 (36.306)

[POST131][511][LTE Broadcast] Final 36.331 CR (Qualcomm)

Scope: Produce a final CR for LTE Broadcast

Intended outcome: CR for agreement in R2-2506344

Deadline:  Short

=> Agreed in R2-2506344 (36.331)

[POST131][512][LTE Broadcast] Final 38.321 CR (Samsung)

Scope: Produce a final CR for LTE Broadcast

Intended outcome: CR for agreement in R2-2506345

Deadline:  Short

=> Agreed in R2-2506348 (36.321)

[POST131][513][LTE Broadcast] Final 38.306 CR (Huawei)

Scope: Produce a final CR for LTE Broadcast

Intended outcome: CR for agreement in R2-2506346

Deadline:  Short

=> Agreed in R2-2506346 (36.306)

[POST131][514][LTE Broadcast] Final 38.300 CR (ZTE)

Scope: Produce a final CR for LTE Broadcast

Intended outcome: CR for agreement in R2-2506347

Deadline:  Short

=> Agreed in R2-2506347 (36.300)

[Post131][601][SONMDT] SONMDT capability CRs (CATT)

Scope:

    - Produce agreeable CRs

Intended outcome:

    - Endorsed CR(s) in R2-2506377 and R2-2506378

Deadline:  Sept. 4th

=> Endorsed in:

- R2-2506377 (38.331)

- R2-2506378 (38.306)

[Post131][602][SONMDT] SONMDT NR RRC CR (Ericsson)

Scope:

    - Produce agreeable CRs

Intended outcome:

    - Agreed CR(s) in R2-2506376

Deadline:

    - Short

=> Agreed in R2-2506612 (38.331)

[Post131][603][SONMDT] SONMDT LTE RRC CR (Huawei)

Scope:

    - Produce agreeable CRs

Intended outcome:

    - Agreed CR(s) in R2-2506379

Deadline:

    - Short

=> Agreed in R2-2506564 (36.331)

[Post131][604][Maint] Clarifications on the applicability of independent gap UE capability (Qualcomm)

Scope:

    - Produce agreeable CRs

Intended outcome:

    - Agreed CR(s)

Deadline:

    - Short

=> Agreed in:

- R2-2506367: 38.306 (Rel-15)

- R2-2506368: 38.306 (Rel-16)

- R2-2506369: 38.306 (Rel-17)

- R2-2506499: 38.306 (Rel-18)

[Post131][999][ Review of RAN3 endorsed CRs (MCC)

Scope: Review and agree RAN3 endorsed stage-2 CRs

Intended outcome: set of agreed CRs

Deadline: two sets of CRs expected from RAN2.

The deadline for the first set:05/09

The deadline for the second set: 10/09

=> Agreed in (1st set):

- R2-2506506 (38.300)

- R2-2506507 (38.300)

- R2-2506508 (36.300)

- R2-2506509 (36.300)

- R2-2506510 (38.300)

- R2-2506511 (38.300)

- R2-2506512 (38.305)

=> Agreed in (2nd set):

- R2-2506593 (38.300)

- R2-2506618 (37.340) (rev of R2-2506594)

- R2-2506595 (37.340)

- R2-2506596 (38.300)

- R2-2506597 (38.300)

- R2-2506598 (37.340)

- R2-2506599 (37.340)

- R2-2506600 (36.300)

- R2-2506601 (38.305)

- R2-2506602 (38.300)

95 short email discussions.

# Long email discussions, for R2-131bis, Sept 26th 10:00 UTC (unless otherwise stated)

Please request R2-131 TDoc numbers for the following email discussions by 3GU according to normal tdoc submission procedure.

POST131][009][TEI19]  LTM Cell Switch (Ericsson)

Intended outcome: Review updated CRs after plenary

Deadline:  Long

[Post131][225][NR_Others] On Rx BSF optimization (CATT)

Intended outcome: Discussion summary on the UAI issue, and endorse the CR for TS38.331

Deadline:  Long

[Post131][410][POS] Stage 2 descriptions for Rel-18 positioning (CATT)

Scope: Determine how to structure descriptions of the Rel-18 positioning subfeatures in 38.305 and work towards an agreeable CR, starting from the proposals in R2-2505124 / R2-2505849.

Intended outcome: Agreeable CR if possible, and report to next meeting

Deadline: Long

[POST131][019][TEI19] NES UE Caps CR (Huawei)

Intended outcome: identify open issues & suggest resolutions

Deadline:

- Initial list of open issues by rapporteur, proposed resolutions for easy open issues or resolution options for other issues: sept. 19th

- Inputs from other companies and identification of issues that require contribution input: Sept. 26th

- Final set of proposals and resolutions for issues that don’t require contribution input: Oct. 1st

[POST131][021][AIoT] MAC spec (Huawei)

Intended outcome: identify open issues & suggest resolutions

Deadline:

- Initial list of open issues by rapporteur, proposed resolutions for easy open issues or resolution options for other issues: sept. 19th

- Inputs from other companies and identification of issues that require contribution input: Sept. 26th

- Final set of proposals and resolutions for issues that don’t require contribution input: Oct. 1st

[POST131][022][AIoT] Stage 2 CR (CMCC)

Intended outcome: identify open issues & suggest resolutions

Deadline:

- Initial list of open issues by rapporteur, proposed resolutions for easy open issues or resolution options for other issues: sept. 19th

- Input from other companies and final set of proposals and resolutions for identified issues that don’t require contribution input: Oct. 1st

NOTE: no contributions from other companies expected

[POST131][023][AI PHY] 37.320 (Huawei)

Intended outcome: identify open issues & suggest resolutions

Deadline:

- Initial list of open issues by rapporteur, proposed resolutions for easy open issues or resolution options for other issues: sept. 19th

- Input from other companies and final set of proposals and resolutions for identified issues that don’t require contribution input: Oct. 1st

NOTE: no contributions from other companies expected

[POST131][025][AI PHY] 38.300 (Vivo)

Intended outcome: identify open issues & suggest resolutions

Deadline:

- Initial list of open issues by rapporteur, proposed resolutions for easy open issues or resolution options for other issues: sept. 19th

- Input from other companies and final set of proposals and resolutions for identified issues that don’t require contribution input: Oct. 1st

NOTE: no contributions from other companies expected

[POST131][026][AI PHY] 38.305 (CATT)

Intended outcome: identify open issues & suggest resolutions

Deadline:

- Initial list of open issues by rapporteur, proposed resolutions for easy open issues or resolution options for other issues: sept. 19th

- Input from other companies and final set of proposals and resolutions for identified issues that don’t require contribution input: Oct. 1st

NOTE: no contributions from other companies expected

[POST131][028][AI PHY] 38.321 (Apple)

Intended outcome: identify open issues & suggest resolutions

Deadline:

- Initial list of open issues by rapporteur, proposed resolutions for easy open issues or resolution options for other issues: sept. 19th

- Inputs from other companies and identification of issues that require contribution input: Sept. 26th

- Final set of proposals and resolutions for issues that don’t require contribution input: Oct. 1st

[POST131][043][AI PHY] UE capabilities (Xiaomi)

Intended outcome: identify open issues & suggest resolutions

Deadline:

- Initial list of open issues by rapporteur, proposed resolutions for easy open issues or resolution options for other issues: sept. 19th

- Input from other companies and final set of proposals and resolutions for identified issues that don’t require contribution input: Oct. 1st

NOTE: no contributions from other companies expected

[POST131][108][NES] (Huawei)

Scope: Update NES 38.300 CR (including this meeting agreements also) with RAN3 CR merged.

Intended outcome: identify open issues & suggest resolutions

Deadline:

- Initial list of open issues by rapporteur, proposed resolutions for easy open issues or resolution options for other issues: sept. 19th

- Input from other companies and final set of proposals and resolutions for identified issues that don’t require contribution input: Oct. 1st

NOTE: no contributions from other companies expected

[POST131][109][NES] (Apple)

Scope: Update NES 38.304 CR (including this meeting agreements also).

Intended outcome: identify open issues & suggest resolutions

Deadline:

- Initial list of open issues by rapporteur, proposed resolutions for easy open issues or resolution options for other issues: sept. 19th

- Inputs from other companies and identification of issues that require contribution input: Sept. 26th

- Final set of proposals and resolutions for issues that don’t require contribution input: Oct. 1st

[POST131][111][NES] (InterDigital)

Scope: Update NES 38.321 CR (including this meeting agreements also).

Intended outcome: identify open issues & suggest resolutions

Deadline:

- Initial list of open issues by rapporteur, proposed resolutions for easy open issues or resolution options for other issues: sept. 19th

- Inputs from other companies and identification of issues that require contribution input: Sept. 26th

- Final set of proposals and resolutions for issues that don’t require contribution input: Oct. 1st

[POST131][112][NES] (ZTE)

Scope: Update NES UE capability CRs (including this meeting agreements also).

Intended outcome: identify open issues & suggest resolutions

Deadline:

- Initial list of open issues by rapporteur, proposed resolutions for easy open issues or resolution options for other issues: sept. 19th

- Input from other companies and final set of proposals and resolutions for identified issues that don’t require contribution input: Oct. 1st

NOTE: no contributions from other companies expected

[POST131][113][MOB] (Apple)

Scope: Update MOB 38.300 CR (including this meeting agreements also) and merge endorsed RAN3 CR

Intended outcome: identify open issues & suggest resolutions

Deadline:

- Initial list of open issues by rapporteur, proposed resolutions for easy open issues or resolution options for other issues: sept. 19th

- Input from other companies and final set of proposals and resolutions for identified issues that don’t require contribution input: Oct. 1st

NOTE: no contributions from other companies expected

[POST131][114][MOB] (China Telecom)

Scope: Update MOB 37.340 CR (including this meeting agreements also) and merge endorsed RAN3 CR.

Intended outcome: identify open issues & suggest resolutions

Deadline:

- Initial list of open issues by rapporteur, proposed resolutions for easy open issues or resolution options for other issues: sept. 19th

- Input from other companies and final set of proposals and resolutions for identified issues that don’t require contribution input: Oct. 1st

NOTE: no contributions from other companies expected

[POST131][115][MOB] (CATT)

Scope: Update MOB UE capability CRs (including this meeting agreements also).

Intended outcome: identify open issues & suggest resolutions

Deadline:

- Initial list of open issues by rapporteur, proposed resolutions for easy open issues or resolution options for other issues: sept. 19th

- Input from other companies and final set of proposals and resolutions for identified issues that don’t require contribution input: Oct. 1st

NOTE: no contributions from other companies expected

[Post131][210][LPWUS] CR for TS 38.304 (CATT)

Intended outcome: identify open issues & suggest resolutions

Deadline:

- Initial list of open issues by rapporteur, proposed resolutions for easy open issues or resolution options for other issues: sept. 19th

- Inputs from other companies and identification of issues that require contribution input: Sept. 26th

- Final set of proposals and resolutions for issues that don’t require contribution input: Oct. 1st

[Post131][212][LPWUS] CR for TS 37.340 (ZTE)

Intended outcome: identify open issues & suggest resolutions

Deadline:

- Initial list of open issues by rapporteur, proposed resolutions for easy open issues or resolution options for other issues: sept. 19th

- Input from other companies and final set of proposals and resolutions for identified issues that don’t require contribution input: Oct. 1st

NOTE: no contributions from other companies expected

[Post131][213][LPWUS] CR for TS 38.321 (Apple)

Intended outcome: identify open issues & suggest resolutions

Deadline:

- Initial list of open issues by rapporteur, proposed resolutions for easy open issues or resolution options for other issues: sept. 19th

- Inputs from other companies and identification of issues that require contribution input: Sept. 26th

- Final set of proposals and resolutions for issues that don’t require contribution input: Oct. 1st

[Post131][214][LPWUS] CR for TS 38.300 (Ericsson)

Intended outcome: identify open issues & suggest resolutions

Deadline:

- Initial list of open issues by rapporteur, proposed resolutions for easy open issues or resolution options for other issues: sept. 19th

- Input from other companies and final set of proposals and resolutions for identified issues that don’t require contribution input: Oct. 1st

NOTE: no contributions from other companies expected

[Post131][215][LPWUS] Draft CR for UE capability (Huawei)

Intended outcome: identify open issues & suggest resolutions

Deadline:

1.	Initial list of open issues by rapporteur, proposed resolutions for easy open issues or resolution options for other issues: sept. 19th

2.	Input from other companies and final set of proposals and resolutions for identified issues that don’t require contribution input: Oct. 1st

NOTE: no contributions from other companies expected

[Post131][216][MIMO_Ph5] CR for TS 38.300 (CMCC)

Intended outcome: identify open issues & suggest resolutions

Deadline:

- Initial list of open issues by rapporteur, proposed resolutions for easy open issues or resolution options for other issues: sept. 19th

- Input from other companies and final set of proposals and resolutions for identified issues that don’t require contribution input: Oct. 1st

NOTE: no contributions from other companies expected

[Post131][217][MIMO_Ph5] CR for TS 38.321 (Samsung)

Intended outcome: identify open issues & suggest resolutions

Deadline:

- Initial list of open issues by rapporteur, proposed resolutions for easy open issues or resolution options for other issues: sept. 19th

- Inputs from other companies and identification of issues that require contribution input: Sept. 26th

- Final set of proposals and resolutions for issues that don’t require contribution input: Oct. 1st

[Post131][219][SBFD] CR for TS 38.300 (CATT)

Intended outcome: identify open issues & suggest resolutions

Deadline:

- Initial list of open issues by rapporteur, proposed resolutions for easy open issues or resolution options for other issues: sept. 19th

- Input from other companies and final set of proposals and resolutions for identified issues that don’t require contribution input: Oct. 1st

NOTE: no contributions from other companies expected

[Post131][221][SBFD] CR for TS 38.321 (Samsung)

Intended outcome: identify open issues & suggest resolutions

Deadline:

- Initial list of open issues by rapporteur, proposed resolutions for easy open issues or resolution options for other issues: sept. 19th

- Inputs from other companies and identification of issues that require contribution input: Sept. 26th

- Final set of proposals and resolutions for issues that don’t require contribution input: Oct. 1st

[Post131][301][R19 NR NTN] Stage2 CR (Thales)

Scope: finalize the running Stage 2 CR and merge with RAN3 CR

Intended outcome: identify open issues & suggest resolutions

Deadline:

- Initial list of open issues by rapporteur, proposed resolutions for easy open issues or resolution options for other issues: sept. 19th

- Input from other companies and final set of proposals and resolutions for identified issues that don’t require contribution input: Oct. 1st

NOTE: no contributions from other companies expected

[Post131][303][R19 NR NTN] 38.304 CR (ZTE)

Scope: finalize the running 38.304 CR

Intended outcome: identify open issues & suggest resolutions

Deadline:

- Initial list of open issues by rapporteur, proposed resolutions for easy open issues or resolution options for other issues: sept. 19th

- Input from other companies and final set of proposals and resolutions for identified issues that don’t require contribution input: Oct. 1st

NOTE: no contributions from other companies expected

[Post131][304][R19 NR NTN] capability CR (Apple)

Scope: finalize the running capability CR

Intended outcome: identify open issues & suggest resolutions

Deadline:

- Initial list of open issues by rapporteur, proposed resolutions for easy open issues or resolution options for other issues: sept. 19th

- Input from other companies and final set of proposals and resolutions for identified issues that don’t require contribution input: Oct. 1st

NOTE: no contributions from other companies expected

[Post131][305][R19 IoT NTN] Stage2 CR (Ericsson)

Scope: finalize the running Stage 2 CR and combined RAN3 CR

Intended outcome: identify open issues & suggest resolutions

Deadline:

- Initial list of open issues by rapporteur, proposed resolutions for easy open issues or resolution options for other issues: sept. 19th

- Input from other companies and final set of proposals and resolutions for identified issues that don’t require contribution input: Oct. 1st

NOTE: no contributions from other companies expected

[Post131][307][R19 IoT NTN] MAC CR (Mediatek)

Scope: finalize the running MAC CR

Intended outcome: identify open issues & suggest resolutions

Deadline:

- Initial list of open issues by rapporteur, proposed resolutions for easy open issues or resolution options for other issues: sept. 19th

- Inputs from other companies and identification of issues that require contribution input: Sept. 26th

- Final set of proposals and resolutions for issues that don’t require contribution input: Oct. 1st

[Post131][308][R19 IoT NTN] 36.304 CR (Nokia)

Scope: finalize the running 36.304 CR

Intended outcome: identify open issues & suggest resolutions

Deadline:

- Initial list of open issues by rapporteur, proposed resolutions for easy open issues or resolution options for other issues: sept. 19th

- Inputs from other companies and identification of issues that require contribution input: Sept. 26th

- Final set of proposals and resolutions for issues that don’t require contribution input: Oct. 1st

[Post131][309][R19 IoT NTN] capability CR (Qualcomm)

Scope: finalize the running capability CR

Intended outcome: identify open issues & suggest resolutions

Deadline:

- Initial list of open issues by rapporteur, proposed resolutions for easy open issues or resolution options for other issues: sept. 19th

- Input from other companies and final set of proposals and resolutions for identified issues that don’t require contribution input: Oct. 1st

NOTE: no contributions from other companies expected

[Post131][310][IoT NTN TDD] Stage2 CR (Iridium)

Scope: finalize the running Stage 2 CR

Intended outcome: identify open issues & suggest resolutions

Deadline:

- Initial list of open issues by rapporteur, proposed resolutions for easy open issues or resolution options for other issues: sept. 19th

- Input from other companies and final set of proposals and resolutions for identified issues that don’t require contribution input: Oct. 1st

NOTE: no contributions from other companies expected

[Post131][312][IoT NTN TDD] MAC CR (Toyota)

Scope: finalize the running MAC CR

Intended outcome: identify open issues & suggest resolutions

Deadline:

- Initial list of open issues by rapporteur, proposed resolutions for easy open issues or resolution options for other issues: sept. 19th

- Inputs from other companies and identification of issues that require contribution input: Sept. 26th

- Final set of proposals and resolutions for issues that don’t require contribution input: Oct. 1st

[Post131][313][IoT NTN TDD] 36.304 CR (Xiaomi)

Scope: finalize the running 36.304 CR

Intended outcome: identify open issues & suggest resolutions

Deadline:

- Initial list of open issues by rapporteur, proposed resolutions for easy open issues or resolution options for other issues: sept. 19th

- Inputs from other companies and identification of issues that require contribution input: Sept. 26th

- Final set of proposals and resolutions for issues that don’t require contribution input: Oct. 1st

[Post131][314][IoT NTN TDD] capability CR (Samsung)

Scope: finalize the running capability CR

Intended outcome: identify open issues & suggest resolutions

Deadline:

- Initial list of open issues by rapporteur, proposed resolutions for easy open issues or resolution options for other issues: sept. 19th

- Input from other companies and final set of proposals and resolutions for identified issues that don’t require contribution input: Oct. 1st

NOTE: no contributions from other companies expected

[Post131][315][R19 NR NTN] MAC CR (Interdigital)

Scope: create the MAC CR

Intended outcome: identify open issues & suggest resolutions

Deadline:

- Initial list of open issues by rapporteur, proposed resolutions for easy open issues or resolution options for other issues: sept. 19th

- Inputs from other companies and identification of issues that require contribution input: Sept. 26th

- Final set of proposals and resolutions for issues that don’t require contribution input: Oct. 1st

[Post131][316][LTE NR NTN mob] Stage2 CR (Samsung)

Scope: finalize the running Stage 2 CR

Intended outcome: identify open issues & suggest resolutions

Deadline:

- Initial list of open issues by rapporteur, proposed resolutions for easy open issues or resolution options for other issues: sept. 19th

- Input from other companies and final set of proposals and resolutions for identified issues that don’t require contribution input: Oct. 1st

NOTE: no contributions from other companies expected

[Post131][318][LTE NR NTN mob] capability CR (vivo)

Scope: finalize the running capability CR

Intended outcome: identify open issues & suggest resolutions

Deadline:

- Initial list of open issues by rapporteur, proposed resolutions for easy open issues or resolution options for other issues: sept. 19th

- Input from other companies and final set of proposals and resolutions for identified issues that don’t require contribution input: Oct. 1st

NOTE: no contributions from other companies expected

[Post131][411][Relay] Rel-19 relay 38.351 CR (OPPO)

Scope: Update the CR in R2-2505353 in accordance with decisions of RAN2#131.

Intended outcome: identify open issues & suggest resolutions

Deadline:

- Initial list of open issues by rapporteur, proposed resolutions for easy open issues or resolution options for other issues: sept. 19th

- Input from other companies and final set of proposals and resolutions for identified issues that don’t require contribution input: Oct. 1st

NOTE: no contributions from other companies expected

[Post131][412][Relay] Rel-19 relay 38.321 CR (InterDigital)

Scope: Update the CR in R2-2505427 in accordance with decisions of RAN2#131.

Intended outcome: identify open issues & suggest resolutions

Deadline:

- Initial list of open issues by rapporteur, proposed resolutions for easy open issues or resolution options for other issues: sept. 19th

- Inputs from other companies and identification of issues that require contribution input: Sept. 26th

- Final set of proposals and resolutions for issues that don’t require contribution input: Oct. 1st

[Post131][413][Relay] Rel-19 relay 38.300 CR (LG)

Scope: Update the CR in R2-2505621 in accordance with decisions of RAN2#131.

Intended outcome: identify open issues & suggest resolutions

Deadline:

- Initial list of open issues by rapporteur, proposed resolutions for easy open issues or resolution options for other issues: sept. 19th

- Input from other companies and final set of proposals and resolutions for identified issues that don’t require contribution input: Oct. 1st

NOTE: no contributions from other companies expected

[Post131][415][Relay] Rel-19 relay 38.323 CR (Ericsson)

Scope: Update the CR in R2-2505796 in accordance with decisions of RAN2#131.

Intended outcome: identify open issues & suggest resolutions

Deadline:

- Initial list of open issues by rapporteur, proposed resolutions for easy open issues or resolution options for other issues: sept. 19th

- Inputs from other companies and identification of issues that require contribution input: Sept. 26th

- Final set of proposals and resolutions for issues that don’t require contribution input: Oct. 1st

[Post131][416][Relay] Rel-19 relay 38.304 CR (MediaTek)

Scope: Update the CR in R2-2506047 in accordance with decisions of RAN2#131.

Intended outcome: identify open issues & suggest resolutions

Deadline:

- Initial list of open issues by rapporteur, proposed resolutions for easy open issues or resolution options for other issues: sept. 19th

- Inputs from other companies and identification of issues that require contribution input: Sept. 26th

- Final set of proposals and resolutions for issues that don’t require contribution input: Oct. 1st

[Post131][417][Relay] Rel-19 relay capability CRs (Samsung)

Scope: Update the CR in R2-2505772 in accordance with decisions of RAN2#131 and develop a complementary CR to 38.331.

Intended outcome: identify open issues & suggest resolutions

Deadline:

- Initial list of open issues by rapporteur, proposed resolutions for easy open issues or resolution options for other issues: sept. 19th

- Input from other companies and final set of proposals and resolutions for identified issues that don’t require contribution input: Oct. 1st

NOTE: no contributions from other companies expected

[POST131][504][XR] Final 38.300 CR (Nokia)

Scope: Produce a final CR for R19 XR and merge with RAN3 CR

Intended outcome: identify open issues & suggest resolutions

Deadline:

- Initial list of open issues by rapporteur, proposed resolutions for easy open issues or resolution options for other issues: sept. 19th

- Input from other companies and final set of proposals and resolutions for identified issues that don’t require contribution input: Oct. 1st

NOTE: no contributions from other companies expected

[POST131][506][XR] Final 38.323 CR (LGE)

Scope: Produce a final CR for R19 XR

Intended outcome: identify open issues & suggest resolutions

Deadline:

- Initial list of open issues by rapporteur, proposed resolutions for easy open issues or resolution options for other issues: sept. 19th

- Inputs from other companies and identification of issues that require contribution input: Sept. 26th

- Final set of proposals and resolutions for issues that don’t require contribution input: Oct. 1st

[POST131][507][XR] Final 38.322 CR (vivo)

Scope: Produce a final CR for R19 XR

Intended outcome: identify open issues & suggest resolutions

Deadline:

- Initial list of open issues by rapporteur, proposed resolutions for easy open issues or resolution options for other issues: sept. 19th

- Inputs from other companies and identification of issues that require contribution input: Sept. 26th

- Final set of proposals and resolutions for issues that don’t require contribution input: Oct. 1st

[POST131][508][XR] Final 38.321 CR (Qualcomm)

Scope: Produce a final CR for R19 XR

Intended outcome: identify open issues & suggest resolutions

Deadline:

- Initial list of open issues by rapporteur, proposed resolutions for easy open issues or resolution options for other issues: sept. 19th

- Inputs from other companies and identification of issues that require contribution input: Sept. 26th

- Final set of proposals and resolutions for issues that don’t require contribution input: Oct. 1st

[POST131][509][XR] Final UE capability CRs (Xiaomi)

Scope: Produce final draft CRs for R19 XR UE capabilities for merging into UE capabilities mega CR

Intended outcome: identify open issues & suggest resolutions

Deadline:

- Initial list of open issues by rapporteur, proposed resolutions for easy open issues or resolution options for other issues: sept. 19th

- Input from other companies and final set of proposals and resolutions for identified issues that don’t require contribution input: Oct. 1st

NOTE: no contributions from other companies expected

[POST131][510][TEI19] UE capability CR for 5GB_CASMuting (Huawei)

Scope: Discuss whether we should add a capability dependency

Intended outcome: identify open issues & suggest resolutions

Deadline:

- Initial list of open issues by rapporteur, proposed resolutions for easy open issues or resolution options for other issues: sept. 19th

- Input from other companies and final set of proposals and resolutions for identified issues that don’t require contribution input: Oct. 1st

NOTE: no contributions from other companies expected

[POST131][512][LTE Broadcast] Final 38.321 CR (Samsung)

Scope: Produce a final CR for LTE Broadcast

Intended outcome: identify open issues & suggest resolutions

Deadline:

- Initial list of open issues by rapporteur, proposed resolutions for easy open issues or resolution options for other issues: sept. 19th

- Input from other companies and final set of proposals and resolutions for identified issues that don’t require contribution input: Oct. 1st

NOTE: no contributions from other companies expected

[POST131][513][LTE Broadcast] Final 38.306 CR (Huawei)

Scope: Produce a final CR for LTE Broadcast

Intended outcome: identify open issues & suggest resolutions

Deadline:

- Initial list of open issues by rapporteur, proposed resolutions for easy open issues or resolution options for other issues: sept. 19th

- Input from other companies and final set of proposals and resolutions for identified issues that don’t require contribution input: Oct. 1st

NOTE: no contributions from other companies expected

[POST131][514][LTE Broadcast] Final 38.300 CR (ZTE)

Scope: Produce a final CR for LTE Broadcast

Intended outcome: identify open issues & suggest resolutions

Deadline:

- Initial list of open issues by rapporteur, proposed resolutions for easy open issues or resolution options for other issues: sept. 19th

- Input from other companies and final set of proposals and resolutions for identified issues that don’t require contribution input: Oct. 1st

NOTE: no contributions from other companies expected

[Post131][601][SONMDT] SONMDT capability CRs (CATT)

Scope:

    - Produce agreeable CRs

Intended outcome:

    - identify open issues & suggest resolutions

Deadline:

- Initial list of open issues by rapporteur, proposed resolutions for easy open issues or resolution options for other issues: sept. 19th

- Input from other companies and final set of proposals and resolutions for identified issues that don’t require contribution input: Oct. 1st

NOTE: no contributions from other companies expected

57 long email discussions.

# Annex H:	History

| Document history |  |  |
| --- | --- | --- |
| Date | TSG RAN WG2 Tdoc | Subject |
| 18.09.2025 |  | First draft version |
| 11.10.2025 |  | Second draft version |
| 12.10.2025 | R2-2506702 | Submitted to RAN2#131bis for approval |
| Author: Dr. Juha Korhonen ETSI Mobile Competence Centre (MCC) Tel. +33 (0)4 92 94 42 00 email: Juha.Korhonen@etsi.org |  |  |