3GPP TSG-RAN WG2 Meeting #131bis                                                                      R2-2507229

Prague, Czech Republic, Oct 13th - 17th 2025

Agenda item:	10.3.3

Source: 	ZTE Corporation, Sanechips

Title: 	Views on common user plane and control plane

Document for:	Discussion and Decision

Introduction

During RAN#108 meeting, the RAN Working Group SID “study on 6G radio” was approved in [1]. Some objectives related to the common user plane and control aspects are listed as follows:

(3) Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] ...... (c) Access stratum security aspects, in alignment with requirements from SA3 [RAN2] ...... (e) Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3] ...... (8) AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4] (a) Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…) NOTE: lead WG depends on the use case. (b) AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including i. LCM procedures [RAN2, RAN1, RAN3, RAN4] ii. Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1]

In this paper, we will discuss the 6G requirements on access stratum security, data transfer of various type of data and AI/ML framework. The general principles and design considerations are analyzed and presented.

Discussion

2.1  Access stratum security

The MAC layer plays an important role in various functions such as RACH procedure, scheduling and dynamic activation/deactivation of various resources, etc. It can be foreseen that MAC layer will take an even more important role in 6G with the introduction of more lower-layer mechanisms to support more complex functionality, higher flexibility, and more extreme performance requirements in 6G.

However, without security protection for MAC layer signaling, the MAC layer functionality faces direct security risks such as risk to user privacy due to lack of confidentiality protection for the critical information contained in MAC signaling. A typical example is the TA information that is included in TA command sent from network to UE and Time advance report sent from UE to network may reveal UE’s distance to base station. Another risk comes from spoofing, i.e. forging MAC signaling sent to the UE, caused by lack of integrity protection for MAC signaling.

Therefore, introducing a security protection mechanism for 6G MAC signaling is critical to protect user and network from security attacks. According to 6G SID [1], one of the objectives for RAN2 is to define the radio interface architecture and procedure in alignment with the AS security requirements from SA3. SA3 has also approved SID on 6G security [3] and one of the work task is to study security and privacy aspects of 6G RAN architecture (including all UE-RAN interactions). Thus MAC layer security requirements and mechanisms should firstly be discussed in SA3.

Considering MAC layer security will incur additional overhead in MAC PDU size (specifically for integrity protection) and extra processing delay (for both integrity protection and encryption), it is reasonable to provide security protection for only some of the 6G MAC signaling. Note that not all information contained in MAC signaling is critical and the security risks caused by security attack to different MAC signaling may also differ.

In order to discuss and determine security protection requirements, especially to determine which kind of MAC signaling needs what kind of security protection, it is necessary to identify the security risks (including risk type and impact range) for each MAC signaling first. Given the diverse content and functionalities that MAC signaling may potentially offer, it would be difficult for SA3 to conduct such identification independently. RAN2 should hence assist in identifying the kind of information that is exchanged in MAC signalling and assist SA3 in designing the right security framework for this signalling.

In this contribution, we try to provide our consideration on the identification of security risk types and impact range based on a high-level categorization of likely MAC signaling in 6G.

Firstly, as 6G MAC signaling has not yet been defined at this stage, RAN2 is recommended to make a reasonable assumptions on a high-level categorization of 6G MAC signaling with reference to the types/functions of NR MAC CEs, and initiate the studies based on these high-level categories.

High level classification of 6G MAC signaling

DL MAC signaling:

- Link resource activation/deactivation (e.g. TCI states, CSI reporting etc)

- Activation/deactivation of radio resource (e.g. SCell activation/deactivation)

- Activation/deactivation of a specific feature (e.g. Duplication)

- Cell change signalling (HO/LTM etc)

- Synchronization related information (e.g. TA values)

- Measurement configuration signalling (e.g. Measurement gaps etc)

UL MAC signaling:

- Signalling for scheduling/resource allocation (SR/BSR/DSR etc)

- UE assistance information for specific configuration (Desired guard symbols etc)

- UL synchronization related information (e.g. TA report)

Depending on the content and function provided by a MAC signaling, the consequence of security risk may differ among MAC signaling categories. It is important to identify the exact risk for further discussion. Following list shows one example classification of security risk types.

Classification of Security Risks

- Risk to Privacy

- Link interruption/degradation

- Denial of service

- UL interference

- HO failure

The impact range is another dimension to evaluate the security risk. Depending on the nature of security risk, some risk types have impact to a single UE within single cell, while some risk types may have impact to multiple UEs or single UE spanning more than one cells.

Classification of Impact Range of Security Threats

- Impact to a single UE in single cell.

- Impact to a single UE spanning more than on cell.

- Impact to multiple UEs or cell/system level impact

Then, we can analysis the security risks of a specific category of MAC signaling, identify the risk types, and determine the impact range. For example, the forgery of signalling related to scheduling and resource allocation, such as forged SR/BSR/DSR, may block or delay the scheduling of UL buffer data.

Based on this analysis, we can make a mapping between MAC signaling categories and corresponding risk type and impact range, as illustrated in the Table 1 below. From this table, we can clearly identify which MAC signaling categories pose more severe security risks, and this can be used as a basis for deciding the security requirements for MAC signaling.

It is suggested that the discussion of MAC security in RAN2 be conducted based on the Table 1 with further optimization on the format and content. Then the results can be informed to SA3 to facilitate SA3’s discussion.

Table 1 Mapping MAC signaling categories and security risk and impact range

|  | Impact to a single UE in single cell |  |  | Impact to multiple users/ more than one cell |  |
| --- | --- | --- | --- | --- | --- |
|  | Risk to user Privacy | Link interruption/ degradation | Denial of service | UL interference | HO failure |
| DL MAC signaling |  |  |  |  |  |
| Link resource activation/deactivation (e.g. TCI states, CSI reporting etc) |  | √ | √ |  |  |
| Activation/deactivation of radio resource (e.g. SCell activation/deactivation) |  | √ | √ |  |  |
| Activation/deactivation of a specific feature (e.g. Duplication) |  | √ | √ |  |  |
| Cell change signalling (HO/LTM etc) | √ |  |  | √ | √ |
| Synchronization related information (e.g. TA values) | √ |  |  | √ |  |
| Measurement configuration signalling (e.g. Measurement gaps etc) |  | √ | √ | √ | √ |
| UL MAC signaling |  |  |  |  |  |
| Signalling for scheduling/resource allocation (SR/BSR/DSR etc) |  | √ | √ |  |  |
| UE assistance information for specific configuration (Desired guard symbols etc) |  | √ | √ |  |  |
| UL synchronization related information (e.g. TA report) | √ |  |  | √ | √ |

Observation 1   From RAN2 perspective, security protection for the low layer signalling should be studied.

The solution for the security protection for low layer command will also be discussed and determined in SA3. However, the type of information transmitted in low layer signalling should be discussed and determined in RAN2 first. The following two alternatives can be studied in RAN2:

- Option 1: MAC CE based solution (similar as 5G): The lower layer command is generated by MAC layer and be transmitted through MAC CE, in which case new security protection mechanism will be required

- Option 2: RRC based solution: The lower layer command will be generated by local RRC (RRC located in DU in case of CU/DU split), and be transmitted as RRC message through SRB, in which case the PDCP security protection for SRB can be reused.

Therefore, to facilitate the discussion in SA3 on the security mechanism design, RAN2 should discuss and determine the solution for the transmission of low layer command first and inform SA3 as soon as possible.

Observation 2 	The solution for the transmission of low layer command (e.g. legacy MAC CE based and/or DU terminated local RRC based) will impact the discussion in SA3 for the security protection of low layer command.

Proposal 1 	RAN2 should identify the type of lower layer signalling that is likely to be specified for 6G and convey this information to SA3 and RAN2 should study the format for the transmission of lower layer signalling (e.g. MAC CE based or Local RRC based), and inform SA3 the corresponding conclusion.

In legacy NR specification, the security keys used for CP and UP are separate algorithm keys, both derived from the same AS layer security key, i.e. KgNB. These algorithm keys are updated at the same time whenever their associated AS layer security key is updated: whenever a new AS layer security key is derived, both CP keys and UP keys are re-derived based on this new AS layer key.

In a RAN architecture with split CU-CP and CU-UP, PDCP entities for DRBs reside in the CU-UP. In some network deployments, a single CU-UP entity can be shared among multiple CU-CPs. With the legacy key derivation mechanism, even if the serving CU-UP of a UE remains unchanged during a handover, the UP key will still be updated whenever the AS key is updated. This leads to traffic interruption due to PDCP re-establishment and RLC/MAC reset that is performed during the handover procedure.

If the security key for CP and UP can be updated separately during handover, the following benefits can be achieved in the CU-CP/CU-UP slit architecture:

- Frequent UP key updates can be avoided

- No need to perform PDCP re-establishment

- RLC reset and MAC reset may be avoided; already received RLC segments need not to be discarded.

We believe this kind of mechanism will bring significant improvement to mobility performance. Although the final requirements and mechanism should be determined by SA3, RAN2 can study the necessity of introduction of such mechanism. The study can be focused on analyzing the impact on mobility performance. UP traffic interruption during handover should be the main evaluation metrics.

Observation 3  Some benefits can be observed to allow the change of CP key without the change of UP key (to minimize the requirement on key refresh and reduce the interruption time during mobility).

On the other hand, whether CU/DU split and CU-CP/CU-UP split are supported in 6G is still pending and decision will be made after June 2026. Thus, RAN2 can decide whether to start the study after RAN plenary makes the decision.

Proposal 2 	RAN2 should study the potential enhancement on the key management for mobility enhancements and inform SA3 of our preference. It is up to SA3 to determine whether such enhancement is acceptable from a security perspective or not.

2.2 Data collection and transfer

According to SA1 6G use cases and requirement [2], 6G system data denotes the data generated and controlled by the 6G system, e.g. AI/ML data, sensing data, etc. Different from the traditional user traffic data which is transmitted through the 3GPP system for user related services, 6G system data should be delivered between data provider and data consumer within the 6G system. Here the data provider and consumer may be UE, RAN, CN, etc. Moreover, the data collection and consumption, data exposure, data storage and retrieval, security and privacy protection should be considered for the 6G system data.

As a matter of fact, various data collection methods are available in 5G, e.g., MDT, L3 measurement and report, L1 measurement and report, UE assistance information, early measurement and report, LPP, etc. The comparison table of these existing data collection methods are captured in [4, TR38.843,Table 7.2.1.3-1]. The existence of multiple, distinct data collection methods create trouble for operators to manage data in a unified and consistent manner. It should be noted that the data collection for AI/ML data has been discussed in 5G. For the network-side data collection for network-side model training, it was agreed that UE may be configured by gNB to log L1 measurement and report them via control plane RRC message. For the UE-side data collection for UE-side model training, three options were mainly discussed in RAN2: 1) Option 1b, UE collects training data and transfers it to the server for data collection; 2) Option 2, UE collects training data and transfers it to Core Network(CN); 3) Option 3, UE collects training data and transfers it to OAM. For Option 2 and 3, CN or OAM may further transfer the training data to the UE server. Until now, no consensus has been reached for the UE-side data collection and it will be further discussed in Rel-20 AI/ML for air interface WI.

Observation 4	Various data collection methods are available in 5G, e.g., MDT, L3/L1 measurement report,  LPP, network-side data collection, etc. These distinct data collection methods increase the specification efforts and create trouble for operators to manage data in a unified and consistent manner.

On the other hand, emerging AI applications(e.g. Gen AI, token, AI agent, etc.) exhibits new traffic characteristics and QoS requirements, such as end-to-end joint latency(including round-trip communication and AI inference), burst data rate and latency, etc. On the other hand, 6G AI service (e.g. AI model training, AI model management or AI inference) [2] can be provided by the 6G network to a UE. When providing AI service to UE, the service performance (e.g. latency, inference accuracy) assurance should be considered.

2.2.1 Data collection

In this section, we focus on the data collection aspects for AI for network, network for AI and sensing respectively.

- AI for network

With regard to AI for network, as shown in Figure 1, the following three cases can be considered:

- UE side model: UE may report AI training data towards RAN/CN/DN for UE-side AI model training and receive well-trained UE-side AI model from RAN/CN/UE server. Here the DN denotes the data network, which includes the UE server, etc. On the other hand, the performance of UE-side model can be monitored by RAN/CN/DN which requires the UE to send the AI model performance data to RAN/CN/DN. Moreover, when multiple AI/ML models are available, more suitable AI/ML model can be selected and transferred to UE based on current situation.

- Network side model: RAN may collect data from UE for AI inference purpose. Moreover, RAN may collect measurement data from UE for the network side model training.

- Two-side model: RAN can collect AI training data from UE and UE receive the well-trained model from RAN.  In addition, RAN may also send data set toward UE for UE side training.

Figure 1 Data collection for AI for network scenarios

- Network for AI

As captured in [2], 6G network may provide AI service (e.g. AI model training, AI model management or AI inference) to UE. For example, UE may ask CN for the AI model training or AI model inference for an authorized application on UE. In this case, CN may collect AI training data from UE and then select suitable computing node for AI model training. Finally UE may receive the well-trained AI model from CN. Moreover,  UE may send the AI model performance data to CN, which may trigger the AI model retraining correspondingly. Furthermore, UE may send the AI inference data to CN for AI inference and then receive the AI inference result from CN.

On the other hand, it is captured in [2] that 6G system shall support secure means to expose 6G system data to authorized UE. As a matter of fact, it is already supported in 5G that AI/ML analytics provided by NWDAF can be exposed to authorized third party via NEF. When it comes to the 6G, it is likely that both RAN and CN may generate the AI/ML analytics and such AI/ML analytics(e.g. network performance analytics) can be exposed to UE to assist UE’s AI related application.

- Sensing

Generally speaking, six sensing modes need to be supported from Day1. To be specific, BS mono-static, BS-BS bi-static, BS-UE bi-static, UE-BS bi-static, UE mono-static, UE-UE bi-static (without sidelink), as shown in Figure 2. On the other hand, sensing can be used for two scenarios, i.e. sensing assisted communication and service-triggered sensing (e.g. UAV detection, environment detection, etc.).

Figure 2 Illustration of sensing modes

For the sensing assisted communication scenario, RAN may collect the sensing measurement data from UE to assist the scheduling or mobility control. Here the sensing measurement data may be generated by UE via sensing mode of BS-UE bi-static and UE-UE bi-static.

For the service-triggered sensing scenario, the data collection may be triggered by one sensing function within the 6G network. As a matter of fact, it is not yet clear where the sensing function resides. The details depends on the architecture progress of SA2/RAN.  Nevertheless, the sensing function may collect the sensing measurement data from UE for the sensing modes of BS-UE bi-static, UE mono-static and UE-UE bi-static. Moreover, the sensing function may collect the sensing measurement data from BS for the sensing modes of  BS mono-static, BS-BS bi-static, and UE-BS bi-static. Based on the collected sensing measurement data from UE and or BS, the sensing function may process the sensing data and then send the sensing result to UE and or BS if necessary.

As shown in Figure 3, the data size for the original sensing measurement data, intermediate data at different split points of sensing data processing and the final sensing result varies greatly. In order to reduce the data size to be reported, split sensing data processing can be considered where UE/RAN pre-processes the sensing measurement data. In this case, the intermediate sensing data and even sensing result can be reported by UE/BS towards the sensing function.

Figure 3 Data volume at different split points of sensing data processing

2.2.2 Data transfer

As analyzed in the data collection part, both AI and sensing related data need to be transferred between UE and RAN, between UE and CN, between UE and DN. The potential data path and data types that need to be transferred over the data path are summarized in Table 2.

Table 2 Comparison table of data path and example data to be transferred

| Data path | AI related data to be transferred | Sensing related data to be transferred |
| --- | --- | --- |
| UE<->RAN | - UE sends AI training data to RAN for UE side model or network side model training at RAN - UE receives UE side model from RAN - UE sends model performance data to RAN - UE sends AI inference data to RAN - UE receives data set from RAN for UE side training | - UE sends sensing measurement data to RAN - UE receives sensing result from RAN |
| UE<->CN | - UE sends AI training data to CN for UE side model training at CN - UE receives UE side model from CN - UE sends model performance data to CN - UE sends AI inference data to CN | - UE sends sensing measurement data to CN - UE receives sensing result from CN |
| UE<->DN (e.g. UE server, application server, etc) | - UE sends AI training data to DN - UE receives UE side model from DN - UE sends AI inference data to DN - UE receives AI inference result from DN | - UE sends sensing measurement data to DN - UE receives sensing result from DN |

Proposal 3	RAN2 to study the AI/ML and sensing data collection between UE and RAN/Core Network/Data network, focusing on the following data types: 1) AI/ML related, e.g. AI training data, AI model, AI inference data, AI inference result, AI model performance data, AI analytics; 2) Sensing related, e.g. sensing measurement data, sensing result, intermediate sensing data (pending on the input from RAN1).

It should be noted that the QoS requirement on priority, latency, reliability and data volume varies greatly for different AI/ML sensing data. As shown in Table 3, certain AI/ML model transmission requires high data rate and high latency while the AI inference result transmission requires low data rate and low latency. On the other hand, the sensing measurement data requires low latency, low reliability with high data volume while the sensing result requires low latency and high reliability.

Table 3 QoS requirements of various AI/ML and sensing data

| Data type | Data volume | Latency | Reliability |
| --- | --- | --- | --- |
| AI training data | Large | Medium, high | High, medium |
| AI inference data/result | small | Low | High, medium |
| AI model | Medium or large | Medium, high | High |
| AI analytics | Small | Low | High |
| Sensing measurement data | Large | Low | Low |
| Sensing result | Small | Low | High |

With regard to the AI/ML and sensing data transfer between UE and DN, it can be regarded as user traffic data which is application level data being transmitted through the 3GPP system. Correspondingly, traditional user plane (e.g. PDU session associated DRB and GTP-U tunnel via UPF) can be reused as baseline. However, for the AI/ML and sensing data transferred between UE and RAN or between UE and CN, they are essentially different from the traditional user traffic data and how to transfer them should be further investigated. To be specific, the following aspects should be considered:

- Radio bearer type: As we know,  traditional SRB is used for high priority signalling transmission. Only priority is associated with SRB. The differentiated QoS requirements for data traffic such as the latency, reliability and data rate, are not supported by traditional SRB. With regard to DRB, it is designed to support traffic with different QoS requirements. However, traditional DRB is associated with PDU session which is not the case for the AI/ML and sensing data between UE and RAN, or between UE and CN. It is FFS whether new radio bearer type should be introduced between UE and RAN.

- Data format: It is not clear whether the AI/ML and sensing data will be in the format of IP packet  or non IP packet. Suppose IP packet is used, IP segmentation, IP based QoS flow mapping, IP address and port number based addressing can be reused. However, for the non-IP packet, the potential segmentation, QoS mapping, and addressing at the termination point should be further studied. On the other hand, it is FFS whether ASN.1 or JSON/XML should be used to define the AI/ML and sensing data.

- QoS handling: For the IP-based AI/ML and sensing data, SDAP can be reused for the mapping from QoS flow to DRB in 5G. For the non-IP based AI/ML and sensing data, it is not yet clear how to map the various AI/ML and sensing data to RB. This actually depends on SA2’s work and should be discussed with SA2 jointly.

- Compression: Due to the large volume of AI training data, AI model and sensing measurement data, it is expected that compression can be supported. As far as we know, UDC is supported in PDCP for the uplink data  compression in 5G. It is necessary to investigate whether the UDC can be reused to support the AI/ML and sensing data compression.

- Encryption/decryption: According to the 6G requirements in [2], security and privacy protection should be supported for AI/ML and sensing data. Generally speaking, the encryption/decryption mechanism defined in PDCP for user plane traffic can be used for AI/ML and sensing data as well.

- Segmentation: In 5G, the maximum supported size of a PDCP SDU is 9000 bytes. Suppose IP packet is used for the AI/ML and sensing data, the IP layer segmentation can be used to meet the size limit of PDCP SDU. However, suppose the non-IP packet is used, it is FFS whether the segmentation and re-assembly should be supported on top of PDCP.

Proposal 4	For the AI/ML and sensing data transfer, RAN2 to study the following aspects: 1) Radio bearer type(e.g. SRB, DRB or new RB type) ; 2) data format (IP based or non-IP based, ASN.1 or JSON/XML); 3) QoS handling; 4) compression; 5) encryption/decryption; 6) segmentation.

As a matter of fact, SA2 is also discussing the 6G data framework. According to the 6G data framework summary [5] in SA2, many companies propose to design end to end 6G system architecture including UE, RAN, core network as both data service consumer and provider to support efficient and scalable data handling functionalities, and to study whether to use CP, UP or a dedicated data plane to transfer large volume of data generated from heterogeneous data sources (UE, RAN, and NF) in the 6G system. RAN2 should work jointly with SA2 on the efficient 6G data collection and transfer framework.

Proposal 5	    RAN2 should work jointly with SA2 on the efficient 6G data collection and transfer framework.

2.3 AI/ML framework

In Rel-18, the AI/ML life cycle management is generally outlined into the following Figure 4. Not all the components of LCM for AI/ML functions are specified within 3GPP network as only offline trained UE side model is focused in NR Rel-19 discussion. Actually the following LCM components for the AI/ML framework have been defined:

- Management framework

- Inference framework

- Data collection

Figure 4 Functional Framework for AI/ML for NR Air interface

Observation 5	In Rel-19 NR discussion for AI/ML LCM framework, only the management component, inference component and data collection component have been defined.

Regarding the management component, it includes three parts in general according to the Rel-19 AI/ML discussion:

- Applicability reporting: it is used for notifying the NW whether the AI/ML model at UE side is applicable for inference. It is necessary for the UE side model which is trained offline with the following reasons:

  - The offline-trained model is trained at a remote UE server, UE needs to download from UE server before using it.

  - The training environment for the offline-trained model may be not consistent with the current inference environment.

- Performance monitoring & functionality management: for the controllability of the UE side model, the metrics provided by performance monitoring is also necessary in 6G.

  - In 5GA, the performance monitoring is performed at NW side, that is, UE calculates the performance metrics and reports them to the NW, then NW determines the functionality management according to the metrics. Alternatively, UE transfers both the inference value and corresponding ground truth value to the NW, and NW determines the functionality management according to the comparison between inference value and ground-truth value.

  - In 5GA, the functionality management is done by using RRC signalling (e.g. to Add or Remove the CSI-ReportConfig for inference) or by using DCI/MAC CE (e.g. to use the SP CSI Report Activation/Deactivation MAC CE).

Regarding the inference framework, it is tightly related to the use case. With the positioning case as an example, the inference framework is designed by using LPP protocol. For the beam management and CSI report use case, the inference framework with CSI-framework is designed, which can be seen that the inference component is use case dependent.

Regarding the data collection component, it has been discussed in subclause 2.2.1 in this contribution.

Observation 6	In 5GA AI/ML model LCM, there is no 3GPP impact from model training component, and the management component (i.e. applicability reporting, performance monitoring, functionality management) defined in the Rel-19 is applicable for the offline trained UE side model; while the inference component defined is use case dependent.

For 6G AI/ML LCM, if the use case in 6G still uses offline-trained UE side model, the current 5G AI/ML framework can be reused, that is, the management component (including Applicability reporting, performance management, functionality management) can be reused. For the inference component, it shall be as same as the 5GA, the inference component in 6G AI/ML LCM shall be the use case dependent.

In 5GA, only offline-trained UE side model is studied, and the AI/ML framework is designed based on it. However, as mentioned in above, even though the offline-trained model has a lower requirement of UE device hardware capability, the generalization issue may be caused for some use cases (e.g. AI/ML based BM, AI/ML based Positioning). In order to mitigate the generalization issue, UE needs to train a large number of AI/ML models beforehand within various training environments to match with the potentially different inference environment. NW needs to disclose its private settings (e.g. masked by associated ID) for UE to determine whether an AIML model has the consistency between training environment and inference environment.

Conversely, the online trained model is trained using the ground-truth value so that there is no concern regarding the generalization issue. However, the on-line model training needs UE to be equipped with more powerful computing hardware and more sustainable battery.

Observation 7	Online-trained UE side model, compared to the offline-trained UE side model, can bring the benefits for both UE vendor and NW vendor. From UE vendor perspective, the generalization issue caused by offline-trained UE side model can be avoided and the offline effort to train a large number of models can be saved; from NW perspective, signalling cost for the associated Id  and offline coordination effort for associated Id allocation can be saved.

For supporting online-trained UE side model, the following enhancement of each component in AI/ML LCM framework in 5GA can be foreseen:

Regarding the model training component, the online mode training shall be configured and controlled by the NW, and how to test the online trained-model shall be also discussed. Besides, for providing the data resources to the model training, the ground-truth data collection for online-model training should be taken into account as well.

Regarding the management component, it is not sure whether the applicability reporting is needed or not as the generalization issue is not existed for the online trained model. Moreover, regarding the model management, except for the activation/deactivation of the model, may be the fine-tuning of an AI/ML model can be a new candidate for the model control.

Regarding the model inference component,  the model inference mechanism may need some enhancements and the coordination between mode training component and model inference component is needed if we support on-line model training/fine-tuning without interrupting the model inference.

Regarding the data collection component, the data collection framework in 5GA for the online-model training shall be enhanced as the requirement of the data collection for the offline model training is totally different from the data collection for the online model training.

In a sum, compared to the offline model training, online model training/fine-tuning need further study on each component (e.g. model training, data collection, management, model inference), as well as the coordination between those components. In this sense, the AI/ML framework in 5GA maybe no longer suitable.

However, for now, there is no use case studied in RAN1 where the online-trained UE side model is required, and RAN2 use case discussion has not yet started. In this sense, we think the AI/ML LCM framework for on-line trained UE side model can wait for the input from RAN1 or outcome of RAN2 use case discussion.

Proposal 6       For 6G AI/ML LCM framework design:

-	Case 1:  the AI/ML LCM framework in 5GA can be the baseline for the offline-trained UE side model LCM;

-	Case 2:  a new AI/ML LCM framework shall be studied in RAN2 for the online-trained UE side model LCM (pending the input from RAN1 and outcome of the RAN2 use case discussion).

As we can see, 6G use case is the key factor for RAN2 to determine what 6G AIML framework will be like. If the use case only applies the offline-trained model, the 5GA AI/ML framework can generally fit in, otherwise, RAN2 need to study how to enhance the 5G AI/ML framework to support the online-trained model.

For the AI/ML use case study in RAN2, the scope includes both use cases inherited from 5G—such as AI/ML-based mobility—and new use cases proposed by companies for 6G. Examples of these new use cases are:

- AI/ML-based traffic pattern prediction

- AI/ML based BSR

- AI/ML based C-DRX

- AI/ML-based PDCP duplication

- AI/ML-based QoS control

- AI/ML-based L2 parameter prediction

For each of these use cases, RAN2 should conduct a case-by-case analysis and for each use case, the proponent shall address the following aspects:

- Use case description: including the motivation, justification, etc.

- Performance gain: quantify the improvement achieved by the AI-based feature compared to non-AI solutions. This assessment will help determine whether the use case is worthwhile for standardization in 3GPP. However, in some aspects such as power saving, the gain is hard to be evaluated. So the methodology of the gain evaluation shall be contained in this aspect as well.

- Specification impact: evaluate the expected impact on 3GPP specifications for supporting the use case in 6G, such as the analysis of the components of the LCM framework, including:

  - Data Collection: Identify which measurements and/or measurement results should be used for model training, inference, and performance monitoring.

  - Model Training: Determine the training approach (e.g., offline, online, or federated learning) and assess whether online training can be network-controlled—and if so, how.

  - Model Inference: Define the input and output data required for inference.

  - Management: Establish methods to ensure environmental consistency between training and inference, define performance monitoring techniques, and specify AI/ML model control mechanisms.

Proposal 7	RAN 2 to conduct a case by case study on 6G AI/ML use cases and the use case proponent shall provide the following aspects for the use case discussion: 1) Use case description, including the motivation, justification; 2) Performance gain, including the methodology for the gain evaluation; 3) Specification impact, including the evaluations based on each component of the LCM.

Conclusion

In this paper, we discussed the general principles and design considerations of access stratum security, data transfer of various type of data and AI/ML framework. And we have the following observations and proposals:

Observation 1   From RAN2 perspective, security protection for the low layer signalling should be studied.

Observation 2 	The solution for the transmission of low layer command (e.g. legacy MAC CE based and/or DU terminated local RRC based) will impact the discussion in SA3 for the security protection of low layer command.

Observation 3  Some benefits can be observed to allow the change of CP key without the change of UP key (to minimize the requirement on key refresh and reduce the interruption time during mobility).

Observation 4	Various data collection methods are available in 5G, e.g., MDT, L3/L1 measurement report,  LPP, network-side data collection, etc. These distinct data collection methods increase the specification efforts and create trouble for operators to manage data in a unified and consistent manner.

Observation 5	In Rel-19 NR discussion for AI/ML LCM framework, only the management component, inference component and data collection component have been defined.

Observation 6	In 5GA AI/ML model LCM, there is no 3GPP impact from model training component, and the management component (i.e. applicability reporting, performance monitoring, functionality management) defined in the Rel-19 is applicable for the offline trained UE side model; while the inference component defined is use case dependent.

Observation 7	Online-trained UE side model, compared to the offline-trained UE side model, can bring the benefits for both UE vendor and NW vendor. From UE vendor perspective, the generalization issue caused by offline-trained UE side model can be avoided and the offline effort to train a large number of models can be saved; from NW perspective, signalling cost for the associated Id  and offline coordination effort for associated Id allocation can be saved.

Proposal 1 	RAN2 should identify the type of lower layer signalling that is likely to be specified for 6G and convey this information to SA3 and RAN2 should study the format for the transmission of lower layer signalling (e.g. MAC CE based or Local RRC based), and inform SA3 the corresponding conclusion.

Proposal 2 	RAN2 should study the potential enhancement on the key management for mobility enhancements and inform SA3 of our preference. It is up to SA3 to determine whether such enhancement is acceptable from a security perspective or not.

Proposal 3	RAN2 to study the AI/ML and sensing data collection between UE and RAN/Core Network/Data network, focusing on the following data types: 1) AI/ML related, e.g. AI training data, AI model, AI inference data, AI inference result, AI model performance data, AI analytics; 2) Sensing related, e.g. sensing measurement data, sensing result, intermediate sensing data (pending on the input from RAN1).

Proposal 4	For the AI/ML and sensing data transfer, RAN2 to study the following aspects: 1) Radio bearer type(e.g. SRB, DRB or new RB type) ; 2) data format (IP based or non-IP based, ASN.1 or JSON/XML); 3) QoS handling; 4) compression; 5) encryption/decryption; 6) segmentation.

Proposal 5	    RAN2 should work jointly with SA2 on the efficient 6G data collection and transfer framework.

Proposal 6       For 6G AI/ML LCM framework design:

-	Case 1:  the AI/ML LCM framework in 5GA can be the baseline for the offline-trained UE side model LCM;

-	Case 2:  a new AI/ML LCM framework shall be studied in RAN2 for the online-trained UE side model LCM (pending the input from RAN1 and outcome of the RAN2 use case discussion).

Proposal 7	RAN 2 to conduct a case by case study on 6G AI/ML use cases and the use case proponent shall provide the following aspects for the use case discussion: 1) Use case description, including the motivation, justification; 2) Performance gain, including the methodology for the gain evaluation; 3) Specification impact, including the evaluations based on each component of the LCM.

Reference

- RP-251881, New SID: Study on 6G Radio, June 2025.

- 3GPP TR 22.870 v0.4.0, Study on 6G use cases and requirements, September, 2025.

- S3-252973, Study on Security for the 6G System, August, 2025.

- 3GPP TR 38.843 v18.0.0, 3GPP TSG RAN Study on AI/ML for NR air interface, December, 2023.

- S2-2508132, [WT#5 Data framework] Summary of WT scope on 6G data framework, August, 2025.