3GPP TSG-RAN WG2 Meeting #131bis	R2-2507266

Prague, Czech Republic, 13 – 17 October 2025

Agenda item:	8.11.2

Source:	Nokia

Title:	MAC Issues - SBFD

WID/SID:	NR_duplex_evo-Core - Release 19

Document for:	Discussion and Decision

# 1	SBFD Random Access Operation

For initial access, the UE selects the index of the preferred SSB index and decodes the associated PBCH for MIB, and obtains relevant information for decoding the SIB1, from where it obtains the RACH configuration for sending Msg1. The RACH configuration contains, among others, the PRACH configuration index, the frequency domain multiplexing of the RACH occasions (ROs), the frequency offset of the ROs, and information about the power ramp up during the RACH procedure. For UEs in RRC_CONNECTED, dedicated RACH configuration, including CFRA, can be indicated by the network. If the CFRA configuration is missing, UEs in RRC_CONNECTED use the common RACH configuration for CBRA.

To enable support of initial access in SBFD, i.e., in the UL subband, RAN1 has been discussing the above aspects of random access procedures in SBFD since RAN1#116. In fact, RAN1 observed that support of initial access in SBFD provides at least one or more following benefits: reduced random access latency, reduced PRACH collision probability or allowing more contiguous frequency resources for PUSCH in UL slots, improved coverage of PRACH with sparse UL resources, and increased cell range of PRACH with sparse UL resources. The WID was updated in RAN#104 [RP-241614] to specify SBFD operation to support random access in SBFD symbols by UEs in RRC_IDLE/INACTIVE mode, along with RRC_CONNECTED mode in RAN1 and RAN2.

# 2	MAC Open issues

## 2.1	MAC-3: UE transmit power continuity during RO type switching

In RAN2#131, how to ensure UE transmit power continuity during RO type switch was discussed. As a result, the following proposal introduces a power offset to compensate for the UE transmit power difference in legacy ROs and additional ROs:

Proposal: For the RO type fallback between legacy RO and additional RO, a power offset given by the difference between the two quantities of preamble power ramping steps is added.

This is trying to solve the following issue as shown in Figure 1:

Figure 1

Following the same principle as for the 2-step RACH to 4-step RACH switch, it was proposed to introduce the following power offset compensation:

By using this approach, the UE power continuity is ensured given that the preamble received target power is the same for both the additional ROs and legacy ROs, as shown in Figure 2:

Figure 2

However, in our view, the current proposal is not completely solving the UE transmit power continuity issue when switching between additional RO to legacy RO or vice versa. The reason is that the agreed proposal only compensates for the potential differences in the power ramping step but doesn’t take into account the potential differences in the preamble received power. In Configuration 2, the UE will receive 2 independent RACH-ConfigCommon configurations, which among others, contain different preambleTargetReceivedPower and/or different powerRampingStep. Thus, the power continuity cannot be achieved by applying the RAN2#131 compensation method as shown in Figure 3.

Figure 3

Observation 1: With Configuration 2, UE receives two separate RACH-ConfigCommon settings. As a result, the current method does not completely solve the UE transmit power continuity issue when switching between additional RO to legacy RO or vice versa as it doesn’t take into account the potential differences in the preamble received power.

Therefore, the UE transmit power continuity is not ensured. This can lead to a larger power ramp-down or power ramp-up of 6 dB, which is not acceptable from UE vendors point of view as per last meeting’s discussion. Therefore, it is required to find a new way of compensating for both the differences in power ramping offsets and preamble received target powers.

Proposal 1: To support the compensation for the difference in preamble received target power between SBFD RO and legacy RO (in addition to the compensation for the power ramping difference that is already supported), for ensuring preamble transmit power continuity, during RO type switching.

TP : The power offset adjustment for UE transmit power continuity between SBFD RO to legacy RO (or vice versa) can be determined as follows:

Section 5.1.4 2> if the Random Access procedure is not completed: 3> if preambleTransMaxRO-Type is applied, and neither contention-free Random Access Resources nor Random Access resources for SI request have been provided for this Random Access procedure, and PREAMBLE_TRANSMISSION_COUNTER = preambleTransMaxRO-Type + 1: 4> if the RO_TYPE is set to 2nd-RO, and set of Random Access resources associated with the same feature or feature combination, and with the same or higher Msg1 repetition number (if the Random Access Preamble is transmitted with repetitions), than the current set of Random Access resources, is available for the first PRACH occasions as defined in TS 38.213 [6]: 5> set the RO_TYPE to 1st-RO; 5> select the set of Random Access resources associated with the same feature or feature combination, and with the same Msg1 repetition number if available, or with the next higher Msg1 repetition number otherwise (if the Random Access Preamble is transmitted with repetitions), for this Random Access procedure; 5> if sbfd-RACH-DualConfig is configured for the Random Access procedure (see TS 38.331 [5]): 6> set PREVIOUS_RO_TYPE_PREAMBLE_POWER_RAMPING_STEP to PREAMBLE_POWER_RAMPING_STEP; 6> (re-)initialize the parameters specified in clause 5.1.1 for the Random Access procedure according to the values configured by RRC for the selected set of Random Access resources; 6> re-initialize PREAMBLE_POWER_RAMPING_STEP and SCALING_FACTOR_BI as specified in clause 5.1.1a; 6> set POWER_OFFSET_RO_TYPE to preambleReceivedTargetPower (not included in sbfd-RACH-DualConfig) - preambleReceivedTargetPower (included in sbfd-RACH-DualConfig) + (PREAMBLE_POWER_RAMPING_COUNTER – 1) × (PREVIOUS_RO_TYPE_PREAMBLE_POWER_RAMPING_STEP – PREAMBLE_POWER_RAMPING_STEP). 4> else if the RO_TYPE is set to 1st-RO, and set of Random Access resources associated with the same feature or feature combination, and with the same or higher Msg1 repetition number (if the Random Access Preamble is transmitted with repetitions), than the current set of Random Access resources, is available for the second PRACH occasions as defined in TS 38.213 [6]: 5> set the RO_TYPE to 2nd-RO; 5> select the set of Random Access resources associated with the same feature or feature combination, and with the same Msg1 repetition number if available, or with the next higher Msg1 repetition number otherwise (if the Random Access Preamble is transmitted with repetitions), for this Random Access procedure; 5> if sbfd-RACH-DualConfig is configured for the Random Access procedure (see TS 38.331 [5]): 6> set PREVIOUS_RO_TYPE_PREAMBLE_POWER_RAMPING_STEP to PREAMBLE_POWER_RAMPING_STEP; 6> (re-)initialize the parameters specified in clause 5.1.1 for the Random Access procedure according to the values configured by RRC for the selected set of Random Access resources; 6> re-initialize PREAMBLE_POWER_RAMPING_STEP and SCALING_FACTOR_BI as specified in clause 5.1.1a; 6> set POWER_OFFSET_RO_TYPE to preambleReceivedTargetPower (included in sbfd-RACH-DualConfig) - preambleReceivedTargetPower (not included in sbfd-RACH-DualConfig) + (PREAMBLE_POWER_RAMPING_COUNTER – 1) × (PREVIOUS_RO_TYPE_PREAMBLE_POWER_RAMPING_STEP – PREAMBLE_POWER_RAMPING_STEP).. Note: Similar changes in 5.1.5 as well.

# 3	Conclusion

Observation 1: With Configuration 2, UE receives two separate RACH-ConfigCommon settings. As a result, the current method does not completely solving the UE transmit power continuity issue when switching between additional RO to legacy RO or vice versa as it doesn’t take into account the potential differences in the preamble received power.

Proposal 1: To support the compensation for the difference in preamble received target power between SBFD RO and legacy RO (in addition to the compensation for the power ramping difference that is already supported), for ensuring preamble transmit power continuity, during RO type switching.