3GPP TSG-RAN WG2 Meeting #131bis	R2-2507431

Prague, Czech Republic, October 13th – 17th, 2025

Agenda Item:	8.1.3

Source:	InterDigital Inc.

Title:	Remaining issues for NW side data collection (N028, H002, H007, Z005, H009)

Document for:	Discussion

# 1. Introduction

In this contribution, we address some of the RRC open issues related to network side data collection that were not settled during the ASN1 review after RAN2-131.

# 2. Discussion

## 2.1 Reuse of A1/A2 events for NW-side logging (N028)

[Description]: It was agreed to reuse A1 and A2 events. The implementation does not reflect that agreement. The proposed change below could be simplified by making the entire structure a choice which can only ever contain different types of events. That is, if we do not think this IE would be extended with anything else, then the outer level can be a CHOICE and the second set of extension markers could be removed.

[Proposed Change]:

CSI-LoggedMeasurementEventTriggerConfig-r19 ::=          SEQUENCE {

eventId        CHOICE {

eventA1      SEQUENCE  {

a1-Threshold    MeasTriggerQuantity,

hysteresis      Hysteresis,

timeToTrigger   TimeToTrigger

},

eventA2     SEQUENCE  {

a2-Threshold    MeasTriggerQuantity,

hysteresis      Hysteresis,

timeToTrigger   TimeToTrigger

},

...

},

...

}

In our view, as the rapporteur has also pointed out, both the proposed changes and the original way it was captured lead to the same outcome from the pov of UE behaviour.

The logged MDT configuration (LoggedMeasurementConfiguration) is the most similar configuration we currently have in RRC that is also have event based configuration, where the event based configuration is captured as below:

LoggedEventTriggerConfig-r16 ::= SEQUENCE {

eventType-r16 			EventType-r16,

loggingInterval-r16 		LoggingInterval-r16,

...

}

EventType-r16 ::= CHOICE {

outOfCoverage NULL,

eventL1 SEQUENCE {

l1-Threshold 	MeasTriggerQuantity,

hysteresis 		Hysteresis,

timeToTrigger 	TimeToTrigger

},

...

}

Thus, for the sake of standard clarity, we prefer the proposal from Nokia in N028, with slight modification to some of the names of the IEs as shown below (changes from the proposal in N028 highlighted in yellow):

CSI-LoggedMeasurementEventTriggerConfig-r19 ::=          SEQUENCE {

csiLoggedMeasurementEventId        CHOICE {

csiLoggedMeasurementEventA1      SEQUENCE  {

a1-Threshold    MeasTriggerQuantity,

hysteresis      Hysteresis,

timeToTrigger   TimeToTrigger

},

csiLoggedMeasurementEventA2     SEQUENCE  {

a2-Threshold    MeasTriggerQuantity,

hysteresis      Hysteresis,

timeToTrigger   TimeToTrigger

},

...

},

...

}

Proposal 1: Align the CSI-LoggedMeasurementEventTriggerConfig IE with the event configuration definitions for RRM and logged MDT, as proposed in RIL: N028, and additionally prefixing the event IDs with csiLoggedMeasurement.

## 2.2 Retaining logged measurements during LTM (H002)

[Description]:

RAN2 made the following agreement:

- RAN2 confirm that the solution agreed in RAN2#130 is applicable to regular HO and CHO (i.e. 1-bit indication corresponding to each candidate cell configuration in RRCReconfiguration is provided).

LTM is not part of the agreement, but it seems there is nothing preventing the network from adding retainLoggedMeasurements also to the LTM candidate cells which also reuse RRCReconfiguraiton containers.

[Proposed Change]: No change is needed, but RAN2 is requested to confirm that retainLoggedMeasurements can also be used for LTM candidate configurations.

One issue that is different in LTM from the normal HO case is the possibility of subsequent LTM, where a UE may receive multiple LTM candidate configurations and may perform LTM from the source to a first target and then to a second target without receiving a reconfiguration in between.

Consider the way the retainLoggedMeasurement is used in the RRC reconfiguration procedure in the latest CR:

3>	if the RRCReconfiguration includes retainLoggedMeasurements:

4>	if the UE has logged measurement entries available in VarCSI-LogMeasReport: [RIL]: Q510, AIML

5>	include csi-LogMeasAvailable in the RRCReconfigurationComplete message;

3>	else:

4>	discard the logged measurement entries included in VarCSI-LogMeasReport, if any;

Assume the UE was configured with LTM candidate target A, B and C, and the retainLoggedMeasurements was set to TRUE in all the LTM configurations of these candidates (e.g., in the reference configuration that is common to the LTM candidate configurations or to the individual delta LTM configuration that is specific to each candidate). When the UE performs the first LTM cell switch (e.g., to cell B), it will indicate that the logged measurement is available. If the UE was requested to send the logged measurements while in cell B, then it will do so and next time it performs cell switch to another cell (e.g., to cell A), it will not indicate that it has logged measurement available because it has already sent the measurements. However, if the UE has not been asked to send the measurements while in cell B, it will indicate that it has logged measurements, and those measurements can be retrieved in cell A, and so on.

If there was a case where the retainLoggedMeasurements was set to TRUE only in some of the candidate cells, then it is likely that the UE may perform cell switch to a target whose LTM configuration doesn’t have retainLoggedMeasurements set to TRUE and thus will discard the logged measurements and if it then switches to a target whose LTM configuration does have retainLoggedMeasurements set to TRUE, it won’t have any logged measurements to send. However, we don’t think this is a very corner case where the network was able to enable LTM between targets A, B and C, but was only possible to do the logged data transfer/routing only between a subset of the target cells.

Considering the above, we propose:

Proposal 2: RAN2 to confirm that no special handling is needed regarding the retraining/discarding of data during LTM (i.e., current retainLoggedMeasurement handling for HO is sufficient also for LTM).

## 2.3 Logged measurement configuration modification and release (H007)

[Description]:

Currently it is possible for the network to provide an updated logged measurement configuration (i.e. reusing the same csi-LoggedMeasurementConfigId as already included in the UE configuration). This may lead to ambiguity in the collected data, i.e. once it is reported it will be unclear to which configuration this data referred to.

Similarly, the UE currently does not discard the collected data for a certain logging configuration even when this configuration is released. If the UE is subsequently configured with a new clogging configuration later on, reusing the ID of the previously released configuration, similar ambiguity exists. Such situation should be avoided.

[Proposed Change]:

It is proposed to clarify that when the UE receives a modified logging configuration or releases a logging configuration, the discards the logged data related to the modified/released logging configuration. This way the ambiguity mentioned above can be avoided.

We agree that if the logged measurement configuration is changed or released, there could be some ambiguity of collected data. However, we think these cases can be easily handled by the network with the need for the modification of UE behaviour.

- release of a data collection configuration: Data collection configuration is released upon explicit release by the network. Thus, network can also request the UE to send the logged data upon releasing the configuration.

- Modification of data collection configuration: For the case of the modification of the network data collection configuration, the network can either retrieve the data before the reconfiguration, or it can configure a new data configuration with a different ID. As the current ID space maxNrofLoggedMeasurementConfigurations allows 48 configurations, and it is very unlikely that the UE will be configured with 48 configurations and doing the logging of all 48 configurations in parallel.

Considering the above, we propose:

Proposal 3: RAN2 to confirm that no special handling of the logged data is needed regarding the modification and release of data collection configuration (i.e., up to network to retrieve the collected data before the modification/release or reconfigure using a different configuration ID).

## 2.4 Interaction with PHY measurements (Z005/H009)

Z005: Start/stop performing L1 measurement in Logged NW side data collection

H009: Interactions with PHY for NW-side data collection

[Description]:

In the current description of text procedure for performing logging, it only describes UE behaviour regrading logging or not logging.

[Proposed change]:

The RILs propose changes whereby the RRC indicates to the PHY to start the L1 measurements only when the conditions to log are fulfilled and stop the measurements when the conditions are no more fulfilled.

We agree to the proposed changes in that if those changes are not implemented, the UE may end up wasting its power performing measurements for logging which are not logged due to data collection, specifically what is needed is:

- Add an explicit indication from the RRC to PHY in the data collection procedure to start the L1 measurements only if the conditions to log are fulfilled (e.g., it is periodic only, or if it is event based and event conditions are fulfilled) and there is also buffer space available, i.e., “instruct lower layer to start the L1 measurements….”

- Add an explicit indication from the RRC to PHY in the data collection procedure to stop the L1 measurements when the logging event conditions are no more fulfilled or the buffer is full, i.e., “instruct lower layer to stop the L1 measurements..”)

When it comes to the exact phrasing of the procedures in RRC, we prefer the proposal form ZTE in Z005, mainly for the sake of readability/clarity.

Proposal 4: Capture in the data logging procedure that the PHY will be instructed to start the L1 measurements only if the conditions to log are fulfilled (i.e., buffer space is available AND (only periodic logging is configured or logging event conditions fulfilled))

Proposal 5: Capture in the data logging procedure that the PHY will be instructed to stop the L1 measurements if the conditions to log are no more fulfilled (i.e., buffer space is not available OR logging event conditions are not fulfilled)

Proposal 6: If Proposal 4 and 5 are agreed, adopt the TP proposed in RIL: Z005.

# 4. Conclusion

In this contribution, we addressed some of the RRC open issues related to network side data collection that were not settled during the ASN1 review after RAN2-131, and the following are proposed:

Proposal 1: Align the CSI-LoggedMeasurementEventTriggerConfig IE with the event configuration definitions for RRM and logged MDT, as proposed in RIL: N028, and additionally prefixing the event IDs with csiLoggedMeasurement.

Proposal 2: RAN2 to confirm that no special handling is needed regarding the retraining/discarding of data during LTM (i.e., current retainLoggedMeasurement handling for HO is sufficient also for LTM).

Proposal 3: RAN2 to confirm that no special handling of the logged data is needed regarding the modification and release of data collection configuration (i.e., up to network to retrieve the collected data before the modification/release or reconfigure using a different configuration ID).

Proposal 4: Capture in the data logging procedure that the PHY will be instructed to start the L1 measurements only if the conditions to log are fulfilled (i.e., buffer space is available AND (only periodic logging is configured or logging event conditions fulfilled))

Proposal 5: Capture in the data logging procedure that the PHY will be instructed to stop the L1 measurements if the conditions to log are no more fulfilled (i.e., buffer space is not available OR logging event conditions are not fulfilled)

Proposal 6: If Proposal 4 and 5 are agreed, adopt the TP proposed in RIL: Z005.

# 5. References

[1]	R2-2506530, Introduction of AIML for NR air interface, RAN2-131, Bengaluru, India, Aug 2025

[2]	ASN1 comments for AIML (https://www.3gpp.org/ftp/Email_Discussions/RAN2/%5BMisc%5D/ASN1%20review/Rel-19%202025-09/38331/CR%20ASN1%20review/AIML?sortby=daterev)