3GPP TSG-RAN WG2 Meeting #132	R2-2508032

Dallas, USA, November 17-21, 2025

Agenda item:	10.3.1.1

Source:	Qualcomm Incorporated

Title:	Discussion on 6G user-plane functionalities

WID/SID:	FS_6G_radio

Document for:	Discussion and decision

# Introduction

In this paper, we discuss essential user-plane functionalities required for 6G and those that need to be improved for anticipated new use cases or can benefit from better designs.

# Discussion

## L2 Baseline

In the last meeting, the first agreement in UP design has identified the need to characterize the main L2 functionality that should be supported as a baseline.

Agreements on UP

- 1.	RAN2 first discuss/agree on the required functionalities for the protocol stack, and only afterwards discuss which protocol layer supports each functionality.  Discussions should focus on standalone architecture, until told otherwise by plenary.  When discussing functionalities companies should focus on identifying the problems being address or new requirements.

To this end, RAN2 L2 User Plane discussions should focus on standalone RAN architecture making sure that design does not block progress by RANP and other WGs ongoing discussions on the several architecture and migration open issues. A first step should be defining a minimum functionality that L2 should support.

This minimum functionality comprises of the minimum set of User Plane features that every UE should support. The exact layering architecture would not be very critical at the beginning but would evolve to accommodate this functionality naturally considering agreements on architecture and migration as they are reached by RANP and other WGs.

The minimum functions proposed below are the functions supported in Rel-15. They are a natural starting point as a baseline to build on. The functions below use TS 38.300 (Rel-15, clauses 6.2- 6.5 [1]) as a reference. The reason we choose Rel-15 as a starting point is that all the features that were developed in (Rel-16 – Rel-19) built on the Rel-15 design and thus were constrained by those design choices. The minimum baseline agreements do not preclude any other functions to be agreed later and added to the minimum L2 functionality. They have been captured without specifying implementation choices occasionally captured in stage 2 spec (e.g. “Header compression and decompression” instead of “Header compression and decompression using the ROHC protocol”). This is to isolate discussions about functions and specific implementation choices which can be discussed separately for every function once there is an agreement to support it.

Proposal 1.  	L2 supports the following minimum set of functionalities as a baseline:

| - Transfer of data - Ciphering and deciphering - Integrity protection & verification - Protocol Error detection and recovery, i.e., ARQ | - Segmentation & Reassembly - Header Compression & decompression - SDU marking, e.g., QFI - SDU discard | - Reordering and in-order delivery - Out-of-order delivery - Duplicate discarding |
| --- | --- | --- |

## SDAP

Aside from the functionality, a critical ongoing discussion in L2 UP design would be to specify the L2 data flow (similar to clause 6.6 in 38.300) [1]. A first step in this design would be specifying the traffic granularity incoming into L2. In NR, the QoS flow granularity was used to identify incoming traffic. Since the QoS flow framework is flexible and can accommodate different protocols (e.g., IP/Ethernet), we expect a similar abstraction to be used in 6G-UP as the traffic granularity incoming to UP.

In-order to offer more QoS flexibility 5G-NR introduced some enhancements over LTE:

- Introduction of QoS flows in RAN that can be mapped to different bearers.

- The ability to dynamically switch flow-to-bearer mapping using in-band SDAP signalling.

While the first enhancement is expected to carry on in 6G, it is worth reflecting on the general SDAP mechanism in light of lessons learned from 5G-NR considering the below agreement last meeting:

Agreement:

UP design should aim to be hardware-processing friendly while keeping memory requirements low and minimize data movements across protocol layer.

SDAP came at a high hardware processing cost since it introduced a mandatory per-packet header check for every DL packet when SDAP is configured, which means a cost that increases proportional with DL data rate. On the other hand, field experience has shown little, if any, practical deployment of the RQoS mechanism. Considering 6G UP design goals of simplification and processing reduction, RAN2 should rethink the mechanism by which NR performed Flow-to-bearer (re)mapping, namely RQoS, and consider available alternatives to perform the same procedure.

Observation 1. 	In 5G, the SDAP RQoS mechanism was not widely deployed. It complicated hardware by mandating a per-packet RQoS check. Additionally, Control Plane may perform flow-to-bearer mapping.

Proposal 2.  	RQoS mechanism is not supported in 6G UP to reduce packet processing.

## MAC

RACH

In NR, RACH partitioning was introduced in Rel-17 to support early indication of feature(s) such as RedCap, SDT, slice, coverage enhancements, etc. Each combination of features requires a separate configuration of RACH resources (aka a RACH partition). When new features that require early indication are introduced in a subsequent release, more RACH partitions need to be added. As the number of such features (N) grow over releases, the total number of required RACH partitions can grow very fast, which is in the order of O(N!). This design approach led to poor scalability and very inefficient use of RACH resources. It also created overly complex signaling for RACH configurations, so much that CRs for fixing related problems were still submitted 18 months after Rel-17 was frozen.

Observation 2. 	In NR, early indication of feature combinations through RACH partitioning leads to very inefficient use of RACH resources.

For these reasons, we think RACH partitioning should be avoided as much as possible in 6G. RAN2 should work with RAN1 and discuss whether Msg1 based early indication can be avoided for most of the expected UE types or services in 6G (e.g. whether they may rely solely on Msg3 based indication). If not, RAN2 and RAN1 should explore techniques for Msg1 transmission that allow early indication to be more efficiently signaled.

Another inefficiency with the 5G RACH design is that allocation of RACH resources does not adapt well to short-term variations in RACH load. Since RACH configuration is provided in SIB1 and can only be updated via system information modification, reconfiguring them to handle changes in access loads is very slow. But field data shows that there are often temporary, short-term changes in access load, e.g. RACH is triggered by many UEs at the same time in the same area due to some events. SIB update simply is too slow to handle this type of change, resulting in high level of access contention and even failed RACH. In Rel-19, dynamic PRACH via paging DCI was introduced to help network save energy. This enhancement can help make RACH resources become adaptive. However, its effectiveness is limited because it can’t be used by UEs in RRC Connected or those already engaged in RACH procedure.

Observation 3. 	In NR, RACH resources do not adapt well to short-term variations in access load, often resulting in high level of contention and access failures.

Therefore, we believe it is important to explore design approaches that offer greater versatility and efficiency than the legacy designs. For example, network may allocate additional RACH resources via Msg2/B when it detects high level of contention. Or allow UE to request RACH resources on demand, i.e. some RACH resources are configured but not activated until they are requested by UE.

#### Proposal 3.  	Study RACH designs that enable more efficient use of resources, e.g. avoid RACH partitions unless truly necessary, dynamic or on-demand allocation of RACH resources, etc.

MAC CE format

Over 5G releases, we have witnessed rapid increase in the size and number of MAC CEs, as many features/procedures prefer to use MAC CE as the means to de-/activate radio configurations (e.g. MIMO), reporting UE status (e.g. LTM). We expect the use of MAC CEs in 6G will continue to increase. In fact, many companies have shown interest in using MAC CE to send a broader range of messages, e.g. PHY-layer control messages, measurement reports, etc. Therefore, we believe it is important for MAC CEs in 6G to be designed with greater flexibility and enabled their use as a versatile signaling mechanism.

One consequence of new payload type is that some of them, such as measurement reports, command for reconfiguration, or those that are important and do not change much over the span of a typical HARQ process, may demand higher reliability. It hence is worth studying whether it is beneficial to increase the reliability of such MAC CEs, by techniques such as MAC-layer retransmission.

#### Observation 4. 	MAC CEs in 6G are expected to be used to send more diverse types of messages, some of which may demand higher reliability.

One of the drawbacks of the 5G MAC CE design is that most MAC CEs were designed individually, without a systematic or unified approach. As their number becomes large, this lack of structure is not friendly to UE implementation and increases their processing time. We therefore think that the 6G design should ensure efficient processing of MAC CEs and keep their processing time low, to better align with the stricter timing requirements expected in 6G.

#### Observation 5. 	MAC CEs in 6G can be designed in a more structured way to enable efficient processing and meet the stricter timing requirements expected in 6G.

Based on the above observations, we think potential areas to explore may include:

- Organizing logical channel IDs for MAC CEs in a more structured manner compared to the flat ID space used in 5G. This approach would enable UE to process them faster, because UE can use hardware simpler than table lookup when parsing a received MAC CE.

- Including a length field in all MAC CEs—regardless of whether their sizes are fixed or variable—could streamline the decoding process for both the MAC CE itself and the TB.

- Supporting different payload formats, to better accommodate different types of messages in MAC CEs (as mentioned above).

In summary, we’d propose that RAN2 discuss the following design objectives for 6G MAC CE design:

- The MAC CE format should be flexible or extensible;

- MAC CEs should remain easy for UE to process and assemble, even as their quantity increases in future releases.

- MAC CEs can be used as a versatile signaling mechanism for various purposes.

Proposal 4.  	MAC CEs in 6G should be designed to be a flexible and versatile signalling mechanism for various purposes, while remaining easy to process and assemble.

MAC PDU format

6G is expected to support much higher link rate than 5G, e.g. with a peak rate of 100Gbps or higher. This implies that the maximum size of a MAC PDU can increase to 500KB or more and a MAC PDU can include up to several hundreds of data packets. Achieving this kind of link rate necessitates more efficient processing of MAC PDUs. In the following, we discuss a few 5G MAC PDU designs that slow the processing of MAC PDUs and how they may be avoided.

In NR, each TB consists of code blocks (CB). The size of a CB is predefined and completely independent from upper-layer procedures. For example, it is possible that the boundary of a CB is in the middle of a MAC subPDU (or a RLC PDU). A TB is not delivered to the upper layers until all its CBs have been successfully received. One of the reasons for this restriction is that in some cases UE has to decode the MAC subheader of a MAC CE or a MAC subPDU before it can determine the starting point of the next MAC CE or MAC subPDU. If such two MAC CEs or MAC subPDUs are not located in the same CB, then it is not useful to deliver a decoded CB ahead of the other.

The drawback of this design can’t be more obvious – as long as a CB fails to decode, the whole TB has to go through HARQ retransmission, and PDUs in other CBs that have already been successfully received have to incur extra delay. This issue becomes more serious when TBs in 6G become larger.

#### Observation 6.  In NR, failure of a single code block (CB) can delay the delivery of PDUs in other CBs. This problem can be more serious in 6G with its much larger TBs.

One possible solution is to introduce some restriction on how MAC subPDUs and MAC CEs are multiplexed into a TB. For example, if each MAC CE or MAC subPDU must be fully contained within a single CB, then the processing between CBs is completely independent, i.e. once a CB is successfully decoded, it can be delivered to upper layers without waiting for other CBs. Or if it is preferred to hide CB from the MAC layer since it is a PHY-layer data unit, MAC layer can have its own data unit, denoted sub-TB. The length of a sub-TB can be either predefined or configured by network. As long as the aforementioned restriction for multiplexing MAC subPDUs and MAC CEs is applied to a sub-TB instead of a CB, the same benefits can be attained.

Another issue is that in NR, a receiver needs to process each MAC subheader to parse a TB (e.g. to determine the start of the next MAC CE or MAC subPDU). This can become a considerable burden for processing in 6G, as a single TB in 6G may contain up to several hundreds of MAC subPDUs.

A potential technique to speed up the processing is to append metadata about MAC CEs or MAC subPDUs at start of a TB (or a CB or a sub-TB). This metadata contains information about the subsequent MAC CEs or MAC subPDUs, e.g. number of them, their individual length, etc. Once the receiver (e.g. UE) decodes this metadata, it knows immediately how to parse the rest of the TB (or the CB or the sub-TB). To further simply processing, metadata can be generated separately for MAC CEs and MAC subPDUs.

#### Observation 7. 	Including metadata about MAC CEs or MAC subPDUs in a TB can help the receiver speed up processing of the TB.

#### Proposal 5.  	Study new MAC PDU formats (e.g. independent CBs, including metadata about MAC CEs or MAC subPDUs in a TB) that can enable faster processing at receiver.

# Conclusion

Based on the above analysis, we respectively request RAN2 to discuss and agree to the following proposals:

Baseline

Proposal 1.  	L2 supports the following minimum set of functionalities as a baseline:

| - Transfer of data - Ciphering and deciphering - Integrity protection & verification - Protocol Error detection and recovery, i.e., ARQ | - Segmentation & Reassembly - Header Compression & decompression - SDU marking, e.g., QFI - SDU discard | - Reordering and in-order delivery - Out-of-order delivery - Duplicate discarding |
| --- | --- | --- |

SDAP

Observation 1. 	In 5G, the SDAP RQoS mechanism was not widely deployed. It complicated hardware by mandating a per-packet RQoS check. Additionally, Control Plane may perform flow-to-bearer mapping.

Proposal 2.  	RQoS mechanism is not supported in 6G UP to reduce packet processing.

RACH

Observation 2. 	In NR, early indication of feature combinations through RACH partitioning leads to very inefficient use of RACH resources.

Observation 3. 	In NR, RACH resources do not adapt well to short-term variations in access load, often resulting in high level of contention and access failures.

#### Proposal 3.  	Study RACH designs that enable more efficient use of resources, e.g. avoid RACH partitions unless truly necessary, dynamic or on-demand allocation of RACH resources, etc.

MAC CE format

#### Observation 4. 	MAC CEs in 6G are expected to be used to send more diverse types of messages, some of which may demand higher reliability.

#### Observation 5. 	MAC CEs in 6G can be designed in a more structured way to enable efficient processing and meet the stricter timing requirements expected in 6G.

Proposal 4.  	MAC CEs in 6G should be designed to be a flexible and versatile signalling mechanism for various purposes, while remaining easy to process and assemble.

MAC PDU format

#### Observation 6.  In NR, failure of a single code block (CB) can delay the delivery of PDUs in other CBs. This problem can be more serious in 6G with its much larger TBs.

#### Observation 7. 	Including metadata about MAC CEs or MAC subPDUs in a TB can help the receiver speed up processing of the TB.

#### Proposal 5.  	Study new MAC PDU formats (e.g. independent CBs, including metadata about MAC CEs or MAC subPDUs in a TB) that can enable faster processing at receiver.

# References

[1] 3GPP TS 38.300 V15.21.0 Stage 2 (Release 15)