3GPP TSG-RAN WG2 Meeting #132	R2-2508049

Dallas, USA, Nov. 17th – 21st, 2025

Source:	vivo

Title:	Discussion on 6GR scheduling and retransmission

Agenda Item:	10.3.1.3

Document for:	Discussion and Decision

- Introduction

In RAN#109, Study on 6G Radio was updated in[1]:

Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

In RAN2#132, there are the following agreements regarding user plane[2]:

Agreements on UP

1.	RAN2 first discuss/agree on the required functionalities for the protocol stack, and only afterwards discuss which protocol layer supports each functionality.  Discussions should focus on standalone architecture, until told otherwise by plenary.  When discussing functionalities companies should focus on identifying the problems being address or new requirements.

2.  	UP design should aim to be hardware-processing friendly while keeping memory requirements low and minimize data movements across protocol layer.

3.	6G user plane should aim to reduce the radio and end-to-end latency for general services (including eMBB)

4.	Study potential benefits and standardized mechanisms for applications/service-awareness in the RAN (e.g. immersive communications, AI mobile traffic).  Understand the issues and shortcoming with the current NR QoS.   Aim to look at a general framework.

5.	Support at least the following scheduling schemes: dynamic grant and configured grant.  Further study configured grant.

6.	Study need for scheduling enhancements to address the SR/BSR/DSR latency

7.	Study how to improve L2 ARQ latency/efficiency in 6GR based on tight coordination with HARQ.  Study should identify enhancements needed for HARQ and L2 ARQ.

Based on the above study scope and the agreements, the following issues regarding uplink scheduling and retransmissions are discussed in this paper:

- Low latency uplink resource allocation

- Buffer Status and Delay Status Reporting

- Fast L2 Retransmission

- LCP enhancement

- Discussion

  - Low latency uplink resource allocation

In RAN2#131bis meeting, RAN2 agreed to support at least DG and CG for scheduling schemes, while details on CG is still FFS.

For dynamic scheduling, the gNB has to rely on the instantaneous buffer status or delay status report from the UE to assist the UL resource allocation. In 5G, both BSR and DSR can trigger SR procedure when there is no uplink resource for BSR/DSR transmission.  The SR resource is a periodic SR-PUCCH resource which can be configured per LCH. The uplink radio resource allocation delay comes from the below aspects, as shown in Fig. 1:

- D1: The delay from BSR/DSR trigger to SR-PUCCH occasion at UE side. In 5G, the minimum SR-PUCCH periodicity can be 2 OFDM-symbol, hence D1 can be very small with dense SR-PUCCH configuration in time domain.

- D2: The scheduling delay from SR reception to small uplink grant transmission to UE at gNB side;

- D3: The delay from UL grant reception end to BSR/DSR transmission using the received small UL grant at UE side;

- D4: The scheduling delay from BSR/DSR reception end to uplink grant transmission to UE at gNB side.

UE

gNB

SR TX

Small grant for BSR/DSR TX

BSR/DSR TX

Successive UL grant for data TX

D1

D2

D3

D4

BSR/DSR is triggered

Fig. 1 Uplink radio resource allocation delay for dynamic scheduling

- For dynamic scheduling, the uplink radio resource allocation delay comes from the SR/BSR/DSR transmission procedure.

To conquer the latency issue (i.e. D1, D2, D3 and D4 illustrated in Fig. 1) due to the SR/BSR/DSR procedure, 5G specified configured grant for uplink radio resource pre-allocation. Using the pre-allocated radio resources via configured grant, the UE can transmit uplink data as well as BSR/DSR without relying on SR transmission. There were two types of configured grant specified in 5G:

  - Type 1 configured grant: periodic uplink UL grants are allocated for a UE by gNB using RRC signaling only and the configured grant is effective once the UE has received the RRC msg. Type 1 configured grants are activated when the UE receives the RRC signaling, without additional activation signaling.

  - Type 2 configured grant: the static parameters for configured grant are provided via RRC signaling while the instantaneous parameters depending on channel quality and traffic variation are carried in the PDCCH to activate/reactivate Type 2 configured grant. Type 2 configured grants are effective when the UE receives PDCCH to activate/reactivate Type 2 configured grant. With Type 2 configured grant, the gNB can adjust some parameters via PDCCH if necessary.

- Type1 and Type2 configured grant were specified in 5G, with which the uplink radio resource allocation delay for SR/BSR/DSR transmission can be avoided.

Configured grant is especially useful for XR/immersive communication traffics with periodic data bursts. However, as the data burst size varies, the configured grants may be more than needed in case of small burst size.

To improve radio resource efficiency, some low latency uplink resource allocation mechanisms have been discussed in 5G, and CG recycling based on UTO-UCI (Unused Transmission Occasion – UCI) was introduced in 5GA R18. However, CG recycling in 5G requires the UE to always include UTO-UCI in the CG PUSCH when CG recycling is configured. In 6GR, we can further consider the feasibility to realize CG cycling based on BSR. If the UE can report the burst size via BSR, the network can determine the CG grants required for the transmission of the burst and determine the redundant CG grants that are recyclable. As the UE just need to transmit BSR once in a MAC PDU, the overhead and complexity can be considerably reduced compared to CG recycling based on UTO-UCI. Anyway, these further enhancements could be further studied.

Both Type 1/2 configured grant in 5G are supported as the starting point, and 6GR further study the related resource efficiency enhancements, for low latency uplink resource allocation.

As discussed above, semi-static uplink radio resource pre-allocation is useful for traffic with periodic bursts. But there would be jitter for burst arrival and burst size variation even for some XR/immersive traffics with periodical data burst, which could result in certain resource inefficiency. Another way that is worth to be studied is to enable UE to estimate the uplink burst arrival information and report the burst arrival information to gNB ahead of the burst arrival. The gNB can then pre-allocate uplink resources for the burst via dynamic scheduling. In such way, the delay due to the SR/BSR procedure can be conquered.

Burst arrival information estimation may comprise both burst arrival time and burst size estimations. Blind burst arrival estimation may result in uncertain deviation of arrival time and burst size. It can be studied to estimate the burst arrival based on the service awareness. For instance, currently, the RAN layers can only know the burst arrival when the burst is finally delivered from the application layer to RAN layers at UE side. However, before the burst has been prepared and delivered to RAN layers from application layer, the application layer will experience the traffic triggering and burst preparation based on the raw data input by the end user. If the application layer can provide some early information of burst to RAN layers before the burst is finally prepared and delivered to RAN layers, the RAN layers can initiate early uplink radio resource request during which the application layer can prepare the burst. Based on the early uplink radio resource request, the NW can pre-allocate the uplink resources based on the estimated burst arrival information. In such way, the uplink radio resource allocation delay due to traditional SR/BSR procedure can be largely mitigated. This could be especially useful for bursts which need long time encoding in application layer (e.g. video frames of XR/immersive communication). This process is shown in Fig. 2.

Time

Time

Application layer

RAN layers

Traffic trigger

Burst arrival info

Burst delivery

Early uplink resource request

Transmit the burst to network

Fig. 2 illustration of early uplink resource request based on burst arrival estimation at UE side.

- Relying on service awareness, the burst arrival information (e.g. arrival time, burst size) from application layer to RAN layers can be estimated at UE side.

RAN2 study the solution of early uplink radio resource request based on the burst arrival information (e.g. arrival time, burst size), which is estimated based on service awareness.

One more direction for low latency uplink resource allocation via dynamic scheduling is to study the feasibility to improve the traditional SR/BSR message design to enable early buffer status information delivery from UE to network.

- For 5G, as illustrated in Fig. 1, there are usually two rounds of uplink radio resource allocation during this procedure and the total delay comprises 4 parts: D1 - from burst arrival to next SR occasion; D2 - from SR RX to small grant TX at gNB side; D3 – from small grant RX end to BSR TX; D4 – from BSR TX end to UL grant RX end.

- For 6GR, one can consider to use UCI to convey the early buffer status information. With buffer status over UCI, the UE can report the buffer status without preceding scheduling request transmission upon a burst arrival. The network can then allocate enough radio resources for the burst directly based on the early buffer status information over UCI (as shown in Fig. 3). Compared to legacy SR/BSR procedure in 5G, there are only two delay parts for uplink radio resource allocation: D1 and D4, i.e. the uplink resource allocation round of small UL grant only for BSR TX can be avoided.

UE

gNB

Burst arrival

Early buffer status over UCI

UL grants for the burst TX

burst TX using the UL grants

D4

D1

Fig. 3 Illustration of uplink resource allocation delay reduction with early buffer status over UCI

- With the early buffer status over UCI, the network can pre-allocate proper amount of uplink resource for the coming data burst upon the UCI reception, which could save the delay to allocate a small UL grant for BSR TX.

To evaluate the performance of early buffer status report over UCI, some system simulations were executed and the related simulation results are provided in Table 1 below. In the study, three cases are simulated, wherein Case 1 is for dynamic uplink scheduling based on traditional SR/BSR procedure with a 3-ms delay (i.e. D1 + D2 + D3 + D4), Case 2 is for dynamic uplink scheduling base on BSR over CG with a 2.5-ms delay (i.e. D1 + D3 + D4) and Case 3 (i.e. the target case) is for early buffer status report over UCI (UCI @ PUCCH/PUSCH) with a 0.5-ms delay for dynamic uplink scheduling (D1 + D4). It can be observed that early BSR over UCI with dynamic scheduling can clearly improve the capacity of XR traffics with tight PDB. The detail simulation settings are provided in the appendix.

Table I. Performance evaluation on early buffer status report over UCI

| Traffic model | Cases | Capacity (#UEs/cell) |
| --- | --- | --- |
| XR traffic: UL video, 10Mbps, 10ms PDB | Case 1. DG with 3ms scheduling delay | 2.33 |
|  | Case 2. CG for BSR reports and DG for corresponding data scheduling (MAC CE based BSR delay is 2.5 ms) | 6.12 |
|  | Case 3. Early BSR report with UCI, and DG for corresponding data scheduling (UCI based BSR delay is 0.5 ms) | 8.79 |

6GR study the early buffer status report over UCI for low latency uplink radio resource allocation in case of dynamic uplink scheduling.

  - Buffer Status and Delay Status Reporting

In NR, Buffer Status Report (BSR) was introduced in R15 and enhanced BSR with finer granularity was introduced in R18. For supporting delay-aware handling at gNB, e.g. scheduling, single entry Delay Status Report (DSR) for reporting at most single pair of buffer status/remaining time per LCG and multiple entry DSR for reporting multiple pairs of buffer status/remaining times per LCG were introduced in R18 and R19 respectively. Similar functionalities need to be supported in 6GR, while the existing BSR/DSR procedure in 5G should be considered as a starting point for 6GR. As BSR, DSR and the related enhancements were introduced gradually in different releases, the UEs of earlier releases cannot benefit from BSR/DSR enhancements in NR. For 6GR, there is an opportunity to review and jointly consider these similar BSR/DSR functions in 6G Day-1as there is no restriction of backward compatibility for 6G Day-1:

- Support BSR in Day-1: As in NR, BSR should be supported as a mandatory capability of UE in 6GR Day-1. With even larger carrier bandwidth and better spectrum utilization efficiency for 6GR than NR, it is supposed that 6GR should be able to provide even higher uplink data rate than NR. This means that 6GR can serve traffics with even higher data rates. For one instance, immersive communication with ultra-HD video of high data rate could be one of the most promising services to be served in 6GR, which means the UL data volume of 6GR can be much larger than NR. Considering this, BSR for 6GR should be designed to report even larger data volume, with the trade-off between overhead and granularity considered.

- Support DSR in Day-1: For 6GR, to achieve better user experience, the delay budget for data transmission of delay sensitive service (e.g. XR) can be even shorter than that in NR, it is essential to support delay aware handling in order to fulfil the tight delay budget. To ensure the uplink data transmission latency management for 6GR is at least not worse than 5G, delay aware handling should be supported in the first release of 6GR. Considering this, DSR function should be supported in the first release of 6GR.

- Consider harmonization of BSR and DSR: In NR, BSR is triggered based on the data arrival/availability and used to report the whole buffer status per LCG, while DSR is triggered based on remaining time threshold and used to report the buffer status of urgent data and the associated remaining times per LCG. Currently, BSR procedure and DSR procedure are relatively independent in NR specification, for instance,

  - Both pending BSR and pending DSR can trigger scheduling request when there is no uplink grant for BSR and DSR transmission, respectively;

  - BSR MAC CE and DSR MAC CE are generated and transmitted separately;

  - BSR cancellation and DSR cancellation are operated separately.

6GR gives us an opportunity to rethink about these two functions. On issue is whether these two functions should be fully isolated as in NR. However, a large portion of the information contained in BSR and DSR is overlapping, especially for the traffic with critical latency requirements, which introduces an additional burden for the UE, in NR. We may consider a harmonized design for BSR and DSR in 6GR. For example, the BSR could carry delay information and be reported as such, or the DSR could include more data volume information for non-delay-critical data. Anyway, such kinds of detailed designs could be further studied.

5G BSR/DSR design should be taken as the starting point for uplink radio resource allocation for 6GR, and harmonized design of BSR and DSR should be considered, from Day-1.

  - Fast L2 Retransmission

In last RAN2 meeting, L2 ARQ latency and efficiency in 6GR has been briefly discussed. It has been agreed to study the ARQ enhancement based on tight coordination with HARQ. Current ARQ retransmission is based on timer triggered status reporting mechanism, which is independent with HARQ transmission. According to current NR RLC parameters setting in the field, the ARQ retransmission normally takes 30ms-50ms, and up to hundreds of ms in the worst case. This can hardly meet the latency requirement for the emerging new services e.g. immersive communication or GenAI services. In NR R19, remaining-time-based RLC retransmission/polling were introduced. However, such add-on solution may have some design limitation e.g. potential radio resource waste for “blind retransmission” and extra status report.

In 6GR, we have to enhance the ARQ transmission to consider of HARQ transmission status information in order to fast trigger ARQ retransmission without utilizing much unnecessary radio resources. As a result, we first investigate current limitation of HARQ information utilization for ARQ in detail. Then the potential enhancements could be concluded.

DL Data Transmission:

For DL transmission, HARQ feedback via UCI is used for RAN node to schedule HARQ retransmission or new transmission. Due to NACK to ACK error, RAN node would trigger improper HARQ new transmission with NDI toggled. UE cannot trigger fast RLC level ARQ status report due to lack of information of RLC PDU to TB mapping in the receiving side even UE can detect such HARQ error. The limitation is illustrated in Fig. 4.

Fig. 4 NACK-to-ACK error for DL data transmission

Two straight forward enhancements could be raised from above analysis. Enhance the reliability of UL HARQ feedback transmission could help to reduce NACK to ACK error. However, whether it could completely solve the problem with no residual error may depend on RAN1 to evaluate the performance and cost of corresponding solution. On the other hand, UE could inform RAN node once it detects potential HARQ to ARQ error, e.g. HARQ new transmission scheduling is received on certain HARQ process after HARQ NACK is sent. This indication could be considered as potential fast ARQ status reporting based on MAC transport block transmission in HARQ to further trigger RAN node ARQ retransmission if there are any AMD PDU in the corresponding TB. Since RAN node as the transmitter could easily identify mapping between the RLC PDU and MAC TB. Fig. 5 illustrates the above potential enhancements and corresponding gain of fast ARQ status indication. Such indication could send over UCI or UL MAC CE, and is only triggered when potential NACK to ACK error detected, so the radio resource overhead could be considered as relatively low.

Fig. 5 fast DL ARQ retransmission with enhanced fast ARQ status based on HARQ transmission

UL Data Transmission:

For UL transmission, RAN node may stop scheduling HARQ retransmission and begin to schedule HARQ new transmission due to link adaptation. However, UE cannot identify whether the previous HARQ transmission is successful or not according to current NR design. RAN node also need to wait the RLC timer expire to trigger the status report due to as the receiver, RAN node is lack of information of RLC PDU to TB mapping even it can detect the HARQ error. The above analyzed limitation is illustrated in Fig. 6.

Fig. 6 UL HARQ retransmission abandon for MCS adaption

Similar as DL transmission, once RAN node decide to give up current HARQ (re-)transmission, an DL indication of fast ARQ status based on MAC transport block transmission in HARQ could send to UE to trigger potential fast ARQ retransmission since UE is the transmitter that has the knowledge of whether and which AMD PDU is included. The enhancement for UL data transmission is illustrated in Fig. 7.

Fig. 7 fast UL ARQ retransmission with enhanced fast ARQ status based on HARQ transmission

ARQ Retransmission Data:

Once fast ARQ retransmission is triggered, current RLC based re-segmentation and retransmission could be re-used. Since the enhanced fast ARQ status discussed above is based on MAC transport block level, ARQ retransmission data rebuilding including re-segmentation or/and multiplexing also on MAC sub PDU level in MAC could be considered. It could reduce interaction between RLC and MAC layer to further speed up ARQ retransmission and simplify L2 packet processing. Fig. 8 provides a general illustration on MAC data rebuilding. Transmitter MAC could fast generate new MAC TB for ARQ retransmission according new grant.

Fig. 8 MAC data rebuild for ARQ retransmission

With above potential enhancements, it is believed ARQ retransmission can be fast triggered with reasonable radio resource overhead that could help to meet performance requirements for hyper reliable and low-latency communication. Moreover, it enables hardware friendly design to save UE buffer occupation and improve data processing with reasonable L2 header overhead.

Support to introduce fast ARQ status indication based on HARQ transmission for both uplink and downlink.

Study solutions to enable MAC data rebuilding including re-segmentation and multiplexing to further speed up ARQ retransmission and simplify L2 packet processing.

  - LCP enhancement

In 5G, LCP procedure is used to configure the uplink radio resource allocation among LCHs. Each LCH has two sets of LCP parameters, if configured:

- LCP restriction parameters, such as allowedSCS-List, maxPUSCH-Duration, configuredGrantType1Allowed, allowedServingCells, allowedCG-List, allowedPHY-PriorityIndex and allowedHARQ-mode, are used for LCH channel selection for a UL grant. Only the data of the selected LCHs in relation to a UL grant are allowed to be transmitted using the UL grant.

- Radio resource amount calculation parameters: priority, prioritisedBitRate (PBR) and bucketSizeDuration (BSD), which are used to evaluate the maximum amount of radio resources that are prioritized to be allocated for this LCH according to the priority descending order of all selected LCHs for a UL grant in the first round of radio resource allocation. If there are still remaining radio resources after the first round of radio resource allocation, the remaining radio resources are allocated among selected LCHs for the UL grant based only the priority descending order of all selected LCHs without considering the rest radio resource amount calculation parameters.

In R19, LCH priority adaptation was introduced to improve timely radio resource allocation for urgent data of an LCH if configured, i.e. when the remaining time for a PDU/PDU set is below a preconfigured threshold, the UE can adapt the LCH to a higher priority (i.e. additionalPriority). In such way, an LCH with urgent data can be allocated more radio resource in time.

However, there are still the following hard limit for timely resource allocation for an LCH with urgent data in 5G:

- Case 1: When there is only UL grant(s) that does not fulfil the LCP restriction parameters of the LCH, the UE is not allowed to use these UL grant(s) for data transmission of the LCH. The UE may have to discard the urgent packets in the LCH at the end when the delay budgets of these packets exhaust. If the UE can temporarily use this UL grant(s) for LCH, there is still considerable probability for successful transmission of the urgent data.

- Case 2: The Bj calculated based on the preconfigured PBR and BSD acts as a hard limit for radio resource allocation for the LCH. When the Bj of LCH 1 reduces to 0 and there are still remaining resources in the first round of resource allocation of a UL grant, LCH 2 that has lower priority than LCH 1 but positive Bj will have higher priority for resource allocation until the Bj of LCH 2 reaches to 0. Such mechanism is used to ensure the fairness between LCHs. However, as the burst size varies and the preconfigured LCP parameters such as PBR and BSD for the LCH may be not always optimal. RAN2 can study the feasibility to allow the UE to loosen the Bj limit for urgent data transmission of an LCH without starvation of the traffics mapped on low priority channels.

In 6GR, in addition to LCH priority adaption according to the remaining time of packets, RAN2 should study the LCH enhancements more widely for even better QoS guarantee of delay sensitive services such as XR/immersive communications, e.g. LCP restriction relaxation and Bj adjustment.

Besides, the existing 5G NR design largely bases the LCH mapping and prioritization decisions on QoS parameters, such as delay budget, packet error rate, etc. This QoS-driven approach has served well for traditional eMBB, URLLC, and mMTC services. However, as the mobile ecosystem evolves toward 6G, application diversity and data heterogeneity are expected to grow significantly. Future 6G services, e.g. multi-modality immersive communication, mobile AI, etc., introduce traffic characteristics that are not purely defined by conventional QoS dimensions, for example: packet importance, inter-flow or inter-packet dependency/correlation.

These aspects directly affect how data should be prioritized, and scheduled at the MAC layer, yet they are not explicitly considered in the current 5G LCH mapping and LCP mechanisms.

Even in 5GA Rel-19, as discussed above, LCP adaptation mainly consider PDU or PDU set delay requirements. However, experience from late 5G releases (e.g., Rel-18/19 XR enhancements) has shown that existing QoS alone cannot consider the full diversity of traffic requirements. As a result, while 5G provides mechanisms for delay-based prioritization, it lacks the capability to express importance and inter-QoS flow/packet correlation, both of which are essential to achieve optimal scheduling efficiency and UE experience in 6G.

In 6GR, it is expected that data streams will be characterized by multi-dimensional traffic characteristics, including not only latency and reliability, but also importance, correlation, etc. Accordingly, the LCH and LCP design in 6G should consider these characteristics.

LCP design in 5G is taken as a starting point, RAN2 further study LCP enhancements considering the traffic characteristics, e.g. delay, importance, inter-QoS flow/packet correlation.

- Conclusion

In this paper, we have discussed 6GR scheduling and ARQ retransmission mechanism and have the following observations and proposals:

For Low latency uplink resource allocation:

Observation 1	For dynamic scheduling, the uplink radio resource allocation delay comes from the SR/BSR/DSR transmission procedure.

Observation 2	Type1 and Type2 configured grant were specified in 5G, with which the uplink radio resource allocation delay for SR/BSR/DSR transmission can be avoided.

Proposal 1:	Both Type 1/2 configured grant in 5G are supported as the starting point, and 6GR further study the related resource efficiency enhancements, for low latency uplink resource allocation.

Observation 3	Relying on service awareness, the burst arrival information (e.g. arrival time, burst size) from application layer to RAN layers can be estimated at UE side.

Proposal 2:	RAN2 study the solution of early uplink radio resource request based on the burst arrival information (e.g. arrival time, burst size), which is estimated based on service awareness.

Observation 4	With the early buffer status over UCI, the network can pre-allocate proper amount of uplink resource for the coming data burst upon the UCI reception, which could save the delay to allocate a small UL grant for BSR TX.

Proposal 3:	6GR study the early buffer status report over UCI for low latency uplink radio resource allocation in case of dynamic uplink scheduling.

For Buffer Status and Delay Status Reporting:

Proposal 4:	5G BSR/DSR design should be taken as the starting point for uplink radio resource allocation for 6GR, and harmonized design of BSR and DSR should be considered, from Day-1.

For fast L2 Retransmission:

Proposal 5:	Support to introduce fast ARQ status indication based on HARQ transmission for both uplink and downlink.

Proposal 6:	Study solutions to enable MAC data rebuilding including re-segmentation and multiplexing to further speed up ARQ retransmission and simplify L2 packet processing.

For LCP enhancement:

Proposal 7:	LCP design in 5G is taken as a starting point, RAN2 further study LCP enhancements considering the traffic characteristics, e.g. delay, importance, inter-QoS flow/packet correlation.

- Appendix: simulation settings for early BSR over UCI

Table I. Simulation assumptions for FR1 Indoor Hotspot scenario

| Parameter | Value |
| --- | --- |
| Scenarios | Indoor Hotspot, 12 nodes in 50 m x 120 m |
| Channel model | InH |
| Carrier frequency | 4GHz |
| Bandwidth | 100 MHz, 1.72% Guard Band |
| Subcarrier spacing | 30 KHz |
| Frame structure | DDDSU (S: 10D:2G:2U), DDDUU |
| BS Antennas (M,N,P,Mg,Ng;Mp,Np) | For 32T: (4,4,2,1,1;4,4), (dH,dV) = (0.5, 0.5)λ |
| UE Antennas (M,N,P,Mg,Ng;Mp,Np) | 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,1/2,2,1,1;1,1/2), (dH, dV) = (0.5, N/A) λ |
| BS antenna pattern | Ceiling-mount pattern, 5 dBi |
| UE antenna pattern | Omnidirectional, 0 dBi |
| BS Power | 24 dBm per 20MHz |
| UE max Power | 23 dBm |
| UE Power | Max Tx power: 23 dBm, (P0 = -80, alpha = 0.8) |
| ISD | 20 m |
| BS height | 3 m |
| UE height | 1.5 m |
| Noise Figure | BS:5 dB, UE:9 dB |
| Max MCS | 256QAM |
| Device deployment | 100% indoor |
| Down-tilt | 90 degrees |
| BS receiver | MMSE-IRC |
| UE receiver | MMSE-IRC |
| Channel estimation | Realistic |
| Target BLER | 10% |
| UE speed | 3 km/h |

Table II.  UL traffic models of Cloud Gaming (CG) and XR

| Traffic model | Mean packet size (Bytes) | STD of packet sizes (Bytes) | Min packet size (Bytes) | Max packet size (Bytes) | Packet arrival interval (ms) | PDB (ms) | Jitter |
| --- | --- | --- | --- | --- | --- | --- | --- |
| pose/control stream | 100 | 0 | 100 | 100 | 4 | 10 | No |
| scene/video/data/audio stream (10Mbps) | 20833 | 2187 | 10416 | 31249 | 16.67 | 10, 30 | No |

- Reference

- RP-252912	Revised SID: Study on 6G Radio

- Chair notes, RAN2#131bis