3GPP TSG-RAN WG2 Meeting #132	                                                             R2-2508101

Dallas, USA, Nov. 17th - 21st, 2025

Source:	CATT, Turkcell

Title:	Functionality management for RRM measurement prediction

Agenda Item:	9.3.2

Document for:	Discussion and Decision

- Introduction

For RRM measurement prediction of UE sided model, following objective are specified [1]:

- Specify support for RRM measurement prediction and measurement event prediction for UE sided models [RAN2]: - Temporal domain prediction case A and case B, and Frequency domain prediction for co-located cases are supported. - L3 Cell level predication is supported Note 1: For UE sided model, spatial domain prediction is not supported. Note 2: For UE sided model, L3 beam level prediction is not supported. - For both RRM measurement prediction and measurement event prediction, specify signalling and protocol aspects to enable LCM functionality management for UE sided model including [RAN2]: - UE capability request and response procedure - Applicability report for both full and partial configuration approach - Inference configuration and inference report - Performance monitoring performed in UE or network side, based on which network can manage UE sided functionality - UE sided data collection - For measurement event prediction: - Measurement event A1~A6 are in the scope in this release Note3: Data transfer and model delivery for UE sided model are out of scope of this WI. And model related choices can be up to UE’s implementation. Note5: The above objectives are to be based on the LCM framework for Rel-19 AI/ML for NR air interface WI as much as possible.

In this contribution, we discuss the RRM measurement prediction of UE sided model.

- Discussion

In this section, we discuss the LCM functionality management of RRM measurement prediction of UE sided model, including:

- UE capability request and response procedure

- Inference configuration and inference report

  - UE capability

In Rel-19 AI/ML PHY, the LCM procedure in Figure 1 has been agreed.

Figure 1: LCM procedure of AI/ML PHY

Based on the AI/ML PHY procedure, the network determines a UE's supported AI/ML functionalities prior to sending the inference configuration. Network can send UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities. And then UE can send UECapablityInformation message to network, containing supported functionalities at the UE side. For AI/ML mobility, a similar procedure should also be supported.

Proposal 1: For AI/ML mobility, network can send UECapabilityEnqiry message to initiate the procedure to a UE reporting its supported AI/ML based RRM measurement prediction capability. And then UE can send UECapablityInformation message to network, containing supported AI/ML based RRM measurement prediction capability at the UE side.

In addition, for Rel-19 AI/ML PHY, different use cases such as BM, CSI prediction, and positioning involve varied UE capabilities. Moreover, even within a single use case, multiple capabilities may exist. For example, in BM, UE capabilities can be categorized into UE-side beam prediction for BM Case1 for inference, UE-side beam prediction for BM Case1 with predicted RSRP for inference, UE-side beam prediction for BM Case2 for inference and UE-side beam prediction for BM-Case2 with predicted RSRP for inference. As for AI/ML mobility, we support use cases including temporal domain prediction case A and case B, and frequency domain prediction for co-located. And each use case considers the following three cell-level RRM measurement prediction:

-	RRM sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated;

-	RRM sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s);

-	RRM sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s).

Therefore, the granularity of UE capability reporting for AI/ML mobility still requires careful consideration.

Proposal 2: RAN2 discusses the granularity of UE capability for AI/ML mobility.

  - Inference

According to TR 38.744 [2], the existing RRM measurement configuration and reporting framework in RRC layer is baseline for inference configuration and report.

Existing RRM measurement configuration and reporting framework in RRC layer is baseline for inference configuration and report. When a full inference configuration is received, UE shall maintain it until it is released by network explicitly.

Proposal 3: RAN2 confirms that the existing RRM measurement configuration and reporting framework in RRC layer is baseline for inference configuration and report.

Inference configuration

According to TR 38.744 [2], inference parameters used for inference and assessing applicability include:

Following inference parameters can be configured to UE for inference and assessing applicability: - PW length for intra-frequency temporal domain case A; - Measured frequency carrier and predicted frequency carrier information for inter-frequency prediction; - Parameter for intra-frequency temporal domain case B to indicate the timing of network’s SSB configuration (but not necessarily the timing for UE to perform or skip measurement); - Optional list of cells for intra-frequency temporal domain case A, for which network expects inference report (if available); - Optional Beam pattern e.g. to save SSB transmission for intra-frequency spatial domain prediction; - Optional associated ID.

Based on the WID, we support the following use case: temporal domain prediction case A and case B, and frequency domain prediction for co-located. In Rel-19, model generalization shows strong performance across UE speeds, frequencies, and cell configurations. Therefore, we need to discuss whether the associated ID is still necessary.

Proposal 4: RAN2 discusses whether the associated ID is required.

For temporal domain prediction case A, we propose:

Proposal 5: RAN2 confirms that following inference parameters for temporal domain prediction case A can be configured to UE for inference and assessing applicability:

-	PW length;

-	Optional list of cells, for which network expects inference report (if available).

For temporal domain prediction case B, in our view, the predicted cell list can be defaulted to the detectable cells within the measurement object. Additionally, the SSB transmission configuration (i.e., the skip pattern) should be indicated to the UE. Therefore, we propose:

Proposal 6: RAN2 confirms that following inference parameters for temporal domain prediction case B can be configured to UE for inference and assessing applicability:

-	Parameter for intra-frequency temporal domain case B to indicate the timing of network’s SSB configuration (but not necessarily the timing for UE to perform or skip measurement).

For inter frequency domain prediction, the focus should be on prediction within the frequency domain itself, without involving the temporal domain. Specifically, this means using frequency f1 at a given moment to predict frequency f2 at the same moment. Regarding the predicted cell list, the detectable cells in the measurement Object corresponding to the predicted frequency can be taken as the default. Therefore, we propose:

Proposal 7: RAN2 confirms that following inference parameters for inter frequency domain prediction can be configured to UE for inference and assessing applicability:

-	Measured frequency carrier and predicted frequency carrier information for inter-frequency prediction.

Based on the WID, both full and partial inference configurations are supported. In our view, the inference parameters used for inference and assessing applicability of proposals 5-7 can be configured at least in the full inference configuration.

Proposal 8: The inference parameters used for inference and assessing applicability of proposals 5-7 can be configured in the full inference configuration.

Condition when to perform inference

In SI discussion [3], when the UE can perform inference was discussed, but no consensus was reached. In the following, we analyze the report procedure of predicted measurement report for FR2-FR2 Intra-frequency temporal domain case A, FR1-FR1 Intra-frequency temporal domain case B and inter-frequency domain respectively.

2.2.2.1	FR2-FR2 Intra-frequency temporal domain case A

In intra-frequency temporal domain case A, continuous measurement results in PW are predicted by continuous historical measurement result(s) in OW and the measurement results in the future can be predicted to improve the handover performance.

Regarding AI/ML mobility, there are 4 options on when to trigger inference:

- Option 1: the inference is performed periodically as the real measurement result is obtained as shown in Figure 2;

Figure 2: Inference is performed periodically

- Option 2: the inference is performed when actual measurement event is triggered. For example, when the A3 entry condition is satisfied by actual measurement result, the inference is performed. And the prediction on the future result is performed which is shown in Figure 3.

Figure 3: Inference is performed when measurement event is triggered by actual measurement

- Option 3: the inference is performed when the moves to the cells edge, e.g. the serving cell RSRP is below the one threshold considering the UE performs handover at the cell edge.

Figure 4: Inference is performed when the UE moves to the cell edge

- Option 4: the inference is performed up to the UE implementation.

In BM of AI/ML PHY, AI/ML functionalities may be activated autonomously when periodic CSI report configuration is configured for the UE or AI/ML functionality activation follows the existing CSI measurement and reporting, i.e., semi-persistent reporting can be activated by MAC CE/DCI and aperiodic CSI reporting can be activated by DCI. When the AI/ML functionality is activated, it is already enabled for performing inference. Hence, there is no other extra condition for the UE to decide when to perform inference in BM.

For FR2-FR2 Intra-frequency temporal domain case A in AI/ML mobility, it is studied to to improve the handover performance (e.g., Ping-pong HO, HOF/RLF, short time of stay, Handover interruption). Regarding handover, it is typically happens at the cell edge, e.g., the channel quality of the neighbour cell is higher than that of the serving cell or the channel quality of the serving cell is below one threshold. Hence, inference performed under some conditions decreases the inference frequency which is beneficial on reducing of AI computation, storage and power consumption as well. Therefore, it is proposed that:

Proposal 9: For FR2-FR2 Intra-frequency temporal domain case A, inference should be performed conditionally, for example, when an actual measurement event is triggered or when the UE moves to the cell edge.

2.2.2.2	FR1-FR1 intra-frequency temporal domain case B/inter-frequency domain

Regarding FR1-FR1 Intra-frequency temporal domain case B/inter-frequency domain, they are studied to reduce measurement efforts in temporal, frequency domain by using predicted measurements.

For FR1-FR1 intra-frequency temporal domain case B, the measurement is skipped to reduce the measurement overhead and measurement results in PW are predicted by historical measurement result(s) in OW. The UE needs to predict the measurement result for the skipped time instance based on the network configuration.

For inter-frequency, the measurement result of one frequency are predicted by the actual measurement result of another frequency to reduce the measurement overhead of different frequencies.

Based on the analysis above, considering no dynamic signaling (e.g. L1/2) is required for inference configuration.  It is proposed that UE performs inference when inference configuration is configured for the UE and no other conditions are needed for FR1-FR1 Intra-frequency temporal domain case B/inter-frequency domain.

Proposal 10: For FR1-FR1 intra-frequency temporal domain case B/inter-frequency domain, UE can perform inference when inference configuration is configured for the UE and no other conditions are needed.

Inference Reporting

According to TR 38.744 [2], the content about inference report is as follows:

UE can be configured with periodic or event triggered reporting of predicted and/or actual measurement result(s). For each reported cell in the measurementReport message: - For intra-frequency temporal domain case A, one or more instances of predicted measurement result(s) in PW can be reported; - For intra-frequency temporal domain case B, the latest actual or predicted measurement result can be reported; - For inter-frequency prediction, the latest predicted measurement result is reported.

For inference reporting, the aforementioned description should be applied as the basis, so we propose:

Proposal 11: RAN2 confirms that UE can be configured with periodic or event triggered reporting of predicted and/or actual measurement result(s). For each predicted cell in the measurementReport message:

- For intra-frequency temporal domain case A, one or more instances of predicted measurement result(s) in PW can be reported,

- For intra-frequency temporal domain case B, the latest actual or predicted measurement result can be reported,

- For inter-frequency prediction, the latest predicted measurement result is reported.

- Conclusion

In this contribution, we provide our views on functionality management for RRM measurement prediction, and the proposals are summarized as follows:

UE capability

Proposal 1: For AI/ML mobility, network can send UECapabilityEnqiry message to initiate the procedure to a UE reporting its supported AI/ML based RRM measurement prediction capability. And then UE can send UECapablityInformation message to network, containing supported AI/ML based RRM measurement prediction capability at the UE side.

Proposal 2: RAN2 discusses the granularity of UE capability for AI/ML mobility.

Inference configuration and reporting

Proposal 3: RAN2 confirms that the existing RRM measurement configuration and reporting framework in RRC layer is baseline for inference configuration and report.

Proposal 4: RAN2 discusses whether the associated ID is required.

Proposal 5: RAN2 confirms that following inference parameters for temporal domain prediction case A can be configured to UE for inference and assessing applicability:

-	PW length;

-	Optional list of cells, for which network expects inference report (if available).

Proposal 6: RAN2 confirms that following inference parameters for temporal domain prediction case B can be configured to UE for inference and assessing applicability:

-	Parameter for intra-frequency temporal domain case B to indicate the timing of network’s SSB configuration (but not necessarily the timing for UE to perform or skip measurement).

Proposal 7: RAN2 confirms that following inference parameters for inter frequency domain prediction can be configured to UE for inference and assessing applicability:

-	Measured frequency carrier and predicted frequency carrier information for inter-frequency prediction.

Proposal 8: The inference parameters used for inference and assessing applicability of proposals 5-7 can be configured in the full inference configuration.

Proposal 9: For FR2-FR2 Intra-frequency temporal domain case A, inference should be performed conditionally, for example, when an actual measurement event is triggered or when the UE moves to the cell edge.

Proposal 10: For FR1-FR1 intra-frequency temporal domain case B/inter-frequency domain, UE can perform inference when inference configuration is configured for the UE and no other conditions are needed.

Proposal 11: RAN2 confirms that UE can be configured with periodic or event triggered reporting of predicted and/or actual measurement result(s). For each predicted cell in the measurementReport message:

- For intra-frequency temporal domain case A, one or more instances of predicted measurement result(s) in PW can be reported,

- For intra-frequency temporal domain case B, the latest actual or predicted measurement result can be reported,

- For inter-frequency prediction, the latest predicted measurement result is reported.

- References

- RP-252899 Revised WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

- TR38.744, Study on Artificial Intelligence (AI)/Machine Learning (ML) for Mobility in NR (Release 19), 3GPP

- R2-2503302, RAN2#129bis Meeting Report