3GPP TSG-RAN WG2 Meeting #132	R2-2508124

Dallas, U.S.A., 17th – 21st November 2025

Agenda Item:	10.3.1.1

Source:	MediaTek Inc.

Title:	On essential functionality for 6G data transfer

Document for:	Discussion and decision

# Introduction

Based on the agreement below from the previous meeting [1], this document discusses what functionality is needed of the 6G stack for the efficient transfer of data.

RAN2 first discuss/agree on the required functionalities for the protocol stack, and only afterwards discuss which protocol layer supports each functionality. Discussions should focus on standalone architecture, until told otherwise by plenary. When discussing functionalities companies should focus on identifying the problems being address or new requirements.

# Discussion

## Essential functionality for data transfer

We list below functionality needed of 6G user plane stack to enable efficient data transfer. For each function, we highlight why it is essential for data transfer and considerations for its design to cater to 6G data transfer requirements.

- Uplink resource acquisition: This functionality is essential for the initiation of UL data transfer. In LTE and NR, this was achieved by BSR, SR and RACH mechanisms which experience significant delays. For 6G, this functionality needs to be redesigned to minimise its latency. We expand on how to achieve this latency reduction in our companion paper [2].

- Reliable data transfer: This functionality is essential to ensure that data reliably reaches the receiver over a lossy physical channel. In LTE and NR, this was achieved by HARQ, with ARQ running on top. ARQ is rather complex functionality that suffers from significant delays and, for 6G, needs to be redesigned to minimise its latency and processing impact. We expand further on how to do so in our companion paper [2].

- Data recovery at mobility: Reliable data transfer should also work as the UE moves across the network and switches between RAN nodes. In LTE and NR, this was achieved by lossless handover functionality. For 6G, we propose to ensure that such functionality is retained.

- Security of data and control information: Security is a basic requirement of 6G data transfer. The security algorithms to use need to follow SA3 conclusions to ensure that they meet 6G requirements such as quantum safe operation. RAN2 needs to design the placement of security functionality in the data pipeline. In addition to what we have for NR, it is highly likely that SA3’s response to our LS [3] would be that we would need to secure L2 control information such as that carried in MAC CEs in NR. This additional security consideration needs to be carefully designed given the time-critical nature of some of this information.

- Data handling differentiation in the RAN: This functionality is essential to provide differentiated data handling in the RAN to meet QoS requirements. In NR this was provided at a radio bearer level which is at a coarser granularity than QoS flows (16 DRBs vs 64 QoS flows) to limit RAN processing requirements. In the field we never see more than 4 DRBs used in NR. For 6G, we propose to continue using such a coarse data differentiation concept as radio bearers from NR to keep processing requirements within reason, and do not see a need to expand the number of radio bearers.

- Multiplexing of radio bearers into a physical layer transmission: This is functionality that multiplexes data with different QoS requirements in the RAN into a physical layer transmission. In NR, this was achieved using the Logical Channel Prioritisation (LCP) function in MAC. Given that we require differentiated data handling, we also need such functionality to map data with differing handling requirements to an uplink transmission. Care needs to be taken during the design of this functionality in 6G, to keep its processing requirements in check.

- Segmentation and reassembly of data: When whole IP data cannot fit into a physical layer transmission resource, segmentation of IP data at the transmitter and reassembly of segmented information at the receiver is needed to maximise the use of physical layer resources. While at low data rates (e.g. due to congestion, or UE being at cell-edge) maximising the use of scarce physical resources is of paramount importance, the same is not true at high data rates. At high data rates, segmentation and reassembly can lead to increased memory requirements and processing overheads due to the need of handling an excessive number of segments. Care needs to be taken when designing this functionality for 6G to ensure that it is only used when absolutely necessary.

- In order delivery: Some transport layer protocols such as TCP react poorly to out of order data by triggering congestion control. This would lead to a sub-par end-user experience despite a good underlying communications link. There is therefore a need to ensure in-order delivery of data in 6G. In NR, reordering is done at a DRB granularity where a single DRB is typically configured to handle all Internet traffic in a UE. This leads to significant head-of-line blocking delays for all traffic of a UE. For 6G, there is a need to redesign reordering functionality to keep head-of-line blocking to a minimum, by limiting the impact of missing data to the specific application or service that the data corresponds to. We expand further on how to do so in our companion paper [4].

- Compression for low bitrate radio bearers: Header compression mechanisms such as Robust Header compression (RoHC) and Ethernet Header compression (EHC) as well as data compression is present in NR to transfer low bitrate applications such as voice efficiently. For 6G, we expect similar services need to be supported and therefore propose to retain compression functionality from NR for low bitrate radio bearers.

- Data discard: Dropping data at the transmitter end is necessary for active queue management (AQM) purposes and to tackle buffer bloat due to stale data in case of latency sensitive applications. As part of 6G design, we need to ensure that the transmitter can discard data as needed, and the receiver is able to cope with discarded data.

Figure 1: An overview of essential 6G user plane functionality

Proposal 1: 6G user plane is designed with the following functionality:

- Fast uplink resource acquisition

- Simplified reliable data transfer

- Data recovery at mobility

- Security of data, RRC and some L2 control information

- Data handling differentiation in the RAN using radio bearers

- Multiplexing of radio bearers into a physical layer transmission

- Segmentation and reassembly of data, while avoiding excessive memory requirements

- In-order delivery for applications

- Compression for low bitrate radio bearers

- Data discard

## Considerations for other functionality

One of the differences of NR compared to LTE was its ability to dynamically switch the mapping of QoS flows to radio bearers using reflective operation, where the uplink mapping would change if the corresponding downlink mapping also changed. The purported advantage of such a mechanism is that it would be fast with limited overhead. While the signalling overhead is may be limited, the cost of this design choice was significant hardware processing overhead at the UE, associated with the per DL packet inspection forced by the design (as also raised in [5]).

UP design should aim to be hardware-processing friendly while keeping memory requirements low and minimize data movements across protocol layer.

This forced processing cost is not in proportion to the value it delivers, given that reflective QoS has not taken off in the field as highlighted in papers [5][6]. To keep the 6G design envelope within reasonable constraints as agreed in the previous meeting [1], we propose that we use control plane signalling to control the mapping of QoS flows to radio bearers instead of reflective mapping mechanisms as in NR.

Observation 1: Reflective QoS functionality results in significant processing overhead that is not proportional to its purported benefits.

Proposal 2: QoS flow to bearer mapping is only updated using control plane signalling, i.e. reflective QoS functionality from NR is not considered for 6G.

Another topic that has been raised by several companies in the previous meeting is the introduction of concatenation to reduce implementation complexity and overheads. Concatenation existed in the RLC layer of the LTE stack, and it resulted in a variable sized RLC header structure with varying bit positions, that was unsuitable for hardware-friendly operations.

Observation 2: Data concatenation can lead to variable-sized headers with varying bit positions that are not hardware-processing friendly.

Another consideration for concatenation is that it would result in larger packets that are more likely to get segmented into multiple segments when passed down to lower layers. This would require larger buffers than in NR to handle the various segments of the concatenated SDU. Increased segmentation would also lead to additional delays in reassembling the concatenated PDU at the receiver. Unless these above considerations are addressed, we propose to retain the NR principle of handling data at a per-packet granularity.

Observation 3: Data concatenation can lead to increased segment handling and reassembly delays

Proposal 3: As in NR, the unit of data handling in the 6G user plane stack remains at a per-packet granularity.

# Conclusion

In this contribution we propose that the following functionality is considered essential for 6G data transfer.

Proposal 1: 6G user plane is designed with the following functionality:

- Fast uplink resource acquisition

- Simplified reliable data transfer

- Data recovery at mobility

- Security of data, RRC and some L2 control information

- Data handling differentiation in the RAN using radio bearers

- Multiplexing of radio bearers into a physical layer transmission

- Segmentation and reassembly of data, while avoiding excessive memory requirements

- In-order delivery for applications

- Compression for low bitrate radio bearers

- Data discard

We also propose these additional considerations based on the following observations.

Observation 1: Reflective QoS functionality results in significant processing overhead that is not proportional to its purported benefits.

Proposal 2: QoS flow to bearer mapping is only updated using control plane signalling, i.e. reflective QoS functionality from NR is not considered for 6G.

Observation 2: Data concatenation can lead to variable-sized headers with varying bit positions that are not hardware-processing friendly.

Observation 3: Data concatenation can lead to increased segment handling and reassembly delays

Proposal 3: As in NR, the unit of data handling in the 6G user plane stack remains at a per-packet granularity.

# References

- R2-2508002 - RAN2#131bis Meeting Report (ETSI MCC)

- R2-2508126 - On latency reduction mechanisms for 6G eMBB traffic (MediaTek)

- R2-2507945 - LS on Early Alignment on Access Stratum security aspects (RAN2)

- R2-2508125 - On service awareness in the access stratum (MediaTek)

- R2-2506809 - Discussion on 6G user plane enhancements (Qualcomm)

- R2-2507157 - 6G UP design (Nokia)