3GPP TSG RAN WG2 #132          	                                                                  R2-2508233

Dallas, USA, Nov 17th – 21st, 2025

Source:	CATT

Title:	6GR UP Functions

Agenda Item:	10.3.1.1

Document for:	Discussion and Decision

- Introduction

This contribution discusses the required functionalities for 6GR UP based on the 5G UP architecture and 6G new requirements. And further discuss the UP design principles to better promote the standardization of the UP protocol.

- Discussion

  - The analysis of the UP functions

For better comprehension, we list the UP functions in 5G specifications first and extract the main functions. By leveraging 5G UP functions, the basic 6G UP functions are identified. Among these functions, three aspects are further considered: 1) whether the functions can be disable or not by RRC (mandatory or optional in UP); 2) whether major optimization is needed; 3) potential "duplicate" functions between sub-layers.

SDAP

The functions listed in 5G specifications[1] are:

-	transfer of user plane data;

-	mapping between a QoS flow and a DRB for both DL and UL;

-	mapping between an MBS QoS flow and an MRB for DL;

-	mapping between a PC5 QoS flow and a SL-DRB for NR sidelink communication;

-	marking QoS flow ID in both DL and UL packets;

-	marking PC5 QoS flow ID in unicast of NR sidelink communication packets;

-	reflective QoS flow to DRB mapping for the UL SDAP data PDUs.

Except the functions for MBS and Sidelink, the main functions are QoS flow to DRB mapping/remapping and reflective QoS.

- QoS mapping: In our understanding, 6G will still handle data packets based on DRBs in air interface. QoS mapping is used to organize data transmission within QoS requirements. It is a mandatory function in UP which is performed in UP natively and cannot be disabled by RRC.

- QoS remapping: QoS remapping is used for DRB management within a cell and during UE mobility. In 5G, End-Marker is introduced due to QoS remapping. In 6G, since multiple QoS flows can be mapped to one DRB as 5G, QoS remapping associated to the End-Marker mechanism is still needed. And QoS remapping should be performed in UP specification natively.

- Reflective QoS: Reflective QoS in RAN is extended from reflective QoS in CN. It can perform QoS mapping and remapping in user plane dynamically as an optimization for RRC configured QoS mapping and remapping. In 6G, we can consider RRC configured way as basic. Whether to adopt reflective QoS can be discussed in SA2 first.

Observation 1: Referring to 5G SDAP functions, QoS mapping and remapping are supported in UP and cannot be disabled. Reflective QoS can be discussed in SA2 first.

PDCP

The functions listed in 5G specifications[2] are:

-	transfer of data (user plane or control plane);

-	maintenance of PDCP SNs;

-	header compression and decompression using the ROHC protocol;

-	header compression and decompression using the EHC protocol;

-	uplink data compression and decompression using the UDC protocol;

-	ciphering and deciphering;

-	integrity protection and integrity verification;

-	timer based SDU discard;

-	PDU Set discard;

-	PSI based SDU discard;

-	for split bearers and DAPS bearer, routing;

-	duplication;

-	reordering and in-order delivery;

-	out-of-order delivery;

-	duplicate discarding.

The main functions in PDCP are: SN maintenance, HC, UDC, security, SDU discard, duplication, reordering and in-order delivery, out-of order delivery and duplication discarding.

- SN maintenance: it is the prerequisite for some other functions such as ciphering, integrity protection, and in-order delivery, etc. SN maintenance is performed in UP natively except TM mode.

- HC: including ROHC and EHC. ROHC and EHC are used for different protocols in which no one can be precluded. HC is an important function to reduce overhead in air interface, thus 6G will inherit the ROHC and EHC mechanisms and procedures in 5G.

- UDC: UDC is used to enhance SIP signaling transmission while optimizing UL resource utilization and reducing latency. In 6G Day1, IMS is also supposed to be supported [5]. Therefore, UDC should be supported to ensure efficient IMS-SIP signaling handling.

- AS Security: including ciphering and integrity protection. AS security is crucial for UE to prevent eavesdropping and manipulation by attackers. Both ciphering and integrity protection should be supported in 6G.

- SDU discard: SDU discard is a function in transmitting side. In 5G, SDU discard includes timer based SDU discard, PDU Set discard and PSI based SDU discard. For the three SDU discard types, PDU set features impact all of them, and NTN impacts the parameter values of discardTimer. In 6G, immersive communication is a key usage scenario. PDU set characteristics should be considered natively. And some other service characteristics such as FEC, synchronized data transmission will be considered based on further discussion. The impacts of NTN should be considered too. Thus, SDU discard needs to be adopted in 6G. Optimization should be considered due to the new services characteristics.

- PDCP duplication: In 5G, PDCP duplication is applied per RB, meaning that once activated, all packets transmitted by the PDCP entity are duplicated. PDCP duplication mechanism significantly enhances reliability. However, its primary drawback lies in reducing resource efficiency. To address this, selective PDCP duplication was proposed in 3GPP Release-16's IIoT work item, where only critical packets within a radio bearer would be duplicated. However, no definitive conclusion was reached due to the lack of QoS parameters finer than QoS flow granularity in Release 16. With the introduction of PDU set in Release-18, selective PDCP duplication can now be implemented based on PDU set information. Moving forward, 6G networks can further study resource efficiency improvement for PDCP duplication by leveraging PDU set-based policies. So PDCP duplication should be adopted in 6G and the optimization such as selective duplication can be considered.

- Reordering and in-order delivery & out-of-order delivery: They are the functions in receiving side. The requirements of in-order delivery depends on higher layer protocols, such as TCP. Both in-order delivery and out-of-delivery should be supported in Uu according to transport layer protocol. As in 5G, the function of reordering and in-order delivery should be supported in only one UP sub-layer, i.e., PDCP sub-layer.

- Duplication discarding: It is a function in receiving side. The function cannot be omitted because the duplicate PDCP PDUs can be received in many cases, such as PDCP duplication. Thus duplication discarding is performed in UP natively.

Observation 2: By leveraging 5G PDCP functions, 6G Day1 should support SN maintenance, HC (ROHC and EHC), UDC, AS security (ciphering and integrity protection), SDU discard, L2 PDU duplication, reordering and in-order delivery, out-of-order delivery and duplication discarding. Among the functions, SN maintenance and duplication discarding are supported in UP and cannot be disabled. SDU discard and L2 PDU duplication need to be optimized considering the service characteristics of immersive communication and NTN.

RLC

The functions listed in 5G specifications[3] are:

-	transfer of upper layer PDUs;

-	error correction through ARQ (only for AM data transfer);

-	segmentation and reassembly of RLC SDUs (only for UM and AM data transfer);

-	re-segmentation of RLC SDU segments (only for AM data transfer);

-	duplicate detection (only for AM data transfer);

-	RLC SDU discard (only for UM and AM data transfer);

-	RLC re-establishment;

-	Protocol error detection (only for AM data transfer).

The main functions are: segmentation, ARQ retransmission, re-segmentation, duplication detection, and RLC SDU discard.

- Segmentation: Segmentation is always needed to accommodate the allocated resources (except TM). It is performed in UP natively and cannot be disabled by RRC.

- L2 ARQ retransmission: Reliability is a critical metric in telecommunication. If relying on L1 HARQ feedback and HARQ retransmission, the reliability targets may be achievable within a certain range. However, considering the reliability requirements are general Macro-scenario targets rather than tailored to specific scenarios, ARQ retransmission as a more robust mechanism is still needed. Additionally, in ARQ retransmission, RLC SDU/SDU segments are retransmitted, allowing for better adaptation to available UL resources and channel conditions, thereby enabling more efficient retransmissions and resource utilization. In last meeting, RAN2 agreed “Study how to improve L2 ARQ latency/efficiency in 6GR based on tight coordination with HARQ. Study should identify enhancements needed for HARQ and L2 ARQ.” Thus, L2 ARQ retransmission is needed in 6G UP and enhancements should be studied.

- Re-segmentation: MAC SDU re-segmentation is used to accommodate in-flight data in the up-to-date allocated resources. It should be supported as long as L2 ARQ retransmission is supported.

- Duplication detection: The precondition of duplication detection is that there are duplicate PDUs delivered from lower layer. It should be decided based on the discussion on HARQ/ARQ tight interworking and final UP protocol structure.

- RLC SDU discard: RLC SDU discard is indicated by PDCP when PDCP SDU discard happens. Whether to keep RLC SDU discard in 6G will be decided based on the final UP protocol structure.

Observation 3: Referring to 5G RLC functions, segmentation and re-segmentation should be supported in UP and cannot be disabled. L2 ARQ retransmission is needed and can be optimized with HARQ. Whether to keep RLC duplication detection and RLC SDU discard depends on the final UP protocol structure.

MAC

The functions listed in 5G specifications[4] are:

-	mapping between logical channels and transport channels;

-	multiplexing of MAC SDUs from one or different logical channels onto transport blocks (TB) to be delivered to the physical layer on transport channels;

-	demultiplexing of MAC SDUs to one or different logical channels from transport blocks (TB) delivered from the physical layer on transport channels;

-	scheduling information reporting;

-	error correction through HARQ;

-	logical channel prioritization;

-	priority handling between overlapping resources of one UE;

-	radio resource selection.

The main functions in above list include: (de)multiplexing of MAC SDUs from/to one or multiple LCHs, scheduling information reporting, HARQ, and LCP. The MAC procedures related to not included in above list are: RACH, UL TA maintenance, scheduling mechanism, and DRX. These procedures should be discussed separately and not included in this contribution.

- (De)multiplexing: 5G removes the concatenation function from the 4G RLC sub-layer and places the function of multiplexing MAC SDUs from one LCH in MAC sub-layer. Thus the multiplexing function is located in MAC sub-layer only. Multiplexing of MAC SDUs onto TBs can maximize the utilization of allocated resources. The multiplexing function should be performed in UP and cannot be disabled by RRC.

- Scheduling information reporting: The scheduling information in 5G includes SR, BSR, DSR, PHR, and TAR. In last meeting, RAN2 agreed that “Study need for scheduling enhancements to address the SR/BSR/DSR latency”. We discuss SR/BSR/DSR in another contribution[6]. PHR is decided in RAN1 and it is important for uplink scheduling. TAR is introduced for NTN and 6G Day1 supports NTN. Thus both PHR and TAR are needed in UP.

- HARQ: HARQ is a basic function for reliability. It will be supported natively. And the enhancements will be discussed considering HARQ/ARQ interworking[6] and RAN1 inputs.

- LCP: LCP In 5G Rel-15, LCP is based on token bucket. The main motivation of using token bucket is to avoid starvation. Furthermore, in 5G XR, DSR and latency based LCP (e.g., additional priority for delay-critical LCH) were introduced to meet the delay-critical service requirements. The DSR and latency based LCP can also avoid starvation in a certain degree, which seem a bit repetitive with token bucket mechanism from the function perspective. Hence, in 6G, it can further evaluate whether token bucket should be reused or new LCP procedure can be considered.

Observation 4: For MAC, (De)multiplexing should be supported in UP and cannot be disabled. Scheduling information report, HARQ and LCP should be supported and enhanced.

Summary of the required functions

According to above analysis, we conclude the essential UP functions below.

Proposal 1: At least the following UP functions should be supported in 6G Day1 and how to support these functions can be further studied:

- Mapping/remapping between QoS flow(s) and a DRB,

- SN maintenance,

- Header compression and decompression (ROHC and EHC),

- UDC,

- AS security ((de)ciphering and integrity protection/integrity verification),

- SDU discard,

- L2 PDU duplication,

- Reordering and in-order delivery,

- Out-of-order delivery,

- Duplication discarding/detection,

- (Re)segmentation,

- L2 ARQ retransmission,

- (De)multiplexing,

- Scheduling information report,

- HARQ,

- LCP.

Proposal 2: The functions should be supported in UP and cannot be disabled including: QoS (re)mapping, SN maintenance, duplication discarding, (re)segmentation, and (de)multiplexing.

Proposal 3: Whether to support reflective QoS depends on SA2 progress.

”Duplicate” functions

Based on the identified UP functions, we further discuss the “duplicate” functions since some companies prefer to minimize duplication of functionalities across different protocol layers.

In 5G specification, there are several functions exist in different sub-layer, including: 1) status report in PDCP and RLC, 2) PDCP duplication and TB repetition, 3) SN in PDCP and RLC, 4) SDU discard in PDCP and RLC, 5) duplication discarding in PDCP and RLC, and 6) ARQ in RLC and HARQ in MAC. Enhancement of HARQ and L2 ARQ is discussed in[6]. SDU discard and duplication discarding are analyzed above that they should be decided based on the final UP protocol structure. Then we discuss three aspects below: status report in PDCP and RLC, PDCP duplication and TB repetition, SN in PDCP and RLC.

- Status report in PDCP and RLC: PDCP status report is used for UE mobility to guarantee lossless transmission and avoid redundant transmission after handover. The parameters of PDCP SN are 32-bit COUNT value and variable-sized bitmap. RLC status report is used for ARQ retransmission and the RLC status report uses SN and SO to indicate the reception status of RLC SDU and RLC SDU segmentation in the receiving side. Therefore, it is not feasible to combine PDCP SR and RLC SR.

(a) PDCP SR                                                                               (b) RLC SR(12bit SN)

Figure 1 5G PDCP Status Report vs. RLC Status Report

- PDCP duplication and TB repetition: PDCP duplication is used to improve the reliability of PDCP PDUs. It was introduced to satisfy ultra-high reliability requirements in 5G URLLC. TB repetition was introduced in RAN1. The main objective of TB repetition is coverage enhancement. Since both the objectives and the granularities are totally different, they should be two separate functions in 6G too.

- SN in PDCP and RLC: In 5G specification, PDCP and RLC maintain SNs separately. For each RLC AMD PDU, it will consume 2-3 bytes for RLC SN. 5G had discussed reusing of PDCP SN and RLC SN but didn’t adopt. One reason is that it is not clear how to perform RLC SN maintenance and RLC reordering in DC architecture. However, if we can identify the potential issues and consider it during 6G RLC design (e.g., optimizing RLC window mechanism and the initial values of the related parameters), it may not be a big deal.

Observation 5: For the “duplicate” functions: 1) Keep separately: PDCP SR and RLC SR, PDCP duplication and TB repetition; 2) Depends on the final UP stack structure: SDU discard and duplication discarding; 3) Combine: PDCP SN and RLC SN.

Proposal 4: SN can be maintained in only one sub-layer.

  - The principles for UP design

In last meeting, RAN2 agreed “UP design should aim to be hardware-processing friendly while keeping memory requirements low and minimize data movements across protocol layer.”

It is not very clear what’s mean “hardware-processing friendly”. One example is fixed headers. The header sizes of 5G UP sub-layers except sidelink are listed below. If fixed headers are applied, it is possible that 14-byte L2 header is always used for a single packet or packet segmentation. It is not a reasonable overhead.

Table 1 Header sizes of 5G UP sub-layers

|  | Header size | Minimal size | Maximal size |
| --- | --- | --- | --- |
| SDAP | - 0 if configured as absent; - 1-byte if configured as present. | 0 | 1 |
| PDCP | - 2-byte (12bit SN) - 3-byte (13bit SN) | 2 | 3 |
| RLC | UM: - 1-byte without segmentation, or with 6 bit SN no SO; - 2-byte with 12 bit SN no SO; - 3-byte with 6 bit SN with SO; - 4-byte with 12 bit SN with SO. AM: - 2-byte with 12 bit SN no SO; - 3-byte with 18 bit SN no SO; - 4-byte with 12 bit SN with SO; - 5-byte with 18 bit SN with SO. | 1 | 5 |
| MAC | For a subPDU, - 1/2/3/4/5-byte | 1 | 5 |
| Sum | / | 4 | 14 |

Observation 6: L2 header overhead is a key consideration in UP design.

Processing complexity is another important factor for UP design. The L2 PDU design with fixed headers is a good way to reduce the processing complexity. But as analyzed above, we don’t think the solution of large fixed headers is feasible considering the air interface overhead and resource efficiency. Another solution for processing complexity is that transparent processing in specific sub-layer when the functions of the sub-layer are not needed. For example, in transmitting side, when the functions of segmentation and SN maintenance are not needed for a PDCP PDU (RLC SDU), if RLC sub-layer can deliver it to lower layer directly instead of adding a useless RLC header, it will be more beneficial to processing complexity and data movements.

Observation 7: UP design should consider processing complexity. Customizable protocol stack with variable sub-layers and functions is a considerable design for both header overhead reduction and processing complexity reduction.

Another consideration for UP design is to “minimize data movements across protocol layer”. One possible solution for minimizing data movements is to combine some 5G sub-layers. For example, the combination of SDAP and PDCP was discussed in last meeting. A SDAP entity is associated to a PDU session. A PDCP entity is associated to a RB. The mapping between SDAP entity and PDCP entity is one-to-multiple because a SDAP entity needs to treat a lot of QoS flows and map one or several QoS flow(s) to a DRB.

If SDAP and PDCP are combined, two possible structures are listed in Figure 2.

(a) Centralized-Distributed structure                                           (b) Distributed structure

Figure 2 Two possible structures for SDAP/PDCP combination

In (a), one centralized part associated to a PDU session replaces 5G SDAP entity, and distributed parts equal to 5G PDCP entities. In this structure, new PDCP specification will combine functions and description in 5G SDAP and PDCP, and specify them as “centralized part” and “distributed part”. Although SDAP functions are simple and SDAP header can be absent. Two sub-layers specifications will be clearer and easy to maintain. For the “data movement”, it is no different between the structure of two sub-layers and the structure of centralized part and distributed parts in one sub-layer.

In (b), PDCP entities are distributed. So each PDCP entity needs to perform QoS flow(s) to DRB mapping and check whether this QoS flow is related to its DRB or not. Both QoS mapping mechanism and QoS remapping mechanism need to be reconsidered and specified. This will likely complicate the UP protocol rather than simplify it.

Similar consideration on RLC and MAC, since L2 ARQ feedback and retransmission for RLC SDU/SDU segmentation is needed, the procedures of ARQ feedback and retransmission and the necessary fields such as SI/SO must be considered in PDU formats design, it is simple to keep RLC sub-layer and MAC sub-layer which perform functions for logical channel and cell group (multi-LCHs) separately.

In general, the functions and relationships of 5G UP sub-layers are clear and easy to maintain. It can be a good starting point for 6G UP protocol structure.

Observation 8: In 5G, SDAP entity is per PDU session; PDCP entity is per DRB; RLC entity is per LCH; MAC entity is per Cell Group. The functions and relationships of UP sub-layer are clear and easy to maintain. It is a good starting point for 6G UP.

Proposal 5: The principles for UP design should include header overhead reduction, processing complexity reduction and facilitation of specification maintenance.

Proposal 6: RAN2 take 5G L2 sub-layers as baseline including SDAP, PDCP, RLC and MAC, further consider the division of functionalities in the sub-layers.

- Conclusion

In this contribution, we provide below observations and proposals:

Observation 1: Referring to 5G SDAP functions, QoS mapping and remapping are supported in UP and cannot be disabled. Reflective QoS can be discussed in SA2 first.

Observation 2: By leveraging 5G PDCP functions, 6G Day1 should support SN maintenance, HC (ROHC and EHC), UDC, AS security (ciphering and integrity protection), SDU discard, L2 PDU duplication, reordering and in-order delivery, out-of-order delivery and duplication discarding. Among the functions, SN maintenance and duplication discarding are supported in UP and cannot be disabled. SDU discard and L2 PDU duplication need to be optimized considering the service characteristics of immersive communication and NTN.

Observation 3: Referring to 5G RLC functions, segmentation and re-segmentation should be supported in UP and cannot be disabled. L2 ARQ retransmission is needed and can be optimized with HARQ. Whether to keep RLC duplication detection and RLC SDU discard depends on the final UP protocol structure.

Observation 4: For MAC, (De)multiplexing should be supported in UP and cannot be disabled. Scheduling information report, HARQ and LCP should be supported and enhanced.

Proposal 1: At least the following UP functions should be supported in 6G Day1 and how to support these functions can be further studied:

- Mapping/remapping between QoS flow(s) and a DRB,

- SN maintenance,

- Header compression and decompression (ROHC and EHC),

- UDC,

- AS security ((de)ciphering and integrity protection/integrity verification),

- SDU discard,

- L2 PDU duplication,

- Reordering and in-order delivery,

- Out-of-order delivery,

- Duplication discarding/detection,

- (Re)segmentation,

- L2 ARQ retransmission,

- (De)multiplexing,

- Scheduling information report,

- HARQ,

- LCP.

Proposal 2: The functions should be supported in UP and cannot be disabled including: QoS (re)mapping, SN maintenance, duplication discarding, (re)segmentation, and (de)multiplexing.

Proposal 3: Whether to support reflective QoS depends on SA2 progress.

Observation 5: For the “duplicate” functions: 1) Keep separately: PDCP SR and RLC SR, PDCP duplication and TB repetition; 2) Depends on the final UP stack structure: SDU discard and duplication discarding; 3) Combine: PDCP SN and RLC SN.

Proposal 4: SN can be maintained in only one sub-layer.

Observation 6: L2 header overhead is a key consideration in UP design.

Observation 7: UP design should consider processing complexity. Customizable protocol stack with variable sub-layers and functions is a considerable design for both header overhead reduction and processing complexity reduction.

Observation 8: In 5G, SDAP entity is per PDU session; PDCP entity is per DRB; RLC entity is per LCH; MAC entity is per Cell Group. The functions and relationships of UP sub-layer are clear and easy to maintain. It is a good starting point for 6G UP.

Proposal 5: The principles for UP design should include header overhead reduction, processing complexity reduction and facilitation of specification maintenance.

Proposal 6: RAN2 take 5G L2 sub-layers as baseline including SDAP, PDCP, RLC and MAC, further consider the division of functionalities in the sub-layers.

- References

- 3GPP TS37.324, Service Data Adaptation Protocol (SDAP) specification, V19.0.0

- 3GPP TS38.323, Packet Data Convergence Protocol (PDCP) specification, V19.0.0

- 3GPP TS38.322, Radio Link Control (RLC) protocol specification, V19.0.0

- 3GPP TS38.321, Medium Access Control (MAC) protocol specification, V19.0.0

- 3GPP TR22.870, Study on 6G Use Cases and Service Requirements, V0.4.0

- R2-2508235, Discussion on L2 Retransmission and Scheduling, CATT