3GPP TSG-RAN WG2 Meeting #132	R2-2508245

Dallas, USA, Nov. 17th - 21st

Agenda Item:	9.3.2

Source: 	NEC

Title:  	Discussion on UE-side RRM measurement prediction

Document for:	Discussion and Decision

- 1 Introduction

The Rel-20 AI/ML mobility WID [1] related to UE-side RRM measurement prediction part is shown as below:

- Specify support for RRM measurement prediction and measurement event prediction for UE sided models [RAN2]:

- Temporal domain prediction case A and case B, and Frequency domain prediction for co-located cases are supported.

- L3 Cell level predication is supported

Note 1: For UE sided model, spatial domain prediction is not supported.

Note 2: For UE sided model, L3 beam level prediction is not supported.

- For both RRM measurement prediction and measurement event prediction, specify signalling and protocol aspects to enable LCM functionality management for UE sided model including [RAN2]:

- UE capability request and response procedure

- Applicability report for both full and partial configuration approach

- Inference configuration and inference report

- Performance monitoring performed in UE or network side, based on which network can manage UE sided functionality

- UE sided data collection

- For measurement event prediction:

  - Measurement event A1~A6 are in the scope in this release

Note3: Data transfer and model delivery for UE sided model are out of scope of this WI. And model related choices can be up to UE’s implementation.

In this contribution, we discuss LCM functionality management for RRM measurement prediction including applicability report procedure, inference configuration and report.

- 2 Discussion

- 2.1 Applicability report procedure

The design on LCM functionality management is important to make AI/ML mobility functionality operate smoothly. One key point is to achieve applicability report procedure so that UE and NW can be aligned for how/when to apply this AI/ML mobility functionality. And our understanding is that the LCM procedure introduced in Rel-19 AI PHY can be taken as baseline, the procedure is shown below.

Fig.1 applicability report procedure

1. The network inquires about UE capability information related to AI/ML mobility

2. The UE indicates its supported functionalities for AI/ML mobility to the network via UECapabilityInformation message.

3. The network may provide inference configuration (including either full inference configuration for AI/ML mobility or partial inference configuration with some parameters related to AI/ML mobility), and optional NW-side additional conditions in RRCReconfiguration.

4. The UE determines the applicable AI/ML functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in the UE.

5. The UE reports its functionality applicability (including applicable and inapplicable indication) in RRCReconfigurationComplete message.

6. Upon reporting the applicable functionalities, the UE applies/activates the corresponding inference configuration (FFS partial configuration case, FFS event-triggered condition,). Upon reporting the inapplicable functionalities, the UE is expecting that NW will release the corresponding inference configuration but the UE still needs to perform the inference (if activated before) until configuration is released.

7. Applicability status can be further updated via UEAssistanceInformation.

Proposal-1: the LCM procedure in Rel-19 AI/ML PHY can be taken as baseline for AI/ML mobility, and the above applicability report procedure can be considered as starting point.

- 2.2 Inference configuration

Based on WID, temporal domain case A, temporal domain case B and frequency domain will be supported. And we discuss inference configuration related to these cases respectively.

Temporary domain case A

Fig.2 Example of intra-frequency temporal domain case A

Temporal domain case A belongs to intra-cell inference, the UE predicts future measurement results within prediction window (PW) based on historical measurement results within observation window (OW), and our understanding is that how many inference inputs are needed can be up to UE implementation depending on UE AI model capability while PW should be explicitly configured by NW to control subsequent UE behaviour after prediction as this PW value should be closely relevant to TTT value, MR periodicity, etc. Also noted that it is promising to take existing RRM measurement framework (i.e., measConfig) as baseline for inference configuration. Normally, the UE is configured with measurement object which is indicated to frequency instead of dedicated cell information, when it comes to AI/ML model inference, frequency granularity is a little rough which would make model perform inference for all cells under this frequency. Therefore, cell information used to indicate which cell should be used for inference can be provided in the inference configuration. Furthermore, companies can further consider if inference periodicity is needed, for example, if periodical MR is configured, this MR periodicity can be considered as inference periodicity while for event prediction, since the MR is only triggered by event fulfilment, there is no MR periodicity before event fulfilment, it is not clear when and how often the UE needs to perform inference.

Proposal-2: the inference configuration for temporary domain case A should consider the following parameters:

- Prediction window (PW), FFS the value form, e.g., ms, measurement or prediction instance

- Cell information, used to indicate which cell should be used for inference

- FFS inference periodicity

Temporary domain case B

Fig.3 Example of intra-frequency temporal domain case B

Temporary domain case B is also intra-cell inference, and this case is for measurement reduction purpose by extending the real measurement instance but predict for those instances which are not real measured. First the NW should indicate which measurement instance should be skipped, therefore the timing of SSB configuration should be provided in inference configuration. And same reason as case A, cell information is also needed to indicate which cell will be used for inference. Furthermore companies should also consider whether inference periodicity is needed or not.

Proposal-3: the inference configuration for temporary domain case B should consider the following parameters:

- Timing information of SSB configuration, e.g., a separate SMTC dedicated for AI/ML

- Cell information, used to indicate which cell should be used for inference

- FFS inference periodicity

Frequency domain

This is inter-frequency inter-cell inference, the UE performs measurements on one frequency (e.g., intra-frequency) and predict measurement results for another frequency (e.g., inter-frequency) based on AI inference, whether multiple frequency inputs is needed can be further studied (i.e., cluster case). In this case, the NW need to provide measured frequency and predicted frequency to UE so that the UE knows how to perform inference, and these frequency can be indicated by enhanced measurement object (MO). Since this is frequency level prediction, it is assumed that the UE will, in spatial domain, predict multiple cell measurement results under predicted frequency, whether more detailed IE, e.g., cell information under predicted frequency is needed or not can be further studied, as well as inference periodicity.

Proposal-4: the inference configuration for frequency domain prediction should consider the following parameters:

- Measured frequency and predicted frequency indicated in enhanced MO

- FFS Cell information under predicted frequency

- FFS inference periodicity

- 2.3 Report

For temporary case A, since the UE may predict multiple instances of predicted measurement results, when the UE initiates measurement report, this report should comprise all these predicted measurement results for providing more information so that facilitate NW handover/mobility decision, the current MR reporting mechanism only allows UE to report the latest measurement result which is not enough for AI/ML measurement prediction design. Therefore, measurement report should be enhanced to carry multiple instances of predicted measurement results with time information.

For temporary case B and frequency domain, basically it doesn’t belong to future prediction and UE could just leverage the latest measurement results (may or may not be predicted results) for RRM management as legacy. But when the UE initiates measurement report, since NW is not aware of whether the reported results is from prediction or real results, an flag/indication can be introduced to indicate whether the reported results is real measured or predicted.

Proposal-5: measurement report should be enhanced to be able to carry multiple instances of predicted measurement results with time information for temporal domain case A and introduce a flag/indication to indicate whether the reported measurement results is real measured or predicted for temporal case B and frequency domain.

- 3 Conclusions

Proposal-1: the LCM procedure in Rel-19 AI/ML PHY can be taken as baseline for AI/ML mobility, and the above applicability report procedure can be considered as starting point.

Proposal-2: the inference configuration for temporary domain case A should consider the following parameters:

- Prediction window (PW), FFS the value form, e.g., ms, measurement or prediction instance

- Cell information, used to indicate which cell should be used for inference

- FFS inference periodicity

Proposal-3: the inference configuration for temporary domain case B should consider the following parameters:

- Timing information of SSB configuration, e.g., a separate SMTC dedicated for AI/ML

- Cell information, used to indicate which cell should be used for inference

- FFS inference periodicity

Proposal-4: the inference configuration for frequency domain prediction should consider the following parameters:

- Measured frequency and predicted frequency indicated in enhanced MO

- FFS Cell information under predicted frequency

- FFS inference periodicity

Proposal-5: measurement report should be enhanced to be able to carry multiple instances of predicted measurement results with time information for temporal domain case A and introduce a flag/indication to indicate whether the reported measurement results is real measured or predicted for temporal case B and frequency domain.

- 4 References

[1] RP-252899 	Revision of WID on AI mobility		OPPO, Interdigital