3GPP TSG-RAN WG2#132	R2-2508270

Dallas, USA, Nov 17th – 21st 2025

Source: 	Fujitsu

Agenda item:	10.3.1.3

Title:	Discussion on scheduling enhancements for 6G

Document for:	Discussion and decision

Introduction

In RAN2#131bis meeting, for 6GR UP, some agreements were made as follows [1]:

Agreements on UP

- 1.	RAN2 first discuss/agree on the required functionalities for the protocol stack, and only afterwards discuss which protocol layer supports each functionality.  Discussions should focus on standalone architecture, until told otherwise by plenary.  When discussing functionalities companies should focus on identifying the problems being address or new requirements.

2.  	UP design should aim to be hardware-processing friendly while keeping memory requirements low and minimize data movements across protocol layer.

3.	6G user plane should aim to reduce the radio and end-to-end latency for general services (including eMBB)

4.	Study potential benefits and standardized mechanisms for applications/service-awareness in the RAN (e.g. immersive communications, AI mobile traffic).  Understand the issues and shortcoming with the current NR QoS.   Aim to look at a general framework.

5.	Support at least the following scheduling schemes: dynamic grant and configured grant.  Further study configured grant.

6.	Study need for scheduling enhancements to address the SR/BSR/DSR latency

7.	Study how to improve L2 ARQ latency/efficiency in 6GR based on tight coordination with HARQ.  Study should identify enhancements needed for HARQ and L2 ARQ.

In this contribution, we would like to provide our views on scheduling enhancements for 6GR.

Discussion

SR/BSR/DSR enhancements

In NR, SR is used by the UE for requesting UL-SCH resources for new transmission when the UE has no UL-SCH resources for new transmission or when the UL-SCH resources for new transmission cannot meet the LCP mapping restrictions. An SR configuration including a set of dedicated PUCCH resources for the SR triggered by BSR, DSR and some other MAC CEs are configured to the UE for the SR transmission. The Buffer Status reporting (BSR) procedure is used to provide the serving gNB with information about UL data volume in the MAC entity. Later in Rel-18 and Rel-19 XR, single-entry and multi-entry Delay Status Reporting (DSR) procedures are introduced to provide the serving gNB with delay status of LCGs, respectively. This delay status for an LCG includes remaining time and the corresponding total amount of delay-critical UL data/delay-reporting UL data for the LCG.

From our perspective, the current SR, BSR and DSR procedures in NR have their individual functions and purposes. However, there are also some drawbacks:

- SR, BSR and DSR procedures are all used to request uplink resources for new transmission for the network, and their functionalities are somewhat overlapping.

- SR and BSR procedures cause scheduling latency, due to one more round-trip-time in the Uu interface.

- Dedicated PUCCH resources for SR for each UE lead to more overhead in radio resources.

Observation 1: SR, BSR and DSR procedures in NR have overlapping functionalities, long scheduling latency and high radio resource overhead.

Therefore, for 6GR, we assume that SR, BSR and DSR procedures can be combined into a unified status reporting procedure to avoid overlapping functionalities, reduce the scheduling latency and reduce the radio resource overhead. How to do the combination can be further studied.

Proposal 1: RAN2 to study that SR, BSR and DSR procedures are combined into a unified status reporting procedure in 6GR.

LCP enhancements

The MAC layer has many basic functions related to data transmission, including (de-)multiplexing, LCP, HARQ, BSR, SR, etc. In 5G NR, the resource allocation (LCP) procedure is designed to meet eMBB service requirement as baseline, targeting to support the prioritized bit rate as the first priority. It is not really friendly to delay-critical services, such as URLLC and XR. Although enhancements for supporting URLLC and XR were specified in 5G later releases, the complexity on UE implementation and uncertainty of the usefulness lead to lack of commercialization of these features.

In the new use cases for 6G, industry and verticals are typical HRLLC scenarios. Thus, it is critical to meet HRLLC service requirements on its reliability and stringent latency. The usage scenario of immersive communication extends the eMBB and covers use cases which provide a rich and interactive video (immersive) experience to users, including the interactions with machine interfaces. 6GR air interface designs for immersive communication need to ensure consistent user experience and improved network capacity, taking into account the composite requirement of high data rate and low latency. In addition, the QoS requirements for immersive communications are different among data streams. Good immersive experience lies on all streams meet their own QoS requirements. Besides, the requirements among different streams are dependent, similar to multi-modality issue discussed for XR in NR. Therefore, composite requirement of data rate, latency, reliability, and capacity should be considered. To achieve this, an adaptive service-aware/QoS-aware resource allocation which can adaptively optimize resource allocation with different targets needs to be studied.

Observation 2: NR LCP-based resource allocation procedure is eMBB-centric and not friendly to XR, HRLLC and immersive communication.

Proposal 2: Study on adaptive resource allocation method to meet variable needs for different services, including services with high data rate, services with stringent latency requirement and services with composite requirement.

Enhancements on HARQ and ARQ retransmissions

In NR MAC layer, the HARQ functionality ensures delivery between peer entities at Layer 1. In NR RLC layer, the ARQ process in AM mode serves as a last resort for failure of the transmission in HARQ process, cause by for example exceeding maximum number of TB retransmissions, NACK to ACK error, PDCCH misdetection, etc. ARQ retransmits RLC SDUs or RLC SDU segments based on RLC status report.

Legacy RLC status report can be triggered by t-Reassembly timer expiration or polling. Normally the t-Reassembly and parameters related to polling will be configured in a conservative manner to avoid interfering with HARQ process. So, the ARQ retransmission latency may be tens of milliseconds which cannot meet the requirements of delay-critical applications. In NR Rel-19, remaining time-based RLC retransmission and remaining time-based polling are introduced to better support low latency for XR services. However, the setting of the remaining time thresholds is challenging, and autonomous retransmission cannot efficiently adapt to the lower layer transmission status. To address these issues and to achieve fast and efficient layer 2 retransmission, it is better to let the ARQ procedure be aware of the status of the HARQ procedure.

The principle of the HARQ and ARQ coordination is to notify the ARQ process when an HARQ failure (considering all retransmissions) happens, so that ARQ retransmission can be performed immediately. How to detect an HARQ failure in MAC layer, and how to operate the ARQ retransmission, e.g., whether it is triggered by a status report, need to be studied.

Observation 3: Legacy ARQ procedure will introduce long retransmission delay. Rel-19 XR enhancements on RLC autonomous retransmission are also not flexible or efficient for RLC retransmission.

Proposal 3: HARQ and ARQ procedures are coordinated so that ARQ procedure is aware of HARQ procedure status to support fast L2 retransmission.

An important issue for HARQ and ARQ coordination is to decide whether ARQ function is kept in the RLC layer or moved to the MAC layer. If ARQ function is kept in the RLC layer, the standard effort may be relatively smaller. If ARQ function is moved to the MAC layer, other functions such as segmentation, reassembly, and re-segmentation will be moved to the MAC layer adaptively. Essentially, no RLC layer is needed as a result. The cross-layer interaction will be reduced, and it may possibly minimize the processing delay for data retransmission. RAN2 needs to study the pros and cons of the two approaches and decide the direction for fast L2 retransmission.

Proposal 4: Study the HARQ and ARQ coordination in the following two directions:

Direction 1: ARQ function is kept in RLC layer.

Direction 2: ARQ function is moved to MAC layer.

Conclusion

In this paper we presented some views on scheduling enhancements for 6G and have come up with the following observations and proposals:

Observation 1: SR, BSR and DSR procedures in NR have overlapping functionalities, long scheduling latency and high radio resource overhead.

Proposal 1: RAN2 to study that SR, BSR and DSR procedures are combined into a unified status reporting procedure in 6GR.

Observation 2: NR LCP-based resource allocation procedure is eMBB-centric and not friendly to XR, HRLLC and immersive communication.

Proposal 2: Study on adaptive resource allocation method to meet variable needs for different services, including services with high data rate, services with stringent latency requirement and services with composite requirement.

Observation 3: Legacy ARQ procedure will introduce long retransmission delay. Rel-19 XR enhancements on RLC autonomous retransmission are also not flexible or efficient for RLC retransmission.

Proposal 3: HARQ and ARQ procedures are coordinated so that ARQ procedure is aware of HARQ procedure status to support fast L2 retransmission.

Proposal 4: Study the HARQ and ARQ coordination in the following two directions:

Direction 1: ARQ function is kept in RLC layer.

Direction 2: ARQ function is moved to MAC layer.

References

- RAN2#131bis RAN2 chair’s note