3GPP TSG-WG2 Meeting #132	R2-2508339

Dallas, USA, November 17 – 21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 5575 | rev | - | Current version: | 17.14.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of remote UE release procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_Relay-Core |  |  |  |  |  | Date: |  |  | 2025-11-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In clause 5.3.5.15.2, if the release is triggered by reception of the sl-RemoteUE-ToReleaseList, the L2 U2N Relay UE indicates upper layers to trigger PC5 unicast link release for the corresponding L2 U2N Remote UE. The L2 U2N Relay UE shall also release the configurations associated with the corresponding L2 U2N Remote UE. 5.3.5.15.2 L2 U2N Remote UE Release The L2 U2N Relay UE shall: 1> if the release is triggered by reception of the sl-RemoteUE-ToReleaseList: 2> for each SL-DestinationIdentity value included in the sl-RemoteUE-ToReleaseList: 3> if the current UE has a PC5 RRC connection to a L2 U2N Remote UE with SL-DestinationIdentity: 4> indicate upper layers to trigger PC5 unicast link release. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In clause 5.3.5.15.2, include the procedure text “release the configurations associated with the L2 U2N Remote UE.” Impact Analysis Impacted 5G architecture options: NR SA This CR is mandatory to implement for UEs supporting Sidelink relay operation Impacted functionality: Sidelink relay operation Inter-operability: If the network implements the change according to the CR and the UE does not, there are no inter-operability issues since the procedures only affect the UE. If the UE implements the change according to the CR and the network does not, there are no inter-operability issues since the procedures only affect the UE. If one UE implements the change according to the CR, but not the other UE, there are inter-operability issues, i.e., one relay UE releases the configurations of a remote UE while another relay UE doesn’t release the configruation of a remote UE. Thus, relay UE behaviors are inconsistent. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the CR is not approved, configurations of remote UEs in a relay UE will not be released. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.5.15.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

##### 5.3.5.15.2	L2 U2N Remote UE Release

The L2 U2N Relay UE shall:

1>	if the release is triggered by reception of the sl-RemoteUE-ToReleaseList:

2>	for each SL-DestinationIdentity value included in the sl-RemoteUE-ToReleaseList:

3>	if the current UE has a PC5 RRC connection to a L2 U2N Remote UE with SL-DestinationIdentity:

4>	indicate upper layers to trigger PC5 unicast link release.

4 > release the configurations associated with the L2 U2N Remote UE.

End of changes