3GPP TSG-RAN WG2 Meeting #132

Dallas, USA, 17 – 21 November 2025	R2-2508349

Agenda item:	10.3.2.2

Source:	Nokia

Title:	RRC structure and configuration in 6GR

WID/SID:	FS_6G_Radio - Release 20

Document for:	Discussion and Decision

# 1	Introduction

The current 5G RRC protocol, despite undergoing multiple revisions across various releases, retains a predominantly monolithic and complex architecture. This structure is characterized by procedural delays that are intrinsic to basic operations, and it often necessitates the repetitive configuration of the same parameters for individual UEs leading to suboptimal radio resource utilization. Therefore, it would be beneficial to study how to enable a signalling structure for 6G RRC that reduces typical RRC configuration overhead while avoiding excessive nesting of configurations.

During RAN2#131bis, it was already agreed to use ASN.1 for the 6G RRC signalling, as well as to study the overall RRC structure and how to improve on the RRC structure and signalling efficiency, including studying how to modularize the RRC. In this document, we discuss these aspects.

Agreements

  - ASN.1 is used for encoding of RRC signaling for 6G air interface (same as NR).

  - Study overall RRC structure, configuration improvements, improve readability of ASN.1 for RRC signalling

  - Study how to efficiently, reliably and unambiguously configure UEs while keeping signalling size small (e.g. improvements to delta signaling or no delta signaling).

  - RAN2 will study modular design of RRC for 6G.  Further study how to modularize RRC e.g. based on features, functions, verticals, etc.

# 2	RRC signalling in 5G

## 2.1	Signalling sizes in 5G RRC

The RRC signalling sizes vary a lot in practice: Some reconfigurations can be very small (e.g. DRX configuration changes or specific L1 parameter are usually small messages) while others are clearly larger (e.g. SCell addition or HO command can be quite large messages). This is illustrated in the Figure below showing some examples from a real network, where the messages have been categorized to three cases: 1) “Call handling” to cover typical cases of call establishment or modification of PCell parameters,  2) “Carrier aggregation” showing the addition, modification and release of SCells, and 3) “Handover” to cover HO commands and related configurations such as measurements.. At the same time, the figure illustrates the (normalized) frequency of the messages to show that some messages are often more frequency than others (e.g. “call handling” purpose is the most common reconfiguration purpose and to “Carrier aggregation” is relatively uncommon).

Figure 1. Illustration of messages sizes and frequency of messages in real network.

The figure illustrates that while HO(-related) message can grow to be quite big in some cases, the variance is also large, and the biggest number of messages is contributed by the overall call handling (i.e. call setup and parameter reconfigurations). Thus, the signalling size implies that it is important to both reduce the maximum typical message size (e.g. for call handling messages) while also avoiding increasing the minimum size of the typical messages (e.g. to avoid making all messages big).

Observation 1: RRCReconfiguration signalling size varies a lot in practice. Reducing the typical message sizes while avoiding increase to the minimum message sizes would be beneficial.

## 2.2	Modularity in 5G RRC

In RAN2#131bis, many companies (including Nokia) were interested in considering more “modular” RRC to help reduce the configuration complexity. For that reason, it bears considering what each company might mean with this as the term “modularity” has multiple meanings. In general, “modularity” refers to a structure than is composed of separate parts that, when combined, form a complete whole (as per Cambridge English dictionary). In terms of programming languages, the term also often refers to the “composability” or “modularization” paradigms that are natively defined in many programming languages, with the intent to allow splitting a program into smaller, manageable components that can be combined to form a larger program. This allows complexity reduction by splitting a large problem into several separate smaller problems, each of which is easier to solve.

In terms of the ASN.1 language, a “module” is a fundamental unit that provides a unique name to a collection of type definitions and/or value definitions/assignments, constituting a data specification (i.e. the ASN.1 schema). In terms of implementation, the UE (and network) must implement the schema defined by the modules in the specification to be able to encode/decode the corresponding ASN.1 signalling, which means that the memory footprint of the signalling depends on the size of the ASN.1 schema. That is, while the ASN.1 is inherently “modular” in the strict sense of the word, there is no enforced way to split the schema into smaller piece.

In 5G RRC, we see both types of modularity in the form of ASN.1 modules and configuration modules.

- Several ASN.1 modules are defined as of Rel-19: NR-RRC-Definitions (the main RRC signalling module), NR-UE-Variables (UE stored information used in the procedural descriptions), PC5-RRC-Definitions (the main sidelink signalling), NR-Sidelink-Preconf (pre-configured sidelink signalling), NR-Sidelink-DiscoveryMessage (sidelink relay discovery signalling) and NR-InterNodeDefinitions (inter-node message exchange about RRC configurations between gNBs) – this is illustrated in Figure 2 below.

Figure 2. Illustration of ASN.1 modules in 5G RRC

- Configuration modules are the definitions of IEs and OCTET STRINGS: For example, the RRCReconfiguration is split into CU-part (MeasConfig and RadioBearerConfig) and DU-part (CellGroupConfig), out of which the latter is also contained within an OCTET STRING. Similarly, the DU configuration contains PHY and UP parts, where PHY configuration is split into “common” part (e.g. PhysicalCellGroupConfig) and BWPs (which contain most of the PHY configurations), and UP configuration is split into a (single) MAC cell group configuration (which is common to all bearers) and RLC bearer configurations (which are DRB-specific) – this is illustrated in Figure 3 below.

Figure 3. Illustration of CU/DU-split in RRC configuration.

The first type of modularity is of primary importance for the actual ASN.1 footprint, as it determines the size of the “codebase” that every UE has to support. This is also the reason e.g. sidelink and UE variables have been separated from the main ASN.1 – UEs not supporting sidelink do not have to comprehend anything related to the operation, and network is not required to comprehend anything related to UE variables. There is also a unidirectional relation between the “common” ASN.1 module and other modules – all the smaller modules IMPORT the definitions from the main ASN.1 module, and to avoid recursive definitions, the IMPORT is only from main module to child module (to avoid issues with recursive definitions).

Observation 2: ASN.1 modularity almost requires a “main” module that is used for IMPORT for other modules.

The second type of modularity is more about configuration complexity and is of primary importance to the network operations as it is the task of network to create the configuration provided to the UE, while UEs primarily need to just parse the provided configuration. This is the primary “modularity” discussed in 3GPP, and where 3GPP can (more) freely define the signalling structure. One thing that is often overlooked is the OCTET STRING (or BIT STRING) construct from the ASN.1 language: By definition, the OCTET STRING is encoded with a length-field that indicates how many bits the encoded content consumes, which means that every OCTET STRING can be treated as a separate “module” for the ASN.1 compiler – that is, each such component can be encoded or decoded as an independent schema. However, this obviously comes with a cost: The length field consumes 1-2 octets in addition to the content, which may or may not be critical (depending on the message).

Observation 3: Using OCTET STRING allows “modularizing” ASN.1 content with the cost of 1-2 bytes for each OCTET STRING.

## 2.3	Delta signalling and need codes in 5G RRC

Delta signalling is one of the most misunderstood concepts in 3GPP RRC specifications: Its purpose is to reduce signalling size, but at the cost of complicated rules on which fields/IEs are subject to delta signalling. The primary reason for the confusion comes from the lack of proper definition in the specifications for “delta signalling”: It is defined only indirectly via the Need Codes, which are described in Figure 4 below.

Figure 4. Definition of need codes in 5G RRC specification

The term delta signalling is not defined in RRC specification but essentially refers to the property of optional fields marked as “Need M”: UE retains the values of the fields even when they are absent in a received RRC (re)configuration message. However, this is further complicated by the rules of the parent fields containing child fields, wherein the absence of the parent field may cause UE to release or retain the child fields (i.e. if the parent is Need R, the child fields are released but if the parent is Need M, the child fields are retained even if they were all Need R fields). This has caused many interpretation issues for both UE and network implementations, and typical mistakes done during specification phase include incorrect handling of need codes (e.g. wrong or forgotten need code, or ambiguous conditional condition with regards to absence of the field). If we consider the frequency of the need codes, Need R is the most commonly used (due to the common use of ENUMERATED{true} – types of fields), while CondX is the second-most used (for complex configuration conditions) and Need M comes as third (with the most common use case being SetupRelease{} – parameterized type), and Need N is most substantially used in combination with the ToAddModList and ToReleaseList – fields. In Table 1 below we have counted (via simple word count) the frequency of occurrence of each case in 38.331 v19.0.0.

| Need code | #Times in RRC | Typical usage |
| --- | --- | --- |
| Need R | 1723 | ENUMERATED {value} |
| Cond | 1250 | - |
| Need M | 925 | SetupRelease {} |
| Need N | 559 | ToAddModList, ToReleaseList |
| Need S | 440 | - |

Table 1. Frequency of occurrence in 5G RRC (based on v19.0.0)

All of this illustrates that it three things:

- Delta signalling is complicated due to Need Codes being complicated, and “delta signalling” only being defined indirectly in RRC specification

- The RRC specification has far fewer instances of delta signalling than it has non-delta signalling (which partly explains also the phenomenon of large message sizes)

- The Need R, Need M and Need N have specific and common use cases

Even though the Need Codes offer essential functionality, the use has created some typical problems such as the assignment of wrong Need Code, forgotten Need Code assignments, mismatches in the assignment of UL vs. DL Need Codes, ambiguities in the specified behaviour of Need S, and the complexity introduced by the conditional conditions. Some companies (e.g. R2-2506957 and R2-2507073) have proposed to either eliminate the need codes entirely or make Need R the “default” need code. While these approaches seem possible, it is still unclear what would be their final impacts and how they would help to keep the signalling size small and avoid typical mistakes. However, the functionality offered by the Need Codes would be needed in the 6G standards as well, even if a different mechanism to using the Need Codes is proposed for 6G.

Observation 4: The functionality offered by the Need Codes would be needed in the 6G standards as well, even if a different mechanism to using the Need Codes is proposed for 6G. Simplifying Need Codes has potential to reduce RRC specification complexity.

Proposal 1: Consider reducing the RRC configuration size by using a modular structure. Discuss “delta signalling” based on detailed proposals.

# 3	RRC Structure and (re)configuration

## 3.1	General considerations for 6G RRC configuration design

To improve on the 5G RRC, the 6G RRC design should be lean: RRC structure should be more scalable and modular (i.e. concise and easily decodable) while also enabling forward-compatibility (as RRC always must provide). To ensure successful 6G, RRC should aim to enable better power efficiency for UEs and networks, while also reduce RRC complexity to provide improved operational latency and smaller typical RRC message size. To achieve this, 6G radio must support fast transitions from power-saving mode (which include both UE and network power-saving operations) to high-performance modes (such as WB/broadband single or multi-layer CA connections with the necessary assistance reference signals). This must be achieved while accounting for the fact that different UE implementations with differing performance characteristics will exist in real-world deployments. Consequently, the system should accommodate both better performing UEs and those performing according to the minimum performance requirements, ensuring that both the network and the UEs benefit when UEs performing better than the minimum.

Therefore, some of the core objectives in designing RRC for 6G could be,

- Efficient Power Saving for UE and Network: Support fast changes in active configuration to enable efficient power saving and reduced configuration overhead.

- Fast Activation of High-Performance Configuration: Ensure rapid activation of RRC configuration that support the highest data rates and/or other performance metrics.

- Reduced Latency During Configuration Transitions: Achieve shorter and more predictable latencies when activating, deactivating, or modifying radio configurations. Improve use of INACTIVE mode to achieve better latency for transition to CONNECTED.

- Reduced Signalling Overhead: Reduce the need for signalling existing configurations repeatedly to allow more robust signalling with less overhead through activation of stored configurations.

Therefore, to achieve the above-mentioned objectives, the design of the 6G RRC configuration needs to undergo some changes with respect to the current design of the 5G RRC configuration.

Figure 5: Aim of 6G RRC: Modular, scalable and sustainable design.

## 3.2	Modular 6G RRC design

### 3.2.1	Definition of modularity

A modular 6G design would mean shaping the RRC signalling around RRC configuration modules (or RRC modules or, simply, modules) that can be encoded and decoded in parallel – i.e. encoding or decoding of module 1 does not require knowledge of module 2 (as also discussed in section 2.2 on modularity definition and usage in 5G).

With modularity, the full RRC configuration of a UE could be viewed as a combination of constituent RRC configuration modules. The aim should be to identify dimensions within the RRC configuration so that each module contains an independent ASN.1 structure so that there are as few linkages to other modules as possible. This form of modularity would allow partial modifications of configurations (modules) and the mapping of RRC configuration components to different RAN functional entities. The modules could have dependent parameters but must allow separating the RRC configuration to parts that are changed at the same time (e.g. L1 configurations, RRM measurements, etc.).

Furthermore, modularity allows for localized failure and recovery, where only part of the RRC configuration fails and must be replaced, while the rest may remain unaffected. This could help to allow mode controlled operation for “partial failures” that were discussed in RAN2#131bis, and could also assist mobility procedures in avoiding full reconfiguration of the UE at the target due to non-comprehended part of the source configuration, as typically such changes would only occur at limited parts of the UE configuration and could thus be localized at a specific module. Figure 6 below illustrates an example of possible configuration modularization structure, with intent to focus on potentially isolated modules.

Figure 6: High-level example of modular design for the 6G RRC configuration.

Observation 5: Building a modular configuration framework for 6G RRC using isolated configuration modules that can be reconfigured without impacting each other may not be always possible, it is still a viable goal.

In terms of practical steps, we would propose to consider the existing Rel-15 NR RRC structure as a starting point so that companies wishing to consider modular approaches would also provide a more detailed exploration of what the modularity would mean to existing NR structure – while this is obviously very unlikely to be exactly the same as what 6GR will bring, it still serves as a good intermediate step where all companies understand how the existing NR RRC worked, and how the modularity would change that.

Proposal 2: RAN2 to discuss modularity based on concrete proposals based in Rel-15 NR RRC structure.

In terms of how to use modules in ASN.1, we see the requirements for a module as the following: For a module, it shall be possible to

- setup, replace and release a module from a configuration

- encode or decode the contents of a module independently from the other configuration

- write the actions for each module operation (i.e. setup, replace, release) in the RRC procedural text

Based on this, modularity could be defined via the use OCTET STRINGs and the parameterized types: Define a parameterized type RRC_Module that allows setup, release and replace actions, and wrap the IE content within OCTET STRINGs. An example of this is shown below in Figure 7.

Figure 7: Definition of a module in ASN.1

Proposal 3: RAN2 to adopt the definition of module that allows the setup, release and replacing of the module contents.

### 3.2.2	Modularizing RRC configuration based on functional view

To modularize the RRC configuration could be viewed as multi-dimensional, with each component addressing specific aspects of network and device interaction. For example, following dimensions could be observed in a typical RRC (re)configuration.

- The PDU session view, is abstracted by Data Radio Bearers (DRBs), focusing on the management of data flows between the user equipment (UE) and the network.

- The Network view divides the UE configuration into two primary components. These are organized based on how network elements handle specific configurations, e.g. similar to how Radio Bearer Config vs. Cell Group Config.

- The MAC and PHY entity view, relates to entity-level dedicated configurations.

- The Cell specific view, which involves configurations for secondary and serving cells (SPCell and SCell) derived from common base containers like ServingCellConfigCommon and ServingCellConfig.

- The Cell specific physical channel view includes cell-specific parameters such as CSI configuration, CORESET, BWP, Search Spaces, TCI states, and physical channel configurations like PDCCH, PUCCH, PDSCH, PUSCH, and PRACH configurations.

- The Measurement view could contain the measurement framework, which includes measurement objects and reporting configurations.

- The Security view could contain CP and UP security keys along with the associated keying material.

The modularization based on the functionality of the module makes it possible to identify cell specific and UE specific modules of an RRC configuration and additionally, helps to make clear module-wise changes based on functional component modules when modifications to a RRC configuration is necessary as an alternative to applying a delta configuration.

The operational limitations imposed by the placement of the modules needs to be considered in defining and dimensioning the modules. If the defined module is at message-level, the overhead would be minimized but flexibility would be low, and at deeper level, the overhead may increase but flexibility would be higher.

Observation 6: A functional view could be used to modularize the RRC configuration. RAN2 should discuss further on the placement of the modules within the RRC structure.

Proposal 4: Study modularization of the 6GR RRC based on a functional view and further discuss the modules to define.

### 3.2.3	Modularity with stored RRC configurations

In 5G RRC (re)configuration is used for sending (almost) the same configuration repeatedly (e.g. when UE stays in the same cell). The 6G control plane should improve latency and energy efficiency without overcomplicating the initial structure.

The storing of RRC configurations even across state transitions and reusing them would reduce signalling overhead, which can help to save power and improve signalling reliability. Adopting a design with reusable, stored configurations as part of 6G RRC could be beneficial in terms of both UE and network performance since it would allow modules of UE configuration to be signalled once and reused afterwards.

5G RRC already has several examples of notable stored, notably in the CSI configuration framework and BWP configuration. As those examples have demonstrated, UEs can activate stored configurations faster than with RRC reconfigurations received via the network and thus allow faster transitions to and from power saving mode with more predictable latency.

Therefore, the potential benefits of a 6G RRC configuration and signalling design based on reusable stored configurations could be:

- Lower latency state transitions: Smaller messages for RRC state transitions using stored configurations enables NW to move UE “fast” to INACTIVE state and, when required, back to CONNECTED. This can allow both network and UE save power by keeping UE more in INACTIVE state.

- Signalling overhead and robustness: By having stored configurations for (typical) initial access cases from INACTIVE, smaller message sizes can be used, which allows network to use more robust MCS due to smaller (DL) message sizes.

- Network and UE efficiency: Network can represent multiple UEs with the same configuration along with a UE-specific (smaller) delta configuration (which also allows UEs to be treated differently if necessary e.g. for field issues). This allows network to save memory.

Figure 8: Benefits of a design with reusable stored RRC configurations.

Stored modular configurations could additionally assist in AI-native operations for RRC by enabling dynamic and efficient configuration selection for current conditions based on prior usage patterns.

Observation 7: Storing RRC configurations even across state transitions and reusing them would reduce signalling overhead and thus saves power. This can allow faster state transitions, better power saving with more predictable latency benefitting both UE and network performance.

Proposal 5: The 6G RRC configuration and signalling design should allow reusable stored configurations based on configuration modules.

Storing multiple configurations optimized for different use cases at the UE, which can be activated in an on-demand manner by the network would be beneficial in 6G. The network can request UE to activate a suitable stored configuration with small overhead based on the UE service requirements, where each stored configuration could consist of a set of functional modules and associated parameters.

The set of stored configurations at a UE could be optimized based on the type of UE (e.g. an eMBB UE or an IOT UE): Different UEs could have different set of stored configurations each optimized for the functionality afforded by the device.

The stored configurations could be optimized based on the needs of a UE:

- For instance, one stored configuration could be tailored for scenarios involving extremely low UE activity, such as single-cell narrowband operation. This configuration could include parameters optimized for minimal resource usage and power consumption.

- Conversely, another stored configuration might be configured for high-performance data throughput, supporting high-bandwidth single-cell or multi-cell (Carrier Aggregation, CA) operations. This configuration could contain parameters aimed at maximizing data rates and resource utilization.

- A basic stored configuration could enable configuring a UE to a default and very basic operational RRC state which could act as a fallback configuration in case of failure.

- Devices with high radio capability could have a stored configuration which enables such high performance at the device.

Figure 9 below illustrates how these configurations could be switched based on the use case.

Figure 9: Example of stored configurations optimized for different use cases.

Proposal 6: Consider multiple stored configurations optimized for different use cases which can be activated in an on-demand manner by the network in designing 6G RRC.

## 3.3	ASN.1 footprint with modular RRC

Keeping the ASN.1 footprint relates to the ASN.1 module definitions (as discussed in section 2.2). However, it will only be possible to fully discuss the footprint once the majority of the configuration (especially L1 configuration) is known. Before that, the main thing RAN2 can discuss is what kinds of ASN.1 module definitions would be feasible. From our perspective, we should retain the current structure with one “common” ASN.1 module that is used by all other modules, but whether this would be as big as in 5G or made more like an independent “general” module could be studied. For example, device families using restricted sets of features could be made to function according to a reduced set of ASN.1 code. A very crude example of this is shown below in Figure 10, but the details would really need much deeper discussion later.

Figure 10: Example of ASN.1 modules for 6GR RRC

Proposal 7: Retain a single “common” ASN.1 module that is used for IMPORTS for other modules. Study whether this module can be made independent from the “main” ASN.1 module defined for RRC messages.

# 4	Conclusions

We have discussed the definition of modularity and RRC configuration structure, with the following observations and proposals:

Observation 1: RRCReconfiguration signalling size varies a lot in practice. Reducing the typical message sizes while avoiding increase to the minimum message sizes would be beneficial.

Observation 2: ASN.1 modularity almost requires a “main” module that is used for IMPORT for other modules.

Observation 3: Using OCTET STRING allows “modularizing” ASN.1 content with the cost of 1-2 bytes for each OCTET STRING.

Observation 4: The functionality offered by the Need Codes would be needed in the 6G standards as well, even if a different mechanism to using the Need Codes is proposed for 6G. Simplifying Need Codes has potential to reduce RRC specification complexity.

Proposal 1: Consider reducing the RRC configuration size by using a modular structure. Discuss “delta signalling” based on detailed proposals.

Observation 5: Building a modular configuration framework for 6G RRC using isolated configuration modules that can be reconfigured without impacting each other may not be always possible, it is still a viable goal.

Proposal 2: RAN2 to discuss modularity based on concrete proposals based in Rel-15 NR RRC structure.

Proposal 3: RAN2 to adopt the definition of module that allows the setup, release and replacing of the module contents.

Observation 6: A functional view could be used to modularize the RRC configuration. RAN2 should discuss further on the placement of the modules within the RRC structure.

Proposal 4: Study modularization of the 6GR RRC based on a functional view and further discuss the modules to define.

Observation 7: Storing RRC configurations even across state transitions and reusing them would reduce signalling overhead and thus saves power. This can allow faster state transitions, better power saving with more predictable latency benefitting both UE and network performance.

Proposal 5: The 6G RRC configuration and signalling design should allow reusable stored configurations based on configuration modules.

Proposal 6: Consider multiple stored configurations optimized for different use cases which can be activated in an on-demand manner by the network in designing 6G RRC.

Proposal 7: Retain a single “common” ASN.1 module that is used for IMPORTS for other modules. Study whether this module can be made independent from the “main” ASN.1 module defined for RRC messages.