3GPP TSG-RAN WG2 Meeting #132	R2-2508363

Dallas, USA, 17th – 21st of November 2025

Agenda item:	10.4

Source:	Nokia, Turkcell

Title:	On the Unified, Reliable and Interruption-free 6G Mobility

WID/SID:	FS_6G_Radio - Release 20

Document for:	Discussion and Decision

# 1	Introduction

RAN 6G Study Item Description (SID) among the others, lists the following objective [1]:

(4) Mobility for 6GR (for all RRC states), including related RRM [RAN2, RAN1, RAN4, RAN3]

During RAN2#131bis discussion the following high-level agreements have been made [2]:

Agreements 1 Study mobility with the following requirements in 6G mobility design: - Minimize interruption time and ensure service continuity (i.e. minimize throughput degradation during mobility). Consider complexity and gains when discussing solutions. - Robustness of mobility procedures - Energy efficiency for both UE and NW 2 Study aspects related to mobility (e.g. early DL/UL synchronization, UE configuration processing, pre-configurations, conditional handover, early CSI acquisition)

In this paper we provide our view on 6G mobility, considering the aforementioned focus areas identified at RAN2#131bis. This paper is a follow up of Nokia’s previous TDoc on 6G Mobility [3].

# 2	Discussion

## 2.1	On the Unification of Mobility and Measurements in 6G

During the discussions at RAN2#131bis a lot has been said about the unification or harmonization of the 6G mobility. However, as usual, the devil was in the details, and it was clear RAN2 companies had different understanding regarding how such unification/harmonization of 6G mobility shall look like. Thus, in the RAN2#131bis meeting notes [2], it is suggested that companies should share more details in the upcoming 3GPP meetings to reach a common understanding what is there to be unified/harmonized (if anything).

In 5G new mobility features have been introduced in each 3GPP release:

- Release 15: Baseline handover

- Release 16: Dual Active Protocol Stack (DAPS) and Conditional Handover (CHO)

- Release 17: Conditional PSCell Addition/Change (CPAC)

- Release 18: L1/L2 Triggered Mobility (LTM), Subsequent CPAC, CHO with conditional PSCell change

- Release 19: Inter-CU LTM, Conditional LTM (CLTM)

- Release 20: Dynamic measurement and reporting change for LTM. LTM SCell activation improvements

As can be noticed, the list of features was enormously lengthy and integrating them well into the Release 15 baseline handover framework was nearly impossible. Thus, almost each of those NR mobility features has its own configuration for the candidate/target cell and related measurements. Most of these also have their own procedure.

Observation 1: The list of mobility features added in consecutive NR releases was extremely lengthy and these were not integrated well with the NR baseline mobility procedure and configurations.

The introduction of L1 measurements in Release 18 for LTM has initiated a lot of discussions in 3GPP on how to handle L1 (new) and L3 (legacy) measurements in parallel and whether these can be somehow integrated when it comes to the configurations or UE requirements. L3 mobility and L3 measurements have been used in many 3GPP generations and were mainly introduced for supporting cell-level mobility. This has happened long before introducing beams and corresponding beam-level mobility, i.e. beam management. LTM has been basically built on top of beam-level mobility as, in one of the implementation options, it relies on L1 measurements for cell switch triggering. This all implies that in NR any proper scheme to unify/merge/reuse these two measurement types was not considered as these were done in separate releases and for slightly different purposes.

Observation 2: Separate measurement configurations for L3 mobility measurements and L1 measurements (introduced for LTM) have increased the NR specification complexity related to interworking between these types of measurements.

Thus, our 6G goal should be to identify the NR functionalities (out of the list provided above) which are worth considering for 6G mobility with respect to key mobility performance parameters and obviously study some additional means wherever needed. Then a unified/single mobility framework should be defined from the key components that would be selected. Similar actions should be performed for measurement related part of 6G study.

When speaking about the unification/harmonization/single procedure, one can identify at least the following areas:

- Unification of measurement and reporting configurations (e.g., a single Measurement Object irrespective of the purpose of the measurements)

  - No need to tightly associate the measurements with candidate configurations, etc.

- Unification of mobility measurement requirements (RAN4-related with the key impact on 6G.133 specification)

- Unification of mobility configurations (e.g., configurations are decoupled from the actual features)

  - As the mobility features for first release of 6G are expected to already address interruption time reduction, robustness and throughput performance improvement during mobility, single framework that consists of modular (expandable) configurations for these features with common configuration management is preferred.

  - The framework defined for first release should be still scalable and it should be possible to easily integrate any future extensions with the baseline functionality.

  - For the single framework for mobility configuration, it is preferred to consider bottom-up approach where the framework is developed for the minimum required functionality, and it is extended for additional scenarios.

  - The unified mobility procedure shall ensure an uninterruptable HO procedure (such as RRC Reconfiguration with sync in 5G) is supported.

Proposal 1: Unification of mobility measurements shall involve measurement and reporting configurations. It is for RAN4 to decide if measurement requirements can be also unified in 6G.

Proposal 2: Unification of mobility shall be done such that different KPIs can be individually maximized for different use-cases. Those involve increased robustness, high throughput during mobility, decreased interruption time as well as energy efficiency.

As typically a (good) picture is worth a thousand words, below we provide a high-level figure explaining how this unification of mobility configuration may look like. In Fig. 1 a simple structure is shown, where the UE is provided with a Candidate Configuration for the target cell. The purple Candidate Configuration box may comprise the typical components, such as RRC configuration, configuration’s identity, candidate cells identifier, etc. It is a mandatory part of the unified configuration as obviously without these information, the UE would be unable to access the target cell. Candidate Configuration is linked with multiple optional boxes, some of which are also shown in Fig. 1. Among such boxes one can find for example: Execution Information, details on Interruption Reduction or Actions to protocol. Execution Information component can tell the UE if the Candidate Configuration is to be applied immediately (i.e. like in NR Release 15 HO), whether there is any condition associated with the execution or if the execution is subject to separate triggering (e.g. MAC-based, like in Release 18 LTM).

Fig. 1. Exemplary structure of unified mobility configuration.

Interruption Reduction component defines if the handover shall be executed with special actions aimed at decreasing the time without any UL/DL transmissions. Namely if early synchronisation should be applied when performing a cell change towards this particular candidate. Eventually, Actions to protocols may instruct the UE what to do with certain parts of its source cell configuration (e.g. whether to reset L2 protocols, etc.).

By keeping such modular design, there is no need to configure separate mobility features in parallel. Instead, the UE might be simply told what to perform (i.e. L2 reset, early sync, MAC-based triggering) for each of the candidates.

## 2.2	On Interruption Time Reduction

During the RAN2#131bis discussions it was considered whether 6G mobility framework should guarantee an interruption time of 0 ms or whether it is sufficient to be close to 0 ms (while it is for further study what “close to” exactly means). 3GPP has had this sort of discussion when each generation of mobile standard was defined from scratch. Thus, we will not reiterate all the details that have been extensively discussed when LTE or NR were designed, such as how to calculate the interruption time and what are the exact components that contribute to the overall interruption length (the most recent analysis of interruption time components can be found in [4], Annex G). However, we would like to emphasize that an NR Release 16 study and specification work has shown that achieving genuine 0 ms interruption time has a non-negligible cost while it turned out not to be commercially successful so far.

Observation 3: DAPS specification in NR Release 16 has proven that achieving 0 ms interruption time during mobility is complex while the commercial success of this feature has been so far limited.

Thus, RAN2 should consider this as a lesson learnt and avoid similar mistakes in 6G. As has been correctly observed during RAN2#131bis discussions, most of the services do not need extremely low interruption time during mobility and what matters more is the minimization of throughput degradation (see the RAN2#131bis notes and agreements [2], quoted also in the Introduction). Furthermore, a significant percentage of handovers in real networks occur when there is actually no urgent data pending in the buffers for the UE. Thus, reducing the interruption to 0 ms would be essential in a relatively small subset of handover occurrences.

Observation 4: According to many opinions shared at RAN2#131bis, minimization of throughput degradation during mobility seems to be more important than reducing the interruption time to 0 ms.

Observation 5: A significant percentage of handovers in real networks occur when there is actually no urgent data pending in the buffers for the UE, so reducing the interruption time is needed for a small subset of overall HO events while it might be critical for some use-cases.

Interruption reduction can be achievable thanks to different NR functionalities, such as early UL or DL synchronization  (LTM-like), early decoding (if the target’s configuration is sent is slightly earlier than in NR Release 15 HO) or RACH-less handover. None of the listed solutions will reduce the interruption time to genuine 0 ms. However, they offer relatively significant reduction (e.g., in intra-FR1 LTM the interruption is reduced to ~20 ms), and are not excessively complex like DAPS.

Considering the aforementioned aspects, we propose the following:

Proposal 3: RAN2 studies the ways to reduce the interruption time, but not to 0 ms. The considered solutions should not be complex in order to increase their chances of commercial deployment.

## 2.3	On Minimisation of Throughput Degradation

As has been briefly highlighted in section “2.2 Interruption Time Reduction”, the equally important aspect is to minimize the user throughput reduction associated with each cell change as this impacts end-to-end user experience. Features such as early CSI acquisition introduced in NR Release 19 for LTM and L3 handover help to minimise the negative impact on the user’s throughput during mobility. Thus, these feature and potentially other new features, need to be studied for 6G mobility under the single framework.

Another mobility use case where UE’s throughput may be reduced is the handover with carrier aggregation (CA). In NR specification when the target’s configuration contains also secondary cells, it is possible to reduce the time needed for  secondary cells activation by means of direct activation in the RRC configuration. It is an attractive solution which shall be considered for 6G along with other potential improvements, such as early synchronisation to the target secondary cells.

As throughput reduction during mobility directly impacts the service continuity, RAN2 needs to study ways to reduce the impact on user’s throughput during mobility in addition to interruption time reduction. To begin with, the areas described above should be taken into account.

Proposal 4: RAN2 to study methods to minimise throughput reduction due to mobility, including early CSI acquisition introduced in Rel-19 and other new 6G solutions.

## 2.4	On Reliability and Robustness

Reliability and robustness was not considered thoroughly during the RAN2#131bis discussions [2]. Most likely not because this is not a problem at all, but rather as the focus has been on reducing the interruption time and trying to understand what “unification” actually means.

The mobility robustness is the ability to perform the handover without failures and the ability to restore the link immediately when there is a suitable neighbouring cell.

The 5G baseline handover (according to Release 15) is significantly robust [6]. However, we have investigated different areas where the handover robustness could be still improved. It is commonly known that the two main mobility robustness issues are:

- Too Early (TE) handover, which is a handover initiated too early. As a result, the handover will not be successful and typically HOF will be declared.

- Too Late (TL) handover, which is a handover that could not be triggered on time, usually due to decreasing serving link quality. It will typically lead to RLF in the source cell.

Below we assume that the network typically can send the HO command as late as possible to avoid TE issues. Thus, we concentrate on listing which components causing TL handovers can be studied for 6G.

Measurement and reporting:

In order to avoid TL handover, the measurements shall be done in a timely manner, and the report has to be triggered when appropriate and delivered quickly and reliably. RAN2 should study features that could improve the reliability of the measurement report used to determine the mobility decision. However, the improvements to measurements and reporting should not result in increased signalling.

Conditional execution of a pre-configured candidate cell configuration is a solution to circumvent the reliability of measurement delivery, as the UE evaluates the measurements and takes corresponding decision by itself, considering the condition provided by the NW. However, in conditional execution the network loses some level of control and cannot decide on the exact time of the HO execution. This in practice means that the NW is unsure when the UE will appear at the target cell.

Observation 6: Conditional execution of cell change leads to NW’s uncertainty regarding the time when the UE will appear in the target cell.

Network preparation delay:

In order to avoid TL handover, the network shall send the HO command with minimal delay to the UE. Network needs to send HO request and get an answer for inter-gNB scenario for preparing the HO, which can be denoted as “network preparation delay”. It might be investigated in 3GPP how this message exchange procedure can be done in a faster way.  However, that is likely more in the RAN3 scope, so RAN2 might inform RAN3 if this is found to be a promising area to be studied.

Preconfiguration of candidate cells is a potential solution to avoid network preparation delay. However, the reduction of the preparation time via the pre-configuration of the UE (via CHO- or LTM-like approaches) leads to challenges in network’s side such as handling NAS interactions, that may lead to the necessity to reconfigure the UE (for example with the addition of a DRB). This kind of event would lead to the cancellation of the prepared Handover in the target gNB and a reconfiguration in the UE. This may cause further delays.

However in some specific scenario, like NTN, when the time of the handover is known in advance, RAN2 needs to consider the use of pre-configured handover.

Observation 7: Preconfiguration of candidate cells can reduce the NW preparation delays and improve the handover robustness. However, it can result in excessive resource consumption and excessive inter-node signalling when the UE needs to be reconfigured prior to handover execution.

HO command signalling:

The handover command sent to the UE is in 5G an RRC Reconfiguration message. It is a big RRC message that has to be sent in cell edge conditions. A reduction in the size of the HO message could improve the reliability through shortening the length of the transmission. This can be discussed after further clarity on the RRC protocol in control plane discussions.

Observation 8: NR RRC message, used to command the handover is typically large in terms of its size and needs to be successfully delivered to the UE in deteriorating cell edge radio conditions.

Loss of coverage:

It can happen that the UE loses the coverage of the serving cell without having any neighbour cells to connect to.

If the lack of coverage leads to a failure, UE would have to perform re-establishment. In case of failure, the UE should be able to recover quickly. In NR, the re-establishment procedure is done in two steps, while the Resume procedure can be done in one step. We think it is beneficial if the re-establishment time can be reduced, for example by applying one-step re-establishment procedure.

Proposal 5: RAN2 is asked to study the aspects that cause degradation of mobility robustness. The list of aspects includes: measurements and reporting, network preparation delay, network resource consumption, handover command signalling, recovery after the loss of coverage.

Proposal 6: RAN2 studies the solutions which improve the mobility robustness. In the study the complexity to both the UE and NW side is considered.

## 2.5	Energy Efficiency for Mobility

In order to achieve OPEX savings for 6G, one key aspect is enabling network energy savings. There are various proposals how to achieve such network energy savings that RAN1 has agreed to study [5]. Those include increased SSB periodicity, on-demand synchronisation signals, multi-carrier/cells/TRPs for NES and on-demand/or periodic SIB1 transmission. Out of those, especially the default SSB periodicity increase, and the use of on-demand synchronisation signals can be critical for mobility in 6G. It is clear that the network can transmit the SSBs more frequently. In any case, before agreeing on the default SSB periodicity RAN2 needs to be involved to ensure key mobility performance indicators are not impacted, e.g. due to the increased SSB periodicity.

Regarding the UE energy efficiency (EE), ensuring at least the same battery life as in the previous generations will be a key challenge, considering the new functionalities in 6G and their respective energy cost. To achieve this RAN1 has agreed to study [5] DL Wake-Up Signal (WUS) based energy efficiency improvements and similarly measurement prediction to decrease UE measurements burden (i.e. similar to NR Release 19 and 20 design). In all of these features the high mobility performance shall be ensured through maintaining the reliability of the measurements with strict and testable RAN4 requirements.

Proposal 7: For NES and UE energy saving features that may impact mobility, evaluations shall be done to verify whether key performance indicators for mobility (such as robustness) are not negatively impacted.

# 3	Conclusion

This document has proposed the following:

Proposal 1: Unification of mobility measurements shall involve measurement and reporting configurations. It is for RAN4 to decide if measurement requirements can be also unified in 6G.

Proposal 2: Unification of mobility shall be done such that different KPIs can be individually maximized for different use-cases. Those involve increased robustness, high throughput during mobility, decreased interruption time as well as energy efficiency.

Proposal 3: RAN2 studies the ways to reduce the interruption time, but not to 0 ms. The considered solutions should not be complex in order to increase their chances of commercial deployment.

Proposal 4: RAN2 to study methods to minimise throughput reduction due to mobility, including early CSI acquisition introduced in Rel-19 and other new 6G solutions.

Proposal 5: RAN2 is asked to study the aspects that cause degradation of mobility robustness. The list of aspects includes: measurements and reporting, network preparation delay, network resource consumption, handover command signalling, recovery after the loss of coverage.

Proposal 6: RAN2 studies the solutions which improve the mobility robustness. In the study the complexity to both the UE and NW side is considered.

Proposal 7: For NES and UE energy saving features that may impact mobility, evaluations shall be done to verify whether key performance indicators for mobility (such as robustness) are not negatively impacted.

# References

- RP-251881 Study on 6G Radio 3GPP TSG RAN Meeting #108 Prague, Czechia, June 9-13, 2025

- RAN2#131bis Chairlady Notes 3GPP TSG-RAN WG2 Meeting #131bis Prague, Czechia, 13th – 17th of October 2025

- R2-2507385 Initial Thoughts on 6G MMM (Mobility, Measurements and Migration) 3GPP TSG-RAN WG2 Meeting #131bis Prague, Czechia, 13th – 17th of October 2025

- 3GPP TS 38.300 NR and NG-RAN Overall description; Stage-2, v18.7.0, October 2025

- RAN2#122bis Chair Notes 3GPP TSG-RAN WG1 Meeting #122bis Prague, Czechia, 13th – 17th of October 2025

- H. Martikainen, I. Viering, A. Lobinger and T. Jokela, On the Basics of Conditional Handover for 5G Mobility 2018 IEEE 29th Annual International Symposium on Personal, Indoor and Mobile Radio Communications (PIMRC), Bologna, Italy, 2018, pp. 1-7, doi: 10.1109/PIMRC.2018.8580946