3GPP TSG-RAN WG2#132											   R2-2508390

Dallas, USA, Nov. 17th – 21st, 2025

Agenda Item:	8.2.2

Source: 	NEC

Title:	        Ambient-IoT remaining issues

Document for:	 Discussion

# Introduction

In this paper, we address remaining issues of ambient IoT MAC layer and propose corresponding text proposals.

# Discussion

## 2.1 New issue 6-3: Paging ID type indication in paging message

During the RAN2#131bis discussion, it has the overall guidance below.

Overall guidance

=>	After December, NBC changes should be avoided as much as possible similar to NR MAC process.

In TS 23.003 [2] and 23.369 [3], structure of AIoT Device Permanent Identifier and Filtering Information are specified. Based on the structure, there is no distinguish field to differentiate whether it is AIoT Device Permanent Identifier or Filtering Information in Paging ID field in A-IoT Paging message. Paging ID type, i.e. choice of Single Device or Group Devicels, is specified in RAN3. Hence, reader knows the information and it is reader’s duty to indicate the corresponding information to the device by A-IoT Paging message.

In TS 33.369 [1], protection of AIoT device identifier privacy could be done for single device inventory and group device inventory. Whether such protection is used for paging ID should also be indicated. Currently, there is no field to indicate the protection usage of corresponding ID.

Since NBC changes should be avoided after December meeting, RAN2 should add the fields to indicate paging ID type and protection related information if upper layer specification doesn’t add corresponding information during this meeting. If in the future meeting, upper layer specification decides to add the corresponding information, we could only change our procedure by reserving the corresponding bits field as dummy in MAC specification to avoid NBC changes.

- Add fields in A-IoT Paging message to indicate paging ID type and protection related information if upper layer specification doesn’t add corresponding information during this meeting.

- Note: It can avoid NBC changes in MAC specification no matter whether the corresponding information is added or not in upper layer specification after December meeting. If the corresponding information is added in upper layer specification after December meeting, RAN2 can only change the fields as reserved bits.

## New issue 6-2: CFA inventory response retransmission

RAN1 has specified the timing (time duration) between the CFA paging message and the responding D2R Upper Layer Data Transfer message. This timing was defined without considering the time required for the authentication procedure. In fact, the authentication procedure, specifically the calculation of RESAIOT, takes a significant amount of time. However, under the current TS, the timing between the CFA R2D message and the responding D2R message is too short. This may result in “no upper layer data available” by the time the D2R Upper Layer Data Transfer message needs to be transmitted after the CFA paging message.

Considering time need for authentication calculation, the timing between CFA R2D and responding D2R message may be too short for the upper layer data to be available.

In that sense, padding will be transmitted in D2R responding message. However, even the delayed inventory report message is generated later, it will not be retransmitted to reader based on current RAN2 MAC procedure.

In such case, D2R Upper Layer Data Transfer message retransmission due to delayed upper layer data available for initial transmission should be supported for CFA inventory. To support retransmission, we have the following text proposal #2.2.

- If RAN2 reaches a consensus on Observation 1, for CFA inventory procedure, D2R Upper Layer Data Transfer message retransmission due to “upper layer data not available” at the time of initial transmission should be supported. The retransmission is scheduled by R2D Upper Layer Data Transfer message.

- Adopt the text proposal #2.2;

- If RAN2 does not reach a consensus on Observation 1, send LS to RAN1 to confirm: whether the design of TR->D takes into account the authentication performed in the upper layer.

- Draft LS is provided in the Annex.

### text proposal #2.2

------------------Start of text proposal #2.2------------------

5.4.3	R2D message reception

Upon reception of an R2D Upper Layer Data Transfer message, the A-IoT MAC entity shall:

1>	if the device has a stored AS ID and the R2D Upper Layer Data Transfer message is addressed to the device (i.e., the value of AS ID field is identical to the stored AS ID):

2>	if the Choice Indication field indicates that the Data SDU field is included (i.e., CI field set to 1):

3>	forward the upper layer data SDU in the Data SDU field to upper layers;

3>	initiate the following D2R message transmission, as specified in clause 5.4.2;

2>	else if the Choice Indication field indicates that the Received Data Size field is included (i.e., CI field set to 0):

3>	if the Received Data Size field is set to 0:

4>	initiate the D2R message transmission procedure as specified in clause 5.4.2;

3>	else:

4>	initiate the D2R segmentation procedure using this information as specified in clause 5.4.4;

1>	else if the device has no stored AS ID, and if CFA procedure has been performed in the current procedure:

2>	if the Choice Indication field indicates that the Data SDU field is included:

3>	set AS ID to the value indicated by the AS ID field and store the AS ID;

3>	forward the upper layer data SDU in the Data SDU field to upper layers;

3>	initiate the following D2R message transmission, as specified in clause 5.4.2.

2>	else if the Choice Indication field indicates that the Received Data Size field is included and the Received Data Size field is set to 0:

3>	set AS ID to the value indicated by the AS ID field and store the AS ID;

3> perform the D2R message transmission procedure as specified in clause 5.4.2.

------------------End of text proposal #2.2------------------

# Conclusion

Observation 1(New issue 6-2):	Considering time need for authentication calculation, the timing between CFA R2D and responding D2R message may be too short for the upper layer data to be available.

Proposal 1(New issue 6-3):	Add fields in A-IoT Paging message to indicate paging ID type and protection related information if upper layer specification doesn’t add corresponding information during this meeting.

	Note: It can avoid NBC changes in MAC specification no matter whether the corresponding information is added or not in upper layer specification after December meeting. If the corresponding information is added in upper layer specification after Decembermeeting, RAN2 can only change the fields as reserved bits.

Proposal 2(New issue 6-2):	If RAN2 reaches a consensus on Observation 1, for CFA inventory procedure, D2R Upper Layer Data Transfer message retransmission due to “upper layer data not available” at the time of initial transmission should be supported. The retransmission is scheduled by R2D Upper Layer Data Transfer message.

	Adopt the text proposal #2.2;

Proposal 3(New issue 6-2):	If RAN2 does not reach a consensus on Observation 1, send LS to RAN1 to confirm: whether the design of TR->D takes into account the authentication performed in the upper layer.

	Draft LS is provided in the Annex.

# References

[1] 3GPP TS33.369-10: "Security aspects of ambient IoT services in 5G".

[2] 3GPP TS23.003-j30: "Numbering, Addressing and Identification".

[3] 3GPP TS23.369-j10: Architecture support for Ambient power-enabled Internet of Things.

# Annex

3GPP TSG-SA3 Meeting #132	R2-25xxxxx

Dallas, USA, Nov. 17th - 21st, 2025

Title:	[Draft] LS on Timing Offset of D2R Responding Message for A-IoT CFA Paging Containing Security Parameter

Response to:	-

Release:	Release 19

Work Item:	Ambient_IoT_solutions

Source:	[to be RAN WG2]

To:	RAN WG1

Cc:	CT WG1, SA WG3

Contact Person:	TBD

- Name:			TBD

- E-mail Address:	TBD

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: None

1. Overall Description:

RAN2 is discussing the need of D2R retransmission for A-IoT CFA paging containing Security Parameter. Currently, D2R retransmission for A-IoT CFA paging is not specified in RAN2.

RAN1 specified the timing offset between A-IoT CFA paging message and responding D2R message. If NAS message is not available at the time for D2R transmission due to time required for authentication procedure, D2R retransmission for delayed NAS message will be needed.

RAN2 would like to ask RAN1 about whether the timing offset considers the time required for authentication procedure and whether the time duration is enough for NAS message generation considering authentication procedure.

2. Actions:

To RAN1:

ACTION: RAN2 respectfully asks RAN1 about whether the timing offset considers the time required for authentication procedure and whether the time duration is enough for NAS message generation considering authentication procedure.

3. Dates of Next RAN2 Meetings:

RAN2#133				9th to 13rd February 2026			Gothenburg, SE

RAN2#133bis				13th to 17rd April 2026				EU