3GPP TSG-RAN WG2 Meeting #132	R2-2508420

Dallas, US, 17th – 21st November 2025

Agenda item	: 9.3.2

Source	: Tejas Networks

Title	: Discussion on RRM measurement prediction

Document for	: Discussion

# 1. Introduction

The study on AI/ML mobility in NR were concluded in RAN2#131 in the scope of Rel-19 SI. The study outcome is captured in TR 38.744 V1.2.0 [3]. In this paper, we discuss various aspects of the LCM framework and measurement event prediction in the context of UE-sided model use cases.

# 2. Discussion

The RRM specification impacts are captured in TR 38.744 section 6, and few of the potential impacts are discussed in detail below.

- 2.1	 LCM functionality management

From Rel-19 study conclusions: - For both RRM measurement prediction and measurement event prediction, specify signalling and protocol aspects to enable LCM functionality management for UE sided model including [RAN2]: - UE capability request and response procedure - Applicability report for both full and partial configuration approach - Inference configuration and inference report - Performance monitoring performed in UE or network side, based on which network can manage UE sided functionality - UE sided data collection - For measurement event prediction: - Measurement event A1~A6 are in the scope in this release Note 2: Data transfer and model delivery for UE sided model are out of scope of this WI. And model related choices can be up to UE’s implementation.

The work item objective states that the LCM frame work should be, as far as possible, be based on the Rel-19 AI/ML for NR air interface work. And Rel-19 AI/ML for air interface work item considered two different types of approach for LCM procedures. Functionality based LCM and model-based LCM. AI/ML mobility study item concluded to consider only functionality-based LCM, that is, model-based LCM is not considered. In functionality-based LCM, network indicates activation/deactivation/fallback/switching of AI/ML functionality via 3GPP signalling (e.g., RRC, MAC-CE, DCI). Models 	may not be identified at the Network, and it is up to UE’s implementation to decide on model related choices.

Observation 1: Only functionality-based LCM is considered for LCM procedures.

Observation 2: Model related choices are up to UE implementation.

The study on AI/ML based mobility during Rel-19 proposed RRM and measurement event prediction for normative work. Additionally, the following scenarios and/or sub-cases are recommended for normative work:

- For UE sided model (RRM and measurement event prediction), intra-frequency temporal domain case A, intra-frequency temporal domain case B and inter-frequency domain prediction for co-located case;

- For network sided model (RRM prediction), at least RRM sub-case 2 of intra-frequency temporal domain case A for inference input report and all scenarios and sub-cases for data collection.

Proposal 1: RAN2 should specify the handling of LCM for all agreed scenarios and/or sub-cases.

    - Cluster-based prediction

Rel-19 Study conclusions: - It is up to UE’s implementation to decide on model related choices including cluster-based vs single-cell-based approach, RRM sub-use cases and OW length - Both cluster approach (where measurement results from more than one cells are used as input to the model) and single cell approach (where measurement results from single cell are used as input to the model) were used by different companies. Both approaches are valid implementations.

As per the Rel-19 study, using a group of measurement values as input for prediction can improve RRM measurement accuracy compared to using a single measurement. And the Rel-19 study concluded that, both cluster approach (where measurement results from more than one cells are used as input to the model) and single cell approach (where measurement results from single cell are used as input to the model) are valid implementation. Furthermore, it is up to UE’s implementation to decide on model related choices including cluster-based vs single-cell-based approach.

Cluster approach example as per TR 38.744,

  - When measurement from single cell in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR and co-located cell in another carrier frequency.

  - When measurement from a group of cells in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR and co-located cell in another carrier frequency.

In line with the above conclusions, for a UE configured to predict the measurement values for a specific cell or frequency, the AI/ML model may use actual measurement values from another cell or frequency along with predicted measurement values from another cell or frequency as inputs to the model. Cluster based approach can benefit from incorporation of both actual and predicted measurement results, as a result enhance the model’s ability to generalize and improve performance in predicting the target measurements.

Proposal 2: Measurement prediction can be based on actual measurement values or predicted measurement values or combination of predicted and measured values.

In scenarios where cluster-based approach is applied, measurement prediction may depend on inputs derived from other cells or frequencies, which can include either actual measurement values or predicted measurement values. Therefore, how to handle LCM in such cases needs to be clearly defined to ensure consistency of prediction and to avoid unnecessary fallback or model switching.

Proposal 3: RAN2 should specify the handling of LCM for cluster-based approaches, where predicted measurements are used as inputs for inference.

- 2.2	Measurement event prediction

For intra-frequency temporal domain case A, one or more predicted measurement results across a prediction window can be reported. This predictive reporting enables the network to anticipate future events, which enhances the accuracy and timeliness of measurement reporting. Furthermore, allows the network to prepare in advance for mobility related actions, significantly reducing handover delays and failures. Rel-19 study on AI/ML mobility have shown that predicting future measurement instances benefits handover performance. Additionally, anticipation of measurement events can help network to make informed and proactive mobility decisions, leading to improved robustness, fewer unnecessary handovers, and better user experience. The prediction of future events can also reveal which target cell is likely to be optimal at a future time. This information can be valuable not only for mobility management but also for efficient resource preparation at the network side. RAN2 may consider extending this further to include prediction of future events, as this can enable shortlisting of potential target cells in advance. And UE may provide additional information to the network (e.g., candidate cells, confidence levels) to further optimize handover.

Observation 3: Event prediction for future time instances improves handover performance by improving robustness, fewer unnecessary handovers, and better user experience.

Observation 4: Prediction of future events can provide information about the future target cell.

Proposal 4: Based on future event prediction, RAN2 should consider shortlisting potential target cells for handover.

# Conclusion

Observation 1: Only functionality-based LCM is considered for LCM procedures.

Observation 2: Model related choices are up to UE implementation.

Observation 3: Event prediction for future time instances improves handover performance by improving robustness, fewer unnecessary handovers, and better user experience.

Observation 4: Prediction of future events can provide information about the future target cell.

Proposal 1: RAN2 should specify the handling of LCM for all agreed scenarios and/or sub-cases.

Proposal 2: Measurement prediction can be based on actual measurement values or predicted measurement values or combination of predicted and measured values.

Proposal 3: RAN2 should specify the handling of LCM for cluster-based approaches, where predicted measurements are used as inputs for inference.

Proposal 4: Based on future event prediction, RAN2 should consider shortlisting potential target cells for handover.

# References

- RP-251968, Revised WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR, Beijing, China, Sept. 15-18, 2025.

- RP-251965, Status Report to TSG, Beijing, China, Sep. 15-18, 2025.

- 3GPP TR 38.744 V1.2.0 (2025-9)