3GPP TSG-RAN WG2 Meeting#132	    			                                                        R2-2508490

Dallas, USA, Nov. 17th – 21th, 2025

Agenda item:	9.2.2

Source:	ETRI

Title:	Discussion on information between UE reader and gNB

Document for:	Discussion and Decision

Introduction

In the last RAN2 meeting, there was an initial discussion on resource configuration in A-IoT Topology 2. As a result of the discussion, the following agreements were made[1].

Agreements

- gNB will provide AIoT information to the UE reader via RRC dedicated signalling based on NGAP AIoT information received from CN.  FFS what information

2.	gNB provides AIoT resource information to be used for AIoT interface transmissions to the reader via RRC dedicated signaling.   The resource allocation can be triggered by CN request.   UE reader may provide assistance information related to AIoT transmissions.  FFS what the information may be useful to be provided to gNB.  It is up to gNB how the information is used.  It is understood that assistance information is not mandated for the gNB to determine resource allocation.

3. 	the reader schedules the AIoT interface transmissions within the resources allocated by the gNB.

In this paper, we discuss the detailed A-IoT procedures for inventory and command services. We then present our views on the information exchange between the UE reader and the gNB in A-IoT Topology 2.

Discussions

Procedure for Inventory service

For inventory service in topology 2, following procedure was proposed as an RRC-based solution during SI phase[2].

Figure 1. Message flow for A-IoT Inventory in Topology 2 - RRC-based solution

RRC Inventory request

The purpose of the Inventory request procedure is to request the gNB to perform A-IoT paging over the A-IoT radio interface. According to RAN3 work, the gNB receives following information from the CN for inventory request:

- Message type;

- AIOTF Identifier;

- A-IoT Correlation Identifier;

- Inventory Request Transfer;

Inventory Request Transfer;

A-IoT Correlation Identifier;

A-IoT Device Identification Requested;

Requested Service Area Information;

Inventory Assistance Information;

    - Expected D2R Message Size, Approximate Number of Target A-IoT Devices and Time Interval for report,

Follow on Command Indication(optional).

Among above parameters, we discuss which information can be included in RRC Inventory request via step 1c.

The Message Type is used for identifying the NGAP message between the gNB and the CN; therefore, it is not necessary to include this parameter in the RRC Inventory request message.

The AIOTF Identifier is a unique identifier of an AIOTF, and the A-IoT Correlation Identifier uniquely identifies an A-IoT session within an AIOTF.

If we consider a scenario where a gNB is connected to multiple AIOTFs (e.g., AIOTF#1 and AIOTF#2) and multiple UE readers (e.g., UE Reader#1 and UE Reader#2) are served by the gNB, as illustrated in Figure 2, the AIOTF Identifier and A-IoT Correlation Identifier can be used to identify the AIOTF and its corresponding A-IoT session. However, such identification can also be achieved through a mapping function at the gNB, such as a Service Mapping Identifier. If the mapping tables between the AIOTF Identifier and A-IoT Correlation Identifier and the Service Mapping Identifier are maintained at the gNB, the Service Mapping Identifier can serve as a unique identifier to distinguish an AIOTF and an A-IoT session within that AIOTF.

Figure 2. A scenario with multiple AIOTFs and multiple UE readers

The A-IoT Device Identification Requested is used to determine the paging length and paging ID in the MAC PDU of the A-IoT Paging message; therefore, it should be included in the RRC Inventory request message.

The Inventory Assistance Information is important for resource configuration, and thus it is necessary to determine whether this information should be delivered to the UE reader.

- If the gNB configures radio resources over the A-IoT interface, the resource configuration is transmitted to the UE reader as part of the parameter of the RRC Inventory request message, instead of the Inventory Assistance Information.

- Conversely, if the UE reader configures the radio resource, the Inventory Assistance Information should be transmitted to the UE reader.

However, since it is expected that radio resources will be managed by the gNB, the Resource Configuration should be included in the RRC Inventory request message.

The Follow-on Command Indication can be useful for identifying subsequent operations on the A-IoT interface when the UE cannot perform simultaneous operations on both the Uu and A-IoT interface.

Proposal 1.  The following information is proposed to be included as parameters in the RRC Inventory request:

- Service mapping identifier;

- A-IoT Device Identification Requested;

- Resource configuration on A-IoT interface (for D2R transmission);

- Follow-on Command Indication(optional).

RRC Inventory response

The purpose of the Inventory response procedure is to enable the gNB to provide response information of the requested Inventory request to the A-IoT CN node. According to RAN3 work, the gNB transmits the following information to the CN for the Inventory reponse:

- Message type;

- AIOTF Identifier;

- A-IoT Correlation Identifier,

- Inventory Response Transfer

A-IoT Correlation Identifier,

Criticality Diagnostics (optional).

To fill the parameters of Inventory response message in step 2b, we discuss which information can be included in RRC Inventory response via step 2a.

Similar to the RRC Inventory Request message, the AIOTF Identifier and the A-IoT Correlation Identifier can be replaced by a Service Mapping Identifier.

Proposal 2. The following information is proposed to be included as parameters in the RRC Inventory response:

- Service mapping identifier.

RRC Inventory report

The purpose of the Inventory Report procedure is to enable the gNB to provide inventory report related information to the A-IoT CN node. According to RAN3 work, the gNB transmits following information to the CN for inventory report:

- Message type;

- AIOTF Identifier;

- A-IoT Correlation Identifier;

- Inventory Response Transfer

A-IoT Correlation Identifier;

Global gNB ID;

Reader Report List;

Inventory Complete Indication.

To fill the parameters of Inventory report message in step 4a or 4b, we discuss which information can be included in RRC Inventory Report via step 4a or 4b.

Similar to the RRC Inventory Request message, the AIOTF Identifier and A-IoT Correlation Identifier can be replaced by a Service Mapping Identifier.

The Reader Report List consists of the UE Reader Identifier and the A-IoT Device Report List. From the gNB perspective, the RNTI and AS ID can serve as unique identifiers for a UE reader and an A-IoT device, respectively. Therefore, it is necessary for the gNB to maintain a mapping table between the RNTI and AS ID, allowing it to identify the relationship between each UE reader and its associated A-IoT devices. The AS ID can also be used for resource configuration in subsequent command services.

The Inventory Complete Indication is used to notify that no follow-on command is to be transmitted, or that all follow-on command transmissions have been completed. Since this information may assist the gNB in resource configuration and overall operation, it should be included as a parameter in the RRC Inventory Report.

Proposal 3. The following information is proposed to be included as parameters in the RRC Inventory Report:

- Service mapping identifier;

- AS ID for A-IoT Device Identification;

- Inventory Complete Indication.

Procedure for Command service

For command service in topology 2, following procedure was proposed as an RRC-based solution during SI phase[2].

Figure 3. Message flow for A-IoT Command in Topology 2 - RRC-based solution

RRC Command Request

The purpose of the Command Request procedure is to request the gNB to transfer a command to the A-IoT device over the A-IoT radio interface. According to RAN3 work, the gNB receives the following information from the CN for the command request:

- Message type;

- AIOTF Identifier;

- A-IoT Correlation Identifier;

- RAN A-IoT Device NGAP ID;

- Command Request Transfer;

A-IoT Correlation Identifier;

RAN A-IoT Device NGAP ID;

A-IoT NAS PDU;

Command Assistance Information(optional).

Among above parameters, we discuss which information can be included in RRC Command Request via step 2c.

Similar to the RRC Inventory Request and RRC Inventory Report messages, the AIOTF Identifier and A-IoT Correlation Identifier can be replaced by a Service Mapping Identifier.

The RAN A-IoT Device NGAP ID can be translated into the appropriate AS ID using the mapping table maintained at the gNB.

The Command Assistance Information field in the Command Request Transfer optionally contains an estimate of the expected D2R message size. Similar to the Inventory Assistance Information in Step 1a of Figure 1, the gNB uses this information to configure radio resources over the A-IoT interface and transmits the corresponding configuration to the UE reader.

Proposal 4. The following information is proposed to be included as parameters in RRC Command request;

- Service mapping identifier;

- AS ID;

- Resource configuration on A-IoT interface (for D2R transmission).

RRC Command Response

The purpose of the Command Response procedure is to enable the gNB to provide command response information of the requested command request to the A-IoT CN node. According to RAN3 work, the gNB transmits the following information to the CN for the command response:

- Message type;

- AIOTF Identifier;

- A-IoT Correlation Identifier;

- RAN A-IoT Device NGAP ID;

- Command Response Transfer;

A-IoT Correlation Identifier;

RAN A-IoT Device NGAP ID

A-IoT NAS PDU;

Criticality Diagnostics (optional).

To fill the parameters of Command response message in step 4b, we discuss which information can be included in RRC Inventory Response via step 4a.

The AIOTF Identifier and A-IoT Correlation Identifier can be derived from the Service Mapping Identifier using the mapping table maintained at the gNB.

The RAN A-IoT Device NGAP ID can also be obtained from the mapping table between the AS ID and the RAN A-IoT Device NGAP ID.

The A-IoT NAS PDU represents the data PDU from the upper layer; therefore, it should be included in the RRC Command Response.

Proposal 5. The following information is proposed to be included as parameters in the RRC Command Response:

- Service mapping identifier;

- AS ID;

- A-IoT NAS PDU.

Conclusions

Based on the discussion in the contribution, we have the following proposals:

Proposal 1.  The following information is proposed to be included as parameters in the RRC Inventory request:

- Service mapping identifier;

- A-IoT Device Identification Requested;

- Resource configuration on A-IoT interface (for D2R transmission);

- Follow-on Command Indication(optional).

Proposal 2. The following information is proposed to be included as parameters in the RRC Inventory response:

- Service mapping identifier.

Proposal 3. The following information is proposed to be included as parameters in the RRC Inventory Report:

- Service mapping identifier;

- AS ID for A-IoT Device Identification;

- Inventory Complete Indication.

Proposal 4. The following information is proposed to be included as parameters in RRC Command request;

- Service mapping identifier;

- AS ID;

- Resource configuration on A-IoT interface (for D2R transmission).

Proposal 5. The following information is proposed to be included as parameters in the RRC Command Response:

- Service mapping identifier;

- AS ID;

- A-IoT NAS PDU.

References

- Chair’s Note, “3GPP TSG-RAN WG2 Meeting #131bis”, Oct. 2025

- 3GPP TR 38.769, Study on solutions for ambient IoT (Internet of Things), 2025.