3GPP TSG-RAN WG2 Meeting #132	R2-2508582

Dallas, USA, Nov 17th – 21st, 2025

Agenda item:		9.7.2 Other

Title:		Discussion on voice support over NB-IoT-NTN

Source:		ETRI

Document for:		Discussion/Decision

# 1. Introduction

During the 109th meeting of RAN, a new WI on IoT-NTN Phase 4 was approved, including the following objectives [1]:

Objectives - Down-select between CP and UP solutions for voice support over NB-IoT-NTN until RAN#110 [RAN2] - Coordination with SA is required. - The objective can be updated with further details based on the outcome of the above down-selection. - Support of semi-persistent scheduling for NB-IoT-NTN for DL and UL data transmission for voice packets [RAN2, RAN1] - Support of necessary modifications to RRC connection setup procedure for NB-IoT-NTN [RAN2] - Support of necessary modifications for emergency call for voice over NB-IoT-NTN [RAN2] - Study and if feasible, specify UE transmit power higher than PC1 (e.g. up to 37dBm) for NB-IoT-NTN [RAN4] - Note: The enhancements to support voice over NB-IoT-NTN via GSO in this work item may be also applicable to NGSO cases without additional specification enhancements - Note: Coordination with SA4 is expected

The first discussion regarding the new WI took place during the 131bisst meeting of RAN WG2 and then, the following agreements were made for the study on voice support over NB-IoT-NTN.

Agreements regarding the reply LS to SA2: 1. Regarding SA2 Q1, RAN2 is not able to make any observation on how many consecutive packets lost or erroneously decompressed will trigger the RoHC state fall back at the compressor when using RoHC, and thinks the implementation of ROHC functions (e.g., compression and decompression) falls outside of RAN2 scope 2. Regarding SA2 Q3, RAN2 responds to SA2 that scheduling methods may handle the voice packets of different sizes (e.g., via semi-persistent scheduling and dynamic scheduling, including over-provisioning of resources in UL), but RAN2 will further study and discuss the details of the scheduling mechanism to avoid the potential issues. 3. Regarding SA2 Q6, RAN2 responds to SA2 that it is technically feasible to support more than 2 DRBs for a UE accessing NB-IoT in Rel-20. 4. Regarding SA2 Q9 and Q10, RAN2 responds to SA2 that it is technically feasible to introduce SRB with RLC UM for voice packet 5. We add a sentence in the same reply LS saying that majority of companies in RAN2 prefer the UP over CP solution (because of the higher impacts on RAN2 specs of the CP solution). RAN2 also understands that the final decision will be taken at the checkpoint in December plenary. Agreements regarding the reply LS to SA4: 1. RAN2 assumes the MAC header size may be 1 to 3 bytes. However, in most common scenario, the total MAC header size is likely to be 3 bytes. 2. For transmitting voice packets via the UP solution RAN2 expects 1 byte for PDCP + 1 byte for RLC UM + the MAC header size 3. For transmitting voice packets via the CP solution RAN2 expects 2 bytes for RRC + 2 bytes for RLC AM (the only mode currently specified for CP) or 1 byte for RLC UM (if RLC UM will be introduced for SRB in case a CP based solution will be selected) + the MAC header size 4. RAN2 will provide further feedback on the expected average RoHC header size 5. We indicate to SA4 that RAN2 has not started work on SPS for voice but thinks that SPS periodicities like 120ms and 240ms periodicities that do not divide 10240 would require additional specification work in RAN2 to resolve the issues

In this paper, we would like to provide our views on CP vs UP solutions for voice support over NB-IoT-NTN and on the possible solutions to avoid/mitigate the potential issues when handling voice packets of different sizes.

# 2. Discussion

2.1	Background

RAN approved the IoT_NTN_Ph4 WID in support of IMS voice over NB-IoT-NTN via GSO (GEO) targeting Rel-20. The WID calls for a down-selection between CP and UP solutions (RAN#110) and lists specific RAN enhancements (SPS for DL/UL, RRC connection setup modifications, emergency call support, UE Tx power study) as part of the core work.

SA has, in parallel, progressed enabling studies: SA4 SID on Ultra-Low Bit-Rate Speech (PHY 1–5 kbps; codec 0.8–4.8 kbps) and SA2 SID on IMS voice over GSO via NB-IoT-NTN, whose WT#1 includes the CP vs UP comparison and aims to conclude by TSG#110 (Dec-2025). Performance requirements for satellite access captured by SA1/SA2 include IMS voice over GEO with [1–3] kbit/s experienced rate (UL/DL) and call setup time ≤ 30 s for handheld UEs. these KPIs may frame the RAN design space for NB-IoT-NTN.

During RAN2 #131bis, many companies provided contributions comparing CP and UP approaches, analyzing resource efficiency, delay, and QoS implications. While both were initially considered feasible, consensus gradually formed that the UP solution provides a more sustainable and technically consistent framework, particularly for IMS-based voice operation under GEO latency.

2.2	Design Constraints

NB-IoT-NTN voice operation over GEO satellites faces several challenges:

- High propagation delay: One-way latency exceeds 270–300 ms, resulting in round-trip time (RTT) around 600 ms.

- Narrow bandwidth: Single-PRB operation limits scheduling flexibility and TBS size.

- Variable traffic pattern: Voice frames exhibit alternating talk and silence periods, leading to irregular payload sizes.

- Limited scheduling feedback: GEO delay makes fast dynamic scheduling infeasible for real-time voice adaptation.

Under such conditions, static SPS wastes resources during silence, while purely dynamic scheduling incurs additional delay before resumption. A flexible hybrid scheduling mechanism, which preserves SPS predictability while allowing dynamic adaptation to traffic and delay variations, is thus required.

2.3	ETRI’s view on CP vs UP solution

The CP solution conveys voice frames encapsulated in NAS messages via the signaling plane, while the UP solution uses dedicated data bearers configured with PDCP, RLC, and MAC.

Table 1 summarizes the comparison in the context of GEO NB-IoT-NTN.

Table 1. Main features of CP and UP Solutions

| Aspect | CP Solution | UP Solution |
| --- | --- | --- |
| Architecture | NAS signaling plane | Dedicated DRB with PDCP/RLC/MAC |
| QoS Handling | Not supported | GBR, latency/jitter control supported |
| IMS Compatibility | Limited / indirect | Fully compatible (VoNR model) |
| Latency Sensitivity | High (signaling overhead) | Predictable, bearer-based |
| Resource Efficiency | High overhead for small packets | Optimizable via SPS |
| Extendability | Static, non-scalable | Easily extensible for voice & IoT data |

While the CP approach offers simplicity and reuse of existing NAS signaling with higher RAN2 spec. impact, it lacks the mechanisms required for precise delay control and QoS management. The UP solution, on the other hand, aligns with the existing VoNR IMS architecture and enables bearer-level control over packet prioritization, scheduling, and compression.

Observation 1. While the CP solution is simpler with higher RAN2 spec. impact, the UP solution ensures the QoS, IMS compatibility, and stability required for reliable GEO NB-IoT-NTN voice service.

During the 131bisst meeting of RAN WG2, most companies recognized that the UP approach provides a more sustainable path forward. However, RAN2 also noted that additional possible enhancement or mechanisms, particularly for SPS adaptation and variable packet handling, need to be developed to make the UP solution practically deployable over GEO channels.

ETRI would like to support this direction and proposes the following enhancement framework.

Proposal 1. RAN2 to down-select the User Plane (UP) solution for IMS voice over NB-IoT-NTN.

2.4	Possible examples of UP-based solutions

The UP solution enables voice delivery through a dedicated Data Radio Bearer (DRB) operating with PDCP, RLC, and MAC layers, similar to VoNR operation in terrestrial NR systems. This configuration allows the use of PDCP header compression, QoS handling, and DRX control, all of which are unavailable in the CP solution. Given the long GEO propagation delay and the need for precise scheduling, the UP solution offers a more deterministic and manageable architecture for maintaining voice quality.

To further improve performance, an adaptive SPS framework specifically tailored for GEO voice scenarios can be considered. The concept introduces two SPS configurations: one optimized for talk periods with shorter periodicity and higher MCS, and another for silence or comfort-noise frames with longer periodicity and smaller transport block size. The network can switch between these configurations based on voice activity detection (VAD) or codec payload size information. This dual-SPS configuration can reduce unnecessary resource occupation and provides better utilization during active speech bursts. Nevertheless, in uplink transmission, the practical gain of adaptive SPS may be limited in GEO environments, where the terminal’s transmit power constraints and the long round-trip delay hinder timely configuration updates and thereby diminish its effectiveness.

In addition, when the silence period persists for a longer duration, the eNB/gNB can temporarily suspend the SPS operation through a newly definable MAC control element, without releasing the SPS context. Once speech resumes, the network simply issues a resume indication, allowing the UE to immediately restart periodic transmissions without performing RRC reconfiguration. This SPS suspend/resume mechanism minimizes radio resource waste while preserving a fast recovery path for the next talk burst.

At the transition from silence to talk, it is considerable that the first speech packet be transmitted through dynamic scheduling before returning to the SPS pattern. This dynamic scheduling assist avoids packet accumulation delay and ensures a smooth transition for real-time conversational service even in a high-RTT environment.

Furthermore, the scheduler may dynamically adjust the number of repetitions or the modulation and coding scheme (MCS) within the same SPS periodicity according to the instantaneous payload size or link quality. This adaptive repetition/MCS control enables more efficient bandwidth utilization, particularly when the codec switches between narrowband and wideband speech modes.

Finally, GEO-specific timing adjustment is needed to ensure proper alignment between UL/DL SPS grants and feedback procedures. The proposed approach scales timer parameters (e.g., suspendTimer, resumeGuardTimer) according to the measured RTT, maintaining correct scheduling timing even under long satellite delays.

Proposal 2. RAN2 to consider the adaptive dual-SPS framework as a UP-based enhancement for voice support over NB-IoT-NTN in GEO scenarios.

# 3. Conclusions

Observation 1. While the CP solution is simpler with higher RAN2 spec. impact, the UP solution ensures the QoS, IMS compatibility, and stability required for reliable GEO NB-IoT-NTN voice service.

Proposal 1. RAN2 to down-select the User Plane (UP) solution for IMS voice over NB-IoT-NTN.

Proposal 2. RAN2 to consider the adaptive dual-SPS framework as a UP-based enhancement for voice support over NB-IoT-NTN in GEO scenarios.

# 4. References

[1]	RP-252473, “Revised WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 4”.