3GPP RAN WG2 Meeting #132                                                                                     R2-2508589

Dallas, US, Nov. 17th – 21st, 2025

Agenda Item:	10.3.3.1

Source:	HONOR

Title:	Discussion on Data transfer requirements for 6GR

Document for:	Discussion and Decision

- Introduction

In RAN#108 meeting, the industry has approved the 6G Study Item (RP-251881[1]), marking the first concrete blueprint for the next generation of wireless system. As a potential key technology of 6G, RAN2 group is responsible for discussing and promoting the study of radio interface protocol architecture and procedures for 6GR.

In RAN2#131-bis [2], RAN2 has made the below agreements regarding data collection framework and proposed to study the common requirements for various types of data:

Agreements

1.  	Study the standardized data collection framework with these requirements as a baseline at least for AI/ML

- The data collected is secured and data integrity and confidentiality for that data is ensured.

- User data privacy, anonymity and user consent is respected.

- The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE side data collection, without the need of Service Level Agreement (SLA) for this purpose

- This includes initiating, terminating, and fully managing data transfer.

- MNO has full visibility for standardized data.

- The design is future-proof and extendable.

- The UE data collection should minimize impact to the UE battery, UE processing and memory utilization.

- UE data collection should minimize impact to user traffic transmission and power saving features

These requirements can apply to both UE and NW sided data collection.  FFS if some don’t apply to both.

2.	Study further requirements of other non-AI/ML use cases/services and understand if we can have more common requirements across different use cases.

3.	Study termination points (i.e. understand who are producers, consumers of the data, data collection points.  Understand requirements of use cases sensing, aI/ml, son/mdt, QoE etc) and protocol used for data transfer (e.g. IP vs. non-IP).

4.	Study model transfer/delivery requirements/functions

In this contribution, we try to analyze the requirements of other non-AI/ML use cases/services and requirements for model/data transfer for 6GR.

- Discussion

  - AI/ML Model Transfer

2.1.1 Overview on 5G AI/ML model transfer

During the previous study of Rel-18 and Rel-19 AI/ML for air interface (AI PHY), RAN2 has extensively discussed the AI/ML model transfer/delivery process, given its fundamental role in the AI/ML LCM framework. AI/ML model transfer is originally used to deliver an AI/ML model to the Inference function (e.g., UE). However, considering the training collaboration among multiple inter-vendors, AI/ML model transfer is also an important feature for supporting two-sided models (e.g., CSI compression). Generally speaking, AI/ML model transfer may occur in the following two scenarios: model (parameter) transfer during the training phase, and model transfer from UE-side training entity to UE after models have been fully trained.

- Model transfer during training phase

  - Model parameter sharing for two-sided model training: Two alternatives (non-OTA, OTA approach) (as shown in Fig. 1 [3]) for the sharing of NW-side dataset/model parameters have been discussion in Rel-19 and initial progress has been made.

Figure 1 Non-OTA and OTA approaches for dataset/model parameter transfer

For alternative 2 (OTA approach), NW-side dataset/model parameters collection entity located at gNB/CN/OAM is responsible for transmitting dataset/model parameters towards UE-side training entity (a server inside MNO or an OTT server) via UE. From RAN2’s perspective, alternative 2 involves more standard impacts, and thus we will focus on the OTA approach for data model (parameter) transfer in the UE part of the two-sided model.

Proposal 1: RAN2 should focus on the OTA approach.

  - Online training: Rel-18 has explored the discussions on AI/ML model training mode, i.e., offline vs. online, along with their actual meaning and feasibility. According to TR 38.843 [4], online training is defined as an AI/ML training process where the model being used for inference is (typically continuously) trained in (near) real-time with the arrival of new training samples. Though online training is deprioritized in Rel-18/19 due to limitations in computing power and energy consumption of 5G devices and network nodes, which make large-scale online training challenging, it is anticipated that 6G networks will possess significantly enhanced computing capabilities, lower latency, and higher reliability. These improvements will better support the resources and real-time interactions required for online training. Therefore, it is supported to consider online training in 6G, which will involve complex model transfer processes, e.g., the parameter exchange between multiple training entities.

Proposal 2: Online training could be re-considered in 6G, which will involve complex model transfer processes.

- Model transfer after models have been trained by training entities

During Rel-18 SI, several solutions for transferring/delivering AI/ML model(s) have been captured in TR 38.843. Based on the location of model storage (training entities), these solutions can be categorized into four types: (1) control plane (CP) and user plane (UP)-based RAN approaches, (2) CP/UP/LPP-based core network (CN) / location management function (LMF) approaches, (3) CP/UP-based OAM approaches, and (4) OTT server approach.

Whether there is a need to consider standardised solutions for transferring/delivering AI/ML model(s) is unclear from the outcome of the present study. Nonetheless, to support AI/ML model transfer/delivery, the following solutions are considered: • Solution 1a: gNB can transfer/deliver AI/ML model(s) to UE via RRC signalling. • Solution 2a: Core Network (except LMF) can transfer/deliver AI/ML model(s) to UE via NAS signalling. • Solution 3a: LMF can transfer/deliver AI/ML model(s) to UE via LPP signalling. • Solution 1b: gNB can transfer/deliver AI/ML model(s) to UE via UP data. • Solution 2b: Core Network (except LMF) can transfer/deliver AI/ML model(s) to UE via User Plane (UP) data. • Solution 3b: LMF can transfer/deliver AI/ML model(s) to UE via UP data. • Solution 4a: OTT server can transfer/deliver AI/ML model(s) to UE (e.g., transparent to 3GPP). • Solution 4b: OAM can transfer/deliver AI/ML model(s) to UE. Note: The relationships between model transfer/delivery solutions and use cases can be derived from what is captured in clauses 7.3.2, 7.3.3, and 7.3.4.

In our opinion, there is no need to further discuss the OTT solution (i.e., solution 4a) since this solution has no impact on 3GPP specifications.

Observation 1: OTT solution (i.e., solution 4a) has no 3GPP impact.

2.1.2 Baseline principles for AI/ML model transfer

During RAN2#129-bis, the following agreements were made for two-sided model:

Dataset/model parameter transfer solution from NW to UE training entity shall follow below principles: - A1 - Size: From RAN2 point of view, aim to define a unified solution (e.g. OTA or non-OTA) to support various sizes of dataset/model parameter transfer (dataset and/or parameter sharing size could range from tens of KBs to hundreds of MBs, but in average around hundreds of MBs); - A2 - Continuity: Service continuity of dataset and/or parameter transfer/delivery during UE mobility needs to be supported; - A3 - Controllability: NW decides on if and when to transfer/delivery the dataset and/or model parameter from NW to UE or UE training entity (a server inside MNO or an OTT server); - A4 - Latency: Relaxed latency requirement and infrequent update; - A5 - Visibility: dataset and model parameter to be understandable by UE/UE-side training entity (a server inside MNO or an OTT server). - A6: Proprietary information of the network and UE should be respected and not disclosed.

Due to time constraints over the AI/ML for the air interface WID, RAN2 has had no time to properly study the model transfer for other use case (e.g., one-sided model). For 6GR, RAN2 should determine the baseline principles for general AI/ML model transfer scenarios based on prior work, which may include:

- Controllability: The MNO has fully control of the model transfer process. The AI/ML model transfer process should consider the initiating (e.g., model/parameter transfer triggered by a model update procedure for model fine-tuning) and terminating (e.g., terminated by a sudden IMS voice) procedure.

- Visibility: The MNO has full visibility for standardized AI/ML model (e.g., model structure and model parameter) during model transfer. In Rel-20, RAN1 is studying the two-sided model for CSI compression, and focuses on case z4 after discussion of several meetings, i.e., model training at NW side and model transfer in open format of a known model structure at UE.

- Continuity: Model transfer continuity during UE mobility (i.e., resume transmission of model (segments) across gNBs) should be supported, even model/parameter sharing may be expected to transfer in days/weeks.

- Considerations on impacts of UE: The AI/ML model transfer should minimize impact to the UE battery, memory utilization, and user traffic transmission.

  - UE battery: AI/ML model transfer should minimize impact to power saving features, e.g., consider the UE’s DRX configuration when gNB delivers the model to UE.

  - UE memory: AI/ML model training should consider the limitation of UE’s memory. Considering multiple AI/ML use cases, there could be several sets of model parameters transferred to UE and stored in the limited memory, which will significantly increase the cost at UE device when supporting the larger memory size as an optional feature. For example, one set of dataset and/or parameter sharing size can be varied from 36KB to 225MB+11.6MB [5].

  - Impact of user traffic: Similar to the data collection procedure, model transfer should minimize impact to user traffic transmission. For offline training, the priority of model transfer task is typically lower than legacy user traffic, with relaxed latency requirement and infrequent update. The network should consider how to ensure the transmission requirement of user traffic when online model transfer is ongoing.

- Security: The procedure of AI/ML model transfer is secured, and the integrity and confidentiality of model should be ensured.

- Extendibility: The AI/ML model transfer framework is future-proof and extendable for future use cases.

Proposal 3: RAN2 should study the standardized AI/ML model transfer framework with these principles as a baseline:

- Controllability: The MNO has full control of the AI/ML model transfer process, including initiating and terminating the model transfer.

- Visibility: The MNO has full visibility for standardized AI/ML model (including model structure and model parameter) during model transfer.

- Continuity: Model transfer continuity during UE mobility (i.e., resume transmission of model (segments) across gNBs) should be supported.

- Considerations on impacts of UE: The AI/ML model transfer should minimize impact to the UE battery, memory utilization, and user traffic transmission.

- Security: The procedure of AI/ML model transfer is secured, and the integrity and confidentiality of model should be ensured.

- Extendibility: The AI/ML model transfer framework is future-proof and extendable for future use cases.

  - Transfer requirements of 6G system data

In the TR 22.870 [6], SA WG1 has defined the 6G system data which is generated and controlled by the 6G system. 6G system data, e.g., AI/ML data, sensing data, energy efficiency information and NF context, is different from user traffic data which is application-level data transmitted transparently between the UE and DN, thus presenting new requirements for 6G data collection framework. For simplicity, all types of data are collectively referred to as 6G system data which includes inference data, monitoring data, training data and model data for AI/ML.

In this section, we first explore the transfer requirements for AI/ML data; then we analyze whether these considerations could be applied in other use case (e.g., sensing) to prompt common transfer requirement of 6G system data.

2.2.1 Transfer requirements for AI/ML data

Transfer requirements for AI/ML model

As outline in section 2.1.1, for OTA approach (i.e., gNB -> NW dataset/model parameters collection entity (if needed) -> gNB -> UE -> UE training entity), RAN2 has identified several candidate solutions, including transferring models from gNB/CN/OAM to UE via CP, and transferring models from CN/OAM to UE via gNB. However, the latter is not recommended since it has no benefit over non-OTA solution and may bring a risk of proprietary information exposure if gNB and CN are not from the same NW vendor. Moreover, the feasibility of CP-based solutions may be compromised by the limited UE RRC buffer size (i.e., 45kB according to TS 38.306), especially when considering the varying model/parameter size.

Observation 2: Factors like model size, security & privacy have been considered to evaluate the different model transfer/delivery solutions in 5G NR.

For CSI compression using two-sided model use case, three types of AI/ML model training collaborations are captured as follows:

- Type 1: Joint training of the two-sided model at a single side/entity, e.g., UE-sided or Network-sided. - Type 2: Joint training of the two-sided model at network side and UE side, respectively. - Type 3: Separate training at network side and UE side, where the UE-side CSI generation part and the NW-side CSI reconstruction part are trained by UE side and network side, respectively.

For Type 2, if joint training involves multiple nodes (e.g., NW-sided and UE sided), they may frequently exchange model parameters (e.g., gradient) since the generation model and reconstruction model should be trained in the same loop for forward/backward propagation. Thus, there exists a strict latency restriction for the model parameter exchange between different training entities.

Observation 3: Latency requirements should be prioritized in some AI/ML model transfer scenarios (e.g., parameter exchange between multiple training entities for online training).

From the above, 5GNR study has considered the model (parameter) size, model transfer Qos (including latency), and security & privacy protection when measuring different model transfer/delivery solutions, and these aspects can be considered as baseline requirements for the 6GR AI/ML model transfer.

- Model size: Model size may vary depending on use cases. It is important to define a unified solution (e.g., OTA) to support various sizes of model (parameter) transfer.

- Transmission latency: The latency requirements may differ significantly across model transfer scenarios, e.g., model parameter sharing between multiple training entities for online training may be time-critical, while transferring a trained model to UE could be done in days or weeks.

- Reliability: Model transfer requires higher accuracy and integrity because models contain complex structures and precise parameters that are critical to their performance and accuracy. In contrast, transmission errors in data transfer have less impact, especially with large datasets, where a few errors may not significantly affect the final analysis.

- Privacy & security protection: It is very likely that infrastructure vendors, device vendors, and chipset vendors will create their own AI/ML models, while model parameters may contain sensitive information, such as the characteristics of user data or the internal state of the model. Thus, privacy protection during model transfer needs to be considered.

Proposal 4: RAN2 should consider transfer requirements of AI/ML model from the following aspects: model size, transmission latency, reliability and privacy & security protection.

Transfer requirements for all types of AI/ML data

In 5G NR, several use cases have been studied on leveraging AI/ML to improve network performance, such as CSI feedback enhancement, beam management (BM) and positioning enhancement. In each scenario, data plays a crucial role in supporting AI/ML model training and model monitoring/management. The data could be AI/ML model for inference or datasets for model training, and it could be delivered from one entity to another entity in any manner.

Typically, AI/ML data could be divided into several types from the following dimensions:

- Based on data utilization: Three types of AI/ML data are mainly considered in 5G NR study on AI/ML air interface, including AI/ML training data, AI/ML inference data and AI/ML monitoring data. AI/ML training data may include the raw data or processed datasets for training/testing/validating AI models, assistance information (e.g., for categorizing the data like timestamps) and exchanged AI model parameters during model training. AI inference data may indicate the input data and output data (e.g., AI analytic results). AI monitoring data may include the calculated performance metrics or data required for performance metric calculation. What’s more, AI/ML data also include the trained AI/ML model transferred from model provider to model consumer. It can be found that there is a large amount of data types defined in 3GPP. In the future, some data types that may be totally new in 6G (e.g., digital twin) may emerge due to the explosive growth of AI applications.

Observation 4: In view of the emerging 6G applications, a wide variety of AI/ML data types will arise.

- Based on transfer requirements: Similar to the AI/ML model, the transfer requirements for AI/ML data could be summarized in terms of latency, reliability, data size and transfer frequency:

  - Time-sensitive & Low reliability & Small data size & High transfer frequency: e.g., monitoring data for UE-sided BM, CSI prediction use cases. UE needs to frequently report L1 measurement results to gNB for performance monitoring.

  - Relaxed latency requirement & High reliability & Large data size & Low transfer frequency: e.g., model transfer after models have been fully trained (i.e., without online/offline fine-tuning).

  - Relaxed latency requirement & Low reliability & Large data size & Low transfer frequency: e.g., training data transfer from UE to UE/NW-sided training entity for UE/NW-sided model training.

  - Time-sensitive & High reliability & Large data size & High transfer frequency: e.g., parameter exchange for online training, and model parameter sharing from NW-sided to UE-sided training entity.

Observation 5: AI/ML data transfer in 6G varies widely in latency, reliability, data size, and transfer frequency to meet different application needs.

- Based on end point pairs (i.e., producer and consumer)

  - UE and CN: e.g., training data transfer for UE/NW-sided model (e.g., BM/CSI prediction) [Producer: UE & Consumer: UE/NW-sided training entity].

  - UE and RAN: e.g., monitoring data transfer for UE/NW-sided (e.g., BM/CSI prediction) or two-sided model (e.g., CSI compression); inference data transfer for NW-sided (e.g., BM) or two-sided model (e.g., CSI compression) [Producer: UE & Consumer: gNB].

  - RAN and CN: e.g., training data transfer for two-sided model (e.g., CSI compression) [Producer: gNB & Consumer: UE-sided training entity].

  - RAN and OAM: e.g., training data transfer for NW-sided model (e.g., BM) [Producer: gNB & Consumer: OAM (NW-sided training entity)].

Observation 6: Compared with 5G user traffic, AI/ML data transfer in 6G involves diverse endpoint pairs, including UE-CN, UE-RAN, RAN-CN, and RAN-OAM.

In 5G, AI/ML LCM process is discussed case by case and data could be transferred via multiple paths, e.g., UCI/DCI/RRC to meet the requirements of different cases. Therefore, there may be repetitive work for new use cases. To address this issue, RAN2 should study how to unify the requirements across various scenarios and then design a common data framework for 6G system data. Based on the discussion in section 2.1 and conclusion in 5G, AI/ML data could be modeled in the following aspects:

- Data size: AI/ML data size may vary depending on the data types and impacts the data transfer process. For example, AI/ML model transfer may demand higher bandwidth and storage capacity due to the large size of model files. Conversely, AI/ML data transfer can be more efficiently managed through techniques such as data cleaning to avoid redundant data transmission.

- Transmission latency: The latency requirements may differ significantly across data transfer scenarios, e.g., model parameter sharing between multiple training entities for online training may be time-critical, while transferring the training data to training entity could be done in days or weeks.

- Reliability: The reliability requirements may differ based on the scenarios, e.g., model transfer requires robust encryption and integrity verification to protect the model’s structure and parameters. Data transfer also needs encryption and privacy protection, but minor transmission errors have less impact, especially with large datasets.

- Transfer frequency: Transfer frequency should be viewed as one of the key data characteristics. For example, model transfer typically occurs less frequently, while frequent transmission may happen for online training, e.g., FL training. Inference/monitoring data is often transferred frequently, particularly in real-time (for monitoring) or near-real-time (for inference) applications.

- Multiple end point pairs: AI/ML data transfer in 6G involves diverse endpoint pairs, including UE-CN, UE-RAN, RAN-CN, and RAN-OAM.

- Privacy & Security protection: The integrity and authenticity of data have a direct impact on the training quality and inference accuracy of AI models. Securing data transmission and preventing any tampering or forgery are essential to improving the performance and reliability of AI systems.

In conclusion, the transfer requirements of AI/ML data can be proposed as follows:

Proposal 5: 6GR should consider the following AI/ML data characteristics when designing 6G system data transfer framework: Data size, transmission latency, reliability, transfer frequency, multiple end point pairs and privacy & security protection.

2.2.2 Transfer requirements for Sensing

For sensing in NR, sensing entity performs the sensing service requested by sensing function (SnF). In general, the SnF will generate the sensing results and send them to the consumer. In certain cases, the SnF may request the sensing entity (e.g., RAN node) to process the sensing data and generate sensing result or partial result in the report. Therefore, the sensing entity collects the sensing data and delivers the sensing data or results to the SnF.

In general, the sensing data transmission has the following characteristics:

- Max sensing service latency: it is the time elapsed between the event triggering the determination of the sensing result and the availability of the sensing result at the sensing system interface.

- Sensing report frequency: sensing reports support the method of one-time, periodic or triggered events. And the periodicity range is from 50ms (tracking) to 600s (environment).

- Security and privacy: the transfer of sensing data needs to support encryption, integrity protection, privacy for preventing other illegal entity to acquire the sensing results.

- Data size: the sensing data includes the channel state information, e.g. distance, azimuth and radial velocity. And the sensing data may be reported per sub-carrier, resulting the large data volume.

In conclusion, the transfer requirements of sensing data can be summarized as follows:

Proposal 6: 6GR should consider the following sensing data characteristics: Data size, latency, report frequency, and privacy & security protection.

Overall, by combining the transfer requirements for AI/ML data and sensing data, the common transfer requirements of 6G system data can be summarized as:

Proposal 7: 6GR should consider the following 6G system data characteristics when designing 6G system data transfer framework: Data size, transmission latency, reliability, transfer frequency, multiple end point pairs and privacy & security protection.

- Conclusions

In this contribution, we discussed the possible requirements of 6G system data transfer, and made the following proposals:

Proposal 1: RAN2 should focus on the OTA approach.

Proposal 2: Online training could be re-considered in 6G, which will involve complex model transfer processes.

Observation 1: OTT solution (i.e., solution 4a) has no 3GPP impact.

Proposal 3: RAN2 should study the standardized AI/ML model transfer framework with these principles as a baseline:

- Controllability: The MNO has full control of the AI/ML model transfer process, including initiating and terminating the model transfer.

- Visibility: The MNO has full visibility for standardized AI/ML model (including model structure and model parameter) during model transfer.

- Continuity: Model transfer continuity during UE mobility (i.e., resume transmission of model (segments) across gNBs) should be supported.

- Considerations on impacts of UE: The AI/ML model transfer should minimize impact to the UE battery, memory utilization, and user traffic transmission.

- Security: The procedure of AI/ML model transfer is secured, and the integrity and confidentiality of model should be ensured.

- Extendibility: The AI/ML model transfer framework is future-proof and extendable for future use cases.

Observation 2: Factors like model size, security & privacy have been considered to evaluate the different model transfer/delivery solutions in 5G NR.

Observation 3: Latency requirements should be prioritized in some AI/ML model transfer scenarios (e.g., parameter exchange between multiple training entities for online training).

Proposal 4: RAN2 should consider transfer requirements of AI/ML model from the following aspects: model size, transmission latency, reliability and privacy & security protection.

Observation 4: In view of the emerging 6G applications, a wide variety of AI/ML data types will arise.

Observation 5: AI/ML data transfer in 6G varies widely in latency, reliability, data size, and transfer frequency to meet different application needs.

Observation 6: Compared with 5G user traffic, AI/ML data transfer in 6G involves diverse endpoint pairs, including UE-CN, UE-RAN, RAN-CN, and RAN-OAM.

Proposal 5: 6GR should consider the following AI/ML data characteristics when designing 6G system data transfer framework: Data size, transmission latency, reliability, transfer frequency, multiple end point pairs and privacy & security protection.

Proposal 6: 6GR should consider the following sensing data characteristics: Data size, latency, report frequency, and privacy & security protection.

Proposal 7: 6GR should consider the following 6G system data characteristics when designing 6G system data transfer framework: Data size, transmission latency, reliability, transfer frequency, multiple end point pairs and privacy & security protection.

- References

- RP-251881

- RAN2#131-bis Chairman notes

- RAN2#129-bis Chairman notes

- TR 38.843, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”, 3GPP

- RAN1 LS R2-2500015

- TR 22.870, “Study on 6G Use Cases and Service Requirements”, 3GPP