3GPP RAN WG2 Meeting #132                                                                                     R2-2508590

Dallas, US, Nov. 17th – 21st, 2025

Agenda Item:	10.3.2.3

Source:	HONOR

Title:	Discussion on Initial and System Access for 6GR

Document for:	Discussion and Decision

- Introduction

RAN#108 has approved the first version of 6G SID and RAN#109 revised the SID with some modification [1]. An initial aspect of this SID is for RAN2 to study the Control Plane architecture and protocol design. Initial Access is an important aspect for Control Plane and RAN2 has reached an agreement on the basic goals and requirements about system information and paging in RAN2#131-bis [2]:

Agreements on System Information - System information design should consider energy efficiency and low complexity for both network and UE. - System information design should ensure that the public warning requirements are met. - Study RAN2 aspects of system information designs (e.g. on-demand SIB and SSB, periodicity of SSBs). - Study how to improve flexibility and extensibility of scheduling SI design. - Study mechanisms to improve SI update mechanism. - Study aspects related to SIB1 size. Understand the issues and desired content/size from RAN2 point of view. - Study areas specific SIB design. Understand 5G pain points and what improvements can be considered. NOTE: this is a starting list

Agreements Paging design should consider energy efficiency and simplity for both UE and Network.

In this contribution, we discuss some issues related to the initial and system access for 6GR, including paging and connection management.

- Discussion

  - Paging

In LTE and 5G NR systems, paging is a critical functionality for the network to notify a UE in the RRC IDLE or RRC INACTIVE state of incoming services or messages (e.g., calls, data, system Information modification, and ETWS/CMAS notification). In LTE, paging was already optimized with Discontinuous Reception (DRX) cycles by not requiring continuous monitoring of the network for UE’s power saving. Specifically, the network determines the Paging Frame (PF) and the specific Paging Occasions (POs) within that frame for a given UE in IDLE/INACTIVE mode, using parameters like the UE’s identity and the configured DRX cycle. In 5G era, new requirements such as supporting massive IoT devices, enabling URLLC, and improving security have emerged, leading to the following key enhancements in 5G paging:

- RRC_INACTIVE and RNA Paging: Unlike LTE only using tracking areas (TA), 5G introduces RNA for UEs in inactive state to enable localized paging without involving the entire TA, reducing signaling overhead and improving efficiency.

- Improved Privacy: LTE paging could reveal IMSI or static TMSI values, making users vulnerable to tracking. 5G addresses this risk by using temporary identifiers (e.g., 5G-S-TMSI / full-RNTI) that are frequently updated.

- Paging early indication (PEI): Introduced in Rel-17, DCI-based PEI informs the UE whether to monitor an upcoming PO. This prevents unnecessary wakeups and achieves distinct energy savings, particularly for IoT UEs.

As agreed in RAN2#131-bis, Paging design for 6GR should consider energy efficiency and simplicity for both UE and Network, which may inherit existing considerations and techniques from 5G NR.

- Paging for UE Power saving on 6GR

To reduce UE power consumption caused by false paging alarms, UEs monitoring the same PO are divided into subgroups based on information like UE’s location, UE’s paging probability, capacity, and network load, etc. A UE shall monitor PDCCH in its PO only if its subgroup is paged, indicated by the associated wake-up signal (WUS).

NG has introduced two WUS-based paging features. In Rel-17, PEI was introduced as the WUS signal to help avoid unnecessary PO reception by indicating whether the UE should decode paging (PDCCH/PDSCH) in its PO. In Rel-19, Low-Power Wake-Up Signal (LP-WUS) further reduces the UE power consumption by allowing UE to remain asleep (i.e, No Wake up) even for DRX OnDuration period when there is no data for the UE and gNB notifies the UE of “No Wake up”.

Mainly, both two mechanisms are similar but differ in implementation: LP-WUS uses a separate low-power wake-up receiver (WUR) to keep the main radio (MR) off, while PEI relies on the MR to receive signals via the PDCCH. It can be seen, PEI and LP-WUS offer overlapping functionalities, and supporting both simultaneously would increase the complexity and power consumption of the UE. Therefore, 6G Day1 should consider the unified Early Indication design before paging message for UE power saving.

Observation 1: PEI and LP-WUS are similar features, and supporting both simultaneously would increase the complexity and power consumption of the UE.

Proposal 1: 6G Day1 should study the unified Early Indication design before paging message for UE power saving.

- Paging for Network Power saving on 6GR

In addition to SSB signals, paging signal acts like an “always-on” signal, and sparse paging occasions may increase BS’s energy consumption and reduce its sleep time, even when no data transmission occurs during the paging occasions. In Rel-19, NES (network energy saving) WI [3] considers the paging adaptation in time domain, which supports non-uniform PO distribution unlike 5G NR. This makes the paging occasions relatively concentrated, enabling longer cell sleeping periods for network and facilitating more clustered RACH responses to paging.

Proposal 2: In 6G, RAN2 can further consider the non-uniform provisioning (in time) of paging occasions to enhance energy saving.

For adaptation of paging occasions, two directions have been discussed in RAN2#125-bis: a) bundle paging frames and b) extend the values of N to have increased interval between PFs (e.g., T/64, T/128 ...) and compensating decrease in number of PFs by increasing POs per PF.

In details, Option A aggregates the PFs that were originally evenly distributed within a paging cycle, enabling the network to wake up once and page more UEs, thereby extending the network’s sleep time. Option B extends the paging frame interval so that the paging frames are more interspaced compared to the legacy interval, while potentially increasing the number of paging occasions within each frame to maintain the same paging capacity. Compared to Option B, Option A requires significant changes to the legacy paging architecture, and thus Option B was chosen as the Rel-19 study baseline.

Considering that 6G Day1 paging design is expected to commence from a new paradigm, the above two methods (bundled PFs, and PFs with extended interval) can be reconsidered when designing the non-uniform PO distribution.

Proposal 3: In 6G, reconsider the paging adaptation techniques (e.g., bundled PFs, and PFs with extended interval) when designing the non-uniform PO distribution.

  - Connection management

In 5G, cell reservation and access restrictions enable the NW to control the initial and system access. Reservation mechanism means the NW could broadcast the cell status and special reservations. By this way, the UE observes the cell reservation status when performing cell selection and reselection. 5G access restriction (i.e., Unified Access Control) considered different access categories or access identities and only allowed the selected ones to send initial access messages for load control reasons.

On one hand, 3GPP RAN usually controls RRC connection requests to protect RAN/CN nodes. For example, 5G introduced a unified access control mechanism to limit the access attempt based on the requirements from both RAN and CN. The UE would obtain the parameters provided by the camped cell before the access. Firstly, the NAS needs to map the service request to one or more access identities and access category. Then, the RRC would perform the access barring check based on the determined access identities and access category. The UAC aims to guarantee the service quality for the connected UEs and for the UEs with higher priority or authority while the overload exceeds the capacity of the system. For 6G, access control mechanism is still needed considering different service type (e.g., emergency call, MT, MO, or other 6G new services), and device type. On the other hand, barring information is carried in MIB (i.e., cellbarred) and SIB1 (barring for different feature or device type) to indicate the cell status and cell reservation. For example, the cell status as "not barred" indicates the UE could treat the cell as a candidate during the cell selection and cell reselection procedures. For 6G, similar mechanism is needed to inform the cell status in advance to prevent the UE from camping on a cell which could not provide the required service.

Proposal 4: For 6G, RAN2 should aim to design a unified access control mechanism considering different service type (e.g., emergency call, MT, MO, or other 6G new services), and device type and NW/UE capability. Barring information could be carried in system information.

- Conclusions

In conclusion, we propose the followings for CP aspects of 6GR.

Observation 1: PEI and LP-WUS are similar features, and supporting both simultaneously would increase the complexity and power consumption of the UE.

Proposal 1: 6G Day1 should study the unified Early Indication design before paging message for UE power saving.

Proposal 2: In 6G, RAN2 can further consider the non-uniform provisioning (in time) of paging occasions to enhance energy saving.

Proposal 3: In 6G, reconsider the paging adaptation techniques (e.g., bundled PFs, and PFs with extended interval) when designing the non-uniform PO distribution.

Proposal 4: For 6G, RAN2 should aim to design a unified access control mechanism considering different service type (e.g., emergency call, MT, MO, or other 6G new services), and device type and NW/UE capability. Barring information could be carried in system information.

- References

- RP-252912 Revised SID: Study on 6G Radio

- RAN2#131-bis Chairman notes

- RP-250343, “Revised WID on enhancements of network energy savings for NR”, RAN Meeting #107, Incheon, Korea, March 12-14, 2025