3GPP TSG-RAN WG2 Meeting #132	R2-2508619

Dallas, USA, 17th –21st November, 2025

Agenda Item:	10.3.2.3

Source:	Huawei, HiSilicon

Title:	Discussion on cell management and system access

Document for: Discussion and Decision

- Introduction

In the last RAN2 meeting the following progress has been made regarding SI, Paging and carrier aggregation:

Agreements on System Information

-	System information design should consider energy efficiency and low complexity for both network and UE.

-	System information design should ensure that the public warning requirements are met.

-	Study RAN2 aspects of system information designs (e.g. on-demand SIB and SSB, periodicity of SSBs)

-	Study how to improve flexibility and extensibility of scheduling SI design

-	Study mechanisms to improve SI update mechanism

-	Study aspects related to SIB1 size.  Understand the issues and desired content/size from RAN2 point of view.

-	Study areas specific SIB design.  Understand 5G pain points and what improvements can be considered.

NOTE: this is a starting list

Agreements

Paging design should consider energy efficiency and simplicity for both UE and Network

Agreements on Spectrum aggregation

1	Study spectrum aggregation of multiple pieces of spectrum and understand the issues that need to be addressed.

2	Study how to use UL and DL spectrum more efficiently (e.g. decoupling of UL and DL, etc).  Understand the deployment scenarios and problems to address

In this contribution, we will further discuss SI, Paging and carrier aggregation design based on the progress.

2	Discussion

2.1	Cell management

In last meeting, there are following agreements on spectrum aggregation.

Agreements on Spectrum aggregation 1 Study spectrum aggregation of multiple pieces of spectrum and understand the issues that need to be addressed. 2 Study how to use UL and DL spectrum more efficiently (e.g. decoupling of UL and DL, etc). Understand the deployment scenarios and problems to address

Firstly, to analyze the issues for spectrum aggregation, we think the discussion should be separated for RRC connected mode and RRC idle mode considering that the requirements and functions are quite different for these two states (RRC IDLE in 5G is less relevant to spectrum aggregation except for SUL). Before discussing the cell model, it is more important to study how to utilize a set of DL/UL carriers efficiently for spectrum aggregation, and to identify what basic functionalities need to be supported based on the set of carriers.

For RRC connected mode, one issue observed from 5G is, the fixed pairing between DL and UL restricts the flexibility to utilize spectrum pieces. In NR, each cell has to have one FDD band or one TDD band, whose DL and UL are associated in a fixed manner. SUL can be configured as an additional UL for a cell. This brings limitation that the UL configuration has to be tightly coupled with DL configuration, which may not be flexible enough for DL/UL spectrum utilization. For example, based on CA features, configuring UE with Rel-18 UL Tx switching across 3 or 4 UL bands requires UE to support 3 or 4 DL carriers (while requiring UE to support 2 DL carriers if SUL is configured). However, in practical networks, large amount of NR UEs only support up to 2 DL carriers. Furthermore, UL Tx switching is performed on the UL carriers which are associated with the activated DL CCs, leading to unnecessary DL power consumption if DL traffic is low.

Another issue for connected mode is the inflexible spectrum usage due to the fact that the PCell cannot be activated/deactivated dynamically. This is because some cell level operations (such as RLF monitoring, SIBs reception, RRM measurement for mobility etc.) could only rely on the PCell. This has negatively affected the UE's power consumption (when the UE has no chance to be scheduled due to load in PCell) and prevented the effective utilization of other frequencies by UE retuning the PCell receiver to receive in other spectrums.

Observation 1: The issues for spectrum aggregation in RRC connected mode include the low flexibility on DL/UL pairing and inflexible spectrum usage due to PCell-defined cell level operation.

Based on the issues above, we understand at least configuration of separate lists for DL carrier(s) and UL carrier(s) is necessary. Starting from separate lists of DL and UL carriers, RAN2 can further study how to support the basic functionalities in connected mode, such as the UL/DL timing, pathloss reference, RLM monitoring, handover and carrier management (e.g. carrier addition/removal) etc. to address the issues identified above. Based on a clear picture on the carrier function definition, the cell modeling can be further discussed on top of that.

In 5G, the concept of spectrum aggregation is not introduced for RRC idle mode. In downlink only PCell is visible to the UE, and in uplink the SUL carrier can be configured additionally to improve coverage performance by selecting between SUL and NUL for the capable UEs. Considering that the basic functions for IDLE shall be agnostic to UE capability, spectrum aggregation at least shall not be mandatory for UE to access the cell. This means all the basic functions such as SSB/SIBs/Paging reception and RACH operation shall be possible from UE side with on one DL/UL carrier. Considering that the 6G targets spectrum higher than 5G, the RACH on other assisted low band carriers for coverage enhancement shall also be supported for capable UEs. Whether there is need to offload paging in downlink can be further evaluated.

Proposal 1:	Separate the discussions on cell definition and spectrum aggregation for IDLE and Connected states:

For UEs in CONNECTED state, start from the configuration of separate DL and UL carrier lists, identify any requirements for DL/UL coupling and aggregation-related issues. Cell modelling and definition can be discussed after the above discussion;

  - Those issues can be related to RLM monitoring reference, handover, carrier management (e.g., carrier addition/removal), SIB reception, and random access.

For UEs in IDLE state, study the need to configure multiple DL/ UL carriers via SIB — for example, supporting paging over multiple DL carriers and random access on multiple UL carriers.

2.2	System information design

In 5G, SI scheduling adopts a mechanism based on the SI messages and SI windows, together with some SI scheduling principles. The details are as follows:

  - Principle 1: Only SIBs having the same periodicity can be mapped to the same SI message.

  - Principle 2: The SI-windows of different SI messages do not overlap.

Due to the constraints of the above principles, SIBs with different periodicities can only be mapped to different SI windows, and these SI windows are sequentially arranged in the time domain, as shown in Figure 1. With this mechanism, the network needs to continue sending SI messages over a relatively long period if the number of SIs is large or the SI window is long, which prevents the network from entering a sleep state even in low load scenarios.

Figure 1. SI windows of different SIBs with different periodicities in 5G

Observation 2-1:	The current SI scheduling mechanism restricts the network's ability to enter sleep mode for energy saving.

In the last RAN2 meeting, it has been agreed that system information design should consider energy efficiency and low complexity for both network and UE. If SIBs with different periodicities can be mapped to the same SI window, the time for network to transmit system information will be reduced, which is beneficial for UE and network energy saving.

As shown in Figure 2 below, SIB2 and SIB4 with a periodicity of 320ms and SIB5 with a periodicity of 640ms are mapped to a same SI window. In some periods, the network sends SIB2, SIB4 and SIB5 within the SI window, while in other periods, it only sends SIB2 and SIB4 within the SI window.

Figure 2. SI windows of different SIBs with different periodicities in 6G

Therefore, to reduce network energy consumption, more concentrated SIB transmission can be achieved by allowing SIBs with different periodicities to be mapped to the same SI window. RAN2 should study the mechanisms on how to map SIBs with different periodicities to the same SI window.

Proposal 2:	In order to reduce the number of SI windows and thereby decrease the duration of SI transmission, RAN2 should study the mechanisms of mapping SIBs with different periodicities into one SI window.

In 5G RRC specification, there are three SI scheduling lists and up to four SI window determination mechanisms for different SIBs or posSIBs summarized as below:

Figure 3. SI window determination mechanisms in 5G

The complexity of the overall SI window scheduling mechanism stems from its initial design, in which simplicity is overly pursued yet results in insufficient utilization of time-domain resources. As the number of SIs increases, the difficulty of window arrangement by implicit SI window arrangement (based on the order in the scheduling list) also increases. The SI window scheduling mechanism has been indeed progressively improved to more efficiently utilize unused time-domain opportunities by introducing window offset and explicit window position, however, this has contributed to the current complexity of the overall SI window mechanism. It would be better to re-design a new SI scheduling mechanism to allow flexible and unified SI scheduling, rather than to further enhance the current SI scheduling mechanism.

Observation 3-1:	The current SI scheduling mechanism has become overly complex due to multiple enhancements aiming at better utilizing time-domain opportunities yet at late 5G stage.

In 5G, SIBs and posSIBs are scheduled by different SI scheduling lists, and SI message of the posSIBs scheduled in posSchedulingInfoList is sequentially placed after SI message of the SIBs scheduled in schedulingInfoList. Thus when PWS SIB transmission is started or stopped, the SI window of SI messages carrying the posSIBs scheduled in posSchedulingInfoList will change, as shown in Figure 4. Specifically, when the network needs to transmit PWS SIB, it immediately modifies SIB1 to include the scheduling information for the PWS SIB, and starts sending the updated SIB1 and PWS SIB. The network will notify UE of SI change and PWS notification in current modification period. The UEs without PWS capability will obtain the updated SIB1 in next modification period. Therefore, in 5G, the UEs without PWS capability do not know the correct SI window location corresponding to the posSIBs scheduled in posSchedulingInfoList and cannot correctly receive the posSIBs scheduled in posSchedulingInfoList during the modification periods where PWS SIB transmission is started or stopped.

Figure 4. Change of SI window corresponding to posSIBs scheduled in posSchedulingInfoList when PWS SIB transmission is started or stopped in 5G

Observation 3-2:	The current SI scheduling mechanism leads to that UEs without PWS capability cannot correctly receive the posSIBs scheduled in posSchedulingInfoList during the modification periods with PWS SIB starting and stopping.

To avoid repeating the above-mentioned issues from 5G, we should design a unified SI scheduling list and a corresponding flexible SI window determination mechanism (for example with flexible window arrangement by mixing implicit and explicit window determination) from 6G Day 1. This solution is then able to efficiently utilize time-domain opportunities while to also ensure its applicability to all non-positioning SIBs and positioning SIBs that may be introduced in the future.

Proposal 3:	RAN2 aims to develop a unified scheduling list incorporating both explicit and implicit window positions for all SIs, including positioning SI (even if not supported in 6G day 1) and non-positioning SI.

In the last RAN2 meeting, it has been agreed to study aspects related to SIB1 size (i.e., 2976 bits physical layer SIB1 size limitation). Before discussing how to resolve the issue, we can firstly analyse the required SIB1 size in different deployment scenarios in 5G. As shown in Table 1, the required SIB1 size varies with the deployment scenario:

Table 1. required SIB1 size for different deployment scenarios in NR

| deployment scenarios | description | size |
| --- | --- | --- |
| basic deployment scenario | basic DL/UL configuration (2PLMNs,2SI, 3 searchspaces, basic RACH etc.) | 800-1200bits |
| typical deployment scenario | one or two big features configured: e.g. basic configuration +SUL and REDCAP | 1200-2200bits |
| complex deployment scenario | more features and more PLMN/SIs configured: e.g. basic configuration additional 2 PLMNs and 2SIs + SUL/ REDCAP/OD-SI/UAC | [3000-4000bits] |

Observation 4-1:	In typical or basic configurations scenarios, the current SIB1 size can already meet the requirements. However, in complex deployment scenarios the SIB1 size would be the limitation to enable more features.

Regarding the potential essential SI content for 6G, a summary is made in the following table highlighting the largest components.：

Table 2. essential SI types

| types |  | description |
| --- | --- | --- |
| camping related info | basic camping info | Cell selection quality info Basic Cell info: PLMN ID/CellID/TAC Cell/BWP PHY info: (SSB,BW, TDD config,SCS,SS,etc) BCCH/PCCH config Service info(eCallOverIMS/IMS-Emergency) |
|  | feature specific camping info | feature specific bar (e.g. IoT, NTN, IAB bar) feature specific basic Cell info (e.g. NPN) feature specific BWP info (e.g. IoT BWP) |
| Access related info | basic Access related info | UL BWP/RACH info UAC info ConnEstfailure UE Timers |
|  | feature specific Access related info | feature specific UL BPW/RACH info (e.g. for IoT and RACH offload by more carriers) |
| SI scheduling info | basic scheduling info | SI scheduling list |
|  | feature specific SI scheduling | on demand SIB info positioning SI |

The highlighted components should remain in 6G essential SI considering the related functions/features are high likely to be incorporated into 6G (even if not in Day1). On the other hand, considering that 6G targets higher frequencies, it is possible that the physical layer SIB1 size limitation will not be relaxed due to the challenging reception performance. To address the issue of the required SIB1 size exceeding physical layer SIB1 size limitation in a few deployment scenarios, one possible method is to move some information (e.g., access-related information) from SIB1 into the OSI. However, this would lead to performance degradation (i.e., increased access delay by consuming extra time resources) in the majority of deployment scenarios (which doesn't require large SIB1 sizes as observed in current deployments). For the appropriate mechanism of essential SI, we think it should neither restrict the configuration of features (when the required SIB1 size is large), nor occupy additional slots and cause unnecessary delays (when the required SIB1 size is not large, e.g., in typical deployment scenarios). The split of essential SI can also be considered together with RRC modularization. For example, at least basic camping/access related configurations and basic SI scheduling configuration shall be included in SIB1 as a minimum common module based on the modular RRC design perspective. Regarding feature specific camping/access related configurations and features specific SI scheduling configuration, it should be supported to flexibly configure this information in SIB1 or in an additional SIB (e.g. sib1-bis or OSI) according to the required size for essential SI.

Proposal 4:	RAN2 should study appropriate mechanisms for essential system information transmission, so that:

when the size of essential system information is not large (in typical deployment scenarios), it can be transmitted directly via SIB1 as today; and

when the size of essential system information is large (in some special scenarios), it can also be transmitted using additional transmission occasions to transmit some essential system information.

In 5G, network notifies UE of SI change (i.e., BCCH modification other than SIB6, SIB7, SIB8 and posSIBs.) via one bit (i.e., systemInfoModification bit) of Short Message in paging DCI. Upon reception of this notification, the UE needs to reacquire SIB1 to determine if their relevant SIB(s) were modified based on per-SIB value tag. In the last RAN2 meeting, the agreement of "Study mechanisms to improve SI update mechanism" was reached. One potential approach proposed by companies last meeting for this direction is to use finer-grained SI modification indications to reduce the UE's unnecessary SIB1 reading for uninterested SIBs update. However, considering the relatively low occurrence rate of SI modification notification (typically occurring only once every few hours), the benefits of such optimization appears quite marginal. In addition, the finer-grained notifications require more bits in the paging DCI, and these bits would be better left for paging optimization to achieve better energy efficiency. In fact, the power consumption overhead for UEs to monitor paging is much higher than that for system information, mainly because paging occurs more frequently.

Observation 5:	Regarding the SI update mechanism, the benefits for optimizing SI update notification are marginal considering that the occurrence rate of SI updates is relatively low.

RAN1 had some agreements related to SSB such as SSB periodicity, clustered SSB, on-demand SSB:

Agreement Study and evaluate NW energy savings and the impact on UE performance and user experience with respect to 20ms and longer periodicities of sync signal(s) at least for initial access with the following consideration, but not limited to: BS assumptions: - Cell-common signaling (e.g., sync signal(s), broadcast PDCCH, SIB-1, SIB, paging, PRACH), e.g., - Clustered provisioning of different cell-common signaling, - On-demand provisioning of different cell-common signaling, - UE-specific signaling (for low, light, medium loads), e.g., - Clustered provisioning with cell-common signaling, - Unclustered provisioning with cell-common signaling,

As shown in our contribution [1], considering SSB period extension (e.g. 160ms) and the clustered common signal (we assume 4*SSB/SIB1/RO/PO in the clustered structure), we observed 50% and 19% network energy saving gain for different power consumption model of BS respectively in unloaded case.

On the other hand, the long sleep time between clustered common signals will bring non-negligible negative impact on UE’s performance for both IDLE UEs and CONNECTED UEs. For IDLE UEs, the increased SSB period will increase cell searching time and also cause UE’s coverage degradation for initial access. For CONNECTED UEs, if lack of downlink reference signals for tracking time/frequency synchronization persists for some time, the residual timing error and carrier frequency deviation at UEs will accumulate. This may result in synchronization accuracy loss and impact the incoming control and data channel reception negatively. In addition, the longer SSB period will increase the UE’s RSRP measurement time and lead to higher UE energy consumption. Therefore, in order to compensate for UE’s synchronization and measurement accuracy degradation caused by long SSB period, it is necessary to insert some on-demand synchronization signal.

However, as the above discussion should be led by RAN1, RAN2 can further study what the RAN2 impacts are after RAN1 progresses on SSB.

Regarding on-demand SIB1, it can be categorized into two cases: multi-carrier (where there is an anchor carrier providing assistance information for the SIB1-less cell) and single-carrier (where the SIB1-less cell provides the configuration for triggering SIB1 transmission). For single-carrier scenario, a new MIB design is needed to include the necessary configuration for SIB1 request, which is RAN1 responsibility. For multi-carrier scenario, RAN2 needs to determine which carrier (anchor carrier of SIB1-less carrier) provides SIB1 request configuration, receives SIB1 request, and transmits SIB1. Since the SIB1 size/content including whether/how to split the SIB1 is still unclear and the benefit is still being evaluated in RAN1, RAN2 can start studying the details after the basic SIB1 design is clear.

Proposal 5:	RAN2 to study the RAN2 impacts of SSB (e.g., SSB periodicity, clustered SSB, on-demand SSB) based on RAN1 progress, and to study on-demand SIB1 after the basic SIB1 design is clear.

On-demand other SI is a feature present in the initial release of 5G with the benefit of saving network’s energy and signaling. However, this feature has not been successfully commercialized in 5G deployments. If a feature is deployed in the field, in most cases a significant number of UEs should have already implemented the feature. The network will get frequent SI requests for the associated SIB, since once a new UE enters a cell, it needs to send a request. In this case, it does not provide significant benefits compared with always broadcasting the SI. So, there is no strong motivation for the network to use this feature.

Another similar feature is area specific SIB. Despite being included in the initial 5G release, this feature was not implemented due to several practical challenges. From the network perspective, it brings operational complexity without clear benefits for network. The network needs to determine area configurations through system information comparison among multiple gNBs, and further, the gNB needs to coordinate real-time SIB updates between cells to ensure SIB consistency within the same SI area. On the UE side, the energy saving gain is limited by inherent design constraints, since the SIB1 cannot be area-specific even if much of its contents are the same (such as configuration in SIB1 among intra-frequency cells). Especially in case that only a few SIBs are broadcast, the energy saving gains are not very significant since the UE power consumption for SIB1 is still there.

The on-demand OSI and area specific SIB are closely integrated and mutually dependent. The area specific SIB is not implemented, there is no motivation to have on-demand OSI due to frequent SI requests. Since the network always broadcast SIBs, there is no motivation to implement area specific SIB to sacrifice network complexity for UE energy saving. However, if area specific SIB is implemented, the frequent SI requests are not needed for the UEs that move from another cell in the same SI area. The network can achieve the real energy saving gain by reducing the broadcast of certain SIBs.

Observation 6:	On-demand OSI and area specific SI are features in initial release of 5G, but not successfully commercialized, the pain points include:

- deploying on-demand SI without deploying area specific SI will significantly reduce the benefits of on-demand SI due to frequent SI requests from UEs;

- for area specific SI:

  - complexity for the network to determine area configurations through system information comparison among multiple gNBs and coordinate any change of SIBs to ensure that area specific SIBs in the same SI area are the same;

  - SIB1 cannot be area specific even if most of the content in SIB1 is the same, the UE power consumption for SIB1 is still there.

As analyzed above, on-demand OSI and area specific SI can be deployed jointly to reduce the SI transmission on the network side and SI acquisition on the UE side. It can bring noticeable energy saving gains for both network and UE. To overcome the complexity on the network for area specific SIBs, the deployment of SI area needs to be simplified. The area management in the network (e.g., identical SI identification and update synchronization) relying on inter-RAN node comparison/coordination should not be mandatory. Instead, mapping mechanisms can be considered to map the same system information to a common value. Moreover, whether the SIB1 can also be area specific can be considered further pending on SIB1 size/content discussion. The ideal case for energy consumption is that only the essential and variable contents of SIBs need to be transmitted/acquired once the cell is changed.

Proposal 6:	Regarding on-demand SI and area-specific SI, RAN2 should:

conduct joint studies on on-demand SI and area-specific SI to maximize energy-saving gains for both the network and UEs;

simplify the deployment of SI areas, e.g., the area management in the network (identical SI identification and update synchronization) does not have to depend on inter-RAN node comparison/coordination.

2.3	Paging design

In last meeting, the following agreement was reached for paging design principle in 6G.

Agreements =>Paging design should consider energy efficiency and simplity for both UE and Network.

In 5G design, PFs/POs are uniformly distributed in time-domain and the network has to frequently transmit paging signals across all time-domain positions, thus the network will be prevented from entering the sleep mode for energy saving. In 5G-A, in order to enable the adaptation of paging occasions in the time domain for energy saving and ensure no backward compatibility issues, features such as N/Ns adjustments have been introduced. The interval between PFs can be increased while maintaining the same number of total POs as in 5G. However, this method can only achieve a limited level of energy saving, and cannot achieve optimal energy efficiency among all possible paging cycles. For PF and PO distribution in 6G, we understand clustering the POs location within closer time-domain position would be beneficial for network energy saving, which enables network to enter the sleep mode for longer time, thereby significantly improves energy efficiency.

Figure 5. PF/PO distribution in time domain

Proposal 7:	For paging enhancements, RAN2 to study the clustered POs distribution for network energy saving.

3	Conclusion

This contribution makes the following proposals:

Observation 1: The issues for spectrum aggregation in RRC connected mode include the low flexibility on DL/UL pairing and inflexible spectrum usage due to PCell-defined cell level operation.

Proposal 1:	Separate the discussions on cell definition and spectrum aggregation for IDLE and Connected states:

For UEs in CONNECTED state, start from the configuration of separate DL and UL carrier lists, identify any requirements for DL/UL coupling and aggregation-related issues. Cell modelling and definition can be discussed after the above discussion;

  - Those issues can be related to RLM monitoring reference, handover, carrier management (e.g., carrier addition/removal), SIB reception, and random access.

For UEs in IDLE state, study the need to configure multiple DL/ UL carriers via SIB — for example, supporting paging over multiple DL carriers and random access on multiple UL carriers.

Observation 2-1:	The current SI scheduling mechanism restricts the network's ability to enter sleep mode for energy saving.

Proposal 2:	In order to reduce the number of SI windows and thereby decrease the duration of SI transmission, RAN2 should study the mechanisms of mapping SIBs with different periodicities into one SI window.

Observation 3-1:	The current SI scheduling mechanism has become overly complex due to multiple enhancements aiming at better utilizing time-domain opportunities yet at late 5G stage.

Observation 3-2:	The current SI scheduling mechanism leads to that UEs without PWS capability cannot correctly receive the posSIBs scheduled in posSchedulingInfoList during the modification periods with PWS SIB starting and stopping.

Proposal 3:	RAN2 aims to develop a unified scheduling list incorporating both explicit and implicit window positions for all SIs, including positioning SI (even if not supported in 6G day 1) and non-positioning SI.

Observation 4-1:	In typical or basic configurations scenarios, the current SIB1 size can already meet the requirements. However, in complex deployment scenarios the SIB1 size would be the limitation to enable more features.

Proposal 4:	RAN2 should study appropriate mechanisms for essential system information transmission, so that:

when the size of essential system information is not large (in typical deployment scenarios), it can be transmitted directly via SIB1 as today; and

when the size of essential system information is large (in some special scenarios), it can also be transmitted using additional transmission occasions to transmit some essential system information.

Observation 5:	Regarding the SI update mechanism, the benefits for optimizing SI update notification are marginal considering that the occurrence rate of SI updates is relatively low.

Proposal 5:	RAN2 to study the RAN2 impacts of SSB (e.g., SSB periodicity, clustered SSB, on-demand SSB) based on RAN1 progress, and to study on-demand SIB1 after the basic SIB1 design is clear.

Proposal 6:	Regarding on-demand SI and area-specific SI, RAN2 should:

conduct joint studies on on-demand SI and area-specific SI to maximize energy-saving gains for both the network (NW) and UEs;

simplify the deployment of SI areas, e.g., the area management in the network (identical SI identification and update synchronization) does not have to depend on inter-RAN node comparison/coordination.

Proposal 7:	For paging enhancements, RAN2 to study the clustered POs distribution for network energy saving.

4	References

- R1-2507063, Views on energy saving for 6GR, Huawei, HiSilicon, Prague, Czech, October 13-17, 2025.