3GPP TSG RAN WG2 Meeting #132                                                            R2-2508626

# Dallas, USA, Nov. 17th-21st, 2025

Agenda item:	10.2.3

Source:	Ofinno

Title:	Deployments, migration, device types and new services in 6G

Document for:	Discussion and decision

# Introduction

This document discusses our views on 6G system with focus on the deployment scenarios/migration strategies, diverse device types and new services. The agreed 6G SID captures the guidance on the scenarios to start focusing RAN work (as highlighted in yellow on the relevant SI objectives copied below).

(1) Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1, RAN2, RAN3, RAN4]

  - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting.

  - Energy efficiency and energy saving: both for network and device.

  - Enhanced spectral efficiency.

  - Enhanced overall coverage, focus on cell-edge performance and UL coverage.

  - Wider channel BW (at least 200MHz) support for 6G deployments at least > 2 GHz, ~ 7 GHz.

  - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band.

  - Target scalable and forward compatible design for diverse device types.

  - Improved spectrum utilization and operations taking into account diverse spectrum allocations

  - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs.

  - Aim at a harmonized 6G Radio design for TN and NTN, including their integration.

  - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc.

Note1: the term stand-alone architecture does not imply any particular Core network architecture, which is up to SA2 discussion.

…

(7) Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR:

- 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3]

- Study if any additional migration option(s) is needed (other than standalone, MRSS, and inter-RAT mobility between NR-6G). [RAN] [RAN2, RAN1, RAN3, RAN4].  RAN plenary starts this study in March 2026 and will make a decision by September 2026 whether to expand WG SI scope to cover additional migration option(s).

- Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4]

Note: Inclusion of LTE/6G interworking/coexistence aspects may be further discussed based on the requirement from RAN plenary

# Discussion

## Deployment Scenarios and Migration Strategy

Figure below provides a high-level overview of 6G standalone and coexistence/interworking with 5G considering also a 6G/5G MRSS (Multi-Radio Spectrum Sharing) scenario.

The initial 6G study work will focus on a single, standalone (SA) architecture. This decision is a direct lesson from 5G, where numerous non-standalone (NSA) options led to unforeseen complexity, delayed SA deployments, and sometimes a suboptimal user experience. The 6G core may interface with an evolved 5G Core (e5GC), reusing its Service-Based Architecture and key network functions where possible to ensure a smooth migration path.

For seamless migration and spectrum utilization, key enablers identified are:

- 5G-6G Multi-RAT Spectrum Sharing (MRSS): This is considered an essential component from Day 1 to enable the deployment of 6G in existing 5G bands, facilitating a gradual refarming of spectrum. The goal is to make MRSS significantly more efficient than the 4G-5G Dynamic Spectrum Sharing (DSS).

- Intra-6G Carrier Aggregation (CA): CA will be crucial for aggregating new and existing spectrum bands to achieve high performance and competitive user experiences from the start.

- Inter-RAT Mobility: Robust mobility between 5G NR and 6GR is critical for service continuity.

While the study will initially prioritize the SA scenario, RAN Plenary will guide any future work on additional migration options, such as dual-stack or dual-connectivity, after March 2026.

- 6G design should prioritize the study of standalone deployment scenario. Interworking will be based on mobility with 5G NR, supported by MRSS and intra-6G CA. For other migration options, RAN2 should also wait for further guidance from RAN plenary.

## Diverse Device Types in 6G

The evolution toward 6G presents a fundamental challenge in designing a system capable of efficiently supporting a wide range of device types, from low power IoT UEs (e.g., sensors and wearables) to high performance UEs (e.g., eMBB, uRLLC, XR, NTN, AI/ML). Lessons learned from 5G reveal that integrating new or reduced capability devices after the system’s initial design leads to significant fragmentation, inefficiency, and scalability issues. In 5G/4G, low end UEs (e.g., (e)RedCap or (e)MTC were introduced as “Day-2” solutions) forced the specifications to retrofit complex mechanisms for simplification and power saving onto a system originally designed for high end UEs. Summarizing, this reactive approach resulted in complicated backward compatibility requirements, increased signaling overhead, and system fragmentation that limited flexibility and market adoption.

A major source of complexity in 5G arose from its highly granular capability signaling and “toolbox-like” feature framework. Each new device type required extensive capability reporting, leading to large UE Capability reporting messages and sometime unnecessary signaling traffic. Moreover, the abundance of optional features and configuration variants caused uncertainty among vendors and operators, slowing adoption and creating a “chicken-and-egg” problem in commercialization. In addition, the lack of early differentiation mechanisms during initial access further contributed to inefficient resource utilization and excessive power consumption for devices that only required limited system information. These shortcomings demonstrate that the 5G model is not inherently scalable or extensible for diverse device ecosystems.

To address these challenges, 6G aims to implement a unified, scalable framework from Day 1 that can natively accommodate a wide spectrum of device types without resorting to add-on solutions. This unified framework should aim to define a common baseline of mandatory functions across all UE categories while reducing the signaling overhead. A hierarchical capability model, where higher-tier UEs inherit lower-tier functionalities, should be considered to streamline design and ensure consistency while higher-tier UEs are not impacted negatively. 6G system could be designed around the lowest-tier UE requirements, ensuring lean operation and power efficiency while preserving high performance for advanced terminals.

Additionally, early identification of device types during cell (re)selection, access control and access procedure is critical to enable optimized resource allocation, energy savings, and faster connection handling (including setup, resume or re-establishment). RAN2 should study the integration of the device-type differentiation or awareness into the cell (re)selection, access control and initial access process aiming to reduce 6G system overhead, prevent network congestion, and ensure more efficient coexistence between diverse device types. The early identification approach should aim to create a unified and scalable mechanism avoiding the fragmentation of the market/solutions.

The later integration of reduced-capability devices in 5G system led to fragmentation, inefficiency, and scalability issues due to complex backward compatibility, excessive signalling, and lack of early device-type differentiation mechanisms.

- 6G system needs to adopt a unified, scalable framework from Day 1 that natively supports any diverse device type. For this, to study how to enable a hierarchical modelling for diverse device types (e.g., a baseline functionality is defined for any kind of devices, common to all the diverse device types) that allow providing differentiated handling (e.g., during cell (re)selection, access control, initial access, etc).

## New Services in 6G

The evolution toward 6G introduces a new generation of data-centric, intelligent, and immersive services that aim to extend beyond the communication capabilities of 5G. While 5G laid the groundwork for eMBB and URLLC, it was not designed to natively support emerging use cases such as AI-driven networks, Integrated Sensing and Communication (ISAC), XR or even metaverse, holographic communication, Digital Twin or other emergency services. The late, “add-on” integration of such services in 5G created fragmented architectures, inefficiencies in data management, and complex operational dependencies. These limitations highlight the necessity for 6G to adopt a unified and scalable system architecture that can accommodate diverse new service requirements from Day 1.

A central limitation in 5G lies in its fragmented data handling approach. Different categories of operational data (e.g., RAN measurements, AI data sets, and sensing information) are currently processed through independent mechanisms, which may lead to duplication and inefficiency. New 6G services will demand the transfer and processing of massive amounts of non-traditional data (e.g., AI models and sensing results), which require fundamentally different handling from conventional user traffic. To meet these needs, 6G proposals consider the study of a unified data framework or a dedicated data plane to efficiently manage, transport, and expose diverse types of operational and service data. This could help to improve the handling of the data and potentially also enable AI-driven automation, adaptive learning, and intelligent decision-making directly within the RAN.

In parallel, 6G plans to study the harmonization and/or integration of Non-Terrestrial Networks (NTN) with Terrestrial Networks (TN) from day 1. In 5G, NTN support was added as a patchwork solution in later releases, resulting in complexity and limited interoperability. For 6G, the goal is a harmonized, when/if possible, TN/NTN design aiming to maximize the commonality in procedures, signaling, and protocol logic while removing dependencies on GNSS or ephemeris-based frameworks. This unified 6G radio interface should consider the support of multi-orbit constellations (e.g., LEO, MEO, GEO) and for both transparent and regenerative payloads, ensuring seamless connectivity across all environments and enabling true global coverage.

Immersive and interactive services (e.g., XR, holographic communication, and mobile generative AI) will also require 6G to consider some service awareness. These applications impose simultaneous demands for high throughput, low latency, and high reliability. Moreover, 6G might need to also study whether/how to provide a QoE awareness and/or QoS framework capable of adapting to user and service contexts. In addition, it is also proposed to consider emerging AI services that rely on token-based communication introduce new services/traffic.

In summary, the 6G RAN must be designed as a unified, intelligent, and extensible platform that supports AI/ML, sensing, NTN integration, immersive services and potentially emergency new services natively. This approach could mark a decisive shift to avoid (or minimize) fragmented evolution toward a holistic architecture that embeds flexibility, scalability, and service-awareness from Day 1. By consolidating diverse functionalities within a harmonized framework, 6G could aim to provide the foundation for a truly data-driven and ubiquitous communication ecosystem.

5G features added in different releases lead to a fragmented integration of emerging services such as XR, NTN and AI which led to inefficiencies and limited scalability. 5G architecture is not prioritized to enhance the support of diverse, data-centric, and intelligent services.

- 6G system needs to define a unified and scalable architecture that natively can support any emerging service (including e.g., AI/ML, sensing, NTN integration, immersive services).

# Conclusion

The observations captured are the following:

Observation 1.	The later integration of reduced-capability devices in 5G system led to fragmentation, inefficiency, and scalability issues due to complex backward compatibility, excessive signalling, and lack of early device-type differentiation mechanisms.

Observation 2.	5G features added in different releases lead to a fragmented integration of emerging services such as XR, NTN and AI which led to inefficiencies and limited scalability. 5G architecture is not prioritized to enhance the support of diverse, data-centric, and intelligent services.

The proposals captured are the following:

Proposal 1.	6G design should prioritize the study of standalone deployment scenario. Interworking will be based on mobility with 5G NR, supported by MRSS and intra-6G CA. For other migration options, RAN2 should also wait for further guidance from RAN plenary.

Proposal 2.	6G system needs to adopt a unified, scalable framework from Day 1 that natively supports any diverse device type. For this, to study how to enable a hierarchical modelling for diverse device types (e.g., a baseline functionality is defined for any kind of devices, common to all the diverse device types) that allow providing differentiated handling (e.g., during cell (re)selection, access control, initial access, etc).

Proposal 3.	6G system needs to define a unified and scalable architecture that natively can support any emerging service (including e.g., AI/ML, sensing, NTN integration, immersive services).