3GPP TSG-RAN WG2 #132	R2-2508674

Dallas, USA, 17 – 21 November 2025

Agenda item:		10.3.3.2

Title:		Discussion on possible approaches for energy efficiency

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

In RAN2#131bis, RAN2 discussed the energy efficiency a little bit with the conclusions on the way forward towards this meeting [1]. In this contribution, we discuss the legacy mechanism for energy efficiency and provide our views for baseline approaches in 6G.

Design goal

1. 	Strive to develop solutions that consider network energy efficiency and UE power saving

For next meeting companies are encouraged to identify what UE/NW power saving features can be considered and improved in 6G while avoiding specifying multiple features for the same purpose. Focus on solutions that have RAN2 impacts.

# 2. Discussion

2.1	General principle

The power consumption performance directly determines the availability, user experience, and ecological scalability of communication services. No matter it is daily calls, mobile office work, HD video/streaming, or IoT data collection, excessive power consumption causing device out of power frequently may interrupt the service process, therefore UE power saving technology is promising to be considered from day one.

With different feature and technology development, e.g., extensive native AI/ML, massive machine-type communication, it will bring exponential growth in network scale, the number of connected devices, and data traffic, which implies that the network energy consumption is becoming increasingly severe problem compared with 5G. It is foreseen that high cost of OPEX of operator will hinder the successful deployment of 6G network. Therefore, NW energy efficiency is promising to be configured from day one.

Currently companies are considering the concept of joint design for both UE power saving and NW energy efficiency. In terms of power consumption, one side enhancement may bring potential harm to other side. For example, for UE power saving, when the PEI, DCP and LP-WUS is deployed, it is friendly for UE power saving but requires the NW to send another wake-up signal to the UE, which oppositely increase NW power consumption. For NW energy saving, when on-demand SSB/SIB is deployed, it is friendly for NW energy efficiency but requires the UE to request on-demand information, which oppositely increase UE power consumption and latency. Having said this, we believe that from design perspective, UE power saving specific or NW energy efficiency specific enhancement/technology still needs to be considered separately, but we need to keep this in mind that when design for one side, the other side impact should be carefully analysed so as to achieve a better trade-off between both sides.

Proposal 1: When designing power saving technology for one side (e.g., UE-side or NW-side), the impact including power consumption on other side should also be considered.

In 6G, RRC_CONNECTED and RRC_IDLE are still promising RRC state while potential RRC_INACTIVE or power saving sub-state will be further studied. For RRC_IDLE, although UEs in RRC_IDLE do not have many services need to be handled (e.g., only for SI, paging, measurement), it dominates most of the time for UEs staying in NW connection, which implies that the power consumption from RRC_IDLE should not be neglect. For RRC_CONNECTED, the power consumption is way too higher compared RRC_IDLE depending on different service type. High traffic communication and massive computing/processing will drop UE battery quickly. It should also be considered to bring some power saving enhancements for RRC_CONNECTED compared with the 5G NR.

Proposal 2: Energy efficiency / power saving features better than the NR should be studied for all RRC states.

2.2	UE power saving

In NR, multiple power saving technology is introduced from RAN2 perspective:

In Rel-15, discontinuous reception (DRX) is introduced to let UE enter sleep mode if there is no data transmission and BWP adaptation is used to dynamically switch the BWP considering data traffic.

In Rel-16, DCP is introduced to further indicate whether the UE should start C-DRX active timer or not. SCell dormancy is to reduce PDCCH monitoring when no data is transmitted in the SCell. And RRM measurement relaxation enables RRC_IDLE/INACTIVE UE to perform relaxed measurement. Furthermore, multiple UE preferences for power saving is included in UAI.

In Rel-17, to further reduce the power consumption of paging monitoring, PEI mechanism is introduced to indicate whether the UE should monitor paging or not. TRS can be configured during RRC_IDLE/INACTIVE for precise synchronization to make the synchronization duration longer. RLM/BFD relaxation is also introduced by UAI mechanism.

In Rel-19, LP-WUS mechanism is introduced by utilizing a separate UE device (which is a low power consumption device) to replace paging monitoring during RRC_IDLE/INACTIVE and PDCCH monitoring during RRC_CONNECTED for more power saving gain, meanwhile, further RRM measurement relaxation is enabled by using LP-SS measurement.

Looking at the NR UE power saving technologies, there are mainly two types of power saving concept. One is trade-off between communication performance (e.g., data latency) and power saving gain by introducing DRX, WUS mechanism. Another one is trade-off between mobility performance (e.g., cell reselection, handover) and power saving gain by introducing measurement relaxation. And the following table is showing some pros and cons for these technologies.

Observation 1: Looking at the NR UE power saving technologies, there are mainly two types of power saving concept. One is trade-off between communication performance (e.g., data latency) and power saving gain by introducing DRX, WUS mechanism. Another one is trade-off between mobility performance (e.g., cell reselection, handover) and power saving gain by introducing measurement relaxation.

Table-1: analysis for power saving technologies

| Technology family | Pros | Cons |
| --- | --- | --- |
| DRX-like e.g., C-DRX, paging DRX | 1. Introduce sleeping time, by deactivates part of device component and no signalling exchange, power saving can be achieved. 2. Facilitate NW centralized schedule during active time, to avoid NW operation during the whole time domain. 3. Long DRX and short DRX can balance power consumption and latency. | 1. May cause data transmission delay during sleeping time. 2. blindly entering active time every cycle decreasing power saving gain 3. Some power consumption in switching between device “turn-on” and “turn-off”. |
| WUS-like e.g., DCP, PEI, LP-WUS | 1. Avoid unnecessary DRX active time, further extend sleeping time by WUS indication. 2. low power consumption monitoring (e.g., LR for LP-WUS) 3. subgroup-based WUS can reduce false alarm issue | 1. NW need to additionally send WUS signal, increase signaling overhead and power consumption. 2. Some risks of WUS detection failure. 2. Some power consumption in switching between device “turn-on” and “turn-off” (e.g., MR/LR for LP-WUS) |
| Measurement relaxation | 1. reduce measurement instance for more power saving gain 2. reduce RF switching time for inter frequency measurement | 1. measurement accuracy is decreased 2. hysteresis of mobility decision, causing HO failure for RRC_CONNECTED, camping problem for RRC_IDLE |

For 6G, DRX-like, WUS-like and measurement relaxation are still three main aspects for UE power saving. These power saving technologies can be taken as baseline and be further enhanced considering their pros and cons. However, it should be noted that in 5G, we have introduced multiple duplicated features e.g., DCP+LP-WUS/PEI+LP-WUS for the same purpose, which increases practical deployment complexity. Therefore, multiple duplicated solution should be avoided.

Proposal 3: DRX-like, WUS-like and measurement relaxation can be considered as baseline to further study 6G UE power saving technologies, meanwhile avoiding duplicated solution for one same purpose.

2.3	NW energy efficiency

In 5G, Network Energy Saving (NES) features were introduced in Rel-18 and Rel-19, for example Cell DTX/DRX, on-demand SSB for SCell, on-demand SIB1 in capacity cell (covered by coverage cell A). All these are attractive, while unfortunately they were introduced in later release and thus it may not be widely available in the commercial system considering the huge number of legacy UEs. These NES features should be introduced from Day1 in 6G. Note that system information aspects should be discussed in the other sub-agenda (10.3.2.3).

The most attractive function from energy saving perspective is the Cell DTX/DRX, which mainly intends to achieve reasonable trade-off between the performance (e.g. cell throughput) and the energy saving in low to medium cell load case. Although 5G NR Cell DTX/DRX targets the RRC_CONNECTED UEs, 6G Cell DTX/DRX should target the UEs in all the RRC states. For each RRC state, there are different UE impact as follows:

- RRC_IDLE/INACTIVE: SSB, SIB1, Other SI, paging, random access, mobility (e.g., measurements, cell reselection)

- RRC CONNECTED: SSB, SIB1, paging short message, random access, C-DRX, UL transmission (e.g., SR/BSR/DSR/CG), mobility (e.g., measurements and report, mobility command)

Note: if new sub-state is supported, the impact depends on its modelling.

For the RRC_IDLE/INACTIVE, the main impact is SSB, SIB1 and paging reception for DL and random access for UL. For the RRC_CONNECTED, the main impact is C-DRX with respect to the data transmission opportunities and mobility related actions (e.g., measurement report, mobility command like HO command, LTM cell switch command). For SSB and SIB1, as long as we take into account RRC_IDLE UEs, the impact to RRC_CONNECTED UEs can be marginal or limited. For the NES study, Cell DTX/DRX should be considered with system information design and firstly intended for low to medium cell load case.

Proposal 4: Cell DTX/DRX should be studied along with system information design mainly for low to medium cell load case and for all RRC states.

# 3. Conclusion

In this contribution, we discussed the legacy mechanism for energy efficiency (e.g., UE power saving, Network energy saving) and made the following proposals for the baseline approaches in 6G.

General:

Proposal 1: when designing power saving technology for one side (e.g., UE-side or NW-side), the impact including power consumption on other side should also be considered.

Proposal 2: energy efficiency / power saving features better than the NR should be studied for all RRC states.

UE power saving:

Observation 1: Looking at the NR UE power saving technologies, there are mainly two types of power saving concept. One is trade-off between communication performance (e.g., data latency) and power saving gain by introducing DRX, WUS mechanism. Another one is trade-off between mobility performance (e.g., cell reselection, handover) and power saving gain by introducing measurement relaxation.

Proposal 3: DRX-like, WUS-like and measurement relaxation can be considered as baseline to further study 6G UE power saving technologies, meanwhile avoiding duplicated solution for one same purpose.

NW energy saving:

Proposal 4: Cell DTX/DRX should be studied along with system information design mainly for low to medium cell load case and for all RRC states.

# References

[1] RAN2#131bis Chair notes