3GPP TSG RAN WG2 #132			R2-2508677

Dallas, USA, Nov. 17th – 21st, 2025

Source: 	Fainity Innovation

Title:	Views on 6GR Energy Efficiency

Agenda item:	10.3.3.2

Document for:	Discussion

Introduction

Study on 6G Radio (6GR) with enumerated objectives in RP-251881 [1] was approved in RAN Plenary #108. As per the meeting content and agenda item in RAN2#131bis [2], 6G Common CP/UP includes security, AI, data framework, and energy efficiency.

The agreements for 6G common CP/UP in RAN2#131bis [2] are listed as follows for energy efficiency.

- Strive to develop solutions that consider network energy efficiency and UE power saving - For next meeting companies are encouraged to identify what UE/NW power saving features can be considered and improved in 6G while avoiding specifying multiple features for the same purpose. Focus on solutions that have RAN2 impacts.

According to the agreements, in RAN2#131bis some directions for study and deign are determined, yet other considerations or proposals are still welcomed. In this contribution, we discuss the energy efficiency aspects and will propose some aspects based on the directions which are agreed as well as the aspects with some other considerations.

Reducing Measurement Efforts

From our perspective, there are two ways to achieve UE’s energy efficiency by reducing the measurement efforts: measurement relaxation and measurement offloading. Measurement relaxation was initially proposed in Release 16 for UE power saving, while the NR version of measurement relaxation mechanism was targeted at the RRC_IDLE and RRC_INACTIVE UEs. As for measurement offloading, considering low-power radio (LR) is introduced in Release 19, the measurement effort can be transferred from SSB to low-power SS.

Measurement relaxation can be summarized that the UE is allowed to measure with a longer requirement under some conditions. In Release 16 measurement relaxation, the conditions include that the UE is near the cell center and that the UE is almost static. In our view, more conditions can be considered to utilize measurement relaxation and further reduce the measurement efforts. For example, the UE can relax the measurement requirement on the neighbor cells when there is adequate number of neighbor cells with proper link qualities. The measurement requirement is designed to guarantee that the information is updated in time so that the UE can always have up-to-date information of the cell quality for performing cell selection/reselection. When the UE identifies that there are ample qualified neighbor cells, the other neighbor cells are less important to the UE because the UE seldom select the other neighbor cells. As such, the UE can relax the measurement requirement on the other neighbor cells (with an even longer period) to reduce the measurement effort. In addition, since RRC_CONNECTED state is the most power-consuming state, we think RAN2 can discuss the conditions and feasibility of measurement relaxation in CONNECTED state to maximize power saving gain, with the assumption that the RRM performance is not degraded (or with acceptable degradation).

Observation#1: For a UE with adequate qualified neighbor cells, RRM measurement requirements on other neighbor cells could be relaxed.

Proposal #1: RAN2 further study the potential extension and enhancement for measurement relaxation, especially in RRC_CONNECTED.

Another consideration is the impact on AI/ML. For example, with the aid of AI/ML, the measurement efforts can be reduced through prediction. Then, the question will be how the reduction on measurement efforts is reflected on the UE’s behavior while the related AI/ML functionalities are activated and the reduction will not degrade the AI/ML performance monitoring, inference reporting, etc. For the question, we need to further discuss what should be defined for the reduction on measurement efforts because different solutions have different energy saving gains and different flexibility. For example, we can use a relaxed measurement to loosen the requirement with an extended periodicity, which may save more energy (if the periodicity is configured properly) but have less flexibility on the detailed measurement behavior if inference is predicted and expect the measurements could be relaxed based on the prediction; or, we can specify a pattern indicating which period the UE is required to perform measurement, which is flexible for adjusting the UE’s measurement by changing the pattern but may cause additional signalling overhead and less power saving gain. Another question is whether the UE can use the inference result of the serving cell to determine S-measurement or relaxed measurement. Since AI/ML may sometimes provide error predictions, the determination of S-measurement and relaxed measurement can therefore be erroneous. However, if the error is within a tolerable range, the impact on RRM performance can be negligible.

Observation#2: For AI/ML-aided prediction, different configuration/behavior on measurement relaxation leads to a tradeoff between energy saving gain and flexibility.

Observation#3: For a UE using AI for serving cell quality prediction, the possibly erroneous predicted results may lead to erroneous determination of s-measurement and relaxed measurement. However, if the error of the predicted result is within a tolerable range, the impact on RRM performance can be negligible.

Proposal#2: RAN2 further study the AI/ML for measurement relaxation, considering (1) the skipping pattern or the relaxed measurement requirement for AI/ML-based measurement and (2) feasibility to apply inference result of serving cell to the evaluation of S-measurement and relaxed measurement.

In Release 19, LR is introduced as an always-on radio, consuming comparably low power, and detecting low-power wake-up signal (LP-WUS) or low-power synchronization signal (LP-SS). The property of LP-SS detection is attractive for energy efficiency design in 6GR since it serves as a reference signal and can be measured. We think, in addition to measurement relaxation, the offloading from main radio (MR) to LR is another feasible aspect to achieve UE-side energy efficiency. The offloading means the UE measures the quality of LP-SS via LR instead of measuring SSB via MR. Since the LP-SS is transmitted with lower power than SSB, it is intrinsic that a UE must be able to receive SSB of a cell if the UE can receive LP-SS of that cell. As a result, we can consider that in case the UE can detect the LP-SS of a neighbor cell, the UE can skip the measurement of SSB of that neighbor cell. Some other offloading scenarios can be further studied, and with the offloading, another issue will be the evaluation; for example, the S-criteria for cell selection/reselection in idle state, or, for another example, the CHO condition. If the LP-SS is measured instead of SSB, it is necessary to have a calibration, or offset, to compare the LP-SS-based measurement and SSB-based measurement.

Observation#4: When a UE can detect LP-SS of a cell, the UE must be able to detect SSB, if transmitted, of that cell.

Proposal#3: Take NR measurement offloading as baseline, RAN2 further study the measurement offloading to LR in following aspects: (1) the condition for offloading to LR, (2) the comparison between LP-SS-based measurement and SSB-based measurement.

Network Energy Saving

In NR, cell DTX/DRX is configured on a per-UE basis. That is, for a RRC_CONNECTED UE, a cell DTRX configuration is preconfigured, and a separate indication of activation/deactivation of the cell DTRX configuration is applied. As such, when multiple connected UEs are in the cell, the NW needs to configure the cell DTRX configuration for each UE, which causes extreme signalling overhead. As a result, we suggest to consider the feasibility of a cell-specific cell DTRX configuration, which applies to all the UEs in all RRC states. This can align all the UE’s knowledge of the NW’s NES status and behaves the same according to the cell-common information.

Observation#5: Although UE-specific cell DTRX configuration provides flexibility, it is not scalable as the same or similar configuration and indication is transmitted on a per-UE basis.

Proposal #4: RAN2 study both the cell-specific DTRX and UE-specific DTRX configuration method.

OD-SSB and SSB-less are other designs in NR NES working item. In NR, OD-SSB and SSB-less concepts are limited to SCell only. That is, OD-SSB and SSB-less is determined by the PCell and is also controlled (e.g., whether to trigger the OD-SSB transmission) by the PCell. The UE still needs to connect to a PCell and obtain the SCell’s SSB configuration, which is time and power consuming. For 6GR, we think a standalone OD-SSB structure can be considered, and the first question is to address the initial access problem because the UE may not detect the cell. An anchor cell can still be introduced, but the UE does not necessarily connect to the anchor cell; instead, the UE can acquire the OD-SSB cell’s information via the RA procedure to the anchor cell. With the standalone OD-SSB cell design, more energy saving gain is expected to be achieved.

Observation#6: CA-based OD-SSB requires the UE to first connect to a PCell, which lengthen the latency.

Proposal #5: RAN2 study the standalone OD-SSB PCell mechanism for NES, and the OD-SSB acquisition could be initiated by UE.

Conclusions

In this contribution, we share our views and observations on 6G and propose:

Observation#1: For a UE with adequate qualified neighbor cells, RRM measurement requirements on other neighbor cells could be relaxed.

Observation#2: For AI/ML-aided prediction, different configuration/behavior on measurement relaxation leads to a tradeoff between energy saving gain and flexibility.

Observation#3: For a UE using AI for serving cell quality prediction, the possibly erroneous predicted results may lead to erroneous determination of s-measurement and relaxed measurement. However, if the error of the predicted result is within a tolerable range, the impact on RRM performance can be negligible.

Observation#4: When a UE can detect LP-SS of a cell, the UE must be able to detect SSB, if transmitted, of that cell.

Observation#5: Although UE-specific cell DTRX configuration provides flexibility, it is not scalable as the same or similar configuration and indication is transmitted on a per-UE basis.

Observation#6: CA-based OD-SSB requires the UE to first connect to a PCell, which lengthen the latency.

Proposal #1: RAN2 further study the potential extension and enhancement for measurement relaxation, especially in RRC_CONNECTED.

Proposal#2: RAN2 further study the AI/ML for measurement relaxation, considering (1) the skipping pattern or the relaxed measurement requirement for AI/ML-based measurement and (2) feasibility to apply inference result of serving cell to the evaluation of S-measurement and relaxed measurement.

Proposal#3: Take NR measurement offloading as baseline, RAN2 further study the measurement offloading to LR in following aspects: (1) the condition for offloading to LR, (2) the comparison between LP-SS-based measurement and SSB-based measurement.

Proposal #4: RAN2 study both the cell-specific DTRX and UE-specific DTRX configuration method.

Proposal #5: RAN2 study the standalone OD-SSB PCell mechanism for NES, and the OD-SSB acquisition could be initiated by UE.

References

- RP- 251881, New SID: Study on 6G Radio

- RAN2#131bis Chair Notes