3GPP TSG-RAN2#132	R2-2508703

Dallas, USA, November 17-21, 2025

Agenda Item:	10.2.2

Source:	Ericsson

Title:	Essential requirements for NTN and way forward in the common design with TN

Document for:	Discussion, Decision

# 1	Introduction

This contribution discusses aspects of “TN/NTN integration” in the study on 6GR [1].

Following the agreement from the previous meeting and agenda for this meeting, we focus on identifying the essential requirements for NTN and critical functionality that is to be considered in the common design with TN. Other NTN-specific features can be left for later. In addition, we discuss how to move forward with “TN/NTN integration” during the study.

# 2	Discussion

It is key that the 6G specifications support NTN in order to enable ubiquitous coverage for basic mobile broadband (MBB), FWA, and IoT services. 6G should support: a harmonized radio interface for TN and NTN, seamless mobility within NTN and between TN and NTN, and GNSS-free operation.

However, NTN differs from TN in several important aspects, necessitating NTN-specific considerations. In the following, we elaborate on the essential requirements (from RAN2 perspective) to support NTN in 6G, that is, to enable a UE to access 6G systems via NTN and seamlessly move between TN and NTN. Moreover, we attempt to link these essential requirements for NTN to the different RAN2 topic areas where they will likely be relevant.

Note that, so far, the RAN2 discussions were organized in five areas, namely: General aspects, CP, UP, common CP/CP, and mobility. In the proposals below, we refer to these areas.

## 2.1	Essential requirements for NTN

### #1 Coping with large and varying UE-gNB propagation delay

In NTN, the UE-gNB propagation delay is not only significantly larger than in TN but also varies in time for non-GSO satellites. Moreover, different UEs in the same cell will experience a different UE-gNB propagation delay and delay variation, i.e., the change of the UE-gNB propagation delay depends on where the UE is located in the cell, even if the UE is not moving. And naturally, a UE will experience different UE-gNB propagation delays for cells served by different satellites. While these effects also exist in TN, they are orders of magnitude stronger in NTN.

Large UE-gNB propagation delay, for example, may significantly impact the UE performance in the presence of (H)ARQ and in-order delivery mechanisms in the UP protocol stack if not accounted for. Likewise, it may cause problems for the timing relationships for certain CP and UP protocols. Time- and spatially varying UE-gNB propagation delay, in turn, may cause a UE to miss a (response) message or to fail in measuring a reference signal if not accounted for.

It is therefore a key requirement for NTN that the 6G protocols allow for large as well as fast time- and spatially varying UE-gNB propagation delay. We expect this requirement to impact primarily the UP discussions, since large UE-gNB propagation delays call for consideration in UP procedure timings, timer value ranges, sequence number spaces, etc. to ensure optimal UE performance. In addition, it may also impact the mobility discussions, since time- and spatially varying UE-gNB propagation delay may need to be considered in the measurement framework, e.g., to facilitate the correct measurement timing.

The 6G protocols must allow for (cope with) large as well as fast time- and spatially varying UE-gNB propagation delay, which is to be considered in the UP discussions and mobility discussions (measurement framework).

### #2 Coping with satellite mobility

Another fundamental difference between TN and NTN is satellite mobility. In TN, the gNB typically does not move. In NTN, however, satellites move with respect to any fixed point on Earth, except for satellites on a geostationary orbit (GEO). This is especially relevant for satellites on a low earth orbit (LEO), which is a very important type of orbit for NTN deployments, particularly for MBB. At 600 km orbital height, for example, the satellite speed is 7.56 km/s [2]. This leads to frequent cell changes (cell-reselection or handover) for all served UEs independent of UE movements. Even for so-called quasi-Earth-fixed cells, this means that a LEO satellite can serve a cell (or an area) for only a few minutes, e.g., 5 minutes, before another satellite takes over (e.g., with a new cell). This we call satellite switch.

In addition, any non-GEO satellite, at some point, loses its connection (the feeder link) to the satellite gateway (GW) and establishes a new connection to a new satellite GW, to stay operational. This we call feeder-link switch.

Satellite switch and feeder-link switch are frequent phenomena in NTN deployments, especially for the important LEO case. It is thus a key requirement for NTN that the 6G protocols enable satellite and feeder-link switches with low interruption time, signaling overhead, but also limited network and UE complexity (including energy consumption) both in Connected and Idle/Inactive mode. We expect this requirement to impact primarily the mobility discussions on unified mobility and measurement frameworks. However, it might also play a role in the CP discussions, e.g., when it comes to system information related issues.

The 6G protocols must facilitate (frequent) satellite and feeder-link switches in NTN, with low interruption time, signaling overhead, and network and UE complexity, both in Connected and Idle/Inactive mode, which is to be considered in the mobility discussions and potentially CP discussions, e.g., system information.

### #3 GNSS-less NTN operation

In previous releases, NTN operation (NR and IoT) requires the UE to be equipped with a GNSS module, which is a disadvantage. 6G NTN cannot fulfil its role as an essential complement to 6G TN while depending on GNSS, which is prone to jamming and spoofing. We need to study how to overcome the dependency on GNSS for mobility, RRM measurement framework, geofencing, etc. In many cases, this dependency can be overcome by avoiding making mechanisms rely on the (exact) UE location.

As an additional motivation: By overcoming the dependency on GNSS, 6G NTN also overcomes the coverage limitations of GNSS (such that 6G NTN coverage is not limited by GNSS coverage, as is so far the case, e.g., for NR NTN coverage) as well as the energy consumption of the GNSS receiver (UE battery lifetime impact), which is particularly important for lower-capability UEs.

We therefore consider it an essential requirement for NTN to operate without GNSS.

RAN2 should start from the assumption that the UE is not (precisely) aware of its own location. The UE may instead, for example, consider a reference point for a cell or a beam, as a starting point, to determine the assumed UE-gNB propagation delay. The CP/UP procedure timings should account for the variance in the actual UE-gNB propagation delay for different UEs within the cell or beam. Note that, as described above, the actual UE-gNB propagation delay will differ from the assumed one depending on where the UE is located with respect to the considered reference point. We thus expect this requirement to have an impact on the CP and UP discussions (also refer to the essential requirement #1 Coping with large and varying UE-gNB propagation delay). Moreover, the same problem exists for measurement timings. GNSS-less NTN operation may therefore also impact the mobility discussions.

In our view, RAN2 should start the discussion on GNSS-less NTN operation in 6G independent of the progress in the RAN1-led study on GNSS-resilient NR-NTN operation [3]. The reason is two-fold: GNSS-resilient NR-NTN operation is limited to the NR-NTN baseline, whereas GNSS-less NTN operation in 6G can be designed from scratch, thus providing more room to derive the best possible design for 6G. More importantly, the RAN1-led study has no (formal) RAN2 impact. Hence, it will not make progress on RAN2 aspects. Instead, it is important that RAN2, from the start, assumes that the UE is not aware of its location (just like in TN) when designing the RAN2 protocols to avoid these dependencies. That way, 6G NTN becomes more robust and we can avoid additional complexity considering the availability of GNSS (or other UE positioning methods) in RAN2 procedures.

6G NTN must operate without GNSS, and it is assumed in the RAN2 discussions that the UE does not have (precise) knowledge of its own location.

### #4 UE location awareness at the network

NTN cells are generally (very) large. NTN LEO cells may have diameters between a few tens of kilometers and up to several hundreds of kilometers, typically about 100 km to 200 km [2]. An NTN MEO or GEO cell can be larger. Without any mechanism, the network only knows that the UE is located somewhere within the (potentially very large) coverage area of the serving cell, but it does not know more than that. There are, however, cases where the network needs to know approximately where (within the serving cell’s coverage area) the UE is located.

One important case is that NTN cells may, due to their sheer size, cover areas in multiple countries. In this case, the network needs to know in which country the UE is located to connect it to a corresponding core network, which is necessary to fulfil regulatory and safety requirements, e.g., lawful intercept and emergency calls. Besides, there may be other cases where network can benefit from some awareness of the UE’s location within the serving cell to properly configure the UE.

We thus conclude that it is an essential requirement for NTN that the network has some sort of UE location awareness. To meet this requirement, it is key that a mechanism introduced to create UE location awareness, if any, does not violate security or privacy and does not require user consent. The network must have an awareness of the UE location with sufficient accuracy for the relevant use case(s), not more. RAN2 may start by considering as a baseline the solution for network-verified UE location specified for NR NTN.

We propose to capture the requirement on UE location awareness at the network in the TR and consider as a baseline the network-verified UE location solution. Yet, this discussion is likely to impact other WGs too.

In 6G NTN, the network must have (means to determine) UE location awareness with sufficient accuracy for relevant use cases, but without raising security/privacy concerns or requiring user consent. This requirement should be captured in the TR, considering as a baseline the network-verified UE location solution.

### #5 Support of inter-satellite links

As mentioned above, 6G NTN shall complement 6G TN to enable truly ubiquitous coverage for basic services. Ubiquitous coverage shall not be limited to certain areas, e.g., areas where it is feasible to deploy a satellite GW, but instead, expands to all areas on Earth including seas/oceans.

Particularly LEO satellites, due to their comparatively low altitude, cannot always maintain a direct connection (feeder link) with a satellite GW. At some point, they will inevitably fly over areas, e.g., oceans, where they cannot reach any satellite GW directly. In order to anyway serve UEs in those areas and provide the desired ubiquitous coverage with 6G, they need to rely on other satellites to forward the traffic. For this reason, the 6G protocols should support inter-satellite links (ISLs). Without the support of ISLs, the important objective on ubiquitous coverage cannot be achieved.

For regenerative payload/architecture, the support of ISLs may not have so much RAN2 impact. Regenerative payload is, in our view, most suitable for supporting ISLs and a preferable starting point for the RAN2 discussions, e.g., considering as a baseline the solutions for regenerative payload introduced previously. For transparent payload/architecture, on the other hand, the appearance and disappearance of ISLs may likely cause changes in the UE-gNB propagation delay (see the essential requirement #1 Coping with large and varying UE-gNB propagation delay), which may call for more RAN2 work, e.g., an adjustment/extension of the common TA or Kmac.

We propose to capture the requirement on the support of inter-satellite links in the TR and consider it in the UP discussions and in the mobility discussions, e.g., in the measurement framework. Nevertheless, the discussion is likely to also impact other WGs.

The 6G protocols must support inter-satellite links. This requirement should be captured in the TR and can be considered in the UP discussions and mobility discussions (measurement framework).

### #6 Assistance information to facilitate TN-NTN mobility

To facilitate seamless mobility between TN and NTN, it may be helpful for the UE if TN cells provide, e.g., via system information broadcast, certain assistance information related to NTN cells, e.g., satellite assistance information. Similarly, it may also be helpful if NTN cells provide certain assistance information related to TN cells. In NR and IoT NTN in previous releases, satellite assistance information has been introduced. For 6G, however, we do not have any baseline yet, neither for NTN nor for TN. It is therefore unclear and impossible to conclude at this point whether such assistance information is an essential requirement for NTN. RAN2 may revisit this topic at a later point.

Due to the lack of a baseline for 6G, it is yet unclear and impossible to conclude whether the provisioning of satellite assistance information is necessary for NTN.

## 2.2	Way forward in the common design for TN and NTN

There is a clear objective to create a common design for 6GR for TN and NTN. This common design needs to address the essential requirements for NTN. However, it is key to prevent any divergence of functionality for TN and NTN in the 6G protocol stack, as this would hinder the commercial success of 6G NTN. In our view, the best way to ensure a common design is to have common discussions. This means there should be no NTN sessions outside of the main room, and the time that is dedicated to NTN discussions in the main room should be minimized. NTN sessions in the main room should be used to address NTN-specific aspects that do not fit or relate to any other discussion, if any. We therefore propose that the NTN-specific requirements, as agreed in RAN2#132, are addressed in the discussions for CP, UP, mobility, etc. in future RAN2 meetings.

Moreover, any NTN-specific requirements or functionality should be well-motivated by clear and convincing (including commercially relevant) scenarios. RAN2 should not jeopardize a simple, common design for 6GR, i.e., should avoid additional specification and implementation complexity, if only to address niche NTN scenarios.

Let us recall that one of the benefits of NTN technologies being developed within 3GPP is that they benefit from the global scalability of 3GPP technologies, namely: The 3GPP standards ensure global interoperability of mobile devices and network equipment from different vendors, which fosters a massive, unified market, thereby driving down unit costs for everyone (through economies of scale). It is therefore crucial that we focus on the main, commercially promising NTN scenarios and that NTN is supported in 6G in a way that does not lead to market fragmentation or slower time to market, especially in the first release.

NTN-specific requirements must be well-motivated by clear and convincing scenarios (addressing the demands of the mass market, not niche markets) and are addressed in the discussions on CP, UP, mobility, etc. in future RAN2 meetings.

# 3	Conclusion

We observed the following:

Observation 1	Due to the lack of a baseline for 6G, it is yet unclear and impossible to conclude whether the provisioning of satellite assistance information is necessary for NTN.

We propose the following:

Proposal 1	The 6G protocols must allow for (cope with) large as well as fast time- and spatially varying UE-gNB propagation delay, which is to be considered in the UP discussions and mobility discussions (measurement framework).

Proposal 2	The 6G protocols must facilitate (frequent) satellite and feeder-link switches in NTN, with low interruption time, signaling overhead, and network and UE complexity, both in Connected and Idle/Inactive mode, which is to be considered in the mobility discussions and potentially CP discussions, e.g., system information.

Proposal 3	6G NTN must operate without GNSS, and it is assumed in the RAN2 discussions that the UE does not have (precise) knowledge of its own location.

Proposal 4	In 6G NTN, the network must have (means to determine) UE location awareness with sufficient accuracy for relevant use cases, but without raising security/privacy concerns or requiring user consent. This requirement should be captured in the TR, considering as a baseline the network-verified UE location solution.

Proposal 5	The 6G protocols must support inter-satellite links. This requirement should be captured in the TR and can be considered in the UP discussions and mobility discussions (measurement framework).

Proposal 6	NTN-specific requirements must be well-motivated by clear and convincing scenarios (addressing the demands of the mass market, not niche markets) and are addressed in the discussions on CP, UP, mobility, etc. in future RAN2 meetings.

# 4	Reference

RP-251881, New SID: Study on 6G Radio, RAN#108, June 9-13, 2025.

3GPP TR 38.821, Solutions for NR to support non-terrestrial networks (NTN), V16.2.0, March 2023.

RP-251933, 	Study on GNSS (Global Navigation Satellite System) resilient NR-NTN (Non-Terrestrial Networks) operation, RAN#108, June 9-13, 2025.