3GPP TSG-RAN WG2 #132	R2-2508706

Dallas, USA, Nov. 17th – 21st, 2025

Agenda Item:	10.4

Source:	InterDigital, Inc.

Title:	Connected Mode Mobility for 6GR

Document for:	Discussion

# Introduction

The study on 6G contains the following mobility related objective [1]:

- Mobility for 6GR (for all RRC states), including related RRM [RAN2, RAN1, RAN4, RAN3]

During the initial discussion of the mobility topic in RAN2-131bis, the following agreements were made [2]:

Agreements

1	Study mobility with the following requirements in 6G mobility design:

-	Minimize interruption time and ensure service continuity (i.e. minimize throughput degradation during mobility). Consider complexity and gains when discussing solutions.

-	Robustness of mobility procedures

-	Energy efficiency for both UE and NW

2	Study aspects related to mobility (e.g. early DL/UL synchronization, UE configuration processing, pre-configurations, conditional handover, early CSI acquisition)

This contribution outlines initial views on mobility in 6G, with a focus on Connected mode operation.

# Discussion

Mobility in 6G Connected mode should be built on a single design for the mobility framework that supports L3-based mobility, LTM and CHO. By consolidating existing mechanisms, streamlining measurements and reconfiguration, and enabling low-latency procedures, 6G can provide robust and efficient mobility suitable for diverse services.

## 2.1	6G Mobility Design Considerations

Connected mode mobility for 6G should be designed from the start as a lean, single design that minimizes signalling while ensuring robustness.

The following drawbacks from NR should be considered:

- Fragmentation - Mobility mechanisms such as L3 HO, CHO, LTM, and C-LTM were developed in parallel. Their coexistence created overlapping procedures, duplicated signalling, and heavy configuration overhead.

- Latency and interruption - In NR, handovers can still cause short service interruptions. For future services such as XR or URLLC, further reductions in latency and interruption time will be important to maintain user experience.

- Measurement duplication - L1 and L3 measurements are configured separately, resulting in duplicated reference signals and higher UE measurement burden.

- Complexity - Features such as CHO and DAPS introduced additional signalling to maintain backward compatibility. A simplified baseline in 6G can help reduce signalling load.

- Limited Deployment - Some advanced mobility features in NR have seen limited deployment so far, partly due to signalling overhead and the need for cross-vendor interoperability.

6G offers an opportunity to streamline design for wider adoption and the following design targets should be considered to achieve that:

- Simplification: Reduces signalling overhead and eases both network and device implementation.

- Robustness: A single design supporting L3/LTM improves mobility success rates and reduces handover failures.

- Low latency: RACH-less handover and early TA acquisition enable seamless mobility with minimal service interruption.

- Modular configuration: Stored and reusable RRC profiles or configuration parts avoiding full reconfiguration at every handover, reducing delay and signalling.

- Service awareness: Mobility procedures can adapt to traffic type (e.g., XR, IoT) or AI-predicted conditions, ensuring optimized performance.

- Scalability: A single framework can support a broad range of devices, from ultra-low-power IoT to high-throughput XR and extend naturally to NTN scenarios.

Observation 1: NR mobility suffers from fragmentation and complexity, and it is service agnostic. 6G should aim for a single, robust, low-latency and scalable framework adaptable to diverse services and devices.

## 2.2	Single design for the Mobility Framework

NR introduced multiple mobility procedures such as L3 handover, Conditional Handover (CHO), Lower layer Triggered Mobility (LTM) and conditional LTM (C-LTM). Each procedure came with its own signalling, configuration, and backward compatibility requirements.

The coexistence of multiple procedures in NR (e.g., L3 HO, CHO, LTM) led to overlapping signalling and increased complexity. A single procedure in 6G can lower signalling overhead and improve interoperability.

A single design for the mobility framework that supports L3-based mobility, LTM and CHO consolidates these approaches into a common baseline, allowing a single configuration per candidate cell valid across mobility types. This reduces redundancy, simplifies specifications, and improves robustness by ensuring consistent UE behaviour.

Proposal 1: Consolidate L3 HO and LTM into a single mobility framework that supports both network-triggered and conditional mobility based on L3/L1 measurements and L3/L2/L1 signalling.

## 2.3	Consolidated Measurement Framework

In NR, L1 and L3 measurements are configured separately (maybe even configured by different network entities, e.g., in the case of split architectures such as CU-DU), which can result in duplicated reference signals and additional UE measurement burden/complexity (e.g., there are lots of similarity between the handling of L1-RSRP measurements for L1 reporting for beam management purposes and L1-RSRP reporting for L3 reporting for mobility).

The fragmentation of measurement frameworks also limits efficiency and prevents easy integration of mobility based on AI/ML (e.g., prediction of measurements and/or measurement events), as data from different layers cannot be harmonized.

A single design framework in 6G would reduce redundancy and simplify measurement configuration. Merging L1 and L3 measurement frameworks into one single structure reduces redundancy, simplifies configuration, and ensures consistent UE behaviour. Shared RS definitions minimize overhead, while enabling AI-assisted prediction allows for proactive handovers and improved quality of experience.

Proposal 2: Consolidate L1 and L3 measurement frameworks into a single structure and use common RS definitions for all RRM measurements.

## 2.4	Separate Triggering conditions, same Signalling Framework

Currently, there are separate signalling for the triggering/reporting conditions for the different mobility mechanisms. For example, for there are separate signalling for the events conditions (E.g., thresholds, TTT, offsets, etc.,) for the case of L3 HO events (e.g., EventAx IEs under EventTriggeredConfig), L3 CHO events (e.g., condEventAx IEs under CondTriggerConfig of ReportConfigNR) and LTM events (e.g., eventLTMx IEs under EventTriggeredConfig of LTM-CSI-ReportConfig).

This leads to a lot of duplication in terms of both configuration signalling as well as UE procedures, as there is a lot of commonalities in the content of these configurations. Such an approach creates unnecessary complexity and makes it harder for UEs to manage multiple mobility procedures efficiently.

A better approach is to unify the configuration of the triggering/reporting conditions for the different mobility mechanisms, while preserving the possibility to configure the different parameters differently for the different mobility mechanisms (e.g., signal level thresholds, time to trigger, etc.,). This way, UEs can evaluate triggers consistently, leading to a simpler specification, reduced signalling overhead and smoother mobility execution.

Proposal 3: Unify the signalling and procedural framework for L3 HO and LTM (both for network triggered and conditionally triggered), while enabling the possibility to configure the triggering and reporting conditions/parameters separately for each.

## 2.5	RACH-less and Early TA Acquisition

One of the high-level agreements regarding the requirements of mobility in 6G that was agreed in RAN2-131bis was the minimization of interruption time and ensuring of service continuity [2].

For latency-sensitive applications (e.g. eXR, URLLC), even tens of milliseconds of interruption can cause service degradation. RACH-less mobility with early TA acquisition is needed to reduce interruption time and improve user experience. This ensures low-latency, seamless handover performance essential for future immersive and mission-critical services.

Traditional handovers require a Random Access (RA) procedure, which introduces delay and signalling overhead. Earlier releases of NR enabled RACH-less HO for very limited scenarios (E.g., intra-gNB or cases where the TA towards the source and target were equal or TA to the target was zero). Inter-gNB RACH-less HO was introduced in rel-19 LTM, but it is still limited to cases where there is a tight co-ordination between the gNBs and ideal backhaul is assumed. RACH-less mobility is thus not yet generalized to non-ideal backhaul, inter-PLMN, inter-RAT, or multi-TRP scenarios. For 6G, a broader framework design should be considered that will enable RACH-less transitions across all relevant deployment types (e.g., as compared to the way it was done in NR where the supported scenarios were expanded with each specification release).

Proposal 4: 6G should enable RACH-less HO (e.g., via early DL/UL sync, TA prediction, etc.,) for all relevant deployment types (e.g., not limited to intra-gNB cases or inter-gNB cases with a tight coordination).

## 2.6	Inter-RAT mobility

To ensure seamless transitions between 5G and 6G radio access technologies, maintain service continuity and promote attraction of investment on deployment during the early stages of 6G, it is essential that RAN2 support inter-RAT mobility between NR and 6GR. From performance perspective, it is desirable to also keep the interruption time of inter-RAT mobility low.

Most of the mobility features of NR (Such as CHO, LTM, early UL/DL sync, etc.,) were not available in the first release of NR (rel-15). Thus, the inter-RAT mobility between NR and 4G, which was also designed in rel-15, didn’t consider these features (also, even in later releases, enhancements to LTE mobility were mostly limited to CHO only).

6G is anticipated to support most of the mobility enhancements of NR (CHO, LTM, early sync, etc.,), as discussed above. Thus, RAN2 could study the possibility of supporting the inter-RAT extension of these mobility mechanisms (e.g., inter-RAT LTM between NR and 6G), while considering the implications of that (E.g., specification complexity such as dependency between the two generations, ensuring that such tight interworking should not restrict 6G’s design flexibility/scalability/evolution, etc.,)

Therefore, the inter-RAT mobility between 5G and 6G should carefully balance performance (e.g., interruption time and robustness), architectural independence (e.g., shared or non-shared core network options), and system complexity (e.g., UE context transition procedures between RATs).

Proposal 5: Inter-RAT mobility between 5G and 6G should be supported and RAN2 to study the mobility features to be supported for inter-RAT mobility (e.g., LTM, Conditional HO, RACH less HO).

## 2.7	Simplified and Modular Reconfiguration During HO

In NR, full RRC reconfiguration may be performed, even when only a subset of parameters changes. This leads to large signalling messages, increased latency, and higher UE processing burden. While delta signalling and reference configurations are supported in NR, in most cases it may not be practical (e.g., due to the complex ASN.1 structure that have several layers of nesting, parameters that are usually updated during handover being not grouped together, etc.,).

As more features are introduced in 6G, reconfiguration size will only grow if the NR RRC model is retained, further burdening devices and networks. As such a modularized RRC configuration/structure may be desirable, which we have discussed in detail in [3] from several angles.

From the point of view of mobility, a better way to organize RRC configurations may be considered where the parts/modules that are likely to remain the same after mobility are grouped together. That way, only the parts/modules that need to be changed during mobility can be signalled to the UE. This will reduce the size of the reconfiguration message, which further reduces the RRC processing delay and handover interruption time, as well as overall system signalling overhead. This also simplifies specification maintenance and makes the framework more adaptable for future features.

It should also be recognized that as a UE changes from one profile or operation mode to another (e.g., due to service type, device mode, or capability update), the set of parameters that need to change based on delta configuration from that profile can differ significantly.

Proposal 6: To facilitate practical delta RRC reconfiguration at HOs, study RRC configuration modularization that groups together parameters/IEs based on their likelihood to change or persist after mobility.

## 2.8	Support for Energy Saving

In RAN2-131bis, one of the agreed high-level requirements for 6G mobility is considerations for both UE and network energy efficiency [2]. To achieve this, mobility procedures will need to operate under conditions where both UEs and cells may be in low-power or sleep modes. This introduces new challenges that were not present in NR, at least until R19, where most reference signals and control channels are always-on.

With network energy saving (e.g., cell/beam DTX, on-demand SSB/SIB), mobility procedures cannot assume that all reference signals are continuously available. Mobility should account for the possibility that certain RS resources (SSB, CSI-RS) are intermittently available. Measurements may need validity ranges or fallback mechanisms. Handover and LTM/CHO should be possible to execute reliably even when candidate cells are in energy-saving state, e.g. by using wake-up signalling, shared configurations, or predictive measurements.

Measurements may need to adapt to cases where SSBs or CSI-RS are transmitted with longer periodicities or only activated on-demand. Procedures for activating additional RS resources when mobility risk is detected (e.g., predicted RLF, UE entering cell edge) may therefore be needed. Without adapting mobility, there is a risk of delayed or failed handover due to missing measurement opportunities, especially for latency-sensitive services. Different services (e.g. IoT vs XR) may tolerate different trade-offs between energy efficiency and robustness; mobility procedures should adapt accordingly.

NR’s mobility framework also does not consider the UE’s data activity or energy savings state (e.g., low power radio operation) in mobility procedures. For example, a UE will perform RRM measurements the same way regardless of the data activity level of the UE and the QoS profiles of the active bearers. However, the mobility needs of a UE that has a very high data activity or active bearers that are of very strict QoS requirements may be different from that of a UE with low data activity or have only best effort bearers. For example, more frequent measurements may be needed in the former case to ensure service is not interrupted or latency requirements are made, while measurements can be relaxed in the latter case without impacting the QoS/QoE.

Proposal 7: 6G mobility framework should consider measurements and procedures that are adapted to 1) fully or partially sleeping serving/candidate cells (i.e., with intermittently transmitted reference signals or system information) and 2) UE’s data activity and power saving state.

# Conclusion

This contribution outlines initial views on mobility in 6G, with a focus on Connected mode operation and makes the following observation and proposals:

Observation 1: NR mobility suffers from fragmentation and complexity, and it is service agnostic. 6G should aim for a single, robust, low-latency and scalable framework adaptable to diverse services and devices.

Proposal 1: Consolidate L3 HO and LTM into a single mobility framework that supports both network-triggered and conditional mobility based on L3/L1 measurements and L3/L2/L1 signalling.

Proposal 2: Consolidate L1 and L3 measurement frameworks into a single structure and use common RS definitions for all RRM measurements.

Proposal 3: Unify the signalling and procedural framework for L3 HO and LTM (both for network triggered and conditionally triggered), while enabling the possibility to configure the triggering and reporting conditions/parameters separately for each.

Proposal 4: 6G should enable RACH-less HO (e.g., via early DL/UL sync, TA prediction, etc.,) for all relevant deployment types (e.g., not limited to intra-gNB cases or inter-gNB cases with a tight coordination).

Proposal 5: Inter-RAT mobility between 5G and 6G should be supported and RAN2 to study the mobility features to be supported for inter-RAT mobility (e.g., LTM, Conditional HO, RACH less HO).

Proposal 6: To facilitate practical delta RRC reconfiguration at HOs, study RRC configuration modularization that groups together parameters/IEs based on their likelihood to change or persist after mobility.

Proposal 7: 6G mobility framework should consider measurements and procedures that are adapted to 1) fully or partially sleeping serving/candidate cells (i.e., with intermittently transmitted reference signals or system information) and 2) UE’s data activity and power saving state.

# References

- RP-251881, “New SID: Study on 6G Radio”, NTT DOCOMO (Moderator), Prague, June 2025.

- Chair Notes, RAN2-131bis, Prague, October 2025

- R2-2508386, RRC Structure and Reconfiguration for 6GR, Interdigital Inc., RAN2-132, Dallas, November 2025