3GPP TSG-RAN WG2 Meeting #132 	R2-2508783

Dallas, USA, Nov. 17th - 21st, 2025

Agenda Item:	10.3.2.1

Source:	Panasonic

Title:	UE context and location area management in non-connected RRC state

Document for:	Discussion

# Introduction

RAN plenary meeting#108 has approved the 6G radio (6GR) study item (RP-251881). The study represents an opportunity to revisit the foundations of radio system design, free from the constraints of backward compatibility, and thus focusing on a transformative leap in capability, efficiency, and simplicity.

In the previous meeting, RRC state modelling was discussed in Control Plane agenda. During the discussion, there were mainly 3 directions for 6GR RRC state design.

- The first one is to keep legacy 3 states and try to simplify the INACTIVE operation such as paging and UE tracking (e.g. R2-2507072, R2-2507146).

- The second one is to unify INACTIVE and IDLE as much as possible to simplify the state design (e.g. R2-2507172, R2-2507270).

- The third direction is to remove INACTIVE, and instead introduce a no-data low-power connected sub-state for CONNECTED (R2-2506857).

In this paper, instead of discussing how to model RRC states (which can be next step discussion), we focus on addressing issue of existing INACTIVE and IDLE states in 5G NR, and propose a unified UE context and location area handling for 6G non-connected state.

# Discussion

## UE tracking area management

In 5G NR, 3 RRC states have been defined: RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED. The following table 1 summarizes the main differences.

Table 1. RRC States in 5G NR

| RRC_IDLE | RRC_INACTIVE | RRC_CONNECTED |
| --- | --- | --- |
| - No RRC connection is established - UE sleeps most of time - UE can wake up to, e.g. monitor paging, perform cell reselction, initiate RRC connection | - Intermediate state between IDLE and CONNECTED - RRC connection has been established, but inactive (suspended) - UE is allowed to sleep in a similar way as in IDLE - From CN perspective, UE is still connected (the same as in CONNECTED) - UE RAN context is kept at both UE and gNB for facilitating the transition from INACTIVE to CONNECTED: | - RRC connection is established - Active data transmission to/from gNB |

One major issue identified by some companies is the duplicated UE tracking and paging management (see. e.g. tdocs mentioned in section 1). More specifically, for both RRC_IDLE and RRC_INACTIVE, network needs to track UE in order to decide the area to send paging message. But two different UE tracking mechanisms are used. For IDLE state, UE tracking is managed by CN (e.g. AMF) via Registration Area. For INACTIVE, UE tracking is managed by RAN (e.g. gNB) via RAN Notification Area (RNA). This is because CN is typically not aware the UE transitions between RRC_INACTIVE and RRC_CONNECTED. If RAN decides to transition UE into INACTIVE, it is RAN’s responsibility to track the UE. As a result, both network and UE needs to maintain two levels of tracking in order to cope with transitioning between IDLE and INACTIVE states, increasing complexity at both UE and network.

In view of the above, it is worth questioning whether 6G can simply the RRC state design by removing duplicated UE tracking mechanism.

For example, it can be considered to let CN to handle UE tracking area for non-connected states (e.g. IDLE and INACTIVE). In this case, paging would be originated from CN based on the location area managed by CN. This can be applied to both IDLE and INACTIVE.

Proposal 1. RAN2 study one single location tracking management for non-connected mode (e.g. both IDLE and INACTIVE).

## UE context management

One important feature of legacy INACTIVE state is to keep UE context in both UE and RAN for fast resume. This feature can be carried on in 6G RRC state design with some enhancements. For example, to better support the unified location tracking and paging mechanism discussed in section 2.1, it can be considered to store the UE RAN context at CN when the network release UE from connected mode into non-connected mode (such as IDLE and INACTIVE). The last serving base station can also decide to keep a copy of UE RAN context to avoid request from CN when RRC resume is performed.

In such case, for DL case when CN pages UE, CN can proactively send the UE context to the base station together with paging request to avoid delay caused by additional UE context request procedure. For UL, the receiving base station can first check whether there is a cashed UE context. If so, the cashed UE context can be used rather than requested from CN.

Proposal 2. RAN2 study managing the UE RAN context at CN for non-connected mode, including

- Uploading UE RAN context to CN when transitioning from connected to non-connected mode

- Delivering UE RAN context to base station for paging

# Conclusion

In this paper, we have discussed location area management and UE context management for 6GR non-connected mode such as IDLE and INACTIVE. The proposals are as follows.

Proposal 1. RAN2 study one single location tracking management for non-connected mode (e.g. both IDLE and INACTIVE).

Proposal 2. RAN2 study managing the UE RAN context at CN for non-connected mode, including

- Uploading UE RAN context to CN when transitioning from connected to non-connected mode

- Delivering UE RAN context to base station for paging