3GPP TSG-RAN-WG2 Meeting #132	                                       R2-2508817

Dallas, USA, November 17th – 21st, 2025

Agenda Item:	9.3.2

Source: 	Sharp

Title:  	Discussion on RRM Measurement Prediction Functionality Management

Document for: 	Discussion

# Introduction

In RAN#108 the new WID for AI/ML Mobility [1] was agreed. During the study item, the following agreements were made in RAN2#130 [2]:

Agreements - Use the beam management agreements as the baseline, including the following aspects (details can be updated after further progress is made in AI/ML PHY BM use case) - Consider Option A and Option B like scheme (if/when AI/ML PHY makes further progress) - Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability. - Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly. - Support the explicit reporting of applicability/inapplicability in the initial report and subsequent reporting when the applicability changes. - Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability (FFS pending AI/ML PHY progress). - No prohibit timer is introduced. - Associated ID should be optionally configurable for training and inference (i.e. it may not mandatorily required for all training/inference configurations). FFS for WI phase further details (what absence means, which use case, whether it is per cell or multiple cells/frequency, terminology).

- The configuration options for the UE applicability report, the triggering message from the NW, and the response message have been reviewed in the agreements. Also, UAI can be used to update the applicability. Concerning the use scenarios and the objects of measurement prediction on AIML mobility may be mercurial, RAN2 should further consider the flexible usage of the messages in the two options, as well as the applicability report message from the UE side, regardless of the initial and the updated applicability report.

Agreements - The following potential “inference parameters” can be considered for inference/measurement configuration, which provides the necessary information for UE to examine the applicability, it may contain - PW length (for temporal case A) - Skipping pattern (e.g. SSB config that indicates SSBs transmitted) (optional). MRRT (for temporal case B ) not discussed in study item, but if RAN4 says otherwise can be considered in WI - Measured and predicted beam pattern (optional). MRRS (for spatial domain prediction) not discussed in study item, but if RAN4 says otherwise can be considered in WI - Measured and predicted frequency (For inter-freq domain prediction) - Model related choices (i.e., cluster-based vs. cell-based, L1 filtering, RRM sub-use cases, OW length, direct vs. indirect) can be up to UE implementation unless a requirement is identified to specify them.

For the applicability report “inference parameters”, in the functionality-based LCM, the UE should jointly examine the inference performance for the applicability. Hence, RAN2 should further consider some performance-related parameters or criteria for applicability reporting on the self-perspective of the UE.

The activation of the semi-persistent and aperiodic report of the prediction measurement from the lower-layer (MAC CE and DCI) is discussed based on the RAN2 #129 agreements [3] as follows.

- Agreements applicability reporting and management Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed. FFS if we report explicit cause If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5). The provided periodic CSI configuration should be consistent with reported UE capabilities FFS option B Semi-persistent and aperiodic CSI reporting of applicable functionality is activated following legacy CSI framework: - Semi-persistent reporting, activated by MAC CE/DCI - Aperiodic CSI reporting, activated by DCI

Following up on the lower-layer triggered mechanism, we suggest RAN2 to further study the lower-layer triggered applicability report mechanism with the merit of the flexible temporal reaction.

We will discuss the issues mentioned previously in detail in the next section.

# Discussion

## Applicable Functionality Reporting

For AIML mobility, the RRM measurements span multiple levels, which include L1-beam level, L3-beam level, and L3-cell level. Therein, the levels also implicitly encompass intra-frequency, intra-cell, inter-frequency, inter-cell, and (across-frequency, across-cell) domains [4]. In [4], the “across-frequency prediction” refers to the case when the frequencies of the measured and the predicted cells of the AI/ML for mobility feature are different, and “across-cell prediction” refers to the case when a group of cells is measured and the RRM of, or an event involving, one or more of these cells is being predicted. The detailed definition of the “types” of the relationship between the measurement and the prediction, e.g., the intra- or the across- frequency and cell could be further defined, and the concept of a greater granularity classification domain (e.g., with inter (cross)-frequency, and inter (cross)-cell) should be considered. As such, the framework for applicability reporting must evolve with greater granularity to accurately match the explicit and implicit level and the “type” of target RRM measurement prediction. Hence, a more specific inference applicability reporting, e.g., cell-specific and/or frequency-specific, should be further studied in RAN2.

Observation 1: The RRM measurements span multiple levels, which include L1-beam, L3-beam, and L3-cell. The levels also implicitly encompass intra-frequency, intra-cell, inter (cross)-frequency, and inter (cross)-cell domains.

Proposal 1: RAN2 to further consider a greater granularity framework for applicability reporting, e.g., cell-specific and/or frequency-specific relationship between the measurement and prediction, to accurately match the explicit and implicit levels and domains of the target RRM measurement prediction.

RAN2 [2] has agreed that both the RAN1-defined Option A and Option B (triggered by CSI-ReportConfig and OtherConfig [6] while taking the beam management as an instance) are considered for applicability reporting, acknowledging the operational distinctions as to how the UE determines the applicability for AIML inference. Option A and Option B are identified as follows:

- Option A: Requires the configuration of one or multiple sets of CSI-ReportConfig to support the full RRM measurement inference configuration.

- Option B: Requires the configuration of OtherConfig to provide inference-related parameter configurations.

The advantage of option B is to predict the applicability by one set or multiple sets of inference-related parameters from the OtherConfig rather than learn the feasibility based on attempting the applicability given from the CSI-ReportConfig. Regarding the flexibility of the reaction time, option B should be an essential option for applicability reporting for in-time reaction.

Observation 2: The advantage of option B is to determine the applicability by the sets of inference-related parameters rather than learn the feasibility based on attempting the applicability given from the CSI-ReportConfig.

Proposal 2: Regarding the flexibility of the reaction time, option B should be an essential option for mobility regarding the shorter applicability determining time compared to Option A.

For the applicability report, regardless of option A and option B, RAN2 should study a more comprehensive RRC message to trigger the applicability report. In legacy RRC procedures, each measurement configuration (e.g., for event-triggered or periodic reports) is typically associated with a measurement identity (i.e., measId via RRCReconfiguration) for tracking, management, and reporting [7]. Hence, RAN2 should consider measurement identity as a basis option to trigger the applicability report. However, the current use of measurement identity for legacy CSI/SSB measurements does not capture the full information needed for AIML inference applicability and functionality-based LCM configuration (for model-based LCM, could be FFS). Studying an AIML-specific inference configuration is worthwhile and would help with clear mapping between configuration, applicability report management, and performance monitoring. Furthermore, uniformly configuring the applicability report via measurement identity or the AIML-specific configuration message may reduce the signalling complexity.

In addition, RAN2 [2] has agreed that the UE shall maintain all the full inference configuration(s), no matter whether the full inference configuration is applicable or inapplicable, until the network releases it explicitly. However, the LCM of different functionality-based models may not be the same, and the applicable time for the RRM measurement prediction configurations may also not align with each other for different purposes (e.g., objective and event). To reduce the signalling overhead for inference configuration releasing, it is essential for RAN2 to further study the determination of the inference configuration retention that should be applied to individual LCM and the RRM measurement prediction configurations at the UE side, ensuring the full inference configuration is maintained at the UE and is validated.

In addition, even though RAN2 has agreed that model-related choices (i.e., cluster-based vs. cell-based, L1 filtering, RRM sub-use cases, OW length, direct vs. indirect) can be up to UE implementation, there are no performance-related parameters for the UE to choose the models, which may lead to performance misalignment. Thus, RAN2 should further consider jointly introducing some baselines regarding the performance results while triggering the applicability report to avoid any misalignment of the RRM measurement prediction performance.

Observation 3: In legacy RRC procedures, each measurement configuration (e.g., for event-triggered or periodic reports) is typically associated with a measurement identity (i.e., measId via RRCReconfiguration) for tracking, management, and reporting. However, the current use of measurement identity for legacy CSI/SSB measurements does not capture the full scope needed for AIML inference applicability and functionality-based LCM configuration (for model-based LCM, could be FFS).

Proposal 3: RAN2 to study the following options for applicability report triggering

- Option 1: The existing measurement configuration.

- Option 2: AIML-mobility specific configuration.

## Applicability Reporting Triggering

Figure 1: A general procedure for the applicability report and inference configurations

RAN2 #129 [3] has agreed that the CSI measurement prediction report can be triggered from lower layers (e.g., MAC CE and DCI), as in semi-persistent (SP) and aperiodic, which follow the legacy CSI framework. Regarding the state of the environment and the UE may change dynamically and unexpectedly, it may not react in time if the applicability report is always triggered from the RRC layer. Currently, RAN2 has agreed that the UE can proactively use UAI to update the applicability after inference if some changes happen (e.g., after step 4 or step 6 in Figure 1). However, regarding the case that the gNB is trying to maintain the inference configuration after UE inference, while acknowledging that something has changed, the current scheme for the gNB to obtain the applicability of UE is to resend the same RRCReconfiguration (step 3 in Figure 1) again via the RRC layer. To reduce signaling overhead and the reaction time, RAN2 should further consider a mechanism for the gNB to fetch the applicability report from the low layers (e.g., MAC CE and DCI). In other words, RAN2 should consider a low-layer applicability status determination mechanism via UAI from the UE for the AI/ML applicability report.

Observation 4: While keeping the same inference configuration, there is no mechanism for the gNB to fetch the applicability report from the UE without sending the same RRCReconfiguration (step 3 in Figure 1), which may lead to signalling redundancy.

Proposal 4: RAN2 to consider the low-layer applicability status determination mechanism via UAI for the AI/ML applicability report.

# Conclusions

Based on the discussion above, we have the following observations:

Observation 1: The RRM measurements span multiple levels, which include L1-beam, L3-beam, and L3-cell. The levels also implicitly encompass intra-frequency, intra-cell, inter (cross)-frequency, and inter (cross)-cell domains.

Observation 2: The advantage of option B is to determine the applicability by the sets of inference-related parameters rather than learn the feasibility based on attempting the applicability given from the CSI-ReportConfig.

Observation 3: In legacy RRC procedures, each measurement configuration (e.g., for event-triggered or periodic reports) is typically associated with a measurement identity (i.e., measId via RRCReconfiguration) for tracking, management, and reporting. However, the current use of measurement identity for legacy CSI/SSB measurements does not capture the full scope needed for AIML inference applicability and functionality-based LCM configuration (for model-based LCM, could be FFS).

Observation 4: While keeping the same inference configuration, there is no mechanism for the gNB to fetch the applicability report from the UE without sending the same RRCReconfiguration (step 3 in Figure 1), which may lead to signalling redundancy.

We also make the following proposals:

Proposal 1: RAN2 to further consider a greater granularity framework for applicability reporting, e.g., cell-specific and/or frequency-specific relationship between the measurement and prediction, to accurately match the explicit and implicit levels and domains of the target RRM measurement prediction.

Proposal 2: Regarding the flexibility of the reaction time, option B should be an essential option for mobility regarding the shorter applicability determining time compared to Option A.

Proposal 3: RAN2 to study the following options for applicability report triggering

- Option 1: The existing measurement configuration.

- Option 2: AIML-mobility specific configuration.

Proposal 4: RAN2 to consider the low-layer applicability status determination mechanism via UAI for the AI/ML applicability report.

# References

[1] RP-251864

[2] Chair notes, RAN2 130 3GPP TSG-RAN WG2 Meeting #130, St.Julians, Malta, 2025

[3] Chair notes, RAN2 129 3GPP TSG-RAN WG2 Meeting #129, Athens, Greece, 2025

[4] R2-2503514, AI/ML for mobility Configuration and Functionality management, Qualcomm Incorporated