3GPP TSG RAN WG2 Meeting #132	R2-2508837

Dallas, USA, Nov. 17th-21st, 2025

Agenda Item:	8.8.2

Source:	CMCC

Title:	Remaining issues on broadcast service for NR NTN

Document for:	Discussion, Discussion

Introduction

In last RAN2 meeting, there were some discussions on support of broadcast service and some agreements were achieved as follows [1]:

Agreements:

For Stage2:

1.	capture the following in the stage 2:

- The deprioritization of the frequency of a service the UE is interested in when the UE is outside ISA

For idle mode:

1.	Update NOTE0j as: ‘NOTE 0j:	The ISA(s) can be ISA(s) provided in SIBxx and/or target service area in USD. It is up to UE’s implementation to decide whether it is inside ISA(s) or not.’

Agreements:

8.	[V206] Change the need code for mbs-SessionAreaList-r19 from Need R to Need S. 	Add in the FD of mbs-SessionAreaList-r19 that UE considers the associated service can be received within the entire cell area if the list is absent

In this contribution, we would like to provide some further considerations on the remaining issue about neighbouring cell information for service continuity which is postponed to this meeting for broadcast service in NTN system.

Discussion

In last meeting, remaining issue about neighbouring cell information for service continuity is not discussed due to limited online TUs, which is postponed again to this meeting.

R2-2507634 Service continuity in MBS NTN Ericsson discussion Rel-19 NR_NTN_Ph3-Core R2-2505822 Proposal 1 Broadcast future neighbour cells providing the same MBS service(s) as the serving cell. Proposal 2 A UE is informed of an area over with uniform MTCH configuration parameters for a certain MBS service regardless the serving cell or satellite. Can be discussed in the next meeting

For proposal 1, per the current specification, a list of neighbour cells providing the same broadcast MBS service(s) as provided in the current cell can already be provided in the MBSBroadcastConfiguration message for TN MBS. Therefore, we could just reuse the existing mbs-NeighbourCellList-r17 in MBSBroadcastConfiguration to provide the future neighbour cells providing the same MBS service(s) as the serving cell for NTN MBS.

Proposal 1: Reuse the existing mbs-NeighbourCellList-r17 in MBSBroadcastConfiguration to provide the future neighbour cells providing the same MBS service(s) as the serving cell for NTN MBS.

For Proposal 2, a new indication (e.g. mtch-StaticConfigSwitch in [2]) is introduced to indicated the UE whether the cell replacing the serving cell provides an MBS service with identical MTCH configuration parameters to minimize service interruptions and lower UE power consumption due to frequent re-acquisition of information upon each cell change. From our perspective, such optimization is based on the per geographical area MTCH settings mentioned in [2]. However, if the MTCH settings are per geographical area, then the new indication is not needed. And if the MTCH settings varies in different areas/cells, the significance of introducing the new indication information is not obvious.

Observation 1: If the MTCH settings are per geographical area, then the new indication is not needed. And if the MTCH settings varies in different areas/cells, the significance of introducing the new indication information is not obvious.

Proposal 2: The optimization that inform UE whether the cell replacing the serving cell provides an MBS service with identical MTCH configuration parameters is not needed.

Conclusion

Based on the discussions mentioned above, in this contribution we provide some discussions on MBS broadcast enhancements for NTN and have the following observations and proposals:

Proposal 1: Reuse the existing mbs-NeighbourCellList-r17 in MBSBroadcastConfiguration to provide the future neighbour cells providing the same MBS service(s) as the serving cell for NTN MBS.

Observation 1: If the MTCH settings are per geographical area, then the new indication is not needed. And if the MTCH settings varies in different areas/cells, the significance of introducing the new indication information is not obvious.

Proposal 2: The optimization that inform UE whether the cell replacing the serving cell provides an MBS service with identical MTCH configuration parameters is not needed.

Reference

- R2-131bis_NES_NR-NTN_IoT-NTN_(Sergio)_ EOM

- R2-2507634	Service continuity in MBS NTN	Ericsson	discussion	Rel-19	NR_NTN_Ph3-Core	R2-2505822