3GPP TSG-RAN WG2 Meeting #132	R2-2508841

Dallas, US, 17th – 21st November, 2025

Agenda Item:	10.2.3

Source:	CMCC

Title:	Guidelines and Views on 6G Design approaches and New services

WID/SID:			FS_6G_Radio

Document for:	Discussion

# 1 Introduction

In RAN2#131bis, there was discussion on 6GR general aspects, such as Design principles, services, architecture and requirements for 6GR, NTN and 6GR, UE capabilities.

This paper aims to analyze what new services to be supported in the 6G with providing RAN2 design guidelines and views for L2 and L3 protocols on scalability, extensibility and future proofness. And some considerations on Device types will be discussed as well for 6GR.

# 2 Discussion

2.1 New Services

As global deployment of 5G networks enters a mature phase, the limitations of existing service frameworks have gradually emerged in addressing future societal and technological demands. Current 5G services, while enabling high-speed connectivity and low-latency interactions, struggle to fully support emerging scenarios such as immersive extended reality (XR) experiences, intelligent sensing, and satellite-terrestrial integration. Therefore, 6G is expected to become a foundational infrastructure for the verticals, smart societies, and human-centric intelligent ecosystems. In our view, Integrated Sensing and Communication (ISAC), AI/ML-based services and immersive communication will be the key services in 6GR to unlock the new capabilities to support the next era of digital transformation.

2.1.1 6GR ISAC

ISAC has been identified as one of the six usage scenarios for IMT-2030, providing sensing services for low-altitude economy, traffic surveillance, intrusion detection and micro-deformation detection while also assisting communication services.  TR 22.837[2] identifies a very wide range of ISAC use cases, and requirements for such are defined in TS 22.137 [3]. The different use cases of ISAC require the different performance requirements, for example, the performance requirements of sensing results for pedestrian/animal intrusion detection are given in Table 1.

Table 1: Performance requirements of sensing results for pedestrian/animal intrusion detection

| Scenario | Sensing service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency[ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |
| Pedestrian/animal intrusion detection on a highway | Outdoor (Highway) | 95 | ≤1 | N/A | N/A | N/A | N/A | N/A | ≤5000 | ≤ 0.1 | ≤5 | ≤5 |
| NOTE: The terms in Table 5.2.6-1 are found in Section 3.1. |  |  |  |  |  |  |  |  |  |  |  |  |

2.1.2 6GR AI/ML

In 5G-A, 3GPP RAN has investigated various AI for RAN use cases that are aimed to optimize network performance through AI/ML function, for example, AI/ML based CSI feedback enhancement, beam management and positioning enhancement and AI/ML for mobility. For 6G, the potential new use cases of AI for 6G RAN could be identified and investigated, e.g. Joint source/channel coding (JSCC), DMRS design or AI receiver, AI/ML based mobility.

Furthermore, RAN for AI is expected to be investigate and support in 6GR, e.g. RAN provides computing resources, RAN related information or AI data for AI services/applications. AI agent, XR rendering and token communication could be considered as the potential RAN for AI use cases, and the requirements of these use cases are summarized in Table 2.

Table 2: Requirements for RAN for AI use cases

| Use Cases | Uplink KPI |  | Downlink KPI |  |  |
| --- | --- | --- | --- | --- | --- |
|  | E2E Latency | Date Rate | E2E Latency | Date Rate | Load |
| XR rendering | 100 ms | 1.5 Mbit/s | 100 ms | 150 Mbit/s | 1.5 MByte/‌frame |
| Embodied AI/ AI agent | 10 ms | 4.7 Mbit/s | 10 ms | 320 Mbit/s | 40 kByte |

2.1.3 6GR Token Communication

As AI applications increase in traffic across the network, supporting mobile AI applications (such as large language models and othe generative AI) has become a new topic during previous RAN2 discussions. Token Communication, a novel communication paradigm designed for AI/ML applications like generative AI, may be a potential research direction for both 6G general design and user plane design.

Currently, mobile AI applications transmit data (including text, images, and videos) through the RAN interface in the same transparent manner as traditional internet services. However, as the fundamental units for AI model processing and information generation, tokens exhibit unique communication patterns that present new opportunities for transmission optimization at the RAN layer. Prior to token communication, TR 22.870 investigated the KPIs for generative AI in 6G, which can serve as a foundation for RAN2 research.

Table 3: Required KPIs for GenAI applications [8]

| Traffic Type | Burst size | Max Allowed latency for a burst (note) | Service bit rate: user-experienced data rate | Delay (note) | Average packet size |
| --- | --- | --- | --- | --- | --- |
| Image based GenAI app | 400 KB | 50 ms | 64 Mbps | 20 ms | > 800B |
| Video based GenAI app | 20 MB | 400 ms | 400 Mbps | 20 ms | > 800B |
| chatbot | 0.5 KB | 20 ms | 200 Kbps | 30 ms | < 800B |
| Note: meanings of some KPIs: max allowed latency for a burst: max latency for sending out the whole packets within a burst delay: transmission latency of a packet |  |  |  |  |  |

2.1.4 6GR Immersive communication

Immersive Communication has been described by ITU-R as a usage scenario that extends the enhanced Mobile Broadband (eMBB) of IMT-2020 and covers use cases that provide a rich and interactive video (immersive) experience to users, including interactions with machine interfaces [3]. Immersive communication use cases will continue to drive requirements on mobile networks. Mixed Reality Gaming is taken as an example, and its performance requirements are shown in Table 4.

Table 4: Performance of Mixed Reality Gaming

| Profile | Characteristic parameter |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit rate down link | Bit rate up link | Application latency | Communication service availability | Overall user density | Activity factor | UE speed |
| Mixed Reality gaming | [50] Mbit/s (note 1) | [50-100] Mbit/s (note 3) | [20] ms (note 2) | Dense Urban [99]% Urban [99]% Rural [98]% | Dense Urban [25,000] /km2 Urban [1000-10,000]/km2 Rural [100]/km2 | 2% (note 4) | 5 km/h |
| NOTE 1: Remote or split rendering for gaming applications, such as rendering of 360-degree video tiles in the cloud and transmitting the images to device, NOTE 2: The local encoding and processing on the glasses is assumed NOTE 3: envisioned 1080P resolution 2 grayscale cameras 1920 x 1080 x 8 bit, a depth sensor 1920 x 1080 x 8 bit with 60 FPS means 1920 * 1080 * 24 * 60 bps = 3Gbps uncompressed. Depending on compression ratios that would lead to 50 Mbps to 100 Mbps NOTE 4: Activity factor 2% is based on 20% uptake of XR service and 10% activity factor for the XR application. |  |  |  |  |  |  |  |

Proposal 1: RAN2 should study the support of new services, e.g., ISAC, AI/ML-based services, token communication and immersive communication, taking NW & UE energy consumption and User experience (QoS/QoE) improvement into consideration.

2.2 Basic Principles for 6GR design

2.2.1 Simplification

As increased new features and functions are introduced, the fixed and coupled protocol stack causes high complexity, high cost and performance limitation encountered during the commercialization of 5G. The guidance of pragmatic and simplified 6G standard design is required. Hence, 5G design as starting point for 6G with simplification and to minimize the adoption of multiple options for the same functionality, focusing on practical user experience, including the following aspects:

- L2 sublayer consolidation for reorganization and reduction of redundant functionalities across protocol layer, as shown in figure 1 and will be illustrated in [5]

- RRC configuration and UE capability signaling reduction, will be illustrated in the following sections;

- Avoid excessive nesting of configurations (e.g. avoid BWPs as basic granularity in Physical Radio resource configuration)

Figure 1: Layer 2 Structure

Proposal 2: 5G design as starting point for 6G with pragmatic simplification:

- Consolidation L2 sublayers to reduce redundancies across protocol layer

- Simplify RRC configuration and UE capability signaling

- Avoid excessive nesting of configurations (e.g. avoid BWPs as basic granularity in Physical Radio resource configuration)

2.2.2 Extensible and flexible

It is crucial to consider both the efficiency and complexity of data collection in 6G. Therefore, a unified RAN data collection framework is essential, which should support diverse data collection from 6G new services, such as AI and sensing, which is illustrated in [5].

Figure 2: Unified RAN data collection framework

A unified signaling structure to support multiple mobility mechanism and measurement is demanded. When looking back to the mobility evolution, it evolves from supporting only single target cell, to support multi candidate cells, then further to support reference configuration, while LTM can be seen as a typical example, since it supports both reference configuration and multiple candidate cells. Hence, it is proposed to simplify and unify signaling design by avoiding current approach where L3, CHO and LTM are independent features, where LTM signaling structure can be the start point for unified signaling structure study. Meanwhile,unified measurement framework having the capabilities of the L3- and L1-measurement frameworks from 5G is required as well.

We propose to design a unified, flexible and concise multi-carrier framework in 6GR to enable efficient multi-carrier operations, which is illustrated in [6]:

Figure 3: A unified multi-carrier framework

Proposal 3: 5G design as starting point for 6G with extensible and flexible:

- Unified forward-compatible design principle for 6G access control, including cell barring mechanism, the definition of access categories and access identities;

- Unified RAN data collection framework for diversified data collected, e.g. AI and sensing;

- Unified mobility mechanism signalling structure with LTM signaling structure as the start point;

- Harmonized measurement framework having the capabilities of the L3- and L1-measurement;

- Unified multi-carrier framework in 6GR to enable efficient multi-carrier operations.

2.2.3 Excellent Performance

To meet a broad range of use cases including AI/ML-driven services, sensing service, Immersive communication, and additional requirements in 6G defined in TR 38.914 [4]. 6GR design should take NW & UE energy consumption and User experience improvement into consideration.

2.2.4 Modular

The 5G system has made significant strides in service diversity through technologies like network slicing. However, its UP and CP protocol fundamentally retain a relatively and rigid architecture. As we move towards the 6G era of interconnected intelligence, the service scenarios will become highly diversified, encompassing immersive communication, AI services, HRLLC, mMTC, ISAC, among others. These services impose vastly different, and often conflicting, extreme requirements on network throughput, latency, reliability, connection density, and energy efficiency. In TR 38.914 [4], from the perspective of scenarios and requirements, it also points out that in addition to providing diverse new 6G services, other factors need to be considered, such as reducing CAPEX/OPEX reduction.

At RAN2#131bis, RAN2 extensively discussed the motivation for a modular protocol design for 6G, and reached the following preliminary conclusions:

Agreements

  - ASN.1 is used for encoding of RRC signaling for 6G air interface (same as NR).

  - Study overall RRC structure, configuration improvements, improve readability of ASN.1 for RRC signalling

  - Study how to efficiently, reliably and unambiguously configure UEs while keeping signalling size small (e.g. improvements to delta signaling or no delta signaling).

  - RAN2 will study modular design of RRC for 6G.  Further study how to modularize RRC e.g. based on features, functions, verticals, etc.

During this meeting, the discussion on modular protocol shifted more toward RRC configuration framework. We have also provided a discussion on RRC modular in [7]. This section of the manuscript will focus more on the modular design guidance for both UP and CP. In general, 6G necessitates a fundamental paradigm shift from a fixed protocol stack to a lean and modular protocol stack design based on the principles of "service-driven, module-decoupling, profile-based configuration." to meet the requirement of pragmatic and simplified 6G.

Proposal 4: It is proposed to define functional modules in RRC configuration, organizing the protocol not in "layers" but in "functional modules", such as "Mobility Management-Module", "Access Control-Module", “L2 Data Handling Module” and “Physical radio resource configuration-Module” and so on.

The proposed modular protocol stack offers features and advantages compared to the conventional 5G protocol stack:

Table 5: Features and advantages of proposed modular protocol stack

| Features | 5G protocol stack | 6G lean and modular protocol stack | Benefits |
| --- | --- | --- | --- |
| Architecture | Fixed, Interlaced | Flexible, Modular, Assembliable | Flexibility, Scalability |
| Configuration | Complex, High Signaling Overhead, Static | Profile-based, Dynamic | Efficiency, Fast Configuration |
| CAPEX/OPEX | "Full-stack" Deployment, Resource Waste | On-demand Activation/Licensing, Function Tailoring | Significant CAPEX/OPEX Reduction |
| Performance | General-purpose Compromise, Hard to Optimize | Tailored to Service, Optimized Trafic Path | Ultimate Performance (Latency, Reliability, etc.)​ |
| Operation | Hard Spec Updating Towards New Features | Modular Insertion of New Features | Towards Flexible Networks Among Protocol Stack |

2.3 6G Device types

To efficiently accommodate the wide diversity of devices envisioned in 6G, it is necessary to define representative device type categories that can serve as a scalable and future-proof framework for capability definition and signalling. The proposed categorization can be based on fundamental radio capability dimensions such as power class, supported bandwidth, and antenna configuration. Three baseline types can be identified to represent the main device types:

Type I: Low-power devices, e.g., LPWA, RedCap, wearables, and simple 2Rx UEs, characterized by limited radio capabilities and minimal energy consumption.

Type II: Moderate-capability devices, similar to traditional eMBB UEs, providing a balance between throughput, coverage, and power efficiency.

Type III: High-capability devices, including FWA, CPE, and high-end XR terminals, featuring advanced radio performance, multiple antennas, and high data rates.

Observation 1: Defining baseline device types while allowing service-driven subtypes supports scalable capability management and reduces signalling overhead in L2 and L3 protocols.

Beyond these baseline categories, new subtypes are expected to emerge as 6G introduces AI-native functions, multi-modal or immersive services, and non-terrestrial, vehicular, or aerial deployments. These emerging service-driven features may impose distinct radio and protocol requirements that are not captured by conventional radio-only categorization. This indicates that device type definitions should evolve from static capability groupings toward a flexible, multi-dimensional classification, where each type or subtype can be associated with a baseline capability profile describing mandatory functions and default RRC behaviour.

Observation 2: Flexible, multi-dimensional categorization ensures forward compatibility, accommodating future AI-native, multi-modal, and non-terrestrial devices without revising the baseline specification.

By allowing UEs to report only delta capabilities when deviating from the baseline, the network can significantly reduce signalling overhead while simplifying configuration interpretation. This approach enables protocol procedures to reference capability templates instead of enumerating individual parameters, facilitating consistent RRC and L2/L3 behaviour across diverse device classes. Compared with the fixed UE category mechanism in LTE, which predefined uplink and downlink capability combinations per category, a flexible 6G device type framework can accommodate evolving device and service characteristics without revising the base specification.

Observation 3: Associating each device type with a baseline capability profile and delta reporting facilitates template-based configuration, simplifying RRC procedures and enabling consistent protocol interpretation across diverse device classes.

Overall, adopting a hierarchical device type classification that integrates both baseline radio capabilities and service-driven subtypes provides a scalable, extensible, and forward-compatible foundation for 6G, allowing future device generations and emerging service paradigms to be supported efficiently.

Proposal 5: Adopt a set of baseline device type categorizations plus delta features to ensure scalability, extensibility, and forward compatibility in 6G networks.

# 3 Conclusion

In this paper, we discuss on basic principles and guidelines on 6G design, new services in 6G, and 6G Device types. We have the following observations and proposals:

New services

Proposal 1: RAN2 should study the support of new services, e.g., ISAC, AI/ML-based services, token communication and immersive communication, taking NW & UE energy consumption and User experience (QoS/QoE) improvement into consideration.

Basic Principles for 6GR design

Proposal 2: 5G design as starting point for 6G with pragmatic simplification:

- Consolidation L2 sublayers to reduce redundancies across protocol layer

- Simplify RRC configuration and UE capability signaling

- Avoid excessive nesting of configurations (e.g. avoid BWPs as basic granularity in Physical Radio resource configuration)

Proposal 3: 5G design as starting point for 6G with extensible and flexible:

- Unified forward-compatible design principle for 6G access control, including cell barring mechanism, the definition of access categories and access identities;

- Unified RAN data collection framework for diversified data collected, e.g. AI and sensing;

- Unified mobility mechanism signalling structure with LTM signaling structure as the start point;

- Harmonized measurement framework having the capabilities of the L3- and L1-measurement;

- Unified multi-carrier framework in 6GR to enable efficient multi-carrier operations.

Proposal 4: It is proposed to define functional modules in RRC configuration, organizing the protocol not in "layers" but in "functional modules", such as "Mobility Management-Module", "Access Control-Module", “L2 Data Handling Module” and “Physical radio resource configuration-Module” and so on.

6G Device types

Observation 1: Defining baseline device types while allowing service-driven subtypes supports scalable capability management and reduces signalling overhead in L2 and L3 protocols.

Observation 2: Flexible, multi-dimensional categorization ensures forward compatibility, accommodating future AI-native, multi-modal, and non-terrestrial devices without revising the baseline specification.

Observation 3: Associating each device type with a baseline capability profile and delta reporting facilitates template-based configuration, simplifying RRC procedures and enabling consistent protocol interpretation across diverse device classes.

Proposal 5: Adopt a set of baseline device type categorizations plus delta features to ensure scalability, extensibility, and forward compatibility in 6G networks.

# 4 Reference

- Draft_RAN2_131bis_MeetingReport_v1.

- 3GPP TR 22.837 Study on Integrated Sensing and Communication (Release 19)

- 3GPP TS 22.137 Integrated Sensing and Communication (Release 19)

- 3GPP TR 38.914 Study on 6G Scenarios and requirements (Release 20)

- R2-2506909, Requirements on general AI/ML framework and data collection, CMCC.

- R2-2506900, Discussion on 6G control plane, CMCC.

- R2-2509077, Modular design for 6GR Protocol, CMCC, NTT DOCOMO, Turkcell, China Unicom, Nokia.

- 3GPP TR 22.870 Study on 6G Use Cases and Service Requirements (Release 20)