3GPP TSG-RAN WG2 Meeting #132	R2-2508954

Dallas, USA, Nov. 17th – 21st

Agenda Item:	8.2.2

Source:			ASUSTeK

Title:	Discussion on Ambient IoT open issues

Document for: 	Discussion and Decision

- Introduction

There are some remaining open issues in the post-meeting discussion after RAN2#131bis meeting. In this contribution, we would like to discuss some of the open issues.

- Discussion

2.1 		Issue 3-7: More data indication handling

In RAN2#131bis meeting [1], there were agreements on devices having no NAS responses to the reader.

- RAN2 confirms, in addition to delayed response, it is valid that in some cases A-IoT NAS doesn’t provide a response at all.

- For cases other than integrity failure, AS will indicate no NAS response expected to reader.   FFS how (e.g. using 0 SDU & MDI, or new indication).

- For integrity failure, for now RAN2 assumes that there is no AS response to the reader.  Ask SA3 ccCT1 whether a similar mechanism (e.g. AS response to the reader) can be used to indicate to reader no NAS response due to integrity failure.

To our understanding, in some cases, CT1 hadn’t decided whether there were NAS responses or not before CT1#157 meeting in October. After that, there were some clarifications regarding the issue.

Case 1: integrity check

It was agreed that, as described in C1-256631 [2], the AIoT NAS layer could indicate the lower layer in case of security failure, which is specified in TS 24.369 [3]:

### 4.2.3 Integrity protection of AIoT NAS signalling messages … In case of an AIoT NAS message fails the integrity check by the AIoT NAS, the AIoT device shall discard the AIoT NAS message and provide an indication to the lower layers. In case of an AIoT NAS message fails the integrity check by the AIoT NAS, the AIOTF shall discard the AIoT NAS message.

Meanwhile, in RAN2#131bis meeting, it was agreed to adopt no AS response approach for integrity check failure for now. To sum up, the intended behaviour for integrity check failure would be the AIoT device NAS layer provides an indication to AIoT device AS layer. After receiving the indication, the AIoT device AS layer does not send a response to the reader.

Case 2: unknown or unforeseen message type

It was agreed that, as described in C1-256647 [4], if the read or write command is not supported by the AIoT device, the STATUS message shall be sent with a cause. And the corresponding text is captured in TS 24.369 [3] as below:

#### 5.3.2.5 Read command procedure not accepted by AIoT device Upon receiving a READ COMMAND message, if the command type specific parameters (e.g. the offset to read application data from the AIoT Device user memory, the length of the application data to read.) is invalid, the AIoT device shall send a READ COMMAND REJECT message with a cause set to #1 "Command type specific parameters invalid". … #### 5.3.3.5 Write command procedure not accepted by AIoT device Upon receiving a WRITE COMMAND message, if the command type specific parameters (e.g. the offset where to write the application data, the length of the application data to write) is invalid, the AIoT device shall send a WRITE COMMAND REJECT message with a cause set to #1 "Command type specific parameters invalid". Upon receiving a WRITE COMMAND message, if the write command procedure cannot be implemented because the energy will be run out, the AIoT device shall send a WRITE COMMAND REJECT message with a cause set to #3 "Low energy". … ## 6.3 Unknown or unforeseen message type If the network receives an AIoT NAS message with a message type not defined for the AIoT NAS protocol or not implemented by the receiver, it shall ignore the AIoT NAS message. If the AIoT device receives an AIoT NAS message with a message type not defined for the AIoT NAS protocol or not implemented by the receiver, it shall return a STATUS message with cause #97 "message type non-existent or not implemented".

As a result, if the AIoT device NAS layer receives a NAS message with an unknown message type, the AIoT device NAS layer would indicate AIoT device AS layer to include the STATUS message in D2R message and send it to the reader. After receiving the D2R message, the reader would forward the NAS message to CN. As a result, no AS layer issue occurs for the unknown message type case.

Case 3: ignored NAS message

In the LS from CT1 C1-256623 [5], it is stated that the AIoT device shall provide an indication to the lower layer when a received AIoT NAS message was ignored. The related description is also captured in TS 24.369 [3]:

### 6.2.1 Message too short When a message is received that is too short to contain a complete message type information element, that message shall be ignored, c.f. 3GPP TS 24.007 [3], and the AIoT device shall provide an indication to the lower layers that the message was ignored.

Observation 1: No AS layer issue occurs for the unknown message type case.

Observation 2: For the case that integrity check fails and the case that AIoT NAS message is ignored, no NAS response is provided from the device to the reader.

We believe the case of ignored NAS message is not intended by the network, and it is supposed to be a corner case. However, during the previous RAN2 meeting, companies would like to indicate this case to the reader to avoid redundant energy consumption. Therefore, to tell apart between “delayed NAS response” and “no NAS response”, we prefer to reuse MDI with 0 SDU. The following solution can be considered:

- “0 SDU & MDI =0” for “no upper layer data available”.

- “0 SDU & MDI =1” for “no upper layer data available due to delay NAS response”.

Proposal 1: (Issue 3-7) The following solution can be considered: “0 SDU & MDI =0” represents “no upper layer data available”, and “0 SDU & MDI =1” represents “no upper layer data available due to delay NAS response”.

2.2 		Issue 6-1

In the LS C1-256624 [6], CT1 pointed out a scenario of an attacker sending false AIoT paging messages to the device. According to previous RAN2 agreement, the device would always respond to the new service indicated by the received paging message applicable for that device. Therefore, the attack would make the device abort any ongoing inventory or command indicated from the network.

We would like to confirm that the R19 device would suffer from this kind of attack, if it always responds to a new A-IoT paging message and abandon the ongoing procedure. In RAN2#130 meeting, this approach was adopted because RAN2 thought it would not cause problem and was simpler to implement. Since there is actually security concerns, RAN2 may need to reconsider the alternative that the device ignores any paging request until the ongoing A-IoT procedure is considered completed.

However, it may be too late to handle this issue in release 19. Since it is expected to design A-IoT procedure considering multi-reader scenario in release 20, RAN2 could postpone the discussion to R20 A-IoT instead.

Proposal 2: (Issue 6-1) RAN2 considers the following two alternatives:

- Alt A: RAN2 reconsiders to ignore any paging request until the A-IoT procedure is considered completed when a device has an ongoing A-IoT procedure.

- Alt B: RAN2 postpones the false paging attack issue to release 20.

- Conclusion

We have following observations and proposals for A-IoT remaining issues:

Observation 1: No AS layer issue occurs for the unknown message type case.

Observation 2: For the case that integrity check fails and the case that AIoT NAS message is ignored, no NAS response is provided from the device to the reader.

Proposal 1: (Issue 3-7) The following solution can be considered: “0 SDU & MDI =0” represents “no upper layer data available”, and “0 SDU & MDI =1” represents “no upper layer data available due to delay NAS response”.

Proposal 2: (Issue 6-1) RAN2 considers the following two alternatives:

- Alt A: RAN2 reconsiders to ignore any paging request until the A-IoT procedure is considered completed when a device has an ongoing A-IoT procedure.

- Alt B: RAN2 postpones the false paging attack issue to release 20.

- Reference

- Report of 3GPP TSG RAN WG2 meeting #131bis, Prague, Czech Republic

- C1-256631, “Clarification on integrity verification failure”

- TS 24.369, v1.1.0, “Ambient IoT Non-Access-Stratum (AIoT NAS) protocol for 5G System (5GS); Stage 3; (Release 19)”

- C1-256647, “Pseudo-CR on "Addition of status procedure"”

- C1-256623, “LS on no Command Response in NAS”

- C1-256624, “LS on handling of inventory and command collision”