3GPP TSG-RAN WG2 Meeting #132	R2-2509018

# Dallas, US, 17-21 November 2025

Agenda Item:	10.3.2.1

Source:	MediaTek Inc.

Title:	RRC connection management and structure

Document for:	Discussion, decision

# 1	Introduction

This document discusses several aspects of connection management and RRC structure for 6G.

# 2	Discussion

## 2.1	Connection Management Procedures

Connection management is the key function of RRC, so we see it as important to study it early in the SI in order to determine whether 5G RRC connection management procedures are suitable for re-use for 6G RRC or if they require fundamental updates/improvements. We list a core set of connection management procedures which should be studied for 6G RRC below.

Proposal 1	RAN2 to study the following core list of connection management procedures of 6G RRC:

- RRC connection establishment;

- RRC connection reconfiguration;

- RRC connection re-establishment for RLF/error recovery;

- RRC connection release (including UE initiated RRC connection release).

Other procedures not directly related to connection management will emerge over time. RRC suspend/resume is not listed at this point, because of the uncertainty on whether/how RRC_INACTIVE will be realized in 6G. The point on UE initiated RRC connection release is further discussed in the next section.

## 2.2	UE Initiated RRC Connection Release

In RAN2#131bis, RAN2 agreed to study RRC_CONNECTED state for 6G RRC [1]:

Based on the above, we think RRC connection management procedures related to state transitions to/from RRC_CONNECTED state should be studied further in order to determine whether the 5G RRC procedures are suitable for re-use for 6G RRC as-is or if they require fundamental updates/improvements. In this section, we discuss RRC release procedure.

One 5G pain point is the lack of UE initiated RRC connection release procedure, where the UE notifies the network that the UE will leave RRC_CONNECTED state. Without such procedure, the UE performs a local RRC connection release procedure upon encountering an unrecoverable error scenario (such as reception of an unsupported configuration prior to AS security activation, or a failure to re-establish the RRC connection), which causes a temporary desync of the UE RRC state between the network and the UE (i.e., the network still thinks the UE is in RRC_CONNECTED whereas the UE has already left RRC_CONNECTED). Furthermore, since power consumption is an important aspect for the UE, the UEs in practice implement proprietary mechanisms in order to achieve the power consumption targets and such mechanisms also include local RRC connection release, typically as a last resort. For the UE power saving, a UE initiated RRC connection release procedure would be especially useful, because the UE knows related aspects not known by the network, such as:

Battery level of the UE;

User's preference for power/battery saving (e.g., a power saving mode is enabled by the user);

Whether further data transfer is expected (e.g., an application was closed/terminated by the user).

5G RRC includes UEAssistanceInformation based procedure where the UE can indicate its preference to leave RRC_CONNECTED for power saving purpose, but the feature relies on availability of network implementation. Furthermore, even if the UE indicated preference to leave RRC_CONNECTED, it does not mandate the network to release the RRC connection.

5G RRC also includes UE initiated RRC connection release procedure for MUSIM purposes. This procedure also relies on UEAssistanceInformation, but the difference to the previous procedure is that the UE leaves the RRC_CONNECTED latest on expiry of related guard timer T346g, regardless of whether the network reacts on UE's indication or not. But, also this feature relies on availability of network implementation and is not deployed in practice; it is also limited for MUSIM purposes by definition.

Observation 1	5G RRC does not include practical procedure for the UE to leave RRC_CONNECTED, which would be especially benefial for UE power saving purposes. Without such procedure, local RRC connection release by the UE causes a temporary desync of the UE RRC state between the network and the UE.

Based on the discussion above, we propose to specify a UE initiated RRC connection release procedure for 6G RRC. The procedure should be 'always available', i.e., the initiation of the procedure by the UE should not depend on enabling of the feature by a configuration parameter sent by the network. Furthermore, the procedure should be based on a principle of the UE indicating the network (rather than requesting a permission) that the UE is going to leave RRC_CONNECTED; in this sense, the 5G RRC's procedure to leave RRC_CONNECTED state for MUSIM purposes could be used as a baseline for further study of the procedure.

Proposal 2	RAN2 to study UE initiated RRC connection release procedure for 6G RRC. The procedure may be initiated for power saving purposes by the UE, but may also be applied for other scenarios where avoidance of a temporary desync of the UE RRC state between the network and the UE provides benefit.

## 2.3	RRC Structure

There have been suggestions that 6G RRC could be split into “upper” and “lower” parts, with different parts of the UE configuration managed and signalled separately. We see this approach as inviting complexity without a clear benefit: Configurations would need to be coordinated between the two sublayers, some functions might be expected to span both parts, and messages would terminate in different ways (potentially at different nodes on the network side in case of a disaggregated RAN architecture) depending on whether they were for “upper” or “lower” RRC.

Proposal 3	6G RRC comprises a single sublayer (no upper/lower split).

Note that proposal 3 does not speak one way or the other to the potential “modularity” of the RRC protocol.  We are open to the possibility of dividing the RRC protocol into functional modules for implementation purposes (e.g., separate ASN.1 modules with cross-references), but from the standpoint of the protocol stack and the termination points, RRC should remain unified.

# 3	Conclusion

Based on the discussion in the previous sections we have following observations and proposals:

Observation 1	5G RRC does not include practical procedure for the UE to leave RRC_CONNECTED, which would be especially benefial for UE power saving purposes. Without such procedure, local RRC connection release by the UE causes a temporary desync of the UE RRC state between the network and the UE.

Proposal 1	RAN2 to study the following core list of connection management procedures of 6G RRC:

- RRC connection establishment;

- RRC connection reconfiguration;

- RRC connection re-establishment for RLF/error recovery;

- RRC connection release (including UE initiated RRC connection release).

Proposal 2	RAN2 to study UE initiated RRC connection release procedure for 6G RRC. The procedure may be initiated for power saving purposes by the UE, but may also be applied for other scenarios where avoidance of a temporary desync of the UE RRC state between the network and the UE provides benefit.

Proposal 3	6G RRC comprises a single sublayer (no upper/lower split).

# 4	References

- R2-250xxxx: “Report of 3GPP RAN2 WG2 meeting #131bis, Prague, Czech Republic”, ETSI MCC, RAN2#131bis