3GPP TSG-RAN WG2 Meeting # 132	R2-2509038

# Dallas, USA, 17th – 21st November 2025

Source:	Google

Title:	Discussion on 6G Mobility

Agenda item:	10.4

Document for:	Discussion & Decision

# 1. Introduction

RAN2 made the following agreements in RAN2#131bis meeting.

Agreements

1	Study mobility with the following requirements in 6G mobility design:

-	Minimize interruption time and ensure service continuity (i.e. minimize throughput degradation during mobility). Consider complexity and gains when discussing solutions.

-	Robustness of mobility procedures

-	Energy efficiency for both UE and NW

2	Study aspects related to mobility (e.g. early DL/UL synchronization, UE configuration processing, pre-configurations, conditional handover, early CSI acquisition)

This contribution presents an analysis of 5G New Radio (NR) mobility mechanisms, which are considered potential candidates for 6G.

# 2. Potential 6G Mobility Mechanisms

## 2.1 Baseline mobility mechanisms

The 5G NR system introduced several crucial mobility mechanisms, including Redirection, Layer 3 Handover, Conditional Handover (CHO), Dual Active Protocol Stack (DAPS), RACH-less HO, and Lean Mobility (LTM). For 6G, the selection of supported mechanisms must balance complexity, reliability, and the need to meet the extremely stringent KPIs.

Given that 6G is projected to support an increasingly diverse range of devices, including massive IoT devices with limited processing power and battery life, the most power- and complexity-efficient handover methods are critical for broad device compatibility and network efficiency. Layer 3 handover and redirection are the foundational mechanisms for mobility with the lowest complexity compared to other mobility mechanisms.

Proposal 1: 	Layer 3 handover and redirection should be supported in 6G as baseline mobility mechanisms.

## 2.2 Zero millisecond mobility interruption time

As shown in [2], 0 ms mobility interruption time is required for 6G. All make-after-break- handover mechanisms don’t fulfil such a strict requirement. To satisfy the 0 ms mobility interruption time and reliable support for high-speed mobility, a make-before-break (MBB) handover mechanism is essential. This is the only way to guarantee service continuity during the transition phase.

Proposal 2: 	6G should support a "make-before-break" handover mechanism to meet 0 ms interruption time.

5G NR’s DAPS is a MBB mechanism that meets the 0 ms interruption goal and has been specified since Release 16. Despite this, DAPS has had zero field deployment in 5G NR and LTE. This may indicate that the 5G NR’s DAPS has significant implementation complexity. As pointed out by many companies during 6G Workshop and RAN Plenary meetings, lessens learnt from 5G deployment should be considered to ensure 6G success. In the last RAN meetings, many companies expressed that they don’t like DAPS either. Given that, RAN2 should study other mechanisms that can meet 0 ms interruption time.

Proposal 3: 	RAN2 should study non-DAPS mobility mechansims that can meet 0 ms interruption time.

## 2.3 Minimize mobility interruption time in make-after-break HO

Some services (e.g., HRLLC services) demand 0 ms interruption time, which necessitates Make-Before-Break solutions (like DAPS). Conversely, a wider range of services are highly sensitive to latency but do not require the absolute zero-interruption guarantee. For these, Layer 3 Handover is insufficient, requiring mechanisms that drastically shorten the handover interruption window. In 5G NR, RACH-less Handover, Conditional Handover (CHO), and L1/L2 Triggered Mobility (LTM) were designed to address this low-latency requirement.

A review of commercial 5G implementation reveals a crucial pattern related to deployment complexity:

- CHO: Specified since Release 16, CHO has seen little to no commercial field deployment in 5G NR. Similar to the deployment challenges faced by DAPS, this lack of adoption strongly suggests that the complexity required for its implementation outweighs the performance benefit in a commercial environment.

- RACH-less Handover: While NR RACH-less Handover was introduced in Release 18, its LTE counterpart (specified in Release 14) has also never achieved widespread deployment. This non-deployment is a strong indicator that the inherent complexity of the RACH-less procedure makes it commercially unattractive.

- LTM: Specified from Release 18, LTM currently lacks extensive field data. Crucially, however, LTM is fundamentally an optimized, performance-enhanced evolution that consolidates the best aspects of the prior mechanisms into a more streamlined framework:

  - RACH-less LTM provides a more efficient and simplified replacement for the legacy RACH-less Handover.

  - Conditional LTM offers functionality highly similar to CHO but with the potential for simpler signaling and superior, consistent performance.

Based on the RAN-endorsed working principle for 6G (RP-250766) which mandates minimizing the adoption of multiple options for the same functionality, RAN2 should prioritize the most efficient and least complex set of features. Given that LTM and conditional LTM function as more efficient replacements for RACH-less Handover and CHO, respectively, we propose harmonizing around the LTM framework.

Proposal 4	LTM and conditional LTM should be supported in 6G, streamlined mechanisms for low-latency mobility.

Proposal 5	CHO and RACH-less handover should be excluded from 6G to reduce specification complexity and avoid standardizing features already proven commercially non-viable in 5G and LTE.

# 3. Conclusion

This contribution analyzes the 5G NR mobility mechanisms against the demanding performance targets of 6G and provides the following proposals:

Proposal 1: 	Layer 3 handover and redirection should be supported in 6G as baseline mobility mechanisms.

Proposal 2: 	6G should support a "make-before-break" handover mechanism to meet 0ms interruption time.

Proposal 3: 	RAN2 should study non-DAPS mobility mechansims that can meet 0ms interruption time.

Proposal 4	LTM and conditional LTM should be supported in 6G, streamlined mechanisms for low-latency mobility.

Proposal 5	CHO and RACH-less handover should be excluded from 6G to reduce specification complexity and avoid standardizing features already proven commercially non-viable in 5G and LTE.

# 4. References

[1] RAN2#131bis Chair note

[2] RP-252882 3GPP TR 38.914 V0.2.0, "Study on 6G Scenarios and Requirements"

# 5. Annex 6G mobility performance targets

The following is excerpted from [2].

### 5.1.11	Mobility

Mobility is the maximum mobile station speed at which a defined QoS can be achieved (in km/h).

The following classes of mobility are defined:

-	Stationary: 0 km/h

-	Pedestrian: 0 km/h to 10 km/h

-	Vehicular: 10 km/h to 120 km/h

-	High speed vehicular: 120 km/h to 500 km/h

-	Airplane: 500 km/h to 1000/1200 km/h

### 5.1.12	Mobility interruption time

Mobility interruption time shall be sufficiently minimized to support a seamless application performance. The minimum requirement for mobility interruption time is 0 ms at least for mobility between TRxPs of a single base station. The proponent can report the Mobility interruption time in other scenarios, where applicable.