3GPP TSG-RAN WG2#83bis	R2-133239

Oct 7--11, 2013, Ljubljana, Slovenia.

Agenda Item:	7.5.1

Source: 	Motorola Mobility

Title:  	Discovery for Device to Device Communication

Document for:	Discussion and Decision

# Introduction

A study item for Device to Device (D2D) Communication has been initiated by 3GPP RAN[1]. RAN2 is expected to design functionality related to discovery and resource allocation for D2D. Mutual discovery of D2D devices is essential to enable D2D communication. Regardless of what communication path is used, the discovery process needs to be power efficient and scalable.

This contribution outlines our views on some of the points of discussion for design of D2D discovery.

# Discussion

## UE States in which Discovery beacon can be transmitted

Transmission of discovery beacon enables other UEs to detect proximity of the transmitting UE and its support of D2D services. As long as resources for beacon transmission are known to the UE, there is nothing inherent in the beacon transmission procedure that requires the UE to be in connected mode. In fact, restricting beacon transmissions to connected mode is not desirable, because this would imply that the UE has to establish a connection to the network (and then follow all connected mode procedures related measurements, mobility etc) even if it is only interested in D2D communication with a nearby UE.

Proposal 1: Transmission of D2D beacon should be allowed in both Idle and Connected modes.

## UE States in which Discovery beacon can be received

Similar to the beacon transmission case, as long as beacon transmission resources are known to the UE, we see no reason to force the UE to be in connected mode to search for and receive beacons. Forcing the UE to be in connected mode for beacon reception implies that whenever the UE is scanning for beacons (which could be extended periods), the UE has to remain connected.

Proposal 2: Reception of D2D beacon should be allowed in both Idle and Connected modes.

Note that performing D2D beacon transmission or reception in idle mode does not imply anything about the UE states in which D2D communication can occur. That is, after discovery, the UE may need to establish a connection to the network before it can perform D2D communication.

## Discovery procedure and Network assistance

Resources for transmission of discovery beacon can be indicated by the network in a System Information Block, as this can support both connected mode and idle mode UEs. It is very important that there is a well defined and small time window for beacon transmissions. Smaller the time window, lower the power consumption at UEs scanning for beacons. While the design of the discovery signal itself is expected to be done in RAN1, RAN2 is expected to design the discovery procedure.

We think a simple approach for discovery should be adequate. For example, the network can signal a small contiguous sequence of sub-frames for beacon transmissions. A UE that intends to transmit a beacon can randomly select a sub-frame (and possibly resource blocks) within the window and transmit the beacon. A UE scanning for discovery beacons may scan during the entire sequence of sub-frames. This avoids complicated approaches such as scanning before transmission of discovery beacon. There can be delays caused by collisions of beacon transmissions of devices and there is a trade-off between collisions and the time window for beacon transmissions. As a first step, RAN2 needs to clearly understand the time limit for discovery under different conditions.

Proposal 3: RAN2 needs to first discuss and agree on a time limit for the discovery process.

## Information in beacon

The purpose of the discovery beacon and the discovery procedure is to enable applications on a device to find a peer application on another device and establish communication. Examples of such applications include social media applications and critical emergency communication applications. The discovery process has to provide the device that is performing the discovery some information that is necessary to decide whether to establish communication with the discovered device. Such information could include: (a) an identifier of the device or the group the device belongs to, (b) Services & Applications supported, (c) location of the device, (d) transmit power of the device. While such information can be obtained through a request/response interaction between the two devices, we think it is natural to include some or all of this information in the discovery beacon itself, to avoid excessive and unnecessary request/response communications.

Proposal 3: The discovery beacon can optionally include the following information about the device performing the discovery beacon transmission:

- An identifier of the device or an identifier of a group the device belongs to;

- Services & Applications supported by the device;

- Location of the device; and

- Transmit power of the device.

## Coordination among cells for Discovery

Coordination between cells is necessary to have an efficient discovery procedure. If the discovery beacon transmission windows of neighbour cells overlap, there can be collisions of beacon transmissions of UEs in different cells, increasing the collision rate. Coordination should ensure that discovery window transmission beacons of neighbour cells do not overlap.

However, it is also necessary to enable a UE attached to cell1 and at the edge of cell1 to discover a UE in neighbouring cell cell2. Information of the discovery beacon transmission window of neighbour cells can be provided to the UE or the UE may be able to obtain it on its own. Overall, this procedure localizes the discovery such that devices that are not close to a neighbour cell do not look for beacons of devices in neighbouring cells and do not experience collision from beacon transmissions in neighbour cells. Devices that are close to the neighbour cell can look for beacons in the time window corresponding its cell and that of the neighbouring cell.

Proposal 4: Cells should be coordinated such that beacon transmissions of UEs in neighbouring cell do not overlap. Beacon transmission resources corresponding to neighbour cells should be provided to the UE.

# Summary

In this contribution we have considered some of the issues related to design of the D2D discovery procedure: UE states in which discovery can be performed, procedure for discovery, contents of the discovery beacon and coordination in the network to enable discovery. Our proposals are below.

Proposal 1: Transmission of D2D beaconss should be allowed in both Idle and Connected modes.

Proposal 2: Reception of D2D beacon should be allowed in both Idle and Connected modes.

Proposal 3: RAN2 needs to first discuss and agree on a time limit for the discovery process.

Proposal 3: The discovery beacon can optionally include the following information about the device performing the discovery beacon transmission:

- An identifier of the device or an identifier of a group the device belongs to;

- Services & Applications supported by the device;

- Location of the device; and

- Transmit power of the device.

Proposal 4: Cells should be coordinated such that beacon transmissions of UEs in neighbouring cell do not overlap. Beacon transmission resources corresponding to neighbour cells should be provided to the UE.

# References

- RP-122009, “Study on LTE Device to Device Proximity”.

- 3GPP TR 22.803v12.2.0, “Feasibility study for Proximity Services (ProSe).